3GPP TSG RAN WG1 #101							R1-20xxxxx

e-Meeting, May 25th – June 5th, 2020

______________________________________________________________________Agenda item: 7.2.4.5

Source: Moderator (LG Electronics)

Title: Text proposal from Email discussion thread #3 for AI 7.2.4.5 Physical layer procedures for sidelink

Document for: Discussion and decision

- Introduction and proposal

RAN1 made the agreements copied in Appendix in [101-e-NR-5G_V2X_NRSL-SL_PHY_Procedure-03] Email discussion/approval regarding HARQ operation + Sidelink CSI. This contribution includes the text proposal endorsed. It is proposed to adopt the text proposal in Section 2 for the following reasons

- Reason for change: RAN1 made the agreements to complete the UE procedure for sidelink HARQ operation and CSI reporting. The text proposal is to implement these agreements.

- Summary of change: SCI formats 2-A and 2-B and the related UE procedure are updated to introduce the fields for HARQ feedback enabling/disabling indication and the cast type indication. CQI table determination is introduced based on the indicated MCS table.

- Consequences if not approved: The specification is incomplete in supporting sidelink HARQ and CSI report.

- Text proposal

  - Text proposal for TS 38.212

===========================<Start of change #1>=======================

## 8.4	Sidelink control information on PSSCH

< Unchanged parts are omitted >

### 8.4.1	2nd-stage SCI formats

< Unchanged parts are omitted >

#### 8.4.1.1	SCI format 2-A

SCI format 2-A is used for the decoding of PSSCH, with HARQ operation when HARQ-ACK information includes ACK or NACK, or when there is no feedback of HARQ-ACK information.

The following information is transmitted by means of the SCI format 2-A:

-	HARQ Process ID – [x] bits as defined in clause 16.4 of [5, TS 38.213].

-	New data indicator – 1 bit as defined in clause 16.4 of [5, TS 38.213].

-	Redundancy version – 2 bits as defined in clause x.x.x of [6, TS 38.214].

-	Source ID – 8 bits as defined in clause x.x.x of [6, TS 38.214].

-	Destination ID – 16 bits as defined in clause x.x.x of [6, TS 38.214].

-  HARQ feedback enabling/disabling indicator – 1 bit as defined in clause 16.3 of [5, TS 38.213].

-  Cast type indicator – 2 bits as defined in Table 8.4.1.1-1.

-	CSI request – 1 bit as defined in clause 8.2.1 of [6, TS 38.214].

Table 8.4.1.1-1: Cast type indicator

| Value of Cast type indicator | Cast type |
| --- | --- |
| 00 | Broadcast |
| 01 | Groupcast |
| 10 | Unicast |
| 11 | Reserved |

#### 8.4.1.2	SCI format 2-B

SCI format 2-B is used for the decoding of PSSCH, with HARQ operation when HARQ-ACK information includes only NACK.

The following information is transmitted by means of the SCI format 2-B:

-	HARQ Process ID – [x] bits as defined in clause 16.4 of [5, TS 38.213].

-	New data indicator – 1 bit as defined in clause 16.4 of [5, TS 38.213].

-	Redundancy version – 2 bits as defined in clause x.x.x of [6, TS 38.214].

-	Source ID – 8 bits as defined in clause x.x.x of [6, TS 38.214].

-	Destination ID – 16 bits as defined in clause x.x.x of [6, TS 38.214].

-  HARQ feedback enabling/disabling indicator – 1 bit as defined in clause 16.3 of [5, TS 38.213].

-	Zone ID – 12 bits as defined in clause x.x.x of [9, TS 38.331].

-	Communication range requirement – 4 bits as defined in clause x.x.x of [9, TS 38.331]

< Unchanged parts are omitted >

============================<End of change #1>=======================

  - Text proposal for TS 38.213

===========================<Start of change #1>=======================

## 16.3	UE procedure for reporting HARQ-ACK on sidelink

A UE can be indicated by an SCI format scheduling a PSSCH reception, in one or more sub-channels from a number of Nsubch PSSCH sub-channels, to transmit a PSFCH with HARQ-ACK information in response to the PSSCH reception. The UE provides HARQ-ACK information that includes ACK or NACK, or only NACK.

A UE can be provided, by periodPSFCHresource, a number of slots in a resource pool for a period of PSFCH transmission occasion resources. If the number is zero, PSFCH transmissions from the UE in the resource pool are disabled.

A UE may be indicated by higher layers to not transmit a PSFCH in response to a PSSCH reception [11, TS 38.321].

If a UE receives a PSSCH in a resource pool and a ZYX the HARQ feedback enabling/disabling indicator field = 1 in the associated a SCI format 0_2-A or SCI format 2-B scheduling the PSSCH reception indicates to the UE to report HARQ-ACK information for the PSSCH reception [5, TS 38.212], the UE provides the HARQ-ACK information in a PSFCH transmission in the resource pool. The UE transmits the PSFCH in a first slot that includes PSFCH resources and is at least a number of slots, provided by MinTimeGapPSFCH, of the resource pool after a last slot of the PSSCH reception.

< Unchanged parts are omitted >

A UE determines a mcs value, for computing a value of cyclic shift α [4, TS 38.211], as in Table 16.3-2 if the UE receives a SCI format 2-A with Cast type indicator field is 01 or 10 or Table 16.3-3 if the UE receives a SCI format 2-B as indicated by a SCI format scheduling a PSSCH reception. The UE applies one cyclic shift from a cyclic shift pair to a sequence used for the PSFCH transmission [4, TS 38.211].

Table 16.3-2: Mapping of HARQ-ACK information bit values to a cyclic shift, from a cyclic shift pair, of a sequence for a PSFCH transmission when HARQ-ACK information includes ACK or NACK

| HARQ-ACK Value | 0 (NACK) | 1 (ACK) |
| --- | --- | --- |
| Sequence cyclic shift | 0 | 6 |

Table 16.3-3: Mapping of HARQ-ACK information bit values to a cyclic shift, from a cyclic shift pair, of a sequence for a PSFCH transmission when HARQ-ACK information includes only NACK

| HARQ-ACK Value | 0 (NACK) | 1 (ACK) |
| --- | --- | --- |
| Sequence cyclic shift | 0 | N/A |

============================<End of change #1>=======================

===========================<Start of change #2>=======================

## 16.5	UE procedure for reporting HARQ-ACK on uplink

A UE can be provided PUCCH resources or PUSCH resources [12, TS 38.331] to report HARQ-ACK information that the UE generates based on HARQ-ACK information that the UE obtains from PSFCH receptions, or from absence of PSFCH receptions.

For SL configured grant Type 1 or Type 2 PSSCH receptions by a UE within a time period provided by periodSlCG, the UE generates one HARQ-ACK information bit in response to the PSFCH receptions to multiplex in a PUCCH transmission occasion that is after a last time resource, in a set of time resources.

For each PSFCH reception occasion, from a number of PSFCH reception occasions that the UE generates HARQ-ACK information to report in a PUCCH or PUSCH transmission, the UE can be indicated by higher layers SCI to perform one of the following and the UE constructs a HARQ-ACK codeword with HARQ-ACK information, when applicable.

If the UE receives PSFCH associated with a SCI format 2-A with Cast type indicator = 10,

-	generate HARQ-ACK information with same value as a value of HARQ-ACK information the UE determines from a PSFCH reception in the PSFCH reception occasion and, if the UE determines that a PSFCH is not received at the PSFCH reception occasion, generate NACK

If the UE receives PSFCH associated with a SCI format 2-A with Cast type indicator = 01,

-	generate ACK when the UE determines ACK from each PSFCH reception for the number of PSFCH reception occasions; otherwise, generate NACK if the UE determines absence of PSFCH reception or determines a NACK value from a PSFCH reception at a corresponding PSFCH reception occasion

-	generate ACK when the UE determines ACK from at least one PSFCH reception for the number of PSFCH reception occasions of a PSFCH resource with an index with MID, as determined in Clause 16.3, for every identity MID of the UEs expected to receive the PSSCH, as indicated by higher layers; otherwise, generate NACK.

If the UE receives PSFCH associated with a SCI format 2-B,

-	generate ACK when the UE determines absence of PSFCH reception for each PSFCH reception occasion from the number of PSFCH reception occasions; otherwise, generate NACK

-	generate ACK when the UE determines ACK from at least one PSFCH reception for the number of PSFCH reception occasions of a PSFCH resource with an index with MID, as determined in Clause 16.3, for every identity MID of the UEs expected to receive the PSSCH, as indicated by higher layers; otherwise, generate NACK.

The UE generates NACK when, due to prioritization, as described in Clause 16.2.4, the UE does not receive PSFCH in any PSFCH reception occasion associated with a PSSCH transmission in a resource provided by a DCI format 3_0 with CRC scrambled by a SL-RNTI or, for a configured grant, in a resource provided in a single period and for which the UE is provided a PUCCH resource to report HARQ-ACK information.

The UE generates NACK when, due to prioritization as described in Clause 16.2.4, the UE does not transmit a PSSCH in any of the resources provided by a DCI format 3_0 with CRC scrambled by SL-RNTI or, for a configured grant, in any of the resources provided in a single period and for which the UE is provided a PUCCH resource to report HARQ-ACK information. The UE generates ACK if the UE does not transmit a PSCCH with a SCI format 0_1 scheduling a PSSCH in any of the resources provided by a configured grant in a single period and for which the UE is provided a PUCCH resource to report HARQ-ACK information.

============================<End of change #2>=======================

  - Text proposal for TS 38.214

===========================<Start of change #1>=======================

### 8.5.2	Channel state information

#### 8.5.2.1	CSI reporting quantities

#### 8.5.2.1.1	Channel quality indicator (CQI)

The UE shall derive CQI as specified in section 5.2.2.1, with the following changes

-	PDSCH replaced by PSSCH

-	uplink slot replaced by sidelink slot

-	downlink physical resource blocks replaced by sidelink physical resource blocks

-	Transport Block Size determination according to Clause 8.1.3.2

-	CSI reference resource according to TODO Clause 8.5.2.3

-	interference measurements are not supported

-	sub-band differential CQI is not supported

-  cqi-Table is determined as indicated by Additional MCS table indicator in a SCI format 1-A,

-  cqi-Table = ‘table1’ if Additional MCS table indicator indicates Table 5.1.3.1-1,

-  cqi-Table = ‘table2’ if Additional MCS table indicator indicates Table 5.1.3.1-2,

-  cqi-Table = ‘table3’ if Additional MCS table indicator indicates Table 5.1.3.1-3

============================<End of change #1>=======================

Appendix: Agreements made in the email discussion [101-e-NR-5G_V2X_NRSL-SL_PHY_Procedure-03]

Agreements:

- SCI format 2-A includes an explicit indication of HARQ feedback enabled/disabled.

- SCI format 2-B includes an explicit indication of HARQ feedback enabled/disabled.

Agreements:

- SCI format 2-A includes a 2-bit information field providing an explicit indication for the cast type

Conclusion:

- It is feasible from L1 signaling perspective to use Groupcast option 1 (i.e., NACK only feedback) when Zone ID or Communication range requirement is not provided, if RAN2 decides to support this operation.

  - No action in RAN1 unless RAN2 informs RAN1 about their decision (to support or not)

  - Note that if RAN2 decides to support it, RAN1 needs to further discuss

Agreemensts:

- Send an LS to RAN2 in response to R1-2003255 to inform

  - The conclusion on Groupcast option 1 when Zone ID or Communication range requirement is not provided.

  - The agreement as above

Agreements:

- The CQI table is derived based on the indicated MCS table

  - No separate configuration