An email discussion started with the following scope:

[101-e-NR-5G_V2X_NRSL-SL_PHY_Procedure-03] Email discussion/approval regarding HARQ operation + Sidelink CSI

  - Issue 3-1: Details of indicating SL HARQ feedback related information

  - Issue 4-2: How to determine the CQI table used for CSI reporting

Till 5/28, with potential TPs by 6/3 – Hanbyul (LGE)

Relevant agreements are as follows:

Agreements:

- SCI format 2-A includes an explicit indication of HARQ feedback enabled/disabled.

- SCI format 2-B includes an explicit indication of HARQ feedback enabled/disabled.

Agreements:

- SCI format 2-A includes a 2-bit information field providing an explicit indication for the cast type

Conclusion:

- It is feasible from L1 signaling perspective to use Groupcast option 1 (i.e., NACK only feedback) when Zone ID or Communication range requirement is not provided, if RAN2 decides to support this operation.

  - No action in RAN1 unless RAN2 informs RAN1 about their decision (to support or not)

  - Note that if RAN2 decides to support it, RAN1 needs to further discuss

Agreemensts:

- Send an LS to RAN2 in response to R1-2003255 to inform

  - The conclusion on Groupcast option 1 when Zone ID or Communication range requirement is not provided.

  - The agreement as above

Agreements:

- The CQI table is derived based on the indicated MCS table

  - No separate configuration

Text proposals

TP for 38.212

## 8.4 Sidelink control information on PSSCH < Unchanged parts are omitted > ### 8.4.1 2nd-stage SCI formats < Unchanged parts are omitted > #### 8.4.1.1 SCI format 2-A SCI format 2-A is used for the decoding of PSSCH, with HARQ operation when HARQ-ACK information includes ACK or NACK, or when there is no feedback of HARQ-ACK information. The following information is transmitted by means of the SCI format 2-A: - HARQ Process ID – [x] bits as defined in clause 16.4 of [5, TS 38.213]. - New data indicator – 1 bit as defined in clause 16.4 of [5, TS 38.213]. - Redundancy version – 2 bits as defined in clause x.x.x of [6, TS 38.214]. - Source ID – 8 bits as defined in clause x.x.x of [6, TS 38.214]. - Destination ID – 16 bits as defined in clause x.x.x of [6, TS 38.214]. - HARQ feedback enabling/disabling indicator – 1 bit as defined in clause 16.3 of [5, TS 38.214] - Cast type indicator – 2 bits as defined in Table 8.4.1.1-1. - CSI request – 1 bit as defined in clause 8.2.1 of [6, TS 38.214]. Table 8.4.1.1-1: Cast type indicator Value of Cast type indicator; Cast type. 00; Broadcast. 01; Groupcast. 10; Unicast. 11; Rerserved. #### 8.4.1.2 SCI format 2-B SCI format 2-B is used for the decoding of PSSCH, with HARQ operation when HARQ-ACK information includes only NACK. The following information is transmitted by means of the SCI format 2-B: - HARQ Process ID – [x] bits as defined in clause 16.4 of [5, TS 38.213]. - New data indicator – 1 bit as defined in clause 16.4 of [5, TS 38.213]. - Redundancy version – 2 bits as defined in clause x.x.x of [6, TS 38.214]. - Source ID – 8 bits as defined in clause x.x.x of [6, TS 38.214]. - Destination ID – 16 bits as defined in clause x.x.x of [6, TS 38.214]. - Zone ID – 12 bits as defined in clause x.x.x of [9, TS 38.331]. - Communication range requirement – 4 bits as defined in clause x.x.x of [9, TS 38.331] - HARQ feedback enabling/disabling indicator – 1 bit as defined in clause 16.3 of [5, TS 38.214] < Unchanged parts are omitted >

TP for 38.213

## 16.3 UE procedure for reporting HARQ-ACK on sidelink A UE can be indicated by an SCI format scheduling a PSSCH reception, in one or more sub-channels from a number of Nsubch PSSCH sub-channels, to transmit a PSFCH with HARQ-ACK information in response to the PSSCH reception. The UE provides HARQ-ACK information that includes ACK or NACK, or only NACK. A UE can be provided, by periodPSFCHresource, a number of slots in a resource pool for a period of PSFCH transmission occasion resources. If the number is zero, PSFCH transmissions from the UE in the resource pool are disabled. A UE may be indicated by higher layers to not transmit a PSFCH in response to a PSSCH reception [11, TS 38.321]. If a UE receives a PSSCH in a resource pool and a ZYX HARQ feedback enabling/disabling indicator field = 1 in a SCI format 0_2-A or SCI format 2-B scheduling the PSSCH reception indicates to the UE to report HARQ-ACK information for the PSSCH reception [5, TS 38.212], the UE provides the HARQ-ACK information in a PSFCH transmission in the resource pool. The UE transmits the PSFCH in a first slot that includes PSFCH resources and is at least a number of slots, provided by MinTimeGapPSFCH, of the resource pool after a last slot of the PSSCH reception. < Unchanged parts are omitted > A UE determines a mcs value, for computing a value of cyclic shift α [4, TS 38.211], as in Table 16.3-2 if the UE receives a SCI format 2-A with Cast type indicator field is 01 or 10 or Table 16.3-3 if the UE receives a SCI format 2-B as indicated by a SCI format scheduling a PSSCH reception. The UE applies one cyclic shift from a cyclic shift pair to a sequence used for the PSFCH transmission [4, TS 38.211]. Table 16.3-2: Mapping of HARQ-ACK information bit values to a cyclic shift, from a cyclic shift pair, of a sequence for a PSFCH transmission when HARQ-ACK information includes ACK or NACK HARQ-ACK Value; 0 (NACK); 1 (ACK). Sequence cyclic shift; 0; 6. Table 16.3-3: Mapping of HARQ-ACK information bit values to a cyclic shift, from a cyclic shift pair, of a sequence for a PSFCH transmission when HARQ-ACK information includes only NACK HARQ-ACK Value; 0 (NACK); 1 (ACK). Sequence cyclic shift; 0; N/A. < Unchanged parts are omitted > ## 16.5 UE procedure for reporting HARQ-ACK on uplink A UE can be provided PUCCH resources or PUSCH resources [12, TS 38.331] to report HARQ-ACK information that the UE generates based on HARQ-ACK information that the UE obtains from PSFCH receptions, or from absence of PSFCH receptions. For SL configured grant Type 1 or Type 2 PSSCH receptions by a UE within a time period provided by periodSlCG, the UE generates one HARQ-ACK information bit in response to the PSFCH receptions to multiplex in a PUCCH transmission occasion that is after a last time resource, in a set of time resources. For each PSFCH reception occasion, from a number of PSFCH reception occasions that the UE generates HARQ-ACK information to report in a PUCCH or PUSCH transmission, the UE can be indicated by higher layers SCI to perform one of the following and the UE constructs a HARQ-ACK codeword with HARQ-ACK information, when applicable. If the UE receives PSFCH associated with a SCI format 2-A with Cast type indicator = 10, - generate HARQ-ACK information with same value as a value of HARQ-ACK information the UE determines from a PSFCH reception in the PSFCH reception occasion and, if the UE determines that a PSFCH is not received at the PSFCH reception occasion, generate NACK If the UE receives PSFCH associated with a SCI format 2-A with Cast type indicator = 01, - generate ACK when the UE determines ACK from each PSFCH reception for the number of PSFCH reception occasions; otherwise, generate NACK if the UE determines absence of PSFCH reception or determines a NACK value from a PSFCH reception at a corresponding PSFCH reception occasion - generate ACK when the UE determines ACK from at least one PSFCH reception for the number of PSFCH reception occasions of a PSFCH resource with an index with MID, as determined in Clause 16.3, for every identity MID of the UEs expected to receive the PSSCH, as indicated by higher layers; otherwise, generate NACK. If the UE receives PSFCH associated with a SCI format 2-B, - generate ACK when the UE determines absence of PSFCH reception for each PSFCH reception occasion from the number of PSFCH reception occasions; otherwise, generate NACK - generate ACK when the UE determines ACK from at least one PSFCH reception for the number of PSFCH reception occasions of a PSFCH resource with an index with MID, as determined in Clause 16.3, for every identity MID of the UEs expected to receive the PSSCH, as indicated by higher layers; otherwise, generate NACK. The UE generates NACK when, due to prioritization, as described in Clause 16.2.4, the UE does not receive PSFCH in any PSFCH reception occasion associated with a PSSCH transmission in a resource provided by a DCI format 3_0 with CRC scrambled by a SL-RNTI or, for a configured grant, in a resource provided in a single period and for which the UE is provided a PUCCH resource to report HARQ-ACK information. The UE generates NACK when, due to prioritization as described in Clause 16.2.4, the UE does not transmit a PSSCH in any of the resources provided by a DCI format 3_0 with CRC scrambled by SL-RNTI or, for a configured grant, in any of the resources provided in a single period and for which the UE is provided a PUCCH resource to report HARQ-ACK information. The UE generates ACK if the UE does not transmit a PSCCH with a SCI format 0_1 scheduling a PSSCH in any of the resources provided by a configured grant in a single period and for which the UE is provided a PUCCH resource to report HARQ-ACK information. < Unchanged parts are omitted >

TP for 38.214

### 8.5.2 Channel state information #### 8.5.2.1 CSI reporting quantities #### 8.5.2.1.1 Channel quality indicator (CQI) The UE shall derive CQI as specified in section 5.2.2.1, with the following changes - PDSCH replaced by PSSCH - uplink slot replaced by sidelink slot - downlink physical resource blocks replaced by sidelink physical resource blocks - Transport Block Size determination according to Clause 8.1.3.2 - CSI reference resource according to TODO Clause 8.5.2.3 - interference measurements are not supported - sub-band differential CQI is not supported - cqi-Table is determined as indicated by Additional MCS table indicator in a SCI format 1-A, - cqi-Table = ‘table1’ if Additional MCS table indicator indicates Table 5.1.3.1-1, - cqi-Table = ‘table2’ if Additional MCS table indicator indicates Table 5.1.3.1-2, - cqi-Table = ‘table3’ if Additional MCS table indicator indicates Table 5.1.3.1-3