3GPP TSG RAN WG1 Meeting #101-e	R1-200XXXX

eMeeting, May 25-June 5, 2020

Agenda Item:	7.2.4.7

Source:	Moderator (FUTUREWEI)

Title:	Text Proposal for TS36.212 to clarify values of SPS activation/release in DCI format 5A

Document for:	Discussion and decision

# Reason for Change

- DCI 5A contains a SPS activation/release field. However, it is not defined which field value corresponds to activation, and which field value correspond to release. The change is to add the meaning of 0 and 1 for this field.

# Consequences if not approved

The UE does not know how to interpret this field.

# Text proposal for TS36.212 (v14.12.0, Section 5.3.3.1.9A)

The change is to add the meaning of 0 and 1.

---------------------------------------BEGIN TEXT CHANGE-------------------------------------------

##### 5.3.3.1.9A	Format 5A

DCI format 5A is used for the scheduling of PSCCH, and also contains several SCI format 1 fields used for the scheduling of PSSCH.

The following information is transmitted by means of the DCI format 5A:

-	Carrier indicator –3 bits. This field is present according to the definitions in [3].

-	Lowest index of the subchannel allocation to the initial transmission -  bits as defined in clause 14.1.1.4C of [3].

-	SCI format 1 fields according to 5.4.3.1.2:

-	Frequency resource location of initial transmission and retransmission.

-	Time gap between initial transmission and retransmission.

-	SL index – 2 bits as defined in clause 14.2.1 of [3] (this field is present only for cases with TDD operation with uplink-downlink configuration 0-6).

When the format 5A CRC is scrambled with SL-SPS-V-RNTI, the following fields are present:

-	SL SPS configuration index – 3 bits as defined in clause 14.2.1 of [3].

-	Activation/release indication – 1 bit, where value 0 indicates release and value 1 indicates activation as defined in clause 14.2.1 of [3].

---------------------------------------END TEXT CHANGE-------------------------------------------