# HST evaluations

Based on the company’s inputs, it is a common understanding that only LLS should be used for HST evaluations. Therefore, it is proposed to limit discussion scope to LLS assumptions only

Proposal #1:

- LLS to be used for Rel-17 HST evaluations

| Company | Comment |
| --- | --- |

# Frequency range

Companies have provided views on the FR that should be used for HST evaluations. Some companies prefer to prioritize FR1 for HST evaluations (ZTE, CATT, IDC, Lenovo/MotM, CMCC, Nokia), while other companies prefer to treat FR1 and FR2 with equal priority (SS, Intel, E///, vivo), i.e.,

- Alt .1: FR1 + FR2, but FR1 is prioritized

- Alt. 2: FR1 + FR2

From simulation perspective, it is better to define assumptions for both FR1 and FR2 and decide possible FR prioritization later.

Proposal #2:

- Define HST simulation assumptions for both FR1 and FR2

| Company | Comment |
| --- | --- |

# HST layout

Two HST layout options were proposed by companies based on TR 38.913 supporting FR1 + FR2 (Samsung, Lenovo/Motorola Mobility, Nokia) and TS 36.101 Annex B.3A with Ds=700m, Dmin=150m supporting FR1 (CMCC, Intel, IDC, CATT, Ericsson, LG, FUTUREWAY, Sony), i.e.,

- Alt 1: TR 38.913 (FR1 + FR2)

- Alt 2: TS 36.101 Annex B.3A (FR1: Ds=700m, Dmin=150m, FR2: TBD)

Proposal #3:

- Companies to provide their views on the preferred TRP layout for HST evaluation for both FR1 and FR2

  - Alt 1: TR 38.913 (FR1 + FR2)

  - Alt 2: TS 36.101 Annex B.3A (FR1: Ds=700m, Dmin=150m, FR2: TBD)

| Company | Comment |
| --- | --- |

# gNB antenna orientation

Two companies provided views on the gNB antenna orientation in HST deployment.

- Alt 1: Bi-directional only

- Alt 2: Unidirectional + Bidirectional

Considering operator’s input on realistic deployment scenario and given RAN4 requirements are only defined for bi-directional model, it seems natural to use bi-direction antenna orientation should mandatory antenna orientation and unidirectional as optional

Proposal #4:

- Use bi-directional as mandatory and uni-directional as optional gNB antenna orientation

| Company | Comment |
| --- | --- |

# Channel model

Some companies proposed to reuse RAN4 channel models (TS 36.101 / TR 36.878) as much as possible (Samsung, Intel, IDC, CATT, Ericsson, LG, FUTUREWEI, CMCC, Sony, OPPO, Nokia), while other companies proposed by extend RAN4 models by including multi-path components using CDL framework (ZTE, CMCC, Huawei/HiSilicon, Lenovo/Motorola Mobility, vivo),i.e.,

- Alt 1: 4 taps - TS 36.101 (Annex B.3A) / TR 36.878 (RAN4)

- Alt 2: Multi-path extension of TS 36.101 (Annex B.3A) / TR 36.878 (RAN4) + CDL TR 38.901

Considering maturity of the model it is recommended to include RAN4 model as part of simulation assumption and continue discussion to finalize multipath extension of RAN4 model using CDL framework

Proposal #5:

- Adopt RAN4 4-taps model based on TS 36.101 (Annex B.3A) / TR 36.878

- Further discuss CDL based multipath extension from RAN4 model, e.g., using the following proposal (CMCC) as a starting point

CDL based channel model proposal for HST: Therefore, we think that some combination of the CDL channel model in TR38.901 and the 4-tap channel model in TS36.101 Annex B.3A could be considered. One simple way could be similar to the suggestion of ZTE, as illustrated in figure 3 below, 2-tap channel model for simplicity could be assumed which is similar to RAN4’s 4-tap assumption in order to reflect the characteristic of SFN-based transmission, and for each tap, CDL channel model in TR38.901 could be used to model the effect of the directional antenna of gNB. - The delay for k’th TRP is modified as τk,n=τk+τn where is the delay of k’th TRP, which can be derived as τk=y(t)-xkC where τn is the delay of the n’th channel cluster as in Table 7.7.1-1~7.7.1-5 in 38.901, and assume the location of the k’th TRP is xk, and the UE’s location is y(t). The delay spread for different TRPs could be modeled as different as suggested by Huawei. - The normalized power for k’th TRP is modified as Pk=10lg1y(t)-xk2i=1,21y(t)-xi2(dB) - To generate the modified angle parameters, the scaling method mentioned in subclause 7.7.5.1 in TS 38.901 is used φn,scaled=ASdesiredASmodelφn,model-μφ,model+μφ,desired where ASmodel=ASdesired could be assumed, and μφ,desired of the k’th TRP is the AOD, AOA, ZOD and ZOA of LOS direction derived based on the locations and antenna heights of UE and TRPs. Fig. 3. Simplified and updated HST-SFN channel model for evaluation

| Company | Comment |
| --- | --- |

# Baseline scheme for comparison

Some companies provided their views on the baseline schemes that should be used for performance comparison, i.e.,

- Alt. 1 Rel-15 SFN

- Alt. 2 Rel-16 URLLC

To avoid lengthy discussion on the baseline scheme, it is recommended that each company to provide details on the baseline scheme used for comparison

Proposal #6:

- Each company to provide baseline scheme used for comparison as part of simulation assumptions

| Company | Comment |
| --- | --- |

Companies are also encouraged to provide additional inputs regarding HST simulation assumptions using the following table below.

# Detailed simulation assumptions:

| Parameter | FR1 |  | FR2 |
| --- | --- | --- | --- |
| Duplexing | FDD | TDD | TDD |
| TRP layout (Ds, Dmin, etc) |  |  |  |
| gNB antenna configuration including number of antennas, pattern, ports, orientation, etc |  |  |  |
| UE antenna configuration including number of antennas, pattern, ports, orientation, etc |  |  |  |
| DMRS type |  |  |  |
| Number of DMRS symbols |  |  |  |
| TDD pattern | N/A |  |  |
| MCS |  |  |  |
| Number of scheduled RBs |  |  |  |
| Propagation condition |  |  |  |
| TRS configuration, TRS periodicity |  |  |  |
| PDSCH / PUSCH mapping |  |  |  |
| Rank |  |  |  |
| BW |  |  |  |
| Carrier frequency or maximum Doppler shift |  |  |  |
| Performance metric |  |  |  |
| Other assumptions or simulation parameters, e.g., correlation am |  |  |  |