3GPP TSG RAN WG1 #101		R1-200xxxx

e-Meeting, May 20th – June 5th, 2020

Source:	Moderator (OPPO)

Title:	Discussion on Issue#a-2 in Email Thread 2

Agenda Item:	7.2.6.2

Document for:	Discussion and Decision

- Introduction

Rel-16 enhancement on MIMO WID includes objectives of enhancing multi-TRP/Panel transmission with ideal and non-ideal backhaul. During the work of rel-16, designs for multiple-PDCCH based and single-PDCCH based multi-TRP/Panel transmission were discussed and specified. This document provides the discussion for Issue #a-2 in multi-TRP email thread 2:

- Discuss issue #a-2 to align the understanding on whether single-DCI and multi-DCI can be configured on one UE simultaneously and discuss whether or not to introduce restriction on simultaneous configuration of multi-DCI based and single-DCI based M-TRP into RAN1 specification

# Issue#a-2: whether configuring multi-DCI based and single-DCI based M-TRP simultaneously

Background:

Companies CATT [4], CMCC [10], OPPO [12], Apple [13] and Ericsson [17] disused the issue of whether multi-DCI based M-TRP and single-DCI based M-TRP can be configured to one UE simultaneously.

- CATT [4] proposed that, when a UE is configured with two different values of CORESETPoolindex, the UE can only be indicated with one TCI state in DCI. [4] proposed corresponding TP.

- CMCC [10] proposed to clarify that single-DCI and multi-DCI based multi-TRP cannot be configured simultaneously and also proposed corresponding TP.

- OPPO [12] proposed that from RAN1 point of view, simultaneous configuration of single-DCI and multi-DCI based M-TRP is not supported and also proposed corresponding TP.

- Apple [13] thinks that simultaneous single-DCI and multi-DCI configuration shall be avoided and they proposed that for multi-TRP operation, the UE is not expected to be configured to operate in single-DCI and multi-DCI operation simultaneously. Corresponding TP is proposed by [13].

- In contrast, Ericsson [17] proposed to conclude that current RAN1 specs allows simultaneous configuration of single-DCI and multi-DCI.

Furthermore, from the comments collected during prep email:

- ZTE: RAN2 has discussed this issue and is waiting for RAN1’s decision.

- HW: we need to a clear conclusion since it leads to very different UE implementation/complexity and NW expectation.

- Nokia commented in FL summary [20] that simultaneous support of both single-DCI and multi-DCI mode is not valid because MAC CE activation for TCI-states are different for single-DCI based multi-TRP and multi-DCI based.

- MediaTek commented that they support that simultaneous configuration of single-DCI and multi-DCI is not supported.

- LG: companies have different understanding on this issue and clarification is critical and needed.

- If the UE supports both schemes, such a restriction may require the network to reconfigure RRC to change the multi-TRP mode.

- SS: UE can report M-DCI or S-DCI capabilities separately and gNB can configure the UE as it prefers. We think there’s no need to add exclusion rule on top of that, either in RAN1 or RAN2 sides.

- CMCC: Agree with FL’s suggestion and we should conclude in this meeting. Support Alt 1. If S-DCI and M-DCI is simultaneously configured, there will be many ambiguous understandings in the current spec, such as the default TCI for PDSCH.

- OPPO: RAN1 discussed this issue and is waiting for RAN1’s decision. Thus, RAN1 should have a clear conclusion

- FUTUREWEI: need clarification for UE support

FL’s assessment:

- We do have different understanding on whether these two multi-TRP mode in RAN1, thus a clear conclusion to align the understanding is necessary.

- From the viewpoint of FL, I agree with Nokia’s comments that simultaneous configuration of these two modes are not valid because of the design of MAC CE for PDSCH state activation in TS 38.321:

  - The MAC CE used for multi-DCI based M-TRP is the MAC CE of Section 6.1.3.14. It has a 1-bit field “CORESET Pool ID” and it only activates one TCI state for each DCI codepoint. So this MAC CE can only be used when only multi-DCI based is configured.

  - The MAC CE used for single-DCI based M-TRP is the MAC CE of Section. It can activate one or two TCI states for each DCI codepoint. But it does not have the bit field of “CORESET Pool ID”. So, this MAC CE can only be used when only single-DCI based mode is configured.

Proposal 1: On configuration of multi-TRP transmission, down-select one from the following two alts:

- Alt 1: Simultaneous configuration of single-DCI based and multi-DCI based M-TRP is not supported. Update TS 38.214 to clarify that.

- Alt 2: No restriction to prevent configuring multi-DCI and single-DCI based M-TRP simultaneously is needed in RAN1 spec TS 38.214.  Whether to support single-DCI +multi-DCI mixed mode operation is up to UE capability.

Please input your views and comments on these two alternatives:

| Company | Views and comments |
| --- | --- |
| Apple | We support Alt1. For Alt2, our understanding is that sDCI and mDCI would be dynamically switched by MAC CE for TCI indication. However, if gNB configures 5 CORESETs by RRC, and uses a MAC CE to enable sDCI mode, UE has to monitor 5 CORESETs even for sDCI mode. This is not aligned with our previous agreement. In addition, it has been agreed that 2-port PT-RS is only applicable for sDCI mode, which is also included in LS of RRC parameters to RAN2. But if gNB uses a MAC CE to enable mDCI mode, this would result in 2-port PT-RS for mDCI mode. From UE perspective, such can be considered as an incorrect configuration. So to switch to mDCI, gNB still has to use RRC reconfiguration to avoid such kind of incompatible configurations. |
| MediaTek | Support Alt 1. During Rel-16 discussion, most of agreements were reached conditioned on either 1) S-DCI based M-TRP transmission or 2) M-DCI based M-TRP transmission. To our best understanding, we seldom make decision assuming both of them are simultaneously configured. If configuring simultaneously is allowed, a UE has to be ready for any possible combinations, e.g., two DCIs supporting Rel-16 URLLC may be received in the same/different slot, or the case with one S-DCI based eMBB + one S-DCI URLLC. It will take huge effort to identify/conclude which combinations are beneficial. It is also too late to introduce capability signaling for this mixed mode operation, and we believe such a capability signaling is not a simple bit and is controversial. |
| OPPO | Support Alt.1. We agree with Apple and MTK. Two different MAC CEs were separately designed for S-DCI and M-DCI based TRP transmission. UE should know which MAC CE to use based on RRC configuration of S-DCI or M-DCI based, e.g. whether two CORESETPoolIndex are configured. It is straightforward that the two schemes are switched via RRC and not simultaneously configured. At least we should make a conclusion in RAN1 as guidance for RAN2 signaling design. |
| ZTE | Alt.1 should be supported. - MACCE issue for Alt.2: In current 38.321, different MACCE formats are designed for S-DCI and M-DCI. In MACCE for MDCI, CORESET pool index is indicated by one bit. In MACCE for SDCI, one TCI pair may exist for each TCI codepoint and there is no such CORESET pool index. If Alt2 is supported, a new MACCE should be supported. That is, a MACCE including TCI pair also need to carry CORESET pool index. We don’t think MACCE enhancement can be considered in such late state. - PTRS issue for Alt.2 as Apple provided. - Alt.2 doesn’t work in FR2 because UE cannot support four receive beams (two beams for each TRP1). |
| CMCC | Support Alt 1 We also think that simultaneous configuration of S-DCI and M-DCI is not valid by considering the MAC CE issue proposed by Nokia and the PTRS issue proposed by Apple. Besides, when S-DCI and M-DCI are simultaneous configured, the PDSCH default beam is also confused, follow the CORESET beam or follow the two TCI states indicated by MAC CE? Considering the limit time for Rel-16, it is not a good way to support Alt 2 at this time, which may cause more problem for current spec. |

| CATT | If simultaneous configuration of single-DCI based and multi-DCI based M-TRP is possible, CORESETpoolindex can be configured for the CORESET intended for S-DCI. Therefore, it would be more flexible to network side, as dynamic switching between S-DCI and M-DCI can be supported. However, the issues listed above should be addressed, if such simultaneous configuration is to be supported. No matter such configuration is supported or not, in our view, we should preclude the overlapped scheduling with more than 2 TCI states. To be more specific, at least it should be clarified that for a UE configured with CORESETs with different IDs, it does not expect to be scheduled with fully/partially overlapped PDSCH resources by CORESETs with different CORESETpoolindices, if two TCI states are indicated by any of the DCI in these CORESETs. So, Alt 1 is preferred. |
| --- | --- |
| Spreadtrum | Support Alt.1. The benefit of Alt.2 is not clear to us while it will bring more spec workload in such late stage, and require much higher UE capability. |
| vivo | From the scenario perspective, dynamic switching between S-DCI and M-DCI based MTRP is applicable to ideal backhaul scenario. S-DCI based MTRP is surely beneficial for URLLC type of services. While M-DCI based MTRP provides scheduling flexibility in ideal backhaul. For example, two TRPs can independently schedule PDSCHs for the UE to achieve higher throughput, and can activate up to 8 TCI states per TRP which is larger than the S-DCI case. So there is need to support both in ideal backhaul network. From the signaling perspective, when a UE is configured with two different values of CORESETPoolIndex, for the UE received the MAC CE of Section 6.1.3.14 command and the activated TCI states of the associating MAC CE are valid. When a UE receives a MAC CE of Section 6.1.3.24 with at least one TCI codepoint contains 2 TCI states later on, the MAC CE of Section 6.1.3.14 command would be invalidated and any DCI with TCI indication which is(are) activated by the new MAC CE command can be transmitted from either TRP from RAN2 understanding so that the UE switches to S-DCI-based MTRP. In our understanding, current specification without further clarification by TS38.214, supporting both S-DCI and M-DCI based MTRP simultaneously is not supported but dynamic switching between the two is supported. In summary, our preference is: Alt 3: No update in TS 38.214, which means dynamic switching between S-DCI and M-DCI based MTRP is supported. |
| QC | Support Alt1. If there are valid use cases for simultaneous operation of s-DCI and m-DCI, it can be further studied in Rel. 17. There can be many different combinations when different schemes of s-DCI as well as non/partial/full-overlapping of m-DCI are considered. There is no time in Rel. 16 to start this big scoping effort. As for dynamic switching between s-DCI and m-DCI (regarding Vivo’s comment above), note that the MAC-CE in 6.1.3.24 cannot suddenly switch m-DCI to s-DCI mode. What happens to HARQ-Ack (both separate and joint) as CORESETPoolIndex determines the procedures? How about PDSCH scrambling or CRS rate matching? What about BD/CCE limits? Are all of these operations supposed to go back to s-DCI mode once the MAC-CE in 6.1.3.24 is received? |

- Reference

- R1-2003397	On remaining issues on M-TRP	vivo

- R1-2003469	Maintenance of multi-TRP enhancements	ZTE

- R1-2003531	Remaining issues on multi-TRP in R16	Huawei, HiSilicon

- R1-2003627	Discussion on remaining issues of multi-TRP/panel transmission	CATT

- R1-2003660	Remaining issues on multi-TRP transmission	MediaTek Inc.

- R1-2003742	Corrections to multi-TRP	Intel Corporation

- R1-2003819	Remaining issues on multi-TRP/panel transmission	Lenovo, Motorola Mobility

- R1-2003881	On Rel.16 multi-TRP/panel transmission	Samsung

- R1-2003928	Text proposals on enhancements on multi-TRP/panel transmission	LG Electronics

- R1-2003954	Remaining issues on multi-TRP/panel transmission	CMCC

- R1-2003987	Discussion on remaining issues of multi-TRP operation	Spreadtrum Communications

- R1-2004047	Text proposals for enhancements on multi-TRP and panel Transmission	OPPO

- R1-2004229	Remaining issues for Multi-TRP enhancement	Apple

- R1-2004265	Maintenance of Rel-16 Multi-TRP operation	Nokia, Nokia Shanghai Bell

- R1-2004311	Remaining issues on multi-TRP transmission	NEC

- R1-2004395	Remaining issues on multi-TRP/panel transmission	NTT DOCOMO, INC

- R1-2004432	Remaining issues on Multi-TRP/Panel Transmission	Ericsson

- R1-2004463	Multi-TRP Enhancements	Qualcomm Incorporated

- R1-2004592	Clarification on Multi-TRP URLLC Scheme 4	Convida Wireless

- R1-2004719  FL summary #2 for Multi-TRP/Panel Transmission Moderator(OPPO)