3GPP TSG RAN WG1 #101		R1-200xxxx

e-Meeting, May 20th – June 5th, 2020

Source:	Moderator (OPPO)

Title:	Discussion on Issue#b-2 in Email Thread 2

Agenda Item:	7.2.6.2

Document for:	Discussion and Decision

- Introduction

Rel-16 enhancement on MIMO WID includes objectives of enhancing multi-TRP/Panel transmission with ideal and non-ideal backhaul. During the work of rel-16, designs for multiple-PDCCH based and single-PDCCH based multi-TRP/Panel transmission were discussed and specified. This document provides the discussion for Issue #b-2 in multi-TRP email thread 2:

- The issue# b-2 to Clarify the relationship between RepetitionNumber-r16/RepSchemeEnabler  and pdsch-AggregationFactor, and also clarify the repetitions are in RepNum16 consecutive slots in Scheme 4.

Issue#b-2

Background:

Companies [4], [12], [17], [18] discuss the issue of relationship between the RepNumR16 of scheme 4 and the parameter pdsch-AggregationFactor that was specified in rel15.  They all propose that the RepNumR16 of scheme 4 and parameter pdsch-AggregationFactor  can not be used simultaneously. But their proposal has some difference:

- [4] proposed that pdsch-AggregationFactor should be overwritten whenever Rel-16 repetition number RepetitionNumber-r16 is indicated by DCI.

- [12] proposed that when at least one entry in pdsch-TimeDomainAllocationList contains RepNumR16, the UE will ignore the AggregationFactor.

- [17] proposed that pdsch-AggregationFactor should be overwritten when Rel-16 repetition number RepetitionNumber-r16 is indicated by DCI.

- [18] proposed that when at least one entry in pdsch-TimeDomainAllocationList contains RepNumR16, the UE does not expect to be configured with AggregationFactor.

[18] also discussed the issue of simultaneous configuration of scheme 2a/2b/3 and pdsch-AggregationFactor and proposed that simultaneous configuration of scheme 2a/2b/3 and pdsch-AggregationFactor is not allowed.

[12], [17] and [19] proposed to clarify that in scheme 4, the PDSCH is repeated in in RepNumR16 consecutive slots.

Proposal 1: Regarding the RepetitionNumber-r16 of scheme 4 and AggregationFactor, down-select from:

- Alt1: pdsch-AggregationFactor is applied only when the Rel-16 repetition number RepetitionNumber-r16 is not indicated by a DCI.

- Alt2: AggregationFactor is applied only when the Rel-16 repetition number RepetitionNumber-r16 is not included in any entry in pdsch-TimeDomainAllocationList.

- Alt3: When at least one entry in pdsch-TimeDomainAllocationList contains RepetitionNumber-r16, the UE does not expect to be configured with AggregationFactor

Proposal 2: When a UE is configured by repetitionSchemeConfig-r16 set to one of 'FDMSchemeA', 'FDMSchemeB' and 'TDMSchemeA', the UE does not expect to be configured with AggregationFactor.

Proposal 3: Clarify that in scheme 4, PDSCH is repeated in RepNumR16 consecutive slots

Please input your views and comments on these 3 proposals:

| Company | Views and comments |
| --- | --- |
| Apple | Since all the parameters are provided by RRC, gNB should provide the correct parameters. Such “ignore” or “overwritten” are not typical ways we used. It seems Alt3 in proposal 1 and proposal 2 & 3 should be fine. |
| MediaTek | We support Alt 3 in Proposal 1, and also support Proposal 2 and Proposal 3. |
| OPPO(updated) | For proposal 1, regarding Apple’s comments, I modify the wording since we only need to clarify in 38.214 when to apply pdsch-AggregationFactor. The application of RepetitionNumber-r16 has been clearly described in spec. It should be noticed that it is possible that RepetitionNumber-r16 is configured in pdsch-TimeDomainAllocationList-ForDCIFormat1_2 but not in pdsch-TimeDomainAllocationList considering it is mainly applied for URLLC. In this case, if DCI format 1_0 or DCI format 1_1 is used to schedule PDSCH, it is not reasonable to forbid gNB to use pdsch-AggregationFactor for PDSCH (which is actually Rel-15 UE behavior). Hence, considering a unified design for pdsch-TimeDomainAllocationList-ForDCIFormat1_2 and pdsch-TimeDomainAllocationList, Alt.1 or Alt.2 is fine to us. Otherwise, we may need another conclusion to clarify if pdsch-AggregationFactor can be configured if RepetitionNumber-r16 is only configured in pdsch-TimeDomainAllocationList-ForDCIFormat1_2. We also support proposal 2 and 3. |
| ZTE | Support Proposal 1 with Alt3, proposal 2 and proposal 3. The reason is that, Alt.1 is not flexible since it doesn’t support repetition number =1 if pdsch-AggregationFactor is configured. Alt.2 causes unnecessary configuration, the benefit is unclear. |
| CMCC | Support Proposal 1 with Alt2, proposal 2 and proposal 3. For proposal 1, in 38.214, the number of repetitions K for PUSCH has been specified as follows. Similar principle can be reused for PDSCH. For PUSCH repetition Type A, when transmitting PUSCH scheduled by DCI format 0_1 or 0_2 in PDCCH with CRC scrambled with C-RNTI, MCS-C-RNTI, or CS-RNTI with NDI=1, the number of repetitions K is determined as - if numberofrepetitions is present in the resource allocation table, the number of repetitions K is equal to numberofrepetitions; - elseif the UE is configured with pusch-AggregationFactor, the number of repetitions K is equal to pusch-AggregationFactor; - otherwise K=1. |
| Spreadtrum | Support Alt.3 in Proposal 1 for its flexibility, proposal 2, and proposal 3. |
| vivo | Support Alt.2 in Proposal 1 which works the same way as UL PUSCH in TS 38.214: ============================================= For PUSCH repetition Type A, when transmitting PUSCH scheduled by DCI format 0_1 or 0_2 in PDCCH with CRC scrambled with C-RNTI, MCS-C-RNTI, or CS-RNTI with NDI=1, the number of repetitions K is determined as - if numberofrepetitions is present in the resource allocation table, the number of repetitions K is equal to numberofrepetitions; - elseif the UE is configured with pusch-AggregationFactor, the number of repetitions K is equal to pusch-AggregationFactor; - otherwise K=1. For PUSCH repetition Type A, in case K>1, the same symbol allocation is applied across the K consecutive slots and the PUSCH is limited to a single transmission layer. The UE shall repeat the TB across the K consecutive slots applying the same symbol allocation in each slot. The redundancy version to be applied on the nth transmission occasion of the TB, where n = 0, 1, … K-1, is determined according to table 6.1.2.1-2. =============================================== For a UE supporting both DCI format 1-1 and 1-2, two TDRA tables are configured, each applies the same rule as Alt.2. We also support Proposal 3. |
| QC | Support Alt3 in Proposal 1, and proposals 2 and 3. Note that for semi-static HARQ-Ack, we agreed on the following in Feb e-Meeting, which means that Alt1 is not allowed. Also, Alt2 is a configuration error case, which should be avoided by the network. For the case of different TDRA tables for DCI formats 1-1 and 1-2 (if this is agreed), still Alt 3 should be followed (if at least one entry of any of the two TDRA tables is configured with RepetitionNumber-r16, AggregationFactor should not be configured/used). Otherwise, Type-1 HARQ-Ack codebook determination requires additional specification efforts. Note that such an impact does not exists for the case of PUSCH in eURLLC (as it does not impact the HARQ-Ack). If the UE is provided pdsch-AggregationFactor and no entry in pdsch-TimeDomainAllocationList includes RepNumR16 in PDSCH-TimeDomainResourceAllocation, NPDSCHrepeat is a value of pdsch-AggregationFactor; otherwise NPDSCHrepeat=1. The UE reports HARQ-ACK information for a PDSCH reception - from slot n-NPDSCHrepeat+1 to slot n, if NPDSCHrepeat>1, or - from slot n-RepNumR16+1 to slot n, if the Time domain resource assignment field in the DCI format scheduling the PDSCH reception indicates an entry in pdsch-TimeDomainAllocationList containing RepNumR16, or - in slot n, otherwise |
| Ericsson | We have similar comments as QC with regards to Proposal 1. Hence, we support Alt 3 of Proposal 1. We also support Proposals 2 and 3. |
| Huawei | Support Proposal 1 with Alt1 or Alt 3, and slightly prefer Alt 1 due to avoid potential reconfiguration if the NW does not want to use dynamic repetition and prefer that Rel-15 can be sufficient. We are fine with proposal 2, and proposal 3 which shall be aligned with Rel-15 repetition mechanism. |
| Nokia | On Proposal 1: Alt.2 Alt.1 is not matching with the existing specification text as highlighted by QC. On other alternatives, it is not clear why there are two variants as Alt.2 and Alt.3 as the outcome suggest there is the same. Alt.2 refer to the case where RepetitionNumber-r16 is not configured (for any entry) and AggregationFactor is configured. As alt.2 behaviour is already captured in HARQ-Ack codebook (mentioned by several companies above), we do not see a strong need on using Alt.3 wording. Proposal 2 & 3: Fine. |
| LG | Support Alt 3 for Proposal 1 and support Proposal 2 and 3. Regarding Proposal 1, Alt 1 and 2 introduce unnecessary priority when both aggregation factor and repetition number are configured. Such configuration can be avoided by capturing “UE does not expect …” as Alt 3 and it is well aligned with Proposal 2 as well. |
| OPPO1 | Regarding QC’s comments, we don’t think any modification is needed for Type-1 HARQ-ACK codebook determination in 38.213. Even when pdsch-AggregationFactor and RepNumR16 are both configured, there is no issue with NPDSCHrepeat=1 according to the rules in 38.213. And I don’t know why we need to restrict configuration of pdsch-AggregationFactor for DCI format 1_0 and 1_1, when gNB configures RepNumR16 for URLLC for DCI format 1_2. Alt3 actually introduces additional scheduling restriction. |

- Reference

- R1-2003397	On remaining issues on M-TRP	vivo

- R1-2003469	Maintenance of multi-TRP enhancements	ZTE

- R1-2003531	Remaining issues on multi-TRP in R16	Huawei, HiSilicon

- R1-2003627	Discussion on remaining issues of multi-TRP/panel transmission	CATT

- R1-2003660	Remaining issues on multi-TRP transmission	MediaTek Inc.

- R1-2003742	Corrections to multi-TRP	Intel Corporation

- R1-2003819	Remaining issues on multi-TRP/panel transmission	Lenovo, Motorola Mobility

- R1-2003881	On Rel.16 multi-TRP/panel transmission	Samsung

- R1-2003928	Text proposals on enhancements on multi-TRP/panel transmission	LG Electronics

- R1-2003954	Remaining issues on multi-TRP/panel transmission	CMCC

- R1-2003987	Discussion on remaining issues of multi-TRP operation	Spreadtrum Communications

- R1-2004047	Text proposals for enhancements on multi-TRP and panel Transmission	OPPO

- R1-2004229	Remaining issues for Multi-TRP enhancement	Apple

- R1-2004265	Maintenance of Rel-16 Multi-TRP operation	Nokia, Nokia Shanghai Bell

- R1-2004311	Remaining issues on multi-TRP transmission	NEC

- R1-2004395	Remaining issues on multi-TRP/panel transmission	NTT DOCOMO, INC

- R1-2004432	Remaining issues on Multi-TRP/Panel Transmission	Ericsson

- R1-2004463	Multi-TRP Enhancements	Qualcomm Incorporated

- R1-2004592	Clarification on Multi-TRP URLLC Scheme 4	Convida Wireless

- R1-2004719  FL summary #2 for Multi-TRP/Panel Transmission Moderator(OPPO)