3GPP TSG RAN WG1 #101		R1-200xxxx

e-Meeting, May 20th – June 5th, 2020

Source:	Moderator (OPPO)

Title:	Discussion on Issue#b-2 in Email Thread 2

Agenda Item:	7.2.6.2

Document for:	Discussion and Decision

- Introduction

Rel-16 enhancement on MIMO WID includes objectives of enhancing multi-TRP/Panel transmission with ideal and non-ideal backhaul. During the work of rel-16, designs for multiple-PDCCH based and single-PDCCH based multi-TRP/Panel transmission were discussed and specified. This document provides the discussion for Issue #b-2 in multi-TRP email thread 2:

- The issue# b-2 to Clarify the relationship between RepetitionNumber-r16/RepSchemeEnabler  and pdsch-AggregationFactor, and also clarify the repetitions are in RepNum16 consecutive slots in Scheme 4.

Issue#b-2

Background:

Companies [4], [12], [17], [18] discuss the issue of relationship between the RepNumR16 of scheme 4 and the parameter pdsch-AggregationFactor that was specified in rel15.  They all propose that the RepNumR16 of scheme 4 and parameter pdsch-AggregationFactor  can not be used simultaneously. But their proposal has some difference:

- [4] proposed that pdsch-AggregationFactor should be overwritten whenever Rel-16 repetition number RepetitionNumber-r16 is indicated by DCI.

- [12] proposed that when at least one entry in pdsch-TimeDomainAllocationList contains RepNumR16, the UE will ignore the AggregationFactor.

- [17] proposed that pdsch-AggregationFactor should be overwritten when Rel-16 repetition number RepetitionNumber-r16 is indicated by DCI.

- [18] proposed that when at least one entry in pdsch-TimeDomainAllocationList contains RepNumR16, the UE does not expect to be configured with AggregationFactor.

[18] also discussed the issue of simultaneous configuration of scheme 2a/2b/3 and pdsch-AggregationFactor and proposed that simultaneous configuration of scheme 2a/2b/3 and pdsch-AggregationFactor is not allowed.

[12], [17] and [19] proposed to clarify that in scheme 4, the PDSCH is repeated in in RepNumR16 consecutive slots.

Proposal 1: Regarding the RepetitionNumber-r16 of scheme 4 and AggregationFactor, down-select from:

- Alt1: pdsch-AggregationFactor is applied only when the Rel-16 repetition number RepetitionNumber-r16 is not indicated by a DCI.

- Alt2: AggregationFactor is applied only when the Rel-16 repetition number RepetitionNumber-r16 is not included in any entry in pdsch-TimeDomainAllocationList.

- Alt3: When at least one entry in pdsch-TimeDomainAllocationList contains RepetitionNumber-r16, the UE does not expect to be configured with AggregationFactor

Proposal 2: When a UE is configured by repetitionSchemeConfig-r16 set to one of 'FDMSchemeA', 'FDMSchemeB' and 'TDMSchemeA', the UE does not expect to be configured with AggregationFactor.

Proposal 3: Clarify that in scheme 4, PDSCH is repeated in RepNumR16 consecutive slots

Please input your views and comments on these 3 proposals:

| Company | Views and comments |
| --- | --- |
| Apple | Since all the parameters are provided by RRC, gNB should provide the correct parameters. Such “ignore” or “overwritten” are not typical ways we used. It seems Alt3 in proposal 1 and proposal 2 & 3 should be fine. |
| MediaTek | We support Alt 3 in Proposal 1, and also support Proposal 2 and Proposal 3. |
| OPPO | For proposal 1, regarding Apple’s comments, I modify the wording since we only need to clarify in 38.214 when to apply pdsch-AggregationFactor. The application of RepetitionNumber-r16 has been clearly described in spec. It should be noticed that it is possible that RepetitionNumber-r16 is configured in pdsch-TimeDomainAllocationList-ForDCIFormat1_2 but not in pdsch-TimeDomainAllocationList considering it is mainly applied for URLLC. In this case, if DCI format 1_0 or DCI format 1_1 is used to schedule PDSCH, it is not reasonable to forbid gNB to use pdsch-AggregationFactor for PDSCH (which is actually Rel-15 UE behavior). Hence, considering a unified design for pdsch-TimeDomainAllocationList-ForDCIFormat1_2 and pdsch-TimeDomainAllocationList, Alt.1 is preferred. Otherwise, we may need another conclusion to clarify if pdsch-AggregationFactor can be configured if RepetitionNumber-r16 is only configured in pdsch-TimeDomainAllocationList-ForDCIFormat1_2. |
| ZTE | Support Proposal 1 with Alt3, proposal 2 and proposal 3. The reason is that, Alt.1 is not flexible since it doesn’t support repetition number =1 if pdsch-AggregationFactor is configured. Alt.2 causes unnecessary configuration, the benefit is unclear. |

- Reference

- R1-2003397	On remaining issues on M-TRP	vivo

- R1-2003469	Maintenance of multi-TRP enhancements	ZTE

- R1-2003531	Remaining issues on multi-TRP in R16	Huawei, HiSilicon

- R1-2003627	Discussion on remaining issues of multi-TRP/panel transmission	CATT

- R1-2003660	Remaining issues on multi-TRP transmission	MediaTek Inc.

- R1-2003742	Corrections to multi-TRP	Intel Corporation

- R1-2003819	Remaining issues on multi-TRP/panel transmission	Lenovo, Motorola Mobility

- R1-2003881	On Rel.16 multi-TRP/panel transmission	Samsung

- R1-2003928	Text proposals on enhancements on multi-TRP/panel transmission	LG Electronics

- R1-2003954	Remaining issues on multi-TRP/panel transmission	CMCC

- R1-2003987	Discussion on remaining issues of multi-TRP operation	Spreadtrum Communications

- R1-2004047	Text proposals for enhancements on multi-TRP and panel Transmission	OPPO

- R1-2004229	Remaining issues for Multi-TRP enhancement	Apple

- R1-2004265	Maintenance of Rel-16 Multi-TRP operation	Nokia, Nokia Shanghai Bell

- R1-2004311	Remaining issues on multi-TRP transmission	NEC

- R1-2004395	Remaining issues on multi-TRP/panel transmission	NTT DOCOMO, INC

- R1-2004432	Remaining issues on Multi-TRP/Panel Transmission	Ericsson

- R1-2004463	Multi-TRP Enhancements	Qualcomm Incorporated

- R1-2004592	Clarification on Multi-TRP URLLC Scheme 4	Convida Wireless

- R1-2004719  FL summary #2 for Multi-TRP/Panel Transmission Moderator(OPPO)