3GPP TSG RAN WG1 #101		R1-200xxxx

e-Meeting, May 20th – June 5th, 2020

Source:	Moderator (OPPO)

Title:	Discussion on Issue#b-10 in Email Thread 3

Agenda Item:	7.2.6.2

Document for:	Discussion and Decision

- Introduction

Rel-16 enhancement on MIMO WID includes objectives of enhancing multi-TRP/Panel transmission with ideal and non-ideal backhaul. During the work of rel-16, designs for multiple-PDCCH based and single-PDCCH based multi-TRP/Panel transmission were discussed and specified. This document provides the discussion for Issue #b-10 in multi-TRP email thread 3:

- Issue #b-10 to correct Description on QCL of DMRS ports of M-TRP PDSCH in 38.211

Issue#b-10: Description on QCL of DMRS ports of M-TRP PDSCH in 38.211

Reason for changes:

In current specification TS 38.211, we have the following description on DMRS in Section 7.4.1.1.2:

In absence of CSI-RS configuration, and unless otherwise configured, the UE may assume PDSCH DM-RS and SS/PBCH block to be quasi co-located with respect to Doppler shift, Doppler spread, average delay, delay spread, and, when applicable, spatial Rx parameters. The UE may assume that the PDSCH DM-RS within the same CDM group are quasi co-located with respect to Doppler shift, Doppler spread, average delay, delay spread, and spatial Rx. The UE may assume that DMRS ports associated with a PDSCH are QCL with QCL Type A, Type D (when applicable) and average gain.

That description can only apply to UE when a PDSCH associated with one TCI-state, for instance, single TRP transmission in rel-15. In contrast such restriction in the description does not apply some multi-TRP transmission.  Contributions [4] and [12] discussed this issue and proposed TP to correct.

[4] suggests that the QCL assumption of DMRS ports is mainly described in TS 38.214 for both single and multiple TCI states. Thus [4] suggests to remove that sentence in TS 38.211.  Furthermore [4] proposes an editorial change that adds the phrase “when applicable”. The TP proposed by [4] is listed as Alt1 below.

[12] suggests also that highlighted description does not apply to single-DCI based multi-TRP transmission when two TCI states are indicated for different DMRS ports in one PDSCH. [4] suggests to add a condition in the description and [4] does not prefer to delete that sentence for backward compatibility to Rel15. The TP proposed by [12] is listed as Alt 2 below.

Thus, for this issue, we have the following two alternative TPs:

- Alt1: TP proposed by [4] for TS 38.211

---------------------------------------Start of text proposal for 7.4.1.1.2 of TS 38.214 -------------------------------------- ##### 7.4.1.1.2 Mapping to physical resources <Unchanged parts are omitted> In absence of CSI-RS configuration, and unless otherwise configured, the UE may assume PDSCH DM-RS and SS/PBCH block to be quasi co-located with respect to Doppler shift, Doppler spread, average delay, delay spread, and, when applicable, spatial Rx parameters. The UE may assume that the PDSCH DM-RS within the same CDM group are quasi co-located with respect to Doppler shift, Doppler spread, average delay, delay spread, and spatial Rx(when applicable). The UE may assume that no DM-RS collides with the SS/PBCH block. ------------------------------------------------------- End of text proposal ------------------------------------------------------

- Alt2: TP proposed by [12] for TS 38.211:

##### 7.4.1.1.2 Mapping to physical resources <Unchanged parts are omitted> In absence of CSI-RS configuration, and unless otherwise configured, the UE may assume PDSCH DM-RS and SS/PBCH block to be quasi co-located with respect to Doppler shift, Doppler spread, average delay, delay spread, and, when applicable, spatial Rx parameters. The UE may assume that the PDSCH DM-RS within the same CDM group are quasi co-located with respect to Doppler shift, Doppler spread, average delay, delay spread, and spatial Rx. Except for a PDSCH associated with two TCI states, the UE may assume that DMRS ports associated with a PDSCH are QCL with QCL Type A, Type D (when applicable) and average gain. The UE may assume that no DM-RS collides with the SS/PBCH block.

Please input your views and comments on these two alternatives:

| Company | Views and comments |
| --- | --- |
| CATT | Regarding the TP proposed in [12], we have the following comments: - Considering the fact that in scheme 3 and 4, DMRS ports associated with a PDSCH are QCL-ed in each transmission occasion, the newly added condition proposed in [12] is still not precise enough. - To address the issue of inaccurate description of QCL in 211, two approaches can be considered. - Alt-1: elaborate all the QCL cases as accurate as possible in 211 - Alt-2: for QCL related descriptions in 211, cite corresponding parts in 214 For the above alternatives, alt-2 is preferred. |

- Reference

- R1-2003397	On remaining issues on M-TRP	vivo

- R1-2003469	Maintenance of multi-TRP enhancements	ZTE

- R1-2003531	Remaining issues on multi-TRP in R16	Huawei, HiSilicon

- R1-2003627	Discussion on remaining issues of multi-TRP/panel transmission	CATT

- R1-2003660	Remaining issues on multi-TRP transmission	MediaTek Inc.

- R1-2003742	Corrections to multi-TRP	Intel Corporation

- R1-2003819	Remaining issues on multi-TRP/panel transmission	Lenovo, Motorola Mobility

- R1-2003881	On Rel.16 multi-TRP/panel transmission	Samsung

- R1-2003928	Text proposals on enhancements on multi-TRP/panel transmission	LG Electronics

- R1-2003954	Remaining issues on multi-TRP/panel transmission	CMCC

- R1-2003987	Discussion on remaining issues of multi-TRP operation	Spreadtrum Communications

- R1-2004047	Text proposals for enhancements on multi-TRP and panel Transmission	OPPO

- R1-2004229	Remaining issues for Multi-TRP enhancement	Apple

- R1-2004265	Maintenance of Rel-16 Multi-TRP operation	Nokia, Nokia Shanghai Bell

- R1-2004311	Remaining issues on multi-TRP transmission	NEC

- R1-2004395	Remaining issues on multi-TRP/panel transmission	NTT DOCOMO, INC

- R1-2004432	Remaining issues on Multi-TRP/Panel Transmission	Ericsson

- R1-2004463	Multi-TRP Enhancements	Qualcomm Incorporated

- R1-2004592	Clarification on Multi-TRP URLLC Scheme 4	Convida Wireless

- R1-2004719  FL summary #2 for Multi-TRP/Panel Transmission Moderator(OPPO)