3GPP TSG RAN WG1 Meeting #101-E															R1-200xxxx

e-Meeting, May 25 – June 05, 2020

Source: 	Moderator (Intel Corporation)

Title:	Summary of email discussions for [101-e-NR-Mob-Enh-01]

Agenda item:	7.2.9

Document for:	Discussion

# Introduction

In this contribution, we summarize the email discussion approved for discussion during RAN1 #101-E. Chairman has approved three email discussion threads for RAN1 #101-E. The following are the approved email discussions:

- [101-e-NR-Mob-Enh-01] Email discussion/approval of Issue #1 (UL cancellation for DAPS) and #3 (UL overlapping transmission) in R1-2003747 by 5/29; if necessary, endorse the associated TPs by 6/4 – Daewon (Intel)

- [101-e-NR-Mob-Enh-02] Email discussion/approval of Issue #5 (Power sharing mode for UL DAPS-HO) in R1-2003747 by 5/28; if necessary, endorse the associated TPs by 6/3– Daewon (Intel)

- [101-e-NR-Mob-Enh-03] Email discussion/approval of Issue #6 (PDCCH monitoring in DL DAPS-HO) in R1-2003747 by 5/28; if necessary, endorse the associated TPs by 6/2– Daewon (Intel)

This contribution summarizes the email discussion for [101-e-NR-Mob-Enh-01].

# Email Discussion [101-e-NR-Mob-Enh-01]

This discussion is regarding the uplink cancellation and uplink transmission overlapping issue in UL DAPS-HO (Issue #1 and #3 from [11]).

Issue and Proposal Summary:

## Issue #1) Uplink cancellation in UL DAPS-HO [1][2][3][5][6][8]

In RAN1 #100bis-e, TP on uplink cancellation rules for UL DAPS was agreed. However, the TP contained various brackets left for confirmation. Several companies have provided input on this issue. The following is a summary of proposals from contributions.

- Proposal [1]:

  - The timeline for cancellation is defined as PUSCH preparation time assuming d2,1 = 1 and SCS is the smallest SCS between the SCS configuration of PDCCH in the target cell and the SCS configuration of the UE transmission on the source cell.

  - Symbol-based cancellation should be supported for DAPS handover.

  - UL cancellation should be mandatory for a UE supporting DAPS handover and no separate UE capability is needed.

  - The following is the proposed TP:

### TP#1-1

15 Dual active protocol stack based handover <---------------------------Other parts are omitted -------------------------------> If - the UE isdoes not provided with UplinkPowerSharingDAPS-HO-mode, and - UE transmissions on the target cell and the source cell overlap the UE transmits only on the target cell, and cancels the transmission to source cell after the PUSCH preparation time Tproc,2 for the corresponding PUSCH processing capability [6, TS 38.214] assuming d2,1 = 1 after a last symbol of a CORESET where the UE detects a DCI format scheduling the transmission on the target cell and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format and the SCS configuration of the UE transmission on the source cell. If the UE transmits PRACH using 1.25 kHz or 5 kHz SCS on the source cell, the UE determines Tproc,2 assuming SCS configuration μ=0. A UE does not expect to cancel a transmission on the source cell in symbols from the set of symbols that occur, relative to a last symbol of a CORESET where the UE detects a DCI format scheduling a transmission on the target cell, after a number of symbols that is smaller than the PUSCH preparation time Tproc,2 for the corresponding PUSCH processing capability [6, TS 38.214] assuming d2,1 = 1 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format and the SCS configuration of the UE transmission on the source cell. If the UE transmits PRACH using 1.25 kHz or 5 kHz SCS on the source cell, the UE determines Tproc,2 assuming SCS configuration μ=0 <---------------------------Other parts are omitted ------------------------------->

- Proposal [2]:

  - The complexity for DAPS cancellation is not the same as cancellation for SFI and suggest relaxing the cancellation timeline.

  - The UE doesn’t need to treat UL transmissions sub-sequent to a cancelled UL transmission in a special manner.

  - The following is the proposed TP:

### TP#1-2

< Start of the text proposal > 15 Dual active protocol stack based handover < Unchanged parts are omitted > If - the UE does not provide UplinkPowerSharingDAPS-HO, and - UE transmissions on the target cell and the source cell overlap the UE transmits only on the target cell, and cancels the transmission to source cell after Tproc,2+2, where Tproc,2 is the PUSCH preparation time for the corresponding PUSCH processing capability [6, TS 38.214] assuming d2,1 = 1 after a last symbol of a CORESET where the UE detects a DCI format scheduling the transmission on the target cell and μ corresponds to the smallest SCS configuration among the SCS configuration of the PDCCH carrying the DCI format, the SCS configuration of the UE transmission on the target cell, and the SCS configuration of the UE transmission on the source cell. If the UE transmits PRACH using 1.25 kHz or 5 kHz SCS on the source cell, the UE determines Tproc,2 assuming SCS configuration μ=0. A UE does not expect to cancel a transmission on the source cell that occurs, relative to a last symbol of a CORESET where the UE detects a DCI format scheduling a transmission on the target cell, after a number of symbols that is smaller than Tproc,2+2 for the corresponding PUSCH processing capability [6, TS 38.214] assuming d2,1 = 1 and μ corresponds to the smallest SCS configuration among the SCS configuration of the PDCCH carrying the DCI format, the SCS configuration of the UE transmission on the target cell, and the SCS configuration of the UE transmission on the source cell. If the UE transmits PRACH using 1.25 kHz or 5 kHz SCS on the source cell, the UE determines Tproc,2 assuming SCS configuration μ=0. < End of the text proposal >

- Proposal [3]:

  - UE either cancels the whole source UL transmission, or keep the whole source UL transmission, depending on the beginning time of the source UL transmission, relative to the ending time of received target CG DCI.

Figure from [3]: Symbol level UL transmission cancellation from agreed TP for DAPS

  - The following is the proposed TP:

### TP#1-3

# 15 Dual active protocol stack based handover If - the UE does not provide UplinkPowerSharingDAPS-HO, and - UE transmissions on the target cell and the source cell overlap the UE transmits only on the target cell, and cancels the whole transmission to source cell if the occasion of the first symbol of source cell transmission is after [the PUSCH preparation time Tproc,2 for the corresponding PUSCH processing capability [6, TS 38.214] assuming d2,1 = 1 after a last symbol of a CORESET where the UE detects a DCI format scheduling the transmission on the target cell and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format and the SCS configuration of the UE transmission on the source cell. If the UE transmits PRACH using 1.25 kHz or 5 kHz SCS on the source cell, the UE determines Tproc,2 assuming SCS configuration μ = 0.] A UE does not expect to cancel a transmission on the source cell [in symbols from the set of symbols] with first symbol that occurs, relative to a last symbol of a CORESET where the UE detects a DCI format scheduling a transmission on the target cell, after a number of symbols that is smaller than the [PUSCH preparation time Tproc,2 for the corresponding PUSCH processing capability [6, TS 38.214] assuming d2,1 = 1 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format and the SCS configuration of the UE transmission on the source cell. If the UE transmits PRACH using 1.25 kHz or 5 kHz SCS on the source cell, the UE determines Tproc,2 assuming SCS configuration μ = 0].

- Proposal [5]:

  - Tproc,2 provides a reasonable timeline requirement for UE for canceling.

  - UE behavior for symbol based cancellation has already existed in Rel-15, there is no apparent reason that UE cannot support it. The system performance is clear better with the symbol based cancellation.

  - Support UL cancellation rules for Msg 3.

  - The following is the proposed TP:

### TP#1-4

Text proposal #1 for section 15 in TS38.213 A UE does not expect to cancel a transmission on the source cell in symbols from the set of symbols that occur, relative to a last symbol of a PDSCH reception conveying a RAR message with a RAR UL grant on the target cell, after a number of symbols that is smaller than msec, where is a time duration of symbols corresponding to a PDSCH processing time for UE processing capability 1 when additional PDSCH DM-RS is configured, is a time duration of symbols corresponding to a PUSCH preparation time for UE processing capability 1 [6, TS 38.214] and the UE considers that and correspond to the smaller of the SCS configurations for the PDSCH on the target cell and the transmission on the source cell. For , the UE assumes [6, TS 38.214].

- Proposal [6]: Consider reformulating the agreement to simply the interpretation. Remove the brackets around the time offsets.

  - The following is the proposed TP:

### TP#1-5

15 Dual active protocol stack based handover < Unchanged parts are omitted > If - the UE does not provide UplinkPowerSharingDAPS-HO, and - UE transmissions on the target cell and the source cell overlap the UE transmits only on the target cell the UE transmits only on the target cell, and cancels the transmission to source cell after the PUSCH preparation time Tproc,2 for the corresponding PUSCH processing capability [6, TS 38.214] assuming d2,1 = 1 after a last symbol of a CORESET where the UE detects a DCI format scheduling the transmission on the target cell and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format and the SCS configuration of the UE transmission on the source cell. If the UE transmits PRACH using 1.25 kHz or 5 kHz SCS on the source cell, the UE determines Tproc,2 assuming SCS configuration μ = 0. The UE does not expect to cancel a transmission on the source cell the PUSCH preparation time Tproc,2 for the corresponding PUSCH processing capability [6, TS 38.214] assuming d2,1 = 1 after a last symbol of a CORESET where the UE detects a DCI format scheduling the transmission on the target cell and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format and the SCS configuration of the UE transmission on the source cell. If the UE transmits PRACH using 1.25 kHz or 5 kHz SCS on the source cell, the UE determines Tproc,2 assuming SCS configuration μ = 0.

- Proposal [8]:

  - UL cancellation behaviour can be based simply to the defined timeline

  - The following is the proposed TP:

### TP#1-6

If - the UE does not provide UplinkPowerSharingDAPS-HO, and - UE transmissions on the target cell and the source cell overlap the UE transmits only on the target cell, and cancels the transmission to source cell after the PUSCH preparation time Tproc,2 for the corresponding PUSCH processing capability [6, TS 38.214] assuming d2,1 = 1 after a last symbol of a CORESET where the UE detects a DCI format scheduling the transmission on the target cell and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format and the SCS configuration of the UE transmission on the source cell. If the UE transmits PRACH using 1.25 kHz or 5 kHz SCS on the source cell, the UE determines Tproc,2 assuming SCS configuration μ=0. A UE does not expect to cancel a transmission to the source cell in symbols that occur, relative to a last symbol of a CORESET where the UE detects a DCI format scheduling a transmission on the target cell, after a number of symbols that is smaller than the PUSCH preparation time Tproc,2 for the corresponding PUSCH processing capability [6, TS 38.214] assuming d2,1 = 1 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format and the SCS configuration of the UE transmission on the source cell. If the UE transmits PRACH using 1.25 kHz or 5 kHz SCS on the source cell, the UE determines Tproc,2 assuming SCS configuration μ=0.

## Issue #3) Overlapping UL transmission between source and target cells [1][2][4][5][8]

The issue for PRACH and PUSCH/PUCCH/SRS overlapping scenarios was discussed in RAN1 #100-E meeting. However, some companies have provided further input to the issue.

Contribution in [1] mentioned that that definition of overlapping slots for PRACH and PUSCH/PUCCH/SRS could be ambiguous when target and source cell are synchronized. Therefore, suggests updating the description.

- Proposed TP from [1]:

### TP#2-1

15 Dual active protocol stack based handover <---------------------------Other parts are omitted -------------------------------> For DAPS operation in a same frequency band, a UE does not transmit PUSCH/PUCCH/SRS to the source MCG in a slot when the transmission would overlapping in time with a PRACH transmission to the target MCG or when a gap between a first or last symbol of a PRACH transmission to the target MCG in a first slot would be separated by less than N symbols from a last or first symbol, respectively, of the PUSCH/PUCCH/SRS transmission to the source MCG in a second slot. N=2 for μ=0 or μ=1, N=4 for μ=2 or μ=3, and μ is the SCS configuration of the active UL BWP for the PUSCH/PUCCH/SRS transmission to source MCG.

Contribution in [2] noted that if the time gap between source and target cell PUSCH/PUCCH/SRS is too short UE may not be able to switch the transmissions, especially if UE is using the same RF chain to serve target and source cells.

Figure from [2]: Gap between UL transmission to source MCG and UL transmission to target MCG

- Proposed TP from [2]:

### TP#2-2

<Unchanged parts are omitted> For DAPS operation in a same frequency band, a UE does not transmit PUSCH/PUCCH/SRS to the source MCG in a slot when the transmission would overlap in time with a PRACH transmission to the target MCG or when a gap between a first or last symbol of a PRACH transmission to the target MCG in a first slot would be separated by less than N symbols from a last or first symbol, respectively, of the PUSCH/PUCCH/SRS transmission to the source MCG in a second slot. N=2 for μ=0 or μ=1, N=4 for μ=2 or μ=3, and μ is the SCS configuration of the active UL BWP for the PUSCH/PUCCH/SRS transmission to source MCG. For DAPS operation in a same frequency band, a UE does not transmit PUSCH/PUCCH/SRS to the source MCG in a slot when a gap between a first or last symbol of a PUSCH/PUCCH/SRS transmission to the target MCG in a first slot would be separated by less than N symbols from a last or first symbol, respectively, of the PUSCH/PUCCH/SRS transmission to the source MCG in a second slot. N=1 for μ=0 or μ=1 or μ=2，N=2 for μ=3, and μ is the SCS configuration of the active UL BWP for the PUSCH/PUCCH/SRS transmission to source MCG.

Additionally, [4] and [5] mentioned the UE behaviour for when the UE needs to transmit PRACH in the source cell and PUSCH/PUCCH/SRS is missing from specification.

- Proposed TP from [4]:

### TP#2-3

15 Dual active protocol stack based handover < Unchanged parts are omitted > For DAPS operation in a same frequency band, a UE does not transmit PUSCH/PUCCH/SRS to source MCG in a slot overlapping in time domain with PRACH transmission to target MCG or when a gap between the first or last symbol of a PRACH transmission to target MCG in a first slot is separated by less than N symbols from the last or first symbol, respectively, of a PUSCH/PUCCH/SRS transmission to source MCG in a second slot. N = 2 for µ=0 or µ=1, N=4 for µ=2 or µ=3, and µ is the SCS configuration of the active UL BWP for PUSCH/PUCCH/SRS transmission to the source MCG. For DAPS operation in a same frequency band, a UE does not transmit PUSCH/PUCCH/SRS to target MCG in a slot overlapping in time domain with PRACH transmission to source MCG or when a gap between the first or last symbol of a PRACH transmission to source MCG in a first slot is separated by less than N symbols from the last or first symbol, respectively, of a PUSCH/PUCCH/SRS transmission to target MCG in a second slot. N = 2 for µ=0 or µ=1, N=4 for µ=2 or µ=3, and µ is the SCS configuration of the active UL BWP for PUSCH/PUCCH/SRS transmission to the target MCG.

- Proposed TP from [5]:

### TP#2-4

For DAPS operation in a same frequency band, a UE does not transmit PUSCH/PUCCH/SRS to the source MCG in a slot when the transmission would overlap in time with a PRACH transmission to the target MCG or when a gap between a first or last symbol of a PRACH transmission to the target MCG in a first slot would be separated by less than N symbols from a last or first symbol, respectively, of the PUSCH/PUCCH/SRS transmission to the source MCG in a second slot. N=2 for μ=0 or μ=1, N=4 for μ=2 or μ=3, and μ is the SCS configuration of the active UL BWP for the PUSCH/PUCCH/SRS transmission to source MCG. For DAPS operation in a same frequency band, a UE does not transmit PRACH to the source MCG in a slot when the transmission would overlap in time with a PUSCH/PUCCH/SRS transmission to the target MCG or when a gap between a first or last symbol of a PUSCH/PUCCH/SRS transmission to the target MCG in a first slot would be separated by less than N symbols from a last or first symbol, respectively, of the PRACH transmission to the source MCG in a second slot. N=2 for μ=0 or μ=1, N=4 for μ=2 or μ=3, and μ is the SCS configuration of the active UL BWP for the PUSCH/PUCCH/SRS transmission to target MCG.

- Proposal in [8]:

  - When PRACH transmission in source cell would be scheduled to occur in same slot or be separated by less than N symbols from PUSCH/PUCCH/SRS in target cell, UE is not required to transmit PRACH in source UL BWP.

  - The following is proposed TP:

### TP#2-5

For DAPS operation in a same frequency band, a UE does not transmit PUSCH/PUCCH/SRS to the source MCG in a slot when the transmission would overlap in time with a PRACH transmission to the target MCG or when a gap between a first or last symbol of a PRACH transmission to the target MCG in a first slot would be separated by less than N symbols from a last or first symbol, respectively, of the PUSCH/PUCCH/SRS transmission to the source MCG in a second slot. For DAPS operation in a same frequency band, a UE does not transmit PRACH in active UL BWP of source MCG in a same slot when the transmission would overlap in time with a PUSCH/PUCCH/SRS transmission to the target MCG or when a gap between the first or last symbol of a PUSCH/PUCCH/SRS transmission in active UL BWP of target MCG in a first slot is separated by less than N symbols from the last or first symbol, respectively, of a PRACH transmission in active UL BWP of source MCG in a second slot. N=2 for μ=0 or μ=1, N=4 for μ=2 or μ=3, and μ is the SCS configuration of the active UL BWP for the PUSCH/PUCCH/SRS transmission to source MCG.

Discussion Summary:

The issues and text proposals made in the submitted contributions can be largely categorized into 3 groups.

Group 1) Clarification on whether uplink cancellation should be for partial transmission (in units of symbols) or for the whole transmission

- Approach A) partial transmission

  - TP #1-1, TP#1-2

- Approach B) whole transmission

  - TP#1-3

Group 2) Supporting uplink cancellation timeline behavior for Msg 3

- RAN1 should determine whether or not such behavior needs to be explicitly defined or not.

- If need to be define whether TP #1-4 is acceptable.

Group 3) Reformulation the existing agreed text for better readability

- It was mentioned that the agreed text is quite difficult to parse and read. Suggests some reformulation (e.g. using ‘after’ in the first paragraph and ‘before’ in the second paragraph.

- TP#1-5

  - Feature lead note: may need to double check whether TP#1-5 is the intended text.

- Feature lead note: if the timeline boundary for determining when UE can perform cancellation and when UE does not expect to receive control signal is the same, the text proposal can be cleaned up. Therefore, FL suggests resolving Group 3 issue (reformulate for clarity) after other issues have been resolved.

Group 4) clarification of overlapping to avoid ambiguity for synchronized source and target MCG scenarios

- RAN1 should determine whether TP#2-1 is acceptable or not.

Group 5) Providing a protection gap between source and target MCG transmissions

Figure from [2]: Gap between UL transmission to source MCG and UL transmission to target MCG

- RAN1 should determine whether specification should support UE behavior so that UE does not need to handle cases with too short time gap between source and target PUSCH/PUCCH/SRS transmissions. Huawei has mentioned that time gap between source and target cell PUSCH/PUCCH/SRS is too short UE may not be able to switch the transmissions, especially if UE is using the same RF chain to serve target and source cells.

- If agreeable, RAN1 should determine whether TP#2-2 is acceptable or not.

Group 6) handling PRACH transmission in source MCG that overlaps with PUSCH/PUCCH/SRS transmission in target MCG

- RAN1 should determine whether specification should support the UE behavior to handle PRACH transmission in source MCG that overlaps with PUSCH/PUCCH/SRS transmission in target MCG. If need to be handled, which TP among TP#2-3, 2-4, or 2-5 is acceptable.

Companies are encouraged to provide comments on the proposal above. Please note comments for Group 1, 2, and 3, and Group 4, 5, and 6 are split into two different comment tables below.

| Company Name | Group 1 (prefer approach A/B) | Group 2 (agree/disagree in principle) | Group 3 (agree/disagree reformulation would help readability) | Comments |
| --- | --- | --- | --- | --- |
| Ericsson | A | Disagree | Agree | For group 1, it is beneficial to stop UL transmission to source as soon as possible. Restarting the transmission to source after the transmission to target is unnecessary. |
| Huawei, HiSilicon | A | Need more discussion | Ok in principle | For group 2, More discussion is needed. Settling down the timeline for the general case should be prioritized. |
| Intel | A | agree | Agree in principle. | We may want to clean up the text a bit, as there were confusions with the TP in the previous meeting. |
| Samsung | A, TP1-1 is preferred | Agree in principle | disagree | For Group 1, as we stated in previous meeting and T-doc. We think partial cancellation benefits both UE & gNB. As for TP1-1 or TP1-2, we choose TP1-1 as we think Tprocess,2 is enough. For Group 2, if agreed, the TP can be further improved once TP associated with Group1&3 group 1 is stable. Group 3: We think the current text already exists in Claus 11.1 of 38.213 for some time and should be acceptable. However, we are okay if company provides better wording without changing the meaning. |
| Apple | B | Disagree | Agree in principle | For group 1, we support to cancel the whole UL transmission to source cell. For the timeline, Tproc,2 is not enough. DAPS is based on the DC structure, each CG would require one modem, thus two moderns need time to exchange the scheduling information. We prefer to re-use the NR-DC defined timeline, i.e., T_offset. If UE supports dynamic power sharing, the UL cancellation is naturally supported without additional effort. The late TP is showing below for your consideration. If - the UE does not provide UplinkPowerSharingDAPS-HO, and - UE transmissions on the target cell and the source cell overlap the UE transmits only on the target cell, and cancels the whole transmission to source cell if the occasion of the first symbol of source cell transmission is after 𝑇offset after a last symbol of a CORESET where the UE detects a DCI format scheduling the transmission on the target cell, where 𝑇offset is defined in Clause 7.6.2, The UE does not expect to have transmissions on the target cell that - are scheduled by DCI formats in PDCCH receptions with a last symbol that is earlier by less than or equal to 𝑇offset from the first symbol of the transmission occasion on the source cell, and - overlap with the transmission occasion on the source cell. For Group 2, we don’t see the necessity to define additional timeline for msg3. If NR-DC timeline is re-used, the offset is larger enough according to UE report capability. For Group 3, the wording can be updated after Group 1 is stable. |

| Company Name | Group 4 (agree/disagree) | Group 5 (agree/disagree in principle) | Group 6 (agree/disagree in principle) | Comments |
| --- | --- | --- | --- | --- |
| Ericsson | Disagree | Disagree | Agree | For group 4: there seems to be no difference. For group 5: unnecessary: the dropping rule is sufficient |
| Huawei, HiSilicon | Need more discussion | agree | agree | For Group4, I failed to understand how the change is motivated by the concerned synchronized source and target MCG scenarios, so appreciated more explanation. For group 5, the dropping rule is only for overlapping case, TP in group 5 intends to cover the case where source and target are not overlapping but with a gap. |
| Intel | Need discussion | disagree | agree | For group 4, not entirely sure what is being changed. For group 5, we assumed the dropping rule will provide enough processing such that UE can cancel transmissions. Once the UE is aware, the cancellation of a transmission should be feasible. The gap (if needed) should be much smaller (a fraction) than the PA transient period. |
| Samsung | Agree and TP 2-1 is acceptable | disagree | Agree in principle 2-3 or 2-5 is preferred | For Group 4, we recalled TP 2-1 is indeed close to the final agreed TP in the email during RAN1-100e. For Group 5, this is a new behavior we haven’t agreed before. From our point of view, this gap is only needed for some UE implementations. We think dealing with this issue could be part of the DAPS HO basic capability and not needed to be in the spec. For Group 6, choice of TP really depends on which prioritization we want under this condition. (target cell PUSCH or source cell PRACH ). We prefer prioritizing for target cell transmission to be consistent with general principle in DAPS-HO. But we open to discuss the prioritization. |
| Apple | Open to discuss | Open to discuss | Agree | For Group 4, don’t see the difference, we are open to discuss it. For group 5, the target cell and source cell are in the same band, not sure the RF retuning time is needed in this case. And if NR-DC timeline is re-used, the target cell UE would check whether UL transmission is collision/overlap before the transmission, so this case can be avoided. |

# Conclusion of Email Discussion [101-e-NR-Mob-Enh-01]

Summary of email discussion outcome:

- xxx

# Reference

- R1-2003330, “Remaining issues on NR mobility enhancements in physical layer,” ZTE

- R1-2003506, “Remaining issues on DAPS-HO,” Huawei, HiSilicon

- R1-2003676, “Remaining issues on Physical Layer Aspects for  DAPS-HO,” MediaTek Inc.

- R1-2003748, “Corrections to Physical layer aspects of NR mobility enhancement,” Intel Corporation

- R1-2003890, “Remaining issues for NR Mobility Enhancement,” Samsung

- R1-2004202, “Remaining issues on mobility enhancements,” Ericsson

- R1-2004235, “On remaining issues on NR mobility enhancements,” Apple

- R1-2004580, “Remaining physical layer aspects of dual active protocol stack based HO,” Nokia, Nokia Shanghai Bell

- R1-2003331, “Discussion on FR2 mobility interruption enhancements,” ZTE

- R1-2004148, “Remaining PHY aspects for CHO,” Huawei, HiSilicon

- R1-2003747, “Issue Summary for NR Mobility Enhancements,” Moderator (Intel Corporation)