3GPP TSG RAN WG1 Meeting #101-E															R1-2004748

e-Meeting, May 25 – June 05, 2020

Source: 	Moderator (Intel Corporation)

Title:	Summary of email discussions for [101-e-NR-Mob-Enh-02]

Agenda item:	7.2.9

Document for:	Discussion

# Introduction

In this contribution, we summarize the email discussion approved for discussion during RAN1 #101-E. Chairman has approved three email discussion threads for RAN1 #101-E. The following are the approved email discussions:

- [101-e-NR-Mob-Enh-01] Email discussion/approval of Issue #1 (UL cancellation for DAPS) and #3 (UL overlapping transmission) in R1-2003747 by 5/29; if necessary, endorse the associated TPs by 6/4 – Daewon (Intel)

- [101-e-NR-Mob-Enh-02] Email discussion/approval of Issue #5 (Power sharing mode for UL DAPS-HO) in R1-2003747 by 5/28; if necessary, endorse the associated TPs by 6/3– Daewon (Intel)

- [101-e-NR-Mob-Enh-03] Email discussion/approval of Issue #6 (PDCCH monitoring in DL DAPS-HO) in R1-2003747 by 5/28; if necessary, endorse the associated TPs by 6/2– Daewon (Intel)

This contribution summarizes the email discussion for [101-e-NR-Mob-Enh-02].

# Email Discussion [101-e-NR-Mob-Enh-02]

This discussion is regarding the power Sharing Mode for UL DAPS-HO (Issue #5 from [11]).

The main focus of the issue to finalize the specification based on agreement made in RAN1 #100bis-e:

Agreement from RAN1 #100bis-e: - gNB can configure for the UE a specific power sharing mode for DAPS - It is assumed that gNB shall only enable a power sharing mode for DAPS among the power sharing modes that the UE indicated support of. - gNB can disable power sharing between target and source MCG - no power sharing between target and source MCG can be indicated by gNB not configuring UplinkPowerSharingDAPS-HO-mode.

Issue and Proposal Summary based on [1][2][4][5][6][7]:

Several companies provided discussion on how to correct the power sharing mode description for UL DAPS-HO. The following are list of proposals and corresponding TPs:

- Proposal [1]: When no power sharing is configured by the network, a UE should cancel the source cell transmission in case of UL collision in the time domain.

  - The following is proposed TP:

## TP #1

15 Dual active protocol stack based handover <---------------------------Other parts are omitted -------------------------------> If the UE indicates UplinkPowerSharingDAPS-HO = Dynamic and is provided UplinkPowerSharingDAPS-HO-mode = Dynamic, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 7.6.2 for UplinkPowerSharingDAPS-HO = Dynamic by considering the target MCG as the MCG and the source MCG as the SCG. If - the UE is not provided with UplinkPowerSharingDAPS-HO-mode, and - UE transmissions on the target cell and the source cell are in overlapping time resources the UE transmits only on the target cell. <---------------------------Other parts are omitted ------------------------------->

- Proposal [2]: When no power sharing is configured by the network, a UE should cancel the source cell transmission in case of UL collision in the time domain.

  - The following is proposed TP:

## TP #2

15 Dual active protocol stack based handover < Unchanged parts are omitted > If the UE is not provided UplinkPowerSharingDAPS-HO-modedoes not provide UplinkPowerSharingDAPS-HO, and - UE transmissions on the target cell and the source cell overlap the UE transmits only on the target cell.

- Proposal [4]:

  - Agreement and capability signaling, the TS38.213 should add a separate text when gNB does not configure UplinkPowerSharingDAPS-HO-mode configuration, which should imply UE always performs dropping of the source cell transmission during transmission overlap in time domain.

  - Text that couples the UE capability with gNB configured mode can be cleaned up by having a generic text that states UE is not expected to be configured with power sharing mode that it does not support.

  - The following is proposed TP:

## TP #3

## 15 Dual active protocol stack based handover < Unchanged parts are omitted > If the UE indicates UplinkPowerSharingDAPS-HO = Semistatic-mode1 and is provided UplinkPowerSharingDAPS-HO-mode = Semi-static-mode1, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 7.6.2 for UplinkPowerSharingDAPS-HONR-DC-PC-mode = Semi-static-mode1 by considering the target MCG as the MCG and the source MCG as the SCG. If the UE indicates UplinkPowerSharingDAPS-HO = Semistatic-mode2 and is provided UplinkPowerSharingDAPS-HO-mode = Semi-static-mode2, the UE determines a transmission power for the target MCG or for the source SCG as described in Clause 7.6.2 for UplinkPowerSharingDAPS-HONR-DC-PC-mode = Semi-static-mode2 by considering the target MCG as the MCG and the source MCG as the SCG. If the UE indicates UplinkPowerSharingDAPS-HO = Dynamic and is provided UplinkPowerSharingDAPS-HO-mode = Dynamic, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 7.6.2 for UplinkPowerSharingDAPS-HONR-DC-PC-mode = Dynamic by considering the target MCG as the MCG and the source MCG as the SCG. [UE is not expected to be provided UplinkPowerSharingDAPS-HO-mode configuration that it did not indicate support of.] If the UE is not provided with UplinkPowerSharingDAPS-HO-mode, and UE transmissions on the target cell and the source cell are in overlapping time resources, the UE transmits only on the target cell. If - the UE does not provides UplinkPowerSharingDAPS-HO, and - UE transmissions on the target cell and the source cell overlap, the UE transmits only on the target cell. UE transmissions on the target cell and the source cell overlap if they are in - overlapping time resources if the carrier frequencies for the target MCG and the source MCG are intra-frequency and intra-band - overlapping time resources and overlapping frequency resources if the carrier frequencies for the target MCG and the source MCG are not intra-frequency and intra-band

- Proposal [5]:

  - The following is proposed TP:

## TP #4

If the UE indicates UplinkPowerSharingDAPS-HO = Semi-static-mode1 and is provided UplinkPowerSharingDAPS-HO-mode = Semi-static-mode1, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 7.6.2 for UplinkPowerSharingDAPS-HO NR-DC-PC-mode = Semi-static-mode1 by considering the target MCG as the MCG and the source MCG as the SCG. If the UE indicates UplinkPowerSharingDAPS-HO = Semi-static-mode2 and is provided UplinkPowerSharingDAPS-HO-mode = Semi-static-mode2, the UE determines a transmission power for the target MCG or for the source SCG as described in Clause 7.6.2 for UplinkPowerSharingDAPS-HO NR-DC-PC-mode = Semi-static-mode2 by considering the target MCG as the MCG and the source MCG as the SCG. If the UE indicates UplinkPowerSharingDAPS-HO = Dynamic and is provided UplinkPowerSharingDAPS-HO-mode = Dynamic, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 7.6.2 for UplinkPowerSharingDAPS-HO NR-DC-PC-mode = Dynamic by considering the target MCG as the MCG and the source MCG as the SCG. <unchanged text omitted> If - the UE does not provide UplinkPowerSharingDAPS-HO, or is not provided UplinkPowerSharingDAPS-HO-Mode and - UE transmissions on the target cell and the source cell are in overlapping time resources or - the UE does not is provided UplinkPowerSharingDAPS-HO-Mode, and - UE transmissions on the target cell and the source cell overlap the UE transmits only on the target cell UE transmissions on the target cell and the source cell overlap if they are in - overlapping time resources if the carrier frequencies for the target MCG and the source MCG are intra-frequency and intra-band - overlapping time resources and overlapping frequency resources if the carrier frequencies for the target MCG and the source MCG are not intra-frequency and intra-band For intra-frequency DAPS HO operation, the UE expects that an active DL BWP and an active UL BWP on the target cell are within an active DL BWP and an active UL BWP on the source cell, respectively. The UE determines intra-frequency as described in Clause 9.2.1 of [10, TS38.133].

- Proposal [6]: When the gNB disables power sharing, the behaviour should be the same as when the UE does not provide UplinkPowerSharingDAPS-HO.

  - The following is proposed TP:

## TP #5

## 15 Dual active protocol stack based handover < Unchanged parts are omitted > If - the UE does not provide UplinkPowerSharingDAPS-HO, or is not provided UplinkPowerSharingDAPS-HO-mode, and - UE transmissions on the target cell and the source cell overlap the UE transmits only on the target cell

- Proposal in [7]:

  - If gNB disables the power sharing between target and source cell, UE would assume the UL transmission is in TDM manner to source and target cell. If any UL transmission collision, the UE behavior is not specified.

  - Define the UE capability for UL transmission cancellation.

  - NR-DC based UL power control adjustment timeline can be considered by UL transmission cancellation in DPAS HO.

  - The following is proposed TP:

## TP #6

15 Dual active protocol stack based handover If - the UE is not provided with UplinkPowerSharingDAPS-HO-mode, UE does not expect the UL transmission on the target cell and source cell are overlapping in time resources Or if - the UE does not provide UplinkPowerSharingDAPS-HO, and - UE transmissions on the target cell and the source cell overlap the UE transmits only on the target cell UE transmissions on the target cell and the source cell overlap if they are in - overlapping time resources if the carrier frequencies for the target MCG and the source MCG are intra-frequency and intra-band - overlapping time resources and overlapping frequency resources if the carrier frequencies for the target MCG and the source MCG are not intra-frequency and intra-band For intra-frequency DAPS HO operation, the UE expects that an active DL BWP and an active UL BWP on the target cell are within an active DL BWP and an active UL BWP on the source cell, respectively.

- Text Proposal in [8]:

## TP #7

If the UE indicates support for UplinkPowerSharingDAPS-HO = Semistatic-mode1 semi-static power sharing mode1 and is provided UplinkPowerSharingDAPS-HO-mode = Semi-static-mode1, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 7.6.2 for UplinkPowerSharingDAPS-HONR-DC-PC-mode = Semi-static-mode1 by considering the target MCG as the MCG and the source MCG as the SCG. If the UE indicates support for UplinkPowerSharingDAPS-HO = Semistatic-mode2 semi-static power sharing mode2 and is provided UplinkPowerSharingDAPS-HO-mode = Semi-static-mode2, the UE determines a transmission power for the target MCG or for the source SCG as described in Clause 7.6.2 for UplinkPowerSharingDAPS-HONR-DC-PC-mode = Semi-static-mode2 by considering the target MCG as the MCG and the source MCG as the SCG. If the UE indicates support forUplinkPowerSharingDAPS-HO = Dynamic dynamic power sharing and is provided UplinkPowerSharingDAPS-HO-mode = Dynamic, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 7.6.2 for UplinkPowerSharingDAPS-HONR-DC-PC-mode = Dynamic by considering the target MCG as the MCG and the source MCG as the SCG. If - the UE isdoes not provided UplinkPowerSharingDAPS-HO-mode, and - UE transmissions on the target cell and the source cell overlap the UE transmits only on the target cell UE transmissions on the target cell and the source cell overlap if they are in - overlapping time resources if the carrier frequencies for the target MCG and the source MCG are intra-frequency and intra-band - overlapping time resources and overlapping frequency resources if the carrier frequencies for the target MCG and the source MCG are not intra-frequency and intra-band

## Discussion Summary:

The text proposals made in the submitted contributions can be largely categorized into 3 groups.

Group 1) Indication of no power sharing between target and source MCG

- Approach A) add new text separate from existing text that handles overlap signals for intra-band and intra-frequency cases.

  - TP#1, TP#3, and TP#4

- Approach B) add the text to the existing text (by stating “or”) that handles overlap signals for intra-band and intra-frequency cases

  - TP #5, and TP#6

- Approach C) change ‘UplinkPowerSharingDAPS-HO’ to ‘UplinkPowerSharingDAPS-mode’ in the existing text handles overlap signals for intra-band and intra-frequency cases (with other minor modification)

  - TP#2, and TP#7

Group 2) Clean-up of existing text by removing the RRC parameter name in “if UE indications UplinkPowerSharingDAPS-HO = xxx”

- Approach A) replace the “if UE indications UplinkPowerSharingDAPS-HO = xxx” with a generic “if UE indicates supports of xxx”

  - TP#7

- Approach B) delete the “if UE indications UplinkPowerSharingDAPS-HO = xxx” and add a generic text that states UE does not expected to be configured with modes that it does not support.

  - TP#3

Group 3) correction of ‘UplinkPowerSharingDAPS-HO’ with ‘NR-DC-PC-mode’

- All mentioned in TP#3, TP#4, and TP#7

- This seems to be editorial in nature and should be straightforward.

Companies are encouraged to provide comments on the issue group 1, group 2, and group 3.

- For example, whether they prefer Approach X in Group 1 issue, Approach B in Group 2 issue, and agrees with Group 3 issue.

- If companies have another resolution for Group 1 and/or 2 issue other than what was mentioned, please provide information as well.

- If companies have concerns with certain approaches, please provide explanation.

- Also, if companies have a merged proposal based on proposal from above companies, please do provide them below as well.

| Company Name | Group 1 (approach A/B/C) | Group 2 (approach A/B) | Group 3 (agree/ disagree) | Comments/Views |
| --- | --- | --- | --- | --- |
| Huawei, HiSilicon | C | B | agree |  |
| Ericsson | B or C | A | agree | It is an error case that the UE is configured with something it does not support, and error cases are typically not described in the RAN1 specifications. |
| Intel | A (or B - TP#6 only) | A | agree | For group 1, the existing cases refer to intra-frequency and intra-band overlapping cases. gNB configuration of no power sharing would need to apply to any case (if configured). Therefore, we think changing the existing text either using approach B or C would not result in the same UE behavior. For example, in approach C, TP#2 and #7, its not clear what happens when the UE is not configured with power sharing mode (i.e. no power sharing) but configured with DAPS in inter-frequency. The description is completely missing. The same situation for TP#5. TP#6 is better in that it does not use the “overlap” definition that is defined by the existing text. For group 2, we are open whether we need to describe error cases. |
| Samsung | A or B (TP#6 only) | A | Agree | For Group 1, we think both approach B or C do not match RAN1-99 agreement on cancellation condition. The reason we came up with TP#6 in [5] is to avoid repeating the sentence “UE transmits only on the target cell”, which will be replaced by the long paragraph in issue#1. |
| Qualcomm | Need further discussion | A | Agree | For Group 1, we would like to have further discussion on UE behavior. If the UE needs to cancel UL Tx when the gNB does not configure any UL sharing mode, the UE needs to support UL cancellation capability FG 21-2d. This is different from the scenario where UE does not indicate the UL power sharing capability where the UE may try to support one of the sharing modes to avoid UL cancellation behavior. |
| Nokia | C | A | Agree | In context of the power sharing modes, like noted in our paper, having the reference to reported UE capability is not absolutely necessary (while that is done also in Section 7.6.2) as it would be erroneous configuration. |
| Apple | Need further discussion | A | Agree | For Group 1, in our understanding, the first thing need to do is to define the UE behaviour for so called “no power sharing”, then how to capture it in the specification is just wording issue. |
| ZTE | A | A | Agree | For Group 1, we have same understanding with Samsung. |
| MTK | A | A | Agree | For Group 1, we can agree on A generally, but we share same view as QC that it may be related to UL cancellation capability FG 21-2d. Besides, we see TP#1 as a general description and not limited to “intra-band and intra-frequency cases”. |

Tproc 2, cancel in MAC, Rel.16 cancel in physical layer

## Discussion Summary of all comments received by May 27, 11pm PDT (May 28, 6am UTC):

Majority of the companies seem to prefer taking approach A for Group 2, and agree with issue discussed in Group 3. Moderator has remove the other aspects from TP#7 and suggests to agree to TP#8 as a conclusion for Group 2 and 3 issue.

Moderator Suggestion for Agreement:

- Agree TP #8 of R1-2004749

## TP #8

If the UE indicates support for UplinkPowerSharingDAPS-HO = Semistatic-mode1 semi-static power sharing mode1 and is provided UplinkPowerSharingDAPS-HO-mode = Semi-static-mode1, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 7.6.2 for UplinkPowerSharingDAPS-HONR-DC-PC-mode = Semi-static-mode1 by considering the target MCG as the MCG and the source MCG as the SCG. If the UE indicates support for UplinkPowerSharingDAPS-HO = Semistatic-mode2 semi-static power sharing mode2 and is provided UplinkPowerSharingDAPS-HO-mode = Semi-static-mode2, the UE determines a transmission power for the target MCG or for the source SCG as described in Clause 7.6.2 for UplinkPowerSharingDAPS-HONR-DC-PC-mode = Semi-static-mode2 by considering the target MCG as the MCG and the source MCG as the SCG. If the UE indicates support forUplinkPowerSharingDAPS-HO = Dynamic dynamic power sharing and is provided UplinkPowerSharingDAPS-HO-mode = Dynamic, the UE determines a transmission power for the target MCG or for the source MCG as described in Clause 7.6.2 for UplinkPowerSharingDAPS-HONR-DC-PC-mode = Dynamic by considering the target MCG as the MCG and the source MCG as the SCG.

For Group 1 issue, companies seem to be somewhat split. There are more companies that prefer approach A or B (TP#6). Moderator would like to ask companies to focus on approach A or B (TP#6 based solution) for further discussion so that we can conclude.

Moderator has taken the relevant text from TP #1, #3, and #4 and merged them as TP #9.

Moderator Suggestion for Agreement:

- Agree on TP #6 or #9 of R1-2004749

## TP #9

15 Dual active protocol stack based handover <unchanged text omitted> If - the UE does not provide UplinkPowerSharingDAPS-HO, or the UE is not provided with UplinkPowerSharingDAPS-HO-mode, and - UE transmissions on the target cell and the source cell are in overlapping time resources the UE transmits only on the target cell. If - the UE does not provides UplinkPowerSharingDAPS-HO, and - UE transmissions on the target cell and the source cell overlap, the UE transmits only on the target cell.

## Discussion Summary after May 27, 11pm PDT (May 28, 6am UTC):

Companies are encouraged to provide comments on the following proposal. Especially, whether they have concerns with the suggested proposal. Also, if the suggestion made are acceptable, then please provide preference between TP#6 and #9.

Moderator Suggestion for Agreement:

- Agree TP #8 of R1-2004749

- Agree on TP #6 or #9 of R1-2004749

| Company Name | Comments/Views |
| --- | --- |
| Qualcomm | We are fine with TP#8 However, we’re not able to agree on either TP#6 or TP#9: - UE behavior 1 (when the gNB does not configure any UL sharing mode, the UE cancels the source transmission): This behavior is fine as long as the UE indicates the support of UL cancellation. Alternatively, we can have UE behavior that when the gNB does not configure any UL sharing mode, the UE expects that the UL transmissions to source and target do not overlap. - UE behavior 2 (when the UE does not provides UplinkPowerSharingDAPS-HO and UE transmissions on the target cell and the source cell overlap, the UE cancels the source transmission): This is not necessarily true. It really depends on the UE RF e.g., if the UE has 2 TX chains, it can do simultaneous transmissions to source and target for this case. |
| Moderator (Intel) | It seems Qualcomm is suggestion a different alternative to what no power sharing mode should describe. I’ve tried to capture what I think Qualcomm was suggesting in TP#10. Companies are encouraged to provide feedback. For the 2nd issue that Qualcomm commented above, the ‘overlap’ definition already defined in specification seems to already handle the situations that Qualcomm is concerned about. It would be good for Qualcomm to provide further feedback whether the ‘overlap’ definition define is sufficient or not. If not sufficient, then what further needs to be changed in specification. |
| MediaTek | Agree on TP #8. For TP #6 or #9, we prefer TP #9. |
| Intel | Agree on TP #8 Ok with TP#9. |
| Samsung | Agree with TP#8. Okay on TP#9. (please ignore our previous comments on TP#6, we mistakenly treated it the same as TP#4) |
| Huawei, HiSilicon | Agree with TP#8. Neither TP#6 nor TP#9 is agreeable to us. If UE does not provide UplinkPowerSharingDAPS-HO, it means UE does not support simultaneous transmission. In other words, gNB is not expected to schedule the uplink to two cells that collide. Otherwise, UE not provided the power sharing modes includes two cases: one is that UE does not support UplinkPowerSharingDAPS-HO, so gNB is not expected to schedule the uplink to two cells that collide; the one is that UE supports UplinkPowerSharingDAPS-HO but NW disables power sharing, for which UE’s behavior to transmit only on the target cell needs pre-requisite support of cancelation and gNB is not expected to schedule the uplink to two cells that collide if UE does not support UL cancelation. Therefore, based on the above understanding, neither TP#6 nor TP#9 and even nor TP#10 reflects the above. |
| Apple | We support TP#8. For TP#6 and TP#9, if our understanding is correct, the TP#10 can be updated as, If the UE does not provide UplinkPowerSharingDAPS-HO, or the UE is not provided with UplinkPowerSharingDAPS-HO-mode, UE is does not expect transmissions on the target and source cell in overlapping time resources. If - the UE does not provide UplinkPowerSharingDAPS-HO, and - UE transmissions on the target cell and the source cell overlap the UE transmits only on the target cell UE transmissions on the target cell and the source cell overlap if they are in - overlapping time resources if the carrier frequencies for the target MCG and the source MCG are intra-frequency and intra-band - overlapping time resources and overlapping frequency resources if the carrier frequencies for the target MCG and the source MCG are not intra-frequency and intra-band [Note: The above has been captured as TP#11 by Moderator] |
| ZTE | Agree with TP#8 For TP #6 or #9, we prefer TP #9. In our understanding, if a UE does not provide UplinkPowerSharingDAPS-HO, it means the UE does not support any power sharing scheme. So, the network will not configure power sharing scheme to the UE. In other words, only keeping ‘UE is not provided with UplinkPowerSharingDAPS-HO-mode’ is sufficient. But, if other companies have concerns, we can accept current TP#9. |
| Nokia | (While TP#8 is been already agreed, just to confirm that we are OK with it). TP#9 would be OK to us as well. For TP#10, it seems to propose UplinkPowerSharingDAPS-HO as an implicit capability for UL cancellation, i.e. if UE does not expect transmission on target and source in overlapping resources, there is no overlapping scheduling allowed. In our understanding UplinkPowerSharingDAPS-HO just determines whether UE can support power sharing between cells. If not and in case of overlap (in time and frequency), UE would transmit to target cell only. |
| Ericsson | We support TP#9 TP#10 is out of scope of this email discussion. |

## TP #10

15 Dual active protocol stack based handover <unchanged text omitted> If the UE does not provide UplinkPowerSharingDAPS-HO, or the UE is not provided with UplinkPowerSharingDAPS-HO-mode, UE does not expect transmissions on the target and source cell in overlapping time resources. If - the UE does not provides UplinkPowerSharingDAPS-HO, and - UE transmissions on the target cell and the source cell overlap, the UE transmits only on the target cell.

## TP #11

15 Dual active protocol stack based handover <unchanged text omitted> If the UE does not provide UplinkPowerSharingDAPS-HO, or the UE is not provided with UplinkPowerSharingDAPS-HO-mode, UE does not expect transmissions on the target and source cell in overlapping time resources. If - the UE does not provide UplinkPowerSharingDAPS-HO, and - UE transmissions on the target cell and the source cell overlap the UE transmits only on the target cell UE transmissions on the target cell and the source cell overlap if they are in - overlapping time resources if the carrier frequencies for the target MCG and the source MCG are intra-frequency and intra-band - overlapping time resources and overlapping frequency resources if the carrier frequencies for the target MCG and the source MCG are not intra-frequency and intra-band

## TP #12

15 Dual active protocol stack based handover <unchanged text omitted> If - the UE supports [UplinkCancellationDAPS-HO], and - the UE does not provide UplinkPowerSharingDAPS-HO or the UE is not provided with UplinkPowerSharingDAPS-HO-mode, and - UE transmissions on the target cell and the source cell are in overlapping time resources, the UE transmits only on the target cell. If - the UE does not support [UplinkCancellationDAPS-HO], and - the UE does not provide UplinkPowerSharingDAPS-HO or the UE is not provided with UplinkPowerSharingDAPS-HO-mode, the UE does not expect transmissions on the target and source cell in overlapping time resources. If - the UE does not provides UplinkPowerSharingDAPS-HO, and - UE transmissions on the target cell and the source cell overlap, the UE transmits only on the target cell.

## Discussion Summary of all comments received by June 02, 11pm PDT (June 03, 6am UTC):

TP#8 has been agreed by Chairman. The only issue left is the discussion on TP#6/#9/#10.

The following is summary of companies views on TP#9, #10, and #11 (proposed by Apple). Based on feedback from Huawei, moderator has formulated TP#12.

TP#6

- No company had provided preference for this. Therefore, moderator suggests to move this out of the discussion.

TP#9

- Supported by: MediaTek, Intel, Samsung, ZTE, Nokia, Ericsson

- Objected by: Qualcomm, Huawei, HiSilicon

  - Main reasons for objection seems to be tied to whether UE is able to support UL transmission cancellation or not.

TP#10 (TP drafted by Moderator based on Qualcomm feedback)

- Supported by:

- Objected by: Huawei, HiSilicon

- Out-of-scope: Ericsson

TP#11 (suggestion from Apple)

- Supported by: Apple

TP#12 (TP drafted Moderator based on Huawei, HiSilicon feedback)

- Supported by:

- Text modified to factor into account potential UL transmission cancellation capability

There seems to good support for TP#9. However, the main concerns for the TP is that the TP does not capture UL cancellation capability behaviors. Therefore, suggests the conclude on either TP#9 or TP#12 depending on the decision of UL cancellation capability.

Moderator Suggestion for Agreement:

- If UL cancellation capability (21-2d feature) is not supported in NR, agree on TP #9 of R1-2004749

- If UL cancellation capability (21-2d feature) is supported in NR, agree on TP #12 of R1-2004749

## Discussion Summary after May 27, 11pm PDT (May 28, 6am UTC):

Moderator suggest focusing the discussion on stabilizing the TP for agreement. Instead of focusing the discussion on whether TP is agreeable or not due to UL cancellation capability, companies are encouraged to provide feedback on both TP#9 and TP#12 assuming feature 21-2d is agreed or not agreed.

Please provide comments on

- whether TP#9 is acceptable if we assume feature 21-2d is not supported in Rel-16 NR, and

- whether TP#12 or TP#13 is acceptable if we assume feature 21-2d is supported in Rel-16 NR

- If neither TP#9 or TP#12/13 is acceptable, please provide an alternative TP.

| Company Name | Comments/Views |
| --- | --- |
| Huawei, HiSilicon | TP#9 needs modification as follows assuming feature 21-2d is not supported in Rel-16 NR: If - the UE does not provides UplinkPowerSharingDAPS-HO but the UE is not provided with UplinkPowerSharingDAPS-HO-mode, and - UE transmissions on the target cell and the source cell overlap, the UE transmits only on the target cell. Because if UE provides UplinkPowerSharingDAPS-HO, it means UE is able to simultaneously transmit the two uplinks, no need to drop one. [Note from Moderator: In order to not confuse the discussion, I’ve put the changes to TP#12 as TP#13 and kept TP#12 as original drafted by Moderator] |
| Moderator (Intel) | Quick question on TP#13 (provided by Huawei). What is the UE behavior if the UE does not support ul-cancellation capability and not provided with UplinkPowerSharingDAPS-HO-mode? It seems some cases are missing from specification. Is that intentional? |

## TP #13

15 Dual active protocol stack based handover <unchanged text omitted> If - the UE supports [UplinkCancellationDAPS-HO], and - the UE does not provides UplinkPowerSharingDAPS-HO and the UE is not provided with UplinkPowerSharingDAPS-HO-mode, and - UE transmissions on the target cell and the source cell overlap, the UE transmits only on the target cell.

# Conclusion of the Email Discussion [101-e-NR-Mob-Enh-02]

Summary of email discussion outcome:

The following were agreed.

Agreement:

- Adopt TP#8 in R1-2004748 for Clause 15 of TS 38.213.

# Reference

- R1-2003330, “Remaining issues on NR mobility enhancements in physical layer,” ZTE

- R1-2003506, “Remaining issues on DAPS-HO,” Huawei, HiSilicon

- R1-2003676, “Remaining issues on Physical Layer Aspects for  DAPS-HO,” MediaTek Inc.

- R1-2003748, “Corrections to Physical layer aspects of NR mobility enhancement,” Intel Corporation

- R1-2003890, “Remaining issues for NR Mobility Enhancement,” Samsung

- R1-2004202, “Remaining issues on mobility enhancements,” Ericsson

- R1-2004235, “On remaining issues on NR mobility enhancements,” Apple

- R1-2004580, “Remaining physical layer aspects of dual active protocol stack based HO,” Nokia, Nokia Shanghai Bell

- R1-2003331, “Discussion on FR2 mobility interruption enhancements,” ZTE

- R1-2004148, “Remaining PHY aspects for CHO,” Huawei, HiSilicon

- R1-2003747, “Issue Summary for NR Mobility Enhancements,” Moderator (Intel Corporation)