3GPP TSG RAN WG1 Meeting #102-e	                                                                               R1-2005257

E-meeting, August 17 – 28, 2020

Agenda Item:	8.8.1.2

Source:	Huawei, HiSilicon

Title:	Evaluation on the baseline performance for FR2

Document for:	Discussion and Decision

- Introduction

The study item of NR coverage enhancement was approved at RAN#86 meeting [1], and most of the evaluation methods and assumptions were agreed in RAN1#101e [2]. In this contribution, we discuss the remaining issues for the FR2 evaluations and provide some baseline performance evaluations.

- Discussions on the remaining issues on the link budget template for FR2

As agreed in the last meeting, the FR2 link budget calculation follows FR1, and our FR1 companion paper [3] suggested to use a link budget template based on IMT-2020 self-evaluation. The remaining issues for coverage evaluation include the array gain calculation for LLS based methodology, number of UE panels, and the parameter assumptions in the link budget.

Consideration on the antenna array gain and UE panel

Agreements: Down selection on the following options for antenna array gain for LLS based methodology for FR1 in next meeting. - Option 1: Antenna array gain is included in the link budget template. - FFS: array gain = 10 * 1og10 (number of antenna elements/number of TxRUs) - FFS: For TDL channel model - FFS: Values reflective of realistic implementation and network operation. - Option 2: Antenna array gain is included in LLS. - FFS: For CDL channel model Agreements: - For link level simulation, adopt the following simulation assumption for eMBB data or VoIP on PUSCH and on PDSCH for FR2 - Number of UE antenna elements : 8, one panel:(M, N, P) = (2,2,2), - FFS: Two panels in link budget, one panel in LLS, 16 for each panel: (M, N, P) = (4,2,2)

For FR2, both the base station and UE use narrow analog beams for data transmission and reception, and the beamforming put different weights on different spatial directions. The obtained array gain highly depends on the matching between the beam direction and the spatial angles of the propagation channel. For example, the power may be 3+ dB attenuated if the cluster angle deviates larger than the half power beamwidth (HPBW) of the main lobe, which reduces the actually obtained array gain. To achieve a more reasonable evaluation result, the antenna array gain should be included in LLS based on CDL channel model for FR2, and the analog beams used for LLS at the BS and UE should be further studied considering that the spatial angles of the channel model. Further, the antenna patterns defined in [4] can be used for LLS.

Proposal 1: The antenna array gain should be included in LLS based on CDL channel model for FR2.

Proposal 2: The analog beams used for LLS at the BS and UE should be further studied, and the beam directed to the strongest cluster in the CDL model can be a starting point.

Although multiple panels may be equipped by an FR2 UE, there are cases that only one panel is effective and the others are blocked. Hence, it is reasonable to use only one panel for both LLS and link budget in FR2 coverage evaluation.

Proposal 3: Only one panel should be considered for the link budget in FR2.

Consideration on the other remaining parameters

In the link budget template based on IMT-2020 self-evaluation, only some parameters are agreed in the last meeting. The remaining undetermined parameters are important for link budget calculation, which includes: the total transmit power, antenna element gain, interference, shadow fading, penetration, and pathloss model. As a startup, the assumptions are listed in Table 1 and most of them are from [4], [5]. In the table, the UE total transmit power (TRP) is assumed to be 16 dBm, which yields a maximum EIRP of 16 + 5 (dBi) + 10log10(8/2) - 1 = 26 dBm (8 is the agreed UE antenna elements number [2], 2 is the UE ports number, 1 is the feeder loss [5]). For FR2, the signal from adjacent cells is with large attenuation, thus the interference density is assumed to be the same as thermal noise for both uplink and downlink.

Proposal 4: The remaining parameters involved in the link budget should be studied, and the values in Table 1 should be considered.

Table 1 Assumptions on the remaining parameters in the link budget

| Parameters | Indoor hotspot | Urban Macro | Suburban |
| --- | --- | --- | --- |
| Total transmit power (dBm) [4] | BS: 23 UE: 16 | BS: 40 UE: 16 | BS: 40 UE: 16 |
| Antenna element gain (dBi) | BS: 5 UE: 5 | BS: 8 UE: 5 | BS: 8 UE: 5 |
| Receiver interference density (dBm/Hz) | All channel: -174 | All channel: -174 | All channel: -174 |
| Shadow fading (dB) [5] | 5.2 | O2O: 4.85 O2I: 6.41 | O2O: 6.61 O2I: 6.3 |
| Penetration margin (dB) | - | 41.55 Derived according to high loss model defined in TS 38.901 [6] | 41.55 Derived according to high loss model defined in TS 38.901 [6] |
| Pathloss model | InH [4] [5] | UMa [4] [5] | Rural [4] [5] With street width 20 m and building height 10 m [7] |

- Baseline coverage performance for FR2

Evaluated configurations on link budget evaluation

For FR2 coverage evaluation, it is considered to focus on a subset of evaluation configurations and technical features. This could facilitate the preparation of the baseline coverage performance. The proposed focused evaluation configuration is shown in Table 2. Link-level simulation is carried to get the required SNR based on the assumptions.

Table 2 evaluated configurations on link budget evaluation

| Parameters | Indoor hotspot | Urban Macro | Suburban |
| --- | --- | --- | --- |
| System configuration | NR 28 GHz TDD DL/UL: DDDSU | NR 28 GHz TDD DL/UL: DDDSU | NR 28 GHz TDD DL/UL: DDDSU |
| Channel state | NLOS | NLOS O-to-O NLOS O-to-I | NLOS O-to-O NLOS O-to-I |
| Antenna settings [2] | BS: (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1) UE: (M, N, P) = (2,2,2) | BS: (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2) UE: (M, N, P) = (2,2,2) | BS: (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2) UE: (M, N, P) = (2,2,2) |
| Analog beamforming | The BS and UE beams, and antenna elements are directed to the strongest cluster in the CDL-A channel model. |  |  |
| Channel model [2] | CDL-A 30 ns | CDL-A 100 ns | CDL-A 100 ns |
| SCS | 120 kHz | 120 kHz | 120 kHz |
| Waveform [2] | PDSCH: OFDM PUSCH: DFT-s-OFDM | PDSCH: OFDM PUSCH: DFT-s-OFDM | PDSCH: OFDM PUSCH: DFT-s-OFDM |
| Bandwidth [2] | PDSCH: 66 RB PUSCH: 30 RB | PDSCH: 66 RB PUSCH: 30 RB | PDSCH: 6 RB PUSCH: 4 RB |
| Data and DMRS configuration [2] | Data channel PDSCH: 12 OFDM symbols PUSCH: 14 OFDM symbols DMRS 1 DMRS symbol, no multiplexing with data PDSCH: Type I w/o additional DMRS PUSCH: Type I with 1 additional DMRS |  |  |
| MCS | DL: 2 UL: 4 | DL: 2 UL: 4, QPSK | DL: 0 UL: 0, QPSK |
| UE speed | 3 km/h | Indoor: 3 km/h Outdoor: 30 km/h | Indoor: 3 km/h Outdoor: 30 km/h |
| Target ISD and cell coverage | Coverage 50 m ISD 70 m | Coverage1: 267.67 m ISD1: 400 m Coverage2: 333.33 m ISD2: 500 m | Coverage 1154.67 m ISD 1732 m |

Required SNR and link budget based on LLS

Based on the assumptions listed in Table 2, the required SNR is simulated and provided in Table 3. The antenna array gain is included in the required SNR. The available pathloss and achieved distance are shown in Figure 1 and Figure 2, and the gap between the achieved and target are given in Table 4 and Table 5, respectively. Based on the results, we have the following observations:

Observation 1: An indoor target coverage of 50 m (ISD 70 m) can be achieved.

Observation 2: For a target O2O urban coverage of ISD=400 or 500m, the downlink is achievable, and the uplink is with 7.32 dB (59 m) or 11.11 dB (125 m) less than the target pathloss (distance), respectively.

Observation 3: For a target O2O suburban coverage of ISD=1732 m, both downlink and uplink are unachievable.

Observation 4: For both urban and suburban, target O2I coverage are unachievable.

Table 3 Required SNR (dB) based on LLS

| Scenario and channels | Indoor hotspot | Urban Macro | Suburban |
| --- | --- | --- | --- |
| PDSCH (O2O) | Indoor gNB serving indoor UE: -25.18 | -24.33 | -23.41 |
| PDSCH (O2I) |  | -24.57 | -23.55 |
| PUSCH (O2O) | Indoor gNB serving indoor UE: -23.14 | -24.53 | -26.84 |
| PUSCH (O2I) |  | -24.71 | -26.83 |

- O2O, Urban with target ISD 500 m

- O2I, Urban with target ISD 500 m

Figure 1 available pathloss for O2O and O2I

- O2O

- O2I

Figure 2 achieved distance for O2O and O2I

Table 4 Achieved pathloss and the gap to target

| Scenario | Available pathloss (dB) |  | Gap to target (dB) |  |
| --- | --- | --- | --- | --- |
|  | PDSCH | PUSCH | PDSCH | PUSCH |
| Indoor (ISD 70 m) | 126.36 | 123.33 | 4.10 | 1.07 |
| Urban (O2O, ISD 400 m) | 150.87 | 133.07 | 10.48 | -7.32 |
| Urban (O2O, ISD 500 m) | 150.87 | 133.07 | 6.69 | -11.11 |
| Suburban (O2O, ISD 1732 m) | 148.19 | 142.37 | -8.06 | -13.88 |
| Urban (O2I, ISD 400 m) | 108.00 | 90.14 | -32.39 | -50.25 |
| Urban (O2I, ISD 500 m) | 108.00 | 90.14 | -36.18 | -54.04 |
| Suburban (O2I, ISD 1732 m) | 107.09 | 101.12 | -49.16 | -55.13 |

Table 5 Achieved coverage and the gap to target

| Scenario | Achieved coverage (m) |  | Gap to target (m) |  |
| --- | --- | --- | --- | --- |
|  | PDSCH | PUSCH | PDSCH | PUSCH |
| Indoor (ISD 70 m) | 80.66 | 67.23 | 30.66 | 17.23 |
| Urban (O2O, ISD 400 m) | 593.53 | 207.91 | 326.87 | -58.75 |
| Urban (O2O, ISD 500 m) | 593.53 | 207.91 | 260.20 | -125.42 |
| Suburban (O2O, ISD 1732 m) | 859.26 | 607.30 | -295.41 | -547.37 |
| Urban (O2I, ISD 400 m) | 47.48 | 16.57 | -219.19 | -250.09 |
| Urban (O2I, ISD 500 m) | 47.48 | 16.57 | -285.86 | -316.76 |
| Suburban (O2I, ISD 1732 m) | 74.18 | 51.96 | -1080.48 | -1102.70 |

- Conclusions

The contribution provides our considerations on FR2 coverage evaluation, and the proposals are listed as following:

Proposal 1: The antenna array gain should be included in LLS based on CDL channel model for FR2.

Proposal 2: The analog beams used for LLS at the BS and UE should be further studied, and the beam directed to the strongest cluster in the CDL model can be a start point.

Proposal 3: Only one panel should be considered for the link budget in FR2.

Proposal 4: The remaining parameters involved in the link budget should be studied, and the values in Table 1 should be considered.

Based on the evaluation results, the observations are listed as following:

Observation 1: An indoor target coverage of 50 m (ISD 70 m) can be achieved.

Observation 2: For a target O2O urban coverage of ISD=400 or 500m, the downlink is achievable, and the uplink is with 7.32 dB (59 m) or 11.11 dB (125 m) less than the target pathloss (distance), respectively.

Observation 3: For a target O2O suburban coverage of ISD=1732 m, both downlink and uplink are unachievable.

Observation 4: For both urban and suburban, target O2I coverage are unachievable.

- References

RP-193240, “New SID on NR coverage enhancement”, RAN#86, Sitges, Spain, December 9 – 12, 2019.

3GPP TSG RAN WG1 Meeting #101-e, “Chairman’s notes of RAN1#101-e”, May, 2020.

R1-2005256, “Evaluation on the baseline performance for FR1”, Huawei, HiSilicon, RAN1#102e, Aug. 2020.

Report ITU-R M.2412-0, Guidelines for evaluation of radio interface technologies for IMT-2020, 10/2017.

RP-191527, “Link Budget Template - SRIT”, Part 3 of RP-191524 3GPP ITU-R Contribution - Final IMT-2020 Submission Overview, AT&T, Telecom Italia, RAN#84, Jun. 2019.

3GPP TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz (Release 16)”, V16.0.0 (2019-10).

Report ITU-R M.2435-1, Guidelines for evaluation of radio interface technologies for IMT-Advanced, 12/2009.