3GPP TSG RAN WG1 #102-e		R1-2005398

e-Meeting, August 17th – 28th, 2020

Source:	vivo

Title:	Considerations on Evaluation Assumptions for Coverage Enhancements

Agenda Item:	8.8.3

Document for:	Discussion and Decision

- Introduction

In RAN1#101e, a set of evaluation assumptions was agreed with some remaining issues to be further discussed. In this contribution, we provide our considerations on evaluation assumptions for coverage enhancements.

- Target for Coverage Enhancement

In the last meeting, it had been discussed that whether a target ISD is needed to identify the target available path loss. As shown in our evaluation results[1][2], it is very challenging to recover the coverage performance if the target ISD is too aggressive, e.g. 500m for urban, which implies that all the UL channels and some of the DL channels should be enhanced. In parallel with the evaluation of the coverage, the target for coverage enhancement also needs to be discussed before determining which physical channels require enhancement. Following candidate coverage recovery targets can be considered.

Option 1: To only improve the physical channel which is observed as coverage bottleneck, achieving a balanced coverage performance across different DL/UL channels.

Option 2: To match a given available path loss target for the specific deployment scenario, such as ISD=350m.

Option 3: Best effort, i.e. no clear target.

Proposal 1: The target for coverage enhancement also needs to be discussed before determine which physical channels require enhancement.

- Evaluation assumptions for physical channels

For PDSCH evaluation, better coverage can be expected if scheduled with lower MCS (e.g. MCS0) and more allocated RBs. However, the DL resources maybe shared by multiple UEs based on network scheduling, it is not realistic to always schedule lowest MCS for cell edge UEs, which may reduce the overall throughput. Alternatively, a relatively low MCS but not the lowest MCS, e.g. around 1/3 code rate, can be assumed for PDSCH evaluation.

Proposal 2: A relatively low MCS, but not the lowest MCS, e.g. around 1/3 code rate, can be assumed for PDSCH evaluation.

For PUSCH, lower MCS and more allocated RBs will not necessarily bring about better coverage since the UL transmission is power limited, more RBs means lower PSDs for each data RE. An optimal MCS and RB number combination maybe identified through evaluation. Nevertheless, best MCS and RB number combination is not practical for gNB scheduling. The best combination will be changed with different channel status, e.g. power delay profile and doppler spread, gNB may not get such detailed channel information in real deployment. Besides, network scheduling is impacted by multiple factors, such as network load, user priorities, traffic delay requirements. Coverage maybe one of the factors, but it is not necessarily the most important one. Therefore, a fixed MCS or a fixed number of allocated RBs for PUSCH can be assumed for PUSCH for simplicity.

Proposal 3: A fixed MCS/RB allocation is preferred for PUSCH evaluation.

PUCCH repetition has been supported in Rel-15. Although PUCCH repetition may provide higher reliability, it may not necessarily lead to ‘better coverage’. For PUCCH repetition, if PUCCH transmission is dropped due to collision with DL symbols or canceled by SFI, the PUCCH transmission is postponed. Furthermore, based on current mechanism, UE is not expected to receive a PDSCH with the same HARQ process before the transmission of HARQ-Ack of previous transmission is finished. In other words, if PUCCH repetition is postponed, the PDSCH reception is also postponed if retransmission is needed. In this case, PDSCH performance is degraded. Besides, PUSCH is one of the bottlenecks in the physical channels, and PUSCH repetition is considered as a baseline for coverage enhancements. Due to limited UL resources in TDD spectrum, PUSCH and PUCCH would be transmitted in the overlapping resources, if repetition for both channels are enabled. Thus, PUSCH is either dropped or multiplexed with PUCCH according to RRC configuration, which means the coverage of PUSCH would be degraded due to PUCCH repetition. Therefore, the number of repetitions for PUCCH evaluation should be limited in coverage evaluation.

Observation 1: Coverage of PDSCH and PUSCH may be degraded if PUCCH repetition is introduced. Hence PUCCH without repetition is preferred in evaluation.

PRACH format 0 and PRACH format B4 has been agreed as the candidate formats for evaluations in FR1. The duration of PRACH format 0 is nearly 1ms, which may be not deployed for some UL/DL configurations. For example, in the frame structure DDDSU (S: 10D:2G:2U) with 30kHz SCS, the PRACH format 0/1/2/3 can not be supported, while PRACH format B4 with 30kHz SCS is feasible for this frame structure. Hence, the PRACH format for coverage evaluation should also take the frame structure into consideration.

Observation 2: PRACH with long duration, e.g. format 0/1/2/3, cannot be deployed in some frame structure. The PRACH format for coverage evaluation should also take the frame structure into consideration.

- PRACH format B4 should be used for frame structure DDDSU with 30kHz SCS.

- Unicast BF gain and broadcast BF gain

The DL coverage is highly dependent on the gNB transmission EIRP. While the EIRP is usually determined based on transmission power and beamforming gain.

In Rel-15/Rel-16, for a UE specific downlink transmission, gNB may select beam with better quality to serve a UE through beam management procedure or taking advantage of channel reciprocity, hence high BF gain can be assumed for transmissions or receptions other than DL broadcast channels.

For DL broadcast channels, e.g. SSBs and PDCCH in type 0-2 CSSs, typically these channels are served by fixed and wider beams, the beamforming gain would be lower compared with UE specific channels. The difference is not considered in ITU link budget template. To reflect the coverage of the broadcast channels more accurately, the BF gain difference between broadcast channel and unicast channel should be reflected in link budget template.

We evaluated the BF gain difference between unicast BF and broadcast BF through system level simulation, and the difference is reflected by the RSRP difference measured at UEs in a serving cell. The number of broadcast beam and the beam width is selected to make sure every position in a cell is covered by a broadcast beam within 3dB beam width. For broadcast beams, there are 4 horizontal beams in FR1, and 64 beams cover 16 horizontal directions and 4 vertical directions in FR2. For unicast beams, 8 DFT based horizontal beams are modeled for FR1, and 128 DFT based beams composing 32 horizon directions and 4 vertical directions are modeled for FR2. The detailed evaluation assumptions for broadcast and unicast beams are given in Appendix.

(a) FR1 2.6GHz                                                             (b) FR1 4GHz

(c) FR2 Urban                                                               (d) FR2 Indoor

Figure 1. CDF of RSRP with broadcast BF and unicast BF

As shown Figure 1, the RSRP of with unicast BF is obviously higher than that of broadcast BF. In FR1, the BF gain of unicast BF is about 8dB higher than broadcast BF. In FR2, the difference is about 8dB and 5dB for urban and Indoor, respectively.

Proposal 4: To reflect the coverage of the broadcast channels more accurately, the BF gain difference between broadcast channel and unicast channel should be considered in link budget template.

- In FR1, the broadcast BF gain is about 8dB lower than unicast BF gain;

- In FR2, the broadcast BF gain is about 8dB and 5dB lower than unicast BF gain, in urban and indoor scenarios, respectively.

- Conclusion

In this contribution, we discussed the evaluation assumptions and methodologies for coverage enhancements. Based on the discussion in previous sections, and we have the following proposals:

Proposal 1: The target for coverage enhancement also needs to be discussed before determine which physical channels require enhancement.

Proposal 2: A relatively low MCS, but not the lowest MCS, e.g. around 1/3 code rate, can be assumed for PDSCH evaluation.

Proposal 3: A fixed MCS/RB allocation is preferred for PUSCH evaluation.

Observation 1: Coverage of PDSCH and PUSCH may be degraded if PUCCH repetition is introduced. Hence PUCCH without repetition is preferred in evaluation.

Observation 2: PRACH with long duration, e.g. format 0/1/2/3, cannot be deployed in some frame structure. The PRACH format for coverage evaluation should also take the frame structure into consideration.

- PRACH format B4 should be used for frame structure DDDSU with 30kHz SCS.

Proposal 4: To reflect the coverage of the broadcast channels more accurately, the BF gain difference between broadcast channel and unicast channel should be considered in link budget template.

- In FR1, the broadcast BF gain is about 8dB lower than unicast BF gain;

- In FR2, the broadcast BF gain is about 8dB and 5dB lower than unicast BF gain, in urban and indoor scenarios, respectively.

- References

- R1-2005393, ‘Evaluation on NR coverage performance for FR1’, vivo.

- R1-2005394, ‘Evaluation on NR coverage performance for FR2’, vivo.

- Appendix

Table 1  Evaluation assumptions for UE RSRP for broadcast BF and unicast BF

| Attributes/ parameters | Assumptions |  |  |
| --- | --- | --- | --- |
| Scenario/ Frequency | 4GHz and 2.6GHz | 28GHz Urban | 28GHz indoor |
| Tx power | 41 dBm for 10 MHz bandwidth | 37 dBm for 40 MHz bandwidth e.i.r.p. should not exceed 73 dBm | 20 dBm for 40 MHz bandwidth e.i.r.p. should not exceed 58 dBm |
| Penetration loss | 20% high loss, 80% low loss | 20% high loss, 80% low loss | N/A |
| ISD | 200 m | 200 m | 20m |
| gNB antenna config | (M,N,P,Mg,Ng)= (12,8,2,1,1) | (M,N,P,Mg,Ng;Mp,Np)= (4,8,2,2,2;1,1) | (M,N,P,Mg,Ng;Mp,Np)= (8,8,2,1,1;1,1) |
| UE antenna config | (M,N,P,Mg,Ng)= (1,2,2,1,1) (dH, dV) = (0.5, N/A) λ | (M,N,P,Mg,Ng)= (2,2,2,1,1) (dH, dV) = (0.5, 0.5) λ | (M,N,P,Mg,Ng)= (2,2,2,1,1) (dH, dV) = (0.5, 0.5) λ |
| BS ant pattern | Table A.2.1-4 in TR 38.802 | Table A.2.1-6 in TR 38.802 | 3-sector BS antenna pattern in Table A.2.1-7 in TR 38.802 |
| UE ant pattern | Omni-direction | Table A.2.1-8 in TR 38.802 | Table A.2.1-8 in TR 38.802 |
| SSB beam directions | Azimuth angle φ(i) = [-3 * pi/12, -pi/12, pi/12, 3 * pi/12] Zenith angle θ(j) = [pi/2] | Azimuth angle φ(i) = [-15*pi/48, -13*pi/48, -11*pi/48, -9*pi/48, -7*pi/48, -5*pi/48, -3*pi/48, -1*pi/48, 1*pi/48, 3*pi/48,5*pi/48, 7*pi/48, 9*pi/48, 11*pi/48,13*pi/48, 15*pi/48,] Zenith angle θ(j) = [9*pi/16,11*pi/16,13*pi/16,15*pi/16] | Azimuth angle φ(i) = [-15*pi/48, -13*pi/48, -11*pi/48, -9*pi/48, -7*pi/48, -5*pi/48, -3*pi/48, -1*pi/48, 1*pi/48, 3*pi/48,5*pi/48, 7*pi/48, 9*pi/48, 11*pi/48,13*pi/48, 15*pi/48,] Zenith angle θ(j) = [9*pi/16,11*pi/16,13*pi/16,15*pi/16] |
| Unicast beam directions | Azimuth angle φ(i) = [-7*pi/24, -5*pi/24, -3*pi/24, -1*pi/24,1*pi/24, 3*pi/24, 5*pi/24, 7*pi/24]; Zenith angle θ(j) = [pi/2]; | Azimuth angle φ(i) = [-31*pi/96, -29*pi/96, -27*pi/96, -25*pi/96, -23*pi/96, -21*pi/96, -19*pi/48, -17*pi/48, -15*pi/96, -13*pi/96, -11*pi/96, -9*pi/96, -7*pi/96, -5*pi/96, -3*pi/48, -1*pi/48,1*pi/96, 3*pi/96,5*pi/96, 7*pi/96, 9*pi/96, 11*pi/96,13*pi/96, 15*pi/96, 17*pi/96, 19*pi/96,21*pi/96, 23*pi/96, 25*pi/96, 27*pi/96,29*pi/96, 31*pi/96] Zenith angle θ(j) = [9*pi/16,11*pi/16,13*pi/16,15*pi/16] | Azimuth angle φ(i) = [-31*pi/96, -29*pi/96, -27*pi/96, -25*pi/96, -23*pi/96, -21*pi/96, -19*pi/48, -17*pi/48, -15*pi/96, -13*pi/96, -11*pi/96, -9*pi/96, -7*pi/96, -5*pi/96, -3*pi/48, -1*pi/48,1*pi/96, 3*pi/96,5*pi/96, 7*pi/96, 9*pi/96, 11*pi/96,13*pi/96, 15*pi/96, 17*pi/96, 19*pi/96,21*pi/96, 23*pi/96, 25*pi/96, 27*pi/96,29*pi/96, 31*pi/96] Zenith angle θj = [9*pi/16,11*pi/16,13*pi/16,15*pi/16] |
| UE beam directions | N/A | Azimuth angle φ(i) = [-1*pi/4, 1*pi/4] Zenith angle θ(j) = [1*pi/4, 3*pi/4] | Azimuth angle φ(i) = [-1*pi/4, 1*pi/4] Zenith angle θ(j) = [1*pi/4, 3*pi/4] |
| UE distribution | Uniformly distributed: 80% indoor, 20% outdoor (in-car) | Uniformly distributed: 80% indoor, 20% outdoor (in-car) | Uniformly distributed indoor |
| UE velocity | Outdoor users (in-car): 30 km/h Indoor users: 3km/h | Outdoor users (in-car): 30 km/h Indoor users: 3km/h | Indoor users: 3km/h |