3GPP TSG RAN WG1 Meeting #102-e                                                                      R1-2005443

e-Meeting, August 17th – 28th May 25th, 2020

Source:              ZTE

Title:                   Enhancement on FR2 MPE mitigation

Agenda Item:     5.2

Document for:   Discussion and Decision

- Introduction

In this contribution, we discuss the detailed solutions on this FR2 MPE mitigation (about the remaining issues raised in [1]), including the new MAC-CE format for MPE event related assistant information.

- Enhancement(s) on FR2 MPE mitigation

  - MAC-CE format for MPE event related assistant information

The P-MPR value is agreed to be reported to the network via a MAC-CE, and, as a part of PHR calculation, the P-MPR value should be reported along with the existing PHR reporting (e,g., Rel-15 single entry PHR MAC-CE as shown in Figure 1). Compared with introducing a new MAC-CE, the spec impact, e.g., mechanisms of configuring, triggering and reporting, can be saved significantly. Specifically, one more P-MPR field can be introduced into the PHR MAC-CE. In order to be distinguished from a normal PHR, one flag field can be used for indicating whether the P-MPR field is included in the MAC-CE.

Figure 1 Example of Rel-15 single entry PHR MAC-CE

Observation-1: The recent mechanism of PHR reporting can be well compatible with P-MPR reporting, considering that the P-MPR is a specific value applied to PHR calculation, like Pc,max that is also reported along with PHR.

PHR reporting with the additional P-MPR value is related to PUSCH transmission (PHR Type-1) of PCell. Further enhancement on other PHR about SRS transmission seems unnecessary, considering that MPE impact is mainly related to UL-data transmission rather than CSI acquisition based on SRS transmission.

- The PHR MAC-CE for PUSCH includes single-entry PHR MAC-CE and multiple-entry PHR MAC-CE, which are related to non-CA/DC and CA/DC cases, respectively. Considering that MPE impact is UE specific and decoupled with the configuration of CA/DC, the P-MPR reporting can be applied to both single-entry PHR MAC-CE and multiple-entry PHR MAC-CE.

Proposal-1: Enhance PHR MAC-CE format(s) to carry P-MPR value for PUSCH-PHR result in SpCell.

- The enhanced PHR MAC-CE format(s) is supported in both single-entry PHR MAC-CE and multiple entry-PHR MAC-CE.

For instance, one example for the enhanced MAC-CE format for P-MPR reporting is provided in Figure 2 as for single-entry PHR MAC-CE, where the enhanced format can be enabled by a RRC command if the UE can support the P-MPR reporting through MAC-CE, and 5 bits are assumed to be used for reporting P-MPR values. The flag F is to indicate whether the P-MPR (PCell) field exists or not.

Figure 2 Enhancement on single-entry PHR MAC-CE with P-MPR reporting

- Conclusion

In this contribution, we provide the following observation and proposals for FR2 MPE mitigation.

Observation-1: The recent mechanism of PHR reporting can be well compatible with P-MPR reporting, considering that the P-MPR is a specific value applied to PHR calculation, like Pc,max that is also reported along with PHR.

Proposal-1: Enhance PHR MAC-CE format(s) to carry P-MPR value for PUSCH-PHR result in SpCell.

- The enhanced PHR MAC-CE format(s) is supported in both single-entry PHR MAC-CE and multiple-entry PHR MAC-CE.

- References

[1] R4-2008479, WF on MPE solutions, OPPO