3GPP TSG RAN WG1 Meeting #102-e                                                               	   R1- 2005716

e-Meeting, August 17th – 28th, 2020

Source:	CATT

Title:	Discussion on coverage recovery

Agenda Item:	8.6.3

Document for:	Discussion and Decision

Introduction

In RAN#86, a study item was approved as a new Rel-17 SI on support of reduced capability NR devices [1]. The SID was updated in [2] in RAN#88e. One of the objectives is to study functionality that will enable the performance degradation of such complexity reduction to be mitigated or limited, including:

- Coverage recovery to compensate for potential coverage reduction due to the device complexity reduction.

  - Note: For FR1, coverage analysis for wearables can include consideration of potential reduced antenna efficiency due to device size limitations as part of the antenna gains. The extent of additional recovery of coverage loss due to reduced antenna efficiency is to be limited to 3 dB

At last meeting, we have the following agreements related to the coverage recovery.

Agreements:

- For FR1, study at least 20MHz maximum UE bandwidth at least for initial access

  - Other bandwidths FFS

- For FR2, study 50MHz and 100 MHz maximum UE bandwidth at least for initial access

  - Other bandwidths FFS

Furthermore, according to the updated SID [2], the lowest bandwidth capability considered should be no less than an LTE Category 1bis modem thus maximum UE bandwidth smaller than 20MHz for FR1 was precluded.

Agreements:

- For FR1, study two antenna configurations for RedCap UEs, namely 1Rx/1Tx and 2Rx/1Tx.

- For FR2, study two antenna configurations for RedCap UEs, namely 1Rx/1Tx and 2Rx/1Tx.

Agreements:

If/when link-level coverage evaluations outside the CE SI are needed,

   The CE SI link-level simulation assumptions can be used as a starting point.

   For calibration purposes, the following settings can be used:

| Parameters | FR1 values | FR2 values |
| --- | --- | --- |
| Scenario and frequency | Urban: 2.6 GHz (TDD) (primary choice) 4 GHz (TDD) (secondary choice) Rural: 700 MHz (FDD) | Indoor: 28 GHz (TDD) |
| Frame structure for TDD | For 2.6 GHz: DDDDDDDSUU (S: 6D:4G:4U) For 4 GHz: DDDSUDDSUU (S: 10D:2G:2U) | DDDSU (S: 10D:2G:2U) |
| Channel model | TDL-C | TDL-A |
| UE velocity | 3 km/h | 3 km/h |

In this contribution, we provide some evaluations to show the performance degradation of UE complexity reduction, and provide preliminary thoughts on coverage recovery for reduced capability NR devices.

Discussion

The following potential UE complexity reduction features are identified [1].

- Reduced number of UE RX/TX antennas

- UE Bandwidth reduction

Note: Rel-15 SSB bandwidth should be reused and L1 changes minimized

- Half-Duplex-FDD

- Relaxed UE processing time

- Relaxed UE processing capability

Some of the UE complexity reduction techniques may negatively impact coverage. To be more specific, reduced number of UE RX antennas may degrade the DL coverage and UE bandwidth reduction may degrade DL/UL coverage. Depending on the UE complexity reduction technique(s) to be adopted, the required coverage recovery can be different.

The performance of SSB, PDCCH and PDSCH are evaluated to quantify the impact on coverage due to reduced number of UE Rx antennas and UE bandwidth. More detailed assumptions can be found in Appendix.

The link-level simulation results for SSB and PDCCH at 2.6G Hz carrier frequency is provided in Table 1.

Table 1: LLS results for SSB and PDCCH

|  | SSB @BLER=1% |  | PDCCH @BLER=1% |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Req. SNR [dB] | Loss [dB] | AL1 |  | AL2 |  | AL4 |  | AL8 |  | AL16 |  |
|  |  |  | Req. SNR [dB] | Loss [dB] | Req. SNR [dB] | Loss [dB] | Req. SNR [dB] | Loss [dB] | Req. SNR [dB] | Loss [dB] | Req. SNR [dB] | Loss [dB] |
| 4Rx | -11.01 | - | 2.2 | - | -1.65 | - | -5.82 | - | -8.30 | - | -10.15 | - |
| 2Rx | -8.57 | 2.44 | 8.68 | 6.48 | 3.87 | 5.52 | -0.50 | 5.32 | -4.83 | 3.47 | -7.56 | 2.59 |
| 1Rx | -5.50 | 5.51 | 16.03 | 13.83 | 10.47 | 12.12 | 3.76 | 9.58 | -1.02 | 7.28 | -4.06 | 6.09 |

Observation 1: For SSB, there is 2.44dB performance loss from 4Rx to 2Rx and 5.51dB performance loss from 4Rx to 1Rx.

Observation 2: For PDCCH, there is 2.59dB performance loss from 4Rx to 2Rx and 6.09dB performance loss from 4Rx to 1Rx.

Observation 3: For a given number of UE Rx antennas, the performance loss for PDCCH is larger for smaller AL.

For PDSCH, we assume BWP size of 100MHz for legacy UE and 20MHz for RedCap UE and the target data rate is 10Mbps. The LLS results for PDSCH are shown in Table 2.

Table 2: LLS results for PDSCH

| PDSCH @BLER=10% | 100MHz BW |  | 20MHz BW |  | Loss from 100MHz to 20MHz [dB] |
| --- | --- | --- | --- | --- | --- |
|  | Req. SNR [dB] | Loss compared with 4Rx [dB] | Req. SNR [dB] | Loss [dB] |  |
| 4Rx | -7.93 | - | -1.17 | - | 6.76 |
| 2Rx | -4.90 | 3.03 | 2.68 | 3.85 | 7.58 |
| 1Rx | -1.30 | 6.63 | 8.66 | 9.83 | 9.96 |

Observation 4: For PDSCH, the bandwidth reduction would increase MCS for a given target data rate leading to degraded performance. The performance loss is larger for a smaller number of UE Rx antennas.

The criteria to determine target of coverage recovery needs to be discussed.

- Alt 1: To compensate the coverage loss for each channel with coverage loss due to UE complexity reduction technique(s)

- Alt 2: To compensate the coverage loss for the bottleneck channel(s) to achieve the same overall MCL of the cell without UE complexity reduction technique(s )

Proposal: clarify the coverage recovery target for RedCap UEs.

Conclusion

This contribution provides our LLS results for SSB, PDCCH and PDSCH considering reduced number of UE Rx antennas and reduced bandwidth with the following observations and proposal:

Observation 1: For SSB, there is 2.44dB performance loss from 4Rx to 2Rx and 5.51dB performance loss from 4Rx to 1Rx.

Observation 2: For PDCCH, there is 2.59dB performance loss from 4Rx to 2Rx and 6.09dB performance loss from 4Rx to 1Rx.

Observation 3: For a given number of UE Rx antennas, the performance loss for PDCCH is larger for smaller AL.

Observation 4: For PDSCH, the bandwidth reduction would increase MCS for a given target data rate leading to degraded performance. The performance loss is larger for a smaller number of UE Rx antennas.

Proposal: clarify the coverage recovery target for RedCap UEs.

References

- RP-193238, New SID on support of reduced capability NR devices, Ericsson, RAN#86

- RP-201386, Revised SID on Study on support of reduced capability NR devices, Ericsson, RAN#88e

Appendix

Table 3: simulation assumptions for PDCCH

| Parameters | Values |
| --- | --- |
| Scenario and frequency | 2.6G Hz |
| Subcarrier Space | 30KHz |
| UE velocity | 3km/h |
| System bandwidth | 100M, 20M |
| PDSCH duration | 12 OS |
| PDCCH duration | 2 OS |
| PDCCH AL | 1，2，4，8，16 |
| Tx-Rx configuration | 1Tx - 1Rx/2Rx/4Rx |
| Channel Model | TDL-C, 300ns |
| Channel estimation | RCE |

Table 4: simulation assumptions for PDSCH

| Parameters | Values |
| --- | --- |
| Scenario and frequency | 2.6G Hz |
| Subcarrier Space | 30KHz |
| UE velocity | 3km/h |
| System bandwidth | 100M, 20M |
| Data rate | 10Mbps |
| Code rate | CR=602/1024 for 20M BW CR=120/1024 for 100M BW |
| DMRS duration | 1OS for 3kmph |
| PDSCH duration | 12 OS |
| PDCCH duration | 2 OS |
| Tx-Rx configuration | 1Tx-1Rx/2Rx/4Rx |
| Channel Model | TDL-C, 300ns |
| Channel estimation | RCE |

Table 5: simulation assumptions for SSB

| Parameters | Values |
| --- | --- |
| Scenario and frequency | 2.6G Hz |
| Subcarrier Space | 30KHz |
| UE velocity | 3km/h |
| Periodicity | 20ms |
| Number of SSBs | 4 |
| Performance metric | Combination of 4 SSBs in 80ms. |
| Tx-Rx configuration | 1Tx – 1Rx/2Rx/4Rx |
| Channel Model | TDL-C, 300ns |
| Channel estimation | RCE |