3GPP TSG RAN WG1 Meeting #102-e	R1-2005808

E-meeting, August 17th – 28th, 2020

Agenda Item:	7.2.2.1.3

Source:	Huawei, HiSilicon

Title:	Maintenance on UL signals and channels

Document for:	Discussion and Decision

Introduction

In this contribution, we propose to fix the remaining issues of uplink signals and channels for Rel-16 
NR-U work item.

PUSCH scheduled by RAR grant/DCI 0_0 with TC-RNTI

In last meeting, the following agreement was achieved:

Agreement:

- As per prior agreement, initial UL BWP is 20 MHz

  - FFS: The case of SUL in licensed band

- For PUSCH scheduled by a RAR UL Grant (e.g., Msg3) or by DCI 0_0 addressed to TC-RNTI (Msg3 re-transmission) when UL Resource Allocation Type 2 is configured, the PUSCH is transmitted as follows:

  - PUSCH is transmitted in the same UL RB set of the active UL BWP as PRACH (Msg1)

- FFS: The case where PRACH is configured in more than one RB set

The initial BWP is indepentently configured for SUL and normal UL(NUL), since there is no LBT issue in licensed band, it is not necessary to restrict the initial UL BWP for SUL in licensed band as 20MHz.

Observation 1: It is not necessary to restrict the initial UL BWP for SUL in licensed band as 20MHz, i.e. no correction is needed.

For PUSCH scheduled by a RAR UL Grant or by DCI 0_0 addressed to TC-RNTI when UL Resource Allocation Type 2 is configured, we have agreed that the PUSCH should be transmited in the same UL RB set where the PRACH is transmitted. Even if the PRACH is configured in more than one RB set, UE will select one occasion to transmit the preamble, which is obviously confined within one RB set, and this has already been reflected clearly in the TS38.213 V16.2.0.

If useInterlace-PUCCH-PUSCH is provided by BWP-UplinkCommon or BWP-UplinkDedicated, the frequency domain resource allocation is by uplink resource allocation type 2 [6, TS 38.214]. A UE processes the frequency domain resource assignment field as follows

-	truncate the frequency domain resource assignment field to the X=6 LSBs if μ=0, or to the X=5 LSBs if μ=1

-	for interlace allocation of a PUSCH transmission, interpret the X MSBs of the truncated frequency domain resource assignment field for the active UL BWP as for the X MSBs of the frequency domain resource assignment field in DCI format 0_0 [6, TS 38.214]

-	for RB set allocation of a PUSCH transmission, the RB set of the active UL BWP is the RB set of the PRACH transmission associated with the RAR UL grant

Observation 2: No correction is needed for the case where PRACH is configured in more than one RB set.

Conclusions

Based on the discussion, we observe that no corrections are needed for the UL-related issues discussed in this document.

Observation 1: It is not necessary to restrict the initial UL BWP for SUL in licensed band as 20MHz, i.e. no correction is needed.

Observation 2: No correction is needed for the case where PRACH is configured in more than one RB set.

References

- [1] 3GPP TS38.214

- [2] 3GPP TS38.213