3GPP TSG RAN WG1 #102-E		R1-2006059

Aug 17th – Aug 28th, 2020

Source:	OPPO

Title:	Enhancement for CSI Feedback

Agenda Item:	8.3.1.2

Document for:	Discussion and Decision

Introduction

In RAN #88e, the WID on enhanced IoT and URLLC support for NR was agreed with following RAN1 objectives:

- Study, identify and specify if needed, required Physical Layer feedback enhancements for meeting URLLC requirements covering

    - UE feedback enhancements for HARQ-ACK [RAN1]

    - CSI feedback enhancements to allow for more accurate MCS selection [RAN1]

Note: DMRS-based CSI feedback is not in scope of this WI

In this contribution, we show our views on the enhancements on Rel-17 CSI feedback enhancement.

CSI feedback procedure

In Rel-15/16, there are period CSI report and A-period CSI report. A-period CSI report is triggered by DCI and the context reported by CSI can be semi-static configured. And the CSI report can be multiplexed with HARQ-ACK in one PUCCH resource or in independent PUCCH resource from HARQ-ACK. For any scheme, the processing time for CSI feedback is longer than the processing time for HARQ-ACK feedback for PDSCH. So, if we want to feedback CSI fast, even together with HARQ-ACK report. The processing time for CSI feedback should be shorten.

Proposal 1: The processing time for CSI feedback should be shorten to support fast CSI report.

A direct way to shorten the processing time for CSI feedback is to reduce report context or simplify the measurement procedure.

For reducing report context, partial of CSI feedback could be considered. To be specific, only CQI is reported. If RI and PMI keep the same, then CQI measurement time will decreased significantly due to UE could not iterate all combinations of {RI, PMI, CQI}.

For simplifying measurement procedure, data reception status can be used as one type of CSI information. And then intermediate output during data decoding, e.g. likelihood ratio value, can be reported. There is no additional measurement procedure and no measurement delay.

Proposal 2: Reducing report context and simplifying the measurement procedure can be considered to reduce processing time for CSI feedback.

To improve UCI transmission efficiency, CSI together with HARQ-ACK reporting can be considered. Meanwhile, to satisfy UCI, especially for HARQ-ACK, reliability requirement, compressed CSI report needs to study:

- Option 1: Absolute value within small CSI range. The small CSI range can be configured by RRC.

- Option 2: Difference value. MCS in the latest PDSCH can be reference point.

Proposal 3: CSI together with HARQ-ACK reporting can be considered and compressed CSI report needs to study.

Conclusions

In this contribution, we show our views on CSI feedback enhancement with following observations and proposals:

Proposal 1: The processing time for CSI feedback should be shorten to support fast CSI report.

Proposal 2: Reducing report context and simplifying the measurement procedure can be considered to reduce processing time for CSI feedback.

Proposal 3: CSI together with HARQ-ACK reporting can be considered and compressed CSI report needs to study.

References

- RP-201310 Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR