3GPP TSG RAN WG1 #102-e	R1-2006161

# e-Meeting, Aug. 17th – Aug. 28th, 2020

Agenda Item:		8.8.1.2

Source:				Samsung

Title:					Baseline coverage performance using LLS for FR2

Document for:	Discussion

Introduction

As listed in the study item objective [1], link-level simulations are to be used for the identification of baseline coverage performance for both DL and UL for the various scenarios and services. This contribution mainly focuses on FR2 preliminary results based on the simulation assumptions agreed in RAN1#101-e meeting. Simulation results for FR1 are given in [2].

Evaluation methodology

As discussed in our contribution on baseline performance for FR1 [2], available path loss in link budget was chosen as the metric to evaluate the coverage for various scenarios in Rel-15 IMT 2020 self-evaluation. In order to take into account the different scenarios, the link budget methodology used in IMT 2020 self-evaluation for 3GPP submission is also preferred for FR2 evaluation. The detailed evaluation procedure for available path loss and corresponding maximum range is provided in Table A-1 in the Appendix. The calculated maximum range would provide evidence to identify the coverage bottleneck of transmission channels in both UL and DL.

Proposal 1. The available path loss should be considered in the link budget calculation for coverage performance in FR2 to take into account the different scenarios and channel.

Simulation assumptions

At the previous RAN1#101-e meeting, the simulation parameters are assumed with the several scenarios, services and channels. In this contribution, we considered the various scenarios, such as urban, suburban, and indoor scenarios. Regarding the channels, we considered the data and control channels for uplink and downlink, such as PUSCH, PUCCH, PDSCH, and PDCCH. In detail, the simulation parameters for FR2 are summarized in the Appendix.

Preliminary results

The following link budget results using the link budget template in Table A-1 in the Appendix have been performed based on the simulation parameter values in Table A-2/3/4/5 in the Appendix. In addition, the required SNR in the link budget template is calculated based on the link-level simulation results at performance target for each channel in Table 2. The baseline performance in dB and coverage distance in meter are defined as available path-loss (29) and maximum range (30) in link budget template in Table A-1, respectively. ISD in meter is evaluated based on the coverage distance in meter. The target performance has to be chosen at the expected ISD for each scenario and the corresponding performance gap represented as

Gap = target path loss – available path loss in baseline performance.

The following sections show the detailed results of the evaluation. We observed that UL channels have inferior performance relative to the DL channels in all considered scenarios. We also observed that when using the same ISD values used in FR1 evaluation for urban scenario, 500m ISD, to compute the target path loss from the path-loss model in [Section 7.4.1, 4], there are large gaps, in the order of tens of dBs, between the baseline performance and the target performance for all considered channels and scenario, however the relative performance of the channels show that UL channels perform worse than the DL channels. When using a smaller ISD value obviously the performance gap reduces. As examples, in the following sections we use 150m for urban and 400m for suburban. For indoor scenario, with ISD of 20m, with the current simulation assumptions, all channels perform above targets of tens of dBs. Further discussion is needed on the evaluation assumptions and target performance for FR2.

Based on the evaluation results so far, we have the following proposal

Observation 1. Considering the relative performance of the channels, it is observed that UL channels perform worse than the DL channels in all considered scenarios.

Observation 2. Preliminary results suggest that performance enhancements are needed for both UL and DL channels.

Proposal 2. Further discussion is needed on the evaluation assumptions for FR2 and to establish target performance.

Urban scenario at 28GHz

Baseline performance is provided for urban scenario with 28GHz carrier frequency. In addition, we consider NLOS O-to-I and NLOS O-to-O channel. For urban scenario, the target ISD is 150m and thus the target performance for ISD=150m is calculated based on the path loss model in [Section 7.4.1, 4] as 118.2dB in terms of path loss.

Table 1. Results for urban scenario with NLOS O-to-I channel at FR2

| Carrier | Service & data rate | Frame structure | Pathloss model | # of PRBs | Required SNR (dB) | Maximum path loss (dB) | Target Performance (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ISD=150m | Gap |
| 28GHz | PUSCH for eMBB | DDDSU | NLOS (O-to-I) | 30 | 3.2 | 96.29 | 118.2 | 21.91 |
|  | VoIP | DDDSU |  | 4 | -7.4 | 115.64 |  | 2.56 |
|  | PUCCH format 1 for UCI 2bits | - |  | 1 | -7.7 | 115.31 |  | 2.89 |
|  | PUCCH format 3 for UCI 11bits | - |  | 1 | -3.9 | 111.51 |  | 6.69 |
|  | PUCCH format 3 for UCI 22bits | - |  | 1 | -1.23 | 108.84 |  | 9.36 |
|  | PDSCH for eMBB | DDDSU |  | 50 | 3.05 | 120.46 |  | -2.26 |
|  | PDCCH | - |  | 48 | -7.88 | 127.99 |  | -9.79 |

As observed in Table 1, there is 21.91 dB gap from target ISD=150m for PUSCH with 5Mbps eMBB service in case of TDD configuration with DDDSU. For VoIP service, there is 2.56 dB gap for 0.02 target residual BLER. For PUCCH, there are 2.89, 6.69, and 9.36 dB gap for format 1 with 2 UCI bits, format 3 with 11 UCI bits, and format 3 with 22 UCI bits, respectively. For downlink channel, there are -2.26 and -9.79 dB gap for PDSCH with eMBB service and PDCCH, respectively. Based on the results, it is observed that for urban scenario with O-to-I channel, PUSCH enhancement in eMBB service and VoIP service and PUCCH enhancement are needed with ISD=150m.

Observation 3. For urban scenario with O-to-I channel for FR2, PUSCH enhancement in both eMBB and VoIP services is needed with ISD=150m.

Observation 4. For urban scenario with O-to-I channel for FR2, PUCCH enhancement is needed with ISD=150m.

Table 2. Results for urban scenario with NLOS O-to-O channel at FR2

| Carrier | Service & data rate | Frame structure | Pathloss model | # of PRBs | Required SNR (dB) | Maximum path loss (dB) | Target Performance (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ISD=150m | Gap |
| 28GHz | PUSCH for eMBB | DDDSU | NLOS (O-to-O) | 30 | 3.2 | 117.21 | 118.2 | 0.99 |
|  | VoIP | DDDSU |  | 4 | -8.75 | 137.91 |  | -19.71 |
|  | PUCCH format 1 for UCI 2bits | - |  | 1 | -7.8 | 136.19 |  | -17.99 |
|  | PUCCH format 3 for UCI 11bits | - |  | 1 | -3.9 | 132.29 |  | -14.09 |
|  | PUCCH format 3 for UCI 22bits | - |  | 1 | -1.1 | 129.49 |  | -11.29 |
|  | PDSCH for eMBB | DDDSU |  | 50 | 3.05 | 141.38 |  | -23.18 |
|  | PDCCH | - |  | 48 | -7.88 | 148.77 |  | -30.57 |

As observed in Table 2, there is 0.99 dB gap from target ISD=150m for PUSCH with 5Mbps eMBB service in case of TDD configuration with DDDSU. For VoIP service, there is -19.71 dB gap for 0.02 target residual BLER. For PUCCH, there are -17.99, -14.09, and -11.29 dB gap for format 1 with 2 UCI bits, format 3 with 11 UCI bits, and format 3 with 22 UCI bits, respectively. For downlink channel, there are -23.18 and -30.57 dB gap for PDSCH with eMBB service and PDCCH, respectively. Based on the results, it is observed that for urban scenario with O-to-O channel, PUSCH enhancement in eMBB service and PUCCH enhancement are needed with ISD=150m.

Observation 5. For urban scenario with O-to-O channel for FR2, PUSCH enhancement in eMBB is needed with ISD=150m.

Suburban scenario at 28GHz

Baseline performance is provided for suburban scenario with 28 GHz carrier frequency. In addition, we consider NLOS O-to-I and NLOS O-to-O (UE speed: 30/120 km/h) channel. For suburban scenario, the target ISD is 400m and thus the target performance for ISD=400m is calculated based on the path loss model in [Section 7.4.1, 4] as 134.85 dB in terms of path loss.

Table 3. Results for suburban scenario with NLOS O-to-I channel at FR2

| Carrier | Service & data rate | Frame structure | Pathloss model | # of PRBs | Required SNR (dB) | Maximum path loss (dB) | Target Performance (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ISD=400m | Gap |
| 28GHz | PUSCH for eMBB | DDDSU | NLOS (O-to-I) | 1 | 0.1 | 114.16 | 134.85 | 20.69 |
|  | VoIP | DDDSU |  | 4 | -7.4 | 115.64 |  | 19.21 |
|  | PUCCH format 1 for UCI 2bits | - |  | 1 | -7.7 | 115.31 |  | 19.54 |
|  | PUCCH format 3 for UCI 11bits | - |  | 1 | -3.9 | 111.51 |  | 23.34 |
|  | PUCCH format 3 for UCI 22bits | - |  | 1 | -1.23 | 108.84 |  | 26.01 |
|  | PDSCH for eMBB | DDDSU |  | 2 | 4.5 | 132.99 |  | 1.86 |
|  | PDCCH | - |  | 48 | -7.88 | 127.99 |  | 6.86 |

As observed in Table 3, there is 20.69 dB gap from target ISD=400m for PUSCH with 50kbps eMBB service in case of TDD configuration with DDDSU. For VoIP service, there is 19.21 dB gap for 0.02 target residual BLER. For PUCCH, there are 19.54, 23.34, and 26.01 dB gap for format 1 with 2 UCI bits, format 3 with 11 UCI bits, and format 3 with 22 UCI bits, respectively. For downlink channel, there are 1.86 and 6.86 dB gap for PDSCH with eMBB service and PDCCH, respectively. Based on the results, it is observed that for suburban scenario with O-to-I channel, PUSCH, PUCCH, PDSCH, and PDCCH enhancement are needed with ISD=150m.

Observation 6. For suburban scenario with O-to-I channel for FR2, PUSCH, PUCCH, PDSCH, and PDCCH enhancement are needed with ISD=400m.

Table 4. Results for suburban scenario with NLOS O-to-O (30km/h) channel at FR2

| Carrier | Service & data rate | Frame structure | Pathloss model | # of PRBs | Required SNR (dB) | Maximum path loss (dB) | Target Performance (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ISD=400m | Gap |
| 28GHz | PUSCH for eMBB | DDDSU | NLOS (O-to-O) | 1 | -0.25 | 135.43 | 134.85 | -0.58 |
|  | VoIP | DDDSU |  | 4 | -8.75 | 137.91 |  | -3.09 |
|  | PUCCH format 1 for UCI 2bits | - |  | 1 | -7.8 | 136.19 |  | -1.34 |
|  | PUCCH format 3 for UCI 11bits | - |  | 1 | -3.9 | 132.29 |  | 2.56 |
|  | PUCCH format 3 for UCI 22bits | - |  | 1 | -1.1 | 129.49 |  | 5.36 |
|  | PDSCH for eMBB | DDDSU |  | 2 | 4.9 | 153.51 |  | -18.66 |
|  | PDCCH | - |  | 48 | -7.88 | 148.77 |  | -13.92 |

As observed in Table 4, there is -0.58 dB gap from target ISD=400m for PUSCH with 50kbps eMBB service in case of TDD configuration with DDDSU. For VoIP service, there is -3.09 dB gap for 0.02 target residual BLER. For PUCCH, there are -1.34, 2.56, and 5.36 dB gap for format 1 with 2 UCI bits, format 3 with 11 UCI bits, and format 3 with 22 UCI bits, respectively. For downlink channel, there are -18.66 and -13.92 dB gap for PDSCH with eMBB service and PDCCH, respectively. Based on the results, it is observed that for suburban scenario with O-to-O (30km/h) channel, PUCCH (including CSI) enhancement are needed with ISD=400m.

Observation 7. For suburban scenario with O-to-O (30km/h) channel for FR2, PUCCH including CSI enhancement is needed with ISD=400m.

Table 5. Results for suburban scenario with NLOS O-to-O (120km/h) channel at FR2

| Carrier | Service & data rate | Frame structure | Pathloss model | # of PRBs | Required SNR (dB) | Maximum path loss (dB) | Target Performance (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | ISD=400m | Gap |
| 28GHz | PUSCH for eMBB | DDDSU | NLOS (O-to-O) | 1 | -1.2 | 136.38 | 134.85 | -1.53 |
|  | VoIP | DDDSU |  | 4 | -6.8 | 135.96 |  | -1.11 |
|  | PUCCH format 1 for UCI 2bits | - |  | 1 | -7.8 | 136.19 |  | -1.34 |
|  | PUCCH format 3 for UCI 11bits | - |  | 1 | -3.9 | 132.29 |  | 2.56 |
|  | PUCCH format 3 for UCI 22bits | - |  | 1 | -1.0 | 129.39 |  | 5.46 |
|  | PDSCH for eMBB | DDDSU |  | 2 | 4.15 | 154.26 |  | -19.41 |
|  | PDCCH | - |  | 48 | -7.88 | 148.77 |  | -13.92 |

As observed in Table 5, there is -1.53 dB gap from target ISD=400m for PUSCH with 50kbps eMBB service in case of TDD configuration with DDDSU. For VoIP service, there is -1.11 dB gap for 0.02 target residual BLER. For PUCCH, there are -1.34, 2.56 and 5.46 dB gap for format 1 with 2 UCI bits, format 3 with 11 UCI bits, and format 3 with 22 UCI bits, respectively. For downlink channel, there are -19.41 and -13.92 dB gap for PDSCH with eMBB service and PDCCH, respectively. Based on the results, it is observed that for suburban scenario with O-to-O (120km/h) channel, PUCCH (including CSI) enhancement are needed with ISD=400m.

Observation 8. For suburban scenario with O-to-O (30km/h) channel for FR2, PUCCH including CSI enhancement is needed with ISD=400m.

Results for indoor scenario at 28GHz

Baseline performance is provided for indoor scenario with 28 GHz carrier frequency with NLOS channel.

Table 6. Results for indoor scenario with NLOS channel at FR2

| Carrier | Service & data rate | Frame structure | Pathloss model | # of PRBs | Required SNR (dB) | Maximum path loss (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 28GHz | PUSCH for eMBB | DDDSU | NLOS | 30 | 2.4 | 127.94 |
|  | VoIP | DDDSU |  | 4 | -7.55 | 146.64 |
|  | PUCCH format 1 for UCI 2bits | - |  | 1 | -7.8 | 147.52 |
|  | PUCCH format 3 for UCI 11bits | - |  | 1 | -4.2 | 143.85 |
|  | PUCCH format 3 for UCI 22bits | - |  | 1 | -1.0 | 140.65 |
|  | PDSCH for eMBB | DDDSU |  | 50 | 2.1 | 137.05 |
|  | PDCCH | - |  | 48 | -7.88 | 144.82 |

Conclusion

This contribution discusses the evaluation methodology for FR2 to identify the baseline coverage performance. In addition, we provide the preliminary results for target scenarios. Following observations and proposals are made:

Proposal 1. The available path loss should be considered in the link budget calculation for coverage performance in FR2 to take into account the different scenarios and channel.

Proposal 2. Further discussion is needed on the evaluation assumptions for FR2 and to establish target performance.

Observation 1. Considering the relative performance of the channels, it is observed that UL channels perform worse than DL channels in all considered scenarios.

Observation 2. Preliminary results suggest that performance enhancements are needed for both UL and DL channels.

Observation 3. For urban scenario with O-to-I channel for FR2, PUSCH enhancement in both eMBB and VoIP services is needed with ISD=150m.

Observation 4. For urban scenario with O-to-I channel for FR2, PUCCH enhancement is needed with ISD=150m.

Observation 5. For urban scenario with O-to-O channel for FR2, PUSCH enhancement in eMBB is needed with ISD=150m.

Observation 6. For suburban scenario with O-to-I channel for FR2, PUSCH, PUCCH, PDSCH, and PDCCH enhancement are needed with ISD=400m.

Observation 7. For suburban scenario with O-to-O (30km/h) channel for FR2, PUCCH including CSI enhancement is needed with ISD=400m.

Observation 8. For suburban scenario with O-to-O (30km/h) channel for FR2, PUCCH including CSI enhancement is needed with ISD=400m.

- References

- RP-200861, Revised SID on NR coverage enhancement, China Telecom

- R1-2006160, Baseline coverage performance using LLS for FR1, Samsung

- TR 38.802, Study on new radio access technology Physical layer aspects (Release 14).

- TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz (Release 16).

- Appendix

Table A-1. Link budget template

| Parameter | Values |
| --- | --- |
| Scenario |  |
| Frame structure |  |
| Carrier frequency (Hz) |  |
| BS antenna heights (m) |  |
| UT antenna heights (m) |  |
| Cell area reliability for control channel |  |
| Cell area reliability for data channel |  |
| Transmission bit rate for control channel (bit/s) |  |
| Transmission bit rate for data channel (bit/s) |  |
| Target packet error rate for the required SNR in item (19a) for control channel |  |
| Target packet error rate for the required SNR in item (19b) for data channel |  |
| Spectral efficiency (bit/s/Hz) |  |
| Pathloss model (select from LoS or NLoS) |  |
| UE speed (km/h) |  |
| Feeder loss (dB) |  |
| Transmitter |  |
| (1) Number of transmit antennas. (The number shall be within the indicated range in § 8.4 of Report ITU-R M.2412-0) |  |
| (2) Maximal transmit power per antenna (dBm) |  |
| (3) Total transmit power = function of (1) and (2) (dBm) (The value shall not exceed the indicated value in § 8.4 of Report ITU-R M.2412-0) |  |
| (4) Transmitter antenna gain (dBi) |  |
| (5) Transmitter array gain (depends on transmitter array configurations and technologies such as adaptive beam forming, CDD (cyclic delay diversity), etc.) (dB) |  |
| (6) Control channel power boosting gain (dB) |  |
| (7) Data channel power loss due to pilot/control boosting (dB) |  |
| (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for downlink) |  |
| (9a) Control channel EIRP = (3) + (4) + (5) + (6) – (8) dBm |  |
| (9b) Data channel EIRP = (3) + (4) + (5) – (7) – (8) dBm |  |
| Receiver |  |
| (10) Number of receive antennas (The number shall be within the indicated range in § 8.4 of Report ITU-R M.2412-0) |  |
| (11) Receiver antenna gain (dBi) |  |
| (11bis) Receiver array gain (depends on transmitter array configurations and technologies such as adaptive beam forming, etc.) (dB) |  |
| (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink) |  |
| (13) Receiver noise figure (dB) |  |
| (14) Thermal noise density (dBm/Hz) |  |
| (15a) Receiver interference density for control channel (dBm/Hz) |  |
| (15b) Receiver interference density for data channel (dBm/Hz) |  |
| (16a) Total noise plus interference density for control channel = 10 log (10^(((13) + (14))/10) + 10^((15a)/10)) dBm/Hz |  |
| (16b) Total noise plus interference density for data channel = 10 log (10^(((13) + (14))/10) + 10^((15b)/10)) dBm/Hz |  |
| (17a) Occupied channel bandwidth for control channel (for meeting the requirements of the traffic type) (Hz) |  |
| (17b) Occupied channel bandwidth for data channel (for meeting the requirements of the traffic type) (Hz) |  |
| (18a) Effective noise power for control channel = (16a) + 10 log((17a)) dBm |  |
| (18b) Effective noise power for data channel = (16b) + 10 log((17b)) dBm |  |
| (19a) Required SNR for the control channel (dB) |  |
| (19b) Required SNR for the data channel (dB) |  |
| (20) Receiver implementation margin (dB) |  |
| (21a) H-ARQ gain for control channel (dB) |  |
| (21b) H-ARQ gain for data channel (dB) |  |
| (22a) Receiver sensitivity for control channel = (18a) ++ (19a) + (20) – (21a) dBm |  |
| (22b) Receiver sensitivity for data channel = (18b) ++ (19b) + (20) – (21b) dBm |  |
| (23a) Hardware link budget for control channel = (9a) + (11) + (11bis) − (22a) dB |  |
| (23b) Hardware link budget for data channel = (9b) + (11) + (11bis) − (22b) dB |  |
| Calculation of available pathloss |  |
| (24) Lognormal shadow fading std deviation (dB) |  |
| (25a) Shadow fading margin for control channel (function of the cell area reliability and (24)) (dB) |  |
| (25b) Shadow fading margin for data channel (function of the cell area reliability and (24)) (dB) |  |
| (26) BS selection/macro-diversity gain (dB) |  |
| (27) Penetration margin (dB) |  |
| (28) Other gains (dB) (if any please specify) |  |
| (29a) Available path loss for control channel = (23a) – (25a) + (26) – (27) + (28) – (12) dB |  |
| (29b) Available path loss for data channel = (23b) – (25b) + (26) – (27) + (28) – (12) dB |  |
| Range/coverage efficiency calculation |  |
| (30a) Maximum range for control channel (based on (29a) and according to the system configuration section of the link budget) (m) |  |
| (30b) Maximum range for data channel (based on (29b) and according to the system configuration section of the link budget) (m) |  |

Table A-2. General simulation parameters

| Parameters | Values |
| --- | --- |
| Scenario and frequency | 28GHz |
| Frame structure for TDD | DDDSU (S: 10D:2G:2U) |
| Subcarrier Space | 120kHz |
| UE velocity | Indoor scenario:3km/h Urban scenario: 3km/h for indoor, 30km/h for outdoor. Suburban scenario: 3km/h for indoor, 30km/h, 120km/h for outdoor. |
| Occupied channel bandwidth for | 100MHz |
| Frequency hopping for PUSCH | w/ or w/o frequency hopping |
| Number of antenna elements for BS | Indoor scenario: 128 (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1) Urban/suburban scenario: 256, (M,N,P,Mg,Ng) = (4, 8, 2, 2, 2) |
| Number of TxRUs for BS | 2 |
| Number of UE Tx/Rx chains | 1T2R, 2T2R |
| Channel model for link-level simulation | TDL-A |
| Delay spread | Indoor scenario: 30ns Urban scenario: 100ns Suburban scenario: 100ns |

Table A-3. Channel specific parameters for PUSCH and PDSCH

| Parameters | Values |
| --- | --- |
| BLER | For eMBB, w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| DMRS configuration | For 30km/h and 120km/h: Type I, 2 DMRS symbol, no multiplexing with data. For 3km/h: Type I, 1 DMRS symbol, no multiplexing with data. |
| Waveform | DFT-s-OFDM for PUSCH, CP-OFDM for PDSCH |
| Repetitions for PUSCH/PDSCH | For eMBB, w/o repetition as baseline For VoIP, w/ repetition and # of actual repetitions = 2 (2/14 symbols in DDDSU with repetition type B) |
| HARQ configuration for PUSCH/PDSCH | For eMBB, no HARQ transmission. For VoIP, w/ HARQ and the maximum number of HARQ transmission = 32 due to latency requirements. |
| PUSCH/PDSCH duration | 14 OS for PUSCH, 12 OS for PDSCH |
| PRBs/TBS/MCS for eMBB for PUSCH | 30 PRBs for 5Mbps and 1 PRBs for 50kbps QPSK modulation |
| PRBs/MCS for VoIP for PUSCH | 4 PRBs for VoIP QPSK modulation |
| PRBs/MCS for eMBB for PDSCH | 50 PRBs for 25Mbps and 2 PRBs for 1Mbps QPSK modulation |
| Frequency hopping | w/o frequency hopping for PUSCH |

Table A-4. Channel specific parameters for PUCCH

| Parameters | Values |
| --- | --- |
| Format | Format 1, 2bits UCI. Format 3, 11/22 bits UCI |
| BLER for PUCCH | For PUCCH format 1: ACK missed detection probability: 1%. For PUCCH format 3: BLER: 1% |
| Number of PRBs for PUCCH | 1 PRB |
| Number of UE transmit chains for PUCCH | 1 |
| Number of repetitions for PUCCH | w/o repetition for PUCCH. |
| PUCCH duration | 14 OFDM symbols |
| DMRS configuration for PUCCH | Additional DMRS symbols for PUCCH Format 3. |
| Frequency hopping | w/ frequency hopping for PUCCH |

Table A-5. Channel specific parameters for PDCCH

| Parameters | Values |
| --- | --- |
| Aggregation level | 16 |
| Payload | 40 bits |
| CORESET size | 2 symbols, 48PRBs |
| Tx Diversity | Precoder cycling |
| BLER for PDCCH | 1% BLER. |