3GPP TSG RAN WG1 #102-e		                                                 R1-2006215

e-Meeting, August 17th – 28th, 2020

Agenda item:	8.5.2

Source: 	CMCC

Title: 	Discussion on achievable positioning latency

Document for:	Discussion and Decision

Introduction

In RAN#86 meeting, a new study item for NR positioning enhancements was approved [1], in which the motivation to provide higher accuracy location requirements resulting from new applications and industry verticals was justified.

In RAN1#101 e-meeting, several agreements regarding the positioning latency were made as follows:

Agreement: - In Rel-17 target positioning requirements for commercial use cases are defined as follows: - Horizontal position accuracy (< 1 m) for [90%] of UEs - Vertical position accuracy (< [2 or 3] m) for [90%] of UEs - End-to-end latency for position estimation of UE (< [100 ms]) - FFS: Physical layer latency for position estimation of UE (< [10 ms]) - In Rel-17 target positioning requirements for IIoT use cases are defined as follows: - Horizontal position accuracy (< X m) for [90%] of UEs - X = [0.2 or 0.5] m - Vertical position accuracy (< Y m) for [90%] of UEs - Y = [0.2 or 1] m - End-to-end latency for position estimation of UE (< [10ms, 20ms, or 100ms]) - FFS: Physical layer latency for position estimation of UE (< [10ms]) - Note: Target positioning requirements may not necessarily be reached for all scenarios Agreement: Physical layer latency can be evaluated through analysis and, optionally, numerical evaluation. Agreement: Higher layer positioning latency can be evaluated in this SI. - FFS: how to evaluate higher-layer positioning latency - FFS: which higher-layers should be included in the evaluation

In this contribution, we focus on the evaluation and analysis on the physical layer latency, and provide our detailed views on the target physical layer latency value.

Discussion on positioning latency

In TS38.305, the overall sequence of events applicable to the UE, NG-RAN and LMF for any location service is defined, as shown in Figure 1, where the completion of this overall procedure is related to the end-to-end latency. Either some entity in 5GC, or the serving AMF, or the UE can request some location service, and the AMF is responsible for transferring the location service request to the LMF. The LMF then can trigger the location procedures with the serving and neighbouring gNB or with the UE to obtain positioning measurements and location estimates, etc. The location service responses are provided by the LMF to the AMF, who can then return the location service response back to the entity that requests the location service. It is noted that the whole process (location clients (entity in 5GC, AMF, UE) -> AMF -> LMF -> gNB/UE -> LMF -> AMF -> location clients) includes long and complicated high layer signalling exchange procedures, and the physical layer procedures happen at steps 3a and 3b.

Observation 1: The current higher layer procedure is long and complicated, and the latency of which is able to be further reduced, e.g., by enabling enhanced higher layer architecture and signalling procedure.

Figure 1: Location Service Support by NG-RAN

For the physical layer procedure, the latency is mainly caused by the following aspects:

- Configuration of measurement gap for DL PRS measurement (if needed): up to RAN4 requirements;

- DL PRS / UL SRS pos periodicity: In Rel-16, the periodicity of the DL PRS can be configured as 2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240 slots; the periodicity of P-SRS pos can be configured as {1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240, 40960, 81920} slots;

- Processing delay of reporting measurements through PUSCH: to be simplified as 1 slot;

Taking the DL PRS measurement as an example, the periodicity of a DL PRS resource set and its corresponding latency in 30kHz SCS and 120kHz SCS are listed as follows:

| DL PRS periodicity (1 resource per resource set) | Physical layer latency (no MG is considered) |  |
| --- | --- | --- |
|  | 30KHz | 120KHz |
| 4 | 2.5ms | 0.625ms |
| 5 | 3ms | 0.75ms |
| 8 | 4.5ms | 1.125ms |
| 10 | 5.5ms | 1.375ms |
| 16 | 8.5ms | 2.125ms |
| 20 | 10.5ms | 2.625ms |
| 32 | 16.5ms | 4.125ms |
| 40 | 20.5ms | 5.125ms |
| 64 | 32.5ms | 8.125ms |
| 80 | 40.5ms | 10.125ms |
| 160 | 80.5ms | 20.125ms |
| 320 | 160.5ms | 40.125ms |
| 640 | 320.5ms | 80.125ms |
| 1280 | 640.5ms | 160.125ms |
| 2560 | 1280.5ms | 320.125ms |
| 5120 | 2560.5ms | 640.125ms |
| 10240 | 5120.5ms | 1280.125ms |

From the above table, it is apparent that to meet a 10ms physical layer latency that identified in the RAN1#101-e meeting agreement, the periodicity of the DL PRS should be carefully configured, and a maximum periodicity of 16 slots and 64 slots is allowed for 30kHz and 120kHz SCS. If a UE expects to measure the DL PRS that is outside of its active DL BWP and the measurement gap should be requested, the allowed DL PRS periodicity becomes shorter. In addition, only 1 DL PRS resource without repetition is considered in the table, to further configure beam sweeping or DL PRS repetition, it is apparent that the DL PRS overhead would be quite heavy.

Observation 2: To achieve a physical layer procedure of less than 10ms, the configuration of the DL PRS periodicity is limited and the DL PRS overhead would be heavy.

In RAN1#101 e-meeting, it has been agreed that for the commercial use cases, the end-to-end latency should be less than 100 ms, and for the IIoT use cases, besides the end-to-end latency requirement of less than 100ms, more stringent requirements of less than 10ms and 20ms were listed as candidate values. Considering some special IIoT use cases, such as the localization of AGVs in logistics and warehousing, whose end-to-end latency is required to be as less as 10ms, the physical layer latency should be less than 10ms in order to achieve the total 10ms end-to-end latency requirements. In such cases, as analysed above, since the periodicity of the DL PRS is limited and its overhead would be an issue, a potential enhancement of supporting aperiodic DL PRS should be considered in Rel-17. On the other hand, considering a common end-to-end latency of less than 100ms, and since the higher layer signalling and its latency is able to be simplified and reduced, we believe that a physical layer latency of less than 50ms should also be supported.

Proposal 1:

- In Rel-17 target positioning requirements for commercial use cases:

- End-to-end latency for position estimation of UE (< 100 ms)

- Physical layer latency for position estimation of UE (< 50 ms)

- In Rel-17 target positioning requirements for IIoT use cases:

- End-to-end latency for position estimation of UE (<10ms)

- Physical layer latency for position estimation of UE (<10ms)

Conclusions

In this contribution, we discuss the NR positioning enhancements in IIoT scenarios, and the following observations and proposals are provided:

Observation 1: The current higher layer procedure is long and complicated, and the latency of which is able to be further reduced, e.g., by enabling enhanced higher layer architecture and signalling procedure.

Observation 2: To achieve a physical layer procedure of less than 10ms, the configuration of the DL PRS periodicity is limited and the DL PRS overhead would be heavy.

Proposal 1:

- In Rel-17 target positioning requirements for commercial use cases:

- End-to-end latency for position estimation of UE (< 100 ms)

- Physical layer latency for position estimation of UE (< 50 ms)

- In Rel-17 target positioning requirements for IIoT use cases:

- End-to-end latency for position estimation of UE (<10ms)

- Physical layer latency for position estimation of UE (<10ms)

References

- RP-193237, New SID NR positioning enhancements, Qualcomm Incorporated, Sitges, Spain.