3GPP TSG RAN WG1#102-e	R1-2006572

eMeeting, August 17th – 28th, 2020

Source:	Sharp

Title:	UE feedback enhancements for HARQ-ACK

Agenda Item:	8.3.1.1

Document for:	Discussion and Decision

- Introduction

In this contribution, we provide discussions on UE feedback enhancements for HARQ-ACK with respect to the following agreed objective in [1]:

1. Study, identify and specify if needed, required Physical Layer feedback enhancements for meeting URLLC requirements covering  UE feedback enhancements for HARQ-ACK [RAN1]

- Discussions on HARQ-ACK feedback enhancements

Although Type-3 HARQ-ACK codebook was introduced as a new type of codebook during Rel-16 specification for NR-U [2], we observed that exploiting Type-3 HARQ-ACK codebook is beneficial to URLLC operation due to the following advantages:

- Can be dynamically triggered by DCI as required

- One-shot feedback for all configured HARQ processes

- Specification on Type-3 HARQ-ACK codebook generation has been completed in NR-U

To adopt Type-3 HARQ-ACK codebook in URLLC operation, one remaining issue is when Type-3 HARQ-ACK codebook meets prioritization of HARQ-ACK codebook. More specifically, the problem is that it is not specified how to handle Type-3 HARQ-ACK codebook with mixed priorities. To solve this problem, there might be several alternatives:

- Alt. 1: Associate Type-3 HARQ-ACK codebook with one priority index

  - 1a: The priority index is predetermined, e.g., priority index 1

  - 1b: The priority index is RRC configured

  - 1c: The priority index is indicated by the Priority field in the triggering DCI format

- Alt. 2: Generate two Type-3 HARQ-ACK codebooks, each corresponding to a priority index

Among the alternatives, we are preferable to Alt. 1c because 1) the flexibility by dynamically indication, 2) integration of Type-3 codebook triggering and prioritization within one DCI format, 3) relatively small specification impact, e.g., no introduction of RRC parameter or modification of pseudo-code. However, at the beginning phase of Rel-17 specification, we suggest to firstly clarify the necessity of adopting Type-3 HARQ-ACK codebook in URLLC operation.

Proposal 1: Support Type-3 HARQ-ACK codebook corresponding to mixed priorities.

- Conclusion

In this contribution, we provide the following proposal:

Proposal 1: Support Type-3 HARQ-ACK codebook corresponding to mixed priorities.

- References

  - RP-201310, “Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR”, Nokia, Nokia Shanghai Bell, July 2020

  - R1-1913599, “Summary of NR-U agreements till RAN1 99”, Qualcomm Incorporated, November 2019