3GPP TSG RAN WG1 Meeting #102-e	R1-2006645

e-Meeting, August 17th – 28th, 2020

Source:		SoftBank

Title:	Views on target performance metric and values for FR1 coverage enhancements

Agenda Item:	8.8.1.1

Document for:	Discussion and Decision

Introduction

This paper shows our views on the target performance metric and values for FR1 coverage enhancements, which is an open issue from RAN1#101-e.

Discussions

Absolute metric vs. relative metric

Absolute metric is a “market demand based” approach because the required value can be brought from outside 3GPP activities, e.g. practical deployments. On the other hand, relative metrics is a “technical based” approach because the coverage bottleneck is defined based on the coverage gap analysis among channels/signals. Therefore, absolute metric (and the specific target value) should be adopted based on the operators’ (market) demand. If the demand and the reason are not clarified/identified in RAN1, relative metric can be adopted.

Proposal:

- Absolute metric (and the specific target value) should be adopted based on the operators’ (market) demand.

If the demand and the reason are not clarified or identified in RAN1, relative metric can be adopted.

Views on absolute metric

The potential performance metrics (i.e. ISD, MPL, and MCL) are convertible to each other if the necessary parameters are clearly defined. Therefore, we don’t see any critical problem to employ either of them as long as all the values are captured in the link budget table, even though our preference is to remove any scenario dependencies from the study as much as possible. If scenario dependent metric (such as ISD) is adopted as a target performance metric, the following aspects should be considered:

Firstly, from our perspective, MCL is anyway needed to evaluate our coverage because of some reasons, e.g. the deployment parameters are not always aligned with the 3GPP model. Therefore, we need to check the validity of the potential target ISD value by converting it to MCL. RAN1 should ensure that the target value is approved after all the simulation parameters and methodologies are stable. Secondly, as long as we understand from the contributions submitted at RAN1#101-e, many companies proposed to adopt ISD for target performance metric, and its target value is the ISD of IMT-2020 model (i.e. 500m for urban and 1732m for rural). However, ISD of IMT-2020 is just informative, and the real life deployment does not strictly follow it. In this sense, we don’t see strong reason to reuse IMT-2020 ISD for this purpose. Even if ISD is agreed as a target performance metric, this should not imply that IMT-2020 ISD is adopted as a target value.

Proposal:

- RAN1 should ensure that the target value is approved after all the simulation parameters and methodologies are perfectly stable, especially when scenario dependent metric (such as ISD) is adopted

If ISD is agreed as a target performance metric, this should not imply that the ISD defined by IMT-2020 scenarios is adopted for a target value.

Target values

As discussed in the last meeting based on R1-2003464, we want to achieve MCL>=147dB for FR1 voice. Note that the definition of MCL follows that of TR36.824. If the definition itself is modified, this value needs to be adjusted accordingly. In addition, if a target performance metric other than MLC is adopted, the target value should correspond to MCL>=147dB.

Proposal:

- For FR1 voice, it is proposed to adopt at least MCL=147dB for target value.

The definition of MCL is based on that of TR36.824. If the definition is modified (e.g. include antenna gain), the value of 147dB needs to be adjusted.

If a target performance metric other than MCL is adopted, the target value should correspond to MCL>=147dB

Conclusion

Our proposals are summarized as follows:

- Absolute metric (and the specific target value) should be adopted based on the operators’ (market) demand.

If the demand and the reason are not clarified or identified in RAN1, relative metric can be adopted.

- RAN1 should ensure that the target value is approved after all the simulation parameters and methodologies are stable, especially when scenario dependent metric (such as ISD) is adopted

If ISD is agreed as a target performance metric, this should not imply that the ISD defined by IMT-2020 scenarios is adopted for a target value.

- For FR1 voice, it is proposed to adopt at least MCL=147dB for target value.

The definition of MCL is based on that of TR36.824. If the definition is modified (e.g. include antenna gain), the value of 147dB needs to be adjusted.

If a target performance metric other than MCL is adopted, the target value should correspond to MCL>=147dB