3GPP TSG RAN WG1 Meeting #102-e	R1-2006945

E-meeting, August 17 – 28, 2020

Agenda Item:	5

Source:	Huawei, HiSilicon

Title:	Discussion on NR SRS carrier switching

Document for:	Discussion and Decision

# Introduction

During RAN2#110-e, RAN2 discussed Rel-15 RRC specification correction on NR SRS carrier switching. However, for Type A NR SRS carrier switching, RAN2 did not achieve consensus on whether the following 2 cases on NR SRS carrier switching are allowed:

- Case 1: SRS carrier switching to a SUL without PUCCH/PUSCH while PUCCH/PUSCH is configured in NUL;

- Case 2: SRS carrier switching to both NUL and SUL, neither of which are configured with PUCCH/PUSCH.

As a result, an LS [1] about this issue has been sent to RAN1. In this contribution our views and analysis towards the issue are provided.

# Discussion

Case 1 and Case 2 above have been supported by the following agreements in RAN1#89 and RAN1#91:

Agreements: - For NR standalone operation for a UE, - NR supports that the UE is allowed to transmit on UL carriers on different frequency ranges but the UE has the capability to only transmit on one of the carriers at a given time in the following case: - case of SRS carrier switching with at least one of the frequency ranges agreed for LTE-NR UL sharing by RAN4 (e.g. refer to R4-1704411)

Agreement: - For a UE configured two ULs in a cell, two TPC bit(s) fields within group common DCI for SRS power control can be configured to the UE, i.e., one for UL and one for SUL;

Therefore, it is not reasonable to restrict further Type A SRS carrier switching from supporting them. We would like to give the proposal as below, and the corresponding LS reply can be found in Appendix.

Proposal 1: Confirm that both Case 1 and Case 2 described in R1-2005209 are supported by Type A SRS carrier switching.

Proposal 2: Inform RAN2 about above according to the draft LS reply in Appendix.

# Conclusions

In this contribution, analysis of the issue of NR SRS carrier switching was presented. The following proposal is provided:

Proposal 1: Confirm that both Case 1 and Case 2 described in R1-2005209 are supported by Type A SRS carrier switching.

Proposal 2: Inform RAN2 about above according to the draft LS reply in Appendix.

# References

R2-2006362(R1-2005209) “LS on NR SRS carrier switching”, Qualcomm, 3GPP RAN2#110 Electronic Meeting, June 1st – 12th, 2020.

# Appendix

# Draft LS reply on NR SRS carrier switching

# Title: Draft Reply LS on NR SRS carrier switching # Response to: R1-2005209/R2-2006362 # Release: Rel-15 # Work Item: NR_newRAT-Core Source: Huawei, [RAN WG1] To: RAN WG2 Cc: Contact Person: Name: Frank Long E-mail Address: frank.longyi {@} huawei.com 1. Overall Description: RAN1 would like to thank RAN2 for the LS and the following question: Question: Whether the following two cases on type A NR SRS carrier switching are allowed. - Case 1: SRS carrier switching to a SUL without PUCCH/PUSCH while PUCCH/PUSCH is configured in NUL; - Case 2: SRS carrier switching to both NUL and SUL, neither of which are configured with PUCCH/PUSCH. RAN1 would like to provide a response as below, Response: Both Case 1 and Case 2 are supported by Type A SRS carrier switching. 2. Actions: To RAN2. ACTION: RAN1 respectfully requests RAN2 to take the above response into account. 3. Date of Next TSG-RAN WG1 Meetings: TSG-RAN WG1 Meeting#103-e October 26th – November 13th 2020 E-Meeting TSG-RAN WG1 Meeting#104 March 1st - 5th 2021 Athens