3GPP TSG RAN WG1 Meeting #102-e															R1-2006982

e-Meeting, August 17th – 28th, 2020

Source: 	Moderator (Intel Corporation)

Title:	Issue Summary for physical layer changes for supporting NR from 52.6 GHz to 71 GHz

Agenda item:	8.2.1

Document for:	Discussion/Decision

# Introduction

In this contribution, we summarize all issues submitted on physical layer changes and aspects for supporting NR from 52.6 GHz to 71 GHz for RAN1 #102-e meeting. Section 2 contain a summary of issues identified from contributions submitted to RAN1 #102-e [1] ~ [31]. The list of issues in Section 2 are not ordered in terms of criticalness/discussion priority.

# Summary of issues

This section provides a categorization of issues mentioned in the submitted contribution for supporting NR from 52.6 GHz to 71 GHz. For each categorized issue, summary of observations and proposals made in the contribution is provided. It should be noted that some observations and proposals made can be placed in multiple categories. However, the moderator has place them based on the main discussion aspect.

## 2.1 General Comments on SI

- From [5]:

  - To support operation between 52.6 GHz and 71 GHz, a common framework should be shared for licensed and unlicensed operation for less standardization workload and specification burden.

  - To figure out the required changes to NR using existing NR waveform, discussion on unlicensed operation can be prioritized.

  - Based on the outcome, licensed operation can be supported by simply removing some unnecessary functionalities or adding essential functionalities if any.

  - If it is possible that a spectrum is for unlicensed operation in a region but for licensed operation in another region, mechanism to allow UE to differentiate two types of operation should be considered at the beginning.

- From [17]:

  - The study item shall support both licensed and unlicensed operation between 52.6 GHz and 71 GHz, and unlicensed band between 57 GHz and 71 GHz (i.e., 60 GHz unlicensed band) should be prioritized.

- From [30]:

  - 60 GHz unlicensed band should be prioritized for this SI study.

  - Short range high data rate D2D deployment scenario should be studied for above 52.6 GHz band.

  - Higher priority should be given for CA case, where above 52.6 GHz is only used for SCell for throughput boosting.

## 2.2 General Comments on Numerology Study

- From [5]:

  - In order to achieve higher efficiency from signaling overhead and resource utilization perspective, wider channel BW beyond 400 MHz should be supported.

  - Furthermore, Lager SCS than 120 kHz can be introduced to have small FFT size in case of wider channel BW and robustness to phase noise at the higher frequency

  - The CP can be scaled following the SCS in the same way as NCP in Rel-15.

  - The tradeoff between performance and cost should be taken into account in the discussion on how wide channel BW and SCS would be supported in the range from 52.6GHz to 71GHz.

- From [9]:

  - The physical channel/signals with the potential impacts by the introducing of higher SCS for data and control channels are as follows,

    - (1) Type0-PDCCH SCS indication in MIB

    - (2) PRACH preamble

    - (3) UE processing capability and the required processing time for higher SCS

- From [15]:

  - For selection of suitable SCS for the 52.6 – 71 GHz frequency range, it is important to perform link level evaluations with 90th percentile RMS delay spreads that are representative of a suitable range of deployment scenarios with different site densities, e.g., up to several tens of ns.

  - Sufficient margin must also be left for other sources of time synchronization error.

- From [19]:

  - Study the impact of channel bandwidth and numerology to physical signal/channel, e.g. the time line, SS/PBCH block, PT-RS and PDCCH monitoring capability

- From [20]:

  - Study further on potential impacts (and relevant handling) due to the shortening of OFDM symbol duration and CP length by adopting larger SCS value.

  - Whether/how to handle impact to cell coverage and/or beam switching time (e.g. by employing the extended CP and/or grouping multiple OFDM symbols as a unit)

- From [21]:

  - Study multiples of 400 MHz up to 2 GHz is considered for above 52.6 GHz.

  - Study potential coexistence issue with other RAT in the spectrum of 52.6 GHz to 71 GHz.

- From [22]:

  - Down-select SCS based on the phase noise reduction requirements of transmission at < 71 GHz, the bandwidth requirements and the cyclic prefix required to mitigate the effect of the beam formed delay spread.

- From [23]:

  - To support various wide bands and use cases for NR operation from 52.6 GHz to 71 GHz, a wide range of numerologies with the carrier aggregation need to be studied, also considering the LBT bandwidth (or the RB set) for co-existing issues, UE capability, processing time and power consumption.

- From [25]:

  - The changes added to the current NR should be minimized.

  - In this sense, only one or two SCSs are sufficient for 52.6 – 71 GHz band in our view

- From [29]:

  - In addition to channel BW and link performance aspects, RAN1 should consider also implementation complexity associated with high SCSs when selecting the supported SCSs for above 52.6 GHz.

  - Extend the numerology scaling framework defined in NR Rel-15 to higher numerologies with an appropriate range of integer values for μ.

  - Maintain the maximum number of RBs supported by NR specification also for NR scenario above 52.6 GHz.

## 2.3 SSB pattern and SSB/CORESET multiplexing

- From [2]:

  - Multiplexing patterns 2 and 3 for SSB and CORESET for Type0-PDCCH better facilitate meeting the OCB requirement in NR-U-60.

- From [7]:

  - SSB pattern could be re-designed whether higher SCS is supported or not. Transmission opportunities, timing and QCI of Rel-17 SSB should be considered.

- From [12]:

  - Introduce groups of SCS in FR2 and all control/data communication will use the SCS from one such group.

  - It is proposed to investigate how to transmit the indication about additional SS/PBCH candidate positions which can become available with existing FR2 numerologies or future new numerologies.

  - It is proposed to investigate efficient transmission of MSI including the multiplexing patterns for both licensed and shared carriers.

- From [14]:

  - When a large subcarrier spacing is defined, SSB pattern and multiplexing of SSB and CORESET0/RMSI need to be updated to accommodate beam switching time.

- From [15]:

  - Do not design for SS/PBCH block sliding within a transmission window for >52.6 GHz operation.

  - For NR operations in the 52.6 – 71 GHz band, consider only 120 and 240 kHz SCS for SS/PBCH blocks, as already supported in Rel-15/16.

  - Consider reusing the SS/PBCH / CORSET0 multiplexing patterns as much as possible.

  - If minor, targeted, enhancements to particular pattern(s) are beneficial, these can be considered.

    - SS/PBCH / CORESET0 multiplexing patterns 2 and 3 are restricted to very small RMSI payloads due to the small number (2) of available OFDM symbols for RMSI PDSCH.

    - SS/PBCH / CORESET0 multiplexing pattern 1, especially with non-zero offset O for the Type0-PDCCH monitoring occasions, is much less restrictive in terms of allowable RMSI payload due to the fact that SS/PBCH and RMSI PDCCH/PDSCH are time division multiplexed.

  - Consider enhancements to SS/PBCH / CORESET0 multiplexing Pattern 1 as follows:

    - (1) Allow (240 kHz, 240 kHz) SCS,

    - (2) Support 6 symbol SLIV in Default Table A starting at OFDM symbols 2 and 8.

- From [17]:

  - RAN1 shall study the SS/PBCH block pattern for the new numerology, taking into account the beam switching time between neighboring SS/PBCH blocks.

- From [20]:

  - Consider the enhancements for the SSB transmission to provide more opportunities in FR-X unlicensed band.

  - Study further how to multiplex SSB and corresponding CORESET#0 in case of using new numerologies such as 240/480 kHz SCSs for the DL signal/channels other than SSB.

- From [25]:

  - Whether to introduce gap symbol(s) for beam switching time should be discussed not only for SSB but also for any signal/channels with beam switching in case that higher SCS such as 960 kHz is supported.

  - For SSB and CORESET multiplexing, following aspects should be discussed:

    - Which SCS(s) is supported for SSB and which combination(s) of SCS between SSB and CORESET#0 is supported;

    - Whether only single numerology is supported as in Rel-16 NR-U or not;

    - Whether the number of supported SCSs for SSB should be minimized.

  - Which multiplexing pattern between SSB and CORESET#0 is supported for each combination of SCS between SSB and CORESET#0:

    - What are minimum channel bandwidth, minimum required CORESET#0 bandwidth and minimum required bandwidth for RMSI PDSCH;

    - Whether beam sweeping overhead should be minimized by FDM between SSB and CORESET#0 and/or RMSI PDSCH

- From [27]:

  - SSB design should be enhanced to match unlicensed band requirements.

- From [28]:

  - SSB pattern in a slot should be further investigated for higher subcarrier spacing (e.g. candidate subcarrier spacings of 480kHz, 960kHz, or 1920kHz) taking into account a beam switching gap due to a RF interruption time of Tx/Rx beams and/or LBT gap in unlicensed spectrum.

  - It should be further studied so that SS/PBCH block and CORESET#0/RMSI can be multiplexed in TDM/FDM within a slot considering multi-beam operation. And it may need to be designed so that it can be closely located without the gap between SSB and CORESET#0/RMSI for the operation of the unlicensed band in terms of channel access.

- From [29]:

  - If SSB design is needed for 960 kHz SCS, design new SSB mapping pattern that allows beam switching gap of 100 ns and/or possible LBT gap between consecutive SSBs.

  - Existing FR2 SSB and Type0-PDCCH multiplexing patterns are a good starting point for above 52.6 GHz operation.

  - If SSB design is needed for 960 kHz SCS, changes would be needed to SSB and RMSI multiplexing patterns, and more specifically on the CORESET Type0-PDCCH time domain allocation to take into potentially required beam switching and/or LBT gap.

- From [31]:

  - Study the window duration/timing granularity to search a SSB set.

## 2.4 SSB numerology

- From [4]:

  - SSB numerology would better to be determined after BWP numerology is selected and supported (SSB, corset 0) numerology pairs need to be determined as well by considering koffset indication and SSB-Coreset 0 multiplexing pattern.

- From [15]:

  - There are several sources of frequency errors, e.g. inter-gNB frequency accuracy, UE initial frequency accuracy, UE frequency drift and Doppler shift, all which scales with the carrier frequency.

  - Thus, to keep the ratio ∆f/SCS similar at different carrier frequencies, the SCS needs to scale accordingly.

  - From a frequency error perspective, an SSB SCS of either 240 kHz or 480 kHz seems reasonable for a 60 GHz carrier frequency.

  - A higher UL SCS puts tighter requirements on UE UL timing accuracy.

  - To avoid further tightening the UE requirement on UL timing error in relation to 1/SCSSSB compared to current specifications, the UL SCS should not be more than twice that of the SSB SCS.

  - This motivates selection of UL SCS to be no greater than 480 kHz assuming the maximum SSB SCS of 240 kHz in the spec today.

  - Extended CP need not be considered for NR operation in 52.6 to 71 GHz.

- From [16]:

  - consider reusing FR2 SCS for initial access phase

- From [29]:

  - Regarding SSB numerologies:

    - Support existing SSB numerologies and

    - study further need for new numerologies for SSB and Type0-PDCCH design.

## 2.5 Cell Search Complexity

- From [4]:

  - frequency domain offset estimation during SSB detection. With increasing of the center frequency, the absolute value for frequency domain offset is increased if assuming the same ratio (e.g. 10ppm).

  - The following aspects should be studied for SSB design: (1) Frequency domain offset estimation; (2) Amount of buffering SSB samples; (3) Beam switching for contiguous candidate SSBs.

## 2.6 TDD Transition Time

- From [3]:

  - A larger fraction of a slot is used for switching between Tx and Rx with higher numerology, which is 7µs.

  - For 240 kHz SCS, 2 symbols would be needed for transition, 4 symbols are needed for 480 kHz SCS, and 7 symbols are needed for 960 kHz SCS. This additional overhead should be accounted.

## 2.7 Cell Coverage

- From [2]:

  - 60 kHz  SCS (support up to 2500m), 120 kHz SCS (support up to 1250m), 	240 kHz SCS (support up to 625m), 480 kHz   SCS	(support up to 313m), 960 kHz  SCS (support up to 156m)

  - 60 kHz SCS can support a coverage close to the maximum expected coverage for the backhauling use cases while increasing the SCS of preamble will reduce the coverage and the maximum cell radius.

- From [4]:

  - Coverage enhancement mechanism should be studied for PDCCH design especially for high SCS.

- From [29]:

  - Support improved PDCCH coverage for the cases of high SCS

  - Consider coverage enhancements for channels and signals with higher SCS.

## 2.8 PRACH

- From [2]:

  - For unlicensed band, new ZC lengths should be considered (for OCB).

- From [4]:

  - Format 0-3 with special SCS is not supported and the candidate PRACH numerologies for format A, B and C are the same as the candidate BWP numerologies. Both coverage and capacity should be studied for PRACH design with new defined numerology.

- From [14]:

  - When a large subcarrier spacing is defined, PRACH configuration related aspects need to be investigated.

- From [17]:

  - RAN1 shall study the scaling/reuse of current PRACH SCS with some enhancement including the non-consecutive RO configuration.

- From [25]:

  - For PRACH sequence, short PRACH sequence supported in Rel-15 NR should be a baseline

- From [29]:

  - Introducing longer sequence lengths for short time domain PRACH preambles, e.g. the ones supported in Rel-16 NR-U (571 and 1151), would allow transmitting device to achieve 40 dBm EIRP maximum in CEPT scenarios c1 and c2.

## 2.9 Addition RF Impairments

- From [4]:

  - Perform modeling of I/Q imbalance in link level evaluation with reasonable sideband suppression value, and study potential enhancement if problem is identified.

  - Perform PAPR evaluation for different channels/signals, and study potential PAPR reduction technique if problem is identified.

## 2.10 PT-RS

- From [1]:

  - For supporting NR operation between 52.6GHz and 71GHz in Rel. 17, no PT-RS configuration should also be supported, depending up on the MCS range, if higher subcarrier spacing values are agreed to be supported.

- From [2]:

  - Block PTRS enables low complexity ICI compensation for smaller SCSs such as 120 kHz and 240 kHz and helps the smaller SCS to perform even better than a larger SCS such as 960 kHz.

- From [4]:

  - DM-RS/PT-RS enhancement should be studied to solve the problem brought by RF impairment such as phase noise, I-Q imbalance and PA non-linear work range.

- From [8]:

  - Methods to eliminate ICI induced by phase noise should be studied for NR operation in the 60 GHz band.

- From [13]:

  - Investigate PT-RS patterns allowing for ICI compensation for CP-OFDM. Support block-based PTRS patterns for OFDM waveform. Support density extension of current Rel.15 PTRS for DFTsOFDM waveform.

- From [16]:

  - further study the need for PTRS enhancement for smaller SCS than 960KHz

- From [17]:

  - RAN1 shall study the enhancement to reference signals (e.g. PT-RS) for the new carrier frequency range, taking into consideration of the impact from the new numerology.

- From [22]:

  - RAN1 to study the need to update Rel-15 PTRS for both OFDM and DFT-S-OFDM to account increased CPE/ICI at higher frequencies.

- From [23]:

  - The spectral efficiency (include guard band, PT-RS overhead, etc.) for large number of carrier aggregation should be studied for NR operation from 52.6 to 71 GHz.

- From [25]:

  - How to allocate resource for RS (e.g. DMRS, PTRS) in frequency domain needs to be considered for higher SCS if introduced

- From [29]:

  - Consider block-PTRS for CP-OFDM. Consider defining new PTRS configurations for DFT-s-OFDM.

## 2.11 DM-RS

- From [1]:

  - For higher SCS values with both 400MHz and 2GHz bandwidth, BLER performance difference between the ideal channel estimation and real channel estimation varies for different SCS values, where, as the subcarrier spacing is increasing, the performance degradation with real channel estimation also increases which could be attributed to the performance of DM-RS configuration with different SCS values.

  - For supporting NR beyond 52.6 GHz with existing waveforms in Rel. 17, if higher subcarrier spacings (numerologies) are adopted, new DM-RS configurations should be studied.

- From [20]:

  - Investigate the necessity to enhance the structure of DM-RS for data as well as control DL/UL channels.

- From [21]:

  - Study enhanced DM-RS designs for a larger subcarrier spacing for PDSCH and PUSCH. Study channel estimation performance impact of PDCCH and PUCCH with a larger subcarrier spacing.

- From [25]:

  - How to allocate resource for RS (e.g. DMRS, PTRS) in frequency domain needs to be considered for higher SCS if introduced. DMRS density in frequency domain may not be sufficient. DMRS ports multiplexing may not work well

- From [31]:

  - Study enhancement of the frequency domain structure of DMRS for NR on 52.6 GHz to 71 GHz.

## 2.12 Processing Timelines - General

- From [3]:

  - If numerologies higher than 120 kHz are introduced, the processing timelines (BWP switching times, HARQ scheduling, UE processing, preparation and computation times for PDSCH, PUSCH/SRS and CSI) and PDCCH monitoring capability should be studied for the new numerologies.

- From [4]:

  - Timeline definition, basic time unit and super long CP per half frame should be discussed for new defined numerology such as (960K, NCP).

- From [7]:

  - If introducing new numerology, the impacts on processing time and scheduling operation should be considered.

- From [10]:

  - For PDSCH/PUSCH processing, N1/N2 values for µ larger than 3 should be defined with consideration of different UE processing capabilities.

  - For PDSCH to HARQ-ACK timing, the value range of k1 should be extended to facilitate SCS higher than 120kHz. UL grant to PUSCH timing, the value range of k2 should be extended to facilitate SCS higher than 120kHz.

  - UE processing capability for PDSCH/PUSCH should be defined for SCS higher than 120kHz.

  - Multiple slot-based UE processing capability for PDCCH blind decoding for should be defined for µ larger than 3.

- From [11]:

  - Determine the processing time when the new numerologies are decided. Study the range of K0, K1, K2 for the new SCS.

- From [14]:

  - When a large subcarrier spacing is defined, processing time related aspects, including PDSCH/PUSCH processing time, CSI computation time, etc., need to be investigated.

- From [15]:

  - UE processing timelines for SCS > 120 kHz need to be further tightened vis-à-vis those for 120 kHz SCS to enable high performance NR operation in 52.6 to 71 GHz.

  - The times provisioned for UE processing grow exponentially with the numerology.

  - Large processing latencies restrict the achievable throughputs, defeating the purpose of enabling large bandwidths with large sub-carrier spacings.

  - RAN1 should investigate the different factors that contribute to the PDSCH processing time and consider possible latency reduction opportunities.

- From [17]:

  - RAN1 shall study the processing timing related procedures for modification/enhancement, taking into consideration of the impact from the new numerology.

  - Timing indication (K0/K1/K2); HARQ procedure with increased value of K0/K1/K2; PDCCH monitoring with practical PDCCH BD capability; Multi-PDSCH/PUSCH scheduling

- From [20]:

  - It would be beneficial in terms of UE implementation complexity or power consumption to perform slot(or symbol)-group level processing instead of every slot(or symbol) processing, e.g. PDCCH monitoring and CSI processing unit availability check.

- From [21]:

  - Study required UE processing time and switching time for larger subcarrier spacings to be introduced. Study enhanced processing time determination methods to reduce the redundant processing time.

- From [22]:

  - RAN1 to modify the UE timing parameter values and their associated signaling.

- From [25]:

  - For higher SCS, the appropriate configuration of k0, k1, k2 need to be discussed to meet UE minimum processing timeline.

  - If the current candidate values don’t meet UE processing limitation, extending, limiting or shifting the range of k0, k1, k2 may be necessary

## 2.13 Processing Timelines – CSI Specific

- From [1]:

  - For supporting NR beyond 52.6 GHz with existing waveforms in Rel. 17, if higher subcarrier spacings (numerologies) are adopted, then potential enhancements should be considered on how to efficiently utilize UE’s limited processing capability to reduce latency and efficiently handle processing/preparation of CSI reports associated with multiple numerologies parallelly.

- From [10]:

  - For CSI computation, Z1/Z2/Z3 value for µ larger than 3 should be defined with consideration of different CSI computation delay requirements.

## 2.14 MAC Buffering

- From [15]:

  - Very larger sub-carrier spacing will induce excessive MAC buffering requirements and causes higher UE implementation costs.

## 2.15 HARQ Processes

- From [15]:

  - Because of larger processing latencies, the numbers of DL and UL HARQ processes may need to be increased.

  - Otherwise, physical layer specification and implementation changes compared to Rel-15 may be needed to sustain high data throughput.

- From [22]:

  - RAN1 to modify the design of the HARQ feedback mechanism to accommodate timeline changes from the increased number of slots due to a possible increase in the SCS.

## 2.16 PDCCH Monitoring

- From [1]:

  - For supporting NR beyond 52.6 GHz with existing waveforms in Rel. 17, if higher subcarrier spacings (numerologies) are adopted, then the PDCCH monitoring capability would be further reduced and the number of PDCCH candidates per slot would be lower.

  - For supporting NR beyond 52.6 GHz with existing waveforms in Rel. 17, if higher subcarrier spacings (numerologies) are adopted, then the PDCCH processing in every slot might not be scalable with increasing subcarrier spacing, due to limitations with UE processing capability.

  - For supporting NR beyond 52.6 GHz with existing waveforms in Rel. 17, if higher subcarrier spacings (numerologies) are adopted, then enhancements to current PDCCH design including the possibility:

    - To introduce new DCI formats should be considered for reduced PDCCH monitoring and efficient scheduling for both UL and DL,

    - To limit the monitoring to specific DCI formats

- From [14]:

  - When a large subcarrier spacing is defined, maximum number of BDs/CCEs for PDCCH monitoring needs to be investigated.

- From [22]:

  - Study mechanisms to limit the increase in PDCCH monitoring complexity with any change in the SCS

- From [29]:

  - Increase of the minimum scheduling/ PDCCH monitoring unit to avoid excessive increase in PDCCH monitoring rate.

  - Support Multi-PDSCH DCI for reaching peak data-rates for the cases of high SCSs.

  - Determine BD/CCE limits based on nominal scheduling/monitoring unit such as slot of e.g. 120kHz (defined in R15)/240kHz (FFS).

## 2.17 Scheduling and DCI Formats

- From [14]:

  - When a large subcarrier spacing is defined, multi-TTI based scheduling can be considered to relax scheduler implementation and higher layer processing burden

- From [15]:

  - For 60GHz operation, reduce the FDRA fields size by supporting larger RBG sizes

  - Consider a gNB initiated polling approach for UL traffic management to reduce UL data latency

  - Consider support of scheduling multiple PDSCH using one DCI for NR operation in 52.6 to 71 GHz

- From [17]:

  - RAN1 shall study more flexible resource allocation in both time and frequency domain for different scenarios, including slot bundling, subcarrier bundling/sub-PRB.

- From [25]:

  - The current granularity in time/frequency domain in Rel-15/16 may be too fine, assuming less opportunity for FDM between UEs due to narrower beam width and larger number of symbols required for coverage performance.

  - How to allocate resource for data in frequency domain needs to be considered especially for higher SCS if introduced. PDSCH/PUSCH allocated on more than 14 symbols would be beneficial.

## 2.18 PUCCH

- From [15]:

  - PUCCH format 0/1/4 enhancements to compensate for the limited transmit power should be studied. Consider enhancements to SR (PUCCH) resource configuration and spatial relation management to reduce UL data latency

- From [26]:

  - In addition to the design issues discussed in RAN1 #101-e, discuss the design of PUCCH to achieve higher EIRP up to maximum allowed EIRP.

- From [29]:

  - Consider support for contiguous multi-PRB allocation for PUCCH format 0 and format 1 or use of PUCCH format 2 and format 3 for SR and before dedicated PUCCH configuration.

## 2.19 UL Interlace Transmission

- From [1]:

  - For supporting NR beyond 52.6 GHz in unlicensed band in Rel. 17, study the enhancement of PRB/sub-PRB interlacing designs for NR with higher SCS, if agreed to be supported.

- From [2]:

  - PRB based interlace resource mapping for PUSCH/PUCCH/SRS should be studied in NR-U-60.

  - More evaluation is required before introducing PRB-based interlacing in NR-U-60.

- From [5]:

  - Similar as NR-U in Rel-16, to maximize transmission power under regulation requirements, interlaced structure should be supported for the SCS and bandwidth of the unlicensed spectrum between 52.6 GHz and 71 GHz.

- From [12]:

  - RAN1 shall study high BW formats, up to 2.16 GHz, for NR-U PUCCH in 60 GHz band. RAN1 shall study the possibility to assign NR-U PUCCH onto partial interlaces for high BW channels.

- From [15]:

  - PRB-based interlacing is not beneficial for SCS ≥ 120 kHz. Sub-PRB interlacing is not beneficial for SCS ≥ 960 kHz.

  - Both PRB and sub-PRB interlacing is not beneficial for large frequency allocations.

  - The support of UL interlace allocation is not considered for operation in >52.6 GHz spectrum

  - To fulfil the OCB requirement specified in EN 302 567, for each of the declared channel bandwidths, the device has to support at least one mode of transmission where the transmission occupies at least 70% of the declared channel bandwidth.

  - Existing NR design fulfills the EN 302 567 OCB requirement

- From [20]:

  - Design wide-band PRACH and interlaced PUSCH/PUCCH considering regulatory requirements such as nominal channel BW, occupied channel BW, maximum allowed output power, and maximum power spectral density.

- From [25]:

  - In unlicensed band, interlaced PUCCH/PUSCH would be necessary.

- From [27]:

  - In order to meet the requirements of minimum OCB, some enhancement on interlace design with unregular RB number might be considered.

- From [29]:

  - No interlaced transmission is defined for 60 GHz unlicenced band.

## 2.20 Transmission Rank

- From [29]:

  - Consider supporting rank-2 SU-MIMO for DFT-s-OFDM.

## 2.21 Beam Switching

- From [25]:

  - sufficient time gap for beam switching between transmissions/receptions with different beam directions may be necessary in case of high SCS.

- From [29]:

  - Study the impacts of beam switching gap on NR physical layer design extended to higher SCSs. The following assumptions are taken when considering need for the explicit beam switching gap:

    - Max 100 ns assumed as beam switching time;

    - If the CP is longer than 100 ns, no explicit gap is needed for the beam switching

## 2.22 Beam Management

- From [2]:

  - Study the use of aperiodic CSI-RS for BFR procedure in NR-U-60.

- From [17]:

  - RAN1 shall study the beam adjustment mechanism in initial access procedure.

- From [20]:

  - Study potential enhancements for beam management CSI-RS or SRS considering beam switching time and coverage loss for large SCS.

- From [25]:

  - SSB beam may not be narrow enough considering large propagation loss. In order to improve the coverage performance of DL transmissions following SSB during initial access, beam refinement during initial access may be beneficial.

  - BFR procedure enhancement needs to be considered with at least following points:

    - The number of candidate beams included in set q1;

    - The minimum time gap to apply new beam configuration after receiving BFR response from gNB; Simultaneous update of beam configuration for multiple SCells;

    - Monitoring aperiodic transmissions for beam failure detection

- From [29]:

  - For P-TRS transmissions in the cell, it would be beneficial to have a mechanism to be able to transmit P-TRSs dropped due to LBT failure.

  - Applied coexistence mechanism(s) should be clarified before impact on beam management and the CSI measurement and reporting framework can be fully evaluated.

  - As the UE moves in a cell, the likelihood of blockage and beam mis-alignment increases with decreasing beamwidths used by the gNB.

  - Connectivity and robustness improvements are being developed for FR2 in the MIMO WID under multi-beam enhancements and multi-TRP agenda items, and those improvements are also expected to be valid solutions above 52.6 GHz operation.

## 2.23 Channelization

- From [7]:

  - When determining supported bandwidths for NR above 52.6 GHz, RAN1 should take co-existence of IEEE 802.11ad/ay into account at least in unlicensed band.

  - In licensed frequency band or in a controlled environment, it can be designed in a unified way with unlicensed band or independently.

  - 400 MHz (and/or its integral multiple e.g. 800/1600 MHz) and 2.16 GHz can be served as candidates of supported bandwidths for Rel-17 NR above 52.6 GHz.

- From [29]:

  - Support operation with CBW=2.16 GHz

## 2.24 Multi-Carrier Operations

- From [6]:

  - Silicon footprint for having large single FFT (using one CC) and multiple smaller FFT (using CA) could be compariable

- From [20]:

  - Consider carrier-group based operation for NR unlicensed band in frequency range above 52.6 GHz, with consideration of multi-RAT coexistence as well as control signalling efficiency.

- From [23]:

  - The signal overhead for scheduling large number of aggregated carriers should be studied for NR operation from 52.6 to 71 GHz.

- From [29]:

  - Support both channel bonding and CA between 2.16 GHz channels

- From [31]:

  - Study whether/how to utilize wide available spectrum such as more than 10 GHz bandwidth in 52.6-71GHz frequency range.

# Summary of Views on Numerology and Bandwidth

The following is a summary of views on bandwidth, subcarrier spacing (SCS), FFT sizes, CP length, and other related issues to numerology.

Table 1. Summary of views on bandwidth, subcarrier spacing (SCS), FFT size, CP length, and related issues to numerology

| Company Name | Bandwidth | SCS (for BWP) | FFT | CP | Notes |
| --- | --- | --- | --- | --- | --- |
| Lenovo, Motorola Mobility | 800 MHz (for 240 kHz) 1600 MHz (for 480 kHz) 3.2 GHz (for 960 kHz) | 240 kHz, 480 kHz, 960 kHz (for ~2 GHz) | Max 4096 | NCP: 240, 480, 960 kHz |  |
| Huawei, HiSilicon |  | 120, 240 kHz | Max 4096 | NCP: 120, 240 kHz [ECP: 480, 960 kHz] | SSB SCS: 120,240 kHz for licensed 240 kHz for unlicensed |
| Futurewei | 400 MHz | 480 kHz | Max 4096 |  | Larger BW achieved using CA |
| vivo |  | 120 kHz, 960 kHz |  | NCP: 120, 960 kHz |  |
| Fujitsu |  |  |  |  |  |
| Sony | 2.16 GHz (unlicensed) 400 MHz (licensed) | 240 or 480 or 960 kHz | 4096 (960 kHz) 8192 (480 kHz) 16384 (240 kHz) |  |  |
| ZTE, Sanechips | 400 x N MHz 2.16 GHz |  | Max 4096 |  | SSB SCS: 240 kHz |
| MediaTek |  |  |  |  |  |
| CATT | 1 GHz | 240 kHz | Max 4096 | NCP: 240 kHz |  |
| Xiaomi |  |  |  |  |  |
| NEC | > 1 GHz | 240 kHz |  |  |  |
| TCL | ≤ 2.16 GHz |  |  |  |  |
| Mitsubishi |  |  |  |  |  |
| Intel |  | 480, 960 kHz |  | NCP: 480, 960 kHz | ECP might be needed depending on MIMO TAE |
| Ericsson | ≤ 1.6 GHz | ≤ 480 kHz |  |  | SSB SCS: 120/240 kHz |
| OPPO |  | 960 kHz |  |  |  |
| Samsung | 400 MHz 21.6 GHz | 960 kHz (2.16 GHz) 120 kHz (400 MHz) | Max 4096 | 120 kHz (NCP) 960 kHz (NCP and/or ECP) |  |
| CMCC | ≤ 1.6 GHz | ≤ 480 kHz | Max 4096 |  |  |
| Spreadtrum |  |  |  |  |  |
| LG Electronics | 800 MHz (240kHz) 1.6 MHz and/or 800 MHz (480kHz) | 240, 480 kHz |  |  | SSB SCS: 120kHz, 240 kHz |
| InterDigital |  |  | Max 4096 Min 512 |  |  |
| Apple | 400 x N MHz | ≤ 480 kHz |  |  |  |
| Convida Wireless |  |  |  |  |  |
| Charter |  | 960 kHz |  |  |  |
| NTT DOCOMO | Max: > 400 MHz Min: > 50 MHz | > 120 kHz | Max 4096 | ECP: 960kHz (if supported) |  |
| Qualcomm | ≤ 400 MHz (120kHz) ≤ 21.6 GHz (960 kHz) | 120 kHz 960 kHz |  |  |  |
| CAICT |  | 240 kHz 480 kHz (FFS) |  |  |  |
| WILUS |  |  |  |  |  |
| Nokia, Nokia Shanghai Bell | 400 x N MHz (N=1,2,3) | 120kHz 960 kHz 1920 kHz (only for OFDM) |  | ECP: 960 kHz |  |

# Reference

- R1-2005239, “Discussion on potential physical layer impacts for NR beyond 52.6 GHz,” Lenovo, Motorola Mobility

- R1-2005241, “PHY design in 52.6-71 GHz using NR waveform,” Huawei, HiSilicon

- R1-2005280, “Considerations on phase noise for numerology selection,” FUTUREWEI

- R1-2005371, “Discussion on requried changes to NR using existing DL/UL NR waveform,” vivo

- R1-2005543, “Consideration on required changes to NR using existing NR waveform,” Fujitsu

- R1-2005567, “Considerations on bandwidth and subcarrier spacing for above 52.6 GHz,” Sony

- R1-2005607, “Discussion on the required changes to NR for above 52.6GHz,” ZTE, Sanechips

- R1-2006989, “On required changes to NR using existing DL/UL NR waveform for operation in 60GHz band,” MediaTek Inc.

- R1-2005699, “System Analysis of NR opration in 52.6 to 71 GHz,” CATT

- R1-2005734, “Physical layer design for NR 52.6-71GHz,” Beijing Xiaomi Software Tech

- R1-2005764, “Study on the required changes to NR using existing DL/UL NR waveform,” NEC

- R1-2005766, “Required changes to NR using existing DL/UL NR waveform,” TCL Communication Ltd.

- R1-2005787, “On phase noise compensation for NR from 52.6GHz to 71GHz,” Mitsubishi Electric RCE

- R1-2006986, “Discussion on Required Changes to NR in 52.6 – 71 GHz,” Intel Corporation

- R1-2005920, “On NR operations in 52.6 to 71 GHz,” Ericsson

- R1-2006026, “discusson on DL/UL NR waveform for 52.6GHz to 71GHz,” OPPO

- R1-2006136, “Design aspects for extending NR to up to 71 GHz,” Samsung

- R1-2006237, “Required changes to NR using existing DL/UL NR waveform in 52.6GHz ~ 71GHz,” CMCC

- R1-2006274, “Discussion on required changes to NR using existing NR waveform,” Spreadtrum Communications

- R1-2006304, “Consideration on required physical layer changes to support NR above 52.6 GHz,” LG Electronics

- R1-2006452, “Consideration on supporting above 52.6GHz in NR,” InterDigital, Inc.

- R1-2006512, “On Required changes to NR above 52.6 GHz using the existing DL/UL NR Waveform,” Apple

- R1-2006628, “On NR operation between 52.6 GHz and 71 GHz,” Convida Wireless

- R1-2006649, “60 GHz DL and UL waveform evaluations,” Charter Communications

- R1-2006725, “Evaluation Methodology and Required Changes on NR from 52.6 to 71 GHz,” NTT DOCOMO, INC.

- R1-2006797, “NR using existing DL-UL NR waveform to support operation between 52p6 GHz and 71 GHz,” Qualcomm Incorporated

- R1-2006853, “Discussions on required changes on supporting NR from 52.6GHz to 71 GHz,” CAICT

- R1-2006885, “Discussion on physical layer aspects for NR beyond 52.6GHz,” WILUS Inc.

- R1-2006907, “Required changes to NR using existing DL/UL NR waveform,” Nokia, Nokia Shanghai Bell

- R1-2006028, “discussion on other aspects,” OPPO

- R1-2006727, “Potential Enhancements for NR on 52.6 to 71 GHz,” NTT DOCOMO, INC.