3GPP TSG RAN WG1 Meeting #103-e	R1-2007581

E-meeting, Oct 26th – Nov 13th, 2020

Agenda Item:	8.8.1.1

Source:	Huawei, HiSilicon

Title:	Evaluation on the baseline performance for FR1

Document for:	Discussion and Decision

- Introduction

Agreements:

- Adopt single link budget template for both FR1 and FR2 based on IMT-2020 self-evaluation with rows for MIL, MCL, MPL, and necessary revisions, including adding/removing/revising/simplifying some parameters

  - [For LLS based methodology,] coverage bottleneck(s) identification is performed using at least [MCL and] MIL.

  - [MCL values can also be considered to compare channels with similar antenna (and antenna array) gain]

Agreements:

- MPL can be used as supplemental information for coverage bottleneck(s) identification

  - The results based on MPL are to be captured in TR

To identify coverage bottleneck channels, link budget calculation based on the IMT-2020 self-evaluation link budget template with necessary revisions is agreed in the RAN#102 e-meeting [1]. According to the agreed evaluation assumptions such as target data rates, channel model, moving speed, DMRS configuration, target BLER etc., key metrics such as MIL and MPL are used for coverage bottleneck identification.

In this contribution, we provide our evaluation observations and analysis on the baseline coverage performance for FR1, aiming to conclude the coverage bottleneck channels and propose coverage enhancement targets.

- Link budget results and analysis

As shown in table 2-1, several typical scenarios are considered with different carrier frequencies, channel models and ISD (Inter site distance) values.

Table 2-1. Simulation scenarios for coverage performance evaluation

| Urban (TDD @4GHz) | Scenario 1-1: NLoS O2I, ISD=400m |
| --- | --- |
|  | Scenario 1-2: NLoS O2I, ISD=500m |
| Urban (TDD @2.6GHz) | Scenario 1-3: NLoS O2I, ISD=400m |
|  | Scenario 1-4: NLoS O2I, ISD=500m |
| Rural (TDD @ 4GHz) | Scenario 2-1: NLoS O2I, ISD=1732m |
|  | Scenario 2-2: NLoS O2O, ISD=1732m |
| Rural (FDD @ 2GHz) | Scenario 3-1: NLoS O2I, ISD=1732m |
|  | Scenario 3-2: NLoS O2O, ISD=1732m |
| Rural with long distance (FDD @ 700MHz) | Scenario 4-1: LoS O2O, ISD=6 km |
|  | Scenario 4-2: LoS O2O, ISD=12 km |

Based on the link level simulation assumptions listed in Appendix A.1~3, the required SNRs for different channels are obtained and shown in Appendix A.4. In the following sections, the MIL and MPL based link level simulation results and identified coverage bottleneck channels are present.

    - MIL based link level simulation results

Agreements:

- For TDL Option 1

  - Definition of MIL

    - Total transmit power - Receiver sensitivity + gNB antenna gain (component 2 + 3 + 4) + UE antenna gain- Rx loss

According to the agreement of MIL definition in [1], we present the MIL values of both downlink and uplink channels under different scenario based on LLS results and link budget calculations.

- Scenario: Urban (TDD @ 4GHz & 2.6GHz)

Figure 2.1.1-1 MIL values of different channels under scenario 1-1~1-4

- Scenario: Rural (TDD @ 4GHz)

Figure 2.1.1-2 MIL values of different channels under scenario 2-1

Figure 2.1.1-3 MIL values of different channels under scenario 2-2

- Scenario: Rural (FDD @ 2GHz)

Figure 2.1.1-4 MIL values of different channels under scenario 3-1

Figure 2.1.1-5 MIL values of different channels under scenario 3-2

- Scenario: Rural with long distance (FDD @ 700MHz)

Figure 2.1.1-6 MIL values of different channels under scenario 4-1~4-2

In light of above results, the MIL values are summarized as follows:

Table 2.1.1-1. MIL based coverage performance of all channels

| MIL(dB) | PUSCH eMBB | PUSCH VoIP (0rep,0retrans,No FH) | PUSCH VoIP (4 rep, 3 retrans, No FH) | PUSCH VoIP (4 rep, 3 retrans, with FH) | Msg3 (No FH) | Msg3 (FH) | PUCCH format1 (2bits) | PUCCH Format3 (11bits) | PUCCH Format3 (22bits) | PDSCH (MCS0) | PDCCH (40bits) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario 1-1~1-4 | 142.38 | 147.51 | 155.61 | 156.26 | 155.92 | 156.22 | 159.76 | 157.36 | 155.46 | 163.56 | 163.71 |
| Scenario 2-1 | 146.57 | 142.74 | 150.84 | 151.49 | 151.15 | 151.45 | 154.99 | 152.59 | 150.69 | 158.29 | 158.94 |
| Scenario 2-2 | 147.04 | 142.49 | 153.89 | 153.99 | 150.75 | 150.85 | 154.99 | 152.49 | 150.69 | 158.64 | 158.84 |
| Scenario 3-1 | 145.34 | 140.49 | 148.34 | 148.94 | 149.25 | 149.55 | 152.79 | 150.29 | 148.44 | 159.52 | 162.02 |
| Scenario 3-2 | 145.99 | 140.59 | 151.99 | 152.14 | 149.00 | 149.10 | 152.74 | 150.24 | 148.19 | 159.87 | 161.92 |
| Scenario 4-1 & 4-2 | 145.28 | 141.23 | 149.28 | 149.28 | 148.69 | 148.89 | 149.23 | 147.23 | 144.43 | 154.26 | 161.11 |

As a smaller MIL value indicates a poorer coverage, based on the relative comparison of MIL values among different channels, we have the following observations:

Observation 1: For VoIP service without repetition, retransmission and frequency hopping, it has the smallest MIL value under almost all channels except urban scenario. By repetition, retransmission and frequency hopping, 8-11dB gain can be obtained. However, the aforementioned gain is under the ideal assumption of 20ms packet arrival period without considering effects of actual packet arrival delay and network jitter on VoIP service.

Observation 2: For eMBB service, PUSCH service has the smallest MIL values as compared to other uplink channels.

Observation 3: PUCCH has >3dB coverage gain over PUSCH with eMBB service under all scenarios.

Observation 4: Obvious MIL gap can be observed between uplink and downlink channels, e.g. the largest MIL unbalance between uplink and downlink can be 21dB for TDD and 22dB for FDD.

In light of above observations, we have following proposal:

Proposal 1: PUSCH is identified as the coverage bottleneck for FR1.

    - MPL based link level simulation results

According to the definition of MPL where parameters related to fading channel are included, we could deduce the available pathloss based on link budget calculations and find out the coverage bottleneck channel by comparing the Available pathloss and Target performance.

- Available pathloss means the maximum pathloss subject to achieving the target data rate in a given evaluation setting.

- Target performance means the pathloss corresponding to a cell radius that is the distance of 23* ISD from the base station for hexagonal cells.

As shown below, MPL based coverage performance based on link budget results for different scenarios are provided:

- Urban scenario (TDD @4GHz, ISD=400m & 500m)

- Rural scenario (TDD @4GHz, NLoS, ISD=1732m)

- Rural scenario (FDD @2GHz, NLoS, ISD=1732m)

- Rural scenario with long distance (FDD @700MHz, LoS, ISD=6km & 12km)

Over all, the MPL based coverage performance of all channels are summarized as follows:

Table 2.1.2-1. MPL based coverage performance of all channels

| Gap | PUSCH eMBB | PUSCH VoIP (0rep,0retrans,No FH) | PUSCH VoIP (4 rep, 3 retrans, No FH) | PUSCH VoIP (4 rep, 3 retrans, with FH) | Msg3 (No FH) | Msg3 (FH) | PUCCH format1 (2bits) | PUCCH Format3 (11bits) | PUCCH Format3 (22bits) | PDSCH (MCS0) | PDCCH (40bits) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario 1-1 (ISD=400m) | -8.75 | -3.62 | 4.48 | 5.13 | 4.79 | 5.09 | 5.55 | 3.15 | 1.25 | 12.43 | 9.50 |
| Scenario 1-2 (ISD=500m) | -12.53 | -7.40 | 0.70 | 1.35 | 1.01 | 1.31 | 1.77 | -0.63 | -2.53 | 8.65 | 5.72 |
| Scenario 1-3 (ISD=400m) | -5.02 | 0.11 | 8.21 | 8.86 | 8.52 | 8.82 | 9.28 | 6.88 | 4.98 | 16.16 | 13.23 |
| Scenario 1-4 (ISD=500m) | -8.80 | -3.67 | 4.43 | 5.08 | 4.74 | 5.04 | 5.50 | 3.10 | 1.20 | 12.38 | 9.45 |
| Scenario 2-1 (ISD=1732m) | -7.07 | -10.90 | -2.80 | -2.15 | -2.49 | -2.19 | -1.98 | -4.38 | -6.28 | 4.65 | 1.98 |
| Scenario 2-2 (ISD=1732m) | -2.55 | -7.10 | 4.30 | 4.40 | 1.16 | 1.26 | 1.55 | -0.95 | -2.75 | 9.05 | 5.41 |
| Scenario 3-1 (ISD=1732m) | -1.25 | -6.10 | 1.75 | 2.35 | 2.66 | 2.96 | 2.88 | 0.38 | -1.47 | 12.93 | 12.11 |
| Scenario 3-2 (ISD=1732m) | 2.42 | -2.98 | 8.42 | 8.57 | 5.43 | 5.53 | 5.33 | 2.83 | 0.78 | 16.30 | 14.51 |
| Scenario 4-1 (ISD=6km) | 14.04 | 9.99 | 18.04 | 18.04 | 17.45 | 17.65 | 16.78 | 14.78 | 11.98 | 23.02 | 28.66 |
| Scenario 4-2 (ISD=12km) | 2.00 | -2.05 | 6.00 | 6.00 | 5.41 | 5.61 | 4.74 | 2.74 | -0.06 | 10.98 | 16.62 |

Notes: Negative gap (Available pathloss - Target performance) refers to insufficient coverage, FH denotes frequency hopping.

Thus, we have following observations for coverage performance:

Observation 5: MPL-based coverage gaps show that coverage of PUSCH with eMBB services are limited under scenarios including urban (TDD, NLoS O2I, ISD=400 & 500m), rural (TDD, NLoS, ISD=1732m) scenarios.

Observation 6: MPL-based coverage gap show that the coverage of PUSCH with VoIP service with no repetition and no retransmission is limited under urban and rural scenarios, the coverage of PUSCH with VoIP service with 4 valid repetitions and 3 retransmission is limited under rural (TDD, NLoS O2I, ISD=1732m) scenario by assuming a physical packet size around 320bits and the packet arrival period of 20ms without considering packet arrival delay and network jitter for VoIP service.

Observation 7：MPL-based coverage gaps show that coverage of PUCCH format 1 with 2bits is only limited under rural (TDD, NLoS O2I, ISD=1732m) scenario, while coverage of PUCCH format 3 with 22 bits payload is limited under urban (TDD, NLoS O2I @4GHz, ISD=500m), rural (TDD, NLoS @4GHz, ISD=1732m) and rural (FDD, NLoS O2I @2GHz, ISD=1732m) scenarios.

Observation 8: MPL-based coverage gaps show that coverage of Msg3 with and without frequency hopping are limited under rural (TDD, NLoS O2I, ISD=1732m) scenario where the coverage of PUSCH is also limited.

Observation 9：MPL-based coverage gaps show that coverage of PDCCH with 40 bits payload can be achieved under all above mentioned scenarios.

Observation 10: MPL-based coverage gaps show that coverage of PDSCH can be achieved under all above mentioned scenarios.

In light of above observations, we have the following proposal:

Proposal 2: PUCCH format 3 with large payload size can be considered in coverage deficit for FR1.

- Conclusions

# In summary, the following observations and proposals are achieved,

Observation 1: For VoIP service without repetition, retransmission and frequency hopping, it has the smallest MIL value under almost all channels except urban scenario. By repetition, retransmission and frequency hopping, 8-11dB gain can be obtained. However, the aforementioned gain is under the ideal assumption of 20ms packet arrival period without considering effects of actual packet arrival delay and network jitter on VoIP service.

Observation 2: For eMBB service, PUSCH service has the smallest MIL values as compared to other uplink channels.

Observation 3: PUCCH has >3dB coverage gain over PUSCH with eMBB service under all scenarios.

Observation 4: Obvious MIL gap can be observed between uplink and downlink channels, e.g. the largest MIL unbalance between uplink and downlink can be 21dB for TDD and 22dB for FDD.

Observation 5: MPL-based coverage gaps show that coverage of PUSCH with eMBB services are limited under scenarios including urban (TDD, NLoS O2I, ISD=400 & 500m), rural (TDD, NLoS, ISD=1732m) scenarios.

Observation 6: MPL-based coverage gap show that the coverage of PUSCH with VoIP service with no repetition and no retransmission is limited under urban and rural scenarios, the coverage of PUSCH with VoIP service with 4 valid repetitions and 3 retransmission is limited under rural (TDD, NLoS O2I, ISD=1732m) scenario by assuming a physical packet size around 320bits and the packet arrival period of 20ms without considering packet arrival delay and network jitter for VoIP service.

Observation 7：MPL-based coverage gaps show that coverage of PUCCH format 1 with 2bits is only limited under rural (TDD, NLoS O2I, ISD=1732m) scenario, while coverage of PUCCH format 3 with 22 bits payload is limited under urban (TDD, NLoS O2I @4GHz, ISD=500m), rural (TDD, NLoS @4GHz, ISD=1732m) and rural (FDD, NLoS O2I @2GHz, ISD=1732m) scenarios.

Observation 8: MPL-based coverage gaps show that coverage of Msg3 with and without frequency hopping are limited under rural (TDD, NLoS O2I, ISD=1732m) scenario where the coverage of PUSCH is also limited.

Observation 9：MPL-based coverage gaps show that coverage of PDCCH with 40 bits payload can be achieved under all above mentioned scenarios.

Observation 10: MPL-based coverage gaps show that coverage of PDSCH can be achieved under all above mentioned scenarios.

Proposal 1: PUSCH is identified as the coverage bottleneck for FR1.

Proposal 2: PUCCH format 3 with large payload size can be considered in coverage deficit for FR1.

- References

[1] RAN1#102-e, “Chairman's Notes RAN1#102-e final”, August 17th – 28th, 2020.

[2] RP-182030, “Consideration on IMT-2020 link budget”, RAN#81, 10 - 13 September, 2018.

- Appendix

- Link level evaluation assumption for PUSCH

Table A.1-1. Simulation parameter setting for PUSCH eMBB

| scenario | Frequency carrier | Channel model | Delay spread | DMRS configuration | Moving speed | Scheduled PRB | MCS | TBS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario 1-1 & 1-2 | 4GHz | TDL-C | 300ns | 1 FL | 3km/h | 30 | MCS3 | 2280 |
| Scenario 2-1 | 4GHz | TDL-C | 300ns | 1 FL | 3km/h | 4 | MCS1 | 184 |
| Scenario 2-2 | 4GHz | TDL-C | 300ns | 1 FL+1 Add | 120 km/h | 4 | MCS1 | 176 |
| Scenario 3-1 | 2GHz | TDL-C | 300ns | 1 FL | 3km/h | 4 | MCS0 | 144 |
| Scenario 3-2 | 2GHz | TDL-C | 300ns | 1 FL+1 Add | 120 km/h | 4 | MCS0 | 128 |
| Scenario 4-1 & 4-2 | 700MHz | TDL-D | 30ns | 1 FL+1 Add | 120 km/h | 4 | MCS0 | 128 |
| Other parameters | - Metric: 10% BLER for eMBB - Antenna number: 1T4R - DFT-S-OFDM - No repetition, No retransmission - 14OS - TDD frame structure: DDDDSUDDSUU - 15KHz SCS for FDD; 30KHz SCS for TDD |  |  |  |  |  |  |  |

Table A.1-2. Simulation parameter setting for PUSCH VoIP

| scenario |  | Frequency carrier | Channel model | Delay spread | DMRS configuration | Moving speed | Scheduled PRB | MCS | TBS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario 1-1 & 1-2 |  | 4GHz | TDL-C | 300ns | 1 FL | 3km/h | 4 | MCS3 | 304 |
| Scenario 2-1 |  | 4GHz | TDL-C | 300ns | 1 FL | 3km/h | 4 | MCS3 | 304 |
| Scenario 2-2 |  | 4GHz | TDL-C | 300ns | 1 FL+1 Add | 120 km/h | 4 | MCS4 | 352 |
| Scenario 3-1 |  | 2GHz | TDL-C | 300ns | 1 FL | 3km/h | 4 | MCS3 | 304 |
| Scenario 3-2 |  | 2GHz | TDL-C | 300ns | 1 FL+1 Add | 120 km/h | 4 | MCS4 | 352 |
| Scenario 4-1 & 4-2 |  | 700MHz | TDL-D | 30ns | 1 FL+1 Add | 120 km/h | 4 | MCS4 | 352 |
| Other parameters | - Metric: 2% rBLER for VoIP - Antenna number: 1T4R - DFT-S-OFDM - (repetition number, retransmission number, frequency hopping) assumptions: - (1,1,no) , (4,3,no), (4,3, yes) - 14OS - TDD frame structure: DDDDSUDDSUU - 15KHz SCS for FDD; 30KHz SCS for TDD |  |  |  |  |  |  |  |  |

- Link level evaluation assumption for PUCCH

Table A.2-1. Simulation parameter setting for PUCCH format1

| scenario | Frequency carrier | Channel model | Delay spread | Scheduled PRB | Performance metric |
| --- | --- | --- | --- | --- | --- |
| Scenario 1-1 & 1-2 | 4GHz | TDL-C | 300ns | 1 | NACK to ACK 0.1% DTX to ACK 1% ACK missed 1% |
| Scenario 2-1 | 4GHz | TDL-C | 300ns | 1 |  |
| Scenario 2-2 | 4GHz | TDL-C | 300ns | 1 |  |
| Scenario 3-1 | 2GHz | TDL-C | 300ns | 1 |  |
| Scenario 3-2 | 2GHz | TDL-C | 300ns | 1 |  |
| Scenario 4-1 & 4-2 | 700MHz | TDL-D | 30ns | 1 |  |
| Other parameters | - TBS= 2 bits - Antenna number: 1T4R - DFT-S-OFDM - No repetitions, No retransmissions - 14OS scheduled - Frequency hopping: enabled |  |  |  |  |

Table A.2-2. Simulation parameters for PUCCH format3

| scenario | Frequency carrier | Channel model | Delay spread | Scheduled PRB | Performance metric |
| --- | --- | --- | --- | --- | --- |
| Scenario 1-1 & 1-2 | 4GHz | TDL-C | 300ns | 1 | BLER=1% |
| Scenario 2-1 | 4GHz | TDL-C | 300ns | 1 |  |
| Scenario 2-2 | 4GHz | TDL-C | 300ns | 1 |  |
| Scenario 3-1 | 2GHz | TDL-C | 300ns | 1 |  |
| Scenario 3-2 | 2GHz | TDL-C | 300ns | 1 |  |
| Scenario 4-1 & 4-2 | 700MHz | TDL-D | 30ns | 1 |  |
| Other parameters | - 22bits payload and 11bits CRC, Polar coding - 11bits payload and RM coding - Antenna number: 1T4R - DFT-S-OFDM - No repetitions, No retransmissions - 14OS scheduled - Frequency hopping: enabled |  |  |  |  |

- Link level evaluation assumption for other channels

Table A.3-1. Simulation parameter setting for Msg3

| scenario | Frequency carrier | Channel model | Delay spread | Moving speed | Without frequency hopping |  | With frequency hopping |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | DMRS configuration | TBS | DMRS configuration | TBS |  |
| Scenario 1-1 & 1-2 | 4GHz | TDL-C | 300ns | 3km/h | 1 FL+2 Add | 56bits | 1 FL+1 Add | 64bits |  |
| Scenario 2-1 | 4GHz | TDL-C | 300ns | 3km/h |  |  |  |  |  |
| Scenario 2-2 | 4GHz | TDL-C | 300ns | 120 km/h |  |  |  |  |  |
| Scenario 3-1 | 2GHz | TDL-C | 300ns | 3km/h |  |  |  |  |  |
| Scenario 3-2 | 2GHz | TDL-C | 300ns | 120 km/h |  |  |  |  |  |
| Scenario 4-1 & 4-2 | 700MHz | TDL-D | 30ns | 120 km/h |  |  |  |  |  |
| Other parameters | - Metric: 10% BLER - Scheduled PRB 2; - MCS 0 - Antenna number: 1T4R - DFT-S-OFDM - No repetition, No retransmission - 14OS |  |  |  |  |  |  |  |

Table A.3-2. Simulation parameter settings for PDSCH

| Scenario | Frequency carrier | Channel model | Delay spread | DMRS configuration | Moving speed | Scheduled PRB | TBS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario 1-1 & 1-2 | 4GHz | TDL-C | 300ns | 1 FL | 3 km/h | 268 | 8208 for D 6792 for S |
| Scenario 2-1 | 4GHz | TDL-C | 300ns | 1 FL | 3 km/h | 28 | 888 for D 704 for S |
| Scenario 2-2 | 4GHz | TDL-C | 300ns | 1 FL+1 Add | 120 km/h | 30 | 848 for D 672 for S |
| Scenario 3-1 | 2GHz | TDL-C | 300ns | 1 FL | 3 km/h | 38 | 1192 |
| Scenario 3-2 | 2GHz | TDL-C | 300ns | 1 FL+1 Add | 120 km/h | 40 | 1128 |
| Scenario 4-1 & 4-2 | 700MHz | TDL-D | 30ns | 1 FL+1 Add | 120 km/h | 40 | 1128 |
| Other parameters | - Metric: 10% BLER - Antenna number: 4T2R - CP-OFDM - No repetition, No retransmission, No frequency hopping - 12OS wideband precoding |  |  |  |  |  |  |

Table A.3-3. Simulation parameter setting for PDCCH

| Scenario | Frequency carrier | Channel model | Delay spread | Moving speed | DCI payload |
| --- | --- | --- | --- | --- | --- |
| Scenario 1-1 & 1-2 | 4GHz | TDL-C | 300ns | 3km/h | 64bits (including 24 bits CRC) AL=16 2 symbols 48 PRB |
| Scenario 2-1 | 4GHz | TDL-C | 300ns | 3km/h |  |
| Scenario 2-2 | 4GHz | TDL-C | 300ns | 120 km/h |  |
| Scenario 3-1 | 2GHz | TDL-C | 300ns | 3km/h |  |
| Scenario 3-2 | 2GHz | TDL-C | 300ns | 120 km/h |  |
| Scenario 4-1 & 4-2 | 700MHz | TDL-D | 30ns | 120 km/h |  |
| Other parameters | - Metric: 1% BLER - Antenna number: 4T2R - No repetition, No retransmission, No frequency hopping |  |  |  |  |  |

- On required SNR of different channels

As we can see from Table 3, the required SNR (19a/19b) of the target channel is one key factor for identifying coverage bottleneck. According to above mentioned method of parameter determination, we conduct link level simulations and required SNR towards different channels are provided as follows:

Table A.4-1. Required SNR of PUSCH and PUCCH towards different scenarios

| Required SNR (dB) | PUSCH (eMBB) | PUSCH (VoIP) |  |  | PUCCH format 1 (2bits) | PUCCH format 1 (11bits) | PUCCH format 3 (22bits) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | no rep, no trans | 4 rep, 3 trans, no FH | 4 rep, 3 trans, with FH |  |  |  |
| Scenario 1-1 & 1-2 | -4.62 | -1 | -9.1 | -9.75 | -10.3 | -7.9 | -6 |
| Scenario 2-1 | -4.83 | -1 | -9.1 | -9.75 | -10.3 | -7.9 | -6 |
| Scenario 2-2 | -5.30 | -0.75 | -12.15 | -12.25 | -10.3 | -7.8 | -6 |
| Scenario 3-1 | -5.6 | -0.85 | -8.7 | -9.3 | -10.1 | -7.6 | -5.75 |
| Scenario 3-2 | -6.25 | -0.7 | -12.10 | -12.25 | -10.05 | -7.55 | -5.5 |
| Scenario 4-1 & 4-2 | -8.3 | -4.25 | -12.30 | -12.30 | -9.3 | -7.3 | -4.5 |

Notes: FH denotes frequency hopping.

Table A.4-2. Required SNR of other channels including Msg3, PDCCH and PDSCH

| Required SNR (dB) | Msg3 (10% BLER, no FH) | Msg3 (10% BLER, FH) | PDCCH (1% BLER) | PDSCH (10% BLER) |
| --- | --- | --- | --- | --- |
| Scenario 1-1 & 1-2 | -6.4 | -6.7 | -9.8 | -4.5 |
| Scenario 2-1 | -6.4 | -6.7 | -9.8 | -4 |
| Scenario 2-2 | -6 | -6.1 | -9.7 | -4.35 |
| Scenario 3-1 | -6.5 | -6.8 | -9.9 | -4.25 |
| Scenario 3-2 | -6.25 | -6.35 | -9.8 | -4.6 |
| Scenario 4-1 & 4-2 | -8.7 | -8.9 | -9.0 | -1.75 |

- On antenna array gain modelling

Agreements:

Further clarify the agreement on antenna gain and antenna gain components including antenna gain correction factors as follows:

- For both TDL option 1 (table A below) and TDL option 2 & CDL (table B below)

  - The gain of antenna gain component 1 is included in LLS results

  - The gain of antenna gain component 2 is included in link budget template

    - The gain is expressed by 10 * log 10( N/k ) - 1

    - For TDL option 2 & CDL, the gain is 0 dB

  - The gain of antenna gain component 3 is included in link budget template

  - The gain of antenna gain component 4 is included in link budget template

    - The gain of antenna gain components 3 and 4 is expressed by Antenna Element Gain + 10 * log 10( M/N ) -2

    - For Tx, One row is used represent the gain of antenna gain component 3 + 4, i.e. row No. (4)

    - For Rx, One row is used represent the gain of antenna gain component 3 + 4, i.e. row No. (11)

    - Note: more appropriate name or explanation will be added to row No.(4) and (11). Details can be discussed when the link budget template is updated.

Given the limited time to evaluate the coverage of numbers of scenarios and channels, the following agreement is reached to reduce the simulation workload and obtain a more accurate antenna array gain estimation:

where the non-idealities of mapping component 2 to component 1 (denoted as 1) and mapping component 3 to component 2 (denoted as 2) can be reported based on observations. In link budget calculation, the number of receiver chain in LLS, number of TxRU and number of antenna elements under various scenarios for BS are listed as follows:

Table A.5-1: antenna array gain loss for unicast channel

| Scenarios | # receive chains in LLS (component 1) | # TxRU (component 2) | 1 (dB) | # antenna element (component 3) | 2 (dB) |
| --- | --- | --- | --- | --- | --- |
| Urban (TDD @4GHz) | 4 | 64 | 1.25 | 192 | 0 |
| Rural (TDD @4GHz) | 4 | 64 | 1.25 | 64 | 0 |
| Rural (FDD @2GHz) | 4 | 32 | 0.25 | 32 | 0 |
| Rural (FDD @700MHz) | 4 | 4 | 0 | 16 | 0 |

For broadcast channel such as PDCCH, the switching beams is wider than the ideal digital beamforming which cause large beamforming gain loss. By SLS method, the beamforming gain of broadcast channel can be modeled more accurately and the beamforming gain loss of broadcast channel as compared to ideal antenna array gain are presented as follows:

Table A.5-2: antenna array gain loss for broadcast channel

| Scenarios | # receive chains in LLS (component 1) | # TxRU (component 2) | 1 (dB) | # antenna element (component 3) | 2 (dB) |
| --- | --- | --- | --- | --- | --- |
| Urban (TDD @4GHz) | 4 | 64 | 6.4 | 192 | 0 |
| Rural (TDD @4GHz) | 4 | 64 | 6.4 | 64 | 0 |
| Rural (FDD @2GHz) | 4 | 32 | 3.4 | 32 | 0 |
| Rural (FDD @700MHz) | 4 | 4 | 0.4 | 16 | 0 |

- On shadow fading and penetration margins

Table A.6-1: values of shadow fading margin and penetration margin

| Parameters | Urban TDD | Rural NLoS TDD | Rural NLoS FDD | Rural with long distance FDD |
| --- | --- | --- | --- | --- |
| (25a) Shadow fading margin for control channel (function of the cell area reliability and (24)) (dB) | 7.56 | 10.45(O2O) 8.45(O2I) | 10.45(O2O) 8.45(O2I) | 6 |
| (25b) Shadow fading margin for data channel (function of the cell area reliability and (24)) (dB) | 4.48 | 6.61(O2O) 5.13 (O2I) | 6.61(O2O) 5.13 (O2I) | 4.79 |
| (27) Penetration margin (dB) | 26.25 | 9(O2O) 14.53 (O2I) | 9(O2O) 13.5 (O2I) | 9 |