3GPP TSG RAN WG1 #103e		R1-2007654

e-Meeting, October 26th – November 13th, 2020

Source:	vivo

Title:	Evaluation on different numerologies for NR using existing DL/UL NR waveform

Agenda Item:	8.2.3

Document for:	Discussion and Decision

- Introduction

The following objective is included in this study item [1]:

- Study of required changes to NR using existing DL/UL NR waveform to support operation between 52.6 GHz and 71 GHz

  - Study of applicable numerology including subcarrier spacing, channel BW (including maximum BW), and their impact to FR2 physical layer design to support system functionality considering practical RF impairments [RAN1, RAN4].

- Identify potential critical problems to physical signal/channels, if any [RAN1].

To achieve the above objective, various link level evaluations for different channels using existing DL/UL NR waveform are conducted. In the past RAN1 #102-E meeting, a set of evaluation assumptions and simulation parameters is agreed.  According to the agreements and conclusions, the link simulations are implemented. Some results are provided together with the discussion on applicable numerology in our companion contribution [2]. In this paper, more detailed evaluation results for PDSCH, PUSCH, SSB and PRACH are provided.

- PDSCH evaluation

As a common understanding, cyclic prefix (CP) plays an important role in NR operation in high frequency band, especially for the system coverage. Whether the CP can eliminate the effect of delay spread (DS) depends on the length of CP which is determined by SCS. To evaluate the impacts of SCS, the BLER performance is evaluated for four cases: 5ns DS, 10ns DS,20ns DS and 40ns. The evaluation assumptions are summarized in Table 8 of the Appendix. The evaluation was conducted with two bandwidths: 400 MHz and 2 GHz bandwidth.

  - 400MHz bandwidth

Firstly, the 400M bandwidth case was evaluated for TDL-A channel. The simulation results were shown below:

Figure 1: 400MHz, 5ns DS of different SCS (left: NCP, right: ECP)

Figure 2: 400MHz, 10ns DS of different SCS (left: NCP, right: ECP)

Figure 3: 400MHz, 20ns DS of different SCS (left: NCP, right: ECP)

Figure 4: 400MHz, 40ns DS of different SCS (left: NCP, right: ECP)

In the above figures, the blue line, red line, green line and yellow line represent the SCS=120 KHz, SCS=240 KHz, SCS=480 KHz and SCS=960 KHz respectively. The circle, asterisk and square represent the QPSK modulation, 16QAM modulation and 64QAM modulation respectively. Figure 1, Figure 2, Figure 3 and Figure 4 represent the 5ns DS, 10ns DS, 20ns DS and 40ns respectively.

The following table gives the detailed SINR values for 10%/1% BLER at 400MHz bandwidth.

Table 1: SINR in dB achieving PDSCH BLER of 10% /1% for 400MHz bandwidth

| MCS | Channel | NCP |  |  |  | ECP |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 120KHz | 240KHz | 480KHz | 960KHz | 120KHz | 240KHz | 480KHz | 960KHz |
| 7 | TDL-A, 5ns | 1.8/ 3.6 | 1.8/ 3.6 | 1.8/ 3.8 | 2.2/ 4.2 | 1.9/ 3.7 | 1.9/ 3.7 | 1.9/ 3.9 | 2.4/ 4.4 |
|  | TDL-A, 10ns | 1.7/ 2.9 | 1.7/ 2.9 | 1.8/ 3.1 | 2.9/ 4.3 | 1.7/ 3.1 | 1.7/ 3.1 | 1.9/ 3.4 | 3.1/ 4.5 |
|  | TDL-A, 20ns | 1.4/ 2.8 | 1.4/ 2.8 | 2.4/ 3.7 | 4.2/ 5.2 | 1.6/ 2.9 | 1.7/ 2.9 | 2.6/ 3.7 | 4.3/ 5.4 |
|  | TDL-A, 40ns | 1.5/ 2.6 | 2.4/ 3.4 | 3.8/ 4.9 | 6.9/ 7.8 | 1.6/ 2.8 | 2.5/ 3.5 | 4/ 4.9 | 7/ 8 |
| 16 | TDL-A, 5ns | 10.3/ 12.2 | 10/ 11.7 | 9.9/ 11.7 | 10/ 11.7 | 10.1/ 12.2 | 9.9/ 11.6 | 9.8/ 11.6 | 9.9/ 11.6 |
|  | TDL-A, 10ns | 10.1/ 11.5 | 9.7/ 11.2 | 9.7/ 11.2 | 10.5/ 11.9 | 10/ 11.5 | 9.6/ 11.1 | 9.5/ 10.9 | 10.3/ 11.7 |
|  | TDL-A, 20ns | 10/ 11.4 | 9.8/ 10.9 | 10.5/ 11.6 | 11.6/ 12.9 | 9.7/ 11.3 | 9.6/ 10.8 | 10.4/ 11.3 | 11.4/ 12.8 |
|  | TDL-A, 40ns | 10/ 11.3 | 10.6/ 11.7 | 11.5/ 12.5 | 14.4/ 16.6 | 9.9/ 11.2 | 10.5/ 11.5 | 11.3/ 12.2 | 13.8/ 15.9 |
| 22 | TDL-A, 5ns |  | 19.8/ | 16/ 18.8 | 15.1/ 17.1 | 23.5/ | 18.5/ | 15.9/ 18.5 | 15/ 17 |
|  | TDL-A, 10ns |  | 19.9/ | 16/ 18.5 | 15.9 17.2 | 24.5/ | 18.9/ | 15.8/ 18.2 | 15.7/ 17.1 |
|  | TDL-A, 20ns |  | 20/ | 16.8/ 19.1 | 17.1/ 18.8 | 25/ | 19/ | 16.6/ 19 | 16.7/ 18.1 |
|  | TDL-A, 40ns |  | 21/ | 18.2/ 21 |  | 26/ | 19.9/ | 17.8/ 20.3 |  |
| Additional report/notes: - NCP and ECP - CP-OFDM waveform for PDSCH - PTRS configuration: one tone per 2 RB; occupy symbol index: 2-13 - DMRS symbol index: 2 - Precoder: random precoder |  |  |  |  |  |  |  |  |  |

In order to evaluate the impact of different SCS, it is necessary to calculate the channel delay and the CP range of different SCS. The maximum delay path granularity for TDL-A channel is 9.6*10-9. Therefore, the maximum delay path for 5ns, 10ns，20ns DS and 40ns DS can be calculated as 5*9.6*10-9=48*10-9, 10*9.6*10-9=96*10-9,  20*9.6*10-9=192*10-9 and 40*9.6*10-9=384*10-9. On the other hand, CP of different SCS can be calculated as follows:

Table 2: The length of NCP and ECP for different SCS

| SCS (kHz) | Symbol length | NCP length（6.57%） | ECP length（20%） |
| --- | --- | --- | --- |
| 120 | 8.33 us | 585.94 ns | 2083.3ns |
| 240 | 4.17 us | 292.97 ns | 1041.7ns |
| 480 | 2.08 us | 146.48 ns | 520.8ns |
| 960 | 1.04 us | 73.24 ns | 260.4ns |

We can observe that, for 5ns DS, all the CP of different SCS can cover the maximum delay path; for 10ns DS, only SCS 960KHz of NCP case exceeds a little bit of CP length; for 20ns DS, SCS 480KHz and SCS 960KHz of NCP cases exceed NCP length; for 40ns DS, only SCS 120KHz of NCP case covers the DS.

Figure 5: DS distribution in typical indoor scenes

Meanwhile, we analyse the DS distribution in typical indoor scenes by using the SLS.  From Figure 5, we can see that the DS of almost 80% users are less than 30ns.

Some remarkable phenomenon can be seen from Figure 1, Figure 2, Figure 3, Figure 4 and Table 1:

- Under the same DS, the performance of NCP is basically the same as ECP. This is because the weak power of the paths which exceed the CP length;

- For high order MCS (e.g., 64QAM), the SCS 120 KHz and SCS 240KHz show poor performance which is due to the ICI effect;

- For low order and medium MCS (QPSK and 16QAM), different SCSs show similar performance with 5ns DS; the performance of SCS 960KHz decreases with the increases of the DS which is due to the CP length;

- For high order MCS (e.g., 64QAM), SCS 960 KHz has the best performance with 5ns, 10ns and 20ns DS; the performance degrades at 40ns DS.

Observation 1: Under the same DS, the performance of NCP is basically the same as ECP for CP-OFDM waveform.

Observation 2: High order modulation is more sensitive to phase noise impact. Higher SCS benefits more with phase noise compensation than lower SCS, especially for high order modulation such as 64QAM.

Observation 3: For CP-OFDM waveform, SCS 960 KHz is the most affected because of its shortest CP coverage when the DS is increased to 40 ns.

  - 2GHz bandwidth

Then, the 2GHz bandwidth case was evaluated. The simulation results were shown below:

Figure 6: 2G, 5ns DS of different SCS (left: NCP, right: ECP)

Figure 7: 2G, 10ns DS of different SCS (left: NCP, right: ECP)

Figure 8: 2G, 20ns DS of different SCS (left: NCP, right: ECP)

Figure 9: 2G, 40ns DS of different SCS (left: NCP, right: ECP)

In the above figures, the blue line and red line represent the SCS=480 KHz and SCS=960 KHz respectively. The circle, asterisk and square represent the QPSK modulation, 16QAM modulation and 64QAM modulation respectively. Figure 6, Figure 7, Figure 8 and Figure 9 represent the 5ns DS, 10ns DS, 20ns DS and 40ns DS respectively.

The following table gives the detailed SINR values for 10%/1% BLER at 2GHz bandwidth.

Table 3: SINR in dB achieving PDSCH BLER of 10% /1% for 2GHz bandwidth

| MCS | Channel | NCP |  | ECP |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 480KHz | 960KHz | 480KHz | 960KHz |
| 7 | TDL-A, 5ns | 1.4/ 2.6 | 1.5/ 2.8 | 1.2/ 2.5 | 1.4/ 2.6 |
|  | TDL-A, 10ns | 1.6/ 2.7 | 2.4/ 3.6 | 1.4/ 2.5 | 2.3/ 3.5 |
|  | TDL-A, 20ns | 2.3/ 3.3 | 3.6/ 4.4 | 2.2/ 3.1 | 3.5/ 4.6 |
|  | TDL-A, 40ns | 3.8/ 4.4 | 6.5/ 7.4 | 3.6/ 4.4 | 6.2/ 7 |
| 16 | TDL-A, 5ns | 9.5/ 10.5 | 9.4/ 10.4 | 9.4/ 10.5 | 9.3/ 10.3 |
|  | TDL-A, 10ns | 9.4/ 10.5 | 10.3/ 11.2 | 9.5/ 11.6 | 10.3/ 12.2 |
|  | TDL-A, 20ns | 10.2/ 11.3 | 11.1/ 11.8 | 10.2/ 11.3 | 11.1/ 11.8 |
|  | TDL-A, 40ns | 11.4/ 12.4 | 13.7/ 14.9 | 11.3/ 12.3 | 13.5/ 14.5 |
| 22 | TDL-A, 5ns | 19.8/ | 15.2/ 17.3 | 15/ 16.8 | 18.5/ |
|  | TDL-A, 10ns | 20/ | 16.4/ 18.2 | 18.6/ | 16/ 17.8 |
|  | TDL-A, 20ns | 21/ 12.3 | 17.5/ 20 | 19.6/ | 17/ 19 |
|  | TDL-A, 40ns | 23.5/ |  | 21.5/ |  |
| Additional report/notes: - NCP and ECP - CP-OFDM waveform for PDSCH - PTRS configuration: one tone per 2 RB; occupy symbol index: 2-13 - DMRS symbol index: 2 - Precoder: random precoder |  |  |  |  |  |

We can observe that:

- For high order MCS (64QAM), the BLER performance of SCS 480KHz with 2GHz bandwidth is worse than that of 400MHz. This is mainly due to the larger ICI under 2GHz bandwidth. While the performance of SCS 960 KHz is maintained in both 400 MHz and  2 GHz bandwidth.

- With the increase of DS, both the BLER performance of SCS 480KHz and SCS 960KHz will degrade, but the changes for SCS 960KHz are more obvious, especially for DS=40ns.

Observation 4: The greater the bandwidth, the greater the number of RBs, resulting in more ICI impact.

Observation 5: For CP-OFDM, the larger SCS is more sensitive to DS.

- PUSCH evaluation

For PUSCH channel of CP waveform, the impact of PN is consistent with that of PDSCH channel. Therefore, the DFT-S-OFDM wave form is mainly studied in this chapter. The evaluation assumptions are summarized in Table 9 of the Appendix

  - 400MHz bandwidth

Firstly, we provide 400MHz simulation results:

Figure 10: 400MHz, 5ns DS of different SCS (left: NCP, right: ECP)

Figure 11: 400MHz, 10ns DS of different SCS (left: NCP, right: ECP)

Figure 12: 400MHz, 20ns DS of different SCS (left: NCP, right: ECP)

Figure 13: 400MHz, 40ns DS of different SCS (left: NCP, right: ECP)

In the Figure 10, Figure 11, Figure 12 and Figure 13, in order to maintain the same PT-RS signal overhead under various SCS, the chunk set of SCS=120 KHz, 240 KHz, 480 KHz and 960 KHz is chunk=5, chunk=4, chunk=3 and chunk=1 respectively.

The following table gives the detailed SINR values for 10%/1% BLER at 400MHz bandwidth.

Table 4: SINR in dB achieving DFT-S-OFDM PUSCH BLER of 10% /1% for 400MHz bandwidth

| MCS | Channel | NCP |  |  |  | ECP |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 120KHz | 240KHz | 480KHz | 960KHz | 120KHz | 240KHz | 480KHz | 960KHz |
| 7 | TDL-A, 5ns | 5.5/ 8.9 | 5.5/ 8.5 | 6.4/ 9.8 | 6.5/ 9.9 | 5.1/ 8.6 | 5.2/ 8.3 | 6.1/ 9.1 | 6.6/ 9.8 |
|  | TDL-A, 10ns | 5.1/ 7.8 | 5.1/ 7.8 | 6.1/ 8.6 | 6.2/ 9 | 5/ 7.5 | 5/ 7.4 | 5.9/ 8.3 | 6.1/ 9 |
|  | TDL-A, 20ns | 5/ 7 | 5/ 7 | 6/ 8.4 | 6.3/ 8.8 | 4.8/ 6.9 | 4.8/ 6.8 | 5.6/ 7.9 | 6.5/ 8.8 |
|  | TDL-A, 40ns | 5/ 6.6 | 5/ 6.6 | 6.2/ 8.1 | 7.5/ 10.1 | 4.8/ 6.5 | 4.8/ 6.5 | 5.9/ 7.7 | 7.3/ 9.9 |
| 16 | TDL-A, 5ns | 12.2/ 15.7 | 12.2/ 15.2 | 12.3/ 15.3 | 12.5/ 15.7 | 12/ 15.4 | 12/ 15.2 | 12.1/ 15.2 | 12.3/ 15.6 |
|  | TDL-A, 10ns | 11.8/ 14.5 | 11.8/ 14.5 | 11.9/ 14.4 | 12.3/ 15 | 11.7/ 14.4 | 11.7/ 14.4 | 11.9/ 14.4 | 12.3/ 14.9 |
|  | TDL-A, 20ns | 11.7/ 14.1 | 11.8/ 14.1 | 11.9/ 14.2 | 13.8/ 15 | 11.6/ 13.8 | 11.7/ 13.9 | 11.9/ 14 | 12.7/ 14.9 |
|  | TDL-A, 40ns | 11.8/ 13.8 | 12/ 13.8 | 12.5/ 14.4 | 17/ | 11.5/ 13.5 | 11.8/ 13.6 | 12.3/ 14.3 | 16.8/ 25 |
| 22 | TDL-A, 5ns | 18.7/ 25 | 18.2/ 22 | 17.4/ 20.5 | 17.4/ 20.7 | 18.4/ 24 | 18/ 21.7 | 17.3/ 20.4 | 17.4/ 20.6 |
|  | TDL-A, 10ns | 18.5/ 23 | 18.3/ 21.7 | 17/ 19.7 | 17.5/ 20 | 18/ 22.5 | 18/ 21.3 | 17/ 19.6 | 17.3/ 20 |
|  | TDL-A, 20ns | 19.5/ | 19.4/ | 17.2/ 19.5 | 18.5/ 21.5 | 19/ 25 | 19/ | 17.1/ 19.4 | 18.1/ 21 |
|  | TDL-A, 40ns | 20.4/ | 20.6/ | 18.4/ 21.1 |  | 19.5/ | 20.2/ | 18/ 20.5 |  |
| Additional report/notes: - NCP and ECP - DFT-S-OFDM waveform for PUSCH - PTRS configuration: - 400MHz：SCS120:chunk5,SCS240:chunk4,SCS480:chunk3, SCS960:chunk1； (chunk1:2*2, chunk2:2*4, chunk3:4*2, chunk4:4*4, chunk5:8*4); occupy symbol index: 2-13 - Precoder: random precoder |  |  |  |  |  |  |  |  |  |

We can observe that:

- For 400MHz bandwidth, compared with CP-OFDM waveform, the PN compensation of DFT-S-OFDM waveform can eliminate the influence of PN more effectively, especially for the small SCS (120KHz and 240KHz).

- For low order MCS (QPSK), the higher SCS, the worse performance, which is mainly due to the less number of PTRS.

- With the increase of DS, the performance of higher SCS will slightly deteriorate (except the SCS 960KHz with 40ns DS).

- Under the same DS, the performance of NCP and ECP is nearly the same.

Observation 6: For 400MHz bandwidth, compared with CP-OFDM waveform, the PN compensation of DFT-S-OFDM waveform can eliminate the influence of PN more effectively, especially for the small SCS (120KHz and 240KHz).

Observation 7: For DFT-S-OFDM waveform, under the same PTRS overhead, the more PTRS number, the better the BLER performance.

Observation 8: For DFT-S-OFDM waveform, with the increase of DS (≤20ns), the performance of higher SCS will slightly deteriorate. When the DS increase to 40ns, the SCS 960 has significant performance loss.

  - 2GHz bandwidth

Then, the 2GHz bandwidth case was researched.

Figure 14: 2G, 5ns DS of different SCS (left: NCP, right: ECP)

Figure 15: 2G, 10ns DS of different SCS (left: NCP, right: ECP)

Figure 16: 2G, 20ns DS of different SCS (left: NCP, right: ECP)

Figure 17: 2G, 40ns DS of different SCS (left: NCP, right: ECP)

The following table gives the detailed SINR values for 10%/1% BLER at 2GHz bandwidth.

Table 5: SINR in dB achieving DFT-S-OFDM PUSCH BLER of 10% /1% for 2GHz bandwidth

| MCS | Channel | NCP |  | ECP |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 480KHz | 960KHz | 480KHz | 960KHz |
| 7 | TDL-A, 5ns | 4.8/ 6.7 | 4.8/ 6.7 | 4.7/ 6.8 | 4.7/ 6.8 |
|  | TDL-A, 10ns | 4.8/ 6.4 | 4.8/ 6.4 | 4.8/ 6.4 | 4.8/ 6.5 |
|  | TDL-A, 20ns | 4.5/ 5.9 | 4.6/ 6.2 | 4.4/ 6 | 4.5/ 6.1 |
|  | TDL-A, 40ns | 4.9/ 6.3 | 5.4/ 7.2 | 4.8/ 6.2 | 5.3/ 7 |
| 16 | TDL-A, 5ns | 11.4/ 13.5 | 11.5/ 13.4 | 11.3/ 13.5 | 11.3/ 13.5 |
|  | TDL-A, 10ns | 11.4 13.2 | 11.5/ 13.3 | 11.4/ 13 | 11.5/ 13.2 |
|  | TDL-A, 20ns | 11.3/ 12.8 | 11.9/ 13.4 | 11.2/ 12.7 | 11.6/ 13.3 |
|  | TDL-A, 40ns | 11.9/ 13.4 | 15.5/ 22 | 11.7/ 13.1 | 14.1/ 17.5 |
| 22 | TDL-A, 5ns | 18.2/ 24.4 | 17.3/ 20 | 18.3/ 25 | 17.5/ 20.3 |
|  | TDL-A, 10ns | 18.3/ 25 | 17.8/ 20.3 | 18.3/ 25 | 18/ 20.9 |
|  | TDL-A, 20ns | 18.2/ 25 | 19/ 22.7 | 18.2/ 25 | 18.5/ 22.3 |
|  | TDL-A, 40ns | 20/ 30 |  | 19/ 25 |  |
| Additional report/notes: - NCP and ECP - DFT-S-OFDM waveform for PUSCH - PTRS configuration: SCS480: chunk5, SCS960: chunk4. (chunk1:2*2, chunk2:2*4, chunk3:4*2, chunk4:4*4, chunk5:8*4); occupy symbol index: 2-13 - Precoder: random precoder |  |  |  |  |  |

In Figure 14, Figure 15, Figure 16 and Figure 17, it is clearly observed that:

- For low order and medium order MCS (QPSK and 16QAM), the SCS 480 KHz and SCS 960 KHz show similar performance under different DS.

- For high order MCS (64QAM), the performance of SCS 960KHz is better than SCS 480 KHz when DS = 5ns and 10ns; the performance of those two SCS is close when DS=20ns, which is mainly due to the effect of DS; the performance of SCS 960KHz becomes poor when DS=40ns.

Observation 9: For DFT-S-OFDM waveform, larger bandwidth (more RB numbers) is more sensitive to PN impact.

- SSB evaluation

In this section, the evaluation results are provided with the evaluation assumptions summarized in Table 12 of the Appendix. For SSB channel, we first perform the coarse and fine frequency offset compensation to identify CELL ID through the detection of PSS and SSS. Then by using the compensated value of the frequency offset to decode the PBCH. In this case, it can be seen that as long as the CELL ID is detected incorrectly, PBCH decoding will not be correct.

  - SSB detection error probability

Figure 18: SSB, 5ns DS of different SCS (left: NCP, right: ECP)

Figure 19: SSB, 10ns DS of different SCS (left: NCP, right: ECP)

Figure 20: SSB, 20ns DS of different SCS (left: NCP, right: ECP)

Figure 21: SSB, 40ns DS of different SCS (left: NCP, right: ECP)

From Figure 18, Figure 19, Figure 20 and Figure 21, we can see that:

- Under the NCP case for DS=5ns, the higher SCS, the better SSB detection performance, which means the larger bandwidth introduces some gain.

- In many cases, the performance of ECP is worse than that of NCP. It is mainly because when doing correlation detection in the time domain, combined with the influence of noise and channel, an extended CP may produce the highest correlation value in its range, resulting in detection errors.

- With the increases of the DS, the performance of SCS 960KHz decreases which is because the CP length cannot cover the DS.

Observation 10: For SSB detection, when the coarse frequency offset searcher range equals to the fine frequency offset compensation value, higher SCS (larger bandwidth) shows better performance.

Observation 11: For SSB channel, CP length and SCS will affect the autocorrelation of PSS sequence, thereby affecting the SSB detection performance.

  - Cell ID detection error probability

Figure 22: Cell ID, 5ns DS of different SCS (left: NCP, right: ECP)

Figure 23: Cell ID, 10ns DS of different SCS (left: NCP, right: ECP)

Figure 24: Cell ID, 20ns DS of different SCS (left: NCP, right: ECP)

Figure 25: Cell ID, 40ns DS of different SCS (left: NCP, right: ECP)

Table 6: SINR in dB achieving cell ID detection probability of 90% by one-shot detection from PSS/SSS

| Channel | NCP |  |  |  | ECP |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 120KHz | 240KHz | 480KHz | 960KHz | 120KHz | 240KHz | 480KHz | 960KHz |
| TDL-A, 5ns | -5.5 | -5.8 | -5.8 | -6.2 | -5.6 | -5.9 | -5.9 | -6.3 |
| TDL-A, 10ns | -5.6 | -5.9 | -5.9 | -5.8 | -5.7 | -5.9 | -5.9 | -5.9 |
| TDL-A, 20ns | -5.6 | -6 | -6 | -5.8 | -5.6 | -5.8 | -5.8 | -5.8 |
| TDL-A, 40ns | -5.4 | -5.4 | -5.4 | -5.8 | -5.5 | -5.4 | -5.6 | -5.6 |
| Additional report/notes: - frequency offset：600k - the number and granularity of the frequency locations: SCS120:22, SCS240:11, SCS480:6, SCS960:3 |  |  |  |  |  |  |  |  |

From above figures, we can see that:

- For DS=5ns case, the SCS 960KHz performs much better than others.

- With the increase of SCS, there is little difference in performance between different SCS.

Observation 12: For Cell ID detection, when the coarse frequency offset searcher range equals to the fine frequency offset compensation value, various SCS and DS have little effect on the cell ID BLER performance.

- PRACH evaluation

In this section, we present the performance of PRACH channel for different SCS. The evaluation results are provided with the evaluation assumptions summarized in Table 13.

In addition to the simulation assumptions in Table 11, the Ncs is calculated based on the equation Ncs=2rc+τ×LRATseq×Ts+ng .Where r is the cell radius, c is the speed of light, τ=0.096 μs for TDL-A with 100ns delay spread,LRA=139, Tseq=2048, Ts=12048×SCS, and ng is the number of guard samples. Therefore, under the 50m coverage, we can calculate the Ncs for different SCS: SCS 120KHz:8, SCS 240KHz:15, SCS 480KHz:34, SCS 960KHz:69. Meanwhile, the TA shall be: SCS 120KHz:82, SCS 240KHz:163, SCS 480KHz:325, SCS 960KHz:649.

By using the parameters above, the simulation results are shown below:

Figure 26: PRACH, 5ns,10ns,20ns and 40ns DS of different SCS

Table 7：SINR in dB achieving PRACH preamble misdetection probability of 1% with less than 0.1% false alarm

| Channel | Misdetection/False alarm |  |  |  |
| --- | --- | --- | --- | --- |
|  | 120KHz | 240KHz | 480KHz | 960KHz |
| TDL-A, 5ns | -2.9/0.0071 | -3.5/0.0038 | -3.7/0.0026 | -3.2/0.0020 |
| TDL-A, 10ns | -3.5/0.0071 | -4.2/0.0038 | -3.8/0.0026 | -2.9/0.0020 |
| TDL-A, 20ns | -4.4/0.0071 | -4.4/0.0038 | -3.5/0.0026 | -2.7/0.0020 |
| TDL-A, 40ns | -4.7/0.0071 | -3.9/0.0038 | -3.2/0.0026 | -1.8/0.0020 |
| Additional report/notes: 1. PRACH format：A1 2. values of Ncs： SCS 120KHz:8, SCS 240KHz:15, SCS 480KHz:34, SCS 960KHz:69 3.Time advance： SCS 120KHz: random in 0-82, SCS 240KHz: random in 0-163, SCS 480KHz: random in 0-325, SCS 960KHz: random in 0-649. |  |  |  |  |

We can see from the Figure 26 that:

- At low SNR, the Higher SCS, the worse BLER performance for all different DS. It is mainly because high SCS leads to poor preamble sequence correlation.

- At high SNR, the BLER performance basically becomes better with the increase of SCS, mainly due to the gain brought by the increase of bandwidth.

- With the increases of the DS, the performance of SCS 480KHz and SCS 960KHz decrease which is because the CP length cannot cover the DS especially for DS=40ns.

Observation 13: For PRACH channel at low SNR, the preamble sequence correlation shows great effect on the detection performance. The higher SCS, the worse the correlation.

Observation 14: For PRACH channel at high SNR, the bandwidth shows great effect on the detection performance. The larger bandwidth, the better the performance.

Observation 15: For PRACH channel, with the increase of DS, the higher the SCS, the more obvious the performance degradation.

- Conclusion

In this contribution, we provide some evaluation results and discuss about the PDSCH/PUSCH channel, SSB channel and PRACH channel with the following observations.

Observation 1: Under the same DS, the performance of NCP is basically the same as ECP for CP-OFDM waveform.

Observation 2: High order modulation is more sensitive to phase noise impact. Higher SCS benefits more with phase noise compensation than lower SCS, especially for high order modulation such as 64QAM.

Observation 3: For CP-OFDM waveform, SCS 960 KHz is the most affected because of its shortest CP coverage when the DS is increased to 40 ns.

Observation 4: The greater the bandwidth, the greater the number of RBs, resulting in more ICI impact.

Observation 5: For CP-OFDM, the larger SCS is more sensitive to DS.

Observation 6: For 400MHz bandwidth, compared with CP-OFDM waveform, the PN compensation of DFT-S-OFDM waveform can eliminate the influence of PN more effectively, especially for the small SCS (120KHz and 240KHz).

Observation 7: For DFT-S-OFDM waveform, under the same PTRS overhead, the more PTRS number, the better the BLER performance.

Observation 8: For DFT-S-OFDM waveform, with the increase of DS (≤20ns), the performance of higher SCS will slightly deteriorate.

Observation 9: For DFT-S-OFDM waveform, larger bandwidth (more RB numbers) is more sensitive to PN impact.

Observation 10: For SSB detection, when the coarse frequency offset searcher range equals to the fine frequency offset compensation value, higher SCS (larger bandwidth) shows better performance.

Observation 11: For SSB channel, CP length and SCS will affect the autocorrelation of PSS sequence, thereby affecting the SSB detection performance.

Observation 12: For Cell ID detection, when the coarse frequency offset searcher range equals to the fine frequency offset compensation value, various SCS and DS have little effect on the cell ID BLER performance.

Observation 13: For PRACH channel at low SNR, the preamble sequence correlation shows great effect on the detection performance. The higher SCS, the worse the correlation.

Observation 14: For PRACH channel at high SNR, the bandwidth shows great effect on the detection performance. The larger bandwidth, the better the performance.

Observation 15: For PRACH channel, with the increase of DS, the higher the SCS, the more obvious the performance degradation.

References

- RP-193259, RAN #86, Reno, Sitges, Spain, December 9 – 12, 2019.

- R1-20057652, Discussions on required changes to NR using existing NR waveform, vivo, RAN WG1 #103, e-Meeting, October 26th – November 13th, 2020.

Appendix: Link level evaluation assumptions

Table 8:  Simulation assumptions in LLS for PDSCH channel

| Parameters | Values |
| --- | --- |
| Carrier frequency | 60 GHz |
| Waveform | CP-OFDM |
| Antenna configuration | TDL-A: 2T2R |
| MIMO Layer/Precoder | 1/random precoder |
| Subcarrier spacing | 120/240/480/960 KHz |
| Phase noise model | TR38.803 Example 2 |
| MCS | NR Rel15 MCS Table |
| MIMO reception algorithm | MMSE |
| Channel model | TDL-A |
| Pre-BF RMS delay spread | 5ns/10ns/20ns/40ns |
| RB allocation | 2GHz: 320/160 400MHz: 256/128/64/32 |
| FFT size | 2GHz: 4096/2048 400MHz: 4096/2048/1024/512 |
| PDSCH symbol indexes | NCP: 2-13 ECP:2-11 |
| DMRS symbol index | 2 |
| DMRS bundling size | 4 |
| PTRS symbol indexes | NCP: 3-13 ECP:3-11 |
| PTRS density | One tone per 2 RBs |
| Channel code | LDPC |
| Channel estimation | MMSE |
| Transmission mode | QPSK:526/1024; MCS 7. 16QAM: 658/1024; MCS 16 64QAM:666/1024; MCS 22 Actual transmission code is list in Table 9 |

Table 9: The actual coding rate in LLS for PDSCH channel

| MCS | 400M bandwidth |  |  |  |  |  |  |  | 2G |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NCP |  |  |  | ECP |  |  |  | NCP |  | ECP |  |
|  | 120KHz | 240KHz | 480KHz | 960KHz | 120KHz | 240KHz | 480KHz | 960KHz | 480KHz | 960KHz | 480KHz | 960KHz |
| 7 | 0.5064 | 0.5069 | 0.5074 | 0.5089 | 0.5127 | 0.5127 | 0.5127 | 0.5127 | 0.5193 | 0.5194 | 0.5109 | 0.5109 |
| 16 | 0.6485 | 0.6491 | 0.6492 | 0.6502 | 0.6379 | 0.6379 | 0.6379 | 0.6379 | 0.6455 | 0.6454 | 0.6496 | 0.6496 |
| 22 | 0.6542 | 0.6542 | 0.6542 | 0.6548 | 0.6444 | 0.6444 | 0.6444 | 0.6444 | 0.6584 | 0.6581 | 0.6499 | 0.6499 |

Table 10: Simulation assumptions in LLS for PUSCH channel

| Parameters | Values |
| --- | --- |
| Carrier frequency | 60 GHz |
| Waveform | DFT-S-OFDM |
| Antenna configuration | TDL-A: 2T2R |
| MIMO Layer/Precoder | 1/random precoder |
| Subcarrier spacing | 120/240/480/960 KHz |
| Phase noise model | TR38.803 Example 2 |
| MCS | NR Rel15 MCS Table |
| MIMO reception algorithm | MMSE |
| Channel model | TDL-A |
| Pre-BF RMS delay spread | 5ns/10ns/20ns/40ns |
| RB allocation | 2GHz: 320/160 400MHz: 256/128/64/32 |
| FFT size | 2GHz: 4096/2048 400MHz: 4096/2048/1024/512 |
| PDSCH symbol indexes | NCP: 2-13 ECP:2-11 |
| DMRS symbol index | 2 |
| PTRS symbol indexes | NCP: 3-13 ECP:3-11 |
| PTRS density | 2GHz: / / /5/4 400MHz: chunk 5/4/3/1 (chunk1:2*2, chunk2:2*4, chunk3:4*2, chunk4:4*4, chunk5:8*4) |
| Channel code | LDPC |
| Channel estimation | MMSE |
| Transmission mode | QPSK:526/1024; MCS 7 16QAM: 658/1024; MCS 16 64QAM:666/1024; MCS 22 Actual transmission code is list in Table 11 |

Table 11: The actual coding rate in LLS for PUSCH channel

| MCS | 400M bandwidth |  |  |  |  |  |  |  | 2G |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NCP |  |  |  | ECP |  |  |  | NCP |  | ECP |  |
|  | 120KHz | 240KHz | 480KHz | 960KHz | 120KHz | 240KHz | 480KHz | 960KHz | 480KHz | 960KHz | 480KHz | 960KHz |
| 7 | 0.5209 | 0.5218 | 0.5072 | 0.5081 | 0.5152 | 0.5152 | 0.5152 | 0.5152 | 0.5139 | 0.5139 | 0.5083 | 0.5083 |
| 16 | 0.6434 | 0.6438 | 0.6438 | 0.6450 | 0.6368 | 0.6368 | 0.6368 | 0.6368 | 0.6480 | 0.6484 | 0.6426 | 0.6426 |
| 22 | 0.6539 | 0.6539 | 0.6539 | 0.6545 | 0.6494 | 0.6494 | 0.6494 | 0.6494 | 0.6525 | 0.6525 | 0.6581 | 0.6581 |

Table 12:  Simulation assumptions in LLS for SSB channel

| Parameters | Value |
| --- | --- |
| Carrier frequency | 60 GHz |
| SCS | 120K, 240K，480k，960k |
| N_FFT | 256 |
| Delay spread | 5ns/ 10ns/ 20ns/40ns |
| CP type | N-CP/E-CP |
| Antenna configuration | TDL-A: 2T2R |
| UE speed | 3 km/h |
| RB allocation | 20 |
| Searcher range | 10ppm (-600k,600k) |
| Searcher step | 120k: 56k 240k: 112k 480k: 224k 960k: 448k |
| Initial frequency offset | 600k |

Table 13: Simulation assumptions in LLS for PRACH channel

| Parameters | Value |
| --- | --- |
| Carrier frequency | 60 GHz |
| SCS | 120K, 240K，480k，960k |
| N_FFT | 2048 |
| Delay spread | 5ns/ 10ns/ 20ns/40ns |
| Antenna configuration | TDL-A: 1T2R |
| UE speed | 3 km/h |
| RB allocation | 20 |
| Preamble RB | 12 |
| PRACH format | A1 |
| Restricted type | unrestricted |