3GPP TSG RAN WG1 #103-e                                                                                   R1-2007742

e-Meeting, October 26th – November 13th, 2020

Source:	ZTE

Title:                 Discussion on baseline coverage performance for FR2

Agenda item:	8.8.1.2

Document for: Discussion and Decision

Introduction

According to the revised SID on NR coverage enhancements [1], one objective is to identify the baseline performance for specific scenarios for FR1, where the scenarios include urban scenario and rural scenario. In the RAN1#101-e and RAN1#102-e meeting, the evaluation methodology, LLS simulation assumptions, link budget template were discussed and the output can be found in [2][3][4][5].

In this contribution, we further discuss the remaining issues on the simulation assumptions/link budget template, and provide the baseline coverage performance for DL/UL channels in different scenarios in FR2.

Discussion on LLS simulation assumptions

- PUCCH carrying HARQ-ACK for Msg4

- In Section 4 of this contribution, we find PUCCH carrying HARQ-ACK for Msg4 could potentially have coverage issues. However, it’s unfortunate that RAN1 missed to discuss the evaluation on this channel and its corresponding simulation assumptions. Compare to normal PUCCH evaluation, the main difference of PUCCH carrying HARQ-ACK for Msg4 is the HARQ-ACK payload is one bit and no PUCCH repetition is supported. As a result, we have the following proposal.

- Proposal 1: For link level simulation of PUCCH format 1 with 1-bit HARQ-ACK for Msg4 in FR2, reuse the simulation assumptions of normal PUCCH with assuming no PUCCH repetition.

Discussion on link budget parameters

- Transmitter/Receiver array gain

In RAN1#102-e, the following agreements are reached for antenna gain modeling of gNB and UE. For gNB, two correction factors 1 and 2 are introduced for antenna gain component 2 and component 3/4 respectively. For UE, one correction factor 3 is introduced for antenna array gain. In the following, we provide our views on determination of the correction factors for FR2.

Agreements: Further clarify the agreement on antenna gain and antenna gain components including antenna gain correction factors as follows: - For both TDL option 1 (table A below) and TDL option 2 & CDL (table B below) - The gain of antenna gain component 1 is included in LLS results - The gain of antenna gain component 2 is included in link budget template - The gain is expressed by 10 * log 10( N/k ) - 1 - For TDL option 2 & CDL, the gain is 0 dB - The gain of antenna gain component 3 is included in link budget template - The gain of antenna gain component 4 is included in link budget template - The gain of antenna gain components 3 and 4 is expressed by Antenna Element Gain + 10 * log 10( M/N ) -2 - For Tx, One row is used represent the gain of antenna gain component 3 + 4, i.e. row No. (4) - For Rx, One row is used represent the gain of antenna gain component 3 + 4, i.e. row No. (11) - Note: more appropriate name or explanation will be added to row No.(4) and (11). Details can be discussed when the link budget template is updated. Table A. antenna gain components for TDL option 1 Agreement: - Antenna array gain at a UE for FR1 and FR2 is clarified as follows: - The meaning of k, N and M: - k is the number of Tx/Rx chains, e.g., number of SRS/CSI-RS ports to be simulated in LLS. - M is the number of antenna elements used both for transmission and reception, i.e., M2 xpol antenna elements. - A formal definition of N is not necessary for UE antenna array gain modeling. - The values for k and the relationship between k and M are clarified as follows: - For FR1, k = M is assumed for the simulations, and - k=1 for Tx (optional k = 2) - k∈{2,4} for Rx - For FR2, there are two possibilities for simulations: - k∈{1,2}; for Tx and k=2 for Rx; or - k=M. - Antenna array gain in transmission/reception to input in link budget template is given by - Antenna Element Gain+10*log10Mk-Δ3, where - Δ3(≥0 dB) is a correction factor to account for various non-idealities impacting the actual antenna array gain, if any - For FR1, Δ3=0. - For FR2, 3 is channel procedure/dependent, and reported by companies. - The values for antenna element gain: - 0 dBi for FR1 - 5 dBi for FR2

Take urban 28GHz as an example,  there are two 2 RF chains with totally 256 elements at gNB side, and 1Tx-2Rx with 4 polarized elements at UE side. In such case, Δ1 is zero,  and Δ2 and Δ3 can be jointly determined by UE’s angular location in reference to the bore sight of the selected DL-UL beam pair via SLS. In Figure 1, the CDF curve of UE’s analog beamforming gain for channels in RRC-connected mode is provided. In the simulation, 128 DL and 4 UL analog beams per sector are formulated for channels in connected mode.

- Figure 1 CDF curve of UE’s analog beamforming gain for 28GHz urban scenario —— DL/UL channels in connected mode

As can be observed in Figure 1, if the 95th percentile beamforming gain is chosen, the correction Δ2+Δ3 is 6.99 dB (ideal beamforming gain of 27.09 dB minus actual beamforming gain of 20.1 dB) for DL/UL channels in RRC-connected mode.

Observation 1: For urban scenario at 28GHz, Δ2 + Δ3 is 6.99 dB for DL/UL channels in RRC-connected mode.

Similarly, the correction factors could be obtained for indoor scenarios. In Table 1 below, the correction factors in different scenarios are summarized. Note that, 64 DL SSB beams and 1 UL beam is assumed for channels in idle mode in the evaluation.

Table 1 Beamforming gain for DL/UL channels in RRC connected mode and RRC-idle mode for FR2

| Scenario | Ideal beamforming gain per TxRU (dB) | Antenna gain correction factors (dB) |
| --- | --- | --- |
| DL/UL channels in RRC connected mode in 28GHz indoor scenario | 24.08 | Δ1=0, Δ2+Δ3 = 5.47 |
| DL/UL channels in RRC idle mode in 28GHz indoor scenario | 24.08 | Δ1=0, Δ2+Δ3 =12.19 |
| DL/UL channels in RRC connected mode in 28GHz urban scenario | 27.09 | Δ1=0, Δ2+Δ3 = 6.99 |
| DL/UL channels in RRC idle mode in 28GHz urban scenario | 27.09 | Δ1=0, Δ2+Δ3 =14.43 |

- Proposal 2: Consider the antenna gain correction factors in Table 1 for link budget calculation in FR2.

- Receiver interference density

- Interference density is highly dependent on the deployment scenarios and carrier frequency. In ITU self-evaluation, this value is only available for urban at 4GHz or rural at 700MHz. In order to get more accurate interference density for FR2, SLS is conducted. In the evaluation, the interference density values of the 5th percentile SINR UEs are provided. More detailed simulation assumptions are provided in Appendix B.

- The interference density values for the interested physical channels are provided in Table 2.

Table 2 Interference density for different physical channel

| Physical Channel | Scenario | Interference density values of the 5th percentile SINR UEs |
| --- | --- | --- |
| PUSCH-eMBB | Indoor scenario, 28GHz(ISD= 20) | -160.80 |
|  | Urban scenario, 28GHz(ISD= 200,O2I) | -168.93 |
|  | Urban scenario, 28GHz(ISD= 200,O2O) | -169.91 |
| MSG3 | Indoor scenario, 28GHz(ISD= 20) | -164.42 |
|  | Urban scenario, 28GHz(ISD= 200,O2I) | -167.6 |
|  | Urban scenario, 28GHz(ISD= 200,O2O) | -171.84 |
| PUCCH | Indoor scenario, 28GHz(ISD= 20) | -164.81 |
|  | Urban scenario, 28GHz(ISD= 200,O2I) | -170.76 |
|  | Urban scenario, 28GHz(ISD= 200,O2O) | -173.04 |
| PDCCH-unicast | Indoor scenario, 28GHz(ISD= 20) | -152.96 |
|  | Urban scenario, 28GHz(ISD= 200,O2I) | -176.50 |
|  | Urban scenario, 28GHz(ISD= 200,O2O) | -172.86 |
| PDCCH-broadcast | Indoor scenario, 28GHz(ISD= 20) | -153.67 |
|  | Urban scenario, 28GHz(ISD= 200,O2I) | -170.70 |
|  | Urban scenario, 28GHz(ISD= 200,O2O) | -168.25 |
| PBCH | Indoor scenario, 28GHz(ISD= 20) | -153.67 |
|  | Urban scenario, 28GHz(ISD= 200,O2I) | -170.70 |
|  | Urban scenario, 28GHz(ISD= 200,O2O) | -168.25 |
| PDSCH-eMBB | Indoor scenario, 28GHz(ISD= 20) | -152.96 |
|  | Urban scenario, 28GHz(ISD= 200,O2I) | -176.50 |
|  | Urban scenario, 28GHz(ISD= 200,O2O) | -172.86 |
| MSG4 | Indoor scenario, 28GHz(ISD= 20) | -153.67 |
|  | Urban scenario, 28GHz(ISD= 200,O2I) | -170.70 |
|  | Urban scenario, 28GHz(ISD= 200,O2O) | -168.25 |

- Proposal 3: Consider the receiver interference density values in Table 2 for link budget calculation in FR2.

- Penetration margin

For penetration margin calculation, Table 7.4.3-1 and Table 7.4.3-2 defined in TR 38.901, which is frequency and penetration material dependent, should be used. For urban scenario, 50% low-loss and 50% high-loss models can be considered.  The calculation based on Table 7.4.3-1 and 7.4.3-2 is shown in Table 3.

Table 3. Penetration margins for FR2

| Urban |  |  |
| --- | --- | --- |
| High | Low | 50% High +50% Low |
| 44.20 | 24.08 | 34.14 |

For indoor case, the typical penetration margin is 0 dB. And for O2O case, the typical penetration margin is 9 dB according to TR 38.901.

Proposal 4: Consider the penetration margin with 34.14 dB for urban O2I, 9 dB for urban O2O, 0 dB for indoor scenario in FR2.

Evaluation results

- Indoor at 28GHz

In Table 4, the evaluation results for indoor scenario at 28GHz are provided. In the evaluation, the following assumptions are used:

- Correction factors for antenna array gain: Using values in Table 1 above.

- Interference density: Using values in Table 2 above.

- Penetration margins: 0 dB is assume for indoor scenario.

- Shadow fading margin: Reusing IMT-2020 values.

- For other MPL specific values: Reusing IMT-2020 values.

Table 4 Evaluation results for indoor scenario at 28GHz

| Evaluation methodology | Item | PUSCH-eMBB | MSG3 | PRACH B4 | MSG4 ACK | PUCCH-2bits | PUCCH-11bits | PUCCH -22bits | PDCCH-unicast | PDCCH-broadcast | PBCH | PDSCH-eMBB | MSG4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Required SNR | 4.34 | -2.01 | -9.82 | -5.35 | -3.45 | -0.90 | 1.71 | -7.97 | -7.97 | -6.55 | -1.88 | -7.12 |
| LLS based methodology | MCL | 100.67 | 120.93 | 124.51 | 127.52 | 125.62 | 123.07 | 120.46 | 101.60 | 102.26 | 100.84 | 96.01 | 101.41 |
|  | MIL | 128.28 | 141.82 | 145.40 | 148.41 | 153.23 | 150.68 | 148.07 | 129.21 | 123.14 | 121.72 | 123.62 | 122.29 |
|  | MPL | 123.80 | 137.34 | 137.84 | 140.85 | 145.67 | 143.12 | 140.51 | 121.65 | 115.58 | 114.16 | 119.14 | 117.81 |
|  | - Target MPL with ISD=20 | 91.82 | 91.82 | 91.82 | 91.82 | 91.82 | 91.82 | 91.82 | 91.82 | 91.82 | 91.82 | 91.82 | 91.82 |
| SLS based methodology | 5th percentile SNR via SLS | 2.94 | 1.31 | -3.18 | - | 0.24 | 1.63 | 2.81 | 10.13 | 10.5 | 10.5 | 10.13 | 10.5 |
|  | Performance gap via SLS | 1.4 | NAN | NAN | - | NAN | NAN | NAN | NAN | NAN | NAN | NAN | NAN |

For LLS based methodology, if the target is defined by MPL with ISD = 20m, no coverage enhancement is needed. For SLS based methodology, only PUSCH with eMBB  needs coverage enhancement.

Observation 2: In indoor scenario at 28GHz,

  - for LLS based methodology, no channel needs coverage enhancement if the target is defined by MPL with ISD = 20m.

  - for SLS based methodology, only PUSCH with eMBB  needs coverage enhancement.

- Urban at 28GHz & O2I

In Table 5, the evaluation results for urban O2I scenario at 28GHz are provided. In the evaluation, the following assumptions are used:

- Correction factors for antenna array gain: Using values in Table 1 above.

- Interference density:  Using values in Table 2 above.

- Penetration margins:  34.14 dB is assumed.

- Shadow fading margin:  Reusing IMT-2020 values.

- For other MPL specific values:  Reusing IMT-2020 values.

Table 5 Evaluation results for urban O2I scenario at 28GHz

| Evaluation methodology | Item | PUSCH-eMBB | MSG3 | PRACH B4 | MSG4 ACK | PUCCH-2bits | PUCCH-11bits | PUCCH -22bits | PDCCH-unicast | PDCCH-broadcast | PBCH | PDSCH-eMBB | MSG4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Required SNR | 3.61 | -2.64 | -10.10 | -5.80 | -4.13 | -1.14 | 1.33 | -8.10 | -8.10 | -6.70 | 1.35 | -5.47 |
| LLS based methodology | MCL | 106.38 | 123.33 | 125.27 | 130.69 | 129.02 | 126.03 | 123.56 | 129.86 | 129.26 | 127.86 | 120.91 | 126.63 |
|  | MIL | 135.48 | 144.98 | 146.93 | 152.35 | 158.12 | 155.13 | 152.66 | 158.96 | 150.92 | 149.52 | 150.01 | 148.29 |
|  | MPL | 96.86 | 106.36 | 105.23 | 110.65 | 116.42 | 113.43 | 110.96 | 117.26 | 109.22 | 107.82 | 111.39 | 109.67 |
|  | - Target MPL with ISD=200m | 123.10 | 123.10 | 123.10 | 123.10 | 123.10 | 123.10 | 123.10 | 123.10 | 123.10 | 123.10 | 123.10 | 123.10 |
| SLS based methodology | 5th percentile SNR via SLS | -16.58 | -8.02 | -15.15 | - | -3.39 | -2.9 | -1.87 | -6.76 | -7.27 | -7.27 | -6.76 | -7.27 |
|  | Performance gap via SLS | 20.19 | 5.38 | 5.05 | - | NAN | 1.76 | 3.2 | NAN | NAN | 0.57 | 8.11 | 1.8 |

For LLS based methodology, if the target is defined by MPL with ISD = 200m, all the channels simulated need coverage enhancement.  For SLS based methodology, almost all the channels simulated need coverage enhancement except for PUCCH format 1 with 2bits and PDCCH.

Observation 3: In urban O2I scenario at 28GHz,

  - for LLS based methodology, all channels need coverage enhancement if the target is defined by MPL with ISD = 200m.

  - for SLS based methodology, channels except for PUCCH format 1 with 2bits and PDCCH need coverage enhancement.

- Urban at 28GHz & O2O

In Table 6, the evaluation results for urban O2O scenario at 28GHz are provided. In the evaluation, the following assumptions are used:

- Correction factors for antenna array gain: Using values in Table 1 above.

- Interference density:  Using values in Table 2 above.

- Penetration margins: 9 dB is assumed.

- Shadow fading margin:  Reusing IMT-2020 values.

- For other MPL specific values:  Reusing IMT-2020 values.

Table 6 Evaluation results for Urban 28GHz & O2O

| Evaluation methodology | Item | PUSCH-eMBB | MSG3 | PRACH B4 | MSG4 ACK | PUCCH-2bits | PUCCH-11bits | PUCCH -22bits | PDCCH-unicast | PDCCH-broadcast | PBCH | PDSCH-eMBB | MSG4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Required SNR | 6.44 | -2.89 | -10.22 | -6.35 | -4.70 | -2.40 | 0.10 | -7.83 | -7.83 | -9.11 | 7.58 | -3.27 |
| LLS based methodology | MCL | 103.91 | 125.06 | 125.63 | 131.80 | 130.15 | 127.85 | 125.35 | 129.30 | 128.44 | 129.72 | 114.39 | 123.88 |
|  | MIL | 133.01 | 146.72 | 147.29 | 153.46 | 159.25 | 156.95 | 154.45 | 158.40 | 150.10 | 151.38 | 143.49 | 145.54 |
|  | MPL | 119.53 | 133.24 | 130.73 | 136.90 | 142.69 | 140.39 | 137.89 | 141.84 | 133.54 | 134.82 | 130.01 | 132.06 |
|  | - Target MPL with ISD=200m | 123.10 | 123.10 | 123.10 | 123.10 | 123.10 | 123.10 | 123.10 | 123.10 | 123.10 | 123.10 | 123.10 | 123.10 |
| SLS based methodology | 5th percentile SNR via SLS | 2.06 | 4.16 | -2.61 | - | 2.23 | 4.13 | 5.96 | 10.61 | 9.76 | 9.76 | 10.61 | 9.76 |
|  | Performance gap via SLS | 4.38 | NAN | NAN | - | NAN | NAN | NAN | NAN | NAN | NAN | NAN | NAN |

For LLS based methodology, if the target is defined by MPL with ISD = 200m, only PUSCH with eMBB needs coverage enhancement. For SLS based methodology, similar observation is found.

Observation 4: In urban O2O scenario at 28GHz, only PUSCH with eMBB needs coverage enhancement for both LLS and SLS based methodology.

Conclusion

According to the analysis given above, we have the following observations and proposals:

Observation 1: For urban scenario at 28GHz, Δ2 + Δ3 is 6.99 dB for DL/UL channels in RRC-connected mode.

Observation 2: In indoor scenario at 28GHz,

  - for LLS based methodology, no channel needs coverage enhancement if the target is defined by MPL with ISD = 20m.

  - for SLS based methodology, only PUSCH with eMBB  needs coverage enhancement.

Observation 3: In urban O2I scenario at 28GHz,

  - for LLS based methodology, all channels need coverage enhancement if the target is defined by MPL with ISD = 200m.

  - for SLS based methodology, channels except for PUCCH format 1 with 2bits and PDCCH need coverage enhancement.

Observation 4: In urban O2O scenario at 28GHz, only PUSCH with eMBB needs coverage enhancement for both LLS and SLS based methodology.

- Proposal 1: For link level simulation of PUCCH format 1 with 1-bit HARQ-ACK for Msg4 in FR2, reuse the simulation assumptions of normal PUCCH with assuming no PUCCH repetition.

- Proposal 2: Consider the antenna gain correction factors in Table 1 for link budget calculation in FR2.

- Proposal 3: Consider the receiver interference density values in Table 2 for link budget calculation in FR2.

Proposal 4: Consider the penetration margin with 34.14 dB for urban O2I, 9 dB for urban O2O, 0 dB for indoor scenario in FR2.

Reference

- 3GPP RAN#88-e, RP-200861, “Revised SID on NR coverage enhancement”, China Telecom.

- 3GPP, RAN1#101-e, Chairman’s notes.

- 3GPP, RAN1#102-e, Chairman’s notes.

- 3GPP, RAN1#102-e, R1-2007479, [102-e-Post-NR-CovEnh-01] Summary on email discussion/approval of remaining simulation assumptions, Moderator (SoftBank, Nokia).

- 3GPP, RAN1#102-e, R1-2007480, [102-e-Post-NR-CovEnh-02] Summary on email discussion/approval of link budget template, Moderator (SoftBank, Nokia).

- 3GPP, RAN1#102-e, R1-2005426, Discussion on baseline coverage performance for FR2, ZTE.

- 3GPP, RAN1#103-e, R1-2007746, Discussion on target performance for NR coverage enhancements, ZTE.

- Appendix-A LLS assumptions and results

- A.1 LLS assumptions and results for PUSCH

Table A.1-1 Simulation assumption for baseline performance of PUSCH for FR2

| Parameters | Values |
| --- | --- |
| Scenario and frequency | 28GHz |
| Frame structure for TDD | DDDSU (S: 10D:2G:2U) |
| Transmission bit rate for data channel (bit/s) | 5 Mbps for eMBB |
| Subcarrier spacing | 120kHz |
| System BW | 100MHz |
| Channel model for link-level simulation | TDL-A |
| Delay Spread | 30ns for indoor, 100ns for urban |
| UE velocity | 3 km/h, 30 km/h |
| Number of antenna elements for BS | Indoor scenario: 128 (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1) Urban/suburban scenario: 256, (M,N,P,Mg,Ng) = (4, 8, 2, 2, 2) |
| Number of TxRUs for BS | 2 |
| Number of UE Tx/Rx chains | 1T2R |
| Occupied channel bandwidth | 30 PRBs |
| PUSCH duration | 14OS |
| DMRS overhead | Type I, 2 DMRS symbol (one front- loaded and one additional), no multiplexing data for O2I Type I, 3 DMRS symbol (one front- loaded and one additional), no multiplexing data for O2O |
| Frequency hopping | w/o frequency hopping |
| Channel estimation | Practical |
| Receiver type | MMSE |
| BLER Target | 0.1 |
| Waveform | DFT-s-OFDM |

- A.2 LLS assumptions and results for MSG3

Table A.2-1 Simulation assumption for baseline performance of Msg3 for FR2

| Parameters | Values |
| --- | --- |
| Scenario and frequency | 28GHz |
| Frame structure for TDD | DDDSU (S: 10D:2G:2U) |
| TBsize | 56bit |
| Subcarrier spacing | 120kHz |
| System BW | 100MHz |
| Channel model for link-level simulation | TDL-A |
| Delay Spread | 30ns for indoor, 100ns for urban |
| UE velocity | 3 km/h, 30 km/h |
| Number of antenna elements for BS | Indoor scenario: 128 (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1) Urban/suburban scenario: 256, (M,N,P,Mg,Ng) = (4, 8, 2, 2, 2) |
| Number of TxRUs for BS | 2 |
| Number of UE Tx/Rx chains | 1T2R |
| Occupied channel bandwidth | 2 PRBs |
| PUSCH duration | 14OS |
| DMRS overhead | Type I, 2 DMRS symbol (one front- loaded and one additional), no multiplexing data for O2I Type I, 3 DMRS symbol (one front- loaded and one additional), no multiplexing data for O2O |
| Frequency hopping | w/o frequency hopping |
| Channel estimation | Practical |
| Receiver type | MMSE |
| BLER Target | 0.1 |
| Waveform | DFT-s-OFDM |

- A.3 LLS assumptions and results for PUCCH

Table A.2-2 Simulation assumption for baseline performance of PUCCH for FR2

| Parameters | Values |
| --- | --- |
| Scenario and frequency | 28GHz |
| Subcarrier spacing | 120kHz |
| System BW | 100MHz |
| Format type | Format 1: 2bits, Format 3: 4bits, 11bits, 22bits. |
| Channel model for link-level simulation | TDL-A |
| Delay Spread | 30ns, 100ns |
| UE velocity | 3 km/h |
| Number of antenna elements for BS | Indoor scenario: 128 (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1) Urban/suburban scenario: 256, (M,N,P,Mg,Ng) = (4, 8, 2, 2, 2) |
| Number of TxRUs for BS | 2 |
| Number of UE Tx/Rx chains | 1T2R |
| Occupied channel bandwidth | 1 PRBs |
| PUCCH duration | 14OS |
| DMRS overhead | Pattern 1: additional DMRS for PUCCH format 3 |
| Channel estimation | Practical |
| Receiver type | MMSE |
| BLER Target | DTX to ACK probability: 1% ; NACK to ACK probability: 0.1%; ACK missed detection probability: 1%; CSI block error probability: 1% |

- A.4 LLS assumptions and results for PDCCH

Table A.4-1 Simulation assumption for baseline performance of PDCCH for FR2

| Parameter | Value |  |
| --- | --- | --- |
| Scenario | Urban | Indoor |
| Delay Spread | 100 ns | 30 ns |
| DCI payload (excluding 24bits CRC) | 40 bits |  |
| Carrier Frequency | 28GHz |  |
| UE speed | O2I:3 km/h, O2O:120 km/h, |  |
| System bandwidth | 100MHz |  |
| Number of antenna elements for BS | Indoor scenario: 128 (M, N, P, Mg, Ng) = (8, 8, 2, 1, 1) Urban/suburban scenario: 256, (M,N,P,Mg,Ng) = (4, 8, 2, 2, 2) |  |
| Subcarrier spacing | 120KHz |  |
| Number of symbols for CORESET | 2 |  |
| REG bundling size | 6 |  |
| Modulation | QPSK |  |
| Channel coding | Polar code (DCI) |  |
| Transmission scheme | 1-port precoder cycling |  |
| Channel model | TDL-A |  |
| Channel estimation | Practical |  |
| Receiver type | MMSE |  |
| BLER target | 1% |  |

- A.5 LLS assumptions and results for PBCH

Table A.5-1 Simulation assumption for baseline performance of PBCH for FR2

| Parameter | Value |  |
| --- | --- | --- |
| Scenario | Urban | Indoor |
| Carrier Frequency | 28GHz | 28GHz |
| UE speed | 3,120 km/h | 3 km/h |
| System bandwidth | 100MHz | 100MHz |
| Number of BS antennas | 2Tx | 2Tx |
| Number of UE antennas | 2Rx | 2Rx |
| Subcarrier spacing | 120KHz | 120kHz |
| Channel model | TDL-A (delay spread: 100ns) | TDL-A (delay spread: 30ns) |
| PBCH payload (excluding 24bits CRC) | 32 bits |  |
| Number of symbols for CORESET | 2 |  |
| Modulation | QPSK |  |
| Channel coding | Polar code (DCI) |  |
| Transmission scheme | 1-port precoder cycling |  |
| Channel estimation | Practical |  |
| Receiver type | MMSE |  |
| BLER target | 1% |  |
| Number of interfering TRPs | 0 TRP |  |
| Combined number | 4 SSBs |  |
| Frequency Offset | Initial acquisition TRP: uniform distribution +/- 0.05 ppm UE: uniform distribution +/- 5, 10, 20 ppm (each company to choose one) Non-initial acquisition TRP: uniform distribution +/- 0.05 ppm UE: uniform distribution +/- 0.1 ppm |  |

- A.6 LLS assumptions and results for PRACH

Table A.6-1 Simulation assumption for baseline performance of PRACH for FR2

| Parameter | Value |
| --- | --- |
| Scenario and frequency | 28GHz |
| Subcarrier spacing | 120kHz |
| System BW | 100MHz |
| Channel model for link-level simulation | TDL-A |
| Delay Spread | 30ns, 100ns |
| UE velocity | 3 km/h, 30km/h |
| Number of UE Tx/Rx chains | 2T2R |
| Occupied channel bandwidth | NR PRACH preamble format 0 : 6 RBs NR PRACH preamble format 4B : 12 RBs |
| Miss-detection target | 1% |
| Preamble | NR PRACH preamble format 0 with Ncs = 167 , Logical sequence index = 22, v=2 for O2I and O2O NR PRACH preamble format B4 with Ncs = 13, Logical sequence index = 1, v=0 for O2O |

- A.7 LLS assumptions and results for PDSCH/Msg 4

Table A.7-1 Simulation assumption for baseline performance of PDSCH for FR2

| Parameters | Values |  |
| --- | --- | --- |
| Scenario | Urban | Indoor |
| Duplexing scheme and frequency | 28 GHz TDD | 28 GHz TDD |
| Transmission bit rate for data channel (bit/s) | 25 Mbps | 25 Mbps |
| Subcarrier spacing | 120kHz | 120kHz |
| Channel model for link-level simulation | TDL-A | TDL-A |
| Pathloss model (select from LoS or NLoS) | NLos | NLos |
| Delay Spread | 100ns | 30ns |
| UE velocity | 120 km/h for O2O 3 km/h for O2I | 3 km/h |
| Frame structure for TDD | DDDSU (S: 10D:2G:2U) |  |
| Number of TXRU for BS | 2 |  |
| Number of TRXU for UE | 2 |  |
| Payload for MSG4 | 1000 bits |  |
| PMI adaption | ideal |  |
| Channel estimation | Practical |  |
| Receiver type | 2D-MMSE |  |
| BLER Target | 10% |  |
| duration | 12 OS |  |

- Appendix-B SLS assumptions and results

Most of the simulation assumptions for SLS can reuse that of defined for LLS as above. In the following, SLS specific simulation assumptions are provided.

Table B-1 Simulation assumptions for system-level simulation for FR2

| Parameters | Values |  |
| --- | --- | --- |
| Scenario | Urban scenario | Indoor scenario |
| Layout | Single layer - Macro lay: Hex. Grid | 120m x 50m |
| Inter-BS distance | 200m | 20m |
| Channel model | UMa in TR 38.901 | Indoor-office in TR 38.901 |
| Min distance of UE2gNB | 35m | 0m for indoor |