3GPP TSG RAN WG1 Meeting #103-E					      R1-2007788

e-Meeting, October 26th – November 13th, 2020

Source:	Fujitsu

Title:	Considerations on Inter-UE Coordination for Mode 2 Enhancements

Agenda item:	8.11.2.2

Document for:	Discussion and Decision

Introduction

The RAN WG approved study item on NR Sidelink Enhancements [1]. The study item includes the following objective:

- Study the feasibility and benefit of the enhancement(s) in mode 2 for enhanced reliability and reduced latency in consideration of both PRR and PIR defined in TR37.885 (by RAN#91), and specify the identified solution if deemed feasible and beneficial [RAN1, RAN2] - Inter-UE coordination with the following until RAN#90. - A set of resources is determined at UE-A. This set is sent to UE-B in mode 2, and UE-B takes this into account in the resource selection for its own transmission. - Note: The study scope after RAN#90 is to be decided in RAN#90. - Note: The solution should be able to operate in-coverage, partial coverage, and out-of-coverage and to address consecutive packet loss in all coverage scenarios. - Note: RAN2 work will start after [RAN#89].

According to the discussion in the e-meeting #102e, we have summarized three proposals. In this contribution, we express our views on those three proposals targeting the enhancement of reliability and the reduction of latency.

Aspects for Study

In the e-meeting #102e, the proposal-3 has been summarized as follows:

- Companies are encouraged to consider at least the following aspects when studying the feasibility and benefit of the enhancement(s) in mode 2

  - Hidden-node problem

  - Exposed-node problem

  - Half duplex problem

  - Consecutive packet loss (as described in WID)

  - [Resource collision (i.e., Time-frequency resource overlapping [and/or Time resource overlapping] caused by the reason other than hidden-node problem].

In response to the above threads, we express our views and selectively propose the corresponding solutions.

Hidden-node problem

The hidden node problem is illustrated in Figure 1, where UE B transmits the data TB to UE A based on sensing procedure. However, the UE A reception is interfered by UE C which is a hidden node to UE B. More specifically, UE B cannot avoid the interference from UE C since UE C is outside the sensing range of UE B. Therefore, the transmissions of UE B and UE C will collide with each other on the selected resources. Here, “collision” means transmission on the overlapped time-frequency resources. Inter-UE coordination can be used to solve the hidden node problem. For example, by sending assistance information, UE A can notify UE B the resources interfered by UE C. When performing resource selection, UE B can avoid using the resources with interference from UE C.

Figure 1: An example of the hidden node problem.

The assistance information for inter-UE coordination should be studied to solve the hidden node problem.

Exposed-node problem

In Rel-16 NR V2X, if a candidate resource overlaps with another resource indicated by an SCI, and the corresponding RSRP measurement result is higher than a threshold, this resource shall be excluded from the candidate resource set. In some cases, such kind of exclusion is unnecessary. Figure 2 gives an example, where UE B wants to transmit the packet(s) to UE A, and UE C wants to transmit the packet(s) to UE D. The transmission of UE B may not actually be interfered even if UE B and UE C have selected overlapping resources, when UE C is far from UE A. Then, UE C can be regarded as an exposed-node for UE B’s transmission. Inter-UE coordination can be used to identify the existing of exposed nodes, e.g., UE B can identify UE C is far from UE A, by comparing the RSRP values corresponding to the same reservation SCI. In such a case, the RSRP values separately reported from UE A can be compared with UE B’s measurements. Alternatively, such an identification can be realized by comparing the geographic locations of UE A and UE C after performing the sensing procedure or acquiring information reported from UE A which includes the geographic location.

Figure 2: An example of the exposed-node problem.

The assistance information for inter-UE coordination should be studied to solve the exposed-node problem.

Half duplex problem

Inter-UE coordination is considered a major WI feature for Rel-17, towards unicast, groupcast, and broadcast communication. Inter-UE coordination can be performed, either prior to or posterior to the initial transmission. The former is to coordinate the resources between the Tx-UEs relying on the information provided by a coordinating UE, in order to avoid resource collision and/or half-duplex impact prior to initial packet transmission. The latter is to assist the Tx-UEs which experience and detect resource collision and/or half-duplex impact posterior to the initial transmission, in order to avoid the resource collision and/or half-duplex impact in retransmission and/or next initial transmission.

Figure 3 exemplifies the TB transmission and retransmission with option-1 based HARQ, within the grouped members of UE-A, UE-B, and UE-C, where the UE-B and the UE-C are Tx-UEs, and the UE-A is a Rx-UE, with three transmission phases. In phase 1, UE-B and UE-C simultaneously transmit the initial data TBs over PSSCH1 and PSSCH2, respectively. In this case, UE-A succeeds in TB reception from UE-B but fails the TB reception from UE-C. Meanwhile, UE-B and UE-C face the half-duplex issue due to the transmission in the same time slot. In phase 2, UE-A only feeds back the NAK over PSFCH2 to UE-C and asks for its retransmission based on the option-1 HARQ process. In phase 3, UE-C retransmits the TB over PSSCH2, and UE-B and UE-A succeed in TB reception. Consequently, UE-C has no opportunity to receive the TB from the UE-B due to the half-duplex constraint.

Figure 3: An example of TB transmission and retransmission with option-1 HARQ.

In order to clarify how much impact does occurs due to the half-duplex constraint, we perform the system level simulation based on the simulation assumptions, listed in Table 1, in Annex. Figure 3 and Figure 4 show the PRR as a function of distance between Tx-UE and Rx-UE, with 100m communication range, in consideration of option-1 HARQ, with the bandwidth of 20MHz and 40MHz, respectively. It can be observed that, even with a short communication range, the PRR cannot be fulfilled with high reliability, such as 99.99%, if the half-duplex issue occurs between the Tx-UEs within the group. However, if the simulation assumes removing the half-duplex impact in option-1 HARQ, then high PRR performance can be achieved. Here, this ideal assumption on half-duplex means that the half-duplex issue still occurs in the initial transmissions, but the Tx-UE can ideally perform the relevant retransmissions in subsequent slots. The purpose of making this ideal assumption in our simulation is to determine the potentially achievable reliability performance in Rel-17.

Consequently, the PRR degradation due to the half-duplex issue is about 1~2% in groupcast, depending on the how many Tx-UEs simultaneously transmit TBs in a slot. It is observed that, the larger the number of Tx-UEs in a slot, the bigger the impact of half-duplex. This implies that, the half-duplex impact could fatally block the realization of stringent PRR reliability in option-1 HARQ.

Figure 4: PRR vs. distance with 20MHz bandwidth, and 100m communication range.

Figure 5: PRR vs. distance with 40MHz bandwidth, and 100m communication range.

The PRR degradation due to the half-duplex issue is about 1~2% in groupcast, depending on the how many Tx-UEs simultaneously transmit TBs in a slot. The larger the number of Tx-UEs in a slot, the bigger the impact of half-duplex.

Rel-17 supports enhancement of Option-1 HARQ process and mitigation of the half-duplex impact, relying on inter-UE coordination mechanism.

In order to mitigate the half-duplex impact, we introduce the concept of a coordinating UE (denoted as Co-UE, alternatively it can be thought of as UE-A, as described in the WID), who enables both Tx-UEs to make the retransmissions in different slots. Each Co-UE needs to behave as follows:

- The Co-UE is in the group, enabling it to receive groupcast transmissions;

- The Co-UE is aware of the situation that both Tx-UEs have been in the transmission mode in a given time slot.

- The Co-UE can transmit SFCI over PSFCH regardless of the status of its own PSSCH reception as long as it can succeed in receiving the corresponding PSCCH.

Figure 6 depicts the details for the TB transmissions and retransmissions with the inter-UE coordinative HARQ feedback process, completed with four phases; i.e., Phase-1 is for the initial TB transmission, Phase-2 is for the SFCI NAK feedback(s), Phase-3 is for the TB1 retransmission, and Phase-4 is for the TB2 retransmission.

Figure 6: TB transmissions and retransmissions with inter-UE coordinative HARQ feedback, with UE-A as the coordinating UE.

More precisely, the procedures for the inter-UE coordinative HARQ feedback process with two Tx-UEs (Tx-UE-B and Tx-UE-C) and one Rx-UE (Rx-UE-A) who are all belonging to the same group can be described as follows.

- The Tx-UE-B and the Tx-UE-C groupcasts the initial TBs in the group in the same time slot.

- The Rx-UE-A as a Co-UE receives both TBs and decodes PSCCH and then PSSCH in the same slot. By checking both source L1 IDs and destination L1 IDs, and the distance between the Tx-UE-B and the Tx-UE-C based on Zone-IDs of both Tx-UEs, Rx-UE-A determines whether to trigger the inter-UE coordinative HARQ feedback process.

- The Rx-UE-A transmits the generated SFCI(s) to the corresponding the Tx-UE(s), over PSFCH(s).

- The Tx-UE-B retransmits the TB as groupcast if the Tx-UE-B receives the SFCI-1 from PSFCH1.

- The Tx-UE-C retransmits the TB as groupcast if the Tx-UE-C receives the SFCI-2 from PSFCH2.

The retransmitted TB from the Tx-UE-C is received by both Tx-UE-B and Rx-UE-A, and the Rx-UE-A decodes the TB if it has also failed in the initial decoding process, otherwise, the Rx-UE-A ignores that TB reception.

The Co-UE should coordinatively transmit SFCIs for the Tx-UEs instead, if the Co-UE detects the half-duplex problem occurred in two Tx-UEs.

In the same scenario, the Co-UE can forward both TBs received from two Tx-UEs, if the Co-UE detects that the half-duplex problem occurred between two Tx-UEs. This forwarding process can be performed by using the same resource reserved by both Tx-UEs in the SCIs of initial transmissions.

The Co-UE should coordinatively forward both TBs received from two Tx-UEs, if the Co-UE detects the half-duplex problem occurred between two Tx-UEs, relying on the same resource reserved in the SCI of initial transmission.

Consecutive packet loss

In the case when UEs perform resource selection independently, it is possibly that UE transmissions collide with each other on the selected resources. This could be even worse if UEs are transmitting periodic traffic due to the occurrence of consecutive collisions. One example is shown in Figure 7 where UE A always collides with UE B. Due to half-duplex constraints, neither UE A nor UE B can identify the collision and even the re-evaluation or pre-emption specified in NR V2X cannot resolve this issue. Therefore, two UEs could keep colliding potentially for a long time, thus leading to the consecutive packet loss.  If a third UE (UE C) is required to receive the transmitted data from UE A or UE B, the reception may keep failing for a long time due to the consecutive collisions between transmissions from UE A and UE B. Inter-UE coordination can be used to avoid consecutive packet loss. For example, UE A can notify other UEs to avoid its own transmissions, or a third UE can notify UE A or UE B to avoid collisions after identifying a collision.

Figure 7: An example of consecutive packet loss.

The assistance information for inter-UE coordination should be studied to avoid consecutive packet loss.

Conditions for Inter-UE Coordination

In the e-meeting #102e, the proposal-2 has been summarized as follows:

- When a set of resources determined at UE-A is sent to UE-B in mode 2 and UE-B takes this into account in the resource selection for its own transmission,

  - for the condition when UE-A sends “a set of resources” to UE-B, at least followings can be considered:

    - Option 1: Based on signaling of triggering or requesting

    - Option 2: Based on a pre-defined or (pre)configured triggering condition(s)

  - FFS details on UE-A behavior of transmitting “a set of resources” when the above option is satisfied, including time domain behavior (e.g., periodic, aperiodic, semi-persistent).

  - FFS details of signaling in 1st option.

  - FFS details of 2nd option.

  - Note: further discussion is necessary on what options will be finally specified.

In response to the above threads, we express our views and selectively propose the corresponding solutions.

The condition when UE-A sends “a set of resources” to UE-B

The assistance information can be provided by a Co-UE (or UE-A as described in WID) to a Tx-UE either in a triggering or pre-defined manner. In the former, as the option-1, a Tx-UE sends the request to the Co-UE, and then the Co-UE passively informs the assistance information to the Tx-UE. In the latter, as the option-2, a Co-UE is aware of the pre-defined condition, and based on the information such as available resources, interference, half-duplex incurrence and congestion status, the Co-UE actively informs the assistance information to associated Tx-UEs.

Option 1: Based on signaling of a trigger or request

With the option-1, firstly, a Tx-UE which has an arrival packet for delivering sends a request to a Co-UE, in order to acquire the assistance information. The request may contain the QoS requirement, cast type, and geographical information, and communication range associated with the arrival packet. Then, the Co-UE informs the assistance information to the Tx-UE, in consideration of what the Tx-UE exactly requested. The assistance information message should be detectable by other non-requesting UEs for reference purposes.

Note that, based on the geographical information of the Tx-UE, the Co-UE is able to figure out the geographical relation in between, which may be used to identify the similarity of the sensing results between theTx-UE and the Co-UE.

The assistance information can be provided by the Co-UE to the Tx-UE in a triggered manner, and the assistance information should be detectable by any other UEs.

The signaling of a trigger or request may rely on two steps: one is related to the information sent from the Tx-UE to the Co-UE, and the other is to the information sent from Co-UE to the Tx-UE.

The resource request procedure from the Tx-UE in the 1st step is as follows:

- Once or before the data packet arrives in the Tx-UE, the Tx-UE decides to acquire a set of resources from the Co-UE, and sends a request to the Co-UE by either MAC-CE or SCI;

- The Tx-UE receives the transmission grant with the set of resources, which is broadcasted from the Co-UE by either MAC-CE or SCI;

  - If SCI is utilized, physical layer entity shall report the granted set of resources to its own MAC layer.

The request by MAC-CE or SCI may contain the information as follows:

- Source ID or group member ID associated with the Tx-UE;

- Destination ID associated with the group;

- QoS requirement (e.g., LCH priority, communication range);

- Cast type (e.g., unicast, groupcast, or broadcast);

- Data packet size;

- Service type (e.g., V2X service ID);

- Traffic types (periodic or aperiodic);

  - The possible resource reservation period if it belongs to a periodic traffic;

The resource grant procedure from the Co-UE in the 2nd step is as follows:

- The Co-UE with a group management function has an ability to gather the Tx-UEs in proximity and creates a group for inter-UE coordination;

- The Co-UE equips a universal MAC entity which enables to select the set of resources and broadcast it to the Tx-UEs who belong to the associated group, by either MAC CE or SCI;

- The Co-UE has an ability to sense the sidelink channel and updates the candidate sets of resources, which potentially assigned/granted for the Tx-UEs in its group.

In order to realize the assignment/grant of the set of resources to the Tx-UEs, the Co-UE can utilize either MAC-CE or SCI, containing the necessary information as follows:

- Source ID or group member ID associated with the Tx-UE, which is identified by the Tx-UE;

- A set of resources in both time and frequency domains.

The assistance information should contain the source ID associated with the Tx-UE, and a set of resources in both time and frequency domains.

Option 2: Based on a pre-defined or (pre)configured triggering condition(s)

With the option-2, a Co-UE may have available resources which are acquired based on either (pre)-configuration or a sensing procedure (in consideration of resource collision and half-duplex impact), and/or the Co-UE may measure the interference or predict/detect the information of resource collision which needs to be avoided in each resource pool operation. Then, the Co-UE considers such information as assistance information, informing all the associated Tx-UEs. The Tx-UE will take such assistance information into account and make the proper selection for the next potential transmission occasion accordingly.

The assistance information can be provided by the Co-UE to the Tx-UEs in the predefined manner, and the assistance information should be detectable by any other UEs.

Interpretation of “A Set of Resources”

In the e-meeting #102e, the proposal-1 has been summarized as follows:

- When a set of resources determined at UE-A is sent to UE-B in mode 2 and UE-B takes this into account in the resource selection for its own transmission,

  - for the definition of “a set of resources”, at least followings can be considered:

    - Resource set which is preferred for UE-B’s transmission

      - e.g.,

        - Resource set which is preferred for UE-A’s reception

        - Resource set which is preferred for intended receiver(s) of UE-B’s transmission

    - Resource set which is preferred not to be used by UE-B’s transmission

      - e.g.,

        - Resource set which is not preferred for UE-A’s reception

        - Resource set with a problem for intended receiver(s) of UE-B’s transmission

  - FFS: whether the “resource set” in above candidates can individually refer to the resources in the past, in the future, or in both past and future.

  - FFS details on how UE-A determines “a set of resources” in the above definitions of “a set of resources”.

  - FFS details on signaling of “a set of resources”, including container used for carrying it either at the physical or at higher layers and including time domain behavior (e.g., periodic, aperiodic, semi-persistent).

  - FFS relation between “a set of resources” and resource pool.

  - FFS how/when UE-B takes “a set of resources” into account in the resource selection for its own transmission.

  - FFS whether/how to support other assistance and/or coordinating information.

  - FFS if “inter-UE coordination” is supported in all cast types.

  - Note: further discussion is necessary on what definitions of “a set of resources” will be finally specified.

- FFS whether/how to handle an impact, if any, caused by the functionality of power consumption reduction to be introduced.

In response to the above threads, we express our views and selectively propose the corresponding solutions.

The definition of “a set of resources”

As we discussed in the last e-meeting, it has been summarized that the resource set should be preferred either for UE-B’s transmission, or for UE-B not to use.

In the former, the resource set can be further classified into the resource set which is preferred for UE-A’s reception, and the resource set which is preferred for intended receiver(s) of UE-B’s transmission. We believe that this classification of resource sets should be supported.

Rel-17 supports the definition of resource set which is preferred for UE-B’s transmission, with the further classification:

  - Resource set which is preferred for UE-A’s reception;

  - Resource set which is preferred for intended receiver(s) of UE-B’s transmission.

In the latter, the resource set can be further classified into the resource set which is not preferred for UE-A’s reception, and the resource set with a problem for intended receiver(s) of UE-B’s transmission. We believe that this classification of resource sets should be supported as well. In what follows, we express our view on those classified resource sets.

Resource set which is not preferred for UE-A’s reception

This kind of notification can be used to avoid the half duplex issue. For example, if a UE A can predict its own transmission time in the future, e.g., in the case where the transmission is periodic, it can generate a set of such resources and notify them to other UEs by broadcast. Then, as shown in Figure 8, UE B has packets to be transmitted to the intended receiver UE A, while UE A has reserved two sets of resources; the resource set #1 targeting for UE B and the resource set #2 targeting for UE C. In this case, UE B should avoid using the same slot in the resource set #1 and resource series #2 if reported from UE A, and mitigating the half duplex issue incurred at UE A.

Figure 8: UE A is the destination UE.

However, when UE B’s intended receiver is not UE A, a different behaviour of UE B should be considered. For example, as shown in Figure 9, only the same slot in resource set #1 targeting for UE B should be avoided so as to mitigate the half duplex issue incurred at UE B; Another case is shown in Figure 10, the half-duplex issue no longer occurs in both UE A and UE B, and in this case, then, the over-exclusion should be avoided.

Figure 9: UE A is not the destination UE.

Figure 10: UE A is not the destination UE.

Resource set which is not preferred for UE-A’s reception can be used to avoid the half duplex issue.

  - FFS: the details on the behavior of UE B about how to take this kind of resource into account during the resource (re)selection procedure, in consideration of the different intended receiver(s).

UE A behaviour to determine a set of resources

For a resource identified by UE A’s sensing procedure, the determination of the resource included in the reported “resource set” should always be clarified. In our opinion, a straightforward way is based on the RSRP measurement result, i.e., for a resource, if the corresponding RSRP value measured by UE A is higher/less than a threshold, the resource should be included in the reported “resource set” and indicated from UE A. To determine the value of this threshold, it can be considered to reuse the priority combination pi, pj , where pi is the priority value received in the SCI format 1-A and  pj is the priority value indicated by the request signaling from UE B. Note that, in this case, we assume that the report procedure triggered by UE B is based on the triggering or requesting mechanism.

For a resource identified by UE A’s sensing procedure, the decision of the report from UE A depends on the level of the corresponding RSRP measurement result in comparison with the RSRP threshold.

  - The indicated priority in the triggering/requesting signaling should be used as prioTX to determine the RSRP threshold, when the reporting is based on signaling of triggering or requesting.

Signalling of “a set of resources”

The message associated with a set of resources provided by Co-UE (or UE-A as described in WID) to Tx-UE may at least contain the information as follows:

- Destination ID: Co-UE informs the assistance information associated with this destination ID to Tx-UEs. This implies that only Tx-UE(s) belonging to the destination ID takes the assistance information into consideration. The destination ID may be not necessary if the assistance information is shared by all the UEs.

- Source ID(s): The source ID belongs to the Tx-UE, which is going to potentially utilize the given assistance information if a Co-UE informs the assistance information to a specific Tx-UE. A Co-UE could include multiple source IDs if the given assistance information is potentially utilized by multiple Tx-UEs.

- A set of resources in both time and frequency domains: This information can be informed by Co-UE to Tx-UE, which may either directly select it as transmission resource, or select part of a set for its transmission resource. The acquiring of this information may reduce the burden in the resource selection procedure at Tx-UE, which is required for power saving, etc.

- Coordinative information: This information can be informed by a Co-UE to Tx-UE(s), which may take it into account for resource collision and/or half-duplex avoidance. It is intended to enable the status detection of resource collision and/or half-duplex occurring either prior to or posterior to the initial transmission.

The assistance information message provided by a Co-UE to a Tx-UE may contain at least the following information; destination ID, source ID(s), a set of resources for transmission, and some other coordinative information for resource collision and/or half-duplex avoidance.

Besides the set of resources reported from UE A to UE B, the information of RSRP and priority (prioRX) can also be reported. Based on sensing, UE A can obtain the information of the reserved resources, the associated RSRPs and the associated priorities. If UE A reports all these sensing results to UE B, UE B can use them for resource selection as if it owns the sensing results by itself. Equivalently, it can be imagined that the sensing range of UE B has been extended, and the additional sensing results can be collected. One usage of reporting sensing results is to address the hidden node issue. More specifically, a hidden node generates interference towards RX UE A and cannot be sensed by TX-UE B. When provided with the sensing results from RX-UE A, TX-UE B can make a better resource selection to avoid the interference from a hidden node towards UE A. With RSRP and priority reported, UE B can increase the RSRP threshold when the number of candidate resources is too smaller. This provides UE B with more freedom to decide whether a reported resource is excluded or not during the resource selection. The details on signalling RSRP and priority can be further studied. Anyway, the reported RSRP and priority should be able to indicate when the related candidate resource will be excluded from the candidate resource set. As shown in Figure 11, the resource exclusion is performed iteratively where the RSRP threshold is increased by 3dB for each iteration. If Th+3 <RSRP≤Th+6, the related candidate resource is excluded on the 1st, 2nd iterations but not excluded on the 3rd, 4th iterations. In this case, the largest iteration number for resource exclusion is Imax=2. What TX-UE B really cares is this largest iteration number Imax. If RX-UE A knows the RSRP threshold, it can be considered that RX-UE A directly indicates the largest iteration number Imax to TX-UE B.

Figure 11: The reported RSRP and priority determines when the resource should be excluded.

The assistance information transmitted from UE A to UE B can indicate the sensing results of UE A including the reserved resources, the associated RSRPs and the associated priorities.

A set of resources is considered as a coordinative resource unit in time and frequency domain in order to avoid the problems of hidden-node, exposed-node, half-duplex between Tx-UEs. The set of resources associated with a Tx-UE could be broadcasted to all the Tx-UEs via SCI or MAC-CE in order to avoid the resource collision. Alternatively, the set of resources associated with a Tx-UE can also be unicasted to a specific Tx-UE via a new PC5-Signaling to update the set of resources related to RRC parameters. The determination of resource set may rely on either the dynamic grant mechanism or the configured grant mechanism, either CG type-1 or CG type-2.

If the dynamic grant mechanism is employed, each Co-UE shall sense the channel, including the resources utilized or reserved by Tx-UEs, and the resources granted by other Co-UE. Alternatively, Co-UEs can exchange the candidate sets of resources potentially granted to Tx-UEs, by establishing a unicast link between Co-UEs. The exact set of resources granted for Tx-UE are indicated by either SCI, or MAC CE.

If the configured grant mechanism is employed, each Co-UE shall exchange the configured multiple sets of resources potentially granted for Tx-UEs, by establishing a unicast link between Co-UEs. The multiple sets of resources can be determined by RRC parameter, with the set ID, indication of time and frequency domain resources. The exact resource in use for Tx-UE are indicated by either SCI, MAC CE, or RRC signaling.

Until now, we haven’t discussed how many resources can/should be reported to the UE-B. We believe that the more the resources reported, the better inter-UE coordination that can be realized. This, however, results in a higher signalling overhead. Thanks to the default functionality of resource selection in mode-2, the UE-B is always able to autonomously select the resource if necessary, and thus, the UE-B does not need to enforce the UE-A to report the resource sets or all the resources which meet the criteria for reporting.

A simple way is to impose a limitation on the number of reported resources; namely, the UE-A should skip to report the resources if the report condition hits the limitation ceiling. To this end, the resources potentially for reporting can be prioritized, whereby the UE-A could efficiently pick up the higher priority resources and finalize the report accordingly.

It should be studied whether/how to limit the number of resources transmitted from UE A to UE B.

Resource selection for its own transmission

Since the set of resources granted for each Tx-UE is broadcasted by the Co-UE, all the Tx-UEs are aware the information of the granted resources if in proximity. This offers more dimensions for the channel sensing and reduces the resource collision in mode-2 transmission.

In the Tx-UE, MAC entity can select the resource for its transmission as

- If the set of resources is not granted for the Tx-UE, MAC entity shall trigger the resource selection procedure.

- If the set of resources is granted for the Tx-UE, MAC entity can simply select the resource from the set.

  - In this case, MAC entity can trigger the resource selection procedure as well, but additionally considering the information of granted set of resources. This may offer more precise resource selection from the granted set by the Tx-UE.

To trigger the resource selection procedure, in slot n, MAC layer provides the following parameters for this PSSCH/PSCCH transmission:

- the resource pool from which the resources are to be reported;

- the set of resources granted for its own Tx-UE by the Co-UE within the above resource pool if any;

- the sets of resources granted for other Tx-UEs by the Co-UE within the above resource pool if any;

- L1 priority;

- the remaining packet delay budget;

- the number of sub-channels to be used for the PSSCH/PSCCH transmission in a slot;

- optionally, the resource reservation interval.

The same procedure for the Tx-UE as in NR-V2X is taken for the resource selection, except for the sensing procedure that the Tx-UE shall additionally exclude any candidate resources, granted for the other Tx-UEs by the Co-UE. The exclusion procedure may follow the rule of priorities of sensing resources; for example, the higher the priority on the reserved resources by SCI in each TB transmission, the lower the priority on the granted set of resources by the Co-UE.

Rel-17 supports the same resource selection procedure except for the sensing procedure that the Tx-UE shall additionally exclude any candidate resources, granted for the other Tx-UEs by the Co-UE.

  - MAC layer should additionally provide PHY layer the sets of resources granted for other Tx-UEs by the Co-UE within the expected resource pool.

As for the behaviour of UE B, it should be studied how UE B performs resource selection when receiving the assistance information from multiple reporting UEs. For example, given a case where UE B transmits the packets to multiple UEs in a broadcast/groupcast manner by utilizing the sensing results reported from these UEs, it should be considered how to utilize these sensing results during resource selection. Since the different reporting UEs may have the different levels of tolerance for interference, it would be more desirable that the RSRP thresholds used in the resource selection are updated independently for different reporting UEs. One example is shown in Figure 12 where UE B receives assistance information from UE A and UE C. Updating the RSRP threshold potentially means the resources with higher interference can become candidate resources. Considering a “near” RX UE (UE A) can endure higher interference than that of a “far” RX UE (UE C), updating the RSRP thresholds simultaneously may introduce excessive interference to a “far” RX UE. By letting the RSRP thresholds be independently updated, UE B can select more compromised resources which are acceptable for both “near” and “far” UEs.

Figure 12: UE B receives assistance information from UE A and UE C.

If a UE receives sensing results from multiple reporting UEs, the RSRP thresholds used in resource selection should be updated independently for different reporting UEs.

Another behaviour of UE B can be considered is, in both LTE Rel-14 and NR Rel-16 sensing procedure, all the candidate resources within the selection window shall be excluded as long as the corresponding slots have not been monitored. With the service of the periodic traffic implemented by the higher layer parameter sl-ResourceReservePeriodList, for example, the slots on which the Tx-UE performs transmission cannot be monitored due to the half-duplex issue. In this case, however, too many resources may be excluded if the UE has performed the frequent transmissions within the sensing window, especially when a large number of short period packets (e.g., less than 100ms) are allowed. A feasible method to reduce the excessive exclusion caused by unmonitored slots is, if the resources reported from UE A includes the sensing results in one or more unmonitored slots of UE B, UE B can take these sensing results of UE A into account during its own resource exclusion procedure instead of excluding all of the candidate resources within the corresponding slots in the selection window. In our opinion, whether this mechanism can work should depend on the geographical relation between UE A and UE B. Figure 13 illustrates an example, where UE B finds UE A in proximity, by the geographical location notified from UE A or the RSRP measurement result of UE A, UE B can take into the consideration of the sensing results in the unmonitored slots reported from UE A.

Figure 13: Illustration of UE B take the sensing results reported from UE A of the unmonitored slots into account.

UE B can take into the consideration of the sensing results in its unmonitored slots reported from UE A when UE B finds UE A in proximity.

  - UE B can find whether UE A is adjacent by the geographical location notified by UE A or the RSRP measurement result of UE A.

Support of assistance and/or coordinating information

In inter UE coordination, in general, Co-UE informs Tx-UE(s) the assistance information relevant to the (pre)-configured resource pool. In NR-V2X, since each resource pool can be shared with different cast services, it is not efficient for RAN1 to develop a solution associated with UE coordination, by isolating the different UEs who are experiencing in different casts. The assistance information potential utilized for specific Tx-UE should be known by other Tx-UEs who are performing different cast services. Therefore, a unified mechanism of informing the assistance information from the Co-UE to the Tx-UEs should be designed, without the limitation to cast types.

In order to realize the universal inter-UE coordination within the resource pool, a unified mechanism for informing the assistance information from a Co-UE to Tx-UEs should be designed, without the limitation to cast types.

A UE as a coordinating UE may be either (pre)-authorized or self-authorized, depending on the deployment scenario. Due to the mobility issue, multiple Co-UEs may be necessary to be involved in the coordination among UEs. More efficiently, the Co-UEs may need to establish a unicast link to exchange the assistance information, so as to avoid the conflict of assistance information issued by the multiple Co-UEs. This does not imply that, nevertheless, the more the Co-UEs involved in UE coordination, the better the UE coordination. One reason is that, with more Co-UEs involved, more resources need to be dedicated for assistance information delivery, incurring the additional transmission burden and the impact on the overall performance. The other reason is that, with more Co-UEs involved, a potential Tx-UE may receive multiple assistance information messages, resulting in a message flood and making the Tx-UE confused in the resource selection.

If a larger number of Co-UEs is (pre)-authorized or self-authorized, additional transmission burden occurs and a potential Tx-UE may receive multiple assistance information messages, resulting in the message flood and making Tx-UE confused in resource selection.

Due to the mobility issue, multiple Co-UE (pre)-authorization procedure should be studied; including how many Co-UEs properly involved in the UE coordination, and what types of messages exchanged between Co-UEs.

Cast types in inter-UE coordination

Groupcast, in general, requires similar reliability to unicast, but merely has fewer available mechanisms to support this. For example, the MAC entity always supports Option-2 HARQ in unicast, while MAC entity supports both Option-1 HARQ and Option-2 HARQ in groupcast but with some conditions. Option-2 HARQ is available for use only when the V2X application layer provides group size and member IDs with a small number of members within the group. The V2X application layer, in addition, should provide accurate and up-to-date information on the group size and the member ID. Hence, the reliability of groupcast in MAC layer mainly relies on the Option-1 HARQ mechanism.

Furthermore, the RLC layer supports UM/AM in unicast, but only supports UM in groupcast. Thus, the fulfilment of the reliability for groupcast merely relies on HARQ in the MAC layer. This makes the enhancements more important in Option-1 and/or Option-2 HARQ for groupcast communication in Rel-17.

The MAC entity always supports Option-2 HARQ in unicast, while the MAC entity supports both Option-1 HARQ and Option-2 HARQ in groupcast but with some conditions.

The RLC layer supports UM/AM in unicast, but only supports UM in groupcast.

The reliability in groupcast shall be enhanced in MAC layer in Rel-17, in order to fulfil the stringent requirements in groupcast communication.

The architecture enhancements in 5GS [2] defines two types of groups; one is denoted as application layer connection-less group, and the other is denoted as application layer managed group. The former considers an application layer group without group formation in the V2X application layer, e.g., sensor sharing. The latter, however, considers an application layer group with group formation and management in the V2X application layer, e.g., platooning, cooperative adaptive cruise control, only in case that the V2X application layer provides a group size and a member ID, and the V2X layer passes them to the AS layer for groupcast control.

As the requirements [3], the application of sensor sharing requires a high reliability and a low latency, for example, 99.99-99.999% reliability and 3-10ms latency if a fully automated driving is applied. This imposes that Option-1 HARQ should be enhanced with some additional mechanism.

In addition, the communication range in NR-V2X groupcast is up to 1000m, which is indicated by the 2nd SCI. The communication range, of course, can be extended by UE-to-UE relay. However, the relay-UE discovery problem involved in groupcast incurs due to the uncertainty of Tx-UE in each transmission, possibly resulting in V2X relay message transmission flood. Therefore, the extension of communication range should be performed from RAN1 perspective.

The cast type used for MAC-PDU transmission is determined in the application layer based on the service type, while the cast type used for transmitting the assistance information from the Co-UE is determined in the MAC layer dependent on the performance achieved by inter-UE coordination. Thus, the cast type used for transmitting assistance information for the Co-UE should be considered differently from that used for MAC-PDU transmission from the Tx-UE.

Our proposal related to the cast types in use when the Tx-UE triggers the request to the Co-UE and when the Co-UE transmits the assistance information to the Tx-UE is as follows:

- For the request from the Tx-UE, the Tx-UE can use the same cast communication applied for the current service with the same destination ID, and reduce the cost of sidelink establishment. Especially, the unicast is reliable but at the cost establishment in many PC5 links. In this case, source L1/L2 ID in the request can be identified by the Co-UE, in any cast.

- For the grant from the Co-UE, the Co-UE broadcasts the granted set of resource to Tx-UE regardless of current cast type. The Co-UE, always broadcasting MAC-CE (indicated by a dedicated LC-ID) or SCI (indicated by a new SCI format), at lease contains the source ID belonging to the Tx-UE (it may not need destination L2 ID for transmission). The reason to employ the broadcast for granting the set of resources to the Tx-UEs is to let all other potential Tx-UEs be aware the information of the granted set of resources, and offer more dimensions for resource selection and/or channel sensing and reduce the resource collision in mode-2, by modifying the step-1 and step-2 in the (re)selection procedure.

For the request from the Tx-UE, the Tx-UE should use the same cast communication applied for the current service with the same destination ID, while for the grant from the Co-UE, the Co-UE always broadcasts the granted set of resource to the Tx-UE by at least containing the source ID regardless of current cast type.

  - Either MAC-CE (indicated by a dedicated LC-ID) or SCI (indicated by a new SCI format) can be utilized for both request and grant.

Conclusions

In this contribution, we have expressed our views on potential solutions towards the enhancement of reliability and the reduction of latency. In summary, we have following list of the observations and the proposals for NR sidelink:

Observation 1: The PRR degradation due to the half-duplex issue is about 1~2% in groupcast, depending on the how many Tx-UEs simultaneously transmit TBs in a slot. The larger the number of Tx-UEs in a slot, the bigger the impact of half-duplex.

Observation 2: If a larger number of Co-UEs is (pre)-authorized or self-authorized, additional transmission burden occurs and a potential Tx-UE may receive multiple assistance information messages, resulting in the message flood and making Tx-UE confused in resource selection.

Observation 3: The MAC entity always supports Option-2 HARQ in unicast, while the MAC entity supports both Option-1 HARQ and Option-2 HARQ in groupcast but with some conditions.

Observation 4: The RLC layer supports UM/AM in unicast, but only supports UM in groupcast.

Proposal 1: The assistance information for inter-UE coordination should be studied to solve the hidden node problem.

Proposal 2: The assistance information for inter-UE coordination should be studied to solve the exposed-node problem.

Proposal 3: Rel-17 supports enhancement of Option-1 HARQ process and mitigation of the half-duplex impact, relying on inter-UE coordination mechanism.

Proposal 4: The Co-UE should coordinatively transmit SFCIs for the Tx-UEs instead, if the Co-UE detects the half-duplex problem occurred in two Tx-UEs.

Proposal 5: The Co-UE should coordinatively forward both TBs received from two Tx-UEs, if the Co-UE detects the half-duplex problem occurred between two Tx-UEs, relying on the same resource reserved in the SCI of initial transmission.

Proposal 6: The assistance information for inter-UE coordination should be studied to avoid consecutive packet loss.

Proposal 7: The assistance information can be provided by the Co-UE to the Tx-UE in a triggered manner, and the assistance information should be detectable by any other UEs.

Proposal 8: The assistance information should contain the source ID associated with the Tx-UE, and a set of resources in both time and frequency domains.

Proposal 9: The assistance information can be provided by the Co-UE to the Tx-UEs in the predefined manner, and the assistance information should be detectable by any other UEs.

Proposal 10: Rel-17 supports the definition of resource set which is preferred for UE-B’s transmission, with the further classification:

a. Resource set which is preferred for UE-A’s reception;

b. Resource set which is preferred for intended receiver(s) of UE-B’s transmission.

Proposal 11: A Resource set which is not preferred for UE-A’s reception can be used to avoid the half duplex issue.

a. FFS: the details on the behavior of UE B about how to take this kind of resource into account during the resource (re)selection procedure, in consideration of the different intended receiver(s).

Proposal 12:  For a resource identified by UE A’s sensing procedure, the decision of the report from UE A depends on the level of the corresponding RSRP measurement result in comparison with the RSRP threshold.

a. The indicated priority in the triggering/requesting signaling should be used as prioTX to determine the RSRP threshold, when the reporting is based on signaling of triggering or requesting.

Proposal 13: The assistance information message provided by a Co-UE to a Tx-UE may contain at least the following information; destination ID, source ID(s), a set of resources for transmission, and some other coordinative information for resource collision and/or half-duplex avoidance.

Proposal 14: The assistance information transmitted from UE A to UE B can indicate the sensing results of UE A including the reserved resources, the associated RSRPs and the associated priorities.

Proposal 15: It should be studied whether/how to limit the number of resources transmitted from UE A to UE B.

Proposal 16: Rel-17 supports the same resource selection procedure except for the sensing procedure that the Tx-UE shall additionally exclude any candidate resources, granted for the other Tx-UEs by the Co-UE.

a. MAC layer should additionally provide PHY layer the sets of resources granted for other Tx-UEs by the Co-UE within the expected resource pool.

Proposal 17: If a UE receives sensing results from multiple reporting UEs, the RSRP thresholds used in resource selection should be updated independently for different reporting UEs.

Proposal 18: UE B can take into the consideration of the sensing results in its unmonitored slots reported from UE A when UE B finds UE A in proximity.

Proposal 19: In order to realize the universal inter-UE coordination within the resource pool, a unified mechanism for informing the assistance information from a Co-UE to Tx-UEs should be designed, without the limitation to cast types.

Proposal 20: Due to the mobility issue, multiple Co-UE (pre)-authorization procedure should be studied; including how many Co-UEs properly involved in the UE coordination, and what types of messages exchanged between Co-UEs.

Proposal 21: The reliability in groupcast shall be enhanced in MAC layer in Rel-17, in order to fulfil the stringent requirements in groupcast communication.

Proposal 22: For the request from the Tx-UE, the Tx-UE should use the same cast communication applied for the current service with the same destination ID, while for the grant from the Co-UE, the Co-UE always broadcasts the granted set of resource to the Tx-UE by at least containing the source ID regardless of current cast type.

a. Either MAC-CE (indicated by a dedicated LC-ID) or SCI (indicated by a new SCI format) can be utilized for both request and grant.

References

- 3GPP TSG RAN #89e, RP-201516, WID revision: NR sidelink enhancement, Electronic Meeting, September 14 - 18, 2020.

- 3GPP TS 23.287, Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services, Release 16, March 2020.

- 3GPP TS 22.886, Study on enhancement of 3GPP Support for 5G V2X Services, Release 16, December 2018.

Annex

The system level simulation is performed based on the simulation assumptions, listed in Table 1.

Table 1: SLS simulation assumptions.

| Attributes | Values or Assumptions |
| --- | --- |
| Number of drops | 30 |
| Simulation length | 5000[slots](5s) + warmup(8000[slots]) |
| Scenario | Base on Highway case of 3GPP TR 37.885 V15.1.0 |
| Channel model | Pathloss：Table 6.2.1-1 of TR 37.885 Shadowing：STD 3dB, Decorrelation distance 25m Fast fading：Section 6.2.3 in TR 37.885 |
| Speed of vehicle | 140km/h |
| The distance between the rear bumper of a vehicle and the front bumper | Average 2.0[s] with min 2[m] (vehicle length 5[m]) |
| Average number of vehicles | 145 on average, in 6 lanes |
| SINR calculation interval | 1 RB, 1 slot |
| Carrier frequency | 5.9[GHz] |
| Bandwidth | 20/40[MHz] (100/200RBs, 1200/2400subcarriers) |
| Subcarrier spacing | 15[kHz] |
| Slot length | 1[ms] (14symbols) |
| Transmission power | 23[dBm] |
| TX Antenna Configuration | 1 antenna |
| RX Configuration | 4 antennas with λ/2 spacing |
| Antenna pattern | Omnidirectional |
| Antenna height | 1.6 [m] (option A, type 2) |
| Antenna gain | 3 [dBi] |
| Noise figure | 9 [dB] |
| Number of DMRS | 4 |
| Subchannel type | PSCCH+PSSCH scheme |
| Size of sub-channel | 50RB f |
| Modulation and Code rate Error curve type of PSCCH | QPSK, Polar coding |
| Modulation and Code rate Error curve type of PSSCH | 16QAM, LDPC 800byte: 50RB, R=0.3024 1200byte: 50RB, R=0.4302 |
| Traffic mode | Traffic: medium traffic intensity Periodic traffic: - traffic generate vehicle rate：50% - Inter-packet arrival time: 50 ms for 20MHz - Inter-packet arrival time: 25 ms for 40MHz - Packet size: 1200 bytes with probability of 0.2 and 800 bytes with probability of 0.8 - Latency requirement: 50 ms |
| Resource selection scheme | - Mode-2 (with SCI decoded, PSSCH-RSRP) |
| Threshold for excluding SCI decoded resources | -128[dBm] |
| SL_RESOURCE_RESELECTION_COUNTER | 1. The counter decremented by one after every transmission 2. Resource reselection is triggered if the counter reaches to zero (probResourceKeep=0.8) |
| Repetition | Chase combining with the same number of sub-channels as initial Tx |