3GPP TSG RAN WG1 Meeting #103-e		                                    R1-2007863

e-Meeting, October 26th – November 13th, 2020

Source:                CATT

Title:	Discussion on PDCCH monitoring reduction

Agenda Item:	8.6.2

Document for:	Discussion and Decision

Introduction

At RAN1#102 e-meeting, PDCCH monitoring reduction was extensively discussed which focus on the evaluation assumptions. The agreements achieved in the last meeting are attached in the appendix.  In this contribution, we provide our views on the PDCCH monitoring reduction for RedCap.

Discussion

Currently, the search space configuration is very flexible and can achieve any intended number of PDCCH candidates. A search space consists of one or multiple PDCCH candidate(s) which have same or different aggregation level(s). The IE to configure the PDCCH candidates for a search space is shown below.

nrofCandidates                          SEQUENCE {

aggregationLevel1                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel2                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel4                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel8                       ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8},

aggregationLevel16                      ENUMERATED {n0, n1, n2, n3, n4, n5, n6, n8}

}                                                                                                   OPTIONAL,   -- Cond Setup

In an extreme case, a search space can only include one PDCCH candidate corresponding to a certain aggregation level. From this perspective, the smaller number of blind decodes and CCE limits can be achieved by the existing configuration. However, it does not help a RedCap UE to reduce the complexity and cost as it still has to support the maximum number of BD and non-overlapping CCEs as legacy NR UEs.

In Rel-15, the UE PDCCH monitoring capability defined in terms of maximum number of BD and maximum number of non-overlapped CCEs per slot as in the following tables defined in TS38.213 is mandatory. In Rel-16, the PDCCH monitoring capability is further enhanced in order to satisfy the requirement of high reliability and low latency.

It is well-known that the PDCCH monitoring capability plays an important role in the UE complexity:

- Blind decoding consumes lots of processing power at UE side, i.e. PDCCH detection on each PDCCH candidate, DCI content parsing, etc.

- The channel estimation for NR-PDCCH is handled in terms of REG-bundle, which constitutes NR-CCE. The more non-overlapping CCEs configured for a RedCap UE, the more complicated for channel estimation.

Considering that the requirements for the reduced capability NR devices are lower than URLLC and eMBB, it is reasonable to introduce a smaller maximum number of PDCCH candidates per slot and a smaller maximum number of non-overlapped CCEs per slot for reduced capability NR devices than those of Rel-15. Accordingly, the gNB can only configure the search space for reduced capability NR devices subject to the PDCCH monitoring capability defined for reduced capability NR devices.

Observation 1: The reduction on maximum number of BDs and non-overlapping CCEs is beneficial for reducing RedCap UE complexity and cost.

Table 1: Maximum number  of monitored PDCCH candidates per slot for a DL BWP with SCS configuration  for a single serving cell [2]

|  | Maximum number of monitored PDCCH candidates per slot and per serving cell |
| --- | --- |
| 0 | 44 |
| 1 | 36 |
| 2 | 22 |
| 3 | 20 |

Table 2: Maximum number  of non-overlapped CCEs per slot for a DL BWP with SCS configuration  for a single serving cell [2]

|  | Maximum number of non-overlapped CCEs per slot and per serving cell |
| --- | --- |
| 0 | 56 |
| 1 | 56 |
| 2 | 48 |
| 3 | 32 |

Furthermore, reduction on power consumption is a fundamental requirement for RedCap use cases. In Rel-16, the relationship between BD/CCE reduction and power saving has been extensively studied. In short, the power saving gain obtained from the BD reduction can be expressed by the following formula compared to the baseline, i.e. the PDCCH-only power for same slot scheduling:

P(α) = α ∙ Pt + (1 – α) ∙ 0.7Pt

α is the ratio of PDCCH candidates to the max number of PDCCH candidates in the reference configuration (α>0) and Pt is the PDCCH-only power for same-slot scheduling. To be specific, the power saving benefits from BD/non-overlapping CCE reductions compared to the current per slot limit in Rel-15 is shown in the following table.

Table 3: Power saving gain obtained from BD reduction assuming SCS=15 kHz

| Baseline(BD) | ~20% BD reduction |  | ~40% BD reduction |  | ~60% BD reduction |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Reduced limit | Power saving gain | Reduced limit | Power saving gain | Reduced limit | Power saving gain |
| 44 | 351 | 6% | 26 | 12.3% | 17 | 18.3% |

Note1: Floor operation is executed when the number of BDs after scaling is not integer.

There is no doubt that power saving gain can be harvested from BD reduction. However, it should also be noticed that less than 20% power consumption reduction can be achieved even if the maximum number of BD is reduced by 60%.

It should be noted that the aforementioned analyses are based on theoretical assumptions wherein a UE only needs to monitor PDCCH in every configured monitoring occasion. Realistic factors, e.g. DRX configuration, traffic type, scheduling, measurement, cannot be reflected which certainly impacts our impression on how much power consumption gain we can obtain from PDCCH monitoring reduction. In Table 4, we provide the simulation results for the following four cases:

- Case1: TDD in FR1, 1 RX is assumed

- Case2: TDD in FR1, 2 RX is assumed

- Case3: TDD in FR2, 1 RX is assumed

- Case4: TDD in FR2, 2 RX is assumed

For each case, different traffic models, i.e. instant messaging, heartbeat and VoIP, are evaluated. The simulation parameters adopted in the email discussion [102-e-Post-NR-RedCap-01] are applied in the simulation.

Table 4: Power saving gain obtained from BD reduction with considering realistic parameters

| Traffic | Scenarios | TDD, 1 Rx, @FR1 | TDD, 2 Rx, @FR1 | TDD, 1 Rx, @FR2 | TDD, 2 Rx, @FR2 |
| --- | --- | --- | --- | --- | --- |
| Instant messaging | Power saving gain at 25% reduction in BDs | 1.8300% | 2.1650% | 4.5341% | 4.8061% |
|  | Power saving gain at 50% reduction in BDs | 3.6700% | 4.1150% | 9.0696% | 9.6111% |
| Heartbeat | Power saving gain at 25% reduction in BDs, 200 ms inactivity timer | 1.1000% | 1.3000% | 2.9649% | 3.3408% |
|  | Power saving gain at 50% reduction in BDs, 200 ms inactivity timer | 2.1960% | 2.6050% | 5.9299% | 6.6789% |
|  | Power saving gain at 25% reduction in BDs, 80 ms inactivity timer | 1.0400% | 1.2330% | 2.7512% | 3.1160% |
|  | Power saving gain at 50% reduction in BDs, 80 ms inactivity timer | 2.0750% | 2.4608%% | 5.4988% | 6.0640% |
| VoIP | Power saving gain at 25% reduction in BDs | 0.9000% | 1.1580% | 2.8823% | 3.1934% |
|  | Power saving gain at 50% reduction | 1.8200% | 2.3170% | 5.7647% | 6.3869% |

It can be observed that the power saving gain from PDCCH monitoring reduction becomes even more insignificant if taking more realistic parameters into consideration.  Generally speaking, PDCCH monitoring reduction can bring more power saving gain for instant messaging as the packet comes randomly and the total power consumption is much lower than heartbeat and VoIP. We have the following detail observations from the evaluation results:

- The power saving gain from PDCCH monitoring reduction is different for different traffics.  The maximum power saving gain (assuming 50% PDCCH monitoring reduction) can be around 4%, 2.6%, 2.4% for instant messaging, heartbeat and VoIP respectively in FR1, and 9.6%, 6.7%, 6.4% respectively in FR2.

  - The packet distribution time and DRX configuration will significantly impact the overall power consumption.

- The power saving gain from PDCCH monitoring reduction is rarely impacted from the number of Rx.

- The power saving gain from PDCCH monitoring reduction is more significant in FR2 wherein the power consumption for PDCCH monitoring is more remarkable than that of FR1.

The power saving gain from PDCCH monitoring would be diverse depending on the traffic model, frequency range, DRX configuration, etc. The maximum power saving gain is less than 10% with reducing 50% PDCCH blind detection. On the other hand, the reduction on the maximum number of BD will significantly increase the PDCCH blocking possibility from UE perspective.

Observation 2: The benefits on power saving and the corresponding penalty of BD reduction should be carefully considered.

As described in SID, there are different use cases wherein the requirements are divergent. For example, the battery life of industrial wireless sensors should last at least few years while up to 1-2 weeks battery life is sufficient for wearable devices. Accordingly, the requirement on the reduction of PDCCH monitoring may be different. Therefore whether to determine multiple sets of PDCCH candidates and non-overlapped CCE limit targeting for different use cases should be further studied.

In SID document, the use case specific requirements are identified as follows:[1]

- Industrial wireless sensors: Reference use cases and requirements are described in TR 22.832 and TS 22.104: Communication service availability is 99.99% and end-to-end latency less than 100 ms. The reference bit rate is less than 2 Mbps (potentially asymmetric e.g. UL heavy traffic) for all use cases and the device is stationary. The battery should last at least few years. For safety related sensors, latency requirement is lower, 5-10 ms (TR 22.804)

- Video Surveillance: As described in TR 22.804, reference economic video bitrate would be 2-4 Mbps, latency < 500 ms, reliability 99%-99.9%. High-end video e.g. for farming would require 7.5-25 Mbps. It is noted that traffic pattern is dominated by UL transmissions.

- Wearables: Reference bitrate for smart wearable application can be 5-50 Mbps in DL and 2-5 Mbps in UL and peak bit rate of the device higher, up to 150 Mbps for downlink and up to 50 Mbps for uplink.  Battery of the device should last multiple days (up to 1-2 weeks).

As described above, the required reliability can be as high as 99.99%. In order to achieve the overall reliability, the reliability of PDCCH transmission should at least have a magnitude as 99.999%.  Furthermore, the number of Rx antennas may be reduced down to 1 and the antenna gain is deteriorated due to the small form size. The performance of PDCCH reception would be degraded remarkably. From this point of view, large aggregation level is still necessary.

Observation 3: A relaxed limit on the number of non-overlapped CCE for a reduced capability NR device may have some impacts on the PDCCH reliability.

We consider the case of burst traffic, where a huge number of RedCap UEs staying in the system.  Consequently, it can be regarded that the number of scheduled UEs in each slot on average is constant. In order to give a clear picture on the PDCCH blockage issue coming with RedCap UE, we further conduct the LLS and SLS. The target scenario is Urban with 4 GHz carrier frequency. The required SNR for aggregation level {1,2,4,8,16} with different number of Rx antennas is obtained from LLS as summarized in Table 5. Furthermore, the geometry curves corresponding to different antenna configurations in urban scenario are provided in the appendix. The detailed assumptions for LLS and SLS can also be found in the appendix.

In Table 8, we give some analyses on PDCCH blockage issue in urban scenario with different UE numbers. We assume a BWP size of 20MHz for RedCap UEs and the SCS is 15 kHz. We assume there is only one PDCCH monitoring occasion in one slot from system perspective, wherein each PDCCH monitoring occasion occupies 3 OFDM symbols. The maximum number of non-overlapping CCEs would be 3*100/6=50. For simplicity, we assume the number of BD is sufficient. The AL distribution in Table 6 is determined by geometry shown in the appendix with the SNR points provided in Table 5. The average CCE number for a PDCCH candidate in urban scenario assuming different number of Rx is shown in Table 7. The PDCCH blocking percentile and PDCCH capacity is shown in Table 8.

Table 5: Required SNR for different AL with diverse Rx configuration

| PDCCH BLER=1% | AL1 | AL2 | AL4 | AL8 | AL16 |
| --- | --- | --- | --- | --- | --- |
|  | Req. SNR [dB] | Req. SNR [dB] | Req. SNR [dB] | Req. SNR [dB] | Req. SNR [dB] |
| 4Rx | 2.2 | -1.65 | -5.82 | -8.30 | -10.15 |
| 2Rx | 8.68 | 3.87 | -0.50 | -4.83 | -7.56 |
| 1Rx | 16.03 | 10.47 | 3.76 | -1.02 | -4.06 |

Table 6: The aggregation distribution for different number of Rx

| PDCCH BLER=1% | AL1 | AL2 | AL4 | AL8 | AL16 |
| --- | --- | --- | --- | --- | --- |
| 4Rx | 70% | 23% | 4.5% | 1% | 1.5% |
| 2Rx | 42% | 18% | 27% | 10% | 3% |
| 1Rx | 20% | 12% | 29% | 29% | 6% |

Table 7: The required CCEs on average for different number of Rx

| Number of Rx at UE side | 1 Rx | 2 Rx | 4 Rx |
| --- | --- | --- | --- |
| Average number of CCEs for a PDCCH candidate | 4.88 | 3.14 | 1.51 |

Table 8: PDCCH blockage coming with RedCap UE

| Number of Rx Number of UEs | 4 Rx at UE side | 2 Rx at UE side | 1 Rx at UE side |
| --- | --- | --- | --- |
|  | Baseline blockage | Blockage | Blockage |
| 20 UEs per slot | 0% | 20.4% | 48.8% |
| 35 UEs per slot | 5.4 % | 54.5% | 70.7% |
| 40 UEs per slot | 17.2% | 60.2% | 74.4% |
| 45 UEs per slot | 26.4% | 64.6% | 77.2% |

From the above simulation results, we have the following observations:

- The reduction of the Rx antennas will increase the PDCCH blockage significantly

  - The required CCEs for a PDCCH candidate on average would increase, i.e. triple compared to 4 Rx if the Rx is reduced to 1 and double compared to 4 Rx if the Rx is reduced to 2.

- The numerous number of RedCap UE and the limited bandwidth also increase the PDCCH blocking possibility

Proposal 1: PDCCH blockage increase due to reduced number of Rx antennas of RedCap UE should be carefully studied.

As analyzed in the previous section, PDCCH blocking would become a significant issue for RedCap UE considering the explosive growth of UE number, the limited bandwidth and the more consumption on physical resources for RedCap PDCCH. It is critical to study the potential mechanisms for reducing PDCCCH blocking possibility. One straightforward way to reduce the blockage is to cut down the total number of required DCIs. Currently one DCI can only schedule a single PDSCH. If a single DCI can be used to schedule multiple PDSCHs corresponding to multiple UEs, the PDCCH blocking issue could be alleviated.  Furthermore, a search space can only be mapped within one CORESET in a slot, which means a PDCCH would be blocked if there is not empty candidate can be found in the search space. If the search space can be mapped to the CORESETs located in different slots, PDCCH blocking possibility can be reduced.

Figure 1: example for group scheduling and CORESET concatenation

Proposal 2: Group scheduling for multiple UEs and CORESET concatenation can be considered to reduce PDCCH blocking possibility.

Currently, the DCI payload size budget is 3+1, i.e. up to 3 different DCI sizes corresponding to C-RNTI/CS-RNTI/MCS-C-RNTI and 1 DCI size corresponding to other RNTI values. The various DCI payload sizes are friendly to support diverse scheduling functionality but also have the penalty of increasing the effort on blind decoding. There are three groups of DCI formats in Rel-16 for unicast scheduling, i.e. DCI format 0_0 and DCI format 1_0, DCI format 0_1 and DCI format 1_1, DCI format 0_2 and DCI format 1_2, and each DCI format may result in different payload sizes. If more than one DCI format can be configured to a RedCap UE, the RedCap UE may have to handle up to three different payload sizes. Subsequently, more blind decodings are needed for the same number of PDCCH candidate. In order to reduce the effort on PDCCH monitoring for RedCap UE, one straightforward way is to reduce the DCI payload size budget.

Assuming there are two search spaces configured in a slot for RedCap UE, i.e. one CSS associated with fallback DCI formats, one USS associated with DCI format 0_1/1_1. Assuming there are 7 and 15 available PDCCH candidates in the CSS and USS respectively. Then the RedCap UE has to handle 7+15*2=37 BDs in the slot. If the DCI payload size budget is reduced to 2+1, there would be unified DCI payload size for DCI format 0_1 and DCI format 1_1. Accordingly, the number of BD would be reduced to 22.

Proposal 3: The DCI payload size budget for RedCap UE can be considered to be reduced.

gNB configures the search space for a UE via semi-static signaling and UE handles the PDCCH monitoring according to the semi-static configuration. In other words, the PDCCH monitoring behavior would be the same unless the RRC configuration is updated. In order to guarantee there are enough spaces for PDCCH link adaptation, the search space configuration is typically redundant. For example, gNB may configure one or multiple PDCCH candidates with very large aggregation level in order to compensate the possible deep loss when channel condition is pretty bad. Another example may be gNB configures excessive PDCCH candidates than necessary to avoid PDCCH blocking in case of burst traffic. However, the channel condition is not always extremely bad and the traffic load is dynamically changing as well. It will be beneficial to further reduce PDCCH monitoring if the search space can be dynamically adapted based on the practical status. One example could be a search space which is configured with 20 PDCCH candidates in total, gNB can use a DCI carrying search space adaptation indication to shut down some of the candidates in the following search space. The detailed design can be found in our companion contribution [3].

Proposal 4: The dynamic adaptation on search spaces via DCI should be considered to further reduce the effort on PDCCH monitoring.

It can be observed that the requirement of reliability and latency are quite different for different RedCap use cases. The end to end latency can be relaxed to up to 500 ms. From this perspective, the values of PDCCH monitoring periodicity supported in Rel-15 and Rel-16 should be reconsidered. For example, at least mini-slot periodicity is not necessary. The restriction on smaller PDCCH monitoring periodicity can further reduce the effort on PDCCH monitoring and lead to lower power consumption accordingly. Furthermore, the subcarrier spacing should also be taken into consideration when determine the periodicity for reduced capability NR devices.

Proposal 5: PDCCH monitoring periodicity for reduced capability NR devices should be restricted on top of that supported in Rel-15 and Rel-16, e.g. more than one PDCCH monitoring occasion within 1 slot is not supported.

Conclusion

This contribution discusses possible PDCCH monitoring reduction solutions and raises some issues related to capability reduction and bandwidth reduction. We have the following observations:

Observation 1: The reduction on maximum number of BDs and non-overlapping CCEs is beneficial for reducing RedCap UE complexity and cost.

Observation 2: The benefits on power saving and the corresponding penalty of BD reduction should be carefully considered.

Observation 3: A relaxed limit on the number of non-overlapped CCE for a reduced capability NR device may have some impacts on the PDCCH reliability.

Furthermore, we propose the following proposals:

Proposal 1: PDCCH blockage increase due to reduced number of Rx antennas of RedCap UE should be carefully studied.

Proposal 2: Group scheduling for multiple UEs and CORESET concatenation can be considered to reduce PDCCH blocking possibility.

Proposal 3: The DCI payload size budget for RedCap UE can be considered to be reduced.

Proposal 4: The dynamic adaptation on search spaces via DCI should be considered to further reduce the effort on PDCCH monitoring.

Proposal 5: PDCCH monitoring periodicity for reduced capability NR devices should be restricted on top of that supported in Rel-15 and Rel-16, e.g. more than one PDCCH monitoring occasion within 1 slot is not supported.

References

  - RP-193238, “New SID on support of reduced capability NR devices”, Ericsson, RAN#86

  - TS38.213, V15.9.0

  - R1-2005721, “PDCCH monitoring adaptation”, CATT, RAN1#102-e

Appendix

Agreements achieved at RAN1#102 e-meeting:

Agreements:

- Use the VoIP traffic model from TR 38.840 as baseline. Other VoIP traffic models are not precluded and companies to report if other VoIP traffic models are assumed in evaluation.

Agreements:

For power saving evaluation of RedCap UEs:

- Reuse the Instant message traffic model from TR 38.840 as baseline. Other Instant traffic models based on FTP model 3 are not precluded and companies to report the mean inter-arrival time and packet size if other instant traffic models are assumed in evaluation.

- FFS: ‘heartbeat’ traffic model

Agreements:

- The scaling factor ‘0.7’ is used for 2 Rx to 1Rx power scaling for power reduction related evaluation.

- For evaluation, the power scaling for PDCCH candidate reduction defined in TR 38.840 is reused for Redcap UEs.

- For power consumption evaluation, the DRX configurations of Instant message and VoIP in TR 38.840 are reused.

- Discussion on reduced maximum number of configurable CORESET technique for power saving is deprioritized in the Redcap power saving sub-agenda

- For power consumption evaluation, use FTP-3 model with 100 Bytes packet size and 60s mean inter-arrival time as baseline for ‘heartbeat’ traffic.

- For power consumption evaluation, reuse the following DRX configuration defined in TS 38.840 for ‘heartbeat’ traffic model:

- C-DRX cycle 640 msec, inactivity timer {200, 80} msec

- FR1 On duration: 10 msec

- FR2 On duration: 5 msec

Agreements: For the PDCCH blocking rate evaluation, at least the following parameters are assumed as baseline:

| Parameters | Assumptions |
| --- | --- |
| Number of candidates for each AL | Each company to report. |
| SCS/BW | FR1: 30KHz/20MHz - 15kHz/20MHz is optional FR2: 120KHz/[100]MHz |
| CORESET duration | 2 symbols, with 3 symbols optional |
| Delay toleration (Slot) | 1 (1: implies that PDCCH is blocked if it can’t be scheduled in the given slot), with 2 optional |
| Aggregation level Distribution | Companies to report (including the necessary UE channel conditions and deployment scenario(s) for the aggregation level distribution) |

Agreements: For Redcap power consumption evaluation:

- Note that 2RX is assumed

| Power State | Alt.4a |
| --- | --- |
| Deep Sleep (PDS) | 0.8 |
| Light Sleep (PLS) | 18 |
| Micro sleep (PMS) | 31 |
| PDCCH-only (PPDCCH) | 50 for same-slot scheduling, 40 for cross-slot scheduling |
| PDCCH + PDSCH (PPDCCH+PDSCH) | 120 |
| PDSCH-only (PPDSCH) | 112 |
| SSB/CSI-RS proc. (PSSB) | 50 |
| Intra-frequency RRM measurement (Pintra) | · [60]Note4 (synchronous case, N=8, measurement only) · [80] Note4 (combined measurement and search) |
| Inter-frequency RRM measurement (Pinter) | [60] Note4 (neighbor cell search power per freq. layer) · [15080] Note4 (measurement only per freq. layer) · Micro sleep power assumed for switch in/out a freq. layer |

Working assumption:

Adopting the following rule for power determination

- Rule 1: ‘Micro sleep’ power of 1 Rx is [0.8]x2 Rx ‘Micro sleep’ power

- Rule 2: For both 1 Rx and 2 Rx configuration,

- P(α) = max (Micro-sleep, α ∙ Pt + (1 – α) ∙ 0.7Pt))

- Pt is the PDCCH-only power for same slot and cross-slot scheduling cases.

Conclusion: It is up to each company to report the power consumption modeling for 3-symbols CORESET configuration and reduced number of non-overlapped CCEs.

Simulation assumption for LLS and SLS:

Table A-1: LLS assumptions

| Parameters | Values |
| --- | --- |
| Scenario and frequency | 2.6G Hz |
| Subcarrier Space | 30KHz |
| UE velocity | 3km/h |
| System bandwidth | 100M, 20M |
| PDSCH duration | 12 OS |
| PDCCH duration | 2 OS |
| PDCCH AL | 1，2，4，8，16 |
| Tx-Rx | 1T - 1R/2R/4R |
| Channel Model | TDL-C, 300ns |
| Channel estimation | RCE |

Table A-2: SLS assumptions

| Parameters | Values |  |  |
| --- | --- | --- | --- |
| Carrier frequency | 2.6 GHz |  |  |
| SCS | 30 |  |  |
| BS antenna height | 25 m |  |  |
| TRP transmit power | 41 dBm for 10 MHz bandwidth |  |  |
| UE power class | 23 dBm |  |  |
| ISD | 400 m/500m |  |  |
| TRP antenna configuration | 96Tx/Rx, (M,N,P,Mg,Ng) = (12,8,1,1,1) | 96Tx/Rx, (M,N,P,Mg,Ng) = (12,8,1,1,1) | 96Tx/Rx, (M,N,P,Mg,Ng) = (12,8,1,1,1) |
| TRP TXRU configuration | 1TXRU, (Mp,Np,P,Mg,Ng) = (1,1,1,1,1) | 1TXRU, (Mp,Np,P,Mg,Ng) = (1,1,1,1,1) | 1TXRU, (Mp,Np,P,Mg,Ng) = (1,1,1,1,1) |
| UE antenna configuration | 4Tx/Rx, (M,N,P,Mg,Ng) = (1,2,2,1,1) | 2Tx/Rx, (M,N,P,Mg,Ng) = (1,1,2,1,1) | 1Tx/Rx, (M,N,P,Mg,Ng) = (1,1,1,1,1) |
| UE TXRU configuration | 4TXRU, (Mp,Np,P,Mg,Ng) = (1,2,2,1,1) | 2TXRU, (Mp,Np,P,Mg,Ng) = (1,1,2,1,1) | 1TXRU, (Mp,Np,P,Mg,Ng) = (1,1,1,1,1) |
| Device deployment | 80% indoor, 20% outdoor (in car) |  |  |
| UE speeds of interest | Indoor users: 3km/h Outdoor users (in-car): 30 km/h |  |  |
| BS noise figure | 5 dB |  |  |
| UE noise figure | 7 dB |  |  |
| BS antenna element gain | 8 dBi |  |  |
| UE antenna element gain | 0 dBi |  |  |
| Thermal noise level | -174 dBm/Hz |  |  |
| Simulation bandwidth | 20 MHz |  |  |
| Number of UEs per cell | 20 UEs per TRxP |  |  |
| UE antenna height | Outdoor UEs: 1.5 m Indoor UTs: 3(nfl – 1) + 1.5; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) |  |  |
| Channel model variant | Channel model A |  |  |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) |  |  |
| Electronic tilt | 96° |  |  |
| Handover margin (dB) | 0 |  |  |

Figure 2: Geometry for urban scenario