3GPP TSG RAN WG1 #103-e		R1- 2007872

e-Meeting, October 26th – November 13th, 2020

Source:	CATT

Title:	 Baseline coverage performance for FR1

Agenda Item:	8.8.1.1

Document for:	Discussion and Decision

Introduction

At RAN1#102 e-meeting, the link level simulation assumptions and the link budget template were further discussed and the following agreements were achieved [1].

Agreements:

Update the agreements as follows:

- For VoIP performance evaluation based on link-level simulation for FR1

  - A packet size of 320bits with 20ms data arriving interval is adopted,

|  | Size (bits) |
| --- | --- |
| Payload | 256 |
| CRC | 16 (TBS size lower than 3824 bits) |
| MAC | 16 (with 12 bits SN size) |
| RLC | 8 (with 6 bits SN size) |
| PDCP | 16 |
| RTP/UDP/IP | 24 (w RoHC) |

  - If applicable, companies report TB size assumed in evaluation

Agreements:

- for SIP invite message

  - Payload of 1500 bytes can be a starting point.

  - The assumptions (TB size, time period etc.) are reported by companies.

  - Contributions R1-2003464 and R1-2005259 are taken into account for the evaluation.

  - In addition, 1 second time period can also be considered

Agreement:

- For the evaluation, it is assumed that Msg. 4 PDSCH payload size is 1040 bits.

Agreements:

- For receiver interference density

  - Up to each company to report for all scenarios as baseline

    - E.g. obtained by SLS, the ones for ITU self-evaluation, etc.

Agreements:

Further clarify the agreement on antenna gain and antenna gain components including antenna gain correction factors as follows:

- For both TDL option 1 (table A below) and TDL option 2 & CDL (table B below)

  - The gain of antenna gain component 1 is included in LLS results

  - The gain of antenna gain component 2 is included in link budget template

  - The gain is expressed by 10 * log 10( N/k ) - 1

  - For TDL option 2 & CDL, the gain is 0 dB

  - The gain of antenna gain component 3 is included in link budget template

  - The gain of antenna gain component 4 is included in link budget template

  - The gain of antenna gain components 3 and 4 is expressed by Antenna Element Gain + 10 * log 10( M/N ) -2

  - For Tx, One row is used represent the gain of antenna gain component 3 + 4, i.e. row No. (4)

  - For Rx, One row is used represent the gain of antenna gain component 3 + 4, i.e. row No. (11)

  - Note: more appropriate name or explanation will be added to row No.(4) and (11). Details can be discussed when the link budget template is updated.

Agreements:

- Define PSD for DL Tx power, which is depend on deployment scenario

  - For 4GHz frequency,

    - For rural with long distance scenario, PSD is 24 and 33 dBm/MHz

    - For rural scenario, PSD is 24 and 33 dBm/MHz

  - For urban scenario, PSD is 24 and 33 dBm/MHz

    - For 2.6 GHz frequency,

  - For rural with long distance scenario, PSD is 33 dBm/MHz

  - For rural scenario, PSD is 33 dBm/MHz

  - For urban scenario, PSD is 33 dBm/MHz

  - For 700MHz, 2GHz frequency

    - For rural with long distance scenario, PSD is 36 dBm/MHz

    - For rural scenario, PSD is 36 dBm/MHz

    - For urban scenario, PSD is 36 dBm/MHz

- Modify the description of row(s) of link budget template:

  - Keep the meaning of Total transmit power (row (3) ) and adding a new row (3 bis):

    - (3bis) means the transmit power for occupied channel bandwidth for control channel (17a) or data channel (17b)

- Companies are requested to set appropriate values for parameters, which is used to determine total transmit power ( row (3) and/or (3bis) ), to satisfy the PSD value above

- Note: RAN1 will further check the consistency of the definition of row(s) in link budget table when the IMT-2020 based link budget tale is updated

Agreements:

For FR1 and FR2:

- Further clarify the Definition of MCL for downlink

  - Total transmit power – Receiver sensitivity + gNB antenna gain (component 2), where

    - Total transmit power corresponds to row No.(3) + {(6) or -(7)} (for control & data channels)

    - Receiver sensitivity corresponds to row No.(22a/22b)

- Further clarify the Definition of MIL for downlink

  - Total transmit power – Receiver sensitivity + gNB antenna gain (component 2 + 3 + 4) + UE antenna gain, where

    - Total transmit power + gNB antenna gain (component 2 + 3 + 4) corresponds to row No.(9a/9b), i.e.

    - (3) + (4) + (5) + (6) – (8) for control channel

    - (3) + (4) + (5) – (7) – (8) for data channel

    - Note: the derivation of (9a/9b) will be modified depending on the discussion on antenna gain & antenna gain correction

    - Receiver sensitivity corresponds to row No.(22a/22b)

    - (Working assumption for FR2) UE antenna gain corresponds to row No.(11)+No(11bis)

  - Note: further refinement/definition of (3) and/or (22a/22b) can be discussed when link budget table is updated.

Agreements:

Definition of MPL for TDL option 1

- MPL = MIL + [(21a/b) H-ARQ gain] – [ (25a/b) Shadow fading margin – (27) Penetration margin ] + [(26) BS selection/macro-diversity gain ] + [(28) Other gains] – [(12) Cable, connector, combiner, body losses (Rx side) ]

- Note1: (8) is not necessary because it is included in the definition of MIL

Note2: (20) is not necessary because it is included in receiver sensitivity, which is used to derive MIL

In this contribution, we provide simulation results for PDCCH/PDSCH/PUCCH/PUSCH in FR1 in different scenarios.

Discussion

As discussed during in RAN1#102 e-meeting, MIL and MCL were adopted to identify the bottleneck channels for LLS based methodology. Furthermore, MPL can be used as supplemental information for coverage bottleneck(s) identification. Accordingly, we provide the baseline performance based on all these three metrics, which are illustrated as below:

- For MPL, the same definition as in IMT-2020 self-evaluation with some modifications.[2]

- For MIL, the template adopted by email discussion [102-e-Post-NR-CovEnh-02] is used to calculate MIL

- For MCL, the template adopted by email discussion [102-e-Post-NR-CovEnh-02] is used to calculate MIL

Target MPL corresponding to target ISD of 400m is applied. Outdoor-to-indoor and NLOS case with 3km/h UE speed is considered in the evaluation. Required SINR for LLS with TDL-C channel model are evaluated.

The channel would be identified as the bottleneck if it satisfies the following condition:

- For MPL, a channel is identified as the bottleneck if the baseline performance is worse than the target performance.

- For MIL, a UL channel is identified as the bottleneck if it is the worst or second worst or third worst channel. It should be noted that even the worst downlink channel has a much better coverage than the best uplink channel. Hence only uplink channels are considered.

- For MCL, a UL channel is identified as the bottleneck if it is the worst or second worst or third worst channel. It should be noted that even the worst downlink channel has a much better coverage than the best uplink channel. Hence only uplink channels are considered.

- Urban scenario

In Urban scenario, PUSCH is the bottleneck channel despite of the TDD configuration as shown in Table 1-3.

Table1: Performance evaluation on Urban-4GHz (DDDSU)

| Scenario | Bottleneck channel | MCL result (dB) | MIL result (dB) | MPL result (dB) | T-MPL ISD=400 | Required SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 4GHz DDDSU | PDSCH with eMBB | 147.79 | 156.56 | 125.83 | 117.97 | -8.8 |
|  | PDSCH with VoIP | 149.52 | 158.29 | 127.56 | 117.97 | -10.53 |
|  | SSB | 148.04 | 156.81 | 126.08 | 117.97 | -9.05 |
|  | PDCCH | 148.63 | 157.4 | 123.59 | 117.97 | -9.64 |
|  | PRACH format0 | 144.83 | 153.6 | 119.79 | 117.97 | -15.08 |
|  | PRACH formatB4 | 143.83 | 152.6 | 118.79 | 117.97 | -17.06 |
|  | PUCCH PF1 | 147.55 | 156.32 | 122.51 | 117.97 | -9.15 |
|  | PUCCH PF3/11bits/rep=1 | 145.6 | 154.37 | 120.56 | 117.97 | -7.2 |
|  | PUCCH PF3/11bits/rep=2 | 147.88 | 156.65 | 122.84 | 117.97 | -9.48 |
|  | PUCCH PF3/22bits/rep=1 | 143.95 | 152.72 | 118.91 | 117.97 | -5.55 |
|  | PUCCH PF3/22bits/rep=2 | 145.84 | 154.61 | 120.8 | 117.97 | -7.44 |
|  | Msg 3 PUSCH | 143.46 | 152.23 | 121.5 | 117.97 | -5 |
|  | eMBB PUSCH | 132.1 | 140.87 | 110.14 | 117.97 | -5.4 |
|  | VoIP PUSCH w/o repetition | 135.75 | 144.52 | 113.79 | 117.97 | -0.3 |
|  | VoIP PUSCH w/ repetition | 142.25 | 151.02 | 120.29 | 117.97 | -6.8 |

Table2: Performance evaluation on Urban-4GHz (DDDSUDDSUU)

| Scenario | Bottleneck channel | MCL result (dB) | MIL result (dB) | MPL result (dB) | T-MPL ISD=400 | Required SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 4GHz DDDSUDDSUU | PDSCH with eMBB | 146.79 | 155.56 | 124.83 | 117.97 | -7.8 |
|  | PDSCH with VoIP | 149.52 | 158.29 | 127.56 | 117.97 | -10.53 |
|  | SSB | 148.04 | 156.81 | 126.08 | 117.97 | -9.05 |
|  | PDCCH | 148.63 | 157.4 | 123.59 | 117.97 | -9.64 |
|  | PRACH format0 | 144.83 | 153.6 | 119.79 | 117.97 | -15.08 |
|  | PRACH formatB4 | 143.83 | 152.6 | 118.79 | 117.97 | -17.06 |
|  | PUCCH PF1 | 147.55 | 156.32 | 122.51 | 117.97 | -9.15 |
|  | PUCCH PF3/11bits/rep=1 | 145.6 | 154.37 | 120.56 | 117.97 | -7.2 |
|  | PUCCH PF3/11bits/rep=2 | 147.88 | 156.65 | 122.84 | 117.97 | -9.48 |
|  | PUCCH PF3/22bits/rep=1 | 143.95 | 152.72 | 118.91 | 117.97 | -5.55 |
|  | PUCCH PF3/22bits/rep=2 | 145.84 | 154.61 | 120.8 | 117.97 | -7.44 |
|  | Msg 3 PUSCH | 143.46 | 152.23 | 121.5 | 117.97 | -5 |
|  | eMBB PUSCH | 133.3 | 142.07 | 111.34 | 117.97 | -6.6 |
|  | VoIP PUSCH w/o repetition | 135.75 | 144.52 | 113.79 | 117.97 | -0.3 |
|  | VoIP PUSCH w/ repetition | 142.25 | 151.72 | 120.99 | 117.97 | -7.5 |

Table3: Performance evaluation on Urban-2.6GHz (DDDDDDDSUU)

| Scenario | Bottleneck channel | MCL result (dB) | MIL result (dB) | MPL result (dB) | T-MPL ISD=400 | Required SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 2.6GHz DDDDDDDSUU | PDSCH with eMBB | 147.79 | 156.56 | 125.83 | 114.22 | -8.8 |
|  | PDSCH with VoIP | 149.57 | 158.34 | 127.61 | 114.22 | -10.58 |
|  | SSB | 147.94 | 156.71 | 125.98 | 114.22 | -8.95 |
|  | PDCCH | 148.88 | 157.65 | 123.84 | 114.22 | -9.89 |
|  | PRACH format0 | 144.81 | 153.58 | 119.77 | 114.22 | -15.06 |
|  | PRACH formatB4 | 143.35 | 152.12 | 118.31 | 114.22 | -16.58 |
|  | PUCCH PF1 | 147.5 | 156.27 | 122.46 | 114.22 | -9.1 |
|  | PUCCH PF3/11bits/rep=1 | 146.07 | 154.84 | 121.03 | 114.22 | -7.67 |
|  | PUCCH PF3/11bits/rep=2 | 148.2 | 156.97 | 123.16 | 114.22 | -9.8 |
|  | PUCCH PF3/22bits/rep=1 | 144.07 | 152.84 | 119.03 | 114.22 | -5.67 |
|  | PUCCH PF3/22bits/rep=2 | 145.9 | 154.67 | 120.86 | 114.22 | -7.5 |
|  | Msg 3 PUSCH | 143.76 | 152.53 | 121.8 | 114.22 | -5.3 |
|  | eMBB PUSCH | 132.2 | 140.97 | 110.24 | 114.22 | -5.5 |
|  | VoIP PUSCH w/o repetition | 135.75 | 144.52 | 113.79 | 114.22 | -0.3 |
|  | VoIP PUSCH w/ repetition | 141.95 | 150.72 | 119.99 | 114.22 | -6.5 |

Observation 1: In Urban scenario, the coverage of PUSCH is the bottleneck.

- Based on MPL evaluation results, eMBB PUSCH is the bottleneck channel in urban scenario for both 2.6 GHz and 4 GHz

  - Up to 7.83 dB performance gap needs to be compensated in terms of MPL

- Based on MIL/MCL, evaluation result, eMBB PUSCH，VOIP PUSCH and MSG3 PUSCH are the bottleneck channel

- Rural scenario

1732m ISD is assumed to calculate the target MPL 3km/h UE speed and 120km/h are both considered in the evaluation.

Table4: Performance evaluation on 4GHz (DDDSU) for Rural-TDLC-3kmph

| Scenario | Bottleneck channel | MCL result(dB) | MIL result(dB) | MPL result(dB) | T-MPL ISD=1732 | Required SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 4GHz DDDSU | PDSCH with eMBB | 151.55 | 155.55 | 138.57 | 131.57 | -12.56 |
|  | PDSCH with VoIP | 149.53 | 153.53 | 136.55 | 131.57 | -10.54 |
|  | SSB | 148.04 | 152.04 | 135.06 | 131.57 | -9.05 |
|  | PDCCH | 148.63 | 152.63 | 132.57 | 131.57 | -9.64 |
|  | PRACH format0 | 148.83 | 152.83 | 132.77 | 131.57 | -15.08 |
|  | PRACH formatB4 | 147.83 | 151.83 | 131.77 | 131.57 | -17.06 |
|  | PUCCH PF1 | 147.55 | 151.55 | 131.49 | 131.57 | -9.15 |
|  | PUCCH PF3/11bits/rep=1 | 145.6 | 149.6 | 129.54 | 131.57 | -7.2 |
|  | PUCCH PF3/11bits/rep=2 | 147.88 | 151.88 | 131.82 | 131.57 | -9.48 |
|  | PUCCH PF3/22bits/rep=1 | 143.95 | 147.95 | 127.89 | 131.57 | -5.55 |
|  | PUCCH PF3/22bits/rep=2 | 145.84 | 149.84 | 129.78 | 131.57 | -7.44 |
|  | Msg 3 PUSCH | 143.46 | 147.46 | 130.48 | 131.57 | -5 |
|  | eMBB PUSCH | 130.40 | 134.4 | 117.42 | 131.57 | -3.7 |
|  | VoIP PUSCH w/o repetition | 135.75 | 139.75 | 122.77 | 131.57 | -0.3 |
|  | VoIP PUSCH w/ repetition | 142.05 | 146.05 | 129.07 | 131.57 | -6.6 |

Table5: Performance evaluation on 2.6GHz (DDDDDDDSUU) for Rural-TDLC-3kmph

| Scenario | Bottleneck channel | MCL result(dB) | MIL result(dB) | MPL result(dB) | T-MPL ISD=1732 | Required SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 2.6GHz DDDDDDDSUU | PDSCH with eMBB | 151.59 | 155.59 | 138.61 | 131.57 | -12.6 |
|  | PDSCH with VoIP | 149.29 | 153.29 | 136.31 | 131.57 | -10.3 |
|  | SSB | 147.94 | 151.94 | 134.96 | 131.57 | -8.95 |
|  | PDCCH | 148.88 | 152.88 | 132.82 | 131.57 | -9.89 |
|  | PRACH format0 | 148.81 | 152.81 | 132.75 | 131.57 | -15.06 |
|  | PRACH formatB4 | 147.35 | 151.35 | 131.29 | 131.57 | -16.58 |
|  | PUCCH PF1 | 147.50 | 151.50 | 131.44 | 131.57 | -9.1 |
|  | PUCCH PF3/11bits/rep=1 | 146.07 | 150.07 | 130.01 | 131.57 | -7.67 |
|  | PUCCH PF3/11bits/rep=2 | 148.20 | 152.2 | 132.14 | 131.57 | -9.8 |
|  | PUCCH PF3/22bits/rep=1 | 144.07 | 148.07 | 128.01 | 131.57 | -5.67 |
|  | PUCCH PF3/22bits/rep=2 | 145.90 | 149.9 | 129.84 | 131.57 | -7.5 |
|  | Msg 3 PUSCH | 143.86 | 147.86 | 130.88 | 131.57 | -5.4 |
|  | eMBB PUSCH | 130.50 | 134.5 | 117.52 | 131.57 | -3.8 |
|  | VoIP PUSCH w/o repetition | 135.55 | 139.55 | 122.57 | 131.57 | -0.1 |
|  | VoIP PUSCH w/ repetition | 141.85 | 145.85 | 128.87 | 131.57 | -6.4 |

Table6: Performance evaluation on 2GHz (FDD) for Rural-TDLC-3kmph

| Scenario | Bottleneck channel | MCL result(dB) | MIL result(dB) | MPL result(dB) | T-MPL ISD=1732 | Required SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 2GHz FDD | PDSCH with eMBB | 150.85 | 163.88 | 146.25 | 125.55 | -11.86 |
|  | PDSCH with VoIP | 150.63 | 163.66 | 146.03 | 125.55 | -11.64 |
|  | SSB | 149.45 | 162.48 | 144.85 | 125.55 | -10.46 |
|  | PDCCH | 150.9 | 161.92 | 140.97 | 125.55 | -9.9 |
|  | PRACH format0 | 136.51 | 152.55 | 131.6 | 125.55 | -14.76 |
|  | PRACH formatB4 | 135.69 | 151.73 | 130.78 | 125.55 | -16.92 |
|  | PUCCH PF1 | 135.51 | 151.55 | 130.6 | 125.55 | -6.1 |
|  | PUCCH PF3/11bits/rep=1 | 137.17 | 153.21 | 132.26 | 125.55 | -7.76 |
|  | PUCCH PF3/11bits/rep=2 | 139.21 | 155.25 | 134.3 | 125.55 | -9.8 |
|  | PUCCH PF3/22bits/rep=1 | 135.21 | 151.25 | 130.3 | 125.55 | -5.8 |
|  | PUCCH PF3/22bits/rep=2 | 137.21 | 153.25 | 132.3 | 125.55 | -7.8 |
|  | Msg 3 PUSCH | 135.27 | 151.31 | 133.68 | 125.55 | -5.8 |
|  | eMBB PUSCH | 132.66 | 148.7 | 131.07 | 125.55 | -6.2 |
|  | VoIP PUSCH w/o repetition | 127.06 | 143.1 | 125.47 | 125.55 | -0.6 |
|  | VoIP PUSCH w/ repetition | 134.26 | 150.3 | 132.67 | 125.55 | -7.8 |

Table7: Performance evaluation on 700MHz (FDD) for Rural-TDLC-3kmph

| Scenario | Bottleneck channel | MCL result(dB) | MIL result(dB) | MPL result(dB) | T-MPL ISD=1732 | Required SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 700MHz FDD | PDSCH with eMBB | 148.29 | 158.31 | 140.68 | 116.43 | -9.3 |
|  | PDSCH with VoIP | 150.59 | 160.61 | 142.98 | 116.43 | -11.6 |
|  | SSB | 149.02 | 159.04 | 141.41 | 116.43 | -10.03 |
|  | PDCCH | 146.64 | 156.66 | 135.71 | 116.43 | -7.65 |
|  | PRACH format0 | 136.3 | 149.33 | 128.38 | 116.43 | -14.55 |
|  | PRACH formatB4 | 135.72 | 148.7 | 127.8 | 116.43 | -16.95 |
|  | PUCCH PF1 | 140.91 | 153.94 | 132.99 | 116.43 | -11.5 |
|  | PUCCH PF3/11bits/rep=1 | 137.33 | 150.36 | 129.41 | 116.43 | -7.92 |
|  | PUCCH PF3/11bits/rep=2 | 139.19 | 152.22 | 131.27 | 116.43 | -9.78 |
|  | PUCCH PF3/22bits/rep=1 | 135.24 | 148.27 | 127.32 | 116.43 | -5.83 |
|  | PUCCH PF3/22bits/rep=2 | 137.33 | 150.36 | 129.41 | 116.43 | -7.92 |
|  | Msg 3 PUSCH | 135.67 | 148.7 | 131.07 | 116.43 | -6.2 |
|  | eMBB PUSCH | 132.96 | 145.99 | 128.36 | 116.43 | -6.5 |
|  | VoIP PUSCH w/o repetition | 127.96 | 140.99 | 123.36 | 116.43 | -1.5 |
|  | VoIP PUSCH w/ repetition | 133.96 | 146.99 | 129.36 | 116.43 | -7.5 |

Table8: Performance evaluation on 4GHz (DDDSU) for Rural-TDLC-120kmph

| Scenario | Bottleneck channel | MCL result(dB) | MIL result(dB) | MPL result(dB) | T-MPL ISD=1732 | Required SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 4GHz DDDSU | PDCCH | 149.36 | 153.36 | 136.29 | 131.57 | -10.37 |
|  | PRACH format0 | 149.27 | 153.27 | 136.20 | 131.57 | -15.52 |
|  | PRACH formatB4 | 148.47 | 152.47 | 135.40 | 131.57 | -17.7 |
|  | PUCCH PF1 | 147.75 | 151.75 | 134.68 | 131.57 | -9.35 |
|  | PUCCH PF3/11bits/rep=1 | 145.80 | 149.8 | 132.73 | 131.57 | -7.4 |
|  | PUCCH PF3/11bits/rep=2 | 149.60 | 153.6 | 136.53 | 131.57 | -11.2 |
|  | PUCCH PF3/22bits/rep=1 | 143.90 | 147.9 | 130.83 | 131.57 | -5.5 |
|  | PUCCH PF3/22bits/rep=2 | 147.80 | 151.8 | 134.73 | 131.57 | -9.4 |
|  | Msg 3 PUSCH | 143.46 | 147.46 | 133.98 | 131.57 | -5 |
|  | eMBB PUSCH | 131.00 | 135 | 121.52 | 131.57 | -4.3 |
|  | VoIP PUSCH w/o repetition | 127.60 | 140.35 | 118.12 | 131.57 | -0.9 |
|  | VoIP PUSCH w/ repetition | 146.35 | 150.35 | 136.87 | 131.57 | -10.9 |

Table9: Performance evaluation on 2.6GHz (DDDDDDDSUU) for Rural-TDLC-120kmph

| Scenario | Bottleneck channel | MCL result(dB) | MIL result(dB) | MPL result(dB) | T-MPL ISD=1732 | Required SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 2.6GHz DDDDDDDSUU | PDCCH | 149.31 | 153.31 | 136.24 | 127.83 | -10.32 |
|  | PRACH format0 | 148.71 | 152.71 | 135.64 | 127.83 | -14.96 |
|  | PRACH formatB4 | 147.99 | 151.99 | 134.92 | 127.83 | -17.22 |
|  | PUCCH PF1 | 147.55 | 151.55 | 134.48 | 127.83 | -9.15 |
|  | PUCCH PF3/11bits/rep=1 | 145.79 | 149.79 | 132.72 | 127.83 | -7.39 |
|  | PUCCH PF3/11bits/rep=2 | 149.16 | 153.16 | 136.09 | 127.83 | -10.76 |
|  | PUCCH PF3/22bits/rep=1 | 143.99 | 147.99 | 130.92 | 127.83 | -5.59 |
|  | PUCCH PF3/22bits/rep=2 | 147.33 | 151.33 | 134.26 | 127.83 | -8.93 |
|  | Msg 3 PUSCH | 143.46 | 147.46 | 133.98 | 127.83 | -5 |
|  | eMBB PUSCH | 131.00 | 135 | 121.52 | 127.83 | -4.3 |
|  | VoIP PUSCH w/o repetition | 136.25 | 140.25 | 126.77 | 127.83 | -0.8 |
|  | VoIP PUSCH w/ repetition | 145.95 | 149.95 | 136.47 | 127.83 | -10.5 |

Table10: Performance evaluation on 2GHz (FDD) for Rural-TDLC-120kmph

| Scenario | Bottleneck channel | MCL result(dB) | MIL result(dB) | MPL result(dB) | T-MPL ISD=1732 | Required SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 2GHz FDD | PDCCH | 149.44 | 162.47 | 143.02 | 125.55 | -10.45 |
|  | PRACH format0 | 137.22 | 153.26 | 133.81 | 125.55 | -15.47 |
|  | PRACH formatB4 | 136.33 | 152.37 | 132.92 | 125.55 | -17.56 |
|  | PUCCH PF1 | 135.51 | 151.55 | 132.1 | 125.55 | -6.1 |
|  | PUCCH PF3/11bits/rep=1 | 137.05 | 153.09 | 133.64 | 125.55 | -7.64 |
|  | PUCCH PF3/11bits/rep=2 | 140.21 | 156.25 | 136.8 | 125.55 | -10.8 |
|  | PUCCH PF3/22bits/rep=1 | 134.99 | 151.03 | 131.58 | 125.55 | -5.58 |
|  | PUCCH PF3/22bits/rep=2 | 138.31 | 154.35 | 134.9 | 125.55 | -8.9 |
|  | Msg 3 PUSCH | 134.47 | 150.51 | 134.9 | 125.55 | -5 |
|  | eMBB PUSCH | 132.96 | 149 | 133.39 | 125.55 | -6.5 |
|  | VoIP PUSCH w/o repetition | 127.26 | 143.3 | 127.69 | 125.55 | -0.8 |
|  | VoIP PUSCH w/ repetition | 136.91 | 152.94 | 137.34 | 125.55 | -10.45 |

Table11: Performance evaluation on 700MHz (FDD) for Rural-TDLC-120kmph

| Scenario | Bottleneck channel | MCL result(dB) | MIL result(dB) | MPL result(dB) | T-MPL ISD=1732 | Required SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 700MHz FDD | PDCCH | 148.15 | 156.16 | 136.71 | 116.43 | -7.15 |
|  | PRACH format0 | 136.55 | 149.58 | 130.13 | 116.43 | -14.8 |
|  | PRACH formatB4 | 135.87 | 148.9 | 129.45 | 116.43 | -17.1 |
|  | PUCCH PF1 | 138.61 | 151.64 | 132.19 | 116.43 | -9.2 |
|  | PUCCH PF3/11bits/rep=1 | 137.14 | 150.17 | 130.72 | 116.43 | -7.73 |
|  | PUCCH PF3/11bits/rep=2 | 140.06 | 153.09 | 133.64 | 116.43 | -10.65 |
|  | PUCCH PF3/22bits/rep=1 | 135.09 | 148.12 | 128.67 | 116.43 | -5.68 |
|  | PUCCH PF3/22bits/rep=2 | 138.51 | 151.54 | 132.09 | 116.43 | -9.1 |
|  | Msg 3 PUSCH | 134.47 | 147.5 | 131.89 | 116.43 | -5 |
|  | eMBB PUSCH | 132.86 | 145.89 | 130.28 | 116.43 | -6.4 |
|  | VoIP PUSCH w/o repetition | 127.06 | 140.09 | 124.48 | 116.43 | -0.6 |
|  | VoIP PUSCH w/ repetition | 138.36 | 151.39 | 135.78 | 116.43 | -11.9 |

Table12: Performance evaluation on 4GHz (DDDSU) for Rural-TDLD-3kmph

| Scenario | Bottleneck channel | MCL result(dB) | MIL result(dB) | MPL result(dB) | T-MPL ISD=1732 | Required SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 4GHz DDDSU | PDSCH with eMBB | 152.79 | 156.79 | 138.65 | 131.57 | -13.8 |
|  | PDSCH with VoIP | 151.99 | 155.99 | 137.85 | 131.57 | -13 |
|  | PDCCH | 150.14 | 154.14 | 133.24 | 131.57 | -11.15 |
|  | PRACH format0 | 152.13 | 156.13 | 135.23 | 131.57 | -18.38 |
|  | PRACH formatB4 | 150.79 | 154.79 | 133.89 | 131.57 | -20.02 |
|  | PUCCH PF1 | 149.87 | 153.87 | 132.97 | 131.57 | -11.47 |
|  | PUCCH PF3/11bits/rep=1 | 147.78 | 151.78 | 130.88 | 131.57 | -9.38 |
|  | PUCCH PF3/11bits/rep=2 | 149.80 | 153.8 | 132.9 | 131.57 | -11.4 |
|  | PUCCH PF3/22bits/rep=1 | 146.14 | 150.14 | 129.24 | 131.57 | -7.74 |
|  | PUCCH PF3/22bits/rep=2 | 148.08 | 152.08 | 131.18 | 131.57 | -9.68 |
|  | Msg 3 PUSCH | 145.36 | 149.36 | 131.22 | 131.57 | -6.9 |
|  | eMBB PUSCH | 133.10 | 137.1 | 118.96 | 131.57 | -6.4 |
|  | VoIP PUSCH w/o repetition | 139.25 | 143.25 | 125.11 | 131.57 | -3.8 |
|  | VoIP PUSCH w/ repetition | 143.85 | 147.85 | 129.71 | 131.57 | -8.4 |

Table13: Performance evaluation on 2.6GHz (DDDDDDDSUU) for Rural-TDLD-3kmph

| Scenario | Bottleneck channel | MCL result(dB) | MIL result(dB) | MPL result(dB) | T-MPL ISD=1732 | Required SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 2.6GHz DDDDDDDSUU | PDSCH with eMBB | 152.71 | 156.71 | 138.57 | 127.83 | -13.72 |
|  | PDSCH with VoIP | 151.79 | 155.79 | 137.65 | 127.83 | -12.8 |
|  | PDCCH | 150.61 | 154.61 | 133.71 | 127.83 | -11.62 |
|  | PRACH format0 | 152.15 | 149.17 | 135.25 | 127.83 | -18.4 |
|  | PRACH formatB4 | 151.01 | 151.01 | 134.11 | 127.83 | -20.24 |
|  | PUCCH PF1 | 150.56 | 154.56 | 133.66 | 127.83 | -12.16 |
|  | PUCCH PF3/11bits/rep=1 | 147.70 | 151.7 | 130.8 | 127.83 | -9.3 |
|  | PUCCH PF3/11bits/rep=2 | 149.70 | 153.7 | 132.8 | 127.83 | -11.3 |
|  | PUCCH PF3/22bits/rep=1 | 146.10 | 150.1 | 129.2 | 127.83 | -7.7 |
|  | PUCCH PF3/22bits/rep=2 | 147.90 | 151.9 | 131 | 127.83 | -9.5 |
|  | Msg 3 PUSCH | 145.36 | 149.36 | 131.22 | 127.83 | -6.9 |
|  | eMBB PUSCH | 132.90 | 136.90 | 118.76 | 127.83 | -6.2 |
|  | VoIP PUSCH w/o repetition | 139.05 | 143.05 | 124.91 | 127.83 | -3.6 |
|  | VoIP PUSCH w/ repetition | 143.85 | 147.85 | 129.71 | 127.83 | -8.4 |

Table14: Performance evaluation on 2GHz (FDD) for Rural-TDLD-3kmph

| Scenario | Bottleneck channel | MCL result(dB) | MIL result(dB) | MPL result(dB) | T-MPL ISD=1732 | Required SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 2GHz FDD | PDSCH with eMBB | 151.55 | 164.58 | 147.06 | 125.55 | -12.56 |
|  | PDSCH with VoIP | 151.74 | 164.77 | 147.25 | 125.55 | -12.75 |
|  | PDCCH | 152.83 | 163.85 | 143.06 | 125.55 | -11.83 |
|  | PRACH format0 | 139.98 | 156.02 | 135.23 | 125.55 | -18.23 |
|  | PRACH formatB4 | 138.74 | 154.78 | 133.99 | 125.55 | -19.97 |
|  | PUCCH PF1 | 140.95 | 156.99 | 136.2 | 125.55 | -11.54 |
|  | PUCCH PF3/11bits/rep=1 | 139.12 | 155.16 | 134.37 | 125.55 | -9.71 |
|  | PUCCH PF3/11bits/rep=2 | 141.03 | 157.07 | 136.28 | 125.55 | -11.62 |
|  | PUCCH PF3/22bits/rep=1 | 137.43 | 153.47 | 132.68 | 125.55 | -8.02 |
|  | PUCCH PF3/22bits/rep=2 | 139.31 | 155.35 | 134.56 | 125.55 | -9.9 |
|  | Msg 3 PUSCH | 134.88 | 152.51 | 134.99 | 125.55 | -7 |
|  | eMBB PUSCH | 134.56 | 150.6 | 133.08 | 125.55 | -8.1 |
|  | VoIP PUSCH w/o repetition | 129.96 | 146 | 128.48 | 125.55 | -3.5 |
|  | VoIP PUSCH w/ repetition | 136.06 | 152.1 | 134.58 | 125.55 | -9.6 |

Table15: Performance evaluation on 700MHz (FDD) for Rural-TDLD-3kmph

| Scenario | Bottleneck channel | MCL result(dB) | MIL result(dB) | MPL result(dB) | T-MPL ISD=1732 | Required SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 700MHz FDD | PDSCH with eMBB | 148.89 | 158.91 | 141.39 | 116.43 | -9.9 |
|  | PDSCH with VoIP | 151.59 | 161.61 | 144.09 | 116.43 | -12.6 |
|  | PDCCH | 148.6 | 158.62 | 137.83 | 116.43 | -9.61 |
|  | PRACH format0 | 140.22 | 153.25 | 132.46 | 116.43 | -18.47 |
|  | PRACH formatB4 | 138.16 | 151.19 | 130.4 | 116.43 | -19.39 |
|  | PUCCH PF1 | 140.91 | 153.94 | 133.15 | 116.43 | -11.5 |
|  | PUCCH PF3/11bits/rep=1 | 139.14 | 152.17 | 131.38 | 116.43 | -9.73 |
|  | PUCCH PF3/11bits/rep=2 | 141.11 | 154.14 | 133.35 | 116.43 | -11.7 |
|  | PUCCH PF3/22bits/rep=1 | 137.47 | 150.5 | 129.71 | 116.43 | -8.06 |
|  | PUCCH PF3/22bits/rep=2 | 139.41 | 152.44 | 131.65 | 116.43 | -10 |
|  | Msg 3 PUSCH | 136.87 | 149.9 | 132.38 | 116.43 | -7.4 |
|  | eMBB PUSCH | 134.36 | 147.39 | 129.87 | 116.43 | -7.9 |
|  | VoIP PUSCH w/o repetition | 129.76 | 142.79 | 125.27 | 116.43 | -3.3 |
|  | VoIP PUSCH w/ repetition | 135.26 | 148.29 | 130.77 | 116.43 | -8.8 |

Table16: Performance evaluation on 4GHz (DDDSU) for Rural-TDLD-120kmph

| Scenario | Bottleneck channel | MCL result(dB) | MIL result(dB) | MPL result(dB) | T-MPL ISD=1732 | Required SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 4GHz DDDSU | PDCCH | 150.70 | 154.7 | 139.59 | 131.57 | -11.71 |
|  | PRACH format0 | 151.10 | 155.10 | 139.99 | 131.57 | -17.35 |
|  | PRACH formatB4 | 150.59 | 154.59 | 139.48 | 131.57 | -19.82 |
|  | PUCCH PF1 | 149.93 | 153.93 | 138.82 | 131.57 | -11.53 |
|  | PUCCH PF3/11bits/rep=1 | 147.80 | 151.8 | 136.69 | 131.57 | -9.4 |
|  | PUCCH PF3/11bits/rep=2 | 149.95 | 153.95 | 138.84 | 131.57 | -11.55 |
|  | PUCCH PF3/22bits/rep=1 | 146.14 | 150.14 | 135.03 | 131.57 | -7.74 |
|  | PUCCH PF3/22bits/rep=2 | 148.34 | 152.34 | 137.23 | 131.57 | -9.94 |
|  | Msg 3 PUSCH | 145.76 | 149.76 | 137.40 | 131.57 | -7.3 |
|  | eMBB PUSCH | 133.30 | 137.3 | 124.94 | 131.57 | -6.6 |
|  | VoIP PUSCH w/o repetition | 139.45 | 143.45 | 131.09 | 131.57 | -4 |
|  | VoIP PUSCH w/ repetition | 147.65 | 151.65 | 139.29 | 131.57 | -12.2 |

Table17: Performance evaluation on 2.6GHz (DDDDDDDSUU) for Rural-TDLD-120kmph

| Scenario | Bottleneck channel | MCL result(dB) | MIL result(dB) | MPL result(dB) | T-MPL ISD=1732 | Required SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 2.6GHz DDDDDDDSUU | PDCCH | 150.47 | 154.47 | 139.36 | 127.83 | -11.48 |
|  | PRACH format0 | 151.32 | 155.32 | 140.21 | 127.83 | -17.57 |
|  | PRACH formatB4 | 148.76 | 152.76 | 137.65 | 127.83 | -17.99 |
|  | PUCCH PF1 | 150.60 | 154.6 | 139.49 | 127.83 | -12.2 |
|  | PUCCH PF3/11bits/rep=1 | 147.80 | 151.8 | 136.69 | 127.83 | -9.4 |
|  | PUCCH PF3/11bits/rep=2 | 150.05 | 154.05 | 138.94 | 127.83 | -11.65 |
|  | PUCCH PF3/22bits/rep=1 | 146.16 | 150.16 | 135.05 | 127.83 | -7.76 |
|  | PUCCH PF3/22bits/rep=2 | 148.27 | 152.27 | 137.16 | 127.83 | -9.87 |
|  | Msg 3 PUSCH | 145.76 | 149.76 | 137.40 | 127.83 | -7.3 |
|  | eMBB PUSCH | 133.30 | 137.3 | 124.94 | 127.83 | -6.6 |
|  | VoIP PUSCH w/o repetition | 139.45 | 143.45 | 131.09 | 127.83 | -4 |
|  | VoIP PUSCH w/ repetition | 148.25 | 152.25 | 139.89 | 127.83 | -12.8 |

Table18: Performance evaluation on 2GHz (FDD) for Rural-TDLD-120kmph

| Scenario | Bottleneck channel | MCL result(dB) | MIL result(dB) | MPL result(dB) | T-MPL ISD=1732 | Required SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 2GHz FDD | PDCCH | 150.8 | 163.83 | 146.77 | 125.55 | -11.81 |
|  | PRACH format0 | 139.12 | 155.16 | 138.1 | 125.55 | -17.37 |
|  | PRACH formatB4 | 138.63 | 154.67 | 137.61 | 125.55 | -19.86 |
|  | PUCCH PF1 | 140.96 | 157 | 139.94 | 125.55 | -11.55 |
|  | PUCCH PF3/11bits/rep=1 | 138.98 | 155.02 | 137.96 | 125.55 | -9.57 |
|  | PUCCH PF3/11bits/rep=2 | 140.97 | 157.01 | 139.95 | 125.55 | -11.56 |
|  | PUCCH PF3/22bits/rep=1 | 137.34 | 153.38 | 136.32 | 125.55 | -7.93 |
|  | PUCCH PF3/22bits/rep=2 | 139.3 | 155.34 | 138.28 | 125.55 | -9.89 |
|  | Msg 3 PUSCH | 136.77 | 152.81 | 139.02 | 125.55 | -7.3 |
|  | eMBB PUSCH | 135.26 | 151.3 | 137.51 | 125.55 | -8.8 |
|  | VoIP PUSCH w/o repetition | 130.46 | 146.5 | 132.71 | 125.55 | -4 |
|  | VoIP PUSCH w/ repetition | 139.96 | 156 | 142.21 | 125.55 | -13.5 |

Table19: Performance evaluation on 700MHz (FDD) for Rural-TDLD-120kmph

| Scenario | Bottleneck channel | MCL result(dB) | MIL result(dB) | MPL result(dB) | T-MPL ISD=1732 | Required SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 700MHz FDD | PDCCH | 158.98 | 158.18 | 141.12 | 116.43 | -11.55 |
|  | PRACH format0 | 139.73 | 152.76 | 135.7 | 116.43 | -17.98 |
|  | PRACH formatB4 | 138.73 | 151.76 | 134.7 | 116.43 | -19.96 |
|  | PUCCH PF1 | 139.03 | 153.99 | 136.93 | 116.43 | -9.62 |
|  | PUCCH PF3/11bits/rep=1 | 141.12 | 152.06 | 135 | 116.43 | -11.71 |
|  | PUCCH PF3/11bits/rep=2 | 137.46 | 154.15 | 137.09 | 116.43 | -8.05 |
|  | PUCCH PF3/22bits/rep=1 | 139.39 | 150.49 | 133.43 | 116.43 | -9.98 |
|  | PUCCH PF3/22bits/rep=2 | 136.91 | 152.42 | 135.36 | 116.43 | -7.5 |
|  | Msg 3 PUSCH | 138.27 | 150 | 136.21 | 116.43 | -8.8 |
|  | eMBB PUSCH | 130.46 | 148.29 | 134.5 | 116.43 | -4 |
|  | VoIP PUSCH w/o repetition | 128.76 | 143.49 | 129.7 | 116.43 | -2.3 |
|  | VoIP PUSCH w/ repetition | 138.36 | 153.09 | 139.3 | 116.43 | -11.9 |

In rural scenario, the coverage of PDCCH, SSB, PDSCH and PRACH can meet the requirement.

For NLOS scenario, PUCCH is the bottleneck channel if TDD is assumed. For O2O scenario, PUCCH is the bottleneck channel when UCI payload = 22 bits and 4 GHz frequency carrier is configured. For LOS scenario, PUCCH is the bottleneck channel when 4 GHz carrier frequency and O2I is assumed.

For PUSCH, the coverage cannot meet the requirement in some limited case as shown in Table 4-Table 19.

Observation 2: In Rural scenario, the PDCCH, SSB, PDSCH, PRACH are not bottleneck. The coverage of PUCCH and PUSCH are bottleneck.

- Based on MPL evaluation results, PUCCH format 3, eMBB PUSCH, VoIP PUSCH, Msg3 PUSCH are bottleneck channels

- Based on MIL/MCL evaluation results, eMBB PUSCH，PUCCH format 3，MSB3 PUCCH and VOIP PUSCH are the bottleneck channels.

- Rural with long distance scenario

Target MPL corresponding to target ISD of 12Km and 30Km are compared. LOS with 120km/h UE speed is considered in the evaluation.

Table20: Performance evaluation on 4GHz (DDDSU) for Rural with long distance

| Scenario | Bottleneck channel | MCL result(dB) | MIL result(dB) | MPL result(dB) | T-MPL ISD=12km | Required SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 4GHz DDDSU | PDSCH with eMBB | 151.79 | 155.79 | 143.43 | 132.43 | -12.8 |
|  | PDSCH with VoIP | 150.52 | 154.52 | 142.16 | 132.43 | -11.53 |
|  | PDCCH | 150.39 | 154.39 | 139.28 | 132.43 | -11.4 |
|  | SSB | 153.19 | 157.19 | 144.83 | 132.43 | -14.2 |
|  | PRACH format0 | 151.13 | 155.13 | 140.02 | 132.43 | -17.38 |
|  | PRACH formatB4 | 150.62 | 154.62 | 139.51 | 132.43 | -19.85 |
|  | PUCCH PF1 | 149.9 | 153.9 | 138.79 | 132.43 | -11.5 |
|  | PUCCH PF3/11bits/rep=1 | 147.93 | 151.93 | 136.82 | 132.43 | -9.53 |
|  | PUCCH PF3/11bits/rep=2 | 150 | 154 | 138.89 | 132.43 | -11.6 |
|  | PUCCH PF3/22bits/rep=1 | 146.28 | 150.28 | 135.17 | 132.43 | -7.88 |
|  | PUCCH PF3/22bits/rep=2 | 148.32 | 152.32 | 137.21 | 132.43 | -9.92 |
|  | Msg 3 PUSCH | 145.86 | 149.86 | 137.5 | 132.43 | -7.4 |
|  | eMBB PUSCH | 133.2 | 137.2 | 124.84 | 132.43 | -6.5 |
|  | VoIP PUSCH w/o repetition | 139.45 | 143.45 | 131.09 | 132.43 | -4 |
|  | VoIP PUSCH w/ repetition | 147.65 | 151.65 | 139.29 | 132.43 | -12.2 |

Table21: Performance evaluation on 4GHz (DDDSUDDSUU) for Rural with long distance

| Scenario | Bottleneck channel | MCL result(dB) | MIL result(dB) | MPL result(dB) | T-MPL ISD=12km | Required SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 4GHz DDDSUDDSUU | PDSCH with eMBB | 151.79 | 155.79 | 143.43 | 132.43 | -12.8 |
|  | PDSCH with VoIP | 150.52 | 154.52 | 142.16 | 132.43 | -11.53 |
|  | PDCCH | 150.39 | 154.39 | 139.28 | 132.43 | -11.4 |
|  | SSB | 153.19 | 157.19 | 144.83 | 132.43 | -14.2 |
|  | PRACH format0 | 151.13 | 155.13 | 140.02 | 132.43 | -17.38 |
|  | PRACH formatB4 | 150.62 | 154.62 | 139.51 | 132.43 | -19.85 |
|  | PUCCH PF1 | 149.9 | 153.9 | 138.79 | 132.43 | -11.5 |
|  | PUCCH PF3/11bits/rep=1 | 147.93 | 151.93 | 136.82 | 132.43 | -9.53 |
|  | PUCCH PF3/11bits/rep=2 | 150 | 154 | 138.89 | 132.43 | -11.6 |
|  | PUCCH PF3/22bits/rep=1 | 146.28 | 150.28 | 135.17 | 132.43 | -7.88 |
|  | PUCCH PF3/22bits/rep=2 | 148.32 | 152.32 | 137.21 | 132.43 | -9.92 |
|  | Msg 3 PUSCH | 145.86 | 149.86 | 137.5 | 132.43 | -7.4 |
|  | eMBB PUSCH | 134.1 | 138.1 | 125.74 | 132.43 | -7.4 |
|  | VoIP PUSCH w/o repetition | 139.45 | 143.45 | 131.09 | 132.43 | -4 |
|  | VoIP PUSCH w/ repetition | 148.55 | 152.55 | 140.19 | 132.43 | -13.1 |

Table22: Performance evaluation on 700MHz (FDD) for Rural with long distance

| Scenario | Bottleneck channel | MCL result(dB) | MIL result(dB) | MPL result(dB) | T-MPL ISD=12km | Required SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 700MHz FDD | PDSCH with eMBB | 151.79 | 161.81 | 148.02 | 127 | -12.8 |
|  | PDSCH with VoIP | 152.61 | 162.63 | 148.82 | 127 | -13.62 |
|  | PDCCH | 147.79 | 157.81 | 140.75 | 127 | -8.8 |
|  | SSB | 150.61 | 160.63 | 146.84 | 127 | -11.62 |
|  | PRACH format0 | 139.57 | 152.6 | 135.54 | 127 | -17.82 |
|  | PRACH formatB4 | 138.49 | 151.52 | 134.46 | 127 | -19.72 |
|  | PUCCH PF1 | 140.85 | 153.88 | 136.82 | 127 | -11.44 |
|  | PUCCH PF3/11bits/rep=1 | 138.89 | 151.92 | 134.86 | 127 | -9.48 |
|  | PUCCH PF3/11bits/rep=2 | 141.01 | 154.04 | 136.98 | 127 | -11.6 |
|  | PUCCH PF3/22bits/rep=1 | 137.27 | 150.3 | 133.24 | 127 | -7.86 |
|  | PUCCH PF3/22bits/rep=2 | 138.39 | 152.33 | 135.27 | 127 | -9.89 |
|  | Msg 3 PUSCH | 136.97 | 150 | 136.21 | 127 | -7.5 |
|  | eMBB PUSCH | 135.26 | 148.29 | 134.5 | 127 | -8.8 |
|  | VoIP PUSCH w/o repetition | 130.46 | 143.49 | 129.7 | 127 | -4 |
|  | VoIP PUSCH w/ repetition | 140.06 | 153.09 | 139.3 | 127 | -13.6 |

It can be observed the coverage of DL channels can meet the requirements even in rural with long distance scenario.

VoIP PUSCH and eMBB PUSCH are the bottleneck channels if the ISD is 12 km in terms of MPL.

Observation 3: The coverage of most of channels can meet the requirements with target ISD 12km except the VOIP PUSCH without repetition, Msg3 PUSCH, PUCCH format3 and eMBB PUSCH.

- Based on MPL evaluation results, eMBB PUSCH and VOIP PUSCH cannot meet the requirements at 4GHz, up to7.59 dB performance gap needs to be compensated.

- Based on  MIL/MCL evaluation results, Msg3 PUSCH, PUCCH format3 and eMBB PUSCH are the bottleneck channel at 4GHz and 700MHz

Per the agreement in RAN1#102 e-meeting, payload size of SIP message is 1500 byte and 1second transmission period can be assumed. The packet size of SIP message can be calculated via 1500*8/50= 240 bits with 20ms data arriving interval. It should be noted that the actual payload size is much smaller than that of VoIP, i.e. 320 bits. With this regard, SIP message carrying by PDSCH is certainly not the bottleneck as downlink VoIP meets the requirement. Accordingly, at least SIP message carried by PDSCH doesn’t need to be evaluated independently.

Regarding performance evaluation of MSG4, based on agreement in RAN1#102 e-meeting, payload size of MSG4 message is 1040bits. In this case, Data rate of MSG4 is about 1Mbps which is lower than data rate (10Mbps) of PDSCH. In this regard, if PDSCH isn’t the bottleneck, MSG4 isn’t the bottleneck as well.

With observation 1, 2, 3, we have the following proposal:

Proposal 1: PUCCH and PUSCH including Msg3 are identified as the bottleneck channels in FR1 which should be further studied.

Conclusion

This contribution discussed the preliminary simulation results for PDCCH/SSB/PDSCH/PRACH/ PUCCH/PUSCH in FR1, we have the following observations.

Observation 1: In Urban scenario, the coverage of PUSCH is the bottleneck.

- Based on MPL evaluation results, eMBB PUSCH is the bottleneck channel in urban scenario for both 2.6 GHz and 4 GHz

  - Up to 7.83 dB performance gap needs to be compensated in terms of MPL

- Based on MIL/MCL, evaluation result, eMBB PUSCH，VOIP PUSCH and MSG3 PUSCH are the bottleneck channel

Observation 2: In Rural scenario, the PDCCH, SSB, PDSCH, PRACH are not bottleneck. The coverage of PUCCH and PUSCH are bottleneck.

- Based on MPL evaluation results, PUCCH format 3, eMBB PUSCH, VoIP PUSCH, Msg3 PUSCH are bottleneck channels

- Based on MIL/MCL evaluation results, eMBB PUSCH，PUCCH format 3，MSB3 PUCCH and VOIP PUSCH are the bottleneck channels.

Observation 3: The coverage of most of channels can meet the requirements with target ISD 12km except the VOIP PUSCH without repetition, Msg3 PUSCH, PUCCH format3 and eMBB PUSCH.

- Based on MPL evaluation results, eMBB PUSCH and VOIP PUSCH cannot meet the requirements at 4GHz, up to7.59 dB performance gap needs to be compensated.

- Based on  MIL/MCL evaluation results, Msg3 PUSCH, PUCCH format3 and eMBB PUSCH are the bottleneck channel at 4GHz and 700MHz

With observation 1, 2, 3, we have the following proposal:

Proposal 1: PUCCH and PUSCH including Msg3 are identified as the bottleneck channels in FR1 which should be further studied.

References

  - RP-193240, New SID on NR coverage enhancement, China Telecom, RAN Meeting #86.

  - R1-2007480, [102-e-Post-NR-CovEnh-02] Summary on email discussion/approval of Phase 1 - link budget template

Appendix: the LLS assumptions

- 5.1 The LLS assumptions

Table 23: simulation assumptions of PUSCH

| Parameters | Values |  |  |
| --- | --- | --- | --- |
| Scenario | Urban | Rural | Rural with long distance |
| Carrier | TDD for 4GHz, 2.6GHz | TDD for 4GHz, 2.6GHz FDD for2GHz. 700MHz | TDD for 4GHz FDD for 700MHz |
| TDD configuration | TDD config 1: DDDSU (S: 10D:2G:2U) TDD confg 2: DDDSUDDSUU (S: 10D:2G:2U) TDD config 3: DDDDDDDSUU (S: 6D:4G:4U) | TDD config 1: DDDSU (S: 10D:2G:2U) TDD confg 2: DDDSUDDSUU (S: 10D:2G:2U) TDD config 3: DDDDDDDSUU (S: 6D:4G:4U) | TDD config 1: DDDSU (S: 10D:2G:2U) TDD confg 2: DDDSUDDSUU (S: 10D:2G:2U) |
| SCS | 30kHz for TDD | 30kHz for TDD, 15kHz for FDD. | 30kHz for TDD, 15kHz for FDD. |
| Number of antenna elements for BS | 192 antenna elements, (M,N,P,Mg,Ng) = (8,4,2,1,1) | 64 antenna elements for 4GHz and 2.6GHz, (M,N,P,Mg,Ng) = (8,4,2,1,1) 32 antenna elements for 2GHz, (M,N,P,Mg,Ng) = (8,2,2,1,1) 16 antenna elements for 700MHz, (M,N,P,Mg,Ng) = (4,2,2,1,1) | 64 antenna elements for 4GHz, (M,N,P,Mg,Ng) = (8,4,2,1,1) 16 antenna elements for 700MHz, (M,N,P,Mg,Ng) = (4,2,2,1,1) |
| Number of TxRUs for BS | 64 TXRUs | 64 antenna elements for 4GHz and 2.6GHz, (M,N,P,Mg,Ng) = (8,4,2,1,1) 32 antenna elements for 2GHz, (M,N,P,Mg,Ng) = (8,2,2,1,1) 16 antenna elements for 700MHz, (M,N,P,Mg,Ng) = (4,2,2,1,1) | 2 or 4 TXRUs for 700 MHz 64TxRUs for 4 GHz |
| Number of Tx chains for BS | 4 | 4 | 4 |
| Number of UE Tx chains | 1 | 1 | 1 |
| Duration of PUSCH | 14 OS | 14 OS | 14 OS |
| Number of DMRS | For eMBB/VoIP: 1 OS For Msg 3: 3 OS | For eMBB/VoIP: 3km/h: 1 OS 120km/h: 2 OS For Msg 3: 3 OS | For eMBB/VoIP: 3km/h: 1 OS 120km/h: 2 OS For Msg 3: 3 OS |
| Occupied channel bandwidth | 30 PRBs for eMBB 4 PRBs for VoIP 2 PRBs for Msg 3 | 4 PRBs for eMBB 4 PRBs for VoIP 2 PRBs for Msg 3 | 4 PRBs for eMBB 4 PRBs for VoIP 2 PRBs for Msg 3 |
| Modulation | QPSK | QPSK | QPSK |
| Coding Rate | TDD config 1: eMBB: CR=251/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; TDD config 2: eMBB: CR=193/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; TDD config 3: eMBB: CR=251/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; | TDD config 1: 3km/h: eMBB: CR=193/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; 120km/h: eMBB: CR=251/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; TDD config 2: 3km/h: eMBB: CR=157/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; 120km/h: eMBB: CR=157/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; TDD config 3: 3km/h: eMBB: CR=193/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; 120km/h: eMBB: CR=251/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; FDD: 3km/h: eMBB: CR=99/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; 120km/h: eMBB: CR=99/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; | TDD config 1: 3km/h: eMBB: CR=30/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; 30km/h: eMBB: CR=30/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; TDD config 2: 3km/h: eMBB: CR=30/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; 30km/h: eMBB: CR=30/1024; VoIP: CR=308/1024; Msg3: CR=120/1024; |
| Channel Coding | LDPC | LDPC | LDPC |
| Channel Mode | TDL-C, 300ns | TDL-C, 300ns TDL-D, 300ns | TDL-D, 30ns |
| UE Mobility: | 3km/h | 3km/h for indoor, 120km/h for outdoor | 3km/h for indoor, 120km/h for outdoor |
| Channel estimation | True | True | True |
| Target BLER | 10% BLER for eMBB 2% BLER for VoIP 10% BLER for Msg3 | 10% BLER for eMBB 2% BLER for VoIP 10% BLER for Msg3 | 10% BLER for eMBB 2% BLER for VoIP 10% BLER for Msg3 |
| PUSCH repetition | w/o repetition for eMBB/Msg 3 w/o repetition and w/ repetition for VoIP: 8 repetitions for TDD config 1 12 repetitions for TDD config 2 8 repetitions for TDD config 3 | w/o repetition for eMBB/Msg 3 w/o repetition and w/ repetition for VoIP: 8 repetitions for TDD config 1 12 repetitions for TDD config 2 8 repetitions for TDD config 3 20 repetitions for FDD | w/o repetition for eMBB/Msg 3 w/o repetition and w/ repetition for VoIP: 8 repetitions for TDD config 1 12 repetitions for TDD config 2 20 repetitions for FDD |
| Waveform | DFT-s-OFDM | DFT-s-OFDM | DFT-s-OFDM |

Table 24: simulation assumptions of PUCCH

| Parameters | Values |  |  |
| --- | --- | --- | --- |
| Scenario | Urban | Rural | Rural with long distance |
| Carrier | 4GHz, 2.6GHz | TDD for 4GHz, 2.6GHz FDD for2GHz. 700MHz | TDD for 4GHz FDD for 700MHz |
| Format type | PUCCH format 1 with 2bits PUCCH format 3 with 11/22 bits | PUCCH format 1 with 2bits PUCCH format 3 with 11/22 bits | PUCCH format 1 with 2bits PUCCH format 3 with 11/22 bits |
| BWP | 100MHz | 100MHz for TDD 20MHz for FDD | 20MHz |
| SCS | 30kHz | 30kHz for TDD 15kHz for FDD | 15kHz |
| Number of antenna elements for BS | 192, (M,N,P,Mg,Ng) = (12,8,2,1,1) | 64, (M,N,P,Mg,Ng) = (8,4,2,1,1) 32, (M,N,P,Mg,Ng) = (8,2,2,1,1) | 16, (M,N,P,Mg,Ng) = (4,2,2,1,1) |
| Number of TxRUs for BS | 4 | 4 | 4 |
| Number of UE Tx chains | 1 | 1 | 1 |
| Duration of PUSCH | 14 OS | 14 OS | 14 OS |
| DMRS configuration for PUCCH | w/o additional DMRS | w/o additional DMRS | w/o additional DMRS |
| Occupied channel bandwidth | 1 PRB | 1 PRB | 1 PRB |
| Channel Mode | TDL-C, 300ns | TDL-D, 300ns for O2O TDL-C, 300ns for O2I | TDL-D, 30ns |
| UE Mobility: | 3 km/h | 120 km/h for outdoor 3 km/h for indoor | 120 km/h |
| Channel estimation | True | True | True |
| PUCCH repetition | 1/2 | 1/2 | 1/2 |
| Frequency hopping | On | On | On |

Table 25: simulation assumptions of PRACH

| Parameters | Values |  |  |
| --- | --- | --- | --- |
| Scenario | Urban | Rural | Rural with long distance |
| Carrier | 4GHz, 2.6GHz | TDD for 4GHz, 2.6GHz FDD for2GHz. 700MHz | TDD for 4GHz FDD for 700MHz |
| Format type | PRACH format 0 PRACH format B4 | PRACH format 0 PRACH format B4 | PRACH format 0 PRACH format B4 |
| SCS | 30kHz | 30kHz for TDD 15kHz for FDD | 15kHz |
| Number of TxRUs for BS | 4 | 4 | 4 |
| Number of UE Tx chains | 1 | 1 | 1 |
| Duration of PUSCH | 14 OS | 14 OS | 14 OS |
| Occupied channel bandwidth | 12 PRB | 12 PRB | 12 PRB |
| Channel Mode | TDL-C, 300ns | TDL-D, 300ns for O2O TDL-C, 300ns for O2I | TDL-D, 30ns |
| UE Mobility: | 3 km/h | 120 km/h for outdoor 3 km/h for indoor | 120 km/h |
| PRACH BLER | 1% missed detection at 0.1% false alarm probability | 1% missed detection at 0.1% false alarm probability | 1% missed detection at 0.1% false alarm probability |

Table 26: simulation assumptions of PDSCH

| Parameters | Values |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario | Urban |  |  |  | Rural |  | Rural with long distance |  |
|  | eMBB |  | VoIP |  | eMBB | VoIP | eMBB | VoIP |
| Carrier: | 4GHz (TDD), 2.6GHz (TDD) |  |  |  | 4GHz (TDD), 2.6GHz (TDD) 2GHz (FDD), 700MHz (FDD) |  | 700MHz (FDD), 4GHz (TDD) |  |
| BWP | 100MHz for 4GHz and 2.6GHz. |  |  |  | 100MHz for 4GHz and 2.6GHz. 20MHz for 2GHz 20MHz for 700MHz. |  | 100MHz for 4GHz. 20MHz for 700MHz. |  |
| SCS | 30kHz |  |  |  | 30kHz for TDD, 15kHz for FDD. |  | 30kHz for TDD, 15kHz for FDD. |  |
| Number of antenna elements for BS | 192 antenna elements for 4GHz and 2.6GHz, (M,N,P,Mg,Ng) = (12,8,2,1,1) (optional) 128 antenna elements for 4GHz, (M,N,P,Mg,Ng) = (8,8,2,1,1) |  |  |  | 64 antenna elements for 4GHz and 2.6GHz (M,N,P,Mg,Ng) = (8,4,2,1,1) 32 antenna elements for 2GHz (M,N,P,Mg,Ng) = (8,2,2,1,1) 16 antenna elements for 700MHz (M,N,P,Mg,Ng) = (4,2,2,1,1) |  | 64 antenna elements for 4GHz and 2.6GHz (M,N,P,Mg,Ng) = (8,4,2,1,1) 32 antenna elements for 2GHz (M,N,P,Mg,Ng) = (8,2,2,1,1) 16 antenna elements for 700MHz (M,N,P,Mg,Ng) = (4,2,2,1,1) |  |
| Number of TxRUs for BS | 64TxRUs for 2.6 and 4 GHz. |  |  |  | 2 or 4 TXRUs for 2GHz, 700 MHz 64TxRUs for 2.6 and 4 GHz. Optional: 32 TXRUs at 2 GHz |  | 2 or 4 TXRUs for 700 MHz 64TxRUs for 4 GHz. |  |
| UE receive chains | 4 for 4GHz/2.6GHz |  |  |  | 4 for 4GHz/2.6GHz 4 for 2GHz 2 for 700MHz |  | 4 for 4GHz; 2 for 700MHz |  |
| length of PDSCH | 12 OS |  |  |  | 12 OS |  | 12 OS |  |
| Configuration of DMRS | 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. |  |  |  | 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. |  | For 120km/h, (Optional: 30km/h): Type I, 2 or 3 DMRS symbol, no multiplexing with data. |  |
| Data rate | 10Mbps Msg4 payload size:1040bits | 320 bits with 20ms data arriving |  |  | 1Mbps Msg4 payload size:1040bits | 320 bits with 20ms data arriving | 1Mbps Msg4 payload size:1040bits | 320 bits with 20ms data arriving |
| PRBs/TBS/MCS [TBD] | full bandwidth | 320 bits with 20ms data arriving |  |  | full bandwidth | 320 bits with 20ms data arriving | full bandwidth | 320 bits with 20ms data arriving |
| Channel Coding | LDPC |  |  |  | LDPC |  | LDPC |  |
| Channel Model | TDL-C for NLOS, |  |  |  | TDL-C for NLOS, TDL-D for LOS. |  | TDL-C for NLOS, TDL-D for LOS. |  |
| DS | 300ns |  |  |  | 300ns |  | 30ns |  |
| UE velocity: | 3km/h |  |  |  | 3km/h |  | 120km/h |  |
| Channel estimation | True |  |  |  | True |  | True |  |
| Target BLER | w/o HARQ, 10% iBLER. |  |  | 2% rBLER. | w/o HARQ, 10% iBLER. | 2% rBLER. | w/o HARQ, 10% iBLER. | 2% rBLER. |
| UL DL configuration if TDD is assumed: | DDDSU (S: 10D:2G:2U) only for 4GHz DDDSUDDSUU (S: 10D:2G:2U) only for 4GHz DDDDDDDSUU (S: 6D:4G:4U) only for 2.6GHz |  |  |  | DDDSU (S: 10D:2G:2U) only for 4GHz DDDSUDDSUU (S: 10D:2G:2U) only for 4GHz DDDDDDDSUU (S: 6D:4G:4U) only for 2.6GHz |  | DDDSU (S: 10D:2G:2U) only for 4GHz DDDSUDDSUU (S: 10D:2G:2U) only for 4GHz DDDDDDDSUU (S: 6D:4G:4U) only for 2.6GHz |  |

Table 27: simulation assumptions of PDCCH

| Parameters | Values |
| --- | --- |
| Carrier | 2GHz/2.6GHz/4GHz/700MHz |
| SCS | 30kHz, 15kHz |
| Aggregation level | 16 |
| Payload | 40 bits + 24CRC bits |
| CORESET size | 2 symbols, 48 PRBs |
| Tx Diversity | 1-port precoder cycling |
| BLER for PDCCH | 1% BLER |
| Number of SSB for broadcast PDCCH of Msg.2 | 4 |

Table 28: simulation assumptions of SSB

| Parameters | Values |
| --- | --- |
| Carrier | 2GHz/2.6GHz/4GHz/700MHz |
| Periodicity | 20ms |
| SCS | 15KHz, 30KHz |
| BLER for SSB | 1% BLER |
| Performance metric | Combination of 4 SSBs in 80ms. Note: UE is not assumed to know the SS/PBCH block index |