3GPP TSG RAN WG1 Meeting #103-e	R1-2008316

E-meeting, October 26 – November 13, 2020

Agenda Item:	8.14.2

Source:	Huawei, HiSilicon

Title:	Initial evaluation results for XR and Cloud Gaming

Document for:	Discussion and Decision

Introduction

eXtended Reality (XR) and Cloud Gaming (CG) are quite often viewed as the important media applications enabled by 5G. Representative forms of XR are Augmented Reality (AR), Mixed Reality (MR) and Virtual Reality (VR) and the areas interpolated among them. 5G NR and its evolution are expected to support applications demanding high throughput and low latency in line with the requirements of XR and CG. In RAN#86, a new study item on XR evaluations for NR [1] was approved. According to the SID [1], the following objectives are to be studied:

The following applications are to be considered as starting points for this study: - VR1: “Viewport dependent streaming” - VR2: “Split Rendering: Viewport rendering with Time Warp in device” - AR1: “XR Distributed Computing” - AR2: “XR Conversational” - CG: Cloud Gaming Note: Use cases in quotes are from TR26.928. The following traffic parameters for the different applications are to be considered as starting point for the study: Traffic characteristics: - UL and DL File Size distribution (e.g., Pareto with given parameters) - UL and DL File arrival time distribution (e.g., Periodic every 1/60 seconds) Traffic requirements: - Round-trip-time or UL and DL one-way Packet delay budget (PDB) - UL and DL Packet error rate (PER) The objective of this study item are as follows: - Confirm XR and Cloud Gaming applications of interest - Identify the traffic model for each application of interest taking outcome of SA WG4 work as input, including considering different upper layer assumptions, e.g. rendering latency, codec compression capability etc. - Identify evaluation methodology to assess XR and CG performance along with identification of KPIs of interest for relevant deployment scenarios - Once traffic model and evaluation methodologies are agreed, carry out performance evaluations towards characterization of identified KPIs Note 1: eURLLC SI/WI work relevant to XR should be taken into consideration. Note 2: Traffic model for the performance evaluation shall be based on the standardization in SA WG4

In this contribution, we provide our initial system level evaluation results.

Example of KPI for XR and CG evaluation

As discussed in our companion paper [2], existing KPIs in RAN1, such as throughput, reliability, and latency, cannot directly reflect the user experience in XR and CG services, and thus not applicable to be KPIs. Therefore, it is proposed that RAN1 needs to identify a KPI that can reflect the user experience in XR and CG services, and such desired KPI is called XR Quality Index (XQI) in our contributions. Furthermore, it is proposed in [2] that XQI should reflect the impact of network transmission so that XQI should be calculated with RAN available statistics and parameters.

In this contribution, an example of XQI calculation based on the RAN-available statistics and parameters is provided. Assume the video source is given and the RAN-available statistics/parameters are PER and PDB, then XQI calculation procedure is as follows:

- Step 1: For different PER/PDB values, generate the degraded videos accordingly

- Step 2: Calculate the scores of the degraded videos according to some existing video quality evaluation method

  - Note: there are many existing video quality evaluation methods that can well reflect user experience. In this contribution, the method in [3] is taken as an example.

- Step 3: After Step 1 and Step 2, some samples of the score with the corresponding PER and PDB are obtained, and then perform function-fitting to obtain the XQI relationship with PDB and PER, i.e., XQI=f(PER, PDB)

Simulation parameters

In our current evaluation, FR1 Dense Urban scenario and DL traffic of XR and Cloud Gaming (CG) are considered. As shown in Table 1, two cases are considered, where Case 1 is for XR service with the typical downlink bitrate 60 Mbps, and Case 2 is for CG service with the typical downlink bitrate 35 Mbps [2]. 
60 FPS (frames per second) is considered for both two cases and thereby the frame arrival interval is 
1/60 s=16.67 ms. The frame size is assumed to follow truncated Gaussian distribution, where the details are summarized in Table 2. More simulation parameters can be found in the Annex.

Note that for the initial evaluation, a frame can be segmented into multiple IP packets for transmission. In RAN, each received IP packet is packaged into a PDCP packet in the PDCP layer. The PDCP packets are multiplexed into transport blocks (TBs) with or without segmentation, which are then delivered in physical layer over the air interface.

Table 1. Two cases: XR and CG

| Simulation cases | DL bitrate | DL Frame Size distribution |
| --- | --- | --- |
| Case 1 (XR) | 60 Mbps | Truncated Gaussian |
| Case 2 (CG) | 35 Mbps | Truncated Gaussian |

Table 2. Detailed traffic model of Case 1 (XR) and Case 2 (CG)

| Traffic model | Case 1 (XR) | Case 2 (CG) |
| --- | --- | --- |
| Frame size distribution | Truncated Gaussian | Truncated Gaussian |
| Mean frame size (bits) | 1000200 | 583450 |
| STD of frame sizes (bits) | 100020 | 58345 |
| Packet arrival interval (ms) | 16.67 | 16.67 |

Evaluation Results

In our simulation, different numbers of users per cell, e.g., 3, 5, 7, 10, 15 and 20, are considered. As discussed in Section 2 and our companion paper [2], XQI is chosen as the KPI and is used to characterize the user experience or XR quality through RAN transmission of a XR/CG user. Tthe performance requirement of a XR/CG user is deemed to be satisfied if the XQI score >= given threshold. In our current simulation, XQI = 70 is chosen as the threshold (the maximum XQI score is 100).

According to the evaluation methodology proposed in [2], the capacity of a network is defined as the maximum number of users per cell for which the system’s user satisfaction rate is >= given threshold, where user satisfaction rate=MsatisfiedMtotal, Msatisfied is number of UEs whose traffic requirement is deemed satisfied, and Mtotal is the total number of UEs. In our current simulation, the user satisfaction rate threshold is chosen as 90%.

The initial system level evaluation results are as follows:

  - Case 1 (XR Service)

Figure 1. Capacity simulation results of Dense Urban for XR

  - Case 2 (CG Service)

Figure 2. Capacity simulation results of Dense Urban for CG

From Figure 1, it can be observed that for XR service in the scenario FR1 Dense Urban MU-MIMO, i.e., Case 1, the network’s capacity is 5 users per cell. While for Case 2 (CG service), the network’s capacity is 16 users per cell as shown in Figure 2. It’s evident that the capacity of CG service is larger than XR service mainly because CG service has lower data rate. In addition, for both VR and CG services, the user satisfaction rate degrades with increasing of the number of UEs per cell.

Observation 1: For XR service in the scenario FR1 Dense Urban MU-MIMO, 5 users per cell can be supported with the XQI threshold 70 at a user satisfaction rate threshold 90%.

Observation 2: For CG service in the scenario FR1 Dense Urban MU-MIMO, 16 users per cell can be supported with the XQI threshold 70 at a user satisfaction rate threshold 90%.

Conclusions

In this contribution, initial system level evaluation results of the scenario of FR1 dense urban for XR and CG service are discussed with the following observations:

Observation 1: For XR service in the scenario FR1 Dense Urban MU-MIMO, 5 users per cell can be supported with the XQI threshold 70 at a user satisfaction rate threshold 90%.

Observation 2: For CG service in the scenario FR1 Dense Urban MU-MIMO, 16 users per cell can be supported with the XQI threshold 70 at a user satisfaction rate threshold 90%.

- References

RP-193241, RAN#86, Sitges, Spain, December 9-12, 2019

R1-2007561, “Discussion on applications, traffic model and evaluation methodology for XR and Cloud Gaming”, Huawei, HiSilicon, RAN1#103-e, October 26 – November 13, 2020

https://carrier.huawei.com/~/media/CNBGV2/download/products/servies/The-Technology-White-Paper-en.pdf

- Annex

Table I. System level simulation assumption for FR1 Dense Urban

| Parameter | value |
| --- | --- |
| Scenarios | Dense Urban |
| Carrier frequency | 3.5 GHz |
| Bandwidth | 100 MHz |
| Subcarrier spacing | 30 KHz |
| Frame structure | [DDDSU DDDSU] |
| Inter Site Distance | 200m |
| BS Antenna height | 25m |
| BS Antennas (M,N,P,Mg,Ng;Mp,Np) | For 64T: (12,8,2,1,1;4,8) |
| UE Antennas (M,N,P,Mg,Ng;Mp,Np) | For 4R: (1,2,2,1,1;1,2) |
| BS antenna pattern | 3-TRxP pattern, 8 dBi |
| UE antenna pattern | Omnidirectional, 0 dBi |
| TX Power | BS : 51 dBm |
| UE Power class | 23 dBm |
| Noise Figure | BS:5 dB, UE:9 dB |
| Scheduling | MU-MIMO Proportional Fair |
| Max MCS | 256 QAM |
| Device deployment | 80% indoor, 20% outdoor |
| UE speed | 3 km/h |