3GPP TSG-RAN WG1 Meeting #103-e	R1-2008326

# e-Meeting, October 26–November 13, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev |  | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the index value range of CORESET for Multi-DCI Transmission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-Core |  |  |  |  |  | Date: |  |  | 2020-10-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In current spec, multi-TRP operation is typically identified through CORESET configuration per active/configured BWP, as the following examples: “If, for an active DL BWP of a serving cell, the UE is not provided CORESETPoolIndex or is provided CORESETPoolIndex with value 0 for one or more first CORESETs and is provided CORESETPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode-r16 = joint, ……” “For each DL BWP configured to a UE in a serving cell, the UE can be provided by higher layer signalling with - P≤3 CORESETs if CORESETPoolIndex is not provided, or if a value of CORESETPoolIndex is same for all CORESETs if CORESETPoolIndex is provided”. Therefore an active BWP configured with different values of CORESETPoolIndex will be regarded as multi-TRP transmission based on multi-DCI. However there’s potential ambiguity for applicable value range of controlResourceSetId, depending on how to interpret “same for ALL CORESETs” within a given set of CORESETs. “- 0<p<12 if CORESETPoolIndex is not provided, or if a value of CORESETPoolIndex is same for all CORESETs if CORESETPoolIndex is provided; - 0<p<16 if CORESETPoolIndex is not provided for a first CORESET, or is provided and has a value 0 for a first CORESET, and is provided and has a value 1 for a second CORESET;” For example, when BWP1 is configured with same CORESETPoolIndex value 0 for all CORESETs in BWP1, and BWP2 is configured with same CORESETPoolIndex value 1 for all CORESETs in BWP2. There are two possilbe intepretaions: - Assumption 1: No matter which BWP is activated, either BWP1 and BWP2, the UE expect to have single-TRP-like transmission so that 0<p<12 is applied to the value range of controlResourceSetId for that serving cell. - Assumption 2: If the interpretation of “being same for all CORESETs” is applied across BWPs per serving cell, e.g. across BWP1 and BWP2, the UE expect to have 0<p<16 applied to the value range of controlResourceSetId for that serving cell. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify applicable value range of controlResourceSetId for BWPs in a serving cell, if a value of CORESETPoolIndex is same (or different) for all CORESETs within each DL BWP of a serving cell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There could be ambiguity between gNB and UE about applicable value range of controlResourceSetId. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 10.1	UE procedure for determining physical downlink control channel assignment

< Unchanged parts are omitted >

For each CORESET, the UE is provided the following by ControlResourceSet:

-	a CORESET index p, by controlResourceSetId, where

-	0<p<12 if CORESETPoolIndex is not provided, or if a value of CORESETPoolIndex is same for all CORESETs within each DL BWP of  a serving cell if CORESETPoolIndex is provided;

-	0<p<16 if CORESETPoolIndex is not provided for a first CORESET, or is provided and has a value 0 for a first CORESET, and is provided and has a value 1 for a second CORESET in any DL BWP of  a serving cell;

< Unchanged parts are omitted >