3GPP TSG RAN WG1 #103-e	R1-2008398

e-Meeting, October 26th – November 13th, 2020

Source:	Sharp

Title:	Link budget analysis for FR1

Agenda Item:	8.8.1.1

Document for:	Discussion and Decision

Background

Link budget template for study of the baseline coverage is agreed in the post meeting e-mail discussion [102-e-Post-NR-CovEnh-02]. We provide analysis of link budget with MCL/MIL/MPL and the maximum range for each scenario.

Link budget analysis

The evaluation assumptions are in the excel file (initialResults.xlsx) attached in the Tdoc. Table 1 to 5 shows analysis on MCL/MIL/MPL for each deployment scenarios.

Table 1: MCL/MIL/MPL for Urban scenario (4 GHz, TDD, DDSUU)

|  | PBCH | broadcast PDCCH | UE-specific PDCCH | msg2 PDSCH | msg4 PDSCH | PDSCH for eMBB | PRACH | msg3 PUSCH | Cell-specific PUCCH | PUCCH (22 bits) | PUSCH for eMBB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCL [dB] | 146.49 | 143.49 | 143.49 | 147.49 | 142.99 | 142.49 | 132.68 | 129.46 | 131.40 | 130.47 | 117.20 |
| MIL [dB] | 159.52 | 156.52 | 167.31 | 160.52 | 156.02 | 166.31 | 153.49 | 150.27 | 152.21 | 151.28 | 138.01 |
| MPL [dB] | 133.98 | 130.98 | 141.77 | 138.06 | 133.56 | 143.85 | 127.95 | 127.81 | 126.67 | 125.74 | 115.55 |
| Max. Range [m] | 593.98 | 497.74 | 940.05 | 755.39 | 579.46 | 1062.61 | 416.39 | 413.00 | 386.06 | 365.59 | 200.55 |

Table 2: MCL/MIL/MPL for Urban scenario (2.6 GHz, TDD, DDDSUDDSUU)

|  | PBCH | broadcast PDCCH | UE-specific PDCCH | msg2 PDSCH | msg4 PDSCH | PDSCH for eMBB | PRACH | msg3 PUSCH | Cell-specific PUCCH | PUCCH (22 bits) | PUSCH for eMBB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCL [dB] | 146.49 | 143.49 | 143.49 | 147.49 | 142.99 | 142.49 | 132.68 | 129.46 | 131.40 | 130.47 | 119.20 |
| MIL [dB] | 159.52 | 156.52 | 167.31 | 160.52 | 156.02 | 166.31 | 153.49 | 150.27 | 152.21 | 151.28 | 140.01 |
| MPL [dB] | 133.98 | 130.98 | 141.77 | 138.06 | 133.56 | 143.85 | 127.95 | 127.81 | 126.67 | 125.74 | 117.55 |
| Max. Range [m] | 740.48 | 620.51 | 1171.91 | 941.71 | 722.38 | 1324.71 | 519.09 | 514.87 | 481.29 | 455.76 | 281.28 |

Table 3: MCL/MIL/MPL for Rural scenario (4 GHz, TDD, DDDSU)

|  | PBCH | broadcast PDCCH | UE-specific PDCCH | msg2 PDSCH | msg4 PDSCH | PDSCH for eMBB | PRACH | msg3 PUSCH | Cell-specific PUCCH | PUCCH (22 bits) | PUSCH for eMBB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCL [dB] | 144.99 | 142.99 | 142.99 | 147.49 | 142.99 | 142.49 | 136.68 | 129.96 | 131.40 | 130.47 | 117.20 |
| MIL [dB] | 158.02 | 156.02 | 162.04 | 160.52 | 156.02 | 161.54 | 152.72 | 146.00 | 147.44 | 146.51 | 133.24 |
| MPL [dB] | 131.59 | 129.59 | 135.61 | 137.41 | 132.91 | 138.43 | 126.29 | 122.89 | 121.01 | 120.08 | 110.13 |
| Max. Range [m] | 807.18 | 717.47 | 1022.91 | 1137.30 | 872.46 | 1207.77 | 590.71 | 483.53 | 432.71 | 409.77 | 228.00 |

Table 4: MCL/MIL/MPL for Rural scenario (2.6 GHz, TDD, DDDSU)

|  | PBCH | broadcast PDCCH | UE-specific PDCCH | msg2 PDSCH | msg4 PDSCH | PDSCH for eMBB | PRACH | msg3 PUSCH | Cell-specific PUCCH | PUCCH (22 bits) | PUSCH for eMBB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCL [dB] | 144.99 | 142.99 | 142.99 | 147.49 | 142.99 | 142.49 | 136.68 | 129.96 | 131.40 | 130.47 | 117.20 |
| MIL [dB] | 158.02 | 156.02 | 162.04 | 160.52 | 156.02 | 161.54 | 152.72 | 146.00 | 147.44 | 146.51 | 133.24 |
| MPL [dB] | 131.59 | 129.59 | 135.61 | 137.41 | 132.91 | 138.43 | 125.79 | 122.89 | 121.01 | 119.58 | 112.13 |
| Max. Range [m] | 1150.82 | 1022.91 | 1458.38 | 1621.46 | 1243.88 | 1721.94 | 842.18 | 689.38 | 616.93 | 584.21 | 325.07 |

Table 5: MCL/MIL/MPL for Rural scenario (0.7 GHz, FDD)

|  | PBCH | broadcast PDCCH | UE-specific PDCCH | msg2 PDSCH | msg4 PDSCH | PDSCH for eMBB | PRACH | msg3 PUSCH | Cell-specific PUCCH | PUCCH (22 bits) | PUSCH for eMBB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCL [dB] | 144.99 | 142.99 | 142.99 | 147.49 | 142.99 | 129.49 | 136.18 | 129.96 | 131.40 | 129.97 | 120.20 |
| MIL [dB] | 155.01 | 153.01 | 156.02 | 157.51 | 153.01 | 142.52 | 146.20 | 139.98 | 141.42 | 139.99 | 130.22 |
| MPL [dB] | 128.58 | 126.58 | 129.59 | 134.40 | 129.90 | 119.41 | 119.77 | 116.87 | 114.99 | 113.56 | 107.11 |
| Max. Range [m] | 1649.24 | 1465.94 | 1750.38 | 2323.73 | 1782.61 | 960.91 | 981.46 | 827.40 | 740.45 | 680.83 | 465.58 |

We found that PUSCH for eMBB is the worst, and the Cell-specific PUCCH and PUCCH (22 bits) are also worse than other channels. Therefore, we propose,

Proposal 1: Coverage for UL channels at least including PUSCH for eMBB, PUCCH with 22 bits and Cell-specific PUCCH should be enhanced in Rel-17.

Conclusion

In this contribution, we have the following proposals:

Proposal 1: Coverage for UL channels at least including PUSCH for eMBB, PUCCH with 22 bits and Cell-specific PUCCH should be enhanced in Rel-17.

References

- Chairman’s note at RAN1#102 e-Meeting, May 2020