3GPP TSG- RAN WG1 Meeting #103-e                                                   R1-2008452

# e-Meeting, October 26th – November 13th, 2020

Agenda Item:	8.13.2

Source:	Apple Inc.

Title:	Views on Rel-17 DSS Multi-cell PDSCH scheduling via a single DCI

Document for:	Discussion/Decision

# Introduction

Dynamic spectrum sharing allows NR to be co-channel deployed with LTE, i.e., NR and LTE share the same spectrum, to address the spectrum availability issue from the operator. However, from performance or user experience perspective, it is very hard for DSS deployment to deliver better performance with NR compared to LTE. In Rel-15/16, multiple solutions are adopted to allow DSS in many areas such as (1) symbol level rate matching, including DMRS, mini-slot scheduling, flexible CORESET starting symbol within a slot, etc. (2) RB level rate matching via RRC configuration (3) RE level PDSCH rate matching to LTE.

In the Rel-17 “New WID on NR Dynamic spectrum sharing (DSS)”, the following objective was approved to address the PCell scheduling restriction with DSS

This work item is limited to FR1, and includes the following objectives for NR Dynamic Spectrum Sharing (DSS): - PDCCH enhancements for cross-carrier scheduling including [RAN1, RAN2] - PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell - Study, and if agreed specify PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI - The number of cells can be scheduled at once is limited to 2 - The increase in DCI size should be minimized - Note: The total PDCCH blind decoding budget should not be changed as a result of this work - Note: These enhancements are not specific to DSS and are generally applicable to cross-carrier scheduling in carrier aggregation

In RAN1#102e meeting, the following agreement was reached [2]

Agreements: - For the study on single DCI scheduling PDSCH on two cells - Consider the following scenarios as baseline for evaluation - UE configured with Inter-band CA with PCell and an SCell - PCell for the UE is operated on a DSS carrier (i.e., same carrier is also used for serving LTE users) - Case 1: Different SCS for PCell and SCell - Case 2: Same SCS for PCell and Scell - Additional scenarios can also be evaluated, e.g. as below - Intra-band CA case with multiple serving cells having same SCS (all cells operated on non DSS carriers) - Inter-band CA case with PCell and more than one SCell (at least the SCells are operated on non DSS carriers) - Note: other combinations not precluded - Note: Further details of evaluation framework (including carrier BW, slot format etc.) to be discussed in next stage

In this contribution, we provide our views on the objective of PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI.

# Existing PDCCH monitoring restrictions

For the current Rel-15/16, there are a few PDCCH monitoring related capability, we listed a few most notable ones

- FG3-1: Basic PDCCH monitoring

  - Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot per scheduled CC for FDD

  - Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot per scheduled CC for TDD

- FG3-5b: Span based PDCCH monitoring

  - Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per scheduled CC per monitoring occasions with the same span for FDD

  - Processing one unicast DCI scheduling DL and two unicast DCI scheduling UL per scheduled CC per monitoring occasions with the same span for TDD

  - Processing two unicast DCI scheduling DL and one unicast DCI scheduling UL per scheduled CC per monitoring occasions with the same span for TDD

- FG11-2: Rel-16 eURLLC enhanced span based PDCCH monitoring

  - Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per scheduled CC per monitoring occasions with the same span for FDD

  - Processing one unicast DCI scheduling DL and two unicast DCI scheduling UL per scheduled CC per monitoring occasions with the same span for TDD

  - Processing two unicast DCI scheduling DL and one unicast DCI scheduling UL per scheduled CC per monitoring occasions with the same span for TDD

We have the following observation, For UE with advanced PDCCH monitoring capability, e.g., FG3-5b or FG11-2, UE can already be scheduled with multiple unicast DL DCI and UL DCI within a slot in the current NR design, especially considering the span based PDCCH monitoring.

Observation 1: For UE with advanced PDCCH monitoring capability, e.g., FG3-5b or FG11-2, UE can already be scheduled with multiple unicast DL DCI and UL DCI within a slot in the current NR design, especially considering the span based PDCCH monitoring.

# Rel-16 Single-DCI Multi-TRP solution

During the Rel-117 DSS enhancement WID plenary discussion, proponent of this objective brought up Single-DCI Multi-TRP solution adopted in Rel-16 eMIMO WI, as a candidate to be reused for DSS enhancement. In fact, Rel-16 Single-DCI Multi-TRP supports single DCI simultaneous PDSCH scheduling from two TRP without increasing the DCI size, i.e., introducing new field in the DCI or increasing the bitwidth of any existing DCI field, however, it has significantly restrictions as summarized below

| Single-DCI Multi-TRP mode | Restriction |
| --- | --- |
| SDM scheme | Only single TB, single RV, one effective PDSCH transmission occasion, maximum 2 layers from each TRP |
| FDMSchemeA | Only single TB, single RV, one effective PDSCH transmission occasion, maximum 2 layers from each TRP |
| FDMSchemeB | Only single TB, two RV, two effective PDSCH transmission occasions, maximum 2 layers from each TRP |
| TDMSchemeA | Only single TB, two RV, two effective PDSCH transmission occasions, maximum 2 layers from each TRP |
| Scheme 4 | Only single TB, up to 16 effective PDSCH transmission occasions, maximum 2 layers from each TRP |

Therefore, we have the following observation

Observation 2: Rel-16 Single-DCI Multi-TRP solutions significantly restrict PDSCH scheduling flexibility, and, is only designed for PDSCH repetition, i.e. single TB case

# Single DCI scheduling PDSCH on multiple cells

Based on the above discussion, we do not observe enough justification and motivation to allow single DCI to schedule PDSCH on multiple cells. Therefore, we have the following proposal

Proposal 1: We do not observe enough justification and motivation to allow single DCI to schedule PDSCH on multiple cells.

# Conclusion

Dynamic spectrum sharing allows NR to be co-channel deployed with LTE, i.e., NR and LTE share the same spectrum, to address the spectrum availability issue from the operator. However, from performance or user experience perspective, it is very hard for DSS deployment to deliver better performance with NR compared to LTE. In the Rel-17 “New WID on NR Dynamic spectrum sharing (DSS)”, one objective was approved to consider PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell.

We have the following observation and proposals regarding the objective of PDCCH of P(S)Cell/SCell scheduling PDSCH on multiple cells using a single DCI.

Observation 1: For UE with advanced PDCCH monitoring capability, e.g., FG3-5b or FG11-2, UE can already be scheduled with multiple unicast DL DCI and UL DCI within a slot in the current NR design, especially considering the span based PDCCH monitoring.

Observation 2: Rel-16 Single-DCI Multi-TRP solutions significantly restrict PDSCH scheduling flexibility, and, is only designed for PDSCH repetition, i.e. single TB case

Proposal 1: We do not observe enough justification and motivation to allow single DCI to schedule PDSCH on multiple cells.

# Reference

- RP-193260, “New WID on NR Dynamic spectrum sharing (DSS)”, 3GPP TSG RAN Meeting #86, Sitges, Spain, December 9-12, 2019

- Chairman Notes, 3GPP TSG RAN WG1 Meeting #102-e, e-Meeting, August 17th – 28th, 2020