3GPP TSG-RAN WG1 #103-e								R1-2008472

e-Meeting, October 26th – November 13th, 2020

Source: 	Apple Inc.

Title:                     Functionality for Coverage Recovery for RedCap

Agenda item:	8.6.3

Document for:	Discussion and Decision

# 1 Introduction

RAN #86 initiated a study item “Support of Reduced Capability NR devices” [1] with the objectives to investigate and evaluate solutions to either reduce UE complexity or lower power consumption in the applicable use cases e.g. industrial wireless sensor, video surveillance and wearables.

Some of cost reduction techniques (e.g. reduced number of Rx antennas and reduced BW) may have impact on the downlink coverage of the device compared to normal NR devices. Thus, it was agreed as part of SID objective to study coverage recovery schemes to compensate the for potential coverage reduction due to the device complexity reduction [1]. In general, the level of coverage compensation is expected to be different for different channels. Extensive discussions on evaluation methodologies for coverage recovery were carried out in RAN1 101 e-meeting, focusing on the possibility to reuse the methodology of IMT-2020 self-evaluation, the scoping of evaluations (i.e. whether to take all relevant DL and UL channel into account), the target data rate and BLER for PDSCH/PUSCH channels as well as how to reuse/align the simulation assumptions made in Rel-17 Coverage Enhancement (CE) study. The following general methodology was agreed at the end of email discussion:

Agreements: If/when coverage evaluations outside the CE SI are needed, - The basic evaluation methodology is based on link-level simulation for FR1. - Step 1: Obtain the required SINR for the physical channels under target scenarios and service/reliability requirements. - Step 2: Obtain the baseline performance based on required SINR and link budget template. - Note: aspects related to identifying target performance and coverage bottlenecks based on target performance metric is to be handled separately. - The evaluation methodology for FR2 is the same as FR1. If/when link-level coverage evaluations outside the CE SI are needed, - The CE SI link-level simulation assumptions can be used as a starting point. - For calibration purposes, the following settings can be used: Parameters; FR1 values; FR2 values. Scenario and frequency; Urban: 2.6 GHz (TDD) (primary choice) 4 GHz (TDD) (secondary choice) Rural: 700 MHz (FDD); Indoor: 28 GHz (TDD). Frame structure for TDD; DDDSUDDSUU (S: 10D:2G:2U) (only for 4GHz) DDDDDDDSUU (S: 6D:4G:4U) (only for 2.6GHz); DDDSU (S: 10D:2G:2U). Channel model; TDL-C; TDL-A. UE velocity; 3 km/h; 3 km/h.

The following was agreed in RAN1 #102 e-meeting:

Agreements For the channel(s) affected by complexity reduction, the following methodology can be used to determine the target performance for coverage recovery - Step 1: Obtain the link budget performance of the channel based on link budget evaluation - Step 2: Obtain the target performance requirement for RedCap UEs within a deployment scenario - FFS on the target performance requirement - Step 3: Find the coverage recovery value for the channel if the link budget performance is worse than the target performance requirement Agreements: - Link budget evaluation for RedCap should include at least PDCCH/PDSCH and PUCCH/PUSCH Agreements: - For initial access related channels, at least Msg2, Msg3, Msg4 and PDCCH scheduling Msg2/4 are included for link budget evaluation - Other initial access related channels are not precluded Agreements: - The impact of small form factor is considered for all the uplink and downlink channels - A 3dB loss of antenna gain is included in link budget calculation for FR1 - FFS on the application to both FDD and TDD bands or only FDD bands Agreements: For RedCap UE, adopt the following target data rates for link budget evaluation for FR1 Rural. - 1 Mbps on DL and 100kbps in UL Agreements: For RedCap UE, down-selection on adopt the following target data rates for link budget evaluation for FR1 Urban. - 2 Mbps on DL and 1Mbps in UL Note: The 2Mbps target data rate in downlink is the scaled value of the 10Mbps in the CE SI by a factor of 0.2 Agreements: For RedCap UEs, the target data rates for link budget evaluation for FR2 are as follows: - 25Mbps for BW 50MHz/100MHz on DL and 5Mbps in UL - Optionally, 12.5Mbps for BW 50MHz as the target data rate for DL, assuming the same DL PSD as that of BW 100MHz - Note: in case of 50MHz BW, the maximum supported DL data rate is half that of the 100MHz BW in DL Agreements: - For link budget evaluation, the antenna gain loss due to the small form factor can be applied to all the FR1 bands - For RedCap coverage analysis, the agreements in the Rel-17 CE SI regarding link budget template and antenna array gain are reused. - Continue to discuss and decide the performance metric in RAN1-103 e-meeting Agreements: - For RedCap coverage evaluation, the Rel-17 CE SI agreements on gNB antenna configuration, # gNB Tx/Rx chains, channel model and delay spread are reused with the following revision and/or addition Parameters; FR1 values; FR2 values. Channel model; TDL-C; TDL-A CDL-A(optional). Delay spread; 300ns; 30ns. UE velocity; 3 km/h; 3 km/h. Antenna correlation; Low; Low. # gNB Tx chains; 2 or 4; 2. # gNB Rx chains; 2 or 4; 2. - For RedCap coverage evaluation, adopt the following table for the reference NR UE. Parameters; FR1 values; FR2 values. # UE Tx chains; 1; 1. # UE Rx chains; Urban: 4 and Rural: 2; 2. UE BW; Urban: 100 MHz (273 PRBs) Rural: 20 MHz (106 PRBs); 100 MHz (66 PRBs). - For RedCap coverage evaluation, adopt the following table for the RedCap UE. - Other UE BWs are not precluded Parameters; FR1 values; FR2 values. # UE Tx chains; 1; 1. # UE Rx chains; 1 or 2; 1 or 2. UE BW; Urban: 20 MHz (51 PRBs) Rural: 20 MHz (106 PRBs); 50 MHz (32 PRBs) or 100 MHz (66 PRBs). Agreements: - For RedCap coverage evaluation, reuse the Rel-17 CE SI agreements on channel specific parameters with the following revision and/or addition - TBS/PRB/MCS of PDSCH (except for Msg2)/PUSCH for the RedCap UE are based on the agreed target data rates or message sizes and reported by companies - Adopt the following table for Msg2 evaluation - Note: the TBS scaling is not precluded in the table entry “PRBs/TBS/MCS” Parameters; Values. PRBs/TBS/MCS; MCS is fixed to zero. Companies to report the used number of PRBs and corresponding TBS value. PDSCH duration; 12 OS. DMRS configuration; Type I, 3 DMRS symbol, no multiplexing with data. Waveform; CP-OFDM. HARQ configuration; No retransmission.

In this contribution, we provided evaluation for some DL and UL channels/signals to investigate the coverage impact of complexity reduction techniques.

# 2. Discussion

## 2.1 Evaluation of coverage performance

In general, the coverage loss due to reduced number of Rx antenna is expected for all DL channels. In addition, 3dB loss due to smaller form factor is expected for all DL/UL channels of Redcap devices as already agreed in RAN1 102 e-meeting. Whether this coverage loss has an effect on area coverage would depend on the network deployment and coverage performance of other channels.

For coverage evaluation, the baseline gNB and UE assumptions mainly reuse the assumption agreed in the coverage extension (CE) study item. Based on analysis conducted in our companion paper [], a number of cost reduction features have no coverage impacts, including half-duplex FDD, relaxing UE processing time and relaxing UE processing capability (e.g. restricting to single MIMO layer or 64QAM modulation for DL). However, reduction of #Rx antennas and bandwidth would cause coverage loss due to lack of spatial diversity gain or increased code rate. For them, we have considered two different scenarios to evaluate the coverage impacts:

- Scenario-1: FR1, Urban, 2.6GHz 30 KHz SCS (TDD)

- Scenario-2: FR1, Rural, 0.7GHz, 15 KHz SCS (FDD)

Regarding the simulation, we first conduct baseline evaluation for the reference NR UE to obtain operational SNR point to fulfil the target performance in accordance with the evaluation assumptions agreed for the two cases. To study the impact of UE complexity reduction techniques, we further perform link-level simulations for the Redcap UEs focusing on the reduced bandwidth and number of Tx/Rx antennas/chains.

### 2.1.1 PDCCH

Table 1: PDCCH Parameters

| Parameters | Values |
| --- | --- |
| DCI payload size | 40 bits + CRC |
| Aggregation level | 16 |
| CORESET | 2 symbols x 48 PRBs |
| Precoding | Precoding cyclic with REG bundle = 6 |
| BLER target | 1% |

The estimated degradation from removing the receiver diversity is summarized in Table 2 for PDCCH for the two scenarios. It should be noted that the bandwidth of CORESET is “48” PRBs, which is identical for the reference NR UEs and Redcap devices. Hence, the coverage loss of PDCCH channel for Redcap is purely caused by the reduced number of Rx antennas from 4 to 2 or 1.

Table 2: PDCCH performance degradation from removing UE receive diversity

| PDCCH @1% BLER | Low correlation |  |  |
| --- | --- | --- | --- |
|  | 4Rx | 2Rx | 1Rx |
| Scenario 1 | -11.7 dB | -8.6 dB | -5.2 dB |
| Scenario 2 | - | -8.7 dB | -5.7 dB |

Observation 1:

- For Rural scenario, reducing Rx antenna number from 2 to 1 for Redcap devices causes ~3 dB coverage loss of PDCCH relative to NR reference UE to fulfil the 1% BLER target.

- For Urban scenario, reducing Rx antenna number from 4 to 2 and 4 to 1 for Redcap devices causes ~3.1 dB and ~6.5dB coverage loss of PDCCH relative to NR reference UE to fulfil the 1% BLER target.

### 2.1.2 PDSCH

Table 3 shows the channel-specific parameters for PDSCH to obtain the target data rates for different deployment scenarios. We use MCS = 0 for both Urban and Rural cases. To fulfil the requirement, we select TBS = 1544 with 41 PRBs. The achievable data rate can be calculated as:

- 2.6 GHz Urban scenario with 30KHz SCS: 1544/0.0005*(1-0.1) * ((14*7+6)/140) = 2.06 Mbps

- 700 MHz Rural scenario with 15KHz SCS: 1128/0.001*(1-0.1) = 1.01 Mbps

Table 3: PDSCH Parameters

| Parameters | Scenario 1 (Urban) |  | Scenario 2 (Rural) |  |  |
| --- | --- | --- | --- | --- | --- |
|  | NR reference | Redcap | NR reference |  | Redcap |
| SCS | 30KHz |  | 15KHz |  |  |
| FDRA | 100 MHz (273 PRBs) | 20 MHz (51 PRBs) | 20 MHz (106 PRBs) |  | Rural: 20 MHz (106 PRBs) |
| TDRA | 12 OFDM symbols |  |  |  |  |
| Waveform | CP-OFDM |  |  |  |  |
| DMRS | Type I, 2 DMRS symbol, no multiplexing with data |  |  |  |  |
| TBS/PRB/MCS | 1544/41/MCS=1 |  |  | 1128/40/MCS=0 |  |
| HARQ configuration | No retransmission |  |  |  |  |
| Rx combing | MRC |  |  |  |  |
| Target data rate | 2Mbps |  |  | 1Mbps |  |

Table 4 summaries the BLER performance for PDSCH for different scenarios to reach the target data rate. We also provided the corresponding link level curves of PDSCH channels in Appendix section for reference.

Table 4: PDSCH performance degradation from removing UE receive diversity

| PDSCH @no HARQ | Target Data Rate (TDR) | Low correlation |  |  |
| --- | --- | --- | --- | --- |
|  |  | 4Rx | 2Rx | 1Rx |
| Scenario 1 | 2Mbps | -7.1 | -3.6 | 1 |
| Scenario 2 | 1Mbps | - | -6.9 | -2.8 |

Observation 2:

- For Rural scenario, reducing Rx antenna number from 2 to 1 causes ~4.1 dB coverage loss of PDSCH relative to NR reference UE to fulfil the 1Mbps target data rate.

- For Urban scenario, reducing Rx antenna number from 4 to 2 and 4 to 1 for Redcap devices causes ~3.5 dB and ~6.1dB coverage loss of PDSCH relative to NR reference UE to fulfil the 1% BLER target.

### 2.1.3 Msg2/Msg4

Table 5 and 6 summarizes the channel specific parameters for Msg2 and Msg4 in the Rural and Urban deployment scenario, which was used in evaluation to meet the target performance requirement for Redcap devices with reflecting the reduced bandwidth restriction. For both Msg2 and Msg4, we use MCS =0.

Table 5: Msg2 Evaluation Parameters

| Parameters | Scenario 1 (Urban) |  | Scenario 2 (Rural) |  |
| --- | --- | --- | --- | --- |
|  | NR reference | Redcap | NR reference | Redcap |
| SCS | 30kHz |  | 15kHz |  |
| TDRA | 12 |  |  |  |
| Waveform | CP-OFDM |  |  |  |
| DMRS | Type I, 3 DMRS symbol, no multiplexing with data |  |  |  |
| TBS/PRB/MCS | 72 bits/3 PRBs/MCS=0 |  |  |  |

Table 6: Msg.4 Evaluation Parameters

| Parameters | Scenario 1 (Urban) |  | Scenario 2 (Rural) |  |
| --- | --- | --- | --- | --- |
|  | NR reference | Redcap | NR reference | Redcap |
| SCS | 30kHz |  | 15kHz |  |
| TDRA | 12 |  |  |  |
| Waveform | CP-OFDM |  |  |  |
| DMRS | Type I, 3 DMRS symbol, no multiplexing with data |  |  |  |
| TBS/PRB/MCS | 1064 bits/37 PRBs/MCS=0 |  |  |  |

A summary of BLER performance for Msg2/Msg4 is provided in Table 7 and Table 8, respectively.

Table 7: Msg2 performance degradation from removing UE receive diversity

| Msg2 @no HARQ | Low correlation |  |  |
| --- | --- | --- | --- |
|  | 4Rx | 2Rx | 1Rx |
| Scenario 1 | -5 | -1.5 | 4 |
| Scenario 2 | - | -2.5 | 2 |

Table 8: Msg4 performance degradation from removing UE receive diversity

| Msg4 @no HARQ | Low correlation |  |  |
| --- | --- | --- | --- |
|  | 4Rx | 2Rx | 1Rx |
| Scenario 1 | -8.2 | -4.6 | -0.3 |
| Scenario 2 | - | -6.7 | -2 |

Observation 3:

- For Rural scenario, reducing Rx antenna number from 2 to 1 causes ~4.5 dB and ~4.7dB coverage loss for Msg2 and Msg4.

- For Urban scenario, reducing Rx antenna number from 4 to 2 for Redcap devices causes ~3.5 dB coverage loss for Msg2/Msg4 transmissions, respectively.

- For Urban scenario, reducing Rx antenna number from 4 to 1 for Redcap devices causes ~9 dB and ~8dB coverage loss for Msg2 and Msg4 transmissions, respectively.

### 2.1.4 PUSCH

Since a single Tx antenna is assumed for NR reference devices and Redcap devices, it is expected to be no performance impact related to reduced number of UE antennas. The coverage performances of different UL channels are evaluated mainly to provide a full picture of DL/UL channels coverage for reference device and Redcap devices.

Table 9: PUSCH Evaluation Parameters

| Parameters | Scenario 1 (Urban) |  |  | Scenario 2 (Rural) |  |
| --- | --- | --- | --- | --- | --- |
|  | NR reference | Redcap |  | NR reference | Redcap |
| SCS | 30kHz |  |  | 15kHz |  |
| TDRA | 14 |  |  |  |  |
| Waveform | DFT-S-OFDM |  |  |  |  |
| DMRS | Type I, 2 DMRS symbol, no multiplexing with data |  |  |  |  |
| TBS/PRB/MCS | 552/30/MCS=4 (308/1024) |  | 128/4/MCS=0 (120/1024) |  |  |
| HARQ | No retransmission |  |  |  |  |
| Target data rate@10% | 1Mbps |  | 100Kbps |  |  |

Table 10 provided the required SNR for PUSCH to achieve different targets for coverage performance evaluation.

Table 10: Summary of required SNR for PUSCH Channel

| Performance Metric | Required SNR |  |
| --- | --- | --- |
|  | W/O FH | W/ FH |
| Scenario 1: 1Mbps | -4.5 dB | -4.6 dB |
| Scenario 2: 100kbps | -7.2 dB | -7.4 dB |

### 2.1.5 Msg3

Table 11: PUSCH Evaluation Parameters

| Parameters | Scenario 1 (Urban) |  |
| --- | --- | --- |
|  | NR reference | Redcap |
| SCS | 30kHz |  |
| TDRA | 14 |  |
| Waveform | DFT-S-OFDM |  |
| DMRS | Type I, 3 DMRS symbol, no multiplexing with data |  |
| TBS/PRB/MCS | 56/2/MCS=1 (157/1024) |  |
| HARQ | No retransmission |  |

Table 12 provided the required SNR for Msg3 to achieve target performance.

Table 12: Summary of required SNR for PUSCH Channel

| Channels | Performance Metric | Required SNR |  |
| --- | --- | --- | --- |
|  |  | w/o FH | W/ FH |
| Msg-3@10% BLER | Scenario 1 | -5.5 dB | -6.3 dB |

### 2.1.6 PUCCH Format 3

Table 13: PUCCH Evaluation Parameters

| Parameters | Values for PUCCH format 3 |
| --- | --- |
| Number of symbols | 14 |
| Payload | 22 bits |
| DMRS | Additional DMRS configured (4 symbols) |
| Performance | 1% BLER |

A summary of required SNR to achieve the performance target is presented in Table 14.

Table 14: Summary of required SNR for PUCCH format 3

| Channels | Performance Metric | Required SNR |  |
| --- | --- | --- | --- |
|  |  | w/o FH | W/ FH |
| PUCCH format 3 @1% BLER | Note: 22-bit Scenario 1: | -3.6 dB | -4.4 dB |

## 2.2 Methodology for coverage analysis

The link budget e.g. MPL for different channel in different deployment scenario e.g. 0.7GHz Rural FDD, 2.6GHz and 4GHz TDD can be further calculated through the link budget tables. The overall amount of compensation should include additional account the up to 3dB lower antenna efficiency for all DL and UL channels as concluded by plenary and included in the revised SID. As expected, compared to NR reference device, the DL coverage loss of Redcap devices is seriously impacted than Uplink channel due to reduced number of Rx antennas.

### 2.2.1 Antenna Array Gain Modelling in Link Budget Template

In RAN1 the 102 e-meeting, how to define the antenna gain was extensively discussed in coverage enhancement session with the following agreements:

Agreements (for both FR1 & FR2): - For the definition of antenna array gain, adopt option 1, i.e. Antenna array gain is included in the link budget template, where there are four antenna gain components - Note: the four components are illustrated below – the figure is for illustration purpose only - FFS which component(s) are NOT part of the definition of antenna array gain

For the coverage performance study of Redcap, the antenna array gain splits were reused in the agreed template for different deployment scenarios. The rows in the below table with “brown” color were left company to report.

For both Rural and Urban deployment scenarios, 4 Tx antennas were assumed at gNB for link level simulation and template coverage analysis.

Figure 1: Simulated of downlink RSRP performance for different number of beams

For coverage purpose, it is of interest to determine the different RSRP differences at the ‘cell-edge’ or for low percentiles in the distribution of users. It is seen that the RSRP difference depends on the number of transmission beams, which can be roughly 10 dB differences between 4 beams (e.g. for broadcast channels) vs. 24 beams (for UE-specific beam formed channels) and 12 dB difference between 4 vs. 48 beams for urban deployment scenario. It plays a key role and one of key factors to determine the bottleneck channels for coverage recovery. In our analysis, a mid-point value, i.e. 8dB difference between beamformed UE-specific channel and broadcast channels, e.g. CSS PDCCH/ Msg2/Msg3/Msg4 were assumed to reflect this fact. Also, it should be noted that practically the gain between different users can be varied in case of narrow beams e.g. 24 beams.

Observation 3:

- In the 2.6Ghz urban deployment scenario, the RSRP differences can be up to 10dB between 4 beams and 24 beams configurations.

### 2.2.2 Link Budget for Redcap Devices

Based on the LLS results for different channels of Redcap devices and the link budget table agreed in post-meeting email discussions, the coverage of reference devices and Redcap devices with 1 or 2 Rx antennas were evaluated and summarized in FIG.2 and FIG.3 for Urban and Rural deployment scenarios, respectively.

As shown in FIG. 2 below, assuming target ISD = 350 and target MPL = 114.4 for TDD Urban scenario on 2.6GHz frequency band, it was observed that coverage enhancements need to be considered at least for Msg2, PUSCH, PUCCH format 3 and Msg3 channels for Redcap devices due to either reduced number of Rx antennas or 3dB antenna efficiency loss caused by smaller form factor. If the ISD is increased to 500m as assumed for LTE system, MPL = 118.07. For this case, the CSS PDCCH coverage needs to be additionally considered to meet the coverage target.

For Rural deployment scenario on 700MHz, as depicted in FIG.2, all DL/UL channels for both Normal UEs and Redcap devices can meet the PL requirement corresponding to ISD = 1732 configuration.

Observation 4:

- For the urban deployment scenario in 2.6GHz frequency, coverage recovery techniques need to be considered for Msg2/Msg3 broadcast channels and PUCCH channels.

- The CSS PDCCH and Msg4 coverages need to be enhanced for Redcap devices in case of ISD = 500.

Figure 2: MPL budget for Scenario-1, Urban TDD 2.6GHz.

Figure 3: MPL budget for Scenario-2, Rural FDD 700MHz frequency.

In RAN1 #102 e-meeting, two options were identified as follows:

Agreements: Down-selection on the following options for the target performance requirement for RedCap UEs in RAN1#103-e (aim for early in the e-meeting): - Option 1: The target performance requirement for each channel is identified by a target MCL or MIL or MPL within a reasonable deployment - Option 3: The target performance requirement for each channel is identified by the link budget of the bottleneck channel(s) for the reference NR UE within the same deployment scenario - Note: The “bottleneck channel(s)” are the physical channel(s) that have the lowest MCL or MIL or MPL - The details for the target performance requirement are FFS

For Opt.3, it was clarified in email discussion that the reference NR UE is NR normal devices without use of Rel-17 coverage enhancement techniques to improve uplink channels coverage. Given UL channel(s) are typically bottleneck from coverage perspective as proved by FIG.1, Opt.3 with targeting to reach the MPL of Rel-16 bottleneck channel of normal NR device would result in the coverage problem for Redcap device in Rel-17 network. To mitigate this issue, the study of coverage recover for RedCap devices should target to reach the similar MPL of normal Rel-17 NR device with enhanced CE techniques introduced in Rel-17 CE study item. In other words, the target coverage performance should be identical as that of Rel-17 CE study item, which can be determined based on a reasonable deployment

Proposal 1:

- Coverage recovery for RedCap devices should strive for reaching a same MPL target as the normal NR devices targeted by the coverage enhancement study item.

## 2.4 Coverage recovery solutions

In this section, we discuss potential coverage recovery solutions that maybe considered. Depending on the coverage evaluation results for different DL/UL channel/signals channels of RedCap devices, the coverage improvement techniques maybe limited to the channels that have reduced coverage compared to the target NR coverage.

Actually, Rel-15 NR already provided extensive supports to address coverage issue for some data channels. For example, NR supports slot-aggregation for PDSCH and PUSCH transmissions since Rel-15, where a same transport block is repeated up to eight slots and provides an effective tool to handle coverage challenges for unicast PDSCH and PUSCH channels in general. However, more studies are still necessary due to the following reasons:

- How to compensate the common DL and UL coverage loss of wearable device due to the 3dB lower antenna efficiency so as to reach comparable MCL target compared to normal NR devices.

- In addition, time-domain repetition or slot aggregation is not supported for broadcast message yet e.g. PRACH, Msg-2/Msg-3 and PUCCH formats prior to dedicated RRC connection completion. How to extend this simple solution to other UL/DL channels should be studied so as to introduce a unified coverage recovery solution for reduced capability UEs and minimize standard efforts.

Furthermore, the repetition number for RedCap devices maybe increased (e.g. be larger than eight) for PDSCH/ PUSCH/PUCCH to accumulate more energy to compensate the additional coverage loss due to reduced number of antennas and bandwidth.

In addition, specific solutions can be explored for different channels. As one example, frequency hopping is another possibility for both DL and uplink channels to compensate for the reduced frequency diversity resulting from the bandwidth reduction for RedCap devices. Combination between repetition and frequency hopping can be taken advantage further to improve performance by allowing multi-slot channel estimation if possible.

Proposal 2:

- Strive for a unified coverage recovery solution for both DL and UL channel (e.g. time-domain repetition).

- Study the need to increase the repetition number for PDSCH/PUSCH/PUCCH channels to improve coverage.

- Investigate the benefit of frequency hopping for both DL and UL channel as well as the combination of frequency hopping and repetition scheme to recover the coverage loss of RedCap devices.

One more consideration for coverage recovery scheme is the configurability. Note that practically only a fraction of the RedCap UEs that in a bad coverage needs to operate the coverage enhancement techniques. It would be beneficial if these techniques are configurable or scalable targeting to these UEs to avoid excessive resource usage. Mechanisms to allow gNB identify these RedCap UEs in earlier stage (e.g. RACH procedure) should be studied to improve the resource efficiency.

Proposal 3:

- Study mechanisms to allow gNB identifying and enabling the coverage enhancement techniques only for the RedCap UEs in a bad coverage.

# 3. Conclusion

In this contribution, we have presented our views on the potential coverage loss due to reduced number of Rx antennas and lower antenna efficiency. Estimates for the resulting downlink control and data channel link performance degradation are also provided in this contribution.

Based on the discussions, we propose the following:

Observation 1:

- For Rural scenario, reducing Rx antenna number from 2 to 1 for Redcap devices causes ~3 dB coverage loss of PDCCH relative to NR reference UE to fulfil the 1% BLER target.

- For Urban scenario, reducing Rx antenna number from 4 to 2 and 4 to 1 for Redcap devices causes ~3.1 dB and ~6.5dB coverage loss of PDCCH relative to NR reference UE to fulfil the 1% BLER target.

Observation 2:

- For Rural scenario, reducing Rx antenna number from 2 to 1 causes ~4.1 dB coverage loss of PDSCH relative to NR reference UE to fulfil the 1Mbps target data rate.

- For Urban scenario, reducing Rx antenna number from 4 to 2 and 4 to 1 for Redcap devices causes ~3.5 dB and ~6.1dB coverage loss of PDSCH relative to NR reference UE to fulfil the 1% BLER target.

Observation 3:

- For Rural scenario, reducing Rx antenna number from 2 to 1 causes ~4.5 dB and ~4.7dB coverage loss for Msg2 and Msg4.

- For Urban scenario, reducing Rx antenna number from 4 to 2 for Redcap devices causes ~3.5 dB coverage loss for Msg2/Msg4 transmissions, respectively.

- For Urban scenario, reducing Rx antenna number from 4 to 1 for Redcap devices causes ~9 dB and ~8dB coverage loss for Msg2 and Msg4 transmissions, respectively.

Observation 4:

- For the urban deployment scenario in 2.6GHz frequency, coverage recovery techniques need to be considered for Msg2/Msg3 broadcast channels and PUCCH channels.

- The CSS PDCCH and Msg4 coverages need to be enhanced for Redcap devices in case of ISD = 500.

Proposal 1:

- Coverage recovery for RedCap devices should strive for reaching a same MPL target as the normal NR devices targeted by the coverage enhancement study item.

Proposal 2:

- Strive for a unified coverage recovery solution for both DL and UL channel (e.g. time-domain repetition).

- Study the need to increase the repetition number for PDSCH/PUSCH/PUCCH channels to improve coverage.

- Investigate the benefit of frequency hopping for both DL and UL channel as well as the combination of frequency hopping and repetition scheme to recover the coverage loss of RedCap devices.

Proposal 3:

- Study mechanisms to allow gNB identifying and enabling the coverage enhancement techniques only for the RedCap UEs in a bad coverage.

# References

- RP-201386		New SID on Support of Reduced Capability NR devices

- TR 38.804

# Appendix

## A1: PDCCH performance

Figure 4: PDCCH performance for FR1, Urban, 2.6GHz 30 KHz SCS (TDD) (Case 1)

Figure 5: PDCCH performance for FR1, Rural, 0.7GHz, 15 KHz SCS (FDD) (Case 2)

## A2: PDSCH performance

Figure 6: PDSCH performance for FR1, Urban, 2.6GHz, 30KHz SCS (TDD)

Figure 7: PDSCH performance for FR1, Rural, 700MHz, 15 KHz SCS (FDD)

## A3: Msg2 Performance

Figure 8: Msg2 performance for FR1, Urban, 2.6GHz, 30 KHz SCS (FDD)

Figure 9: Msg2 performance for FR1, Rural, 0.7GHz, 15 KHz SCS (FDD)

## A4: Msg4 Performance

Figure 10: Msg4 performance for FR1, Urban, 2.6GHz, 30 KHz SCS (FDD)

Figure 11: Msg4 performance for FR1, Rural, 0.7GHz, 15 KHz SCS (FDD)

## A5: PUSCH performance

Figure 12: PUSCH performance for FR1, Rural, 0.7GHz, 15 KHz SCS (FDD), 100kbps Target Date Rate

Figure 13: PUSCH performance for FR1, Rural, 2.6 GHz, 30 KHz SCS (TDD), 1Mbps Target Date Rate

## A6: Msg3 Performance

Figure 14: PUSCH MSG-3 performance

## A7: PUCCH performance

Figure 15: PUCCH Format 3 performance (22-bit payload)

## A8: MIL/MPL/MCL for Urban Scenario-1

## A9: MIL/MPL/MCL for Rural Scenario-2