3GPP TSG- RAN WG1 Meeting #103-e                                                   R1-2008477

# e-Meeting, October 26th – November 13th, 2020

Agenda Item:	8.7.3

Source:	Apple Inc.

Title:	Power saving evaluation for RLM/BFD relaxation

Document for:	Discussion

# Introduction

- Study and specify, if agreed, enhancements on power saving techniques for connected-mode UE, subject to minimized system performance impact [RAN1, RAN4].

b) Study the feasibility and performance impact of relaxing UE measurements for RLM and/or BFD, particularly for low mobility UE with short DRX periodicity cycle, and specify, if agreed, relaxation in the corresponding requirements [RAN4]

    - Note: Supplementary RAN2 work, if needed, can be triggered by RAN4 LS

In Rel-17 UE power saving WI [1], additional enhancement in CONNECTED state is proposed:

In the last RAN1 meeting RAN1#102e, the following agreement on evaluation methodology is reached related to RLM/BFD [2].

Agreements:

- Reusing power model in TR38.840 for evaluation of DCI-based power saving adaptation schemes.

  - Note: company reporting additional power model for missing state or update is not precluded.

Agreements:

- Company should report assumptions used for periodic measurement activities for the Rel-17 DCI-based power saving adaptation evaluation.

  - The periodic activities defined in TR38.840 can be reused.

  - Measurement for RLM/BFD every C-DRX cycle can be optionally modelled

# Performance evaluation of potential RLM relaxation gain

During rel-16 UE power saving, PDCCH-WUS is enabled to allow a UE to skip one DRX cycle when there is no traffic. However, during RRC-connected mode, the UE is required to perform radio link monitor based on DRX cycle and RLM-RS configuration. When DRX cycle is larger than RLM-RS periodicity, the UE need to wake up to perform RLM measurement even though WUS indicating the DRX ON duration can be skipped. This periodic measurement activities consume large amount of UE power, and can be relaxed in certain condition. In this paper, we study the potential power saving gain of RLM relaxation based on [3].

RLM-RS can be SSB, or CSI-RS, or a combination of both. A UE is not required to perform RLM outside of active BWP. A UE should perform L1 measurement per TIndication_interval.  TS38.133 requirement allows 1.5x scaling factor for RLM measurement, however for simplicity purpose, it was agreed in RAN1 102e that for power evaluation purpose, the UE is modelled to perform RLM measurement every DRX cycle.

The power model of SSB or CSI-RS processing is listed in Table 20 of 38.840 [3]. SSB or CSI-RS processing power per slot is 100 for FR1 and 175 for FR2. The following scaling defined in [3] are also used in the evaluation when RLM-RS configuration is aligned with DRX ON duration.

  - "PDCCH+PDSCH" concurrent in the same slot as "SSB or CSI-RS processing", assume "PDCCH+PDSCH" power

  - For "PDSCH only" concurrent in the same slot as "SSB or CSI-RS processing", assume "PDSCH only" power

  - For "PDCCH-only" and "SSB or CSI-RS processing" concurrent in a slot, the slot-averaged power is 0.85x the sum of the respective power,

    - For FR1, 0.85*(100+100) = 170

    - For FR2, 0.85*(175+175) = 300

The DRX configuration and traffic model used for power consumption evaluation is shown in Table I. PDCCH-WUS and cross-slot scheduling is included as Rel-16 baseline feature. Power consumption without RLM measurement assumes no RLM measurement, which is used to show relative power consumption used for RLM measurement.

Table I: Simulation Assumption

| Parameters | Value |
| --- | --- |
| DRX configuration | DRX cycle 40ms, IAT: 20ms, 5ms ON duration. |
| Traffic: | FTP 3 traffic model: 200ms inter-arrival, 0.5Mbytes packet |
| RLM-RS | SSB with 20ms periodicity |

Network configuration of RLM-RS and DRX cycle can impact RLM power consumption. More UE power consumption is expected when RLM-RS has a constant offset with DRX ON duration, as shown in Fig. 1. In this case, a UE always need to wake up for measurement outside of DRX ON duration. The power consumption is shown in Table II for the offset case. Comparing with and without RLM measurement, roughly 25% power consumption is spent on RLM measurement due to constant offset between SSB and DRX on duration. With RLM measurement relaxation factor 4, roughly 20% power saving gain is observed.

Fig. 1 Non-overlapping RLM-RS and DRX On duration

Table I: Average power consumption when RLM-RS is not aligned with DRX on duration

|  | Average Power Consumption | Statistics |
| --- | --- | --- |
| R16 baseline, no RLM | 18.53 | PDCCH only 10.35%, PDCCH + PDSCH 1.42%, micro 0.21%, light 6.20%, deep 81.82% |
| R16 baseline, with RLM | 25.02 | PDCCH only 10.34%, PDCCH + PDSCH 1.42%, micro 10.16%, light 4.37%, deep 72.46%, RLM only 1.24%, |
| R16 baseline, with RLM relax 2 | 21.74 (13.11%) | PDCCH only 10.34%, PDCCH + PDSCH 1.42%, micro 5.14%, light 5.29%, deep 77.18% RLM only 0.61% |
| R16 baseline, with RLM relax 4 | 20.11 (19.62%) | PDCCH only 10.34%, PDCCH + PDSCH 1.42%, micro 2.63%, light 5.78%, deep 79.51%, RLM only 0.30% |

When network configure RLM-RS is aligned with DRX On duration as shown in Fig. 2, the configuration itself save UE power used for RLM measurement based on the scaling method defined in [3]. The power consumption is shown in Table II. There is 10% UE power consumption for R16 baseline with RLM performance by configuring overlapping SSB with DRX on duration. With RLM measurement relaxation factor 4, roughly 16% power saving gain is observed.

Fig. 2 Overlapping RLM-RS and DRX On duration

Table II: Average power consumption when RLM-RS is aligned with DRX on duration

|  | Average Power Consumption | Statistics |
| --- | --- | --- |
| R16 baseline, no RLM | 18.53 | PDCCH only 10.35%, PDCCH + PDSCH 1.42%, micro 0.21%, light 6.20%, deep 81.82% |
| R16 baseline, with RLM | 22.67 | PDCCH only 10.34%, PDCCH + PDSCH 1.21%, micro 5.18%, light 6.20%, deep 75.82%, RLM only 1.03%, PDCCH with RLM 0.01%, PDCCH + PDSCH with RLM 0.21% |
| R16 baseline, with RLM relax 2 | 20.16 (11%) | PDCCH only 10.35%, PDCCH + PDSCH 1.22%, micro 2.23%, light 6.20%, deep 79.38%, RLM only 0.42%, PDCCH + PDSCH with RLM 0.20% |
| R16 baseline, with RLM relax 4 | 19.02 (16%) | PDCCH only 10.35%, PDCCH + PDSCH 1.25%, micro 0.88%, light 6.20%, deep 81.02% RLM only 0.14%, PDCCH + PDSCH with RLM 0.17% |

Observation: Overlapping RLM-RS configuration with DRX on duration by network can use UE power saving

Observation: Roughly 11% to 13% power saving gain is observed with 2x relaxation. Roughly 16% to 20% power saving gain is observed when 4x relaxation is used.

Proposal 1: Feasibility and performance impact of RLM/BFD relaxation should be studied for UE power saving.

# Conclusion

Observation: Overlapping RLM-RS configuration with DRX on duration by network can use UE power saving

Observation: Roughly 11% to 13% power saving gain is observed with 2x relaxation. Roughly 16% to 20% power saving gain is observed when 4x relaxation is used.

Proposal 1: Feasibility and performance impact of RLM/BFD relaxation should be studied for UE power saving.

# Reference

- RP-200938, UE power saving enhancements, MediaTek Inc, July 2020

- Chairman Notes, 3GPP TSG RAN WG1 Meeting #102-e, e-Meeting, August 17th – 28th, 2020

- 3GPP TR 38.840: Study on UE Power Saving (Rel-16).