3GPP TSG-RAN1 WG1 Meeting # 103	R1-2008527

# e-Meeting, October 26th – November 13th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [Draft] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.202 | CR | xxxx | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-16 CR on uplink transmission combination with PUCCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | - |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-10-16 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR is mirror of R1-2008527 for Rel-15. Note that there are discussions of PUCCH grouping capabilties in RAN1 (NR-CA) and cell grouping capabilies in RAN1/RAN2 (NR-DC) for Rel-16, which might be involved with 202 specification. Section 6 and 6.1 describe UL transmission combinations. However, NR-DC case is not considered in these sections. The following are issues in the sections. - It is unclear whether simultaneous transmissions of two PUCCHs are possible for NR-DC or not. - Note 6: Simultaneous PRACH with PUCCH (or PUSCH or SRS) is not supported in NR-DC case. - Note 7: Simultaneous SRS with PUCCH (or PUSCH) is not supported in NR-DC case. - Note 8: Simultaneous PUCCH and PUSCH(s) are not supported in NR-DC case. To clarify A, k definition should be updated. The following two options are considerable for this clarification: Option 1: k is parameter within a cell group. Current table 6.1-2 means that simultaneous transmissions of two PUCCHs are not supported in NR-DC, which would not be aligned with RAN1 common understanding. Update on ‘Supported Combinations’ column is necessary. Option 2: k is parameter across cell groups. It is still nuclear that simultaneous transmissions of two PUCCHs are possible and the PUCCHs are transmitted in different cell groups. The possible case for NR-DC needs to be clarified explicitly. Simultaneous transmissions of two PUCCHs are related to UE capability for the number of cell groups (e.g. ca-ParametersNRDC) but option 2 is difficult to describe it only with k; thereby we take option 1. To clarify B/C/D, NR-DC case should be added in Note 6/7/8. Note that, this specification (TS38.202) describes NR aspect only. It is understood from the initial page (‘NR;’) and the first section (‘5G-NR’) of 38.212. Therefore, EN-DC is not captured explicitly here. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Parameter k is clarified as the number of PUCCH groups within a cell group. - Parameter k and relative are multiplied by parameter j and relative due to the k definition. Additional parameters are defined in Note 6/7/8. - Note 6 covers NR-DC as well as inter-band CA. According to this update, capability is not only that for pararell transmission. - Note 7 covers NR-DC as well as inter-band CA. According to this update, capability is not only that for pararell transmission. - Note 8 covers multiple cell groups case (i.e. NR-DC) as well as multiple PUCCH groups case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Supported uplink transmission combination with PUCCH is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6, 6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6	Simultaneous transmission and reception of physical channels and physical signals

This clause describes the requirements from the UE to send and receive multiple physical channels and physical signals simultaneously depending on the capabilities and service requirements. The following notation is used between both the uplink and downlink clauses below.

-	p is the number of uplink carriers configured for the UE on which physical channels can be transmitted

-	p' is the number of uplink carriers configured for the UE on which SRS can be transmitted

-	q is the number of downlink carriers configured for the UE

-	j is the number of cell groups configured for the UE.

-	k is the number of PUCCH groups within a cell group configured for the UE.

6.1	Uplink

The tables 6.1-1 and 6.1-2 describe the possible combinations of physical channels and SRS that can be sent in simultaneously in the uplink by one UE. Table 6.1-1 introduces notation for a "Transmission Type" which represents a physical channel or sounding reference signal, and any associated transport channel. Table 6.1-2 describes the combinations of these "Transmission Types" which are supported by the UE depending on capabilities [8, TS 38.306], and enumerates how many of each can be transmitted simultaneously.

Table 6.1-1: Uplink "Transmission Types"

| "Transmission Type" | Physical Channel or SRS | Associated Transport Channel | Comment |
| --- | --- | --- | --- |
| A | PRACH | RACH | Note 1, Note 3 |
| B | PUCCH | N/A |  |
| C | PUSCH | UL-SCH | Note 2, Note 3 |
| D | SRS | N/A |  |
| Note 1: RACH corresponds to contention based. Note 2: UCI on PUSCH without UL-SCH is possible. Note 3: For SCell, MsgA PRACH and MsgA PUSCH is not supported. |  |  |  |

Table 6.1-2: Uplink "Transmission Type" combinations

| Supported Combinations | Comment |
| --- | --- |
| j x A | Note 1 |
| j x k x B | Note 1, Note 2 |
| p x C | Note 3, Note 4 |
| p' x D | Note 3, Note 5 |
| j× A + j×k×B | Note 6 |
| j× A + p×C | Note 6 |
| j× A + p'×D | Note 6 |
| j×k× B + p×C | Note 8 |
| j×k× B + p'×D | Note 7 |
| p× C + p'×D | Note 7 |
| Note 1: The number of cell groups j in the supported combination is subject to UE capability. Note 2: The number of PUCCH groups k in the supported combination is subject to UE capability. Note 3: The number of carriers p, and p' in the supported combinations are subject to UE capability. Note 4: In the case there is one SUL carrier, then p-1 would be supported. Note 5: UE may be configured with p' but may also have capability to simultaneously sound less than this number. Note 6: Simultaneous PRACH with PUCCH (or PUSCH or SRS) is supported only in the case of inter-band CA and NR-DC, with j≤j, k≤k, p≤p, and p'≤p' depending on the configuration, and subject to UE capability. Note 7: Simultaneous SRS with PUCCH (or PUSCH) is supported only in the case of inter-band CA and NR-DC, with j≤j, k≤k, p≤p, and p'≤p' depending on the configuration, and subject to UE capability. Note 8: Simultaneous PUCCH and PUSCH(s) is supported only in the case that multiple cell groups or multiple PUCCH groups are configured and the respective PUCCH and PUSCH(s) are transmitted in the different cell groups or in the different PUCCH groups, with j<j, k<k, and p≤p. j, k, and p are subject to UE capability for supported number of cell groups, PUCCH groups, and UL carriers, respectively. j, k, and p depend on configuration. |  |