3GPP TSG RAN WG1 Meeting #103-e	R1-2008723

e-Meeting, October 26th – November 13th, 2020

Agenda item:		7.2.6

Source:		Nokia, Nokia Shanghai Bell

Title:	Maintenance of Rel-16 Multi-TRP operation

Document for:		Discussion and Decision

# 1	Introduction

In this contribution, we discuss issues related to Rel-16 multi-TRP operation.

# 2	Discussion

We identified one remaining issue related to multi-DCI based multi-TRP and discussed as follows.

## PDCCH monitoring priority with QCL-Type D

The following text in 38.213 [1] restricts the monitoring of the PDCCH from two TRPs (with QCL-Type D) when there are overlapping monitoring occasions. And the rule applies across all CORESETs (regardless of coresetPoolIndex).

If a UE

-	is configured for single cell operation or for operation with carrier aggregation in a same frequency band, and

-	monitors PDCCH candidates in overlapping PDCCH monitoring occasions in multiple CORESETs that have same or different QCL-TypeD properties on active DL BWP(s) of one or more cells

the UE monitors PDCCHs only in a CORESET, and in any other CORESET from the multiple CORESETs having same QCL-TypeD properties as the CORESET, on the active DL BWP of a cell from the one or more cells

For multi-DCI based multi-TRP, the UE may need to monitor PDCCH candidates in overlapping PDCCH monitoring occasions in multiple CORESETs that have different QCL-TypeD properties. Otherwise, the usefulness of multi-DCI based multi-TRP in non-ideal FR2 scenarios become somewhat restricted. On the other band, this may be only applicable for the UEs capable of reception of two different QCL-TypeD.

| 16-2c | Simultaneous reception with different Type-D | Supports simultaneous reception with different QCL Type-D RSs. | Per band | FR2 only | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- |

Considering the importance of this issue, we have the following proposal.

Proposal 1: For a UE capable of simultaneous reception with different QCL-TypeD, the PDCCH monitoring priority rule based on QCL-TypeD is applied within CORESETs of the same coresetPoolIndex.

- TP provided in Annex I

# 3	Conclusions

In this contribution, our proposals are as follows,

Proposal 1: For a UE capable of simultaneous reception with different QCL-TypeD, the PDCCH monitoring priority rule based on QCL-TypeD is applied within CORESETs of the same coresetPoolIndex.

- TP provided in Annex I

# References

[1] 		3GPP TS 38.213 V16.3.0

# Annex I

## TP for Proposal 1: PDCCH monitoring priority with QCL-Type D

-----------------------------------------Start of Text Proposal to TS 38.213---------------------------------- ## 10.1 UE procedure for determining physical downlink control channel assignment ### --------------- Unchanged parts omitted ------------- If a UE - is configured for single cell operation or for operation with carrier aggregation in a same frequency band, and - monitors PDCCH candidates in overlapping PDCCH monitoring occasions in multiple CORESETs that have same or different QCL-TypeD properties on active DL BWP(s) of one or more cells, and - not configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet on active DL BWP(s) of one or more cells the UE monitors PDCCHs only in a CORESET, and in any other CORESET from the multiple CORESETs having same QCL-TypeD properties as the CORESET, on the active DL BWP of a cell from the one or more cells - the CORESET corresponds to the CSS set with the lowest index in the cell with the lowest index containing CSS, if any; otherwise, to the USS set with the lowest index in the cell with lowest index - the lowest USS set index is determined over all USS sets with at least one PDCCH candidate in overlapping PDCCH monitoring occasions - for the purpose of determining the CORESET, a SS/PBCH block is considered to have different QCL-TypeD properties than a CSI-RS - for the purpose of determining the CORESET, a first CSI-RS associated with a SS/PBCH block in a first cell and a second CSI-RS in a second cell that is also associated with the SS/PBCH block are assumed to have same QCL-TypeD properties - the allocation of non-overlapping CCEs and of PDCCH candidates for PDCCH monitoring is according to all search space sets associated with the multiple CORESETs on the active DL BWP(s) of the one or more cells - the number of active TCI states is determined from the multiple CORESETs When the UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet, on active DL BWP(s) of one or more cells, the above applied separately for CORESETs within the same coresetPoolIndex. ### --------------- Unchanged parts omitted ------------- --------------------------------------- End of Text Proposal ------------------------------------