3GPP TSG-RAN WG1 Meeting #103-e	R1-2008772

# e-Meeting, October 26–November 13, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR |  | rev |  | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on sub-slot partition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_L1enh-URLLC-Core |  |  |  |  |  | Date: |  |  | 2020-10-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to the agreement in the RAN1 #98 meeting, at least the following two sub-slot configurations for PUCCH are supported: “2-symbol*7” and “7-symbol*2”. However, only the length of sub-slots is defined in the specification by subslotLength-ForPUCCH, it is not clear how to divide a slot into multiple sub-slots, i.e. the location of the sub-slots in a lot, which will have impact on the determination of the location for PUCCH transmisison. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the sentence “a slot is divided into consecutive sub-slots with the number of symbols of each sub-slot as indicated by subslotLengthForPUCCH-r16”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The definition of sub-slot is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 9	UE procedure for reporting control information

< Unchanged parts are omitted >

In the remaining of this Clause, if a UE is provided subslotLength-ForPUCCH-r16, a slot is divided into consecutive sub-slots with the number of symbols of each sub-slot as indicated by subslotLength-ForPUCCH-r16, and a slot for an associated PUCCH transmission is replaced by a sub-slot.

< Unchanged parts are omitted >