3GPP TSG RAN WG1 #103-e                                                                                   R1-2008927

E-meeting, October 26th – November 13th, 2020

Agenda Item:	8.12.2

Source:	Lenovo, Motorola Mobility

Title:	Discussion on reliability improvement for RRC-CONNECTED UEs

Document for:	Discussion & Decision

# Introduction

The contribution is focused on reliability improvement issues for RRC-CONNECTED UEs in Rel-17 NR MBS.

In RAN1#102-e meeting, regarding HARQ-ACK feedback and CSI feedback as well as scheduling mechanism, the corresponding agreements are made as below only from a high level concept point of view:

Agreements:

- For RRC_CONNECTED UEs, HARQ-ACK feedback is supported for multicast and no additional evaluation is needed to justify this.

  - FFS: The detailed HARQ-ACK feedback solutions, e.g., ACK/NACK based, NACK-only based.

  - FFS: HARQ-ACK feedback can be optionally disabled and/or enabled.

Agreements:

- For RRC_CONNECTED UEs, at least support group-common PDCCH with CRC scrambled by a common RNTI to schedule a group-common PDSCH, where the scrambling of the group-common PDSCH is based on the same common RNTI.

o   FFS: whether to support UE-specific PDCCH to schedule a PDSCH for MBS.

Agreements:

- For RRC_CONNECTED UEs, define/configure common frequency resource for group-common PDSCH.

  - FFS: whether to reuse the BWP framework or not

  - FFS: the relation between the common frequency resource and UE dedicated BWP, e.g., the common frequency resource is a MBS specific BWP, or the common frequency resource is confined within UE’s dedicated BWP, etc.

  - FFS: whether more than one common frequency resource can be configured per UE

Agreements:

- For RRC_CONNECTED UEs, at least support FDM between unicast PDSCH and group-common PDSCH in a slot based on UE capability.

  - FFS: TDM or SDM in a slot.

Agreements:

- For RRC_CONNECTED UEs, at least support slot-level repetition for group-common PDSCH.

  - FFS: whether enhancement is needed

Agreements:

- For RRC_CONNECTED UEs, existing CSI feedback can be used for multicast transmission.

  - FFS: whether enhancement is needed

Hence, in this contribution, more detailed views are present from our side.

# Discussion

## HARQ-ACK feedback

HARQ-ACK feedback is widely adopted in wireless communication systems to improve transmission efficiency and reliability. As highlighted in the WID, the HARQ-ACK feedback from UEs corresponding to downlink multicast transmission is essential for the multicast services in order to satisfy the QoS requirement, e.g., reliability. Furthermore, the highlighted objective also stated that the level of reliability should be based on the requirements of the application/service which is provided by MBS. That is to say, the HARQ-ACK feedback for MBS PDSCH can be enabled or disabled according to the reliability requirements.

The set of objectives includes: - Specify RAN basic functions for broadcast/multicast for UEs in RRC_CONNECTED state [RAN1, RAN2, RAN3]: - Specify a group scheduling mechanism to allow UEs to receive Broadcast/Multicast service [RAN1, RAN2] - This objective includes specifying necessary enhancements that are required to enable simultaneous operation with unicast reception. - Specify support for dynamic change of Broadcast/Multicast service delivery between multicast (PTM) and unicast (PTP) with service continuity for a given UE [RAN2, RAN3] - Specify support for basic mobility with service continuity [RAN2, RAN3] - Assuming that the necessary coordination function (like functions hosted by MCE, if any) resides in the gNB-CU, specify required changes on the RAN architecture and interfaces, considering the results of the SA2 SI on Broadcast/Multicast (SP-190625) [RAN3] - Specify required changes to improve reliability of Broadcast/Multicast service, e.g. by UL feedback. The level of reliability should be based on the requirements of the application/service provided. [RAN1, RAN2] - Study the support for dynamic control of the Broadcast/Multicast transmission area within one gNB-DU and specify what is needed to enable it, if anything [RAN2, RAN3]

Generally, for a PDSCH carrying multicast service, there are two options for HARQ-ACK feedback transmission from UE to gNB:

- Option 1: Group NACK transmission

  - If the PDSCH is successfully received, then a UE does not transmit HARQ-ACK feedback to gNB;

  - If the PDSCH is not successfully received, then a UE transmits NACK to gNB.

  - A group of UEs share the same resource to transmit NACK.

- Option 2: UE-specific ACK/NACK transmission

  - If the PDSCH is successfully received, then a UE transmits ACK feedback to gNB;

  - If the PDSCH is not successfully received, then a UE transmits NACK to gNB.

  - Each UE is provided with a specific PUCCH resource for the UE to transmit ACK/NACK to gNB.

It is obvious that Option 1 can minimize the HARQ-ACK feedback overhead because only a sequence indicating NACK is enough as the HARQ-ACK feedback overhead. At gNB side, only a single common PUCCH resource for transmitting a NACK sequence needs to be configured to the group of UEs so as to enable this group-NACK mechanism. Furthermore, when gNB detects the sequence on the PUCCH resource for a given MBS PDSCH, the gNB has to retransmit the MBS PDSCH in subsequent transmission in a group-manner since the gNB can’t differentiate which UE transmits the NACK feedback on the common PUCCH resource shared by the group of UEs. Hence, as long as only one UE does not correctly received the MBS PDSCH, the gNB has to retransmit the MBS PDSCH to the all the UEs in same group. In addition, the standardization effort for group-NACK feedback mechanism is high compared to legacy HARQ-ACK feedback for unicast PDSCH reception.

Observation 1: Group NACK-only feedback can minimize HARQ-ACK feedback overhead while gNB can’t differentiate which UE transmits the NACK.

On the contrast, Option 2 has minor standardization effort and gNB can differentiate each UE’s ACK or NACK at the cost of UE-specific PUCCH resource reservation. However, when a large number of UEs are receiving the multicast transmission, it will consume too much PUCCH resources if a specific PUCCH resource is configured for each UE for HARQ-ACK feedback corresponding to the multicast transmission. In that sense, it does make sense to adopt a group-based NACK transmission as Option 1.

Observation 2: UE-specific ACK/NACK feedback requires minor standardization effort while may consume large PUCCH resources due to per UE HARQ-ACK feedback.

Generally speaking, either Option 1 or Option 2 has its own pros and cons. In order to fully exploit the benefit of each option, it is better to allow the gNB switch between Option 1 and Option 2 in case gNB decides to enable HARQ-ACK feedback for a given MBS transmission. E.g., gNB can flexibly adjust the feedback options based on the number of interested UEs, when the number of UEs is relatively small, Option 2 is better than Option 1; otherwise, Option 1 is better than Option 2; when the UE-specific PUCCH resource overhead is acceptable to gNB, then it is better for the gNB to adopt Option 2; otherwise, it is better to adopt Option 1.

Based on above discussion, we have below proposals:

Proposal 1: Both Option 1 (group-NACK transmission) and Option 2 (UE-specific ACK/NACK transmission) are supported for NR MBS.

On the other hand, as mentioned in the agreement, there is one FFS issue whether to enable or disable the HARQ-ACK feedback for MBS PDSCH. Considering the reason why HARQ-ACK feedback is introduced for MBS is for the purpose of satisfying MBS service reliability requirements, from the standard point of view, it does make sense to introduce a mechanism which allows gNB to enable or disable the HARQ-ACK feedback based on gNB’s scheduling policy and service QoS. In case of high reliability requirements, gNB can enable the HARQ-ACK feedback; otherwise, gNB can disable the HARQ-ACK feedback.

Additionally, in order to fully exploit the benefit of Option 1 or Option 2, it is necessary to further allow the gNB to dynamically switch between Option 1 and Option 2 in case gNB decides to enable HARQ-ACK feedback for a given MBS transmission. As mentioned above, gNB can flexibly adjust the feedback options based on the number of interested UEs in connected mode so as to reach a tradeoff between transmission reliability and resource overhead.

Based on above discussion, we have below proposals:

Proposal 2: HARQ-ACK feedback for MBS can be enabled or disabled.

Proposal 3: Switching between Option 1 (group-NACK transmission) and Option 2 (UE-specific ACK/NACK transmission) are supported for NR MBS.

## MBS PDSCH repetition

Another method for improving transmission reliability is to use transmission repetition. In NR Rel-15, for cell edge UE, 2, 4 or 8 repetitions of one TB can be transmitted for reliability purpose. In detail, the concrete number of transmission repetitions is configured by gNB via RRC signaling in a semi-static manner. Therefore, it is straightforward to introduce transmission repetition for MBS PDSCH based on reliability requirements. The number of transmission repetitions can be configured by RRC signaling as Rel-15.

Proposal 4: The number of transmission repetitions for MBS PDSCH is configured by RRC signaling.

# Conclusion

In this contribution, we focus on the reliability improvement for MBS transmission and have below proposals:

Proposal 1: Both Option 1 (group-NACK transmission) and Option 2 (UE-specific ACK/NACK transmission) are supported for NR MBS.

Proposal 2: HARQ-ACK feedback for MBS can be enabled or disabled.

Proposal 3: Switching between Option 1 (group-NACK transmission) and Option 2 (UE-specific ACK/NACK transmission) are supported for NR MBS.

Proposal 4: The number of transmission repetitions for MBS PDSCH is configured by RRC signaling.

# References

Chairman’s notes on RAN1#102-e meeting

RP-201038, ‘New Work Item on NR support of Multicast and Broadcast Services’, Huawei

R1-2008926, “Discussion on group scheduling for RRC-CONNECTED UEs”, Lenovo, Motorola Mobility

R1-2006233,	“Discussion on group scheduling mechanisms in NR MBS”, CMCC