3GPP TSG RAN WG1 #103-e		                                    R1-2008936

e-Meeting, October 26th – November 13th, 2020

Agenda Item:	8.3.1.2

Source:	InterDigital Inc.

Title:	CSI feedback enhancements for enhanced URLLC/IIoT

Document for:	Discussion and Decision

# Introduction

The work item on Enhanced IIoT and URLLC support for NR [1] has an objective on studying, identifying and specifying CSI feedback enhancements to allow for more accurate MCS selection. In RAN1#102-e, many candidate CSI enhancement schemes were discussed [4] and RAN1 agreed to study and evaluate further a subset of those. RAN1 also agreed on a set of baseline system-level simulation simulations building on TR38.824.

This contribution presents evaluation results for a set of candidate enhancement schemes in two scenarios. Based on observations from these results, the contribution proposes to support at least a new CSI reporting scheme based on measurements taken from PDSCH (Case 2 from RAN1#102-e agreement) and to study triggering mechanisms such as A-CSI on PUCCH for limiting uplink overhead increase of the new reporting scheme.

# Evaluated schemes

System-level simulations are run for the following scenarios:

- Rel-15 enabled use case with reliability 99.999%, packet size 200 bytes and FTP model 3 (100 packets/s)

- Factory automation with reliability 99.9999%, packet size 32 bytes and periodic traffic (500 packets/s)

The baseline CSI reporting and scheduling scheme is as follows:

| CSI report periodicity | 20 ms |
| --- | --- |
| CSI latency between resource and report | 3 ms |
| CSI report quantity | Subband PMI and subband CQI (full CQI) |
| Scheduling/outer-loop link adaptation | Schedule within best subband Select MCS from best CQI + bias, where bias is adjusted as follows - Bias increases by 0.1 dB if ACK is reported - Bias decreases by 0.9 dB if NACK is reported |

The following schemes are evaluated:

- Worst subband CQI – Select MCS based on worst CQI (gNB still schedules within best subbands)

- Soft-ACK – Bias decreases by 0.9 dB if UE reports that codeword SINR was within 2 or 3 dB of SINR threshold for target BLER. Adjustment occurs for every DL assignment.

- Slow soft-ACK – Same as Soft-ACK, except that adjustment occurs for every 10 DL assignments

- Interference threshold – UE reports average interference level measured from CSI-IM over 10 slots. Scheduler applies most conservative MCS if interference-to-noise ratio is above 20 dB

Each scheme is also evaluated for CSI report periodicity of 1 ms to assess the potential gain of obtaining timely channel information before scheduling. Additional simulation assumptions are available in Appendix.

# Evaluation results

Evaluation results for each scheme are shown in Table 1 to Table 3 below for the packet success %, the average PDSCH resource usage and the BLER of initial transmission.

Table 1. Packet success %

| Scenario/Scheme | Baseline | Worst subband CQI | Soft ACK | Slow soft ACK | Interference threshold |
| --- | --- | --- | --- | --- | --- |
| Rel-15 enabled 20 ms periodicity | 99.985 | 99.993 | 99.998 | 100 | 99.995 |
| Rel-15 enabled 1 ms periodicity | 99.989 | 99.996 | 99.999 | 99.999 | 99.994 |
| Factory automation 20 ms periodicity | 100 | 100 | 100 | 100 | 100 |
| Factory automation 1 ms periodicity | 100 | 100 | 100 | 100 | 100 |

Table 2. Average PDSCH resource usage (PRBs)

| Scenario/Scheme | Baseline | Worst subband CQI | Soft ACK | Slow soft ACK | Interference threshold |
| --- | --- | --- | --- | --- | --- |
| Rel-15 enabled 20 ms periodicity | 5.06 | 6.02 | 6.23 | 6.20 | 7.75 |
| Rel-15 enabled 1 ms periodicity | 5.06 | 5.92 | 6.10 | 6.04 | 7.64 |
| Factory automation 20 ms periodicity | 3.70 | 4.87 | 4.64 | 4.64 | 9.73 |
| Factory automation 1 ms periodicity | 3.89 | 5.29 | 5.04 | 5.06 | 9.74 |

Table 3. BLER of initial transmission

| Scenario/Scheme | Baseline | Worst subband CQI | Soft ACK | Slow soft ACK | Interference threshold |
| --- | --- | --- | --- | --- | --- |
| Rel-15 enabled 20 ms periodicity | 14e-4 | 11e-4 | 3.2e-4 | 2.6e-4 | 9.7e-4 |
| Rel-15 enabled 1 ms periodicity | 12e-4 | 8.1e-4 | 2.6e-4 | 2.1e-4 | 6.5e-4 |
| Factory automation 20 ms periodicity | 27e-4 | 56e-4 | 71e-4 | 62e-4 | 0.9e-4 |
| Factory automation 1 ms periodicity | 26e-4 | 70e-4 | 38e-4 | 37e-4 | 8.4e-4 |

One may make the following observations from the above:

- Results do not significantly improve when reducing the periodicity of the CSI reporting. This can be explained by the fact that channel variations for the desired signal are relatively slow for the considered scenarios (most UEs with 3km/h speed), so that reporting in every 20 ms is sufficient to track the channel.

- In the “Factory automation” scenario, all schemes including baseline allow for 100% packet success rate. When looking at the average PDSCH resource usage, the baseline scheme provides the best results. This is likely due to the periodic nature of the traffic which results in consistent interference for a given UE. One can also note that this scenario can tolerate somewhat higher BLER for the initial transmission. In these conditions, enhancements addressing the problem of interference burstiness should not be expected to improve performance.

- In the “R15 enabled use case” scenario, all schemes provide some improvement in terms of packet failure but only the “Soft-ACK” schemes can satisfy the requirement. All schemes result in some increase resource utilization. The largest increase occurs for “interference threshold” and may be due to falling back to overly conservative MCS when the interference threshold is exceeded. The “slow Soft-ACK” scheme does not degrade performance (and is even slightly better) than the “Soft-ACK”. This suggests that it is not necessary to provide the additional feedback for every single PDSCH, and additional uplink overhead can be kept rather small.

Observation 1: Reporting of subband PMI/CQI with a periodicity less than 20 ms does not improve performance in the considered Rel-15 enabled and Factory automation use cases.

Observation 2: Factory automation use case does not require CSI enhancement to satisfy packet error rate requirement.

Observation 3: R15 enabled use case requires CSI enhancements to satisfy packet error rate requirement.

Observation 4: Packet error rate requirements can be satisfied with enhancements to report additional information based on PDSCH SINR (Soft-ACK).

Observation 5: The benefits of reporting Soft-ACK information are maintained even if the information is not reported for every PDSCH assignment.

These observations suggest that RAN1 should focus on at least schemes consisting of new reporting based on measurements taken from PDSCH for CSI enhancements. Such schemes have potential to significantly improve performance when interference is bursty. RAN1 should also investigate triggering mechanisms supporting these schemes (possibly involving A-CSI on PUCCH or other mechanisms) to limit the increase of uplink overhead.

Proposal 1: Support new CSI reporting scheme based on measurements taken from PDSCH (Case 2).

Proposal 2: RAN1 to study triggering mechanisms (e.g. A-CSI on PUCCH or other) for limiting uplink overhead increase of new CSI reporting scheme.

# Conclusion.

This contribution presented evaluation results for a set of candidate enhancement schemes in two scenarios. Observations and proposals are as follows:

Observation 1: Reporting of subband PMI/CQI with a periodicity less than 20 ms does not improve performance in the considered Rel-15 enabled and Factory automation use cases.

Observation 2: Factory automation use case does not require CSI enhancement to satisfy packet error rate requirement.

Observation 3: R15 enabled use case requires CSI enhancements to satisfy packet error rate requirement.

Observation 4: Packet error rate requirements can be satisfied with enhancements to report additional information based on PDSCH SINR (Soft-ACK).

Observation 5: The benefits of reporting Soft-ACK information are maintained even if the information is not reported for every PDSCH assignment.

Proposal 1: Support new CSI reporting scheme based on measurements taken from PDSCH (Case 2).

Proposal 2: RAN1 to study triggering mechanisms (e.g. A-CSI on PUCCH or other) for limiting uplink overhead increase of new CSI reporting scheme.

# References

RP-201310, “Revised WID: Enhanced IIoT and URLLC support for NR”, Nokia, Nokia Shanghai Bell.

3GPP TR38.824, “Study on physical layer enhancements for NR URLLC”, v16.0.0.

3GPP TR 38.802, “Study on New Radio Access Technology; Physical layer aspects”, v14.2.0.

R1-2007286,	“Feature lead summary#3 on CSI feedback enhancements for Rel-17 URLLC/IIoT”,	Moderator (InterDigital)

# Appendix

Agreements from RAN1#102-e

Agreement:

- CSI feedback enhancement for Multi-TRP transmission is not to be discussed further under IIoT/URLLC enhancement WI

Agreements:

- Baseline assumptions are used as the required minimum to be simulated for the evaluation of candidate CSI enhancement schemes

  - Reuse the assumptions in TR 38.824 and TR 38.901 as a starting point

  - Companies shall report additional parameters (e.g., CSI measurement settings, CSI reporting schemes) used in their evaluation

  - FFS details of baseline assumptions

- Companies can bring additional simulation results with other set(s) of assumptions

Agreements:

- Study/evaluate further on following CSI enhancement schemes in terms of technical benefit, specification and implementation impacts.

  - New triggering methods for A-CSI and/or SRS

  - New reporting based on one or more of the following:

    - Case 1: channel/interference measurement for new CSI reporting, considering aspects such as one or more of the following:

      - Reporting more accurate interference characteristics

      - Reduced CSI feedback overhead (e.g., reporting interference measurement only)

      - Enhanced CSI reporting such as WB/SB CQI

    - Case 2: other measurement (other than channel/interference) for additional information

      - E.g., PDCCH/PDSCH decoding, recommended HARQ RV sequence, etc.

    - It targets to help gNB scheduler for better link adaptation of (re)transmission

  - [Reduced CSI computation time/complexity]

  - [CSI feedback for PDCCH]

  - Other CSI enhancement schemes that enable accurate MCS selection are not precluded

- Detailed assumptions of the proposed CSI enhancement schemes should be provided by the proponent, such as

  - Reporting values

  - Triggering conditions for the reporting

  - Associated measurement resource

  - Uplink resource to be used for the reporting

  - How to use the reported information at the gNB scheduler

  - CSI-RS overhead and CSI reporting frequency

  - CSI reporting latency/timeline

  - Etc.

Agreements:

- Consider Table 1 as baseline assumption for system level simulation for evaluating CSI enhancement schemes

  - The uses cases in Table 1 is for simulation purposes and it does not preclude a CSI enhancement scheme which is beneficial for the other URLLC use cases

- No baseline assumption is used for link level simulation

  - Companies are encouraged to use one of LLS assumption tables in Section A.3 in TR38.824 for any link level simulation

Table 1. Baseline SLS assumption for CSI enhancement schemes in URLLC/IIoT

| Parameters | Values |
| --- | --- |
| Performance metric | Option-1 (section 5.1 of TR 38.824) Additional metrics (it is up to company to bring results with additional metric): - MCS prediction error (e.g., difference of a scheduled MCS and an ideal MCS) - DL/UL signaling overhead - CCDF of latency samples from all UEs - BLER of 1st transmission - Resource utilization - Spectral efficiency |
| Use cases | Following two use cases can be considered for new triggering method and new reporting. Companies are encouraged to evaluate the following cases in descending priority: - Rel-15 enabled use case (e.g. AR/VR) in TR 38.824 - Reliability: 99.999 - Latency: 4ms (200bytes) - Traffic mode: FTP model 3 (100p/s) - Factory automation in TR 38.824 - Reliability: 99.9999 - Latency: 1ms (32bytes) - Traffic mode: Periodic deterministic traffic model with arrival interval 2ms - Rel-15 enabled use case (e.g. AR/VR) in TR 38.824 - Reliability: 99.999 - Latency: 1ms (32bytes) - Traffic mode: FTP model 3 (100p/s) - Assumptions for eMBB and URLLC UEs sharing the same carrier is used (as in A2.5 of TR 38.824) |
| Simulation assumptions | Following simulation assumption is used based on the use case selected: - Rel-15 enabled use case with UMa (Table A.2.4-1 in TR 38.824) - Factory automation at 4GHz (Table A.2.2-1 in TR38.824) with following update: - Channel model is replaced with InF (InF-DH) in TR 38.901 - Companies can bring results with other InF scenarios additionally - Layout is replaced with BS deployment in Table 7.8-7 in TR 38.901 |
| Transmission scheme | Multiple antenna ports Tx scheme - Companies report the details of Tx scheme used |

Additional simulation assumptions

| - Parameters | - Value |
| --- | --- |
| - Layout | - R15 AR/VR use case- Single layer (Macro) - Factory automation - Indoor Factory (InF-DH) |
| - Carrier frequency | - 4 GHz |
| - System bandwidth | - 20 MHz |
| - Channel model | - 3D Uma and InF |
| - Bs Tx power | - 49 dBm for Outdoor UMa - 24 dBm for Indoor factory |
| - Subcarrier spacing | - 30 kHz |
| - Antenna configuration | - 4 x 4 - 32 antenna elements at the gNB - 4 antenna elements at the UE |
| - User distribution | - R15 AR/VR use case - 80% indoor, 20% outdoor, 10 UEs per cell for UMa - Factory automation - 15 UEs per cell for Indoor factory |
| - Scheduler | - Time-domain PF SU-MIMO |
| - Traffic model | - R15 AR/VR use case - FTP Model 3 (Poisson arrival with packet arrival of 100p/s) and packet size of 200 bytes. - Factory automation – Periodic deterministic with 2ms interarrival (500p/s) and packet size of 32 bytes |
| - HARQ/repetition | - Adaptive HARQ retransmissions. - Maximum 2 HARQ transmissions including retransmission. |
| - Channel Estimation | - Realistic |
| - UE receiver | - MMSE-IRC |