3GPP TSG RAN WG1#103-e                                	R1-2009057

e-Meeting, October 26th – November 13th, 2020

Source: 	Asia Pacific Telecom

Title: 	Timing relationship enhancements in NTN

Agenda item:	8.4.1

Document for:	Discussion and Decision

# Introduction

In RAN1#102-e, the following agreements were achieved.

Agreement in RAN1#102-e Introduce K_offset to enhance the following timing relationships: - The transmission timing of DCI scheduled PUSCH (including CSI on PUSCH). - The transmission timing of RAR grant scheduled PUSCH. - The transmission timing of HARQ-ACK on PUCCH. - The CSI reference resource timing. - The transmission timing of aperiodic SRS. Note: Additional timing relationships that require K_offset of the same or different values can be further identified. Agreement in RAN1#102-e For Koffset used in initial access, the information of Koffset is carried in system information. - FFS implicit and/or explicit signaling of Koffset in system information. - FFS a cell specific Koffset value used in all beams of a cell and/or each beam in a cell uses a beam-specific Koffset value. - FFS whether/how to update Koffset after initial access. Agreements in RAN2#111-e Both 2-step and 4-step RACH are supported in Rel-17 NTN. FFS enhancements to RACH to accommodate the NTN environment.

As a recommendation from the moderator, the following discussion on RAN1#103-e was encouraged.

- Moderator recommendation on Issue #3: On MAC CE timing relationship, companies are encouraged to conduct more investigations and provide input to RAN1#103-e.

- Moderator recommendation on Issue #4 – 2-step RACH timing relationship: RAN1 to wait for RAN2 decision on whether to support 2-step RACH for NTN or not, before discussing 2-step RACH timing relationship.

In this contribution, we will provide further discussion on

- MAC CE timing relationship

- 2-step RACH timing relationship

- implicit and/or explicit signaling of Koffset in system information

# Discussion

## MAC CE timing enhancement

### Action time on Rel-16 spec

Rel-16 NR includes the following MAC CE commands that support activation and deactivation:

- Timing Advance Command MAC CE

- DRX Command MAC CE

- SCell Activation/Deactivation MAC CE

- PUCCH spatial relation Activation/Deactivation MAC CE

- SP ZP CSI-RS Resource Set Activation/Deactivation MAC CE

- TCI States Activation/Deactivation for UE-specific PDSCH MAC CE

- Aperiodic CSI Trigger State Sub-selection MAC CE

- SP CSI reporting on PUCCH Activation/Deactivation MAC CE

- SP CSI-RS/CSI-IM Resource Set Activation/Deactivation MAC CE

- SP SRS Activation/Deactivation MAC CE

Note that there exist four different wordings (and three different timing) in Rel-16 for the action time

- given a specific slot number with no TA, e.g., slot p=n+k'+1; or

- timing after a given slot, e.g., the first slot that is after slot  q=n+k'; or

- given a specific UL slot number, e.g., UL slot r=m+k+1; or

- given a specific processing time in absolute time, e.g., s=m+4ms (after 4ms),

where n refers to a slot for HARQ-ACK, m refers to a slot for a timing advance command reception, and k and k’ are related to UE processing time.

Figure 1: MAC CE activation timing in Rel-16

Figure 1 shows different MAC CE action time supported in Rel-16, where

- action #1: MAC CE action time for SCell, PUCCH spatial relation, SP CSI reporting, and SP SRS

- action #2: MAC CE action time for SP ZP CSI-RS, TCI States, Aperiodic CSI, SP CSI-RS/CSI-IM

- action #3: MAC CE action time for Timing Advance Command

- action #4: MAC CE action time for DRX Command

Based on our understanding, the UE DL slot p shall not be a correct MAC CE action time, wrong interpretation to us.

Different MAC CE action time can be found in Rel-16 NR based on MAC CE types.

Different MAC CE action time based on MAC CE types shall be supported in NTN

### Action time Interpretation

Rel-16 spec text is not crystal clear about the MAC CE action timing. The ambiguity comes from whether UE shall determine the action time before or after timing advance is applied. To handle this, the terms Logical Time and Actual Time are introduced based on RAN1#98-Bis consensus, where

- Logical Time means that all the following is assumed to be zero

  - DL-to-DL timing differences between CCs

  - UL-to-UL timing differences across different TAGs

  - UL timing advance

- Actual Time means that values observed by the UE are assumed for

  - DL-to-DL timing differences between CCs

  - UL-to-UL timing differences across different TAGs

  - UL timing advance

In the consensus, MAC CE action time was categorized into Logical Time. That means UE shall make a logical MAC CE action in a slot number based on the spec text and then actually apply the MAC CE after UL timing advance.

Figure 2: Consensus made after RAN1#98-Bis for MAC action time

Figure 2 shows an example when UE receives a MAC CE command on DL slot m. UE shall determine a MAC CE action time by Logical Time by assuming no TA applied, then apply the command after TA applied.

- Step 1: UE determines UL slot p as MAC CE action time by Logical Time that assumes TA = 0

  - determine logical MAC CE reception on UL slot m, where the uplink slot m is the last slot overlapping with the slot of PDSCH reception that provides the MAC CE.

  - determine logical HARQ-ACK slot n = m + k1, where k1 is scheduling offset

  - determine logical MAC CE action time as the end of the slot boundary on the slot n + k’, where k’ is UE processing time (shown as the start of the slot boundary on the slot p)

- Step 2: UE implements UL slot p as MAC CE activation time by Actual time after applying TA ≠ 0

  - apply the timing advance value on the UL frame

  - wait and transmit HARQ-ACK on actual UL slot n

  - wait and apply the MAC CE on actual UL slot p

Good support can be found in TS 38.213 with the sentence as: for a timing advance command received on uplink slot n. This sentence is infeasible in practice and R1-1809257 pointed out that this “uplink” shall be a typo, however, it was not. It implies the following sentence: a timing advance command received on logical uplink slot n, i.e., received on actual downlink slot n with no timing advance.

Based on this interpretation, the actual action time on the DL slot q and UL slot p does not introduce additional latency regarding long propagation delay and the corresponding scheduling offset K_offset.

Do not introduce K_offset for MAC CE action time.

However, to reuse this action time when HARQ-ACK is disabled, some scheduling restrictions may be needed, e.g., a DCI format may still need K1 to indicate MAC CE action time, even if the HARQ-ACK is disabled. Another concern is UE may act a MAC CE command early than NW receives the corresponding HARQ-ACK. If this issue is valid, then a simple alternative is to consider MAC CE action time based on the NW UL timeline.

To prevent the MAC CE action time before the HARQ-ACK that NW receives, MAC CE action time based on the NW timeline shall be considered.

### TA Command MAC CE action time

For TA Command MAC CE, the action time is based on a PDSCH reception.

Figure 11 shows an example of TA Command MAC CE action time. Note that the action time is not based on HARQ-ACK. Therefore, it may not support large TA values, e.g., TA > 5 ms for DL/UL SCS = 15 kHz, where the latency of 5 ms is derived based on a table provided in section 5.1.

Figure 3: Activation on TA Command MAC CE when a TA value is greater than 5 ms

The limit is due to the action latency is determined by k=Nslotsubframe,μ⋅NT,1+NT,2+NTA, max+0.5Tsf , where

- NT,1 is a time duration in ms of  symbols corresponding to a PDSCH processing time for UE processing capability 1 when additional PDSCH DM-RS is configured,

- NT,2 is a time duration in ms of N2 symbols corresponding to a PUSCH preparation time for UE processing capability 1,

- NTA,  max is the maximum timing advance value in ms that can be provided by a TA command field of 12 bits,

- Nslotsubframe,μ is the number of slots per subframe,

- 0.5 is the MAC processing latency in ms,

- Tsf is the subframe duration of 1 ms, and

- N1 and N2 are determined to the minimum SCS among the SCSs of all configured UL BWPs for all uplink carriers in the TAG and of all configured DL BWPs for the corresponding downlink carriers.

The term of NTA, max has a value of 2 ms if UL SCS = 15 kHz, which may not cover requirements in NTN, e.g., the max TA value may require 25.77 ms for LEO. Note that UE may also need some processing time to apply the received TA command. Therefore, enhancement on TA command MAC CE shall be FFS.

NTA,max for Timing Advance Command MAC CE with 2ms as the maximum value cannot cover NTN requirements.

The term of NTA,max used for TA command MAC CE shall be further enhanced.

## 2-step RACH timing enhancement

For 2-step RACH, after MsgA transmission, a UE may receive a RAR message including fallbackRAR or successRAR. For fallbackRAR, the UE may continue MSG2, MSG3, and Msg4 of Type-1 RA procedure when the UE detects a RAR UL grant. For successRAR, the UE may transmit a PUCCH with HARQ-ACK information having ACK value, where

- a PUCCH resource for the transmission of the PUCCH is indicated by PUCCH resource indicator field of 4 bits in the successRAR from a PUCCH resource set that is provided by NW; and

- a slot for the PUCCH transmission is indicated by a PDSCH-to-HARQ_feedback timing indicator field of 3 bits in the successRAR having a value 𝑘 from {1, 2, 3, 4, 5, 6, 7, 8} and the slot is determined as 𝑛 + 𝑘 + ∆, where 𝑛 is a slot of the PDSCH reception and ∆ is as defined for PUSCH transmission, e.g., ∆ = 2 for μ=0; and

- the UE does not expect the first symbol of the PUCCH transmission to be after the last symbol of the PDSCH reception by a time smaller than NT,1+0.5 msec where NT,1 is the PDSCH processing time.

Figure 4 shows an example when UE receives a fallbackRAR within the MsgB-RAR window, where 1st offset and 3rd offset might be derived by UE, and 2nd offset and 4th  could be provided by NW. In our example, C-RNTI is provided for PDCCH monitoring, the UE may transmit HARQ-ACK information for the received Msg4, meanwhile, NW shall guarantee proper scheduling regarding required TA and UE processing time.

Figure 4: Example of a fallbackRAR reception within the MsgB-RAR window

However, detail on 1st offset, 3rd offset, and 4th offset is unclear. More discussion shall be needed in RAN1.

Figure 5 shows another example when UE receives a successRAR within the MsgB-RAR window. Note the HARQ-ACK information shall be ACK value only. Timing enhancement on 2nd offset might need some discussion in RAN1.

Figure 5: Example of a successRAR reception within the MsgB-RAR window

Figure 6 shows another example when the MSGA includes the C-RNTI MAC CE. In this case, UE may receive a DCI format in the PDCCH within the MsgB-RAR window. The DCI format in PDCCH shall provide the 2nd offset for HARQ-ACK information, either ACK or NACK values, for the received PDSCH, and NW shall guarantee that the 2nd offset is long enough to cover the 2nd TA value required for the PUCCH transmission. Details shall be discussed further.

Figure 6: Example of C-RNTI MAC CE included in the MSGA transmission

In short, timing enhancement on 2-step RACH might be more complicated than 4-step RACH. RAN1 shall decide whether the discussion shall be started as soon as possible.

Timing enhancement on 2-step RACH shall start in RAN1#103-e.

## Signaling of K_offset in system information

For initial access, K_offset broadcasted in system information may only be used for Msg3 transmission before RRC connection is built, i.e., no UL timing sync. For Msg3 scheduling, there are two proposals can be found in t-docs:

- using the maximum propagation/differential delay of the cell; and

- using the reported delay from the UE based on PRACH resource selection.

For case 1, implicit and explicit signaling approaches are feasible. However, for case 2, only explicit signaling might work. Explicit signaling in system information may provide more flexibility for further enhancement.

Another concern is that K_offset is supposed to cover the autonomous TA, which is still unclear at this stage whether the TA shall cover the service link, the link between gateway/gNB and UE, or the link between UE and a referent point in the air or on the ground. To address this concern, explicit signaling might provide better flexibility.

Signaling on K_offset in initial access shall be explicit or RAN1 shall wait for more progress on Msg3 scheduling in RAN2.

# Conclusion

In this contribution, we have the following observations

Observation 1	Different MAC CE action time can be found in Rel-16 NR based on MAC CE types.

Observation 2	NTA,max for Timing Advance Command MAC CE with 2ms as the maximum value cannot cover NTN requirements.

Based on these observations, we have the following proposals

Proposal 1	Different MAC CE action time based on MAC CE types shall be supported in NTN

Proposal 2	Do not introduce K_offset for MAC CE action time.

Proposal 3	To prevent the MAC CE action time before the HARQ-ACK that NW receives, MAC CE action time based on the NW timeline shall be considered.

Proposal 4	The term of NTA,max used for TA command MAC CE shall be further enhanced.

Proposal 5	Timing enhancement on 2-step RACH shall start in RAN1#103-e.

Proposal 6	Signaling on K_offset in initial access shall be explicit or RAN1 shall wait for more progress on Msg3 scheduling in RAN2.

# Reference

Chairman’s Notes, 3GPP TSG RAN WG1 #102 e-meeting

Chairman’s Notes, 3GPP TSG RAN WG2 #111 e-meeting

3GPP TR 38.821 V16.0.0, Solutions for NR to support non-terrestrial networks, December 2019.

# Appendix

## Latency on transmission timing adjustments

R1-1808965 proposed to simplify the description in section 4.2 of TS 38.213 of time between the reception of a TA command and the application of the TA command, by providing a table for the different sub-carrier spacings. The proposal is given below.

For a TA command received on the uplink slot n, the corresponding adjustment of the UL transmission timing applies from the beginning of uplink slot n+k. Slots n and n+k are to the minimum subcarrier spacing among the subcarrier spacing of all configured UL BWPs for all uplink carriers in TAG. The value k is given in Table 1.

Table 1: "k", a number of UL slots between TA command and application of TA, as a function of UL and DL SCS.

| fUL | fDL | k |
| --- | --- | --- |
| 15 KHz | 15 KHz | 5 |
| 15 KHz | 30 KHz | 5 |
| 15 KHz | 60 KHz | 5 |
| 30 KHz | 15 KHz | 7 |
| 30 KHz | 30 KHz | 5 |
| 30 KHz | 60 KHz | 5 |
| 60 KHz | 15 KHz | 11 |
| 60 KHz | 30 KHz | 8 |
| 60 KHz | 60 KHz | 8 |
| 60 KH | 120 KHz | 8 |
| 120 KHz | 60 KHz | 13 |
| 120 KHz | 120 KHz | 11 |