3GPP TSG RAN WG1 Meeting #103-e                            	        R1- 2009151

E-Meeting, October 26th – November 13th, 2020

Agenda Item:	8.7.1.2

Source:	Spreadtrum Communications

Title:	Consideration on TRS/CSI-RS occasion(s) for idle/inactive UEs

Document for:	Discussion and decision

Introduction

In RAN 86 meeting, it was agreed to specify means to provide potential TRS/CSI-RS occasion(s) available in connected mode to idle/inactive-mode UEs, and minimize system overhead impact [1].

- Specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects [RAN2, RAN1] - Study and specify paging enhancement(s) to reduce unnecessary UE paging receptions, subject to no impact to legacy UEs [RAN2, RAN1] - NOTE: RAN1 to check and update, if needed, evaluation methodology in RAN1 #100 meeting - Specify means to provide potential TRS/CSI-RS occasion(s) available in connected mode to idle/inactive-mode UEs, minimizing system overhead impact [RAN1] - NOTE: Always-on TRS/CSI-RS transmission by gNodeB is not required

In this paper, we discuss our views on this aspect.

Reasons to introduce TRS/CSI-RS in idle/inactive mode

When UE operates in RRC-IDLE or RRC-INACTIVE state, UE might need to perform RRM measurement, cell re-selection if triggered and paging monitoring every paging cycle. In general, UE is expected to perform the following activities in RRC-IDLE or RRC-INACTIVE state:

- Loop convergence, e.g., AGC/TTL/FTL and etc.

- RRM measurement for cell reselection.

- Paging monitoring

In LTE, all above activities could be performed based on CRS in each PO. For NR UE, these activities are based on SSB, which may not be presented close to PO. Therefore, in normal case (as shown in Figure 1), UE may need to wake up two times in every paging cycle. One is to perform AGC/TTL/FTL and RRM for serving cell based on SSB (SMTC), the other one is to monitor paging occasion.

Figure 1: Case-1: UE power consumption in each paging cycle

In worse case, e.g., for the poor channel condition or lager paging cycle (as shown in Figure 2 and Figure 3), UE may need to wake-up to receive 2 SSB burst or  3 SSB burst for AGC, T/F tracking and serving cell measurement before PO monitoring.

Figure 2: Case-2: UE power consumption in each paging cycle

Figure 3: Case-3: UE power consumption in each paging cycle

From above analysis, UE may need to wake up multiple times.

Observation 1: UE may need to wake up multiple times for AGC, T/F tracking, serving cell measurement and PO monitoring in each paging cycle.

TRS/CSI-RS occasion(s) for idle/inactive UEs

Function of TRS/CSI-RS

In RAN1#102e, it agreed that idle/inactive UE may use the TRS/CSI-RS occasion(s) that are shared to it for AGC, time/frequency tracking [2]. The function of RRM measurement for serving cell, RRM measurement for neighbor cell, paging reception indication need further study.

Agreements: Idle/inactive UE may use the TRS/CSI-RS occasion(s) that are shared to it for functionalities such as: - AGC, time/frequency tracking - FFS: RRM measurement for serving cell, RRM measurement for neighbor cell, paging reception indication

As mentioned above, UE may need to wake-up multiple times for AGC, T/F tracking, serving cell measurement and paging monitoring in each paging cycle. If UE can perform AGC, T/F tracking and RRM measurement for serving cell based on TRS/CSI-RS, the number of times UE wakes up will be significantly reduced in each paging cycle. For Case-1, in our evaluation, TRS/CSI-RS transmission with function of RRM measurement for serving cell for idle/inactive mode UE can bring 12%~17% power saving gains. Therefore, UE can perform serving cell measurement based on TRS/CSI-RS in idle/inactive mode should be supported.

For paging reception indication, it is a consensus that WUS like indication is categorized in " a. Study and specify paging enhancement(s)" and not "b. Specify means to provide potential TRS/CSI-RS occasion(s)". Furthermore, if the availability of TRS/CSI-RS for idle/inactive mode is informed to the UE, the TRS/CSI-RS cannot be used as a paging reception indication.

Proposal 1: UE can perform serving cell measurement with TRS/CSI-RS in idle/inactive mode.

TRS/CSI-RS configuration

In RAN1#102e, it agreed that the configuration of TRS/CSI-RS occasion(s) for idle/inactive mode UE(s) is provided by higher layer signaling.

Agreements: The configuration of TRS/CSI-RS occasion(s) for idle/inactive mode UE(s) is provided by higher layer signalling - FFS higher layer signalling candidates (e.g., SIB, dedicated RRC, RRC release message, etc.) - FFS for other signalling candidates (e.g., pre-configuration, etc.) - FFS for detailed configuration parameters (e.g., whether and how to reduce the signalling overhead for configuration, etc.)

TRS/CSI-RS configuration for idle/inactive mode UE should consider both system overhead impact and signaling overhead. There are two TRS/CSI-RS configuration options for idle/inactive mode UE:

- Cell specific TRS/CSI-RS configuration, e.g., via RMSI broadcasts TRS/CSI-RS configuration

- UE-Specific RRC signaling, i.e., gNB configures RS when the UE is in CONNECTED mode via UE specific RRC signaling (e.g., via RRC release message)

Before making decision on method for TRS/CSI-RS configuration, it would be better to discuss necessary parameters which should be conveyed by higher layer signaling. In our view, both cell specific and UE-specific TRS/CSI-RS configuration should be considered. We propose to hold on and discuss on this issue until other design principle is determined.

Proposal 2: Both cell specific and UE-specific TRS/CSI-RS configuration should be considered.

Availability indication of TRS/CSI-RS

In RAN1#102e, it agreed that Further study whether and how to inform the availability of TRS/CSI-RS to idle/inactive mode UE.

Agreements: Further study whether and how to inform the availability of TRS/CSI-RS to idle/inactive mode UE (implicitly or explicitly). - Note: Availability corresponds to the information for whether TRS/CSI-RS is actually transmitted or not.

As we know, the purpose of introducing TRS/CSI-RS is to prevent UE from waking up too early to get the finer frequency and time synchronization for paging reception. UE should know whether the TRS/CSI-RS is available in advance, otherwise, the benefits of introducing TRS/CSI-RS occasion(s) for idle/inactive UEs cannot be obtained. Furthermore, if there is no availability indication of TRS/CSI-RS to let UE know that TRS is available in advance, then UE needs to detect TRS/CSI-RS blindly. Blind detection of TRS/CSI-RS may cause additional power consumption. In our view, gNB needs to inform the availability of TRS/CSI-RS to idle/inactive mode UE in advance. In idle/inactive mode, indication of the availability of TRS/CSI-RS can be included in existing physical layer signal/channel, e.g., paging DCI/paging PDSCH.

Proposal 3: gNB needs to inform the availability of TRS/CSI-RS to idle/inactive mode UE in advance.

Evaluation

In this section, we will simply evaluate the energy-saving gain brought by TRS/CSI-RS transmission in idle/inactive mode.

Evaluation methodology

Baseline assumption in our evaluation is assumed as the following:

- SCS=30KHz

- Paging cycle=1280/640ms

- SSB burst periodicity=20ms

- UE bandwidth=20MHz

In RAN1#102e, it agreed that the following power consumption model for FR1 is utilized for the evaluations of Rel-17 UE power saving enhancements in idle/inactive mode.

Table 1: Power consumption for different power state

| Power State | Relative Power (FR1 reference from TR 8438.840) | Relative Power (Idle/inactive-mode operation with reception bandwidth 20 MHz) |
| --- | --- | --- |
| Deep Sleep (PDS) | 1 | 1 |
| Light Sleep (PLS) | 20 | 20 |
| Micro sleep (PMS) | 45 | 45 |
| PDCCH-only (PPDCCH) | 100 | 50Note |
| PDCCH + PDSCH (PPDCCH+PDSCH) | 300 | 120 |
| PDSCH-only (PPDSCH) | 280 | 112 |
| SSB/CSI-RS proc. (PSSB) | 100 (synchronization or serving cell measurement) | 50 |
| Intra-frequency RRM measurement (Pintra) | · 150 (synchronous case, N=8, measurement only; Pintra, meas-only) · 200 (combined search and measurement; Pintra, search+meas) | · [60] (synchronous case, N=8, measurement only; Pintra, meas-only) · [80] (combined search and measurement; Pintra, search+meas) |
| Inter-frequency RRM measurement (Pinter) | · 150 (measurement only per freq. layer; Pinter, meas-only) · 150 (neighbor cell search power per freq. layer; Pinter, search-only) · Micro sleep power assumed for switch in/out a freq. layer | · [60] (measurement only per freq. layer; Pinter, meas-only) · [150] (neighbor cell search power per freq. layer; Pinter, search-only) · Micro sleep power assumed for switch in/out a freq. layer |
| Note: Power scaling to 20MHz reception bandwidth follows the rule in Section 8.1.3 of TR 38.840, i.e., max{reference power * 0.4, 50}. |  |  |

Paging traffic model in our evaluation is assumed as the following table:

Table 2: Paging traffic model for evaluation

| Paging rate | 0.01 |
| --- | --- |
| UEs per group | 100 |
| Group paging rate | 10% |

To simplify the evaluation, for Case-1, no neighbor cell measurement is assumed; For Case-2 and Case-3, one 5ms SMTC window is assumed for neighbor cell measurement after PO. In our evaluation, the TRS/CSI-RS transmission occasion is assumed before the PO.

Evaluation results

To evaluate the power gains of introducing TRS/CSI-RS transmission in idle/inactive mode, six different test cases are assumed as the following table:

Table 3: Six different test cases

|  | Evaluation assumptions |
| --- | --- |
| Test 1 | Paging cycle=1.28s |
|  | Case-1 |
| Test 2 | Paging cycle=1.28s |
|  | Case-2 |
| Test 3 | Paging cycle=1.28s |
|  | Case-3 |
| Test 4 | Paging cycle=640ms |
|  | Case-1 |
| Test 5 | Paging cycle=640ms |
|  | Case-2 |
| Test 6 | Paging cycle=640ms |
|  | Case-3 |

For each test case, we evaluate the power saving gain with TRS/CSI-RS transmission compared to the scenario without TRS/CSI-RS transmission in idle/inactive mode. The evaluation time is 100 paging cycles for each test case. The power saving gains for different test cases are shown in the following tables:

Table 4: The evaluation results of test 1

|  | Without TRS/CSI-RS transmission | With TRS/CSI-RS transmission |
| --- | --- | --- |
| PDCCH-only energy | 4500 | 4500 |
| PDCCH+PDSCH energy | 1200 | 1200 |
| SSB or TRS/CSI-RS processing | 12000 | 12000 |
| Deep sleep | 126000 | 128000 |
| Light sleep | 18000 | NA |
| Deep sleep to non-sleep | 45000 | 45000 |
| Light sleep to non-sleep | 10000 | NA |
| Neighbor cell measurement | NA | NA |
| Total Power consumption | 216700 | 190700 |
| Power saving gains | 12.0% |  |
| Note1: No neighbor cell measurement is assumed. Note2: UE can perform serving cell measurement with TRS/CSI-RS. |  |  |

Table 5: The evaluation results of test 2

|  | Without TRS/CSI-RS transmission | With TRS/CSI-RS transmission |
| --- | --- | --- |
| PDCCH-only energy | 18000 | 18000 |
| PDCCH+PDSCH energy | 4800 | 4800 |
| SSB or TRS/CSI-RS processing | 22000 | 22000 |
| Deep sleep | 123500 | 125500 |
| Light sleep | 64000 | 28000 |
| Deep sleep to non-sleep | 45000 | 45000 |
| Light sleep to non-sleep | 30000 | 20000 |
| Neighbor cell measurement | 30000 | 30000 |
| Total Power consumption | 337300 | 293300 |
| Power saving gains | 13.0% |  |
| Note: One 5ms SMTC window is assumed for neighbor cell measurement after PO. |  |  |

Table 6: The evaluation results of test 3

|  | Without TRS/CSI-RS transmission | With TRS/CSI-RS transmission |
| --- | --- | --- |
| PDCCH-only energy | 18000 | 18000 |
| PDCCH+PDSCH energy | 4800 | 4800 |
| SSB or TRS/CSI-RS processing | 32000 | 22000 |
| Deep sleep | 121500 | 123500 |
| Light sleep | 200000 | 80000 |
| Deep sleep to non-sleep | 45000 | 45000 |
| Light sleep to non-sleep | 40000 | 20000 |
| Neighbor cell measurement | 30000 | 30000 |
| Total Power consumption | 391300 | 303300 |
| Power saving gains | 22.5% |  |
| Note: One 5ms SMTC window is assumed for neighbor cell measurement after PO. |  |  |

Table 7: The evaluation results of test 4

|  | Without TRS/CSI-RS transmission | With TRS/CSI-RS transmission |
| --- | --- | --- |
| PDCCH-only energy | 4500 | 4500 |
| PDCCH+PDSCH energy | 1200 | 1200 |
| SSB or TRS/CSI-RS processing | 12000 | 12000 |
| Deep sleep | 62000 | 64000 |
| Light sleep | 18000 | NA |
| Deep sleep to non-sleep | 45000 | 45000 |
| Light sleep to non-sleep | 10000 | NA |
| Neighbor cell measurement | NA | NA |
| Total Power consumption | 152700 | 126700 |
| Power saving gains | 17.0% |  |
| Note1: No neighbor cell measurement is assumed. Note2: UE can perform serving cell measurement with TRS/CSI-RS. |  |  |

Table 8: The evaluation results of test 5

|  | Without TRS/CSI-RS transmission | With TRS/CSI-RS transmission |
| --- | --- | --- |
| PDCCH-only energy | 18000 | 18000 |
| PDCCH+PDSCH energy | 4800 | 4800 |
| SSB or TRS/CSI-RS processing | 22000 | 22000 |
| Deep sleep | 59500 | 61500 |
| Light sleep | 64000 | 28000 |
| Deep sleep to non-sleep | 45000 | 45000 |
| Light sleep to non-sleep | 30000 | 20000 |
| Neighbor cell measurement | 30000 | 30000 |
| Total Power consumption | 273300 | 229300 |
| Power saving gains | 16.1% |  |
| Note: One 5ms SMTC window is assumed for neighbor cell measurement after PO. |  |  |

Table 12: The evaluation results of test 6

|  | Without TRS/CSI-RS transmission | With TRS/CSI-RS transmission |
| --- | --- | --- |
| PDCCH-only energy | 18000 | 18000 |
| PDCCH+PDSCH energy | 4800 | 4800 |
| SSB or TRS/CSI-RS processing | 32000 | 22000 |
| Deep sleep | 57500 | 59500 |
| Light sleep | 100000 | 40000 |
| Deep sleep to non-sleep | 45000 | 45000 |
| Light sleep to non-sleep | 40000 | 20000 |
| Neighbor cell measurement | 30000 | 30000 |
| Total Power consumption | 327300 | 239300 |
| Power saving gains | 26.9% |  |
| Note: One 5ms SMTC window is assumed for neighbor cell measurement after PO. |  |  |

As seen from the above evaluation results, TRS/CSI-RS transmission before PO can bring obvious power saving gain, especially when paging cycle is small. The power saving gain mainly comes from the power reduction of light sleep and the corresponding state transition.

Observation 2: TRS/CSI-RS transmission for idle/inactive mode UE can bring obvious power saving gains.

Conclusion

As summary, we have the following proposals.

Proposal 1: UE can perform serving cell measurement with TRS/CSI-RS in idle/inactive mode.

Proposal 2: Both cell specific and UE-specific TRS/CSI-RS configuration should be considered.

Proposal 3: gNB needs to inform the availability of TRS/CSI-RS to idle/inactive mode UE in advance.

Observation 1: UE may need to wake up multiple times for AGC, T/F tracking, serving cell measurement and PO monitoring in each paging cycle.

Observation 2: TRS/CSI-RS transmission for idle/inactive mode UE can bring obvious power saving gains.

Reference

- RP-193239, “New WID: UE Power Saving Enhancements”, MediaTek Inc.

- 3GPP RAN1#102e Chairman’s Notes, August 17 - 28, 2020.