3GPP TSG RAN WG1 Meeting #103-e	R1-2009654

E-meeting, October 26-November 13, 2020

Agenda Item:	8.12.2

Source:	Moderator (Huawei)

Title:	FL summary#3 on improving reliability for MBS for RRC_CONNECTED UEs

Document for:	Discussion and Decision

# Introduction

This summary summarizes the contributions submitted in AI 8.12.2 to discuss how to improve the reliability for MBS for RRC_CONNECTED UEs.

Regarding this aspect, the following agreements were made during the RAN1#102-e meeting.

Agreements:

- For RRC_CONNECTED UEs, HARQ-ACK feedback is supported for multicast and no additional evaluation is needed to justify this.

  - FFS: The detailed HARQ-ACK feedback solutions, e.g., ACK/NACK based, NACK-only based.

  - FFS: HARQ-ACK feedback can be optionally disabled and/or enabled.

Agreements:

- For RRC_CONNECTED UEs, at least support slot-level repetition for group-common PDSCH.

  - FFS: whether enhancement is needed

Agreements:

- For RRC_CONNECTED UEs, existing CSI feedback can be used for multicast transmission.

  - FFS: whether enhancement is needed

This summary includes three high level aspects to address HARQ-ACK feedback, PDSCH repetition, and CSI feedback as agreed in the last meeting. In each of high level issue, a sub-level list of issues are organized.

For each of listed issue, proposal(s) is/are suggested from moderator’s perspective according to the submitted individual company’s proposal(s). Companies are welcome to make comments in the table “collect views”. The proposals may be updated in a second round according to the comments collected in the previous round so as to strive to converge to consensus.

People can use “navigation pane” to quickly overview the organization of the summary and proposal(s) for each issue for discussion and provide views/comments into the table of “collect view” under each proposal.

# HARQ-ACK feedback

## HARQ-ACK feedback option

Background

The last meeting agreed HARQ-ACK feedback is supported and to FFS the detailed HARQ-ACK feedback solution, e.g., ACK/NACK, NACK-only based.

Agreements:

- For RRC_CONNECTED UEs, HARQ-ACK feedback is supported for multicast and no additional evaluation is needed to justify this.

  - FFS: The detailed HARQ-ACK feedback solutions, e.g., ACK/NACK based, NACK-only based.

  - FFS: HARQ-ACK feedback can be optionally disabled and/or enabled.

Submitted Proposals

(Futurewei) Proposal 2:

  - Support NACK-only HARQ-ACK operation.

(Futurewei) Proposal 4:

  - UE-specific, ACK-NACK based feedback is supported at least for small groups

(Huawei) Proposal 1:

  - ACK/NACK feedback mode should be adopted to NR MBS as baseline.

(Huawei) Proposal 2:

  - NACK only feedback mode could apply to use cases for a large number of UEs and even IDLE/ INACTIVE states.

(Chengdu TD-Tech) Proposal 1:

  - HARQ-ACK feedback with the shared PUCCH resource is supported for the PTM bearer of MBS.

(Chengdu TD-Tech) Proposal 2:

  - ACK/NACK feedback with the dedicated PUCCH resource is supported for the PTM bearer of MBS.

(Vivo) Proposal 2:

  - For RRC_CONNECTED UE, both NACK only and ACK/NACK feedback are supported for multicast.

(CMCC) Proposal 1:

  - First decide whether to support multiplexing between HARQ-ACK feedback for PTM and PTP and other UCIs before making the decision to support ACK/NACK based HARQ-ACK feedback for group-common PDCCH based group scheduling.

(CMCC) Proposal 3:

  - ACK/NACK based HARQ-ACK feedback for UE-specific PDCCH based group scheduling can be supported.

(CMCC) Proposal 4:

  - NACK-only based HARQ-ACK feedback for group-common PDCCH based group scheduling can be supported.

(CMCC) Proposal 5:

  - NACK-only based HARQ-ACK feedback for UE-specific PDCCH based group scheduling can be supported, especially for the case with large number of UEs in a MBS group.

(LG) Proposal 1:

  - Support the following solutions based on configuration for support of MBS HARQ feedback:

    - Solution 1: NACK-only HARQ feedback on PUCCH.

    - Solution 2: HARQ feedback enabled/disabled indicator in DCI

    - Solution 3: UL Configured Grant for HARQ feedback

    - Solution 4: RACH based HARQ feedback

(Samsung) Proposal 1:

  - HARQ-ACK feedback for MBS PDSCH receptions is as for unicast PDSCH receptions. FFS for NACK-only HARQ-ACK feedback.

(OPPO) Proposal 4:

  - The following options can be considered for Broadcast/Multicast service

    - Option 1: NACK only feedback, shared NACK resource

    - Option 2: ACK and NACK feedback, separate ACK and NACK resource

    - Option 3: ACK and NACK feedback, shared NACK resource and separate ACK resource.

    - Option 4: ACK and NACK feedback, shared ACK resource and separate NACK resource.

(Potevio) Proposal 2:

  - ACK/NACK based HARQ-ACK feedback mechanism should be more suitable for the case with fix number of UEs in a group.

(Potevio) Proposal 3:

  - NACK only based feedback should be used for cases with a large number of UEs receiving MBS.

(ZTE) Proposal 8:

  - NR MBS should at least support ACK/NACK feedback.

(Nokia) Proposal 4:

  - NACK-only feedback on group-common PUCCH resources along with CSI reporting is used as the HARQ-ACK feedback solution for PTM operation for UEs in RRC_CONNECTED mode.

(Google) Proposal 1:

  - Support ACK/NACK based HARQ feedback as the baseline feature for Rel. 17 MBS. NACK-only based HARQ feedback can also be supported for UL resource limited system.

(Lenovo) Proposal 1:

  - Both Option 1 (group-NACK transmission) and Option 2 (UE-specific ACK/NACK transmission) are supported for NR MBS.

(Lenovo) Proposal 3:

  - Switching between Option 1 (group-NACK transmission) and Option 2 (UE-specific ACK/NACK transmission) are supported for NR MBS.

(MediaTek) Proposal 1:

  - HARQ feedback Opt 1 should be supported in order that gNB can use unicast or multicast based retransmission for each UE.

(MediaTek) Proposal 2:

  - HARQ ACK/NACK feedback mode can be configured when UEs in one MBS group are relatively small.

(MediaTek) Proposal 3:

  - Common NACK only feedback mode should be configured when more UEs exist in one MBS group.

(MediaTek) Proposal 4:

  - Separate NACK only feedback mode should be configured for reducing ACK UE’s power consumption.

(MediaTek) Proposal 5:

  - NR multicast HARQ-ACK feedback can support more than one HARQ feedback mode based on different usage scenarios.

(Intel) Proposal 2:

  - For RRC_CONNECTED UEs, NR MBS supports both ACK/NACK based and NACK-only HARQ feedback. The configuration of ACK/NACK and NACK only mode can be done using the following options

    - Option 1: Semi-static RRC configuration of ACK/NACK or NACK only mode

    - Option 2: The configured PUCCH resource can contain additional indication that the UE is expected to transmit only NACK on the configured resource

    - Option 3: If UE has no dedicated PUCCH resource configuration, the UE uses cell-specific PUCCH resource and is expected to only transmit NACK

(Convida) Proposal 1:

  - Different HARQ feedback schemes should be considered for NR multicast for the UEs in RRC_CONNECTED state. Shared ACK-NACK based scheme can be considered as another alternative of the NACK only based scheme.

(Convida) Proposal 2:

  - Supporting HARQ feedback for NR broadcast service to improve the reliability can be considered for the UEs in RRC_CONNECTED state.

(Qualcomm) Proposal 1:

  - For RRC_CONNECTED UEs, support both group NACK and UE-specific ACK/NACK for HARQ feedback and corresponding retransmission.

    - FFS: PUCCH resource allocation for multicast feedback

    - FFS: Type 1, 2, 3 HARQ-ACK codebook for multicast feedback

(Ericsson) Proposal 1:

  - The HARQ feedback solution for PTM is an ACK/NACK based solution where each UE in a PTM group has its own PUCCH resource to send HARQ feedback

### 1st round discussion

FL’s Comments

From the submitted contributions, there is a clear majority view of supporting both ACK/NACK and NACK-only based HARQ-ACK feedback for RRC_CONNECTED UE. No company proposed to not support the other option between these two when proposing one option as the baseline, but rather some propose to consider or FFS the other option. 1 company proposes ACK/NACK based is for UE-specific PDCCH based group but whether for group-common PDCCH depends on UCI multiplexing of PTP and PTM.

7 companies proposed PUCCH resource related aspects for the proposed HARQ-ACK option(s). For example, PUCCH resource for NACK-only can be group-common or separate resources. For ACK/NACK, each UE has its own PUCCH resource, UEs can share a common resource for ACK and/or a different common resource for NACK. PUCCH resource will be discussed in section 2.2 specifically.

Further, companies analysed or proposed which option is supported in which cases. For example, ACK/NACK based feedback is deemed to be supported for small groups and otherwise NACK-only based is supported.

When both options are supported, 2 companies proposes to switch or configure between ACK/NACK and NACK-only with three options. 1 company proposed also RACH based HARQ feedback and UL configured grant for HARQ feedback.

FL’s Proposal:

#### Proposal 2.1: (HARQ-ACK options)

For RRC_CONNECTED UEs, support both ACK/NACK based and NACK-only based HARQ-ACK feedback for multicast.

- For ACK/NACK based,

  - From per UE perspective, UE feedback ACK or NACK.

  - From UEs within the group perspective, ACK resources are not shared and NACK resources are not shared.

- For NACK-only based,

  - From per UE perspective, UE only feedback NACK.

  - From UEs within the group perspective, NACK resources are shared.

- FFS other HARQ-ACK feedback options

Collect views:

| Company | Comments |
| --- | --- |
| LG | We are OK with the proposal |
| Huawei/HiSilicon | Support. |
| OPPO | Generally, we are fine with the proposal. While for 2nd sub-bullet of ACK/NACK based case, it is related to the PUCCH resource. We suggest to keep it open now. - For ACK/NACK based, - From per UE perspective, UE feedback ACK or NACK. - From UEs within the group perspective, ACK resources are not shared and NACK resources are not shared. - FFS shared or separated resource for ACK. - FFS shared or separated resource for NACK. |
| MTK | We generally fine with this proposal. But for NACK only mode, we also suggest to keep open discussion. We also think separated NACK resource is ok as mentioned in our Tdoc, which can differentiate which UE is NACK UE, so that retransmission can adopt PTP or PTM instead of only support PTM retransmission in common NACK mode. Besides, it also can reduce ACK UE’s power consumption. So, we suggest modify the NACK only based as follows: - For NACK-only based, - From per UE perspective, UE only feedback NACK. - From UEs within the group perspective - FFS shared resource for NACK. - FFS separated resource for NACK. |
| Lenovo, Motorola Mobility | We are OK with the proposal |
| CATT | We are generally OK with the proposal 2.1 where MBS connected UEs support ACK/NACK and NACK-only feedback mechanism, but there are some parts need to be clarified: - The proposal is from per UE perspective but not per MBS service perspective. From per MBS service perspective, do all the UEs receiving the same MBS service use the same or different feedback mechanism? If it can be different, then the case can be like: for a MBS service, the group of UEs receiving this service use different feedback mechanism, some of the UEs use ACK/NACK, while the rest of the UEs use NACK-only feedback. We also need to clarify the use case and scenarios/benefit. For the FFS sub-bullet, is there any other possible feedback mechanism besides ACK/NACK and NACK-only? If yes, could we list some of the potential extra options in the FFS for further contribution/discussion guidance? |
| Nokia, NSB | We generally support the proposal. |
| Convida | We prefer to do down-selection from all the candidate HARQ schemes rather than first agreeing on some of them and FFS for the others. As an alternative to NACK only based HARQ, we propose shared ACK-NACK approach where from per UE perspective, UE feedback ACK or NACK; from UEs within the group perspective, ACK resources are shared and NACK resources are shared. As we discussed in our contribution, as compared to the NACK only based approach, shared ACK-NACK scheme could avoid the misunderstanding issue in case the UEs missed the scheduling DCI. It is obvious that shared ACK-NACK approach has clear advantages than the NACK only based approach, we believe that shared ACK-NACK approach should be considered and supported. |
| Intel | We are OK to support the current proposal with the understanding that the second FFS from the agreement i.e., the configurability of HARQ/ACK feedback will be further discussed once the schemes are clarified. |
| Futurewei | We are generally OK with the proposal. The details of how the resources are configured can be FFS. |
| Qualcomm | We support the proposal. The suggested changes by OPPO and MTK can be included in FFS. Regarding CATT’s first comment, it could be a concern. Maybe we can add: FFS: configuration/selection of ACK/NACK-based and NACK-only based HARQ-ACK feedback for the UEs in a group. |
| ZTE | Considering the workload the pretty limited TU budget, it is preferred to priority ACK/NACK based solution from our perspective because NACK-only based solution may have more spec impact than ACK/NACK based solution especially for multiplexing issues. Besides, the 2nd bullet of the first bullet is little misleading, which can be interpreted that ACK resources can’t be used for UE to report NACK. Based on the above comments, we would like to update the proposal as below. For RRC_CONNECTED UEs, support both ACK/NACK based and NACK-only based HARQ-ACK feedback for multicast. - For ACK/NACK based, - From per UE perspective, UE feedback ACK or NACK. - From UEs within the group perspective, ACK/NACK resources are not shared. - For NACK-only based, - From per UE perspective, UE only feedback NACK. - From UEs within the group perspective, NACK resources are shared. - Prioritize ACK/NACK based solution - FFS other HARQ-ACK feedback options |
| vivo | We are fine with the proposal in principle. For ACK/NACK based mode, we think ACK and NACK can share the same resource. For example, in NR R15/16, ACK and NACK share the same resource in most cases, except for the case with PUCCH format 0 where different CS are used. ZTE’s update for PUCCH resource is fine to us. |
| CMCC | We are fine with the proposal. Regarding we also discuss group scheduling mechanisms, e.g., PTM scheme 1 and PTM scheme 2, and in our Tdoc [6], although we propose both NACK-only based and ACK/NACK based HARQ-ACK feedback, we think the different HARQ-ACK feedback solutions are suitable for different PTM schemes, e.g., NACK-only based is more suitable for PTM scheme 1 (GC-PDCCH based), and ACK/NACK based is more suitable for PTM scheme 2 (UE-specific PDCCH based) Therefore, we think the use case of two HARQ-ACK feedback should also be discussed, we suggest modify this proposal as follows: For RRC_CONNECTED UEs, support both ACK/NACK based and NACK-only based HARQ-ACK feedback for multicast. - For ACK/NACK based, - From per UE perspective, UE feedback ACK or NACK. - From UEs within the group perspective, ACK resources are not shared and NACK resources are not shared. - FFS the group scheduling mechanism for initial transmission is used - For NACK-only based, - From per UE perspective, UE only feedback NACK. - From UEs within the group perspective, NACK resources are shared. - FFS the group scheduling mechanism for initial transmission is used - FFS other HARQ-ACK feedback options |
| Spreadtrum | We are ok with the proposal. |
| Apple | Generally, the proposal is ok for us. Just want to clarify the sub-bullet “From UEs within the group perspective, ACK resources are not shared and NACK resources are not shared. ” , our understanding is only one PUCCH resource is enough for transmission of ACK or NACK. Do we really allocate two resources for ACK and NACK separately? |

### 2nd round discussion

FL’s Comments

CATT’s comment is noted, however, I did not see proposal submitted talk about the case as CATT commented (but correct me if I missed it), which sounds complicated also. Further, if any company is interested in such cases, welcome to discuss it in the paper first.

CMCC’s comment is noted. However, in my opinion I don’t think involving scheduling in this issue discussion sounds good idea. This proposal aims to keep the HARQ-ACK feedback more general or immune to be affected by other issues. For example, for whatever scheduling schemes the HARQ-ACK feedback option suits, this is what the HARQ-ACK feedback option itself is.

The common ground I can find is to support both ACK/NACK based and NACK-only based. However, ACK/NACK could mean different sub-options depending on the resources used for the feedback, so does NACK-only. This is the reason I describe the options from per UE perspective or per UEs within the group perspective in terms of resources for the feedback. Based on the comments received, we can try the update into two manners reflected in two candidates as follows.

Candidate 1 for more progress, based on the common ground, support both options and FFS other options.

Candidate 2 for more open discussion by keeping more points FFS.

Note: Part of proposal 2.2-1/2 regarding the resources from UEs within the group perspective is merged into the proposals in this section.

People can comment which candidate is preferred.

FL’s Proposal: (two candidates)

#### [High priority] Proposal 2.1-r1: (candidate 1)

For RRC_CONNECTED UEs receiving multicast, support both ACK/NACK based and NACK-only based HARQ-ACK feedback for multicast.

- For ACK/NACK based,

  - From per UE perspective, UE feedback ACK or NACK.

  - From UEs within the group perspective, ACK/NACK resource are orthogonal among the UEs.

- For NACK-only based,

  - From per UE perspective, UE only feedback NACK.

  - From UEs within the group perspective, NACK resources are common and shared among the UEs.

- Prioritize ACK/NACK based solution

- FFS other HARQ-ACK feedback options

- FFS: configuration/selection of ACK/NACK-based and NACK-only based HARQ-ACK feedback

#### [High priority] Proposal 2.1-r1: (candidate 2)

For RRC_CONNECTED UEs receiving multicast, support both ACK/NACK based and NACK-only based HARQ-ACK feedback for multicast.

- For ACK/NACK based,

  - From per UE perspective, UE feedback ACK or NACK.

  - From UEs within the group perspective,

    - FFS shared or separated resource for ACK.

    - FFS shared or separated resource for NACK.

- For NACK-only based,

  - From per UE perspective, UE only feedback NACK.

  - From UEs within the group perspective,

    - FFS shared resource for NACK.

    - FFS separated resource for NACK.

- FFS other HARQ-ACK feedback options

- FFS: configuration/selection of ACK/NACK-based and NACK-only based HARQ-ACK feedback

Collect views:

| Company | Comments |
| --- | --- |
| LG | We generally prefer candidate 1. However, we prefer to: - prioritize NACK-only common PUCCH solution for HARQ-ACK to PTM PDSCH transmission; and - prioritize ACK/NACK based UE specific PUCCH solution for HARQ-ACK to PTP PDSCH transmission; and - FFS for other cases. |
| Huawei/HiSilicon | We prefer candidate 1. |
| CMCC | We cannot accept these candidates. Group-common PDCCH based group scheduling mechanism (i.e., PTM scheme 1) was agreed in last meeting, and with the current proposal, it basically means both ACK/NACK based and NACK-only based HARQ-ACK feedback are supported for PTM scheme 1. However, as we commented in 1st round, we are not convinced by the motivation and spec impact of supporting ACK/NACK-based HARQ-ACK feedback for PTM scheme 1. The reasons are further explained as following: - Large spec impact: Besides the spec efforts regarding the PUCCH resource configuration which is needed to provide orthogonal PUCCH resources for UEs in the same group and it is also hard to share the PUCCH resources with unicast, more spec efforts are envisioned regarding the multiplexing/prioritization between HARQ-ACK feedback for PTM and HARQ-ACK feedback for PTP or other UCIs when they overlap in the same slot, which we have not analysed carefully. If we consider there could be different priorities for the multicast and unicast, it will be much more complicated. - Motivation is not clear: More importantly, the motivation of introducing ACK/NACK based feedback for PTM scheme 1 is not clear. If the number of UEs in the same group is large, it’s not realistic to use ACK/NACK based HARQ-ACK feedback because we need to reserve large PUCCH resources for ACK/NACK feedback, NACK-only feedback should be used in this case. Some companies may argue that there could be one or two cell edge UEs which may usually feedback NACK in the group, if ACK/NACK based feedback is supported in this case, network can use PTP for retransmission to the cell edge UEs to improve the reliability. However, in this case, based on CSI feedback or RSRP, network should directly use PTP for these cell edge UEs instead of put them in the group together with other UEs to use PTM transmission, don’t forget the dynamic switch of PTM and PTP is in the scope of WID which is under discussion in RAN2. If the number of UEs in the same group is very small, PTP can be used which do not need additional ACK/NACK feedback for PTM scheme 1. Therefore, we do not see clear motivation for supporting ACK/NACK based HARQ-ACK feedback for PTM scheme1. We can only see the motivation to support NACK-only based HARQ-ACK feedback at this stage. |
| Nokia, NSB | We support Proposal 2-1-r1 Candidate 1 with minor modifications. Reasons why we prefer candidate 1: 1.Our analysis shows very significant PUCCH resource savings (see [13]) to be made using shared NACK resources for the NACK only approach. 2.We have not seen any technical analysis to justify the claimed benefits of separated NACK resources. Suggested modifications: 1. Delete “prioritize ACK/NACK” based solution, since there is only a slim majority of companies that support that option and so far, we have not seen technical analysis (unlike for the NACK only solution) to justify the prioritization. |
| Ericsson | We prefer candidate 1. Our view is that a Nack-only solution should be down-prioritized until a clear need is shown. The Nack only solution has the benefit of a lower resource use from the network perspective, since less UL will be sent. However, this is only true if all UEs are in the same channel conditions in multicast. Since the PDSCH is group-based, a single NACK will trigger a group retransmission. As the number of UE gets larger, the probability of a retransmission grows to 1. Thus For group based PDSCH, UL NACK-only feedback is not very useful. FFS of other HARQ-ACK feedback options is OK |
| Qualcomm | We prefer candidate 1. But we are not sure of ‘prioritize ACK/NACK’ for now, since it is related with PUCCH resource design. |
| Intel | We prefer candidate 1. We do not think ACK/NACK based solution needs to be prioritized since currently we have agreed PTM scheme 1 in the last meeting which naturally lends itself to NACK-only solution. So the bullet should be removed from candidate 1. |
| OPPO | We prefer candidate 2. As we have explained in the 1st round discussion, there are some benefit for separate resource for NACK-only based feedback. NACK-only based feedback mechanism, which is also seen as distanced based feedback, was discussed and introduced in R16 NR SL. In NR MBS, gNB can configure different PUCCH resource for different distance range, the UEs within the same distance range can share the same PUCCH resource. Based on the detection of NACK feedback of per distance range, gNB can determine the UE within which distance range cannot receive correctly, so that it can adjust the transmission parameters, such as TX power of MCS level. At current stage, we prefer to keep the discussion of PUCCH resource open. |
| Futurewei | Support candidate 1. However, we do not support the sub-bullet about prioritizing ACK/NACK based solution. Our preference is to work towards NACK-only based HARQ-ACK solution for the agreement we already have regarding PTM scheme 1 (i.e. group-common PDCCH based group scheduling) |
| MTK | Support Proposal 2.1-r1: (candidate 2). As we commented in 1st round discussion, we still think there are some advantages when separate NACK resource is used for NACK only feedback mode. |
| ZTE | Support candidate 1. As analysis in R1-2008827, most of features for supporting ACK/NACK feedback in NR MBS can refer to the existing feedback mechanism for NR unicast, and some of them can be guaranteed by implementation of gNB. While for supporting NACK only feedback, lots of new issues will be raised, especially for multiplexing rule among NACK-only and other unicast UL transmissions. Furthermore, ACK/NACK feedback provides more options for retransmission, i.e., both PTP-based and PTM-based retransmission, which may improve the spectrum efficiency. So we suggest that the ACK/NACK feedback mechanism should be prioritized for NR MBS from the perspective of standardization complexity. |
| vivo | Considering the discussion on PTM scheme 2 did not go well. We think ACK/NACK based HARQ-ACK is not suitable for group-common PDCCH scheduling. Before we have identified the spec impact and give good solutions for the combination of PTM scheme 1 and ACK/NACK feedback, we can’t agree to support ACK/NACK based feedback. |
| Spreadtrum | We prefer candidate 1. |
| Lenovo, Motorola Mobility | We support Candidate 1 if the bullet of “prioritize ACK/NACK” based solution”. We see both options are useful feature for MBS HARQ-ACK feedback. |

### 1st Checkpoint

FL’s Comments

Per the comments received, CMCC/vivo not acceptable to the proposal is noted and without prioritizing ACK/NACK based and NACK-only based all others seem fine with the proposal with different candidate preference. Regarding CMCC/vivo’s comment, the proposals in this section is tried to not be affected by the discussion of AI 8.12.1. In companies’ mind who are fine with the proposal, ACK/NACK is also applied to PTM scheme 1 (i.e., group-common PDCCH scheduling a group-common PDSCH) and some company also commented large spec impact for NACK-only based.

Still majority prefer candidate 1 and few prefers candidate 2.

For progress, FL still suggests two candidate proposals with FL’s note for the 1st checkpoint with candidate 1 for more progress and candidate 2 for more points FFS.

FL’s Proposal for 1st checkpoint: (two candidates)

#### [High priority] Proposal 2.1-r2: (candidate 1)

For RRC_CONNECTED UEs receiving multicast,

- Support ACK/NACK based HARQ-ACK feedback for multicast,

  - From per UE perspective, UE feedback ACK or NACK.

  - From UEs within the group perspective, ACK/NACK resource are orthogonal among the UEs.

- Support NACK-only based HARQ-ACK feedback for multicast,

  - From per UE perspective, UE only feedback NACK.

  - From UEs within the group perspective, NACK resources are common and shared among the UEs.

- FFS other HARQ-ACK feedback options

- FFS: configuration/selection of ACK/NACK-based and NACK-only based HARQ-ACK feedback

[FL’s Note: CMCC/vivo’s concern is that ACK/NACK is not applied to PTM scheme 1 but seems all others view it is applicable]

#### [High priority] Proposal 2.1-r2: (candidate 2)

For RRC_CONNECTED UEs receiving multicast,

- Support ACK/NACK based HARQ-ACK feedback for multicast,

  - From per UE perspective, UE feedback ACK or NACK.

  - From UEs within the group perspective,

    - FFS shared or separated resource for ACK.

    - FFS shared or separated resource for NACK.

- Support NACK-only based HARQ-ACK feedback for multicast,

  - From per UE perspective, UE only feedback NACK.

  - From UEs within the group perspective,

    - FFS shared resource for NACK.

    - FFS separated resource for NACK.

- FFS other HARQ-ACK feedback options

- FFS: configuration/selection of ACK/NACK-based and NACK-only based HARQ-ACK feedback

[FL’s Note: CMCC/vivo’s concern is that ACK/NACK is not applied to PTM scheme 1 but seems all others view it is applicable]

Collect concerns:

| Company | Comments |
| --- | --- |
| CATT | We also prefer candidate 1 with current version. Regarding our comment during first round discussion: from the perspective of per MBS service, whether the group of UEs can use different feedback mechanism. As FL answered that there is no contribution discussed about this possible mechanism. By considering about designing complexity, we also think the UEs in a group per MBS service should use the same feedback mechanism. So we suggest adding a clarification of this understanding. |
| Apple | We prefer candidate 1. |
| Chengdu TD Tech, TD Tech | We prefer to candidate 2 with the hope to add the RED item. For RRC_CONNECTED UEs receiving multicast, - Support ACK/NACK based HARQ-ACK feedback for multicast, - From per UE perspective, UE feedback ACK or NACK. - From UEs within the group perspective, - FFS shared or separated resource for ACK. - FFS shared or separated resource for NACK. - Support NACK-only based HARQ-ACK feedback for multicast, - From per UE perspective, UE only feedback NACK. - From UEs within the group perspective, - FFS shared resource for NACK. - FFS separated resource for NACK. - How to feedback NACK with the PUCCH resource is FFS - FFS other HARQ-ACK feedback options - FFS: configuration/selection of ACK/NACK-based and NACK-only based HARQ-ACK feedback |
| MTK | We prefer option 2. We also think separated NACK resource is ok as mentioned in our Tdoc, which can differentiate which UE is NACK UE, so that retransmission can adopt PTP or PTM instead of only support PTM retransmission in common NACK mode. Besides, the sub-group scheduling mechanism is FFS. If it adopts the sub-group scheduling for transmission, separate ACK or separate NACK for each group can be further study. From our understanding, the candidate 1 is the subset of candidate 2. Thus, we can keep option 2 at current stage and other issues can be further discussed in next stage or next meeting. |
| CMCC | As the comment in 2nd round, we cannot accept these candidates. Group-common PDCCH based group scheduling mechanism (i.e., PTM scheme 1) was agreed in last meeting, and with the current proposal, it basically means both ACK/NACK based and NACK-only based HARQ-ACK feedback are supported for PTM scheme 1. However, as we commented in 1st round, we are not convinced by the motivation and spec impact of supporting ACK/NACK-based HARQ-ACK feedback for PTM scheme 1. The reasons are further explained as following: - Large spec impact: Besides the spec efforts regarding the PUCCH resource configuration which is needed to provide orthogonal PUCCH resources for UEs in the same group and it is also hard to share the PUCCH resources with unicast, more spec efforts are envisioned regarding the multiplexing/prioritization between HARQ-ACK feedback for PTM and HARQ-ACK feedback for PTP or other UCIs when they overlap in the same slot, which we have not analysed carefully. If we consider there could be different priorities for the multicast and unicast, it will be much more complicated. - Motivation is not clear: More importantly, the motivation of introducing ACK/NACK based feedback for PTM scheme 1 is not clear. If the number of UEs in the same group is large, it’s not realistic to use ACK/NACK based HARQ-ACK feedback because we need to reserve large PUCCH resources for ACK/NACK feedback, NACK-only feedback should be used in this case. Some companies may argue that there could be one or two cell edge UEs which may usually feedback NACK in the group, if ACK/NACK based feedback is supported in this case, network can use PTP for retransmission to the cell edge UEs to improve the reliability. However, in this case, based on CSI feedback or RSRP, network should directly use PTP for these cell edge UEs instead of put them in the group together with other UEs to use PTM transmission, don’t forget the dynamic switch of PTM and PTP is in the scope of WID which is under discussion in RAN2. If the number of UEs in the same group is very small, PTP can be used which do not need additional ACK/NACK feedback for PTM scheme 1. Therefore, we do not see clear motivation for supporting ACK/NACK based HARQ-ACK feedback for PTM scheme1. We can only see the motivation to support NACK-only based HARQ-ACK feedback at this stage, we suggest remove the ACK/NACK based HARQ-ACK feedback in the proposal, and we still maintain the FFS other HARQ-ACK feedback options. #### [High priority] Proposal 2.1-r2: (candidate 1) For RRC_CONNECTED UEs receiving multicast, - Support ACK/NACK based HARQ-ACK feedback for multicast, - From per UE perspective, UE feedback ACK or NACK. - From UEs within the group perspective, ACK/NACK resource are orthogonal among the UEs. - Support NACK-only based HARQ-ACK feedback for multicast, - From per UE perspective, UE only feedback NACK. - From UEs within the group perspective, NACK resources are common and shared among the UEs. - FFS other HARQ-ACK feedback options - FFS: configuration/selection of ACK/NACK-based and NACK-only based HARQ-ACK feedback #### [High priority] Proposal 2.1-r2: (candidate 2) For RRC_CONNECTED UEs receiving multicast, - Support ACK/NACK based HARQ-ACK feedback for multicast, - From per UE perspective, UE feedback ACK or NACK. - From UEs within the group perspective, - FFS shared or separated resource for ACK. - FFS shared or separated resource for NACK. - Support NACK-only based HARQ-ACK feedback for multicast, - From per UE perspective, UE only feedback NACK. - From UEs within the group perspective, - FFS shared resource for NACK. - FFS separated resource for NACK. - FFS other HARQ-ACK feedback options FFS: configuration/selection of ACK/NACK-based and NACK-only based HARQ-ACK feedback |
| Lenovo, Motorola Mobility | We support Candidate 1. Regarding the bullet of FFS other HARQ-ACK feedback options, can we delete that for more progress? Since NACK-only and ACK/NACK cover the possible schemes, maybe we don’t need this FFS. |
| LG | We prefer candidate 1 only. |
| Convida | We are not OK with either candidate. We think only ACK/NACK based HARQ-ACK feedback needs to be supported. As we discussed in our 1st round feedback, ACK/NACK based scheme with shared resource for ACK and shared resource for NACK has clear advantages over the NACK only based scheme. Therefore, NACK only based scheme doesn’t need to be supported. Other alternative or enhancement could be considered for low overhead approach including ACK/NACK based scheme with shared resource for ACK and NACK, etc |
| vivo | We are not OK with either candidate. Reasons for not supporting ACK/NACK based HARQ-ACK: - No motivation. Until now, we only have agreed to support PTM scheme 1 for initial transmission and people can only agree to support PTM scheme 1 for retransmission according the discussion in 8.12.1. Many people have concern on PTP retransmission or sub-group common retransmission. In this case, since both initial and retransmission are group -common based, then gNB doesn't need to know which UE sends NACK and NACK only based feedback is enough. there is no need for ACK/NACK based HARQ-ACK at all. - Large spec impact but with no clear solution. One issue that needs attention is that since there is only one single PUCCH resource indicator (PRI) in DCI in PDCCH, but the PUCCH resource for different UE need to be different, we need to make sure that the single PUCCH resource indicator (PRI) can point to different PUCCH resource for different UE in PTM group so that there is no PUCCH resource collision. And similar issue for other parameters such as K1 and TPC. This needs a new mechanism. But until now, I haven't see any proposal to give detailed methods to solve PUCCH resource allocation, k1 or TPC indication issues. Anyway, we need to study how to indicate different PUCCH resource with one group-common PRI if companies want to support ACK/NACK based feedback for group-common PDCCH. So my suggestion is we can study how to indicate different PUCCH resources with one group-common PRI first. Or decide whether support PTP for retransmission or whether support PTM scheme 2 for initial transmission first. And then we can go back to discuss this proposal. |
| Samsung | Do not support an agreement at this stage for NACK-only HARQ-ACK feedback. There are concerns that such mechanism is unnecessary and problematic and possible applicable scenarios have not been discussed. |
| Ericsson | We prefer candidate 1, and reiterate our concern regarding NACK-only. |
| Qualcomm | We support candidate 1. |
| Nokia, NSB | Candidate 1 |
| Spreadtrum | We support candidate 1. |

### 3rd round discussion

FL’s Comments

FL suggests focusing on candidate 2 for the 3 round discussion and FFS whether the reource is shared or  separate. Since for each option there are some companies having sustained concern to support, FL suggests the following proposal for GTW for further discussion.

For companies who have not expressed the preference, please add the company name under each for support or object before GTW.

FL’s Proposal:

#### [High priority] Proposal 2.1-r3: (candidate 2)

For RRC_CONNECTED UEs receiving multicast, at least for PTM scheme 1

- Support ACK/NACK based HARQ-ACK feedback for multicast,

  - From per UE perspective, UE feedback ACK or NACK.

  - From UEs within the group perspective,

    - FFS shared or separated resource for ACK.

    - FFS shared or separated resource for NACK.

  - FFS details, e.g., PRI, k1, TPC, etc.

[support: CATT, Apple, MTK, Lenovo, Motorola Mobility, LG, Convida, Samsung, Ericsson, Qualcomm, Nokia, NSB, Huawei, HiSilicon, Spreadtrum, ZTE, Futurewei]

[Object: CMCC, Vivo]

- Support NACK-only based HARQ-ACK feedback for multicast,

  - From per UE perspective, UE only feedback NACK.

  - From UEs within the group perspective,

    - FFS shared resource for NACK.

    - FFS separated resource for NACK.

[support: CATT, Apple, MTK, Lenovo, Motorola Mobility, LG, Ericsson, Qualcomm, Nokia, NSB, Huawei, HiSilicon, Vivo, CMCC, Spreadtrum, ZTE, Futurewei]

[Object: Samsung, Convida]

- FFS: configuration/selection of ACK/NACK-based and NACK-only based HARQ-ACK feedback

Collect concerns:

| Company | Comments |
| --- | --- |
| Samsung | We have concerns on “Support NACK-only based HARQ-ACK feedback” - Unknown performance – we understand that performance evaluation for HARQ-ACK is not necessary, but that is for Rel-16 based mechanisms - Certainly not as reliable as conventional HARQ-ACK - Errors due to random phase combinations of receptions from multiple UEs need to be simulated - Likely to require changes in gNB receiver implementation - Lead to errors due to PDCCH DTX (even severe if the number of UE in a group is larger) - Does not work in case of multiplexing with other UCI - PUCCH overhead for conventional HARQ-ACK is not a problem |
| CMCC | As the comment before, we only see the motivation to support NACK-only based HARQ-ACK feedback for PTM scheme 1 at this stage. We support the following proposal proposed by moderator in the email,which adding FFS on ACK/NACK based HARQ-ACK feedback. For RRC_CONNECTED UEs receiving multicast, at least for PTM scheme 1 - FFS: Support ACK/NACK based HARQ-ACK feedback for multicast, - From per UE perspective, UE feedback ACK or NACK. - From UEs within the group perspective, - FFS shared or separated resource for ACK. - FFS shared or separated resource for NACK. - Support NACK-only based HARQ-ACK feedback for multicast, - From per UE perspective, UE only feedback NACK. - From UEs within the group perspective, - FFS shared resource for NACK. - FFS separated resource for NACK. - FFS other HARQ-ACK feedback options FFS: configuration/selection of ACK/NACK-based and NACK-only based HARQ-ACK feedback |
| LG | We also support the following proposal proposed by moderator in the email,which adding FFS on ACK/NACK based HARQ-ACK feedback. For RRC_CONNECTED UEs receiving multicast, at least for PTM scheme 1 - FFS: Support ACK/NACK based HARQ-ACK feedback for multicast, - From per UE perspective, UE feedback ACK or NACK. - From Ues within the group perspective, - FFS shared or separated resource for ACK. - FFS shared or separated resource for NACK. - Support NACK-only based HARQ-ACK feedback for multicast, - From per UE perspective, UE only feedback NACK. - From Ues within the group perspective, - FFS shared resource for NACK. - FFS separated resource for NACK. - FFS other HARQ-ACK feedback options FFS: configuration/selection of ACK/NACK-based and NACK-only based HARQ-ACK feedback |
| Lenovo, Motorola Mobility | We support this proposal. The FFS in the first bullet seemingly covers too detailed issues. Not sure whether “K1” means the RRC configured K1 set or the HARQ-ACK feedback timing indicator in the scheduling DCI. - FFS details, e.g., PRI, k1, TPC, etc. |
| MTK | As the discussed before, we still think HARQ/ACK and NACK only feedback mode can be supported because it doesn't have a measurement criteria to differentiate “many UEs” and “small UEs” for one MBS group. gNB can flexibly choose appropriate feedback mode beased on different services and reliability requirements. We also think the FFS in first bullet (FFS details, e.g., PRI, k1, TPC, etc.) is too detailed at current stage, we should focus on whether to support these two feedback options. We prefer the proposal summarized by moderator in the email: For RRC_CONNECTED UEs receiving multicast, at least for PTM scheme 1 - FFS: Support ACK/NACK based HARQ-ACK feedback for multicast, - From per UE perspective, UE feedback ACK or NACK. - From Ues within the group perspective, - FFS shared or separated resource for ACK. - FFS shared or separated resource for NACK. - Support NACK-only based HARQ-ACK feedback for multicast, - From per UE perspective, UE only feedback NACK. - From Ues within the group perspective, - FFS shared resource for NACK. - FFS separated resource for NACK. - FFS other HARQ-ACK feedback options FFS: configuration/selection of ACK/NACK-based and NACK-only based HARQ-ACK feedback |
| Spreadtrum | We are ok with the proposal |
| CATT | We support this proposal with some modifications. - I think majority companies’ views are supporting candidate 1 instead of 2, since how the HARQ-ACK resources can be allocated is next step after determination of the feedback mechanism (ACK/NACK-based and/or NACK-only based). By considering the progress, we can leave them under FFS but with general description as follows. - We support both ACK/NACK and NACK-only based feedback schemes - NACK-only based feedback scheme has a natural drawback that is DTX, so ACK/NACK-based feedback can be another alternative when the reliability requirement is high. - How to quantificat the UE number is large or small, e.g. 100 is large, and 5 is small. Then how about 45? What kind of transmission scheme should be used, PTM or PTP? The system cannot guarantee that the number of UEs is either large or small. - When both feedback schemes are supported, either one of them can be configured/selected according to different scenarios, such as UE number, communication range, or reliability requirement. - Clarification: All the UEs of receiving the same multicast service should use the same feedback scheme. We need to explicitly clarify that this clarification is a common understanding by avoiding too divergent views in the future discussion/design. - Clarification: All the UEs of receiving the same multicast service should use the same PUCCH resource multiplexing mechanism. e.g. All UEs use separate PUCCH resource mechod with unicast for a MBS service. - For the “FFS other HARQ-ACK feedback options”: I think the original intention of this FFS is not preclude any other potential feedback mechanism including combination of ACK/NACK-based and NACK-only based feedback (Please correct me if my understanding is not appropriate/correct). By deleting this FFS, does it mean that the feedback mechanism is only selection from ACK/NACK and/or NACK-only without considering other possibilities? We would like to suggest updating the proposal as follows: For RRC_CONNECTED UEs receiving multicast,  FFS: Support ACK/NACK based HARQ-ACK feedback for multicast,  From per UE perspective, UE feedback ACK or NACK.  From UEs within the group perspective,  FFS shared or separated resource allocation for ACK/NACK.  FFS shared or separated resource for NACK.  Support NACK-only based HARQ-ACK feedback for multicast,  From per UE perspective, UE only feedback NACK.  From UEs within the group perspective,  FFS shared resource allocation for NACK.  FFS separated resource for NACK.  FFS other HARQ-ACK feedback options FFS: configuration/selection of ACK/NACK-based and NACK-only based HARQ-ACK feedback |
| vivo | As we comment in the 2nd round discussion, for intial transmission and retransmission, we only agree to support PTM scheme 1, there is no need to differentiate which UE is NACK from gNB perspecitive and no need for supporting ACK/NACK-based feedback. We also support the following proposal proposed by moderator in the email,which adding FFS on ACK/NACK based HARQ-ACK feedback. For RRC_CONNECTED UEs receiving multicast, at least for PTM scheme 1 - FFS: Support ACK/NACK based HARQ-ACK feedback for multicast, - From per UE perspective, UE feedback ACK or NACK. - From Ues within the group perspective, - FFS shared or separated resource for ACK. - FFS shared or separated resource for NACK. - Support NACK-only based HARQ-ACK feedback for multicast, - From per UE perspective, UE only feedback NACK. - From Ues within the group perspective, - FFS shared resource for NACK. - FFS separated resource for NACK. - FFS other HARQ-ACK feedback options FFS: configuration/selection of ACK/NACK-based and NACK-only based HARQ-ACK feedback |
| Nokia, NSB | We support the proposal with the amendments suggested by CMCC/LG/VIVO |
| Futurewei | From the above list, majority of the companies would like to both ACK/NACK and NACK-only, of course, depending on the scenarios. We support both and in our opinion we should NOT prioritize one or the other and consider both on equal priority. Few clarifications/editorials: We also think the FFS in first bullet (FFS details, e.g., PRI, k1, TPC, etc.) is too detailed at current stage For both ACK/NACK and NACK-only - From UEs within the group perspective, - FFS shared or separated resource allocation details for ACK/NACK. .. - From UEs within the group perspective, - FFS shared resource allocation details for NACK. |
| Qualcomm | Thanks for FL for trying to make progress. We support candidate 1 but we can live with candidate 2 for progress. To share our understanding on vivo’s concern: - “one single PUCCH resource indicator (PRI) in DCI in PDCCH, but the PUCCH resource for different UE need to be different, we need to make sure that the single PUCCH resource indicator (PRI) can point to different PUCCH resource for different UE in PTM group so that there is no PUCCH resource collision”we think it is feasible by indicating PUCCH resource configuration via unicast RRC signaling. To share our understanding on CMCC’s concerns: - “based on CSI feedback or RSRP, network should directly use PTP for these cell edge UEs instead of put them in the group together with other UEs to use PTM transmission”CSI feedback is used for open-look LA. But ACK/NACK feedback for dynamic PTP retransmission can combat with bursty interference. The UE-specific CSI and ACK/NACK feedback are complementary to each other and should be discussed in RAN1. - “If the number of UEs in the same group is large, it’s not realistic to use ACK/NACK based HARQ-ACK feedback because we need to reserve large PUCCH resources for ACK/NACK feedback,” we think it is unnecessary to mandate all the UEs to use ACK/NACK in this case. - “it is also hard to share the PUCCH resources with unicast, more spec efforts are envisioned regarding the multiplexing/prioritization between HARQ-ACK feedback for PTM and HARQ-ACK feedback for PTP or other UCIs” we think it is another level of details since we haven’t agree to enable multiplexing/prioritizing unicast/multicast feedback yet. But to address the concerns from vivo and CMCC, is it possile to add a condition of “if PTP can be used for retransmission” for the first sub-bullet as a compromise? #### [High priority] Proposal 2.1-r3: (candidate 2) For RRC_CONNECTED UEs receiving multicast, at least for PTM scheme 1 - FFS: Support ACK/NACK based HARQ-ACK feedback for multicast if PTP can be used for retransmission, - From per UE perspective, UE feedback ACK or NACK. - From Ues within the group perspective, - FFS shared or separated resource for ACK. - FFS shared or separated resource for NACK. - Support NACK-only based HARQ-ACK feedback for multicast, - From per UE perspective, UE only feedback NACK. - From Ues within the group perspective, - FFS shared resource for NACK. - FFS separated resource for NACK. - FFS: configuration/selection of ACK/NACK-based and NACK-only based HARQ-ACK feedback |
| Intel | We also prefer candidate 1 and would like to echo QC’s comment in reponse to vivo’s concern. Furthermore, for NACK only case, we are not sure of the benefits of separate PUCCH resource configuration. But we can live with candidate 2 for progress. For ACK/NACK based feedback, the “FFS details, e.g., PRI, k1, TPC, etc.” should be removed for now. Building on QC’s proposal we suggest the following: #### [High priority] Proposal 2.1-r3: (candidate 2) For RRC_CONNECTED UEs receiving multicast, at least for PTM scheme 1 - FFS: Support ACK/NACK based HARQ-ACK feedback for multicast if PTP can be used for retransmission, - From per UE perspective, UE feedback ACK or NACK. - From UEs within the group perspective, - FFS shared or separated resource for ACK. - FFS shared or separated resource for NACK. - FFS: PUCCH resource configuration for ACK/NACK feedback e.g., shared or separate PUCCH resources - Support NACK-only based HARQ-ACK feedback for multicast, - From per UE perspective, UE only feedback NACK. - From Ues within the group perspective, - FFS shared resource for NACK. - FFS separated resource for NACK. - FFS: PUCCH resource configuration for NACK only feedback FFS: configuration/selection of ACK/NACK-based and NACK-only based HARQ-ACK feedback |

## HARQ-ACK feedback resource

Background

Resources for HARQ-ACK feedback was not discussed in the last meeting. Resources may depend on the HARQ-ACK feedback options also. In addition, when discuss HARQ-ACK feedback resource for MBS, the proposals need to clarify the discussion is from per UE perspective which may receive both unicast and MBS, or from UEs within the group perspective, for which the resource for UEs within the group may be common or different.

Submitted Proposals

(Futurewei) Proposal 1:

  - A common PUCCH resource for MBS HARQ feedback is defined.

    - This common PUCCH resource is in addition to any PUCCH that NR unicast may have.

    - The common PUCCH resource is indexed based on the HARQ process number

    - The common-RNTI scrambled DCI PRI field indicate orthogonal PUCCH resources per HARQ process ID to transmit UL feedback.

(Futurewei) Proposal 5:

  - If there are unicast and multicast TBs in the same slot, the HARQ-ACK feedback mechanism should allow more than one PUCCH with HARQ-ACK in the same slot and/or to consider multiplexing/prioritization

(Chengdu TD-Tech) Proposal 1:

  - HARQ-ACK feedback with the shared PUCCH resource is supported for the PTM bearer of MBS.

(Chengdu TD-Tech) Proposal 2:

  - ACK/NACK feedback with the dedicated PUCCH resource is supported for the PTM bearer of MBS.

(CATT) Proposal 5:

  - For HARQ-ACK feedback mechanism in MBS, the following methods can be considered:

- UE-specific PDCCH to indicate PUCCH resources for common PDSCH.

- Group-common PDCCH to indicate PUCCH resource for common PDSCH.

- Multiple group-common PDCCHs to indicate PUCCH resources for common PDSCH.

- Group-common PDCCH to indicate UE-specific periodic PUCCH resources.

- DCI of UE-specific PDCCH to indicate PUCCH resources for MBS.

(OPPO) Proposal 1:

  - For Broadcast service, a shared PUCCH resource for all UEs to do UL feedback is preferred.

(OPPO) Proposal 2:

  - For Groupcast service, both shared and separate PUCCH resource among UEs within the group are supported.

(Apple) Proposal 1:

  - Configure each UE in the group a dedicated PUCCH resource for MBS service.

(ZTE) Proposal 1:

  - Regarding ACK/NACK feedback for NR MBS, an orthogonal PUCCH resource should be determined for each UE in the same MBS group sharing the same K1 and PRI.

(ZTE) Proposal 2:

  - Regarding ACK/NACK feedback for NR MBS, UE determines the PUCCH resource for ACK/NACK feedback for NR MSS based on the last unicast PDCCH if UE receives both unicast and multicast at the same time.

(ZTE) Proposal 7:

  - If NACK-only feedback is supported for NR MBS, at least the following issues should be addressed.

    - PUCCH resource configuration and determination

    - Time domain overlapping between multiple NACK only PUCCHs

    - Time domain overlapping between NACK only PUCCH and unicast PUCCH/PUSCH

    - Processing order of NACK only related multiplexing

(Ericsson) Proposal 2:

  - Dedicated PUCCH resource is configured for PTM traffic HARQ feedback for each UE in PTM group.

### 1st round discussion

FL’s Comments

7 companies discussed PUCCH resource for HARQ-ACK feedback for MBS. Some companies see it from per UE perspective and some others from all Ues within the group perspective. Also, depending on the option for HARQ-ACK feedback, PUCCH resource for MBS might be different.

Given there is no clear majority view, FL lists the options for collecting companies’ view and suggests discussing the two proposals separately, for ACK/NACK based and NACK-only based, respectively.

FL’s Proposal

#### Proposal 2.2-1: (ACK/NACK based)

For ACK/NACK based HARQ-ACK feedback, PUCCH resource is,

- from per UE perspective,

  - Option 1: shared with PUCCH resource for unicast

  - Option 2: separate from PUCCH resource for unicast

- from Ues within the group perspective, orthogonal among the Ues

Collect views:

| Company | Comments |
| --- | --- |
| LG | We are OK with the proposal |
| Huawei/HiSilicon | Support the proposal. It is ACK/NACK based HARQ feedback, so option 1 seems more straightforward in our opinion. |
| OPPO | Similar view as proposal 2.1, whether separate or shared resource from UEs within the group perspective can keep open. In R16 NR SL, distance based SL feedback is introduced. In that case, the UE within a communication range requirement can perform SL feedback and using shared resource. This mechanism can also applied to MBS. The UE corresponding to different distance from gNB can perform feedback and using shared resource. - from Ues within the group perspective, orthogonal among the Ues - Option 1: shared PUCCH resource among Ues - Option 2: separate PUCCH resource among Ues |
| MTK | We are generally fine with the proposal. If there are many UEs in one MBS group, each UE need to feedback the ACK/NACK, which will use many PUCCH resource and have potential impact for unicast resource. Thus, we prefer option 2. |
| Lenovo, Motorola Mobility | We are OK with the proposal |
| CATT | We are generally OK with proposal 2.2-1. We would like to suggest adding clarification to each option about the scheduling mechanism/conditions, since it is not quite clear of the current two options. For PTM scheme 2, PDCCH is scrambled with C-RNTI, and it is feasible/flexible to indicate PUCCH for MBS and unicast per UE. For PTM scheme 1, a group-common PDCCH is used to schedule a group of UEs, and it is not feasible to indicate the PUCCH resources for each UE that can share PUCCH resource with its own unicast resource. For ACK/NACK based HARQ-ACK feedback, PUCCH resource is, - from per UE perspective, - Option 1: For PTM transmission scheme 2, shared with PUCCH resource for unicast - Option 2: For PTM transmission scheme 1, separate from PUCCH resource for unicast - from Ues within the group perspective, orthogonal among the Ues |
| Nokia, NSB | In general we support the proposal, however we feel it would be helpful to clarify that support of the options is in part dependent on the choice of feedback mechanism. |
| Convida | We prefer to postpone the discussion of this proposal until conclusion has been drawn for proposal 2.1. |
| Intel | We are ok support the proposal but with the understanding that to down-select among options, we need to further consider the details on whether CSS or USS is used for receiving the scheduling DCI and the HARQ feedback scheme being used. |
| Futurewei | We are OK with the proposal |
| Qualcomm | A general comment on the wording. It would be better to clarify that the proposals we are discussing in 8.12.2 are for multicast reception of RRC_CONNECTED UEs. It is ok to list up the options. But it is not clear whether we will down select one of the two options or possible to consider both Opt1 and Opt2. Shall we say Proposal 2.2-1: (ACK/NACK based): For multicast reception of RRC_CONNECTED UEs, the PUCCH resource for ACK/NACK based HARQ-ACK feedback is, - from per UE perspective, down select one of the following options: - Option 1: shared with PUCCH resource for unicast - Option 2: separate from PUCCH resource for unicast - Option 3: Option 1 or Option 2 based on configuration. - from Ues within the group perspective, orthogonal among the Ues |
| ZTE | We are generally fine with the proposal. Regarding Option1 and Option2, our understanding is that at least Option1 should be supported because it is expected that unicast and multicast will be supported in the same cell, Option1 is more efficient in terms of the PUCCH resource utilization. Below is some minor updates from our side. For RRC_CONNECTED Ues, PUCCH resource for ACK/NACK based HARQ-ACK feedback for multicast is, - from per UE perspective, - Option 1: shared with PUCCH resource for unicast - Option 2: separate from PUCCH resource for unicast - from Ues within the group perspective, orthogonal among the Ues |
| vivo | We are generally fine with the proposal. We share same view with CATT that the PUCCH with option 1 or option 2 may depend on the scheduling mechanism. |
| CMCC | Generally OK with the proposal. As the comment in proposal 2.1, two PTM schemes is discussed in AI 8.12. 1, and PTM scheme 1 (GC-PDCCH based) was agreed, the PUCCH resource configuration/determination schemes are different under these two PTM schemes, so we suggest adding the group scheduling mechanism in the proposal, and we can further discuss the PUCCH resource configuration in the future meeting if PTM scheme 2 are also supported For ACK/NACK based HARQ-ACK feedback under group-common PDCCH scheduling mechanism, PUCCH resource is, - from per UE perspective, - Option 1: shared with PUCCH resource for unicast - Option 2: separate from PUCCH resource for unicast - from Ues within the group perspective, orthogonal among the Ues |
| Spreadtrum | We are ok with the proposal. At least Option1 should be supported which is more efficient for PUCCH resource utilization, and the resource details can be further discussed. |
| Apple | We are ok with the proposal. Just to clarify, do we intend to down-select the option 1 and option 2 further? The two options are related to how to define the common frequency resource. |

#### Proposal 2.2-2: (NACK-only based)

For NACK-only based HARQ-ACK feedback, PUCCH resource is,

  - from per UE perspective, separate from PUCCH resource for unicast

  - from Ues within the group perspective, common among the Ues

Collect views:

| Company | Comments |
| --- | --- |
| LG | We are OK with the proposal |
| Huawei/HiSilicon | Support. |
| OPPO | OK |
| MTK | Support the proposal. |
| Lenovo, Motorola Mobility | We are OK with the proposal |
| CATT | We are OK with this proposal, and we suggest considering about PUCCH format in this proposal as well. For PUCCH format, one of the current PUCCH formats can be selected, or a new PUCCH format can also be designed for supporting MBS service feedback mechanism. So we suggest adding “FFS PUCCH format” to this proposal. |
| Nokia, NSB | OK with the proposal but with some additional bullet (suggestion below) to accommodate PUCCH shared for unicast and Mcast feedback - from the per UE perspective, a PUCCH resource that is shared with unicast |
| Convida | We prefer to postpone the discussion of this proposal until conclusion has been drawn for proposal 2.1. |
| Intel | We are ok to support the proposal, with an FFS on PUCCH formats to be used as suggested by CATT. |
| Futurewei | We are OK with the proposal |
| Qualcomm | Support the proposal with minor wording changes for clarification: Proposal 2.2-2: For multicast reception of RRC_CONNECTED UEs, the PUCCH resource for NACK-only based HARQ-ACK feedback is, - from per UE perspective, separate from PUCCH resource for unicast - from Ues within the group perspective, common among the Ues |
| ZTE | We prefer to discuss the detailed design for NACK-only based HARQ-ACK feedback after the discussion of Proposal 2.1. If NACK-only based HARQ-ACK feedback is introduced, then we are basically fine with the above proposal. |
| Vivo | We are fine with the proposal. |
| CMCC | Generally OK with the proposal. Similar view as in proposal 2.2-1, we suggest adding the group scheduling mechanism in the proposal For NACK-only based HARQ-ACK feedback under group-common PDCCH scheduling mechanism, PUCCH resource is, - from per UE perspective, separate from PUCCH resource for unicast from UEs within the group perspective, common among the UEs |
| Spreadtrum | We are ok with the proposal. |
| Apple | We are ok with the proposal. |

### 2nd round discussion

FL’s Comments

For either HARQ-ACK feedback option, PUCCH resources from UEs within the group perspective is merged into proposals in section 2.1 so that the PUCCH resources for HARQ-ACK for MBS is focused on the relation with PUCCH resources for HARQ-ACK for unicast in the proposals of this section.

CATT and vivo comment is noted. However, the proposals in this section focuses only on PUCCH resources and its relation to PUCCH resources for unicast. The relation with the scheduling mode is not the intention of the proposals and could be discussed later separately.

Nokia’s comment is noted. However, for NACK-only based HARQ-ACK feedback, I am not sure whether PUCCH resource for HARQ-ACK feedback for multicast shared with PUCCH resources for HARQ-ACK feedback for unicast makes sense. More clarification is needed. Note that the focus of proposals in this section is PUCCH resource for HARQ-ACK feedback.

I tried to make the proposals immune to be affected by the scheduling scheme, but when we talk about the PUCCH resource for multicast and its relation to PUCCH resource for unicast, it should be understand that group-common PDCCH scheduling is assumed, so I made it clear in the updated proposals basically as CMCC suggested.

FL’s Proposal:

#### [High priority] Proposal 2.2-1-r1: (ACK/NACK based)

For RRC_CONNECTED Ues receiving multicast, for ACK/NACK based HARQ-ACK feedback for group-common PDCCH scheduling, PUCCH resource for HARQ-ACK feedback from per UE perspective is, down-select one of the following options:

- Option 1: shared with PUCCH resource for HARQ-ACK feedback for unicast

- Option 2: separate from PUCCH resource for HARQ-ACK feedback for unicast

- Option 3: Option 1 or option 2 based on configuration

Collect views:

| Company | Comments |
| --- | --- |
| LG | We are fine with Proposal 2.2-1-r1. |
| Huawei/HiSilicon | Support |
| CMCC | Don’t support this proposal. As the comment in proposal 2.1-r1, we do not see the motivation of ACK/NACK based HARQ-ACK feedback for PTM scheme 1. |
| Nokia, NSB | OK with proposal. Preference for Option 3 |
| Ericsson | Ok with the proposals, we can downselect the options at a later stage. |
| Qualcomm | We are fine with the proposal. |
| Intel | Ok with current proposal. Downselection can be made later |
| OPPO | We are fine with Proposal 2.2-1-r1 |
| Futurewei | Ok with the proposal. As clarified in 2nd round Proposal 2.1r1 this ACK/NACK is not treated with any higher priority. Our preference is NACK-only based will be prioritized and, if time permitting, the proposed ACK/NACK based solution will be worked on. |
| MTK | We are fine with the proposal |
| ZTE | We realize that this proposal may be confusing because it is not clear whether the PUCCH configuration should be separate/shared or the actual PUCCH resource for transmission should be separate/shared. Our understanding is that this proposal is about PUCCH configuration. Thus, we propose the following For RRC_CONNECTED Ues receiving multicast, for ACK/NACK based HARQ-ACK feedback for group-common PDCCH scheduling, PUCCH resource configuration for HARQ-ACK feedback from per UE perspective is, down-select one of the following options: - Option 1: shared with PUCCH resource for HARQ-ACK feedback for unicast - Option 2: separate from PUCCH resource for HARQ-ACK feedback for unicast - Option 3: Option 1 or option 2 based on configuration |
| vivo | Don’t support this proposal. For ACK/NACK based HARQ-ACK feedback for group-common PDCCH scheduling, we have the same concern as CMCC. We prefer to deprioritize the discussion for ACK/NACK based HARQ-ACK feedback for group-common PDCCH scheduling. |
| Spreadtrum | We are fine with the proposal. |
| Lenovo, Motorola Mobility | We support this proposal. |

#### [High priority] Proposal 2.2-2-r1: (NACK-only based)

For RRC_CONNECTED Ues receiving multicast, for NACK-only based HARQ-ACK feedback for group-common PDCCH scheduling, PUCCH resource for HARQ-ACK feedback from per UE perspective is separate from PUCCH resource for HARQ-ACK feedback for unicast.

- FFS PUCCH format

Collect views:

| Company | Comments |
| --- | --- |
| LG | We are fine with Proposal 2.2-2-r1. |
| Huawei/HiSilicon | Support |
| CMCC | Fine with this proposal |
| Nokia, NSB | We are strongly against this proposal. REASON: This seems to rules out the alternative feedback option that could exist when there is simultaneous unicast, the option of NACK feedback multiplexed with unicast ACK-NACK feedback, as promoted by other companies (e.g. Futurewei proposal 6). SUGGESTION: For RRC_CONNECTED Ues receiving multicast, for NACK-only based HARQ-ACK feedback for group-common PDCCH scheduling, PUCCH resource for HARQ-ACK feedback from per UE perspective can either be: - Separated from PUCCH resource for HARQ-ACK feedback for unicast. - Multiplexed with the PUCCH resource for HARQ-ACK feedback for unicast. - FFS PUCCH format. |
| Ericsson | Ok with proposal. |
| Qualcomm | We are fine with the proposal. |
| Intel | OK with proposal |
| OPPO | OK |
| Futurewei | OK with the proposal |
| MTK | We are fine with the proposal |
| ZTE | Similar comments. We realize that this proposal may be confusing because it is not clear whether the PUCCH configuration should be separate/shared or the actual PUCCH resource for transmission should be separate/shared. Our understanding is that this proposal is about PUCCH configuration. Thus, we propose the following For RRC_CONNECTED Ues receiving multicast, for NACK-only based HARQ-ACK feedback for group-common PDCCH scheduling, PUCCH resource configuration for HARQ-ACK feedback from per UE perspective is separate from PUCCH resource for HARQ-ACK feedback for unicast. - FFS PUCCH format |
| vivo | Fine with this proposal |
| Spreadtrum | We are fine with the proposal. |
| Lenovo, Motorola Mobility | We support this proposal. |

### 1st Checkpoint

FL’s Comments

CMCC/vivo’s comment that ACK/NACK based HARQ-ACK feedback being not applied to group-common PDCCH scheduling is noted. As explained earlier, seems others/majority view ACK/NACK based HARQ-ACK feedback is applicable to group-common PDCCH scheduling. As moderator for progress, the proposal with FL’s note is still suggested for 1st checkpoint with ZTE’s comment reflected.

To Nokia’s comment to NACK-only based, the motivation of the proposal is from resource configuration perspective, the actual resource used for PUCCH transmission is another issue and will be discussed in section 2.4 UCI multiplexing/prioritizing, i.e., even though the resources are configured separately, for actual transmission, NACK-only is multiplexed with unicast ACK-NAKC or other ways will be discussed in section 2.4.

FL’s Proposals for 1st checkpoint:

#### [High priority] Proposal 2.2-1-r2: (ACK/NACK based)

For RRC_CONNECTED Ues receiving multicast, for ACK/NACK based HARQ-ACK feedback for group-common PDCCH scheduling, PUCCH resource configuration for HARQ-ACK feedback from per UE perspective is, down-select one of the following options:

- Option 1: shared with PUCCH resource for HARQ-ACK feedback for unicast

- Option 2: separate from PUCCH resource for HARQ-ACK feedback for unicast

- Option 3: Option 1 or option 2 based on configuration

[FL’s Note: CMCC/vivo’s concern is still ACK/NACK based HARQ-ACK feedback is not applied to group-common PDCCH scheduling.]

Collect concerns:

| Company | Comments |
| --- | --- |
| CATT | We are OK with the updated proposal and we slightly prefer to use option 3. - For option 3, besides higher layer configuration on the PUCCH resource sharing or differentiation, we also think it be possible to use PDCCH to indicate sharing or separation dynamically. For example, when a UE is configured with separate MBS PUCCH resources with unicast PUCCH resources but there is still a possibility that a following unicast PDCCH can indicate those MBS HACK/ACK feedback can be merged within unicast PUCCH. This is also discussed in our contribution R1-2007836. Therefore, we suggest updating the last sub-bullet of option 3 as: - Option 3: Option 1 or option 2 based on configuration or PDCCH indication - Thanks FL for noting the comments of vivo and CATT in first round discussion. By reading the current main bullet, “from per UE perspective” is clearer. But there is another potential case raised on this proposal: within one PTM scheme (e.g. 1 or 2), can the group of Ues use different options? For example, for a MBS service using PTM scheme 1 (group-common PDCCH scheduling), some of the Ues use option 1 while the rest of the Ues use option 2. This case can happen according to the current proposal with “from per UE perspective”. However, for PTM scheme 1, the PUCCH resources might be shared by the group of Ues, how can it be realized this shared PUCCH resource that can also be shared with per UE’s unicast PUCCH resource? Therefore, we would like to clarify that the configuration on the options is based on per UE or per MBS service or per PTM scheme? |
| Apple | We are ok with the proposal. For option3, our understanding is that the configuration is based PTM or PTP. |
| Chengdu TD Tech, TD Tech | We prefer to option 2. |
| CMCC | Same comment as proposal 2.1, we don’t support this proposal. |
| Convida | We prefer to postpone the discussion of this proposal until conclusion has been drawn for proposal 2.1. |
| vivo | We don’t support this proposal. It depends on conclusion for proposal 2.1. |
| Ericsson | Ok with proposal. |
| Qualcomm | We are fine with the proposal to list up options. |
| Nokia, NSB | OK with proposal |
| Spreadtrum | OK with proposal |

#### [High priority] Proposal 2.2-2-r2: (NACK-only based)

For RRC_CONNECTED Ues receiving multicast, for NACK-only based HARQ-ACK feedback for group-common PDCCH scheduling, PUCCH resource configuration for HARQ-ACK feedback from per UE perspective is separate from PUCCH resource for HARQ-ACK feedback for unicast.

- FFS PUCCH format

Collect concerns:

| Company | Comments |
| --- | --- |
| CATT | We prefer to add the following sub-bullet back from the previous proposal: - From Ues within the group perspective, common among the Ues From per UE perspective, the MBS PUCCH (NACK-only) resource can be separate with unicast, which is current proposal. But the above condition is also necessary to clarify how the group of UEs use the NACK-only PUCCH resource. |
| Chengdu TD Tech, TD Tech | We agree with the proposal. |
| Convida | We prefer to postpone the discussion of this proposal until conclusion has been drawn for proposal 2.1. |
| Samsung | Postpone decision – more discussion is needed to determine whether NACK-only HARQ-ACK feedback is necessary or functional and corresponding scenarios. |
| Qualcomm | We are fine with the proposal. |
| Spreadtrum | We are fine with the proposal. |

### 3rd round discussion

FL’s Comments

To address comments from CATT, regarding the first comment, CATT suggested modifying option 3 as “Option 3: Option 1 or option 2 based on configuration or PDCCH indication”, the response from FL is that the proposal intends to discuss PUCCH configuration for MBS, whether it is shared or separate from that for unicast. The comment seems more related to the actual PUCCH resource used which is a different thing and relates to the discussion of UCI multiplexing. Regarding the second comment, whether the configuration is per UE or per MBS or PTM scheme can be details of the configuration that could be discussed later if needed. This proposal only focuses on the configuration from per UE perspective and the relation with the configuration for unicast. Regarding the comment on NACK-only based, CATT suggested adding the sub-bullet “From Ues within the group perspective, common among the Ues”. FL’s response is that from per Ues within the group perspective is more related to the different NACK-only options, so it is better to be discussed in the HARQ-ACK feedback options and this proposal focuses on the configuration relation to the unicast.

Since people have concerns (CMCC/vivo concerne on ACK/NACK based and Samsung concerned on NACK-only based) the corresponding HARQ-ACK feedback may not be supported, from FL’s perspective, we can discuss them in parallel for more progress assuming the HARQ-ACK feedback option is supported, so “if supported” is added in the following updated proposals.

FL’s Proposals:

#### [High priority] Proposal 2.2-1-r3: (ACK/NACK based)

For RRC_CONNECTED Ues receiving multicast, for ACK/NACK based HARQ-ACK feedback if supported for group-common PDCCH scheduling, PUCCH resource configuration for HARQ-ACK feedback from per UE perspective is, down-select one of the following options:

- Option 1: shared with PUCCH resource for HARQ-ACK feedback for unicast

- Option 2: separate from PUCCH resource for HARQ-ACK feedback for unicast

- Option 3: Option 1 or option 2 based on configuration

#### [High priority] Proposal 2.2-2-r3: (NACK-only based)

For RRC_CONNECTED Ues receiving multicast, for NACK-only based HARQ-ACK feedback if supported for group-common PDCCH scheduling, PUCCH resource configuration for HARQ-ACK feedback from per UE perspective is separate from PUCCH resource for HARQ-ACK feedback for unicast.

- FFS PUCCH format

Collect concerns:

| Company | Comments |
| --- | --- |
| Samsung | As we commens our concerns on Proposal 2.1-r3, for Proposal 2.2-2-r3, following FFS should added for further discussion. - FFS: Performance, impact of PDCCH DTX, impact on gNB receiver |
| CMCC | #### For [High priority] Proposal 2.2-1-r3: (ACK/NACK based) We cannot agree it assuming that the ACK/NACK based feedback is supported. It’s not reasonable to first agree the concrete solutions before we agree to support the functionality especially considering that some companies have technical concerns regarding the motivation of the functionality. Even if we have the common understanding on the possible motivation, we also have to discuss and compare the technical effect between the ACK/NACK based feedback and other potential alternatives (e.g., the dynamic switch between PTP/PTM which is clearly stated to be supported in Rel-17). This discussion should be postponed before we have the common understanding regarding the motivation of supporting ACK/NACK based feedback for PTM scheme 1 and the potential spec impact. We have to make sure the potential benefit we get from the ACK/NACK based feedback deserves the substantial spec effort. |
| LG | If we cannot agree Proposal 2.2-1-r3 right now, we are fine to agree 2.2-2-r3 at this meeting and postpone Proposal 2.2-1-r3 for the next meeting. |
| CATT | We are generally fine with the FL’s two proposals, but we still have some further clarification/modification suggestions: - For proposal 2.2-1-r3 (ACK/NACK based) - We support option 3 that shared and separate PUCCH resource methods can be both used. In the main bullet, it is saying that “PUCCH resource configuration…”, so we think that option 3 can be updated with more general explanation as follows. - For our previous comment that “Option 3: Option 1 or option 2 based on configuration or PDCCH indication”: based on our understanding of the design, a UE can be configured with both shared PUCCH and separate PUCCH resources with unicast. A PDCCH can be used to indicate that a cetain PUCCH for MBS can be merged with a unicast, where the PUCCH for MBS was intended to use separate resources. I understand FL’s explanation that our proposed modification is more related to UCI multiplexing. To make the proposal more clear without ambiguous, we would like to suggest the following update on the proposal (only for Option 3). #### [High priority] Proposal 2.2-1-r3: (ACK/NACK based) For RRC_CONNECTED Ues receiving multicast, for ACK/NACK based HARQ-ACK feedback if supported for group-common PDCCH scheduling, PUCCH resource configuration for HARQ-ACK feedback from per UE perspective is, down-select one of the following options: - Option 1: shared with PUCCH resource for HARQ-ACK feedback for unicast - Option 2: separate from PUCCH resource for HARQ-ACK feedback for unicast - Option 3: Option 1 and option 2 ------------------------------------------------------------- - For proposal 2.2-2-r3 (NACK-only based) - We support the updated proposal. |
| vivo | We can postpone the discussion for ACK/NACK based feedback. We are fine with Proposal 2.2-2-r3. |
| Nokia,NSB | We are fine with proposal 2.2-2-r3. We prefer to postpone the discussion on 2.2-1-r3. |
| Intel | We ok with Proposal 2.2-2-r3 For 2.2-1-r3, it might be enough to downselect between Option 1 and 2. We are ok to postpone this discussion. We might need to also consider codebook design impact. |

## HARQ-ACK codebook

Background

HARQ-ACK codebook was not discussed in the last meeting. This discussion is about how the HARQ-ACK feedback is generated, including the cases of UE receiving unicast and MBS and the cases of UE receiving more than one MBS.

Submitted Proposals

(CATT) Proposal 6:

  - Rel-17 MBS HARQ-ACK codebook determination can reuse the current NR Type-1 semi-static and Type-2 dynamic codebook determination mechanism as baseline.

(CATT) Proposal 7:

  - Joint codebook determination is considered when Type-1 codebook is used for both MBS and unicast.

(CMCC) Proposal 2:

  - If ACK/NACK based HARQ-ACK feedback is to be supported for group-common PDCCH based group scheduling, and if no more than one PTM PDSCH are FDMed in one slot, both semi-static HARQ-ACK codebook and dynamic HARQ-ACK codebook can be considered for PTM.

(CMCC) Proposal 6:

  - Do not support semi-static HARQ-ACK codebook design when multiple PTM PDSCHs are FDMed in one slot.

(ZTE) Proposal 3:

  - Regarding ACK/NACK feedback for NR MBS for UEs only receiving MBS service, consider the following mechanisms.

    - Semi-static HARQ-ACK codebook based on the SLIVs

    - Dynamic HARQ-ACK codebook. UE generates one sub-codebook per MBS service and concatenates all the sub-codebooks together.

(ZTE) Proposal 4:

  - Regarding ACK/NACK feedback for NR MBS for UEs receiving both unicast and MBS service, UE generates sub-codebook for unicast and MBS service separately and concatenates the sub-codebooks together.

(Intel) Proposal 4:

  - Further study the following options for MBS HARQ codebook design

    - Joint codebook with unicast

    - Separate HARQ codebook for NR MBS

(Ericsson) Proposal 3:

  - When dynamic HARQ feedback codebook is configured, the DAI in PDCCH is counted independently between PTP and PTM and between different PTM group which can be distinguished according to the RNTI used for PDCCH.

### 1st round discussion

FL’s Comments

5 companies submitted proposals regarding the HARQ-ACK codebook for MBS.

Regarding HARQ-ACK codebook for MBS, both type 1 and type 2 codebooks can be supported, but 1 company proposed type 1 codebook is not supported when more than one PTM are FDM-ed  in the same slot. 1 company proposed details on how to generate dynamic codebook for UE receiving more than one MBS.

Regarding HARQ-ACK codebook determination for MBS and unicast, joint codebook or separate codebook are two options. 1 company proposed how to generate the joint codebook. 1 company proposed the DAI counting is independently between unicast and MBS.

FL suggests separately discussing the two cases, one for codebook for MBS and the other for codebook for MBS and unicast.

FL’s Proposal

#### Proposal 2.3-1: (codebook type for MBS)

Both Type-1 and Type-2 codebook are supported for MBS,

- FFS HARQ-ACK codebook if more than one MBS PDSCH are FDM-ed within one slot.

- FFS HARQ-ACK codebook if UE supports more than MBS services.

Collect views:

| Company | Comments |
| --- | --- |
| LG | We are OK with the proposal |
| Huawei/HiSilicon | We agree to support both Type-1 and Type-2. |
| OPPO | OK |
| MTK | We are fine with the proposal. |
| Lenovo, Motorola Mobility | We are OK with the proposal |
| CATT | We are generally OK with this proposal with the following clarification: For the second FFS: a word “one” might be missed. The second FFS bullet can be completed as “FFS HARQ-ACK codebook if UE supports more than one MBS services.” After complementation on this FFS, it is still unclear to us. If a UE supports more than one MBS services in a slot, what is the intention/potential design needed? |
| Nokia, NSB | OK |
| Convida | We are OK with the direction, but we prefer to postpone the discussions of this proposal until other more fundamental issues and proposals, e.g., proposal 2.1 and proposal 2.2, have been addressed and concluded. |
| Intel | OK |
| Futurewei | Generally Ok with the proposal. However, the discussion regarding multiple MBS in a slot, HARQ-ACK feedback, and available PUCCH resources need clarity before codebook determination. |
| Qualcomm | We suggest the following changes, where we think HARQ-ACK codebook Type 3 could be considered for aperiodic multicast feedback in at least (not limited to) unlicensed band. Proposal 2.3-1: Both Type-1 and Type-2 HARQ-ACK feedback codebook are supported for multicast reception of RRC_CONNECTED UEs, - FFS HARQ-ACK codebook if more than one MBS PDSCH are FDM-ed within one slot. - FFS HARQ-ACK codebook if UE supports more than one MBS service. - FFS HARQ-ACK codebook Type 3 is supported or not. |
| ZTE | We are generally fine with the proposal. |
| vivo | We agree to support both Type-1 and Type-2 codebook for MBS. In addition, we think enhanced type 2 and type 3 codebook should not be precluded. According to NRU discussion, enhanced type 2 and type 3 codebook are extended to licensed band. For MBS, these two types of codebook can also be used. For example, gNB can trigger UE to feedback type 3 HARQ-ACK codebook to get the information of PDSCH reception states in a period of time and use this information for link adaption. Another example for enhanced type 2 codebook, gNB can group multicast PDSCH as group 0 and unicast PDSCH as group 1, and then gNB can trigger UE to feedback HARQ-ACK for multicast PDSCH by UE-specific PDCCH, which can solve the issue that group common PDCCH can't indicate UE-specific PUCCH resource with one PRI. |
| CMCC | We are fine with the proposal |
| Spreadtrum | We are ok with the proposal. |
| Apple | We want to clarify this proposal is only applying for multiple MBS PDSCH case or multiplexing HARQ-ACK for MBS PSDSCH and unicast PDSCH. |

#### Proposal 2.3-2: (codebook for MBS and unicast)

For HARQ-ACK codebook determination, when HARQ-ACK feedback is available for both unicast and MBS, further down-select among:

- Option 1: generate a joint codebook for both MBS and unicast

- Option 2: generate codebook for MBS separately from codebook for unicast

- Option 3: combination of Option 1 and Option 2.

- FFS details.

Collect views:

| Company | Comments |
| --- | --- |
| LG | We are OK with the proposal |
| Huawei/HiSilicon | Both option 1 and 2 and even option 3 can be considered. Details for FFS may relate to proposal 2.4-2 also, but at this stage, we are ok with the proposal. |
| OPPO | OK |
| MTK | We are fine with the proposal. |
| Lenovo, Motorola Mobility | It depends on detailed scheduling option for MBS PDSCH. For group common DCI scheduling group common PDSCH, HARQ-ACK feedback for MBS has to be separated from HARQ-ACK codebook for unicast; For UE-specific DCI scheduling group-common PDSCH, HARQ-ACK feedback for MBS can be constructed in same codebook with unicast. So option 3 is preferred if option 3 means both options are supported. |
| CATT | We are generally OK with the proposal which needs some clarifications on the options: - For Option 1: Joint codebook generation can be used only when MBS and unicast are both configured with Type-1 codebook. As it is discussion in our contribution R1-2007836, when both MBS and unicast are configured using Type-1 codebook, the size of codebooks are pre-determined semi-statically, and it is feasible to manage the HARQ-ACK feedback information bits jointly for both MBS and unicast. - For option 2: when separate codebooks generation is used, multiplexing of codebooks between MBS and unicast should be considered. - For option 3: what is the meaning of combination? It should be clarified how “combination” is applied for option 1 and 2. According to the above analysis, we suggest that the proposal can be updated as: For HARQ-ACK codebook determination, when HARQ-ACK feedback is available for both unicast and MBS, further down-select among: - Option 1: generate a joint codebook for both MBS and unicast when Type-1 codebook is used. - Option 2: generate codebook for MBS separately from codebook for unicast, FFS codebooks multiplexing mechanism. - Option 3: combination of Option 1 and Option 2. - FFS details. |
| Nokia, NSB | OK |
| Convida | We are OK with the direction, but we prefer to postpone the discussions of this proposal until other more fundamental issues and proposals, e.g., proposal 2.1 and proposal 2.2, have been addressed and concluded. |
| Intel | We are ok with Options 1 and 2 and FFS. The meaning of option 3 is not clear to us. Without further details, we suggest to remove option 3 or have it as part of the FFS. |
| Futurewei | Generally ok with the proposal. The multiplexing/prioritization of HARQ-ACK in the Section 2.4 should be finalized before codebook determination. |
| Qualcomm | We are fine with the proposal. |
| ZTE | We are fine with the above proposal. |
| vivo | From our understanding, for option 1, it can also include that UE generates two sub-codebooks separately for MBS and unicast, and then concatenates the sub-codebooks together. Here, is it categorized option 3? It would be appreciated if FL can explain more for option 1/3. |
| CMCC | We are fine with the proposal |
| Spreadtrum | We are ok with the proposal. |
| Apple | The proposal should clarify that three options are used for ACK/NACK based feedback. |

### 2nd round discussion

FL’s Comments

All comments are noted and are reflected in the updated proposals for this round. To CATT’s comment, FFS HARQ-ACK codebook if UE supports more than one MBS service is to address one submitted proposal that the entire codebook for HARQ-ACK for multicast is generated by concatenating sub-codebooks that might be generated per each MBS service.

Regarding HARQ-ACK codebook for MBS and unicast, only “from per UE perspective” is added into the updated version. More clarification for the three options. Option 2 is always separately generated codebook. Option 1 can be Type-1 or Type- 2 HARQ-ACK codebook. For Type-2 codebook in option 1, I was thinking the codebook is generated separately at first but multiplexed together to be as a joint codebook, so in this sense, it can be option 3 as well. It pretty much depends on the details. Also, proposal 2.3-2-r1 can be medium priority since we are also discussing multiplexing/prioritizing which should be prioritized over proposal 2.3-2-r1.

FL’s Proposal

#### [High priority] Proposal 2.3-1-r1: (codebook type for MBS)

Both Type-1 and Type-2 HARQ-ACK codebook are supported for RRC_CONNECTED UEs receiving multicast,

- FFS HARQ-ACK codebook if more than one MBS PDSCH are FDM-ed within one slot.

- FFS HARQ-ACK codebook if UE supports more than one MBS service.

- FFS whether enhanced Type-2 and/or Type-3 HARQ-ACK codebook is supported or not.

Collect views:

| Company | Comments |
| --- | --- |
| LG | We are fine with Proposal 2.3-1-r1. |
| Huawei/HiSilicon | Support. Both Type-1 and Type-2 might be sufficient and enhanced Type-2 and Type-3 HARQ-ACK codebook may not be necessary in our view. |
| CMCC | Don’t support this proposal. As the comment in proposal 2.1-r1 and 2.2-1-r1, we only support NACK-only based HARQ-ACK feedback, there is no HARQ-ACK codebook issue. |
| Nokia, NSB | OK with updated Proposal |
| Ericsson | We prefer to postpone deciding on this issue once Proposal 2.2-1-r1 is resolved, as it has a direct impact on the design of the codebooks. |
| Qualcomm | Looks ok. |
| Intel | OK with current proposal |
| OPPO | OK |
| Futurewei | OK with the current proposal. However, the discussion regarding multiple MBS in a slot, HARQ-ACK feedback (proposal 2.1 and 2.2), and available PUCCH resources need clarity before codebook determination. |
| MTK | We are fine with the proposal |
| ZTE | Although we are fine with the proposal, it also looks reasonable to wait for the outcome of Proposal 2.2-1-r1. |
| vivo | This proposal depends on the discussion for Proposal 2.1-r1. It assumes that ACK/NACK based HARQ-ACK feedback is supported. So, at least we should add “if ACK/NACK based HARQ-ACK feedback is supported,” in the main bullet. |
| Spreadtrum | We are fine with the proposal. |
| Lenovo, Motorola Mobility | We support this proposal. |

#### [Medium priority] Proposal 2.3-2-r1: (codebook for MBS and unicast)

For HARQ-ACK codebook determination from per UE perspective, when HARQ-ACK feedback is available for both unicast and MBS, further down-select among:

- Option 1: generate a joint codebook for both MBS and unicast

- Option 2: generate codebook for MBS separately from codebook for unicast

- Option 3: combination of Option 1 and Option 2.

- FFS details.

Collect views:

| Company | Comments |
| --- | --- |
| LG | We are fine with Proposal 2.3-2-r1. |
| Huawei/HiSilicon | Both option 1 and 2 and even option 3 can be considered. Details for FFS may relate to proposal 2.4-2 also, but at this stage, we are ok with the proposal. |
| CMCC | Don’t support. Similar comment as Proposal 2.3-1-r1. |
| Nokia, NSB | OK with updated Proposal |
| Ericsson | Same comment as for 2.3-1-r1. We prefer waiting until the issue of Proposal 2.2-1-r1 is settled. |
| Qualcomm | Ok to us |
| Intel | Option 3 is still not clear to us. Suggest to move it to FFS. |
| OPPO | OK |
| Futurewei | Generally ok with the proposal. The multiplexing/prioritization of HARQ-ACK in the Section 2.4 should be finalized before codebook determination. As Intel mentioned, Option 3 is not clear, so suggest to move it as FFS. |
| MTK | We are fine with the proposal |
| ZTE | Although we are fine with the proposal, it also looks reasonable to wait for the outcome of Proposal 2.2-1-r1. |
| vivo | Similar comment as Proposal 2.3-1-r1. At least we should add “if ACK/NACK based HARQ-ACK feedback is supported,” in the main bullet. |
| Lenovo, Motorola Mobility | We support this proposal. |

### 1st Checkpoint

FL’s Comments

According to the comments collected, some companies concern the proposals in this section may be affected by discussion in section 2.1, section 2.2 or section 2.4, so prefer to discuss the proposal regarding HARQ-ACK codebook later.

FL’s suggestion for 1st checkpoint:

#### FL’s suggestion

Wait for progress of proposals in section 2.1, section 2.2 or section 2.4 and discuss later for 2nd checkpoint.

Collect concerns:

| Company | Comments |
| --- | --- |
| CATT | We agree with FL’s suggestion and wait until other sections have some progresses. For Proposal 2.3-2-r1: (codebook for MBS and unicast), we still think that the “joint codebook” in option 1 should be clarified what is “joint” meaning. As FL mentioned, option 1 using type-2 code book means that separately generate codebooks but multiplexing among the codebooks. We think that this is belonging to option 2. |
| Apple | Support FL’s suggestion. |
| Chengdu TD Tech, TD Tech | We agree with the FL’s suggestion for the 1st checkpoint. |
| Ericsson | Support the FL suggestion |
| Qualcomm | ok |
| Spreadtrum | Support the FL suggestion |

## UCI multiplexing/prioritization

Background

This was not discussed in the last meeting. Current specification supports two types of HARQ-ACK feedback corresponding to different priorities. The HARQ-ACK feedback with the same priority will be multiplexed and different priorities will be prioritized. When discussing the HARQ-ACK feedback for MBS, some relevant issues need to be discussed. For example, whether MBS is one priority or can be more than one priority; what the priority is between unicast and MBS; is it multiplexing or prioritizing between feedback for unicast and MBS when determining the feedback resources.

Submitted Proposals

(Futurewei) Proposal 6:

  - For the cases when HARQ-ACK feedback is available for both unicast and multicast transmissions, the multiplexed or prioritized HARQ-ACK is sent in UE-specific PUCCH resource and not in common PUCCH resource.

(Huawei) Proposal 6:

  - Priority indication and/or configuration schemes should be introduced for NR MBS. Moreover, the prioritization and multiplexing rules defined in URLLC can be the starting point for NR MBS.

(Vivo) Proposal 6:

  - Regarding HARQ-ACK for multicast PDSCH, the followings need to be discussed/decided.

    - Aggregated HARQ-ACK feedback, i.e., 1-bit HARQ-ACK for multiple PDSCHs can be considered

    - In case of simultaneous multicast and unicast traffic for the same UE, whether HARQ-ACK for multicast PDSCH and unicast PDSCH can be multiplexed in one HARQ-ACK CB

(CATT) Proposal 8:

  - The current multiplex mechanism (i.e. concatenates the TB-based HARQ-ACK codebook followed by the CBG-based HARQ-ACK codebook) can be reused for MBS and unicast codebook determination.

(Samsung) Proposal 7:

  - Support configuration between multiplexing and prioritization of MBS HARQ-ACK feedback and unicast UCI.

(OPPO) Proposal 6:

  - Multiplexing HARQ-ACK information for unicast and one/multiple MBMS in a single feedback channel should be supported.

(ZTE) Proposal 5:

  - Regarding ACK/NACK feedback for NR MBS for UEs receiving both unicast and MBS service, the existing multiplexing methods can be reused between unicast and MBS.

(Qualcomm) Proposal 2:

  - Support multiplexing of UE-specific ACK/NACK for unicast and multicast transmission based on UE capability.

    - FFS: Type 1, 2, 3 HARQ-ACK codebook for multiplexing unicast and multicast feedback

### 1st round discussion

FL’s Comments

8 companies submitted proposals regarding the relation between HARQ-ACK feedback for MBS and HARQ-ACK feedback for unicast when the feedback occasions are in the same slot.

For discussing UCI multiplexing (especially for HARQ-ACK feedback) between unicast and MBS, we also need to discuss the priority for MBS and the priority between unicast and MBS also unless HARQ-ACK feedback for MBS is always multiplexed with unicast.

Since this issue is firstly discussed in this work item, and there is no clear majority view. FL suggests the following proposals for further discussion.

FL’s Proposal

#### Proposal 2.4-1: (Priority for MBS and unicast)

Number of priority for MBS is, for further down-selection, options are:

- Option 1: One

- Option 2: two

- Option 2: More than two

- FFS the priority between MBS and unicast

Collect views:

| Company | Comments |
| --- | --- |
| LG | We are OK with the proposal |
| Huawei/HiSilicon | Both options 2 and 3 can be considered at this stage, because we see different MBS services have different requirements about latency and reliability as follows: MBS use cases; Latency; Reliability. V2X; 5-100ms; 90% to 99.9999%. Living Video; 150ms; 99.9%. IOT Software update; Latency Tolerant; Higher reliability is beneficial. Industry applications; 0.5ms; 99.9999%. |
| OPPO | OK |
| MTK | Support the proposal. |
| CATT | This proposal is not clear to us. It should be clarified before we discuss/down-select the options. - What is the “number of priority” means? - Option 1: when the priority number is 1, does it mean that there is no priority differentiation among the MBS services, in other words that all MBS services has the same priority? |
| Nokia, NSB | We are OK with this proposal --- though there is a typo --- Option 3: More than two |
| Convida | We are OK with the direction, but we prefer to postpone the discussions of this proposal until other more fundamental issues and proposals, e.g., proposal 2.1 and proposal 2.2, have been addressed and concluded. |
| Intel | The wording of this proposal is not clear to us. What is meant by “number of priority”? We also prefer to discuss UCI multiplexing details after basic issues are agreed i.e., proposal 2.1- 2.3 |
| Futurewei | Further clarification required on where the prioritized feedback is sent whether it is UE-specific PUCCH or common PUCCH. |
| Qualcomm | We prefer to postpone the proposal 2.4-1 for now. It is dependent on how many and what kind of multicast services that a UE can support (UE capability). It is also related with GC-PDCCH monitoring in CSS or USS or new MSS. We can discuss the details of priority after deciding the basic design. |
| ZTE | From our perspective, it is too premature to discuss the priority issue here. The priority is introduced to indicate different codebooks, while the basic codebook design is still FFS. In this case, we would propose not to discuss this issue in this meeting. |
| vivo | We are OK with the proposal for guidance. |
| CMCC | Similar to Qualcomm, we suggest further to discuss this proposal |
| Spreadtrum | We also prefer to postpone the discussion on the priority issue. |
| Apple | Could the FL clarify the number of priority? This priority is to be defined among MBS services or among MBS PDSCHs and unicast PDSCH. |

#### Proposal 2.4-2: (multiplexing/prioritizing)

For the cases of HARQ-ACK feedback is available for MBS and unicast, for determining the PUCCH resource, down-selection will be done among:

- Option 1: Multiplexing is applied

- Option 2: Prioritizing is applied

- Option 3: Combination of Option 1 and Option 2

Collect views:

| Company | Comments |
| --- | --- |
| LG | We are OK with the proposal |
| Huawei/HiSilicon | We prefer Option 3. Since we see MBS also has different requirements regarding latency and reliability, it is natural to have MBS defined with more than one priorities. Similar to what have been specified for URLLC, the UCI with the same priority is multiplexed and the UCI with low priority will be dropped, so we think it should be option 3. |
| OPPO | OK |
| MTK | We are fine with the proposal. |
| Lenovo, Motorola Mobility | We are OK with the proposal |
| CATT | We are OK with this proposal and we support option 1. - For option 2: 2 cases can be discussed - Case 1 (Dropping): PUCCH of MBS and PUCCH of unicast have collision. When prioritizing is applied, the PUCCH corresponding to its PDSCH with lower priority will be dropped. That means when option 2 is enabled, option 1 is disabled. - Case 2 (joint codebook): PUCCH of MBS and PUCCH of unicast have collision. Dropping may degrade the reliability of the system. Joint codebook generation can be considered under this case. The “combination” mechanism should be also clarified on how it works with both options supported. |
| Nokia, NSB | OK |
| Convida | We are OK with the direction, but we prefer to postpone the discussions of this proposal until other more fundamental issues and proposals, e.g., proposal 2.1 and proposal 2.2, have been addressed and concluded. |
| Intel | OK. However, we prefer to discuss UCI multiplexing details after basic issues are agreed i.e., proposal 2.1- 2.3 |
| Futurewei | Generally ok with the proposal. Same as previous reply “Further clarification required on where the prioritization is sent whether it is UE-specific PUCCH or common PUCCH.” PUCCH resource details need to be worked out before discussing the prioritization and multiplexing within those resources. |
| Qualcomm | The proposal 2.4-2 could be considered later after discussing the PUCCH resources for multicast HARQ-ACK feedback. |
| ZTE | From our perspective, it is too premature to discuss the multiplexing/prioritizing issue here. The multiplexing/prioritizing issue is highly depending on the detailed HARQ-ACK design, we may need to come back to this issue once we have some basic understanding on how the HARQ-ACK for multicast looks like. |
| vivo | We also prefer to postpone the discussions of this proposal. Regarding Prioritization, we think it can include two different options: - Option 2-1: For the cases of HARQ-ACK feedback for MBS and unicast are indicated in the same slot, UE will generate one HARQ-ACK codebook either for MBS or unicast - Option 2-2: For the cases of HARQ-ACK feedback for MBS and unicast are indicated in the same slot, UE determines HARQ-ACK PUCCH resources separately MBS and unicast, if the PUCCHs for MBS and unicast are overlapped in time-domain, then UE prioritizes one PUCCH over the other PUCCH. |
| CMCC | As the comment in proposal 2.1, two PTM schemes are still discussed in AI 8.12.1, the solutions for determining the PUCCH resource may be different under two PTM schemes, so we suggest postpone discuss this proposal. |
| Spreadtrum | We also prefer to postpone the discussions of this proposal. |
| Apple | Multiplexing or prioritizing is related to the HARQ-ACK feedback is NACK-only or ACK/NACK based feedback. So this proposal can discuss later after feedback mode is decided. |

### 2nd round discussion

FL’s Comments

The proposals are rephrased to be clearer. To address Futurewei’s comment, the priority is defined for instructing HARQ-ACK feedback for multicast only or for multiplexing/prioritizing HARQ-ACK feedback for multicast and unicast, so it could be UE-specific PUCCH or common PUCCH resources.

This proposals can be medium priority according to the comments received.

FL’s Proposal

#### [Medium priority] Proposal 2.4-1-r1: (Priority for MBS and unicast)

For RRC_CONNECTED UEs receiving multicast, multicast is defined with, for further down-selection, options are

- Option 1: One priority

- Option 2: two priorities

- Option 3: More than two priorities

- FFS the priority between multicast and unicast

Collect views:

| Company | Comments |
| --- | --- |
| LG | We are fine with Proposal 2.4-1-r1 |
| Huawei/HiSilicon | Ok with the proposal. Both options 2 and 3 can be considered at this stage, because we see different MBS services have different requirements about latency and reliability. |
| CMCC | Fine with this proposal. |
| Nokia, NSB | Can it be clarified what the difference is between one, two and more priorities is? For example, does one priority mean all multicast services (there could be multiple) are sent at the same priority level? |
| Ericsson | We can delay this discussion until the need for multiple priorities is identified. |
| Qualcomm | We think it can be low priority. |
| Intel | Similar comment as 1st round. Clarification needed on what priority means here. Discussion on this is lower priority and can be handled later. |
| OPPO | OK |
| Futurewei | OK with the proposal |
| MTK | We are fine with the proposal |
| ZTE | We think it is too premature to discuss this proposal for now. |
| vivo | Fine with this proposal in principle. One clarification, the priority here is defined for the HARQ-ACK of multicast PDSCH. Because for unicast PDSCH in URRLC, two PHY-layer priorities are supported for HARQ-ACK, it is different from the priority of PDSCH. We are also discussing to support TDM or FDM of unicast PDSCH and multicast PDSCH or multiple multicast PDSCH based on UE’s capability, if UE is not capable to receive, the handling rule for multiple PDSCH may have no relation with the priority here. |
| Spreadtrum | We also hope it can be handle later. |
| Lenovo, Motorola Mobility | This can be discussed after the outcome of 2.3. |

#### [Medium priority] Proposal 2.4-2-r1: (multiplexing/prioritizing)

For RRC_CONNECTED UEs receiving multicast to determine the PUCCH resource, for the cases of HARQ-ACK feedback is available for both multicast and unicast, down-selection will be done among:

- Option 1: Multiplexing is applied

- Option 2: Prioritizing is applied

- Option 3: Combination of Option 1 and Option 2

Collect views:

| Company | Comments |
| --- | --- |
| LG | We are fine with Proposal 2.4-2-r1. |
| Huawei/HiSilicon | We are OK with updated proposal 2.4-2-r1 and we prefer Option 3. Since we see MBS also has different requirements regarding latency and reliability, it is natural to have MBS defined with more than one priorities. Similar to what have been specified for URLLC, the UCI with the same priority is multiplexed and the UCI with low priority will be dropped, so we think it should be option 3. |
| CMCC | We should limit this proposal to the case of NACK-only feedback. For ACK/NACK feedback, we should postpone the discussion. |
| Nokia, NSB | Ok with the proposal. Preference for Option 3. |
| Ericsson | We prefer to wait that the HARQ resource sharing discussion is settled first. (Proposal 2.2-1-r1) |
| Qualcomm | We think it can be low priority. |
| Intel | This can be low priority for this meeting and can be handled later |
| OPPO | Agree with QC and Intel to set it as low priority |
| Futurewei | OK with the proposal |
| ZTE | We think it is too premature to discuss this proposal for now. |
| vivo | We think it can be low priority. If only NACK based HARQ-ACK feedback is supported for multicast, it may be difficult to do multiplexing. |
| Spreadtrum | We also hope it can be handle later. |
| Lenovo, Motorola Mobility | This can be discussed after the outcome of 2.3. |

### 1st Checkpoint

FL’s Comments

To address Nokia’s comment regarding priority for MBS, the intention of the proposal is to define the total number of priorities defined for multicast to further discuss the multiplexing/prioritizing the UCI of unicast. The proposal is updated to reflect this intention.

Further, to address the comment of “premature to discuss it”, from moderator’s perspective, this is the issue we have to face/solve to further discuss multiplexing or prioritizing the UCI of unicast. Granted the need for multiple priorities is not identified, the total number of priority will be one as in option 1. So this proposal is quite inclusive and FL suggests this proposal for the 1st checkpoint.

As to the proposal of multiplexing/ prioritizing, to address comment from CMCC, the options and maybe FFS also apply to both ACK/NACK based or NACK-only based HARQ-ACK feedback in general. I tried to make this proposal immune to be affected by other proposals. Hence, if ACK/NACK based is agreed, this proposal is applied to it and otherwise it is applied to NACK-only. If both ACK/NACK based or NACK-only based are agreed, the proposal is applied to both. Though some companies commented it is premature to discuss it or it should be low priority, FL still suggests it for 1st checkpoint since the proposal being inclusive itself seems stable.

FL’s Proposals for 1st checkpoint:

#### [Medium priority] Proposal 2.4-1-r2: (Priority for MBS and unicast)

For RRC_CONNECTED UEs receiving multicast, from the total number of priorities defined for multicast perspective, for further down-selection, options are

- Option 1: One priority

- Option 2: two priorities

- Option 3: More than two priorities

- FFS the priority between multicast and unicast

Collect concerns:

| Company | Comments |
| --- | --- |
| CATT | Generally fine with the proposal. We still think that one priority means no differentiation of priority. |
| Apple | We have the same undertsanind as CATT, one priority means no priority. |
| Ericsson | We can delay this discussion until the need for multiple priorities is identified. |
| Qualcomm | Again, we think it is low priority. In current stage, we are not sure there is a need to define number of priorities just for multicast. It could be possible to rely on network implementation to decide priority without differentiating unicast and multicast data. |
| Nokia, NSB | OK with this proposal (and thanks to the FL for the explanation) |

#### [Medium priority] Proposal 2.4-2-r1: (multiplexing/prioritizing)

For RRC_CONNECTED UEs receiving multicast to determine the PUCCH resource, for the cases of HARQ-ACK feedback is available for both multicast and unicast, down-selection will be done among:

- Option 1: Multiplexing is applied

- Option 2: Prioritizing is applied

- Option 3: Combination of Option 1 and Option 2

Collect concerns:

| Company | Comments |
| --- | --- |
| CMCC | As FL’s comments, the detailed HARQ-ACK feedback scheme (NACK-only based or ACK/NACK based) has not be agreed at this stage, we suggest to postpone this proposal, and wait for progress of proposals in section 2.1. |
| Lenovo, Motorola Mobility | Agree with CMCC, we can delay this discussion after the detailed HARQ-ACK schemes are determined. |
| Ericsson | Agree with CMCC, the discussion should be postpone until the HARQ solution from P.2.1 is more stable. |
| Qualcomm | Low priority |
| Nokia, NSB | Low priority |

## Enable/disable HARQ-ACK feedback

Background

The last meeting agreed the following:

Agreements:

- For RRC_CONNECTED UEs, HARQ-ACK feedback is supported for multicast and no additional evaluation is needed to justify this.

  - FFS: The detailed HARQ-ACK feedback solutions, e.g., ACK/NACK based, NACK-only based.

  - FFS: HARQ-ACK feedback can be optionally disabled and/or enabled.

Submitted Proposals

(Futurewei) Proposal 7:

  - HARQ-ACK feedback enabling and disabling is supported.

(Huawei) Proposal 3:

  - Disabling and enabling HARQ feedback should be introduced to NR MBS which can help with a good trade-off between HARQ feedback overhead and system reliability for NR MBS.

(Huawei) Proposal 4:

  - Disabling and enabling HARQ feedback is indicated by DCI.

(Huawei) Proposal 5:

  - The function of enabling/disabling HARQ-ACK feedback is optional.

(Vivo) Proposal 1:

  - HARQ-ACK feedback for multicast should be RRC configurable.

(CATT) Proposal 4:

  - A UE can only feedback HARQ-ACK information according to its interested/received MBS services.

(Samsung) Proposal 3:

  - A gNB can be configured by higher layers a UE whether or not to report HARQ-ACK information for SPS PDSCH receptions.

(OPPO) Proposal 5:

  - Zone and communication range based HARQ-ACK feedback should be considered for Broadcast/Multicast service.

(Nokia) Proposal 8:

  - HARQ-ACK feedback can be optionally disabled and / or enabled per UE.

(Lenovo) Proposal 2:

  - HARQ-ACK feedback for MBS can be enabled or disabled.

(MediaTek) Proposal 8:

  - NR multicast HARQ-ACK feedback can be disabled.

(MediaTek) Proposal 9:

  - NR multicast HARQ-ACK disable/enable indicator can be defined in DCI with 1bit.

(Intel) Proposal 1:

  - For NR MBS, HARQ feedback should be configurable i.e., it can be enabled or disabled. The following options can be considered for such configuration

    - Semi-static configuration through RRC signaling

    - Dynamic indication using a single bit in the scheduling DCI for the groupcast transmission

(Convida) Proposal 3:

  - HARQ feedback enabling and disabling should be supported in NR multicast and broadcast.

(Qualcomm) Proposal 3:

  - For RRC_CONNECTED UEs, HARQ-ACK feedback can be optionally enabled/disabled by RRC signaling.

    - The configuration of HARQ-ACK feedback can be configured for a given G-RNTI (corresponding to a service) or for a UE receiving a service.

### 1st round discussion

FL’s Comments

12 companies submitted proposals regarding enabling/disabling HARQ-ACK feedback for MBS. Basically all companies support enabling/disabling the HARQ-ACK feedback for MBS.

2 companies propose DCI indicates enabling/disabling, 3 companies propose higher layer signalling or RRC configures it, and 1 company proposes both options can be considered.

Also, 3 companies propose enabling/disabling function is optional. 1 company propose Zone and communication range based HARQ-ACK feedback should be considered for MBS.

FL’s Proposal

#### Proposal 2.5:

Optionally enabling/disabling HARQ-ACK feedback for MBS is supported, further down-select between:

- Option 1: DCI indicates enabling/disabling

- Option 2: RRC configures enabling/disabling

- FFS how to implement enabling/disabling optionally

Collect views:

| Company | Comments |
| --- | --- |
| LG | We are OK with the proposal |
| Huawei/HiSilicon | We agree with this proposal and we prefer Option 1 to dynamically control the feedback load. |
| OPPO | OK |
| MTK | Option 1. |
| Lenovo, Motorola Mobility | Option 1 is supported. |
| CATT | The proposal should be clarified in the following two aspects: - What does the word “Optionally” mean in the main bullet? - Adding option 3: support both DCI indication and RRC configuration enabling/disabling. DCI indication has low latency and high flexibility compared with RRC configuration. But RRC configuration has lower signalling overhead. Based on different use cases/scenarios, option 1 or 2 can be selected. Optionally enabling/disabling HARQ-ACK feedback for MBS is supported, further down-select between: - Option 1: DCI indicates enabling/disabling - Option 2: RRC configures enabling/disabling - Option 3: Support both option 1 and option 2 - FFS how to implement enabling/disabling optionally |
| Nokia, NSB | OK if an option 3 is added (Option 3: Both 1 and 2) |
| Convida | Our understanding on ‘optionally enabling/disabling HARQ function’ is that the RRC configures whether the function of the HARQ enabling/disabling is supported or not while the DCI is used to indicate whether HARQ is enabled or disabled. If our understanding is correct, we suggest to replace the FFS sub-bullet with addition of option 3: ‘RRC configures the enabling/ disabling function and DCI indicates enabling /disabling.’ If this is not the correct understanding, we suggest to clarify what does ‘implement enabling/disabling optionally’ mean. |
| Intel | Configuration by a combination of RRC and DCI should also be listed as an option i.e., Option 3 which supports Option 1 + 2. If this is added, we are ok to support this proposal We also believe this topic should be prioritized along with issues 2.1 – 2.3 |
| Futurewei | Option 1 is supported. Not clear what the “Optionally” in the proposal referring to. “Enabling/disabling HARQ-ACK feedback for MBS is supported” is sufficient I think. |
| Qualcomm | We think Opt2 (by RRC signaling) should be the baseline considering different multicast service QoS requirement. FFS Opt1 (by DCI signaling), which we need to study the potential impacts on UE behavior. |
| ZTE | We are generally fine with the proposal. We prefer to make the current proposal more general. Optionally enabling/disabling HARQ-ACK feedback for MBS is supported, further down-select between: - Option 1: DCI based solution - Option 2: RRC based solution - FFS how to implement enabling/disabling optionally |
| vivo | We are OK with the proposal in principle. Combination of Option 1 and Option 2 can also be considered. For example, using RRC to enable HARQ-ACK feedback and using DCI to dynamically disable. |
| CMCC | OK with this proposal. |
| Spreadtrum | We are ok with suggested proposal by CATT. |
| Apple | We prefer at this stage the proposal is high level, as we don’t know the design detail of DCI, such as - Option 1: dynamic indication of enabling/disabling, e.g, via DCI or MAC CE - Option 2: semi-static indication of enabling/disabling, e.g., via RRC signalling |

### 2nd round discussion

FL’s Comments

Last meeting agreed enabling/disabling is an optional functionality but how to reflect it is optional is a point for FFS. Based on the comments received in the 1st round, it seems combination option 1 and option 2 is what people have in mind to reflect it is optional, so option 3 is added in the update.

ZTE’s comment is noted. Since option 3 (combination of DCI+RRC) is added, we need to make it clear how DCI+RRC works in option 3 otherwise it may be meaningless, and also option 1 meaning DCI indicating enabling/disabling and option 2 meaning RRC configures enabling/disabling seem the common understanding in people’s mind and I did not see other proposals submitted could mean other ways. People can correct me if I missed something.

Qualcomm comment is noted. Since keeping three options seems fair enough to the majority, I accordingly updated the proposal in Proposal 2.5-r1.

FL’s Proposal:

#### [High priority] Proposal 2.5-r1:

Optionally enabling/disabling HARQ-ACK feedback for MBS is supported, further down-select between:

- Option 1: DCI indicates enabling/disabling

- Option 2: RRC configures enabling/disabling

- Option 3: RRC configures the enabling/ disabling function and DCI indicates enabling /disabling

Collect views:

| Company | Comments |
| --- | --- |
| LG | We are fine with Proposal 2.5-r1. |
| Huawei/HiSilicon | We support Option 3. |
| CMCC | Fine with this proposal. |
| Nokia, NSB | Ok with the proposal. Preference for Option 3. |
| Ericsson | Ok with the proposal. |
| Qualcomm | Ok to us |
| Intel | Ok with the proposal |
| OPPO | OK |
| Futurewei | OK with the proposal |
| MTK | We are fine with the proposal |
| ZTE | Fine with the proposal. |
| vivo | We are fine with the proposal |
| Spreadtrum | We are fine with the proposal |
| Lenovo, Motorola Mobility | We are OK with the proposal. |
| Chengdu TD Tech, TD Tech | We think RRC can indicate the cell-level enabling/disabling of the HARQ-ACK feedback. The dynamic switching between enabling and disabling of the HARQ-ACK feedback for each MBS can be determined in the MAC layer. DCI format or MAC CE can be used for the dynamic switching in the MAC layer. For the DCI format based method, the specific description needs to be provided for further selection. For example, the new DCI format or the new field in the DCT format needs to be support? |

### 1st Checkpoint

FL’s Comments

No concerns collected in the second round discussion.

FL’s Proposal for 1st checkpoint:

#### [High priority] Proposal 2.5-r1:

Optionally enabling/disabling HARQ-ACK feedback for MBS is supported, further down-select between:

- Option 1: DCI indicates enabling/disabling

- Option 2: RRC configures enabling/disabling

- Option 3: RRC configures the enabling/ disabling function and DCI indicates enabling /disabling

Collect concerns:

| Company | Comments |
| --- | --- |
| CATT | OK with the proposal. But still, “optionally” in the main bullet is still needed to be clarified. If optionally enabling/disabling is supported, what is the default action? Default action is enabling or disabling? |
| Apple | We prefer at this stage the proposal is high level, as we don’t know the design detail of DCI, such as - Option 1: dynamic indication of enabling/disabling, e.g, via DCI or MAC CE - Option 2: semi-static indication of enabling/disabling, e.g., via RRC signalling - Option 3: dynamic indication of enabling/disabling and semi-static indication of enabling/disabling |
| Chengdu TD Tech, TD Tech | How about MAC CE to enable and disable the HARQ-ACK feedback? |
| Convida | We also support to include MAC CE based enabling and disabling in the proposal. |
| Ericsson | Ok with the proposal. |
| Qualcomm | We are fine with the proposal. |
| Nokia, NSB | OK with the proposal |
| Spreadtrum | We are fine with the proposal. |

### 3rd round discussion

FL’s Comments

CATT comment on “optional” in the main bullet is deleted in the following update preproposal, which seems stable now. For Apple’s comment, FL suggests having a concrete proposal, if more options beyond DCI/MAC-CE and RRC, we can bring them up now for further concrete discussion later.

In case of any more concern, you can provide them in the following table.

FL’s Proposal:

#### [High priority] Proposal 2.5-r2: (enable/disable HARQ-ACK)

Enabling/disabling HARQ-ACK feedback for MBS is supported, further down-select between:

- Option 1: DCI/MAC-CE indicates enabling/disabling

- Option 2: RRC configures enabling/disabling

- Option 3: RRC configures the enabling/ disabling function and DCI/MAC-CE indicates enabling /disabling

Collect concerns:

| Company | Comments |
| --- | --- |
| LG | Both DCI/MAC-CE options would support dynamic control of enabling/disabling. We think that it is beneficial to support dynamic control of enabling/disabling, e.g. based on QoS characteristic of each TB as in NR SL. Thus, we support DCI. However, we are skeptical about MAC-CE. So, you may want to change as follows: #### [High priority] Proposal 2.5-r2: (enable/disable HARQ-ACK) Enabling/disabling HARQ-ACK feedback for MBS is supported, further down-select between: - Option 1: Dynamic signalling such as DCI/MAC-CE indicates enabling/disabling - Option 2: RRC configures enabling/disabling - Option 3: RRC configures the enabling/ disabling function and DCI/MAC-CE indicates enabling /disabling |
| Nokia/NSB | Support the proposal, but suggest modifying “DCI/MAC-CE” to clarify what that actually means “ffs” “and” “or” |
| Intel | We support the proposal with DCI. We can have an FFS for MAC-CE since it’s not clear if we require UE specific or group-common MAC-CE signaling. #### [High priority] Proposal 2.5-r2: (enable/disable HARQ-ACK) Enabling/disabling HARQ-ACK feedback for MBS is supported, further down-select between: - Option 1: Dymainc indication of enabling/disabling e.g., via DCI - Option 2: RRC configures enabling/disabling - Option 3: RRC configures the enabling/ disabling function and indication of enabling /disabling is dynamic e.g. via DCI - FFS: Dynamic indication via MAC-CE |

## Retransmission

Background

Retransmission was not discussed in the last meeting. The HARQ-ACK feedback option may also affect how to retransmit MBS, i.e., via PTP or PTM.

Submitted Proposals

(Futurewei) Proposal 3:

  - Allow scheduling more than one MBS TBs within a single DCI.

(ZTE) Proposal 6:

  - Both PTM-based and PTP-based retransmissions can be considered for NR MBS.

(Google) Proposal 2:

  - Support UE-specific PDCCH scheduling broadcast/multicast PDSCH for MBS initial transmission and retransmission.

(MediaTek) Proposal 10:

  - HARQ feedback Opt 1 should be supported in order that gNB can use unicast or multicast based retransmission for each UE.

(MediaTek) Proposal 11:

  - For common NACK only feedback mode, the retransmission only support multicast mechanism.

(Intel) Proposal 5:

  - For ACK/NACK based HARQ operation, support UE specific CBG based retransmission. Other advanced retransmission schemes are not precluded.

(Ericsson) Proposal 4:

  - PTP based retransmissions can be soft combined with earlier PTM transmissions

### 1st round discussion

FL’s Comments

6 companies submitted proposals regarding the retransmission for MBS. Basically, retransmission for MBS can be group-common or via unicast. For NACK-only based HARQ-ACK feedback and the resource for NACK-only are shared, retransmission will be group-common. For the case of NACK-only resource are specific from UEs within the group perspective, retransmission can be group-common or UE-specific. For the cases of ACK/NACK based HARQ-ACK feedback, retransmission can be group-common or UE-specific.

Therefore, the retransmission for MBS may depend on the HARQ-ACK feedback option or the resources for HARQ-ACK feedback. We either discuss this issue when the discussion of feedback options and feedback resources have progress, or keep the retransmission discussion in general or high level. At this stage, FL suggests the latter.

FL’s Proposal

#### Proposal 2.6:

Retransmission of group-common PDSCH for MBS supports, for the purpose of down-selection, options are:

- Option 1: group-common PDCCH scheduled group-common PDSCH

- Option 2: UE specific PDCCH scheduled PDSCH

- Option 3: both option 1 and option 2

- FFS CBG based retransmission

Collect views:

| Company | Comments |
| --- | --- |
| LG | This proposal seems duplicated with Proposal 2-3 in Summary#1 provided by CMCC. Option 2 can be further clarified whether PDSCH is group common or UE specific. |
| Huawei/HiSilicon | Support the proposal in principle but ok to coordinate with another agenda where this issues should be discussed. |
| OPPO | Agree with LG that option 2 should be clarified further about the PDSCH |
| MTK | We support option 3 as commented in Summary#1 provided by CMCC |
| Lenovo, Motorola Mobility | Agree with LG that Option 2 should be further clarified. |
| CATT | We are generally OK with this proposal, and we support option 3. As LG mentioned, this proposal can be discussed in another AI 8.12.1. |
| Nokia, NSB | Agree with LG comments – for information, proposal 2-3 is copied below. 3.2.3 Proposal 2-3 from Summary#1 [Moderator’s recommendation] Proposal 2-3: For RRC_CONNECTED UEs, if initial transmission of MBS service is based on PTM transmission scheme 1, at least support retransmission can use PTP transmission or PTM transmission scheme 1. • FFS: whether to support PTM transmission scheme 2 for retransmission. • FFS: How to indicate the association between PTM scheme 1 and PTP transmitting the same TB. • FFS: If multiple retransmission schemes are supported, then can different retransmission schemes be supported simultaneously for different subgroups of UEs in the same MBS group? |
| Convida | We are generally OK with the proposal. We agree with LG that clarification is needed for option 2 |
| Intel | Ok with LG’s clarification |
| Futurewei | Option 2 require further clarification if PDSCH is a group-common or UE-specific. Also, this discussion of retransmission should be aligned with Email thread-01 Proposal 2-3. |
| Qualcomm | As addressed by other companies, the current proposal is overlapped with some discussion in 8.12.1. |
| ZTE | The discussion is duplicated with that in group scheduling. As this is more like a scheduling issue, we slightly prefer to discuss this issue under group scheduling session. |
| vivo | We support option 3 as commented in Summary#1 provided by CMCC. |
| CMCC | We suggest to discuss this proposal in AI 8.12.1 |
| Spreadtrum | We support option 3 as commented in Summary#1 provided by CMCC. |
| Apple | We agree the option 2 need to be clarified. Option 3 is preferred. |

### 2nd round discussion

FL’s Comments

As discussed in the GTW call, this issue is discussed both in AI 8.12.1 and AI 8.12.2. From moderator’s perspective, I think this issue can be discussed in both agendas with different angles or focuses.

The proposal is rephrased in Proposal 2.6-r1 from the angle of HARQ-ACK feedback options. With two open HARQ-ACK feedback options, retransmission for multicast can be option 1/2/3.

I see this proposal can be medium priority due to the retransmission scheme depending on the HARQ-ACK feedback option.

FL’s Proposal

#### [Medium priority] Proposal 2.6-r1:

From the perspective of RRC_CONNECTED UEs receiving multicast, with ACK/NACK based and NACK-only based as two open options for HARQ-ACK feedback, retransmission of group-common PDSCH for multicast supports, for the purpose of down-selection, options are:

- Option 1: group-common PDCCH scheduled group-common PDSCH

- Option 2: UE specific PDCCH scheduled PDSCH

- Option 3: both option 1 and option 2

- FFS CBG based retransmission

Collect views:

| Company | Comments |
| --- | --- |
| LG | We are fine with Proposal 2.6-r1. Option 2 can be further clarified whether PDSCH is group common or UE specific. |
| Huawei/HiSilicon | OK with the proposal and support Option 3. With ACK/NACK based feedback, both Option 1 and Option 2 apply. With NACK-only based HARQ-ACK feedback, option 1 applies. |
| CMCC | We also think Option 2 should be further clarified whether the PDSCH is group common or UE specific. If it is UE-specific PDSCH, as we explained in Proposal 2.1, we may need to further study including the motivation. |
| Nokia, NSB | Option 2 needs clarification as highlighted by LG. |
| Ericsson | We would prefer to wait with the proposal. Our preference would be option1. For option 2, if the intention is to also cover a group PDSCH, the option is being discussed in AI 8.12.1 (PTM Scheme 2), so we suggest waiting for a decision on the issue before discussing retransmission for PTM scheme 2. We’re ok with having CBG retransmission FFS.” |
| Qualcomm | We are fine with the proposal. |
| Intel | Generally ok. For option 2, both group-common and UE Specific PDSCH can be listed as sub-options in line with comments from LG and Nokia. |
| OPPO | OK |
| Futurewei | Generally OK with the proposal. Option 2 require further clarification if PDSCH is a group-common or UE-specific. |
| MTK | We are generally fine with the proposal |
| ZTE | The discussion is duplicated with that in group scheduling. As this is more like a scheduling issue, we slightly prefer to discuss this issue under group scheduling session. Besides, there may be more schemes defined in the scheduling session. The current Options listed in the proposal seem not in line with that defined in the scheduling session. |
| vivo | We are fine with the proposal in principle. We also prefer to make clarification for Option 2. |
| Spreadtrum | We are ok with the proposal in principle. We also prefer to make clarification for Option 2. |
| Lenovo, Motorola Mobility | Generally, we are OK with the proposal. Clarification is needed for Option 2 whether it is UE-specific PDSCH or group-common PDSCH. |

### 1st Checkpoint

FL’s Comments

Comments collected are asking clarification for PDSCH scheduled by UE specific PDCCH, so far no option is intended to be precluded. The updated proposal 2.6-r2 with this clarification is suggested for 1st checkpoint.

FL’s Proposal for 1st checkpoint:

#### [Medium priority] Proposal 2.6-r2:

From the perspective of RRC_CONNECTED UEs receiving multicast, with ACK/NACK based and NACK-only based as two open options for HARQ-ACK feedback, retransmission of group-common PDSCH for multicast supports, for the purpose of down-selection, options are:

- Option 1: group-common PDCCH scheduled group-common PDSCH

- Option 2: UE-specific PDCCH scheduled PDSCH

  - Alt 1: PDSCH is UE-specific PDSCH

  - Alt 2: PDSCH is group-common PDSCH

- Option 3: both option 1 and option 2

- FFS other options

- FFS CBG based retransmission

Collect concerns:

| Company | Comments |
| --- | --- |
| CATT | We prefer to add option 3 back with considering both option 1 and option2. We can understand that the third bullet “FFS other options” does not preclude any other option. But since option 3 was there without strong objections during the first and second round discussion, we can keep it in the proposal, because the main bullet said that down-selection. |
| Apple | We prefer Option3, not so clear why option 3 is removed, one objects this option. Network can indicate UE whether option1 or option 2 is applied. - Option 3: both option 1 and option 2 |
| CMCC | Prefer Option 3, option 1 and option 2 have different use cases, and gNB can decide to use which option. |
| Lenovo, Motorola Mobility | Maybe it is better to clarify the scheme of initial transmission of the group-common PDSCH. Is it scheduled by group-common PDCCH or UE-specific PDCCH? If it is scheduled by group-common PDCCH, it is obvious that option 2 can’t work because gNB doesn’t know which UE sends NACK. |
| vivo | Option 3 is preferred. It can be up to gNB to decide which option for retransmission based on different situation. And it is benifical for RU accoding to our simulation. |
| Ericsson | We would prefer to wait with the proposal. Our preference would be option1. For option 2, if the intention is to also cover a group PDSCH, the option is being discussed in AI 8.12.1 (PTM Scheme 2), so we suggest waiting for a decision on the issue before discussing retransmission for PTM scheme 2. We’re ok with having CBG retransmission FFS.” |
| Qualcomm | We prefer Option 3. |
| Nokia, NSB | Ok with updated proposal. |
| Spreadtrum | We prefer Option 3. |

### 3rd round discussion

FL’s Comments

Comments are reflected in the following updated proposal.

FL’s Proposal:

#### [Medium priority] Proposal 2.6-r3: (Retransmission)

From the perspective of RRC_CONNECTED UEs receiving multicast, at least for PTM scheme 1 initial transmission, with ACK/NACK based and NACK-only based as two open options for HARQ-ACK feedback, retransmission supports, for the purpose of down-selection, options are:

- Option 1: group-common PDCCH scheduled group-common PDSCH

- Option 2: UE-specific PDCCH scheduled PDSCH

  - Alt 1: PDSCH is UE-specific PDSCH

  - Alt 2: PDSCH is group-common PDSCH

- Option 3: both option 1 and option 2

- FFS other options

- FFS CBG based retransmission

Collect concerns:

| Company | Comments |
| --- | --- |
| Samsung | We prefer to remove “with ACK/NACK based and NACK-only based as two open options for HARQ-ACK feedback” as these are still not supported yet. |
| Lenovo, Motorola Mobility | CBG-based retransmission can not work when NACK only feedback is adopted. Considering 2, 4, 6 or 8 bits are needed for CBG-based retransmission, it is better not to support CBG-based feedback for MBS. |
| CATT | We are supporting this proposal. As we agreed in AI 8.12.1, the total number of re-transmissions could be more than one. I would like to suggest to add “s” to the word “retransmission” in the main bullet and last sub-bullet of FFS. |
| Nokia, NSB | This agreement should be postponsed until “Proposal 2.1-r3” is resolved, e.g. the current wording implies both ACK/NAK and NACK-only have equal priority, but depending on Proposal 2.1-r3, this may not be the case. |
| Intel | Agree with Nokia. Can we discussed after HARQ schemes are finalized. |

# PDSCH repetition

Background

The last meeting agreed the following:

Agreements:

- For RRC_CONNECTED UEs, at least support slot-level repetition for group-common PDSCH.

  - FFS: whether enhancement is needed

Submitted Proposals

(Futurewei) Proposal 8:

  - For broadcast, if slot-level repetition is the only method for improving reliability, this feature can be enabled. FFS for multicast.

(Huawei) Proposal 7:

  - The PDSCH repetition mechanism and CSI feedback mechanism used in NR unicast can be applied to NR MBS without any enhancements.

(Vivo) Proposal 12:

  - For PDSCH repetition of group-common PDSCH, the PDSCH repetition factors for different MBS services should be separately configured.

(CATT) Proposal 9:

  - To support multi-beam transmission in MBS, gNB can transmit same MBS data on all SSB beams.

(CATT) Proposal 10:

  - UE can receive MBS data from neighbor SSB-beam, and the soft-combination is used to improve the reliability of MBS receptions.

(CMCC) Proposal 7:

  - For slot-based PDSCH repetition for NR MBS, the repetition number is configured by RRC signalling.

(Apple) Proposal 2:

  - To enhance the MBS reception reliability, the following schemes can be considered for RRC_CONNECTED UEs

    - Dynamic indication of MBS PDSCH repetition number

    - Frequency hopping

    - Cross-slot channel estimation

    - Enhanced CSI report

(ZTE) Proposal 9:

  - Slot-level repetition of group-common PDSCH is supported for UEs in RRC_IDLE/INACTIVE states.

(Nokia) Proposal 5:

  - Slot-level blind transmissions with different redundancy versions are followed by adaptive HARQ feedback-based retransmissions, in order to reduce PUCCH resource consumption without decreasing the performance of the system significantly compared to fully adaptive scheme with only adaptive HARQ-based retransmissions.

(Lenovo) Proposal 4:

  - The number of transmission repetitions for MBS PDSCH is configured by RRC signaling.

(Intel) Proposal 7:

  - NR MBS supports dynamic indication of number of repetitions for PDSCH transmissions with slot-based repetitions

(Qualcomm) Proposal 4:

  - Support independent repetition configuration for GC-PDSCH with different G-RNTIs.

(Qualcomm) Proposal 5:

  - Support semi-static and dynamic slot-level repetition for GC-PDSCH.

    - Semi-static and dynamic repetition for GC-PDSCH are not simultaneously configured for the GC-PDSCH associated with same G-RNTI

    - FFS: gap in between repeated GC-PDSCH slots

(Ericsson) Proposal 5:

  - The Rel.16 NR unicast framework for PDSCH repetition is reused for multicast/PTM.

### 1st round discussion

FL’s Comments

12 companies submitted proposals regarding PDSCH repetition for MBS for improving reliability.

1 company proposed slot-level repetition is only for broadcast and FFS for multicast. It is unclear so far what broadcast and multicast mean and what the difference is in RAN1. FL suggest discussing it later when the difference is clearer and maybe guidance from RAN2 is needed. Also, RRC_CONNECTED UE is the focus for the discussion of this agenda.

There are some proposals that repetition number is indicated by RRC, DCI, or RRC/DCI but not simultaneously. Also, there is single proposal related to SSB based beams, a single proposal for blind transmission, and a single proposal related to configuration for different MBS services.

Per FL’s understanding, PDSCH aggregation configured by RRC is supported since Rel-15 and Rel-16 introduced that PDSCH repetition can be indicated also via DCI as agreed for URLLC in mTRP but which seems mTRP specific, for example, 38.214 states the follows:

When a UE is configured by the higher layer parameter repetitionNumber-r16 in PDSCH-TimeDomainResourceAllocation-r16, the UE may expect to be indicated with one or two TCI states in a codepoint of the DCI field 'Transmission Configuration Indication' together with the DCI field "Time domain resource assignment' indicating an entry which contains repetitionNumber-r16 in PDSCH-TimeDomainResourceAllocation-r16 and DM-RS port(s) within one CDM group in the DCI field "Antenna Port(s)".

Since the last meeting has agreed to support slot-level repetition for group-common PDSCH, whether the repetition is referring to Rel-15 slot aggregation, Rel-16 repetition for mTRP or something different from Rel-15 and Rel-16 can be further discussed.

FL’s Proposal

#### Proposal 3:

For slot-level repetition for group-common PDSCH, for indicating the repetition number, further down-select among:

- Opt 1: by DCI

- Opt 2: by RRC

- Opt 3: either Opt 1 or Opt 2.

- FFS independently configured for different GC-PDSCH with different G-RNTIs.

- FFS relation with SSB.

Collect views:

| Company | Comments |
| --- | --- |
| LG | We are OK with the proposal. |
| Huawei/HiSilicon | Support the proposal. |
| OPPO | OK |
| MTK | We are fine with this proposal. |
| Lenovo, Motorola Mobility | Option 2 is preferred. |
| CATT | We are generally OK with the proposal, but we need further clarification on the two FFS bullets. - FFS independently configured…: Different MBS services have different repetition number, even in the same SSB. - FFS relation with SSB: A MBS service can have different repetition number under different SSBs; a MBS service should have the same repetition number under different SSBs. |
| Nokia, NSB | OK |
| Convida | We support to reuse the Rel-15 and Rel-16 mechanism as much as possible. |
| Intel | Option 3 in current proposal is not clear since main bullet suggests that options are for down-selection. If we choose, Option 3, what does it imply? Both RRC and DCI are supported, if not then which one? Suggest to remove Option 3 or make it RRC+DCI Additionally, while the dynamic indication for repetition number was specified in the mTRP agenda in Rel-16, it may not be used only for mTRP operation. It utilizes the TDRA table to indicate repetition and similar functionality can be adopted for multicast. The FFS points can be made more generic i.e., FFS: Further enhancements for configuration of slot-level repetition. |
| Futurewei | We are OK with the proposal |
| Qualcomm | So far RAN1 only agreed the slot-level repetition for GC-PDSCH of CONN UEs. We agree with Intel’s concern and suggest to rewording the proposal as Proposal 3: For slot-level repetition for group-common PDSCH of RRC_CONNECTED UEs, for indicating the repetition number, further down-select among: - Opt 1: dynamically by RRC+DCI - Opt 2: semi-statically by RRC - Opt 3: either Opt 1 or Opt 2. - FFS independently configured for different GC-PDSCH with different G-RNTIs. - FFS enhancements for configuration of slot-level repetition. |
| ZTE | We are generally fine with the proposal. Slot-level repetition can also be supported for RRC_IDLE/INACTIVE, we are not sure whether this should be reflected here. |
| vivo | We prefer to reuse PDSCH repetition mechanism in NR R15/R16 as much as possible. We are not clear for the motivation of the last FFS, we prefer to delete the last FFS. |
| CMCC | OK with this proposal |
| Spreadtrum | We prefer to reuse the Rel-15 and Rel-16 mechanism as much as possible. |
| Apple | We propose to add another option, indication the repetition number via the MAC CE |

### 2nd round discussion

FL’s Comments

All comments are noted and tried to be reflected in the updated proposal.

To CATT’s comment, I hesitate associating the discussion in this section with SSB. As commented in the first GTW call, scheduling multicast/broadcast associated with SSB should be discussed for IDLE/INACTIVE UEs first. I tend to agree with Intel’s comment to generalize the FFS to be further enhancements for configuration of slot-level repetition.

FL’s Proposal

#### [High priority] Proposal 3-r1:

For slot-level repetition for group-common PDSCH of RRC_CONNECTED UEs, for indicating the repetition number, further down-select among:

- Opt 1: by DCI

- Opt 2: by RRC

- Opt 3: by RRC+DCI.

- FFS details for each option.

- FFS further enhancements for configuration of slot-level repetition

Collect views:

| Company | Comments |
| --- | --- |
| LG | We are fine with Proposal 3-r1. |
| Huawei/HiSilicon | Support |
| CMCC | Fine with this proposal. |
| Nokia, NSB | Option 2 needs clarification as highlighted by LG. |
| Ericsson | OK with the proposal. Opt2 is already well supported by legacy NR unicast, but there may be possible extentions. |
| Qualcomm | We are fine with the proposal. |
| Intel | We are ok with the proposal |
| OPPO | OK |
| Futurewei | OK with the proposal |
| MTK | We are fine with the proposal |
| ZTE | OK |
| vivo | We are fine with the proposal |
| Spreadtrum | We are fine with the proposal |
| Lenovo, Motorola Mobility | We are OK with the proposal. |
| Chengdu TD Tech, TD Tech | The description of each option is too simple to show the impact of each option on the specification and implementation complexity. The following examples are given to show our consideration. (1) ” by DCI format “ means a new DCI format or a new field in the existing DCI format? (2) When or under which scenarios to use “by RRC”? (3) What about MAC CE for the dynamic update of the repetition times of the group common PDSCH? (4) If the scenarios for “ by RRC” and the scenarios for “by DCI” are not same, these two options don’t exclude from each other. |

### 1st Checkpoint

FL’s Comments

After the second round discussion, the proposal seems stable. Proposal 3-r1 with no further update is suggested for the 1st checkpoint.

FL’s Proposal for 1st checkpoint:

#### [High priority] Proposal 3-r1:

For slot-level repetition for group-common PDSCH of RRC_CONNECTED UEs, for indicating the repetition number, further down-select among:

- Opt 1: by DCI

- Opt 2: by RRC

- Opt 3: by RRC+DCI.

- FFS details for each option.

- FFS further enhancements for configuration of slot-level repetition

Collect concerns:

| Company | Comments |
| --- | --- |
| Apple | We like to update the proposal including indicated by MAC CE. As the which DCI format is used by MSB is not clear, DCI or MAC CE can be used to indicate the repetition number dynamically. - Opt 1: by DCI/MAC CE - Opt 2: by RRC - Opt 3: by RRC+DCI/MAC CE. |
| Chengdu TD Tech, TD Tech | We think MAC CE can be considered as a possible method for discussion |
| Ericsson | OK with the proposal. Opt2 is already well supported by legacy NR unicast, but there may be possible extentions. |
| Qualcomm | Fine with the proposal. MAC-CE is not used for unicast PDSCH repetition. It can be included in the last FFS. |
| Nokia, NSB | OK with the proposal |
| Spreadtrum | We are fine with the proposal |

### 3rd round discussion

FL’s Comments

The comment of MAC-CE is reflected in the following updated proposal which should be stable now.

In case of any more concern, you can provide them in the following table.

FL’s Proposal:

#### [High priority] Proposal 3-r2: (PDSCH repetition)

For slot-level repetition for group-common PDSCH of RRC_CONNECTED UEs, for indicating the repetition number, further down-select among:

- Opt 1: by DCI/MAC-CE

- Opt 2: by RRC

- Opt 3: by RRC+DCI/MAC-CE

- FFS details for each option.

- FFS further enhancements for configuration of slot-level repetition

Collect concerns:

| Company | Comments |
| --- | --- |
| Samsung | It’s not clear to say DCI/MAC-CE. Is this meaning the combination of DCI/MAC-CE? Or can be it indicated by either DCI or MAC-CE? MAC-CE should be put as other option, for example, option 4: by MAC-CE, option 5: by RRC + MAC-CE. Since indication method is not critical at this stage, it is preferable to put all FFS details of signling methods instead of make a list of all possible options. |
| LG | On Proposal 3-r2, we think that using dynamic control of repetition number for a retransmission e.g. when some UEs requiring more repetitions in the group already sent ACK to the previous transmission. Thus, we support DCI. However, we are skeptical about MAC-CE. So, you may want to change as follows: #### [High priority] Proposal 3-r2: (PDSCH repetition) For slot-level repetition for group-common PDSCH of RRC_CONNECTED UEs, for indicating the repetition number, further down-select among: - Opt 1: by dynamic control e.g. DCI/MAC-CE - Opt 2: by RRC - Opt 3: by RRC+DCI/MAC-CE - FFS details for each option. - FFS further enhancements for configuration of slot-level repetition |
| vivo | We prefer to reuse PDSCH repetition mechanism in NR R15/R16 as much as possible. We are not clear for the motivation to use MAC CE for indication while it will bring additional spec efforts, we prefer to 3-r1 version. |
| Nokia/NSB | Similar comment to Samsung, what DCI/MAC-CE could mean, needs to be clarified. Does “DCI/MAC-CE” mean “DCI and/or MAC-CE”? |
| Intel | Similar comment as in Proposal 2.5-r2. We prefer to have FFS on MAC-CE |

# CSI feedback

Background

The last meeting agreed the following:

Agreements:

- For RRC_CONNECTED UEs, existing CSI feedback can be used for multicast transmission.

  - FFS: whether enhancement is needed

Submitted Proposals

(Huawei) Proposal 7:

  - The PDSCH repetition mechanism and CSI feedback mechanism used in NR unicast can be applied to NR MBS without any enhancements.

(CATT) Proposal 1:

  - CSI measurement/ report, if applied, for MBS can reuse the Rel-15 mechanism in unicast. CSI report for unicast and MBS is not differentiated by UE.

(CMCC) Proposal 8:

  - CSI feedback mechanism in Rel-15/16 can be used for NR MBS, and no additional enhancements are needed.

(ZTE) Proposal 10:

  - Further study whether enhancement is needed for existing CSI reporting mechanism and reporting quantity so that network can choose the most appropriate PMI/RI for multicast transmission that is targeting a group of UEs.

(Nokia) Proposal 1:

  - In case NACK-only based HARQ feedback on group-common PUCCH resources is used as the feedback mechanism of multicast transmission, CSI reporting by the UEs is used to perform link adaptation.

(Nokia) Proposal 2:

  - When using NACK-only based HARQ feedback along with CSI reporting, CQI measurements are done based on actual (time-averaged) BLER measurements at the UEs, rather than (instantaneous) CSI-RS measurements.

(Nokia) Proposal 3:

  - New compact CSI report formats are defined for multicast transmission, where only a CQI or CQI along with an RI can be reported, and these formats are used in CSI reporting when NACK-only based HARQ feedback on group-common PUCCH resources is used.

(Nokia) Proposal 6:

  - For PTM, an rBLER target is used, instead of an iBLER target.

(Nokia) Proposal 7:

  - The configuration for CQI reporting for PTM is extended to include not only the reliability target but also the number of HARQ transmissions per transport block after which the reliability target should be met.

(Intel) Proposal 6:

  - No further enhancements to NR CSI feedback mechanism is needed for NR MBS

(Qualcomm) Proposal 6:

  - For RRC_CONNNECTED UES, configure the CSI-RS resource per Multicast BWP

    - CSI-RS bandwidth is limited within the Multicast BWP.

    - CSI-RS power is associated with GC-PDSCH power

(Qualcomm) Proposal 7:

  - Support GC-PDCCH to trigger A-CSI-RS transmission in Multicast BWP.

(Ericsson)	Proposal 6:

  - The existing Rel. 15/16 framework of CSI feedback is reused for multicast/PTM with no further additions.

### 1st round discussion

FL’s Comments

8 companies submitted proposals regarding the CSI feedback. 5 companies propose CSI measurement and report for unicast is sufficient so no further enhancement is needed. 1 company proposes FFS whether enhancement is needed. 1 company proposed new CSI report formats and new CQI measurement for NACK-only based HARQ-ACK feedback. 1 company proposed some enhancements related to CSI-RS configuration, A-CSI-RS transmission triggering, and SRS configuration.

Given this situation, FL suggests either concluding no enhancement or FFS. Maybe more discussion or clarification is needed since the enhancement is firstly proposed, FL proposes FFS for the 1st round discussion.

FL’s Proposal

#### Proposal 4:

FFS whether CSI feedback enhancement is needed for MBS, including but not limited:

- New CQI measurement

- New CSI report formats

- Targeted BLER

- CSI-RS configuration

- A-CSI-RS transmission triggering

Collect views:

| Company | Comments |
| --- | --- |
| LG | We think that CSI measurement and report for unicast is sufficient so no further enhancement is needed. |
| Huawei/HiSilicon | Ok with the proposal. CSI feedback for unicast might be sufficient to serve both unicast and MBS when UE report CSI for unicast anyway, so no enhancement is needed in our opinion. |
| OPPO | OK |
| MTK | Fine with the proposal. |
| Lenovo, Motorola Mobility | No further enhancement is needed. |
| CATT | CSI feedback can be supported in MBS, but enhancement is not necessary. - We do NOT support “new CQI measurement/CSI report format/target BLER”. - We think the current CSI-RS configuration framework can be reused. - For A-CSI-RS transmission triggering, it is supported in unicast. For MBS, PDCCH scrambled with G-NRTI can be also supported containing A-CSI-RS to trigger CSI feedback of MBS. |
| Nokia, NSB | OK If the 2-1 proposal is supported and includes NACK only feedback, then we strongly support this proposal since we show in [13] clear benefits in to supporting optimized CSI feedback for NACK only based group feedback. |
| Convida | We support to limit the scope of enhancement for CSI feedback for MBS in Rel-17 considering the tight schedule and timeline. We suggest not to support new CSI feedback mechanism unless clear and significant gain can be observed. |
| Intel | Since this entire proposal is FFS, we can postpone discussion to future meetings without limiting the scope. In our understanding current unicast CSI is a good baseline. |
| Futurewei | No further CSI feedback enhancement is needed. |
| Qualcomm | We think it is necessary to list up the potential enhancements for further study. Also, we suggest to also add: FFS SRS configuration. For CATT’s comment 3), we appreciate your support. To our knowledge, the aperiodic NZP-CSI-RS can only be triggered by UL unicast DCI. The A-CSI-RS triggered by GC-PDCCH is one of the enhancements for multicast. |
| ZTE | We are fine with the above proposal. |
| vivo | Technically, we think (Nokia) Proposal 2 is reasonable. For group-common PDSCH scheduling, it is not possible for gNB to make link adaption based on an individual UE’s CQI which is affected by instantaneous interference fluctuations, the current CQI based on CSI-RS measurements is redundant for gNB. So, it is better to report CQI measurements based on actual (time-averaged) BLER measurements at the UEs. |
| CMCC | No enhancement is needed. The CSI measurement and report framework designed in Rel-15/16 is flexible enough, and can be directly used for NR MBS for RRC_CONNECTED UEs, e.g., network can configure a proper CSI measurement and report configuration for UE to report the CSI for MBS transmission. Currently, we have not identified any issues which needs further enhancement for CSI feedback. |
| Spreadtrum | No further CSI feedback enhancement is needed. |
| Apple | We are fine with this proposal. |

### 2nd round discussion

FL’s Comments

All comments are noted. So far the update is only adding “SRS configuration” to the list.

Since still the majority views no enhancement is needed, I made it medium priority at this stage.

FL’s Proposal

#### [Medium priority] Proposal 4-r1:

FFS whether CSI feedback enhancement is needed for MBS, including but not limited:

- New CQI measurement

- New CSI report formats

- Targeted BLER

- CSI-RS configuration

- A-CSI-RS transmission triggering

- SRS configuration

Collect views:

| Company | Comments |
| --- | --- |
| LG | We think that no further enhancement is needed. |
| CMCC | No further enhancement is needed. |
| Nokia, NSB | We strongly support this proposal. We ask companies who are against this proposal to either present technical reasons/evidence why they are against this proposal and/or question the extensive results we present in our contribution. In our contribution [13], we propose that the CQI measurements are not done based on any (instantaneous) CSI-RS measurements, but rather be based on actual (time-averaged) BLER measurements at the UE, since CSI-RS measurements can be affected by instantaneous interference fluctuations based on which the gNB should not adapt its multicast transmission. Our results indicate that it is better to only adapt the MCS slowly and rely on HARQ retransmissions to cater for fast fading fluctuations. This is especially necessary when using NACK-only mode group-common HARQ-ACK feedback, where the gNB cannot estimate BLER of the UEs due to lack of UE-specific resources. From the analysis we presented in [13], we observed that with 100-500ms CSI reporting periods, the system performance is the almost the same as the equivalent UE-specific ACK-NACK configuration in terms of SE and PLR, but with a fraction of the UL OH. Moreover, having a CSI reporting period of hundreds of milliseconds is another indicator why instantaneous fluctuation of CSI-RS measurements need not to be tracked. |
| Ericsson | Support |
| Qualcomm | We are fine with the proposal. |
| Futurewei | No further CSI feedback enhancement is needed. |
| MTK | We are fine with the proposal |
| ZTE | OK |
| vivo | We are fine with the proposal |
| Spreadtrum | No further enhancement is needed. |
| Lenovo, Motorola Mobility | No further enhancement is needed. |
| Chengdu TD Tech, TD Tech | The further discussion on the detailed method of each option is needed before the conclusions or selection can be derived. |

### 1st Checkpoint

FL’s Comments

Companies further expressed whether further enhancement is needed in the second round discussed. As the proposal itself, it seems stable so is suggested for 1st checkpoint.

FL’s Proposal for 1st checkpoint:

#### [Medium priority] Proposal 4-r1:

FFS whether CSI feedback enhancement is needed for MBS, including but not limited:

- New CQI measurement

- New CSI report formats

- Targeted BLER

- CSI-RS configuration

- A-CSI-RS transmission triggering

- SRS configuration

Collect concerns:

| Company | Comments |
| --- | --- |
| Apple | We are ok with this proposal. |
| LG | No further enhancement is needed. |
| Convida | No further enhancement is needed. |
| Ericsson | Support |
| Qualcomm | We support the proposal. It is to clarify the study scope for productive discussion in next meeting. |
| Nokia, NSB | Support |
| Spreadtrum | No further enhancement is needed. |
| CATT | We are OK with this proposal. |

# Other miscellaneous proposals

Submitted Proposals

PUCCH format

(CATT) Proposal 2:

  - Different PUCCH format can be configured by gNB to adapt different coverage requirement.

Impact on DCI contents

(Samsung) Proposal 4:

  - The DCI format scheduling MBS PDSCH reception does not include a TPC command field.

Configuration perspective

(Samsung) Proposal 5:

  - A gNB can provide separate configurations of open-loop power control parameters for PUCCH transmission with unicast HARQ-ACK and for PUCCH transmission with MBS HARQ-ACK.

HARQ process management

(Google) Proposal 3:

  - Clarify the HARQ process assignment (e.g. HARQ ID) when a UE indicates parallel MBS services with HARQ in the MBS interest indication message.

(Intel) Proposal 3:

  - For NR MBS, no additional HARQ processes are defined and MBS shares HARQ process ID with unicast i.e., the total of 16 HARQ processes is unchanged.

Other techniques for reliability

(Qualcomm) Proposal 8:

  - Support beam management for multicast assisted by unicast connection.

(Qualcomm) Proposal 9:

  - Consider SRS configuration for CSI measurement of multicast transmission in Multicast BWP.

Proposals may be in RAN2’s charge

(CATT) Proposal 4:

  - A UE can only feedback HARQ-ACK information according to its interested/received MBS services.

Proposals in AI 8.12.1/8.12.3 charge

(LG) Proposal 2:

  - Allow gNB to use UE specific scheduling of a MBS TB by CSS Type 3 or USS with C-RNTI in UE’s active BWP.

(ZTE) Proposal 9:

  - Slot-level repetition of group-common PDSCH is supported for UEs in RRC_IDLE/INACTIVE states.

FL’s Comments

There are some other miscellaneous proposals proposed but from a single or a couple of companies. FL suggests discussing them later when more companies are interested in or can be discussed in other above issues, other agendas, or other WGs when applicable.

# Summary

## Proposals for GTW on Nov. 10

The following proposals are suggested for GTW treat with or without update based on the “concerns” collected in the section of “3rd round discussion” of the relevant issue.

#### [High priority] Proposal 2.1-r4: (HARQ-ACK option)

For RRC_CONNECTED UEs receiving multicast, at least for PTM scheme 1

- Support ACK/NACK based HARQ-ACK feedback for multicast [if PTP can be used for retransmission],

  - From per UE perspective, UE feedback ACK or NACK.

  - From UEs within the group perspective,

    - FFS: PUCCH resource configuration for ACK/NACK feedback e.g., shared or separate PUCCH resources.

[support: CATT, Apple, MTK, Lenovo, Motorola Mobility, LG, Convida, Samsung, Ericsson, Qualcomm, Nokia, NSB, Huawei, HiSilicon, Spreadtrum]

[Object: CMCC, Vivo]

- Support NACK-only based HARQ-ACK feedback for multicast,

  - From per UE perspective, UE only feedback NACK.

  - From UEs within the group perspective, further down-select between:

    - FFS: PUCCH resource configuration for NACK only feedback.

[support: CATT, Apple, MTK, Lenovo, Motorola Mobility, LG, Ericsson, Qualcomm, Nokia, NSB, Huawei, HiSilicon, Vivo, CMCC, Spreadtrum]

[Object: Samsung, Convida]

- FFS: configuration/selection of ACK/NACK-based and NACK-only based HARQ-ACK feedback

#### [High priority] Proposal 2.2-1-r3: (ACK/NACK based)

For RRC_CONNECTED UEs receiving multicast, for ACK/NACK based HARQ-ACK feedback if supported for group-common PDCCH scheduling, PUCCH resource configuration for HARQ-ACK feedback from per UE perspective is, down-select one of the following options:

- Option 1: shared with PUCCH resource for HARQ-ACK feedback for unicast

- Option 2: separate from PUCCH resource for HARQ-ACK feedback for unicast

- Option 3: Option 1 or option 2 based on configuration

#### [High priority] Proposal 2.2-2-r3: (NACK-only based)

For RRC_CONNECTED UEs receiving multicast, for NACK-only based HARQ-ACK feedback if supported for group-common PDCCH scheduling, PUCCH resource configuration for HARQ-ACK feedback from per UE perspective is separate from PUCCH resource for HARQ-ACK feedback for unicast.

- FFS PUCCH format

#### [High priority] Proposal 2.5-r3: (enable/disable HARQ-ACK)

Enabling/disabling HARQ-ACK feedback for MBS is supported, further down-select between:

- Option 1: DCI

- Option 2: RRC configures enabling/disabling

- Option 3: RRC configures the enabling/ disabling function and DCI indicates enabling /disabling

- [FFS]Option 4: MAC-CE indicates enabling/disabling

- [FFS]Option 5: RRC configures the enabling/ disabling function and MAC-CE indicates enabling /disabling

#### [High priority] Proposal 3-r3: (PDSCH repetition)

For slot-level repetition for group-common PDSCH of RRC_CONNECTED UEs, for indicating the repetition number, further down-select among:

- Opt 1: by DCI

- Opt 2: by RRC

- Opt 3: by RRC+DCI

- [FFS]Opt 4: by MAC-CE

- [FFS]Opt 5: by RRC+MAC-CE

- FFS details for each option.

- FFS further enhancements for configuration of slot-level repetition

#### [Medium priority] Proposal 2.6-r4: (Retransmission)

From the perspective of RRC_CONNECTED UEs receiving multicast, at least for PTM scheme 1 initial transmission, retransmission supports, for the purpose of down-selection, options are:

- Option 1: group-common PDCCH scheduled group-common PDSCH

- Option 2: UE-specific PDCCH scheduled PDSCH

  - Alt 1: PDSCH is UE-specific PDSCH

  - Alt 2: PDSCH is group-common PDSCH

- Option 3: both option 1 and option 2

- FFS other options

- FFS CBG based retransmission

# References

R1-2007557	Improving reliability for MC/BC services	FUTUREWEI

R1-2007563	Mechanisms to improve reliability for RRC_CONNECTED UEs	Huawei, HiSilicon

R1-2007638	Study on the reliability for RRC_CONNNECTED UEs	CHENGDU TD TECH LTD.

R1-2007692	Discussion on mechanisms to improve reliability for RRC_CONNECTED UEs	vivo

R1-2007836	Discussion on reliability improvement mechanism for RRC_CONNECTED UEs in MBS	CATT

R1-2008035	Discussion on reliability improvement	CMCC

R1-2008065	Mechanisms to improve reliability of Broadcast/Multicast service	LG Electronics

R1-2008193	On mechanisms to improve reliability for RRC_CONNECTED UEs	Samsung

R1-2008243	UL feedback for RRC-CONNECTED UEs in MBMS	OPPO

R1-2008450	Discussion on MBS reliability improvement for RRC_connected UEs	Apple

R1-2008715	Reliability improvement for RRC_CONNECTED UEs in MBS	Potevio

R1-2008827	Mechanisms to Improve Reliability for RRC_CONNECTED UEs	ZTE

R1-2008883	Reliability Improvements for RRC_CONNECTED UEs	Nokia, Nokia Shanghai Bell

R1-2008893	Views on improving reliability for RRC_CONNECTED UEs in MBS	Google Inc.

R1-2008927	Discussion on reliability improvement for RRC-CONNECTED UEs	Lenovo, Motorola Mobility

R1-2008962	Discussion on HARQ operation for NR MBS reliable transmission	MediaTek Inc.

R1-2009001	Mechanisms to Improve Reliability for NR-MBS	Intel Corporation

R1-2009166	On reliability enhancement for NR multicast and broadcast	Convida Wireless

R1-2009275	Views on reliability enhancement for Multicast RRC_CONNECTED UEs	Qualcomm Incorporated

R1-2009306	Discussion on reliability mechanisms for NR MBS	Ericsson