3GPP TSG RAN WG1 #103e	R1- 2009770

October 26th – November 13th, 2020

Agenda item:	7.2.5

Source: 	Moderator (Qualcomm)

Title: 	Summary #2 of the Remaining Issues on HARQ and Scheduling Enhancements for URLLC

Document for:	Discussion and Decision

# 1	Introduction

Based on the discussions during the preparation phase, it is agreed to discuss the following topics during the RAN1 #103e:

103-e-NR-L1enh-URLLC-04] Email discussion/approval on remaining issues on scheduling/HARQ enhancements – Kianoush (Qualcomm)

- Issue 4: Intra-UE prioritization and multiplexing order

- Issue 5: Order of multiplexing and prioritization due to conflicts with semi-static DL and SSB symbols

# 2         Issue #4

## 2.1     Unaligned Timeline for Cancellation and Overriding

In [6], some changes to the previous working assumption are proposed. In particular, it is mentioned that:

“Processing time of cancellation and overriding are different and processing time of overriding, i.e. N3 is shorter than processing time of cancellation, i.e. Tproc,2+d, as shown in Figure 2. So when both cancellation and overriding occur, to ensure enough time to perform cancellation and overriding, UE determines and generates final uplink channel at the deadline of cancellation. However, if there is a DCI indicating a new PUCCH, which overrides the cancelled PUCCH, UE has to look back cancelled PUCCH, it increases UE complexity significantly. To avoid UE complexity, overriding should be avoided after deadline of cancellation.”

It is further mentioned that a similar problem happens for multiplexing and overriding in Rel-15. The processing time for overriding, i.e. N3 is shorter than processing time of multiplexing, i.e. Tproc,2. Hence, to ensure enough time to perform both multiplexing and overriding and avoid UE complexity, there is a restriction in the specification that overriding does not occur after deadline of multiplexing as copied below:

“A UE does not expect to detect a DCI format scheduling a PDSCH reception or a SPS PDSCH release, or a DCI format including a One-shot HARQ-ACK request field with value 1, and indicating a resource for a PUCCH transmission with corresponding HARQ-ACK information in a slot if the UE previously detects a DCI format scheduling a PUSCH transmission in the slot and if the UE multiplexes HARQ-ACK information in the PUSCH transmission.”

To address the issue, the following TP is proposed:

--------------------------------------- Start of TP 38.213V16.3.0 section 9------------------------------------- <unchanged text omitted> A UE does not expect to detect a DCI format scheduling a PDSCH reception or a SPS PDSCH release, or a DCI format including a One-shot HARQ-ACK request field with value 1, and indicating a resource for a PUCCH transmission with corresponding HARQ-ACK information in a slot if the UE previously detects a DCI format scheduling a PUSCH transmission in the slot and if the UE multiplexes HARQ-ACK information in the PUSCH transmission, or if the UE previously detects a DCI format scheduling a PUCCH transmission for HARQ-ACK information in the slot and if the UE cancels the HARQ-ACK information in the PUCCH transmission. <unchanged text omitted> ------------------------------------------ End of TP 38.213V16.3.0 section 9-----------------------------------

Please share your views on the proposed TP in the table below.

| Company | Comment |
| --- | --- |
| OPPO | The intention of proposal is to reuse Rel-15 solution on unaligned processing time for multiplexing and overriding to solve unaligned processing time for prioritization and overriding in Rel-16. The benefit is low UE complexity. One implementation is to perform overall multiplexing/overriding/prioritization last moment, shown in the following figure, and the proposal could avoid look-back operation or double processors for HP and LP. In Rel-15, whatever multiple uplink channels multiplexing or mix of multiplexing and overriding, overall multiplexing/overriding can be allowed from perspective of spec, and it is also an easy and efficient implementation. Similarly, overall prioritization/overriding should be a baseline to avoid additional complexity. |
| Nokia, NSB | In our view, the current spec seems to be clear, and thus no spec change is needed. |
| HW/HiSI | According to our understanding, the WA should be confirmed as it is, there is no need for this TP. This issue was discussed last meeting. The N3 is shorter than the cancellation time-line. Step 1 of the WA assumption implies that overriding is performed similar to R15 as if HP channels do not exist. UCI is handled separately from the PUCCH. When the PUCCH is cancelled the UCI can still be transmitted if the final resource according to the N3 deadline is determined. |
| DOCOMO | It seems the following agreement at RAN1#101-e meeting is related to the issue. By following the agreement, UE is not expected to transmit the 3rd LP PUCCH in OPPO’s figure. Agreement If a UE is expected to cancel a scheduled low priority PUCCH/PUSCH due to a first DCI scheduling an overlapping high priority channel, the UE is not expected to transmit the scheduled low priority PUCCH/PUSCH due to a second DCI scheduling UCCH/PUSCH that is received after the first DCI. Note: The collision between HP PUSCH and LP PUSCH is not covered by this agreement. |
| ZTE | Share the same view with DOCOMO. The case mentioned by OPPO would not happen. |
| Samsung | Support the TP in principle. Oppo’s proposal is actually according to the agreement cited above by Docomo. |
| Ericsson | We are not sure if TP is needed and if it is needed, whether the proposed TP served the purpose. In our understanding, when HARQ-ACK is multiplexed on PUSCH, the HARQ was supposed to be transmitted in PUCCH and then due to scheduling a PUSCH, is multiple on PUSCH. That means the corresponding codebook is constructed and it is not updating, or in other words DCI 4 does not come after UL grant. |
| vivo | We suggest to confirm WA. If an intermediate LP PUCCH is canceled, the corresponding UCI can still be transmitted in another LP PUCCH or LP PUSCH following the timeline of PUCCH resource update, or UCI multiplexing on PUSCH. There is no need for this TP. |
| CATT | We share the same understanding as vivo. We do not think the TP is needed. |
| Intel | We agree with others above that the WA can be confirmed. This case seems covered by the spec text we already have in 213 corresponding to the agreement cited by DCM. The only difference we see is that the proposal from Oppo seems to be a stronger variant prohibiting DCI 4 (in the example from Oppo) from being transmitted to the UE, while the current spec only says that the UE is not expected to take DCI 4 into account to re-evaluate the prioritization. However, in practice there should not be a difference since, with current specs, gNB should not send the DCI 4 to override the (1st and) 2nd LP PUCCHs. |
| Apple | We support the intention of the TP. Regarding the agreement quoted by DCM and the corresponding spec text, if it is the common understanding that the sentence prevents DCI 4 from happening, we would be fine with no TP. (Note that such a case was not explicitly discussed when we made the agreement. The agreement was made to address the case where there may be a later HP DCI that changes the overlapping situation.) From companies’ comments, it seems that there are two different understandings. Some companies think the spec already prevents DCI 4 from happening. But some other companies think DCI 4 can happen, and the 3rd LP PUCCH should be transmitted. The different understanding should be addressed. |
| Ericsson | We understand OPPO’s concern that perhaps spec allows the behavior as illustrated in the figure, but from our point of view, this is a strange behavior from the network. Why the network has to send DCI 4 after DCI 3? Or why not using PUCCH3 from the beginning? BUT, if the intention is to disallow such behavior which is also in line with the spirit of the agreement, we would be fine. But the TP has to be updated. Currently, the spec specifies cancellation of a transmission (i.e. PUCCH), not the content. Hence, it wount be clear to the reader what it is meant. |
| ZTE | After the discussion, I think we align on this issue. |
| DOCOMO2 | The following is copied from our email past: Regarding "Unaligned timeline for cancellation and overriding," we agree that the 3rd PUCCH shown in the figure presented by OPPO CAN be scheduled. We checked the spec carefully and your understanding is correct. Thanks a lot Kianoush and all for sharing your views. |

### 2.1.1   Summary of the Discussions

So far, the status of the discussions is as follows:

- 78 companies (Nokia/NSB, HW/HiSi, DCM, ZTE, Ericsson, vivo, CATT and Intel) think that no change is needed.

- 3 companies (OPPO, Samsung and Apple) proposed to clarify the UE behaviour.

Further, 3 companies (DCM, ZTE, Intel) mentioned that the 3rd PUCCH in the figure presented by OPPO above will not be sent by the UE. From the feature lead point of view, that is not the case. A UE can still be scheduled with the 3rd PUCCH, and the 2nd PUCCH, even though it is cancelled, can be overridden by the 3rd grant as long as the PUCCH overriding timeline is satisfied.

## 2.2     Order of Multiplexing and Prioritization

In [8], a change for the following part of the intra-UE cancellation text from Section 9 of TS 38.213 is proposed:

“where

-	the overlapping is applicable before or after resolving overlapping among channels of larger priority index, if any, as described in Clause 9.2.5

-	the UE expects that the transmission of the first PUCCH or the first PUSCH, respectively, would not start before Tproc,2+d1 after a last symbol of the corresponding PDCCH reception”

The reason for change is as follows:

“The intention of the description “before or after” here means that the prioritization of different priority UL channels is conducted before the multiplexing of overlapping HP UL channels in Clause 9.2.5 of TS 38.213 if any, and after the multiplexing of overlapping HP UL channels in the Clause 9.2.5 if any when the outcome of the multiplexing would overlap with the LP channel. Although this description comes from the email discussion “[102-e-NR-Editor-NR_L1enh_URLLC-38.213],” we think it is still misleading. For example, it could be interpreted that “before or after” means UE conducts the prioritization of different priority UL channels only before the multiplexing of overlapping HP UL channels or only after the multiplexing of overlapping HP UL channels. There is ambiguity of UE behavior with the interpretation. Therefore, the following TP is proposed to avoid future misunderstanding of the spec.”

---------------------------------Start of Text Proposal on TS 38.213 v16.3.0----------------------- #### 9 UE procedure for reporting control information <Unchanged parts are omitted> When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes, the UE first resolves the overlapping for PUCCH and/or PUSCH transmissions of smaller priority index as described in Clause 9.2.5. Then, - if a transmission of a first PUCCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a transmission of a second PUSCH or a second PUCCH of smaller priority index, the UE cancels the transmission of the second PUSCH or the second PUCCH before the first symbol that would overlap with the first PUCCH transmission - if a transmission of a first PUSCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a transmission of a second PUCCH of smaller priority index, the UE cancels the transmission of the second PUCCH before the first symbol that would overlap with the first PUSCH transmission where - the overlapping is applicable before resolving overlapping among channels of larger priority index, if any, as described in Clause 9.2.5 - the overlapping is applicable after resolving overlapping among channels of larger priority index, if any, as described in Clause 9.2.5 - the UE expects that the transmission of the first PUCCH or the first PUSCH, respectively, would not start before Tproc,2+d1 after a last symbol of the corresponding PDCCH reception - Tproc,2 is the PUSCH preparation time for a corresponding UE processing capability assuming d2,1=0 [6, TS 38.214], based on μ and N2 as subsequently defined in this Clause, and d1 is determined by a reported UE capability <Unchanged parts are omitted> --------------------------------------End of Text Proposal on TS 38.213 v16.3.0------------------

In [10], it is mentioned that the highlighted part in following text related to intra-UE prioritization and multiplexing is not clear:

When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes, the UE first resolves the overlapping for PUCCH and/or PUSCH transmissions of smaller priority index as described in Clause 9.2.5. Then, - if a transmission of a first PUCCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a transmission of a second PUSCH or a second PUCCH of smaller priority index, the UE cancels the transmission of the second PUSCH or the second PUCCH before the first symbol that would overlap with the first PUCCH transmission - if a transmission of a first PUSCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a transmission of a second PUCCH of smaller priority index, the UE cancels the transmission of the second PUCCH before the first symbol that would overlap with the first PUSCH transmission where - the overlapping is applicable before or after resolving overlapping among channels of larger priority index, if any, as described in Clause 9.2.5 - the UE expects that the transmission of the first PUCCH or the first PUSCH, respectively, would not start before Tproc,2+d1 after a last symbol of the corresponding PDCCH reception - Tproc,2 is the PUSCH preparation time for a corresponding UE processing capability assuming d2,1=0 [6, TS 38.214], based on μ and N2 as subsequently defined in this Clause, and d1 is determined by a reported UE capability

In particular, it is mentioned that “it is not clear when the UE should perform the cancellation of low-priority channel due to the reception of a PDCCH scheduling a high-priority channel that overlaps with the low-priority channel, i.e. whether the UE should cancel the low-priority channel before or after performing high-priority channels multiplexing triggered by the (newly) scheduled high-priority channel, if any. From the Working Assumption, it is s clear that the UE should perform high-priority channels multiplexing triggered by the newly scheduled high-priority channel, if any, after cancelling the transmission of low-priority channel.”

To clarify, the following TP is proposed:

# UE procedure for reporting control information <omitted text> When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes, the UE first resolves the overlapping for PUCCH and/or PUSCH transmissions of smaller priority index as described in Clause 9.2.5. Then, - if a transmission of a first PUCCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a transmission of a second PUSCH or a second PUCCH of smaller priority index, the UE cancels the transmission of the second PUSCH or the second PUCCH before the first symbol that would overlap with the first PUCCH transmission - if a transmission of a first PUSCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a transmission of a second PUCCH of smaller priority index, the UE cancels the transmission of the second PUCCH before the first symbol that would overlap with the first PUSCH transmission where - the overlapping is applicable before or and after resolving overlapping among channels of larger priority index, if any, triggered by the first PUCCH or the first PUSCH scheduled by a DCI format in a PDCCH reception, as described in Clause 9.2.5 - the UE expects that the transmission of the first PUCCH or the first PUSCH, respectively, would not start before Tproc,2+d1 after a last symbol of the corresponding PDCCH reception - Tproc,2 is the PUSCH preparation time for a corresponding UE processing capability assuming d2,1=0 [6, TS 38.214], based on μ and N2 as subsequently defined in this Clause, and d1 is determined by a reported UE capability <omitted text>

Is there any ambiguity in the following clause of the specification?

“the overlapping is applicable before or after resolving overlapping among channels of larger priority index, if any, as described in Clause 9.2.5”

Please share your views in the table below. If the answer is yes, please share your proposals considering the TPs copied in this section from [8] and [10].

| Company | Comment |
| --- | --- |
| Nokia, NSB | Yes. The current specs may create ambiguity regarding when the UE should perform the cancellation of low-priority channel due to the reception of a PDCCH scheduling a high-priority channel that overlaps with the low-priority channel, e.g. whether the UE should cancel the low-priority channel before, after or before&after performing high-priority channels multiplexing, if any. Based on the WA, the UE should cancel the low-priority channel before high-priority channels multiplexing. In addition, from the WA, if the resulting high-priority channel overlaps with a low-priority channel, the UE should cancel this low-priority channel as well (i.e. after). Therefore, we think the UE would need to do it before AND after as described here. Our proposed TP in [10] helps clarifying the above aspects. |
| HW/HiSi | According to our understanding, this issue was discussed and concluded during the CR phase during last meeting. |
| DOCOMO | Yes and we are fine with either of the TPs as long as there is no ambiguity on UE behavior. We acknowledge that this issue was discussed and concluded during the CR phase at the last meeting. However, as discussed in our contribution [6], the current description is misreading for someone who has not been involved this topic. In order to future misunderstanding, we think it is better to update the description. |
| ZTE | The intention of the proposals from DOCOMO and Nokia is clear and reasonable. I slightly prefer the TP from DOCOMO. |
| Samsung | Do not agree with the proposed CRs. There is no ambiguity to “A or B” – it means “A or B”. For the CR from [8], it is unclear whether the new/second sub-bullet is connected with an ‘and’ or with an ‘or’ from the original/first sub-bullet. A connection with ‘and’ (that is probably intended) would not be correct. A connection with ‘or’ produces identical text as the current one. For the CR from [10], ‘and’ is not correct. |
| Ericsson | We discussed “before or after” during the CR review and if there is a risk to interpret “either before or after”. Based on clarifications, we got convinced that there is no ambiguity. In the sense that it is not up to UE to determine overlapping is either before or after. The UE checks before and if applicable, after. |
| vivo | We agree with views from Samsung. Current description seems clear. |
| Spreadtrum | We are fine to keep the current description. |
| CATT | There can be potential misunderstanding in our view so we prefer to clarify in the spec. |
| Intel | Do not see a need to change current description. |
| Apple | We think the current text can be ambiguous for anyone who has not been involved in the discussion. When there are obvious different interpretations on “or” vs “and”, we have to admit there is ambiguity. To avoid issues in the future, we prefer to clarify. We do not feel the TP from either [8] or [10] is crystal clear. The phrase that seems a bit difficult to interpret is “the overlapping is applicable”. An alternative suggestion is: “- the UE performs the procedure if there is overlapping before resolving overlapping among channels of larger priority index, if any, as described in Clause 9.2.5 - the UE performs the procedure if there is overlapping after resolving overlapping among channels of larger priority index, if any, as described in Clause 9.2.5.” “and” between the two bullets does not seem necessary. In short, we are open to consider any wording that improve clarity. |
| DOCOMO2 | As long as at least 5 companies consider there can be ambiguity, we think it is better to make the description clearer although we agree that it is already clear enough for anyone who has been involved in the discussion. For the modification, we support the description proposed by Apple to refine “the overlapping is applicable.” In response to Samsung’s comment, the intention is “and” but it is not needed because the existing other sub-bullet related to timeline also implies a connection with “and” without anything. Regarding whether “and” is correct or not, we think it should be correct because there is “if any.” In short, the description can be interpreted as “’A’ happens if any and ‘B’ happens if any” and it captures the following 4 cases: 1) If the condition for “A” is met, but those for “B” is not met => only “A” is performed. 2) If the condition for “A” is not met, but those for “B” is met => only “B” is performed. 3) If the conditions for both “A” and “B” are met => both “A” and “B” are performed 4) If none of the conditions is met => both “A” and “B” are not performed Therefore, we think the description suggested by Apple should be adopted. |
| Ericsson | Our main priority is that not to have any misunderstanding when it comes to implementation since it would cause mis-alignment between UE and gNB. In fact, that was our main concern for all the discussion in the last meeting. If there are vendors that still find the text ambiguous, we would be definitely fine to make it clear. Because it is not important if the text is clear for NW vendor. We would rather all the UE vendors have the same understanding. Therefore, as long as there is misunderstanding, which seems to be from the discussion to be the case, we would prefer to update the text to avoid that. Apple TP seems to be fine and resolve the potential ambiguity. |

### 2.2.1   Summary of the Discussions

So far, the status of the discussions is as follows:

- 6 companies (Nokia/NSB, DCM, ZTE, CATT and Apple and Ericsson) think that the current specification is not clear.

- 5 companies (HW/HiSi, Samsung, Ericsson, vivo, Spreadtrum and Intel) mentioned that no specification update is needed.

Feature lead comment: From the discussions during the CR phase, it is understood that a UE checks the overlap between a HP channel and a low priority channel before multiplexing. If there is an overlap, the LP channel gets cancelled. If not, a UE performs multiplexing across the HP channels. If then there is an overlap with a LP channel, the LP channel gets cancelled. It was explained by the spec. editor that “before or after” should be interpreted as explained here. Hence, the feature lead recommendation is to not consider any update to the current specification.

# 3         Issue #5

On handling the collisions with semi-static DL and SSB symbols, the following agreement was reached previously:

Agreement:

After the UE determines the overlapping PUCCH or PUSCH for multiplexing/prioritization, the UE cancels the PUCCH or PUSCH that has overlapping with semi-static configured DL symbols or SSB symbols, and then the multiplexing/prioritization is performed among the non-cancelled overlapping transmissions.

This means that (as RAN1 common understanding):

- Step 1: A UE follows Rel-15 behaviors for any intermediate procedure to determine the overlapping PUCCH or PUSCH for multiplexing/prioritization

- Step 2: UE cancels the ones that collides with semi-static DL symbols,

- Step 3: UE performs multiplexing/prioritization among the non-cancelled overlapping channels.

One issue that was raised before is that what should the UE behavior be if the final channel after step 3 is overlapping with the semi-static DL or SSB symbols.

To address this issue, one of the two ways can be followed:

- Option #1: After step 2, all overlapping PUCCH resources are removed by the UE, i.e., multiplexing operation of step 3 does not assume these resources as available.

- Option #2: If the outcome of step 3 overlaps with semi-static DL/SSB symbols, the transmission is cancelled by the UE.

Question #1: Which of the Option #1 or Option #2 should be adopted? Please share your response in the table below.

| Company | Option #1 or Option #2 | Reason |
| --- | --- | --- |
| OPPO | Option 2 if necessary | 1) Option 1 is not preferred due to it is not clear and it leads additional issue. Does all overlapping PUCCH resource mean all PUCCH resource in PUCCH-Config? It needs new procedure to solve issue from absent multiplexing PUCCH resource. 2) Last meeting, we delay this discussion due to we have diverse understanding on order of multiplexing and cancellation by semi-static DL symbol and SSB. Although it is an agreement, it looks strange that the order of multiplexing and dynamic SFI or DL grant and the order of multiplexing and dynamic SFI or DL grant are different. |
| HW/HiSi |  | Slight preference for Option #2. This seems to be aligned with what has been discussed in earlier meetings, that new resources could be determined during step #3 and that these channels should be removed if they overlap with semi-static DL/SSB. Before giving our support for any of the option 2, we would like to ensure that we really understand Option 1 what additional PUCCH resources can be can be cancelled by Option #1 that have not already been removed by following Step 2? |
| DOCOMO |  | Share similar view with HW/HiSi. Besides, as we discussed in the email, we prefer to clarify the common understanding on the 3 steps. In our understanding, the following Understanding #1’ may be the common understanding based on the discussion. Understanding #1' Step 1: Overlapping PUCCH/PUSCHs are determined (No multiplexing) Step 2: Overlapping PUCCH/PUSCHs collied with semi-static DL symbols are cancelled. (Option #1) After step 2, all overlapping PUCCH resources are removed by the UE, i.e., multiplexing operation of step 3 does not assume these resources as available. Step 3: UE performs multiplexing/prioritization among the non-cancelled overlapping channels. Step 3.1: Remaining PUCCH/PUSCHs of low priority after step 2 are multiplexed Step 3.2: If there is collision between high and low priority PUCCH/PUSCHs, low priority channels are dropped. Step 3.3: Remaining PUCCH/PUSCHs of high priority are multiplexed (Step 3.4: If there is collision between high and low priority PUCCH/PUSCHs, low priority channels are dropped.) Step 3.5 (Option #2): If there is collision between high and/or low priority PUCCH/PUSCHs and semi-static DL symbols, it/they will be dropped. |
| ZTE |  | One question for option 1, regarding “all overlapping PUCCH resources are removed by the UE”, literally, the resources are removed by UE, so how about gNB status? Should gNB indicates only the available resource to UE except the removing resources? From my perspective, gNB could guarantee the resource after step 3 would not overlap with semi-static DL symbols, so neither option1 nor option2 is our choice. |
| Samsung | Option 2 | As phrased, option 1 and option 2 do not appear exclusive. However, we have same understanding as explained by Docomo. |
| Ericsson | Option1/Option 2 | As Samsung mentioned these two options are not exclusive. We also share the same understanding as DCM and as it is well explained, the procedure includes both options. |
| vivo | Option1/Option 2 | In our views, Understanding #1’ is the common understanding. Both option 1 and option 2 should be adopted. |
| Spreadtrum |  | Firstly, the cancelation in step 2 means the required PUCCH/PUSCH resources. If colliding with semi-static DL symbols/SSB, they are canceled. Not all the PUCCH resources in PUCCH-config from our understanding. Secondly, we agree with the procedure suggested by DCM, which the final check is still in step 3, not after step 3. The multiplexing output is not collide with DL/SSB symbols. Thirdly, the SFI/DL grant was agreed to do after multiplexing operation, this should be step 4. |
| CATT |  | We are not clear about the option#1 in step 2 in DOCOMO’s understanding #1’. Does it mean that all the PUCCH resources colliding with semi-static DL symbols are removed? If so, why there can be collision after step 3? We do not agree to remove all the PUCCH resources colliding with semi-static DL symbols in step 2 which is unnecessary in our view. If the resource after multiplexing is dynamically indicated, it should not collide with DL symbols and gNB would just select a proper PRI to indicate a valid PUCCH resource. Removing all the PUCCH resources colliding with semi-static DL symbols in step 2 does not help. If multiple CSI on overlapping PUCCHs are multiplexed on a multi-CSI resource and the multi-CSI PUCCH resource collides with semi-static DL symbol, it is the same as in Rel-15 and the multi-CSI PUCCH is dropped. |
| Intel | Option 2 | Slight preference for Option 2. However, to the description from DCM, if Option #1 is followed (i.e., all PUCCH resources in PUCCH-Config with overlaps with semi-static DL and SSB symbols are removed), then does that not render Step #3.5 redundant? On the other hand, with Option #1, there would be possibilities of the multiplexed UCI not finding a suitable resource as all PUCCH resources from PUCCH-Config with overlap with SSB/semi-static DL symbols have been removed, which seems like an error case that the gNB should then avoid. In this regard, Option 2 seems a bit less restrictive for gNB scheduler, and a proper gNB implementation should as such aim to avoid scenarios when the final PUCCH may be dropped due to overlap with SSB/semi-static DL symbols. |
| Apple |  | It is not clear to us what Option 1 means exactly (either in the original formulation or in DCM’s description). We typically cancel a PUCCH/PUSCH transmission, not removing a PUCCH resource. What does it mean by removing overlapping PUCCH resources from UE behavior perspective? Does it mean the gNB cannot indicate these resources any more? Without further understanding of Option 1, we would think Option 2 is the right way to go. |
| DOCOMO2 | Option 2 | Thanks a lot for the discussion and sorry for the confusion it might lead. Originally, we put Option1 and Option 2 into Understanding #1’ in order to 1) summarize the common understanding on the already agreed steps and, 2) summarize the potential processing order for the two options to be discussed. That is why we put brackets to both Option1 and Option2 there. Based on the discussion so far, the intention of Option1 is not clear. But if it means PUCCH resources are removed and those cannot be used for the following steps, i.e. step 3, it may cause an error case as Intel pointed out. Thus, we prefer Option 2. |

In addition, please share your view on the following behavior in the table below:

“If a group of overlapping channels consists of at least one dynamically granted PUCCH or PUSCH, neither any of the intermediate resources or the final one can overlap with a semi-static DL/SSB symbols.”

| Company | Comment |
| --- | --- |
| HW/HiSi | According to our understanding, final and intermediate granted resources could overlap with semi-static DL/SSB (but should not be transmitted). |
| Samsung | Final and intermediate resources can overlap with DL/SSB (e.g. UE misses some DL DCI and determines wrong number of resources for HARQ-ACK – other cases also exist) |
| Ericsson | We also think that the final and intermediate could overlap w DL/SSB. |
| vivo | We think that the final and intermediate could overlap with semi-static DL/SSB symbols. |
| Spreadtrum | Support - Intermediate resource means PUCCH resource with dynamic HARQ-ACK only. Since other PUCCH resource collide with DL/SSB has been removed in step 2. This PUCCH is indicated by gNB by DCI. We don’t understand why network use a usable resource to transmit. - Similarly, final dynamically PUCCH or PUSCH resources are also scheduled by DCI, if they are collide with DL or SSB, they cannot be transmitted anymore. So it is unreasonable to schedule a wasted resource. |
| CATT | Dynamic scheduled transmission should not conflict with semi-static UL/DL symbols. |
| Intel | Agree with others above that the intermediate/final resources could overlap with SSB/semi-static DL symbols. |
| Apple | Our original understanding is that dynamically scheduled transmission should not conflict with semi-static DL/UL configurations (except for error/corner cases e.g. when some DCIs are missed), but we were not able to find any specification text that explicitly impose the constraint. We do not see any need to allow dynamic transmission to conflict with semi-static configuration. Any reasonable gNB implementation would not do it. |
| DOCOMO2 | Agree with other companies. |

## 3.1     Summary of the Discussions

On the two options presented, the current status is as follows:

- No company is in favor of Option 1.

- 5 companies (OPPO, HW/HiSi, DCM, Intel and Qualcomm) are supporting Option 2.

- 1 company (ZTE) does not support any of the two options.

- 3 companies (Samsung, Ericsson and vivo) proposed that both options should be included in the final solution.

As explained before too, if Option 1 is adopted, then, the final check of overlapping channels is not needed. This is because under Option 1, all the resources that are overlapping with DL symbols and SSB are assumed to be non-existent by a UE.

Given the current situation, the recommendation from the feature lead is to adopt Option 2, but continue the discussion based on the steps summarized by DCM to finalize the proposal. In particular, it is important to decide whether Steps 3.1-3.4 are consistent with our earlier decisions on the order of multiplexing and prioritization.

Question: Can the following steps be considered for handling the collisions with semi-static DL and SSB? Is Step 3 consistent with the earlier decision on ordering of multiplexing and prioritization involving dynamically granted channels?

- Step 1: Overlapping PUCCH/PUSCHs are determined (No multiplexing)

- Step 2: Overlapping PUCCH/PUSCHs collied with semi-static DL symbols are cancelled.

- Step 3: UE performs multiplexing/prioritization among the non-cancelled overlapping channels.

  - Step 3.1: Remaining PUCCH/PUSCHs of low priority after step 2 are multiplexed

  - Step 3.2: If there is collision between high and low priority PUCCH/PUSCHs, low priority channels are dropped.

  - Step 3.3: Remaining PUCCH/PUSCHs of high priority are multiplexed

  - Step 3.4: If there is collision between high and low priority PUCCH/PUSCHs, low priority channels are dropped.

- Step 4 If there is collision between high and/or low priority PUCCH/PUSCHs and semi-static DL symbols, it/they will be dropped.

Please share your responses in the table below.

| Company | Comment |
| --- | --- |
| Nokia, NSB | 1. Question for clarification: Is step 3.2 only applicable to ‘scheduled UL transmission’ (e.g. not for CG-PUSCH) – as this seems to be related to step 2 of the working assumption? If so, maybe worth clarifying… 2. Order of step 3.4 and step 4: We would prefer the 2nd dropping with semi-static symbols in step 4 before the intra-UE prioritization in step 3.4, to not in addition unnecessarily drop LP PUCCH/PUSCH due to HP PUCCH overlapping with DL symbols. i.e. something like: - Step 3: UE performs multiplexing/prioritization among the non-cancelled overlapping channels. - Step 3.1: Remaining PUCCH/PUSCHs of low priority after step 2 are multiplexed - Step 3.2: If there is collision between high and low priority PUCCH/PUSCHs, low priority channels are dropped. - Step 3.3: Remaining PUCCH/PUSCHs of high priority are multiplexed - Step 3.4 If there is collision between high and/or low priority PUCCH/PUSCHs and semi-static DL symbols, it/they will be dropped. - Step 3.45: If there is collision between high and low priority PUCCH/PUSCHs, low priority channels are dropped. - Step 4 If there is collision between high and/or low priority PUCCH/PUSCHs and semi-static DL symbols, it/they will be dropped. |
| DOCOMO | We support the proposed steps by FL and Nokia (slightly preferred), and Step 3 is consistent with the earlier decision. However, it would be better to clarify more about Step 1 to have common understanding. In short, we have same understanding with FL that UE only determines overlapping PUCCH/PUSCHs in Step 1 (i.e. no multiplexing) but we are still trying to understand what exactly UE does in this step. FL kindly provided his understanding and examples in the email thread as follows: For example, if there is a dynamically granted PUCCH which is overlapping with DL symbols/SSB, the UE considers it as an intermediate channel and if it gets a DCI overriding that, it will follow that command until it figures out what the final PUCCH resource is. As another example, if there are two CSI reports in a slot and a UE is configured with multi-CSI PUCCH resource, the UE multiplexes them and considers the multi-CSI resource as the final resource. But, the UE does not perform any other multiplexing at this point. The first example seems valid but the second may be incorrect because the UE behavior on multi-CSI PUCCH resource is written in section 9.2.5, which should be performed in Step 3. At the same time, we have the second thought that it may not need to consider Step1 deeply as the current spec has similar description for the order of multiplexing/prioritization in section 9 as follows: When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes, the UE first resolves the overlapping for PUCCH and/or PUSCH transmissions of smaller priority index as described in Clause 9.2.5. We would like to hear other companies’ views on Step 1 to catch RAN1 common understanding. |
| HW/HiSi | Question for clarification only: Is it common understanding that channels that have not participated in the multiplexing, but which (for some reason) collide with semi static DL/SSB are also canceled in step 4? If yes, do we need to add a note for that or is this already or are we only discussion overlapping channels here and for non-overlapping channels we go directly in Section 11 of the spec? |
| Apple | On the question raised by DCM on step 1, for the two examples provided, my understanding is that HARQ-ACK PUCCH resource overriding is considered as part of step 1, but the resolution of multiple CSIs in a slot belongs to step 3. Basically, anything other than overriding is not considered in step 1. Indeed, this is good to clarify. |
| Ericsson | We prefer the steps proposed by FL. We are not supportive of the changes by Nokia, although doesn’t change the outcome. The reason is that in Step 4, the transmission is compared with TDd pattern. The comparison within the procedure compared the logical PUCCH/PUSCH resources with TDD pattern. Hence, we should resolve overlapping and them compare the outcome with TDD. This is a cleaner approach. |
| ZTE | Although we have different view on this issue, we are fine with FL’s proposal to close the thread soon. For DOCOMO’s concern on multi-CSI PUCCH resource, we also have the same question. For Nokia’s revision, it is not bad to keep LP channel while FL’s version could not keep LP and HP if collision. But the condition of Nokia’s proposal is so rare, so both of the proposals could be accepted. |
| Spreadtrum | Question for step 2, is it handled one time only before step 3, or can be several times when overlapping PUCCH resources changing? If only once, the UCI of the later involved PUCCH resources which collide with DL/SSB still can be multiplexed on PUCCH as shown in the Figure below. However, from our understanding, the PUCCH with CSI1 should be dropped before multiplexing. |
| CATT | We support the proposal from FL. Comparing the proposal from Nokia, we think the original proposal is cleaner and potential benefit from the updated proposal would be quite limited, if any since it helps only when the HP channel overlaps with semi-static DL symbol while the LP channel does not. |
| vivo | We have the same question and same understanding as Spreadtrum. |
| Apple 2 | Related to what is included in step 1, we think HARQ-ACK overriding for LP is included in step 1. However, HARQ-ACK overriding for HP should NOT be included in step 1. The reason is that the overriding timeline (N3) is too short for cancellation, and the UE cannot afford to wait after the overriding timeline if a LP needs to be cancelled. Regarding Nokia’s proposal, we prefer the moderator’s original proposal, with similar reason as Ericsson. Given that any reasonable gNB should avoid the conflict of dynamically scheduled UL resource and semi-static DL symbols, there is no need to over-optimize. We would prefer to keep it relatively cleaner. |
| DOCOMO2 | Related to non-overlapping UL channels with other UL channels but colliding with semi-static DL or SSB, we have the same understanding as Spreadtrum. We think the question is same as the one HW/HiSi raised above. Such a non-overlapping UL channel should be cancelled in step 2. |
| DOCOMO3 | In response to the FL question “There were a couple of comments for multi-CSI PUCCH too. It is mentioned by some companies that dropping should be done before multiplexing. Could DCM or others supporting DCM’s understanding please explain?”, whether multi-CSI PUCCH is dropped in Step.2 or Step.3 depends on the overlapping condition. Please see the following figure as an example. Case 1 is the one provided by Spreadtrum, where CSI1 is overlapped with semi-static DL symbol. In this case, CSI1 is dropped in Step.2, i.e. before multiplexing in Step.3. On the other hand, no CSI is overlapped in Case 2. In this case, the multiplexing for multi-CSI should be done in Step.3 as it is written in section 9.2.5. Note that SR is added for Case 2 to show “Step2” procedure. The SR can be assumed as different priority from other UL channels, e.g. SR is HP and others are LP. |
| Nokia / NSB 2 | Just here to clarify our question on step 3.2 – if this applies to any HP transmission or is only applicable for a HP transmission scheduled by a DCI. First, we got the working assumption from last time, that clearly specifies that we are having a different handling of order for scheduled and non-scheduled HP transmission: Working assumption - Multiplexing/overriding/etc. is performed similar to Rel.15 as if HP channels do not exist; this means that LP operations, multiplexing/overriding/etc., are performed before cancellation. - A UE cancels the transmission of a LP channel including any intermediate scheduled LP transmission that does not overlap with any LP channel, if any DCI schedules an overlapping HP transmission with the LP channel, before performing multiplexing/overriding HP channels if any. This is also clearly reflected in the current specifications based on the CR approved last meeting (the two different parts are the same parts currently fine-tuned in the UCI email thread with respect to PUCCH repetition). Therefore, we would like to understand now (or are puzzled) where in the steps now there is a difference between: - A scheduled HP transmission overlapping with some LP transmissions (based the 2nd bullet of the WA) - A HP transmission not scheduled by a DCI overlapping with some LP transmissions (other cases) |
| Apple 3 | On issue #5, some good questions/comments have been raised. A first attempt from our side to modify it to capture our understanding of the intended behavior: - Step 1: For LP, final PUCCH resource for HARQ-ACK is determined based on the overriding procedure. - Step 2: Each individual LP PUCCH/PUSCH (before any multiplexing) that collides with semi-static DL symbols and/or SSB symbols is cancelled. - Step 3: For LP, overlapping PUCCH/PUSCH channels are multiplexed. - Step 4: For HP, each individual HP PUCCH/PUSCH (before any multiplexing, and (including or not including?) any intermediate PUCCH resource for HARQ-ACK in the middle of the overriding procedure) that collides with semi-static DL symbols and SSB symbols is cancelled. - Step 5: If there is collision between HP and LP PUCCHs/PUSCHs, LP channels are cancelled. - Step 6: For HP, final PUCCH resource for HARQ-ACK is determined based on the overriding procedure. (If it collides with semi-static DL symbols and/or SSB symbols, it is cancelled?) - Step 7: For HP, overlapping PUCCH/PUSCH channels are multiplexed. - Step 8: If there is collision between HP and LP PUCCHs/PUSCHs, LP channels are cancelled. - Step 9: If there is collision between high and/or low priority PUCCH/PUSCHs and semi-static DL symbols, it/they will be dropped. The assumptions behind the description: - The first checking of HP/LP overlapping is also done before final HP A/N overriding. - Multiple CSI on PUCCH per slot handling is in step 3. - When I try to differentiate in step 4 that HP HARQ-ACK overriding is not applied when checking the overlapping with LP, it becomes unclear when the check against semi-static DL symbols should be applied (shown as in red above). To be more specific, do we check it for any intermediate HARQ-ACK resource? If yes, then the procedure for HP and LP becomes different. - Step 4 is written in a way that includes all the individual channels (this step is related to Klaus’s question, which I did not try to address here). I think this is something that requires more clarification. Let us take the following example: DCI1 points to PUCCH1 for HARQ-ACK, which overlaps with SR (PUCCH2). When HARQ-ACK and SR are multiplexed, it uses PUCCH3. Then later on DCI2 comes, and it overrides the previous HARQ-ACK resource and points to PUCCH4 for HARQ-ACK. It still overlaps with SR (PUCCH2), and multiplexing of HARQ-ACK and SR results in PUCCH5. So the final HP transmission is PUCCH5. Now the question is which channels are considered in step 4 and which channels are considered in step 7? Or a simpler question is what channels are considered in step4/7 combined? Is it correct understanding that if a LP overlaps with any of the PUCCH1/2/3/4/5, it gets cancelled? If not, which PUCCHs should be considered for cancelling LP? This is not currently clear in the steps above, because I don’t know which one is the correct answer (or the intended behavior). |

Further, based on the following two clauses, it is understood that the intermediate channels can be scheduled to be overlapping with semi-static DL symbols or SSB. If the final channel is overlapping with semi-static DL symbols or SSB, the UE does not transmit.

“For a set of symbols of a slot that are indicated to a UE as downlink by tdd-UL-DL-ConfigurationCommon, or tdd-ULDL-ConfigurationDedicated, the UE does not transmit PUSCH, PUCCH, PRACH, or SRS when the PUSCH, PUCCH,PRACH, or SRS overlaps, even partially, with the set of symbols of the slot.”

“For operation on a single carrier in unpaired spectrum, for a set of symbols of a slot that are indicated to a UE by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon, for reception of SS/PBCH blocks, the UE does not transmit PUSCH, PUCCH, PRACH in the slot if a transmission would overlap with any symbol from the set of symbols and the UE does not transmit SRS in the set of symbols of the slot.”

# 4        Summary of RAN1 #103e

Conclusion:

In the following clause from Section 9 of TS 38.213:

“where

- The overlapping is applicable before or after resolving overlapping among channels of larger priority index, if any, as described in Clause 9.2.5”

the meaning of “before or after” should be interpreted as follows: A UE checks the overlap between a HP channel and a low priority channel before multiplexing. If there is an overlap, the LP channel gets cancelled. If not, a UE performs multiplexing across the HP channels. If then there is an overlap with a LP channel, the LP channel gets cancelled.

Conclusion:

- On the issue of cancellation due to collision with semi-static DL symbols and SSB, re-open the topic in RAN1#104-e but only discuss it during the second phase if there is a clear, workable, proposal available. Otherwise, conclude during the first phase that RAN1 could not reach a consensus and that the UE behavior remains the same as in Rel. 15.

# 5        References

[1] R1-2007635, “Corrections on scheduling and HARQ,” Huawei, HiSilicon

[2] R1-2007705, “Maintenance of scheduling/HARQ for NR URLLC,” Ericsson

[3] R1-2007734, “Text proposals for intra-UE multiplexing and PUSCH preparation procedure time,” ZTE

[4] R1-2008109, “Remaining issues of enhancements to scheduling/HARQ,” Spreadtrum

[5] R1-2008607, “Remaining issues on HARQ and scheduling for URLLC,” Qualcomm

[6] R1-2008277, “Remaining issues on scheduling and HARQ,” OPPO

[7] R1-2008304, “Out-of-order CBG based PUSCH re-transmission and PHR/power scaling with UL cancellation,” Nokia, NSB

[8] R1-2008535, “Corrections on scheduling/HARQ for Rel-16 URLLC,” NTT DOCOMO

[9] R1-2008672, “Enhancement for scheduling/HARQ,” vivo

[10] R1-2008303, “Maintenance of Rel-16 URLLC intra-UE prioritization enhancements (incl. draft CRs),” Nokia/NSB

[11] R1-2007816, “Remaining issues on intra-UE prioritization,” CATT