3GPP TSG RAN WG1 Meeting #104-e	R1-2100110

e-Meeting, January 25th – February 5th, 2021

Source: 	ZTE

Title: 	Discussion on Cross-Carrier Scheduling from SCell to PCell

Agenda item:	8.13.1

Document for:	Discussion/Decision

Introduction

During RAN#86 meeting, a new work item on DSS (dynamic spectrum sharing) was approved for Rel-17 [1]. One of the objectives of this WI is to support PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell. In RAN1 #103e meeting, some progress were made for SCell scheduling PCell [2].

In this contribution, we provide our analysis on how to support PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell.

General issue

Configuration of SCell scheduling PCell

With current NR spec listed below, P(S)Cell cannot be scheduled by SCell as schedulingCellInfo can only be configured as own in the CrossCarrierSchedulingConfig for P(S)Cell. Furthermore, if cif-Presence is set to true, the default CIF value for indicating a grant or assignment for this P(S)Cell is 0. When schedulingCellInfo for a SCell is configured as other in the CrossCarrierSchedulingConfig, that means this SCell is a scheduled cell and also it will be configured with cif-InSchedulingCell and schedulingCellId, wherein the candidate value for cif-InSchedulingCell is [1,...,7].

CrossCarrierSchedulingConfig ::= SEQUENCE { schedulingCellInfo CHOICE { own SEQUENCE { -- Cross carrier scheduling: scheduling cell cif-Presence BOOLEAN }, other SEQUENCE { -- Cross carrier scheduling: scheduled cell schedulingCellId ServCellIndex, cif-InSchedulingCell INTEGER (1..7) } }, ... }

In case that PDCCH on SCell can be used to schedule PDSCH or PUSCH on P(S)Cell, P(S)Cell should be additional configured as a scheduled cell of a SCell. With current structure of schedulingCellInfo, only own or other can be configured for a serving cell and only own can be configured for P(S)Cell. As a result, schedulingCellInfo for P(S)Cell should be updated and configured as a scheduling cell and scheduled cell simultaneously. Two options can be considered. One is adding the third choice of own and other, the other is reusing the choice of other with re-interpretation of this configuration. The details should be determined in RAN2.

Proposal 1: For SCell scheduling PCell, PCell should be configured as the scheduling cell and the scheduled cell simultaneously by RRC parameter CrossCarrierSchedulingConfig.

USS configuration of SCell scheduling PCell

In Rel-15/Rel-16, the search space ID is applied to associate the search space in scheduling cell and search space in scheduled cell. And for a scheduled cell in the case of cross carrier scheduling, except for nrofCandidates in the IE SearchSpace, all the optional fields are absent (regardless of their presence conditions). In other words, in case that Cell #1 cross-carrier schedules Cell #2 with search space #n (SS #n), UE monitors the corresponding PDCCH on Cell #1 based on the MO configurations for SS #n of Cell #1 and acquires the candidate number based on SS #n of Cell #2.

For SCell scheduling PCell, the same approach can be reused. For USS configuration on PCell, at least two options can be considered.

      - Option 1. A USS set configured on PCell can only be used for either self-scheduling or CCS from SCell.

      - Option 2. A USS set configured on PCell can only be used for both self-scheduling and CCS from SCell.

For Option 1, if a USS set on PCell is only configured with searchSpaceId and nrofCandidates, this USS set can only be used for CCS from sSCell. If a USS set on PCell is configured with other optional fields besides nrofCandidates, this USS set can only be used for self-scheduling from PCell.

For Option 2, a USS set configured with other optional fields besides nrofCandidates can be used for both self-scheduling and CCS from SCell. Then the description of the IE SearchSpace for CCS from SCell should be updated such that except for nrofCandidates in the IE SearchSpace, all the optional fields are ignored (regardless of their presence conditions).

Option 1 may need more search space configuration compared with Option 2. However, since up to 10 USS sets can be configured in a BWP, the number of search space sets are sufficient. It is preferred to apply the straightforward option, i.e., Option1.

Proposal 2: For SCell scheduling PCell and for USS configuration, search space ID is used to associate the search space configuration for PCell and search space configuration for SCell.

- A USS set configured on PCell can only be used for either self-scheduling or CCS from SCell.

- If the USS #n on PCell is used for cross-carrier scheduling from SCell, UE acquires the MO configuration from USS #n on SCell and acquire the candidate number from USS #n on PCell, which is the same as in Rel-15/Rel-16.

BD/CCE limit handling

In the scope of Rel-17 DSS, there is a restriction that the total PDCCH blind decoding budget should not be changed as a result of this work. There are two PDCCH blind decoding budget, one is  or , the other is  or . For each scheduled cell, the UE is not required to monitor on the active DL BWP with SCS  configuration  of the scheduling cell more than  PDCCH candidates or more than  non-overlapped CCEs per slot.

In Rel-16 CA, there is only one scheduling cell for a scheduled cell. And the BD/CCE limit on the scheduling cell for the scheduled cell is determined based on the numerology of the scheduling Cell. In Rel-17 DSS with support of SCell scheduling PCell, there are two scheduling cells (PCell and sSCell) for a scheduled cell (PCell), how to handle the BD/CCE limit are analyzed below.

M_max/C_max

As shown in Figure 1 for the scheduled cell (PCell), there are M1 candidates on the first scheduling cell (PCell) with self-scheduling and M2 candidates on the second scheduling cell (sSCell) with CCS. Since the SCS on the two scheduling cells are same and equal to 15kHz, the M_max of the scheduled cell (PCell) should be same as =44 with μ=0 and (M1 + M2) for monitoring should not be larger than M_max. Note, same mechanism can be used for C_max.

Figure 1 SCell-schedule-PCell with same SCS

In case the SCS of two scheduling cells are different, as shown in Figure 2, there are several options as listed below to determine the M_max of the scheduled cell (PCell) with two scheduling cells (PCell and sSCell). Assume that the SCS of the first scheduling cell (PCell) is μ1 and the SCS of the second scheduling cell (sSCell) is μ2. The same mechanism can be used for determination of C_max.

Figure 2 SCell-schedule-PCell with different SCS

- Option 1: Determine the M_max by the SCS of the first scheduling cell (PCell), that is M_max = ;

- Option 2: Determine the M_max by the SCS of the second scheduling cell (sSCell), that is M_max = ;

- Option 3: Determine the M_max by the maximum SCS of the two scheduling cells, that is M_max = ;

- Option 4: Determine the M_max by the minimum SCS of the two scheduling cells, that is M_max = ;

- Option 5: Determine the M_max by the SCS of the two scheduling cells with scaling factor for each scheduling cell, that is M_max =  +  per slot with SCS = μ1;

Comparing the above options, Option 1 – Option 4 can be classified as only using one of the scheduling cells to determine the M_max of the scheduled cell (PCell). Option 5 can be classified as using both of the scheduling cells to determine the M_max of the scheduled cell (PCell). For all the five options, the SCS of the scheduling cell(s) is/are taken into account for determining the M_max of the scheduled cell (PCell) which is aligned with the intention in Rel-16 CA. While different options will lead to different value of M_max of the scheduled cell (PCell). Option 3 is slightly preferred with more candidates than other options which is beneficial in terms of flexible scheduling for network implementation.

For further consideration, the maximum candidate number (M1_max) on the first scheduling cell (PCell) for the scheduled cell (PCell) and the maximum candidate number (M2_max) on the second scheduling cell (sSCell) for the scheduled cell (PCell) can be defined if needed. How to define the M1_max and M2_max should be further studied. For example, M1_max = , M2_max = , wherein, P1+P2 = 1. Then M1 candidates for PCell self-scheduling should not be larger than M1_max and M2 candidates for SCell scheduling PCell should not be larger than M2_max.

Proposal 3: For SCell scheduling PCell,

- Determine the M_max/C_max by the maximum SCS of PCell and sSCell.

- M1_max/C1_max and M2_max/C2_max for PCell and sSCell for scheduling PCell, can be defined respectively.

M_total/C_total

If a UE is configured with  downlink cells (Note, assume without M-TRP, that is = and =0, and also assume without span based monitoring) with DL BWPs of the scheduling cells using SCS configuration , where , a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE is not required to monitor more than  PDCCH candidates or more than  non-overlapped CCEs per slot on the active DL BWP(s) of scheduling cell(s) from the  downlink cells.

In case that PDCCH of SCell scheduling PDSCH or PUSCH on P(S)Cell is supported, PDCCH blind decoding will be performed on two cells for this P(S)Cell. In this case, parameter  will be calculated twice. For example, assume 6 cells are configured for a UE shown in Figure 3, wherein the SCS = 15kHz for PCell #1 and the SCS = 30kHz for SCell #2. Two cases are compared below.

- Case 1: PCell cannot be scheduled by SCell (the red arrow in Figure 3 is not presented), M_total_15khz = floor(4*44*1/6)=29;  M_total_30khz=floor(4*36*5/6)=120;

- Case 2: PCell can be scheduled by SCell (the red arrow in Figure 3 is presented), M_total_15khz = floor(4*44*1/6)=29;  M_total_30khz=floor(4*36*6/6)=144;

Figure 3 CA with SCell-schedule-PCell

The blind decoding complexity of case 2 is increased compared with case 1 mainly because the PCell is calculated twice. Since there is a restriction that the total PDCCH blind decoding budget should not be changed as a result of this work, this issue can be resolved by considering the following two potential solutions.

Option 1: The number of PCell is counted as N1 < 1 and N2 < 1 for the two scheduling cells (PCell and sSCell) respectively, wherein, N1 + N2 = 1.

Option 2: The number of PCell is still count as 1 for only one of the scheduling cells, i.e., PCell.

Option 1 seems more accurate with consideration on both scheduling cells, while option 2 is simple for BD/CCE splitting among numerologies. Note, if the SCS of the two scheduling cells (PCell and sSCell) are same, then the result of M_total based on option 1 and option 2 are same, regardless the specific value of N1 and N2. Option 1 is slightly preferred due to BD/CCE splitting on both scheduling cells.

Proposal 4: For SCell scheduling PCell, the scheduled cell (PCell) with two scheduling cells (PCell and sSCell) should be counted as one cell in CA scaling calculation.

PDCCH overbooking/dropping

In Rel-15/Rel-16, overbooking is only allowed on PCell/PSCell to guarantee the usage of candidates in USS sets with high-efficiency regardless of the non-dense CSS configuration. It should be noted that the overbooking is only for PDCCH scheduling PCell, i.e., overbooking is not allowed for PDCCH on PCell scheduling a SCell if the SCell is CCS by the PCell.

In Rel-17 DSS with support of SCell scheduling PCell, there are two scheduling cells (PCell and sSCell) for a scheduled cell (PCell). No matter USS sets are partially or fully moved to sSCell, overbooking should be also supported for PDCCH scheduling the PCell. As a result, overbooking should be supported on the two scheduling cells (PCell and sSCell) for the same scheduled cell (PCell). That means PDCCH overbooking on sSCell is also supported only for the PDCCH scheduling the PCell.

Proposal 5: For SCell scheduling PCell, overbooking is supported on both PCell and sSCell for scheduling the PCell.

If only M_max is defined and M1_max/M2_max is not defined, it is enough for the Alt.1 (i.e., all USS with non-fallback DCI are moved to SCell) in section 3.3 and USS with fallback DCI is also absent on PCell. In this case, M1 candidates in CSS sets are all monitored and candidates dropping will only occur among M2 candidates in USS sets. The candidates dropping can be performed on the second scheduling cell by USS index with the budget determined by a function of M_max minus M1. Moreover, M1_max and M2_max are useless in this case, because there is no need to define a budget only for the candidates in CSS sets. Meanwhile, usage of candidates in USS sets will be low-efficiency if M1_max and M2_max are defined, because the remaining candidates of M1_max minus the candidates in CSS sets cannot be utilized by the USS sets beyond the budget of M2_max.

In case of Alt.1 in section 3.3 and USS with fallback DCI present on PCell, or Alt.2 (i.e., parts of USS with non-fallback DCI are moved to SCell) in section 3.3, similar mechanism as above can be used for both scheduling cells simultaneously. That is, candidates dropping can be performed on both scheduling cell by USS index with the budget determined by a function of M_max minus candidates of CSS in M1. For simplified considering, this case may be more applicable with M1_max and M2_max defined. Then candidates dropping on each scheduling cell for the PDCCH scheduling PCell can be performed according to M1_max and M2_max respectively.

Proposal 6: For SCell scheduling PCell, before determine how to handle candidates dropping, USS allocation and BD/CCE limit budget should be determined first.

sSCell activation/deactivation and dormancy

In Rel-16, any SCell can be deactivated or switched to dormant BWP. In Rel-17 DSS with support of SCell scheduling PCell, whether sSCell can be deactivated or switched to dormant BWP are analyzed below.

For Alt.1 in section 3.3, if USS with fallback DCI is absent on PCell, sSCell cannot be deactivated or switched to dormant BWP otherwise, there is no DCI scheduling PDSCH/PUSCH on PCell. If USS with fallback DCI is present on PCell, sSCell can be deactivated or dormant at the sacrifice of system performance because only fallback DCI can be used to schedule PDSCH/PUSCH on PCell. Meanwhile, sSCell cannot be indicated to dormancy by non-fallback DCI because there is no non-fallback DCI on PCell.

For Alt.2 in section 3.3, sSCell can be deactivated or switched to dormant BWP flexibly as non-fallback DCI formats can be configured on both PCell and sSCell.

The motivation of Rel-17 DSS with SCell scheduling PCell is offloading PDCCH to sSCell to alleviate PDCCH blocking on PCell. PCell scheduled by sSCell is a configurable function which should not impact PCell baseline scheduling, especially when sSCell is deactivated or dormant. So sSCell activation/deactivation and sSCell dormancy should be supported as in Rel-15/Rel-16. If sSCell cannot be deactivated or dormant, UE will always keep monitoring at least on two cells.

Proposal 7: For SCell scheduling PCell, mechanisms of sSCell activation/deactivation and sSCell dormancy are the same as in Rel-15/Rel-16.

Activation/deactivation of sSCell scheduling PCell/PSCell

In section 2.5, in case of sSCell activation or non-dormancy, sSCell scheduling PCell is always enabled if configured. And in case of sSCell deactivation or dormancy, sSCell scheduling PCell is disabled too.

However, the motivation of this two functions are different, one is for power saving while another is for PDCCH offloading. Considering PDCCH capacity is sufficient or not on PCell is dynamic, activation/deactivation of scheduling from sSCell to PCell/PSCell can be supported instead of RRC reconfiguration.

For Alt.1 in section 3.3, if USS with fallback DCI is also absent on PCell, scheduling from sSCell to PCell/PSCell cannot be deactivated due to lack of DCI for scheduling PDSCH/PUSCH on PCell. If USS with fallback DCI is present on PCell, scheduling from sSCell to PCell/PSCell can be deactivated at the sacrifice of system performance because only fallback DCI can be used to schedule PDSCH/PUSCH on PCell.

For Alt.2 in section 3.3, scheduling from sSCell to PCell/PSCell can be activated/deactivated flexibly depending on PDCCH capacity variation as non-fallback DCI formats can be configured on both PCell and sSCell.

Above all, except for all USS sets are allocated on sSCell, activation/deactivation of CCS from sSCell to PCell/PSCell can be supported instead of RRC reconfiguration.

Proposal 8: For SCell scheduling PCell, dynamic activation/deactivation of scheduling from sSCell to PCell/PSCell can be supported to avoid frequent RRC reconfiguration.

Search space and DCI formats handling

Type 3 CSS

In RAN1 #103e meeting, the following conclusion was achieved. It was agreed that the configuration of Type 3 CSS set for DCI formats 2_0, 2_1, 2_2, 2_3, 2_4 and applicability of the information in the DCI formats are the same as in Rel-15/Rel-16.

Conclusion - When CCS from sSCell to PCell/PSCell is configured, the configuration of Type 3 CSS set for DCI formats 2_0, 2_1, 2_2, 2_3, 2_4 and applicability of the information in the DCI formats are the same as in Rel-15/Rel-16 - FFS: DCI format 2_5 and DCI Format 2_6 handling - Note: The SCell configured with CCS to Pcell/PSCell is referred to as ‘sSCell’

Except for DCI formats 2_0, 2_1, 2_2, 2_3 and 2_4, DCI format 2_5, 2_6 and fallback DCI (DCI format 1_0/0_0) can also be associated with Type 3 CSS. The handling of remaining DCI formats in Type 3 CSS is analyzed below.

- Type 3 CSS with DCI format 2_5

DCI format 2_5 is used for notifying the availability of soft resources for IAB-DU. Group common DCI format 2_5 in Type 3 CSS can be configured on PCell/PSCell or SCell, which is similar as DCI format 2_0/2_1/2_2/2_3/2_4. So there is no additional issues to decide regarding the location of Type 3 CSS with DCI format 2_5.

- Type 3 CSS with DCI format 2_6

Group common DCI format 2_6 in Type 3 CSS can only be configured on PCell/PSCell because this format is used for SCell dormancy indication. If DCI format 2_6 in Type 3 CSS can be configured on the SCell used for scheduling PCell/PSCell, whether this SCell should be always active like PCell/PSCell and whether this SCell can be indicated as dormancy should be further discussed. To reduce the specification impact and standarization effort, it is preferred to keep the DCI format 2_6 on Pcell/PSCell only.

- Type 3 CSS with fallback DCI

In Type 3 CSS, fallback DCI with CRC scrambled by C-RNTI/CS-RNTI/MCS-C-RNTI can also be configured for unicast data scheduling only for the primary cell. In case Type 3 CSS is configured on SCell, fallback DCI in Type 3 CSS cannot be configured. In case Type 3 CSS is configured on PCell, whether fallback DCI in Type 3 CSS can be configured or not depends on network implementation. More specifically, fallback DCI in Type 3 CSS can be kept on PCell/PSCell if configured.

Proposal 9: For SCell scheduling PCell, Type 3 CSS with DCI format 2_5/2_6 and fallback DCI can be reused with no changes compared to Rel-15/Rel-16, i.e.,

- Type 3 CSS with DCI format 2_5 can be configured on PCell/PSCell or SCell.

- Type 3 CSS with DCI format 2_6 can only be configured on PCell/PSCell.

- Type 3 CSS with fallback DCI can only be configured on PCell/PSCell.

USS with fallback DCI

To ensure that RRC_IDLE UEs can still camp on this PCell successfully, network has to configure type 0/0A/1/2 CSS with fallback DCI in the PCell. While for the USS with fallback DCI for scheduling PCell, three alternatives can be considered.

- Alt.1 Leave it on PCell

- Alt.2 Move it to sSCell

- Alt.3 Don’t support the USS with fallback DCI for scheduling PCell

Firstly, considering the spec impact, Alt.2 is not preferred because fallback DCI doesn’t support cross-carrier scheduling from the first NR release. Meanwhile, UE needs to monitor fallback DCI for PCell’s PDSCH/PUSCH scheduling on PCell for robust scheduling. In case Alt.3 is supported, fallback DCI with CRC scrambled by C-RNTI/CS-RNTI/MCS-C-RNTI for unicast scheduling can only be transmitted in the Type 0/0A/1/2/3 CSS, but there are also many restrictions. For example, only aggregation level 4/8/16 can be used for Type 0/0A/2 CSS; Type 1 CSS can be only used if the UE has not been provided a Type 3 CSS or a USS and the UE has received a C-RNTI; Type 3 CSS can be used if this optional search space is configured and only configured on PCell. From this perspective, Alt.3 would be too restrictive. As a result, Alt.1 is preferred due to flexible scheduling.

Proposal 10: For SCell scheduling PCell and for USS, fallback DCI scheduling PCell’s PDSCH/PUSCH is restricted on PCell/PSCell only.

USS with non-fallback DCI

During RAN1#103e meeting, the following agreements were achieved.

Agreements: - Discuss in RAN1#104-e how to handle ‘DCI formats 0_1,1_1,0_2,1_2 scheduling PDSCH/PUSCH on PCell/PSCell’ from USS set(s), when CCS from sSCell to PCell/PSCell is configured.. Below alternatives can be considered in the discussion (other alternatives are not precluded) - Alt 1: UE cannot be configured to monitor DCI formats 0_1,1_1,0_2,1_2 on PCell/PSCell USS set(s), and can be configured to monitor them only on the sSCell USS set(s) - Alt 2: UE can be configured to monitor DCI formats 0_1/1_1/0_2/1_2 on PCell/PSCell USS set(s), and/or on sSCell USS set(s). The PDCCH monitoring is based on following alternatives (other alternatives are not precluded) - Alt 2-1: - UE can monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously - Alt 2-2: - Dynamic switching of PDCCH monitoring of DCI formats 0_1,1_1,0_2,1_2 between monitoring on PCell/PSCell USS sets and monitoring on sSCell USS sets is supported - FFS: Details of switching mechanism - UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS sets simultaneously - Alt 2-3: - UE does not monitor the same DCI format on both PCell USS set(s) and sSCell USS sets simultaneously. UE can monitor some DCI formats on sSCell USS sets and other DCI formats on PCell/PSCell USS sets simultaneously - Alt 2-4: - The USS set(s) on PSCell/PCell and the USS set(s) on sSCell are configured such that UE does not monitor DCI formats 0_1,1_1,0_2,1_2 on both PCell USS set(s) and sSCell USS set(s) simultaneously - FFS following aspects - Impact of sSCell activation/deactivation and sSCell dormancy - Impact on BD/CCE limit handling - Whether PDCCH overbooking on sSCell is supported or not supported and impact (if any) on overbooking handling on PCell/PSCell - Impact from different numerologies between PDCCH on the PCell/PSCell and that on the sSCell - Whether or not to have mechanism for activation/deactivation of scheduling from sSCell to PCell/PSCell - USS configuration details (e.g. handling of USS type (self-scheduling, cross carrier scheduling) for a USS set configured for scheduling of PCell/PSCell)

Regarding USS with non-fallback DCI (DCI format 0_1/1_1/0_2/1_2), Alt.1 is to move all USS sets with DCI format 0_1/1_1/0_2/1_2 to sSCell, while Alt.2 is to move parts of the USS sets with DCI format 0_1/1_1/0_2/1_2 to sSCell and leave the remaining still on PCell.

Alt.1 may be simpler while Alt.2 can offer higher flexibility for network implementation. Alt.1 is benefit to offload PDCCH to sSCell as much as possible, while Alt.2 is robust in case of worse coverage on sSCell.

Both Alt.1 and Alt.2 are feasible, combined with some related issued analyzed in section 2, a summary is listed below.

Summary

For Alt.1, and if USS with fallback DCI is also absent on PCell, only define M_max for the scheduled cell (PCell) with two scheduling cells (PCell and sSCell) is enough (i.e., M1_max and M2_max are not needed), the candidates dropping can be performed on the second scheduling cell by USS index with the budget determined by a function of M_max minus M1. sSCell cannot be deactivated or switch to dormant BWP due to lack of DCI for scheduling PDSCH/PUSCH on PCell, which is the same reason of that scheduling from sSCell to PCell/PSCell cannot be deactivated.

For Alt.1, and if USS with fallback DCI is present on PCell, M1_max and M2_max defined on each of the two scheduling cells (PCell and sSCell) for the same scheduled cell (PCell) is more applicable for candidates dropping on each scheduling cell respectively. sSCell can be deactivated or dormant at the sacrifice of system performance, which is the same reason of that scheduling from sSCell to PCell/PSCell can be deactivated.

For Alt.2, M1_max and M2_max defined on each of the two scheduling cells (PCell and sSCell) for the same scheduled cell (PCell) is more applicable for candidates dropping on each scheduling cell respectively. Since non-fallback DCI on both scheduling cells, sSCell can be activated/deactivated or switch to dormant BWP flexibly without impact the scheduling PDSCH/PUSCH on PCell, and scheduling from sSCell to PCell/PSCell can be activated/deactivated flexibly depend on PDCCH capacity variation.

All the potential combination above are workable, considering Alt.2 in section 2.3 is more flexible for network and the UE complexity is not increased with the unchanged BD/CCE budget, Alt.2 is slightly preferred.

For the Alt.2-1 to Alt.2-4, the Alt.2-1 is a baseline function, Alt.2-2 to Alt.2-4 can be used to reduce or distribute monitoring on two scheduling cells and can be determined further if Alt.2 is supported.

Proposal 11: For SCell scheduling PCell and for USS with non-fallback DCI,

- Alt.2 is preferred with more flexibility for network and non-increased BD/CCE budget.

- Alt.2-1 to Alt.2-4 can be determined further if Alt.2 is supported.

Conclusion

In this contribution, the high-level consideration and detailed consideration of SCell-schedule-PCell from our side are provided with the following proposals.

Proposal 1: For SCell scheduling PCell, PCell should be configured as the scheduling cell and the scheduled cell simultaneously by RRC parameter CrossCarrierSchedulingConfig.

Proposal 2: For SCell scheduling PCell and for USS configuration, search space ID is used to associate the search space configuration for PCell and search space configuration for SCell.

- A USS set configured on PCell can only be used for either self-scheduling or CCS from SCell.

- If the USS #n on PCell is used for cross-carrier scheduling from SCell, UE acquires the MO configuration from USS #n on SCell and acquire the candidate number from USS #n on PCell, which is the same as in Rel-15/Rel-16.

Proposal 3: For SCell scheduling PCell,

- Determine the M_max and C_max by the maximum SCS of PCell and sSCell.

- M1_max/C1_max and M2_max/C2_max for PCell and sSCell for scheduling PCell, can be defined respectively.

Proposal 4: For SCell scheduling PCell, the scheduled cell (PCell) with two scheduling cells (PCell and sSCell) should be counted as one cell in CA scaling calculation.

Proposal 5: For SCell scheduling PCell, overbooking is supported on both PCell and sSCell for scheduling the PCell.

Proposal 6: For SCell scheduling PCell, before determine how to handle candidates dropping, USS allocation and BD/CCE limit budget should be determined first.

Proposal 7: For SCell scheduling PCell, mechanisms of sSCell activation/deactivation and sSCell dormancy are the same as in Rel-15/Rel-16.

Proposal 8: For SCell scheduling PCell, dynamic activation/deactivation of scheduling from sSCell to PCell/PSCell can be supported to avoid frequent RRC reconfiguration.

Proposal 9: For SCell scheduling PCell, Type 3 CSS with DCI format 2_5/2_6 and fallback DCI can be reused with no changes compared to Rel-15/Rel-16, i.e.,

- Type 3 CSS with DCI format 2_5 can be configured on PCell/PSCell or SCell.

- Type 3 CSS with DCI format 2_6 can only be configured on PCell/PSCell.

- Type 3 CSS with fallback DCI can only be configured on PCell/PSCell.

Proposal 10: For SCell scheduling PCell and for USS, fallback DCI scheduling PCell’s PDSCH/PUSCH is restricted on PCell/PSCell only.

Proposal 11: For SCell scheduling PCell and for USS with non-fallback DCI,

- Alt.2 is preferred with more flexibility for network and non-increased BD/CCE budget.

- Alt.2-1 to Alt.2-4 can be determined further if Alt.2 is supported.

Reference

RP-193260, New WID on NR Dynamic spectrum sharing (DSS), RAN #86.

RAN1 #103e meeting, chairman notes.