3GPP TSG RAN WG1#104-e                                                  	            R1-2100275

e-Meeting, January 25th – February 5th, 2021

Agenda Item:	8.1.2.2

Source:	Lenovo, Motorola Mobility

Title:	Enhancements on Multi-TRP inter-cell operation

Document for:	Discussion

Introduction

Inter-cell multi-TRP operation has been discussed in Rel-16, however, without agreement due to the time limitation. QCL/TCI-related enhancements for inter-cell multi-TRP operations will be included in NR Rel-17 as the following approved WI for MIMO enhancement[1]:

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2: - Identify and specify features to improve reliability and robustness for channels other than PDSCH (that is, PDCCH, PUSCH, and PUCCH) using multi-TRP and/or multi-panel, with Rel.16 reliability features as the baseline - Identify and specify QCL/TCI-related enhancements to enable inter-cell multi-TRP operations, assuming multi-DCI based multi-PDSCH reception - Evaluate and, if needed, specify beam-management-related enhancements for simultaneous multi-TRP transmission with multi-panel reception - Enhancement to support HST-SFN deployment scenario: - Identify and specify solution(s) on QCL assumption for DMRS, e.g. multiple QCL assumptions for the same DMRS port(s), targeting DL-only transmission - Evaluate and, if the benefit over Rel.16 HST enhancement baseline is demonstrated, specify QCL/QCL-like relation (including applicable type(s) and the associated requirement) between DL and UL signal by reusing the unified TCI framework

The following agreement is achieved in RAN1#102e[2]:

Agreement Study the following aspects of QCL /TCI-related enhancement to enable inter-cell multi-DCI based multi-TRP operation. - Details on configuration of non-serving cell RS; - Allowed source and target RS types for RS transmitted from the non-serving cell TRP ; - Allowed QCL types for RS transmitted from the non-serving cell TRP ; - Measurement and reporting related to QCL/TCI enhancement except for that in 8.1.1, if any; - Clarification on potential UE behavior for associating/multiplexing non-serving cell RS with other RS/channels; - Other details not precluded.

The following agreements are achieved in RAN1#103e[3]:

Agreement For QCL /TCI related enhancement for enhanced inter-cell multi-TRP operations, support RRC configuration of non-serving cell information - Non-serving cell information can be associated with the TCI state and/or QCL-info at least when “neighbor cell SSB” is used as “QCL referenceSignal ” - FFS: Whether beam indication enhancement is needed in addition to QCL-info enhancement - FFS: Whether the association is explicit or implicit. Agreement The information provided by SSB-Configuration-r16/ssb-InfoNcell-r16 and/or MeasObject can be starting point for providing non-serving cell information. For future meetings Consider rate matching behavior related to non-serving cell SSB.

In this paper, we provide our views on the enhancement on inter-cell multi-TRP operation.

Discussion

For the inter-cell multi-TRP scenario provided in Figure 1, Cell#1 and Cell#2 are covered by SSB signals transmitted from TRP#1 and TRP#2. TRP#1 and TRP#2 are two different gNBs configured with different PCID. SSB set#1 is transmitted from TRP#1 and is associated with PCID#1, and SSB set#2 is transmitted from TRP#2 and is associated with PCID#2. A UE is located in the cell edge of Cell#1 as well as Cell#2, and the UE is served by TRP#1 and TRP#2 in multi-DCI based multi-TRP mode. However, the UE only access Cell#1 through the random access procedure. It means that from the UE perspective, TRP#1 is the serving cell for the UE and TRP#2 is a non-serving cell. So the UE can only obtain SSB indices from SSB set#1 associated with PCID#1, and cannot obtain SSB indices from SSB set#2 associated with PCDI#2 without decoding the system information of Cell#2.

Figure 1 Inter-cell multi-TRP scenario

It has been agreed that SSB from a non-serving cell can be configured as the source QCL RS for the PDCCH and PDSCH reception from a non-serving cell in multi-DCI based multi-TRP mode. In detail, Figure 2 provides the example of QCL chain configuration for DMRS of PDCCH and PDSCH transmitted from TRP#1 and TRP#2 in Figure 1.

For the PDSCH/PDCCH transmitted from TRP#1, UE expects the NW configuring SSB indices from SSB set#1, e.g., SSB#1-1, as the source QCL-TypeC and QCL-TypeD RS for the TRS, e.g., TRS#1-1, used for corresponding DMRS reception. For PDSCH/PDCCH transmitted from TRP#2, the UE expects the NW configuring SSB indices from SSB set#2, e.g., SSB#2-1, as the source QCL-TypeC and QCL-TypeD RS for the TRS, e.g., TRS#2-1, used for corresponding DMRS reception.

Figure 2 QCL-chain for inter-cell multi-TRP operation

The remaining issue is how to configure the SSB from a non-serving cell as the QCL RS in the TCI state. For a UE supporting positioning, SSB index from a non-serving cell can be configured as the spatial relation info for the SRS resources used for positioning by introducing SSB-Configuration-r16/ssb-InfoNcell-r16. The same mechanism can be introduced for TCI state. A example RRC configuration of TCI state if provided in Table 1.

Table 1 RRC configuration for enhanced TCI state

TCI-State ::= SEQUENCE { tci-StateId TCI-StateId, qcl-Type1 QCL-Info, qcl-Type2 QCL-Info OPTIONAL, -- Need R ... } QCL-Info ::= SEQUENCE { cell ServCellIndex OPTIONAL, -- Need R bwp-Id BWP-Id OPTIONAL, -- Cond CSI-RS-Indicated referenceSignal CHOICE { csi-rs NZP-CSI-RS-ResourceId, ssb-IndexServing SSB-Index ssb-Ncell SSB-InfoNcell-r16 }, qcl-Type ENUMERATED {typeA, typeB, typeC, typeD}, ... }

The higher layer parameter ssb-IndexServing indicates a SSB index from the serving cell, while ssb-Ncell indicates a SSB from a non-serving cell. In order to assist the UE to obtain correct SSB index information from a non-serving cell without blind decoding of system information, the necessary information of the non-serving cell can be configured by SSB-InfoNcell-r16 and SSB-Configuration-r16 by RRC signaling provided in Table 3.

Table 3 RRC configuration for non-serving cell’s information

SSB-InfoNcell-r16 ::= SEQUENCE { physicalCellId-r16 PhysCellId, ssb-IndexNcell-r16 SSB-Index OPTIONAL, -- Need S ssb-Configuration-r16 SSB-Configuration-r16 OPTIONAL -- Need S } SSB-Configuration-r16 ::= SEQUENCE { ssb-Freq-r16 ARFCN-ValueNR, halfFrameIndex-r16 ENUMERATED {zero, one}, ssbSubcarrierSpacing-r16 SubcarrierSpacing, ssb-Periodicity-r16 ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2,spare1 } OPTIONAL, -- Need S sfn0-Offset-r16 SEQUENCE { sfn-Offset-r16 INTEGER (0..1023), integerSubframeOffset-r16 INTEGER (0..9) OPTIONAL -- Need R } OPTIONAL, -- Need R Sfn-SSB-Offset-r16 INTERGE (0..15) ss-PBCH-BlockPower-r16 INTEGER (-60..50) OPTIONAL -- Cond Pathloss }

Proposal 1: SSB index from a non-serving cell can be directly configured in QCL-info and SSB-InfoNcell-r16/SSB-Configuration-r16 are used to provide the non-serving cell’s information for the UE to obtain the correct SSB information.

In order not to increase the UE complexity, the UE expects that the non-serving PCID configured in SSB-InfoNcell-r16/SSB-Configuration-r16 is associated with a neighboring cell configured in the measurement objects for mobility. Furthermore, when SMTC is configured for this neighboring cell, the UE only assume that the SSB indices within the SMTC is transmitted by the cell and the SSB indices outside the SMTC are not transmitted, so the configured non-serving cell’s SSB index should be within the SMTC configured for this cell.

Proposal 2: The non-serving PCID configured in SSB-InfoNcell-r16/SSB-Configuration-r16 is associated with a neighboring cell configured in the MO.

Proposal 3: The configured non-serving cell’s SSB index is within the SMTC configured for this cell.

In multi-DCI based multi-TRP, different CORESETs are configured for different TRPs and a higher layer parameter CORESETPoolIndex is configured for each CORESET for TRP differentiation. It is reasonable that we assume CORESETPoolIndex=0 is associated with the serving cell’s PCID and CORESETPoolIndex=1 is associated with another non-serving PCID different from the serving PCID. Based on this assumption, the UE’s behavior can be simplified. For example, the UE always assume that

- the TRS contained in the TCI state activated for CORESET(s) associated with CORESETPoolIndex = 0 is QCLed with a SSB from the serving cell with QCL-TypeC and QCL-TypeD, and the TRS contained in the TCI state activated for CORESET(s) associated with CORESETPoolIndex = 1 is QCLed with a SSB from the non-serving cell with QCL-TypeC and QCL-TypeD.

- the TRS contained in the activated TCI states associated with CORESETPoolIndex=0 is QCLed with a SSB from the serving cell with QCL-TypeC and QCL-TypeD, and the TRS contained in the activated TCI states associated with CORESETPoolIndex=1 is QCLed with a SSB from the non-serving cell with QCL-TypeC and QCL-TypeD.

So, we have the following proposals:

Proposal 4: In inter-cell multi-DCI based multi-TRP scenario, CORESETPoolIndex=0 is associated with the serving PCID and CORESETPoolIndex=1 is associated with a non-serving PCID different from the serving PCID.

Proposal 5: The UE assumes that TRS contained in the TCI state activated for PDCCH/PDSCH transmitted from TRP associated with a non-serving PCID is QCLed with a SSB index from this non-serving cell.

Multi-DCI based multi-TRP UL transmission is also supported in NR Rel-16. For example, each PUCCH resource may be associated with a CORESETPoolIndex for TRP-specific HARQ-ACK and CSI reporting. In the scenario provided in Figure 1, the spatial relation and the PL-RS used for UL signals transmitted to TRP#2 should be QCLed with a SSB associated with the non-serving PCID. The similar feature has been supported for SRS resources used for positioning, where SSB or DL-PRS from a non-serving cell can be configured as the spatialRelationInfo and PL-RS for the SRS resources used for positioning. We believe this can be extended to all UL signals. For example, SSB resources from a non-serving cell or CSI-RS resources QCLed with a SSB from a non-serving cell can be configured as the spatialRelationInfo and the PL-RS for PUCCH resource and SRS resources.  The necessary information of the non-serving cell can be configured by higher layer parameter SSB-InfoNcell-r16 and SSB-Configuration-r16.

Proposal 6: SSB from a non-serving cell can be configured as the spatial relation and PL-RS for PUCCH resources and SRS resources.

As the scenario in Figure 1, SSB from the non-serving Cell#2 can be configured as the spatialRelationInfo and PL-RS for PUCCH and SRS targeting TRP2. When CSI-RS resource is configured as the spatialRelationInfo and PL-RS for PUCCH and SRS resource targeting TRP#2, the UE shall assume the CSI-RS is QCLed with a SSB from the non-serving cell.

Proposal 7: When CSI-RS resource is configured as the spatialRelationInfo and/or PL-RS for PUCCH and/or SRS resource targeting a TRP associated with a non-serving PCID, the UE assumes that the CSI-RS is QCLed with a SSB index from the non-serving cell.

Conclusion

Proposal 1: SSB index from a non-serving cell can be directly configured in QCL-info and SSB-InfoNcell-r16/SSB-Configuration-r16 are used to provide the non-serving cell’s information for the UE to obtain the correct SSB information.

Proposal 2: The non-serving PCID configured in SSB-InfoNcell-r16/SSB-Configuration-r16 is associated with a neighboring cell configured in the MO.

Proposal 3: The configured non-serving cell’s SSB index is within the SMTC configured for this cell.

Proposal 4: In inter-cell multi-DCI based multi-TRP scenario, CORESETPoolIndex=0 is associated with the serving PCID and CORESETPoolIndex=1 is associated with a non-serving PCID different from the serving PCID.

Proposal 5: The UE assumes that TRS contained in the TCI state activated for PDCCH/PDSCH transmitted from TRP associated with a non-serving PCID is QCLed with a SSB index from this non-serving cell.

Proposal 6: SSB from a non-serving cell can be configured as the spatial relation and PL-RS for PUCCH resources and SRS resources.

Proposal 7: When CSI-RS resource is configured as the spatialRelationInfo and/or PL-RS for PUCCH and/or SRS resource targeting a TRP associated with a non-serving PCID, the UE assumes that the CSI-RS is QCLed with a SSB index from the non-serving cell.

References

- RP-193133 New WID: Further enhancements on MIMO for NR.

- Chainman Notes of RAN1#102e

- Chainman Notes of RAN1#103e