3GPP TSG RAN WG1 Meeting #104-e	R1-2100283

e-Meeting, January 25th – February 5th, 2021

Source:	ZTE

Title:	Discussion on closed-loop power control with an absolute TPC command

Agenda Item:	7.1

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss the problem of closed-loop power control with an absolute TPC command, and provide our views on this problem. Note that this issue was discussed in RAN1 #103e meeting and reached a conclusion (i.e., Issue#3 as follows) that “continue discussions in future meetings for Rel-15” [3].

Outcome of the email discussion in [103-e-NR-7.1CRs-10] - Rejected: issue#4 (R1-2007762), issue#5 (R1-2007803), issue#8 (R1-2008123), issue#9 (R1-2008425, R1-2008424), issue#16 (R1-2008657, R1-2008658), issue#22 (R1-2007759, R1-2007758), issue#20 (R1-2008786) - Continue discussions in future meetings for Rel-15: issue#7 (R1-2007805), issue#10 (R1-2008507), issue#18 (R1-2008767), Issue#3 (R1-2007760, R1-2007761), issue#12 (R1-2008597), issue#13 (R1-2008598), issue#15 (R1-2008656), issue#17 (R1-2008659) - Continue discussions in future meetings for Rel-16: issue#24 (R1-2008121), issue#14 (R1-2008599)

Considering the concerns from some companies about backward compatibility, we propose to discuss this issue for Rel-16 only rather than both Rel-15 and Rel-16.

- Discussion

In NR, there are two types of closed-loop power control (CL-PC): accumulated CL-PC and absolute CL-PC. In 38.213[1] as shown below, we have the following observation.

- It is clearly specified how to obtain TPC command(s)  for a given PUSCH transmission with accumulated CL-PC.

- However, for absolute CL-PC (as highlighted before), the UE behavior of determining TPC command for a given PUSCH transmission with an absolute CL-PC is not specified and thus unclear.

- is the PUSCH power control adjustment state for active UL BWP of carrier of serving cell and PUSCH transmission occasion if the UE is not provided tpc-Accumulation, where - The values are given in Table 7.1.1-1 - is a sum of TPC command values in a set of TPC command values with cardinality that the UE receives between symbols before PUSCH transmission occasion and symbols before PUSCH transmission occasion on active UL BWP of carrier of serving cell for PUSCH power control adjustment state , where is the smallest integer for which symbols before PUSCH transmission occasion is earlier than symbols before PUSCH transmission occasion - If a PUSCH transmission is scheduled by a DCI format 0_0 or DCI format 0_1, is a number of symbols for active UL BWP of carrier of serving cell after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUSCH transmission - If a PUSCH transmission is configured by ConfiguredGrantConfig, is a number of symbols equal to the product of a number of symbols per slot, , and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP of carrier of serving cell ... - is the PUSCH power control adjustment state for active UL BWP of carrier of serving cell and PUSCH transmission occasion if the UE is provided tpc-Accumulation, where - absolute values are given in Table 7.1.1-1 - If the UE receives a random access response message in response to a PRACH transmission on active UL BWP of carrier of serving cell as described in Clause 8

According to our knowledge, in TS 38.213, is only defined as in the following paragraphs. Based on email discussion in last meeting, the following unclear aspects were identified for further discussion in the following meetings.

- Q1: For DG (dynamic grant) PUSCH transmission, besides the DCI format scheduling the PUSCH, whether the TPC in DCI format 2_2 can also be applied?

- Q2: For CG (configured grant) PUSCH transmission, which DCI format 2_2 is used to provide TPC, e.g., latest DCI format 2_2, or latest DCI format 2_2 'K_PUSCH,min' symbols before transmission for CG PUSCH?

- For the PUSCH power control adjustment state for active UL BWP of carrier of serving cell in PUSCH transmission occasion - is a TPC command value included in a DCI format that schedules the PUSCH transmission occasion on active UL BWP of carrier of serving cell or jointly coded with other TPC commands in a DCI format 2_2 with CRC scrambled by TPC-PUSCH-RNTI, as described in Clause 11.3

In our views, regarding Q1, we think that absolute TPC is obtained only according to the DCI format scheduling the DG PUSCH. Then, regarding Q2, for CG PUSCH transmission, a straightforward way is to obtain  based on the latest DCI 2_2 that the UE receives KPUSCH, min symbols before the CG PUSCH, which is similar to the method for SRS with an absolute CL-PC.

- Note that  is equal to the product of a number of symbols per slot, , and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP  of carrier  of serving cell . It has already been specified as for CG PUSCH transmission in an accumulative CL-PC.

- For the sake of presentation, we copy the paragraphs corresponding to the absolute TPC command for SRS as follows. Here it can be observed that the absolute TPC command for SRS (including the case that there is no PUSCH transmission in CA) is determined according to the DCI format 2_3 which is  symbols before a first symbol of SRS transmission occasion  as highlighted.

- 7.3.1 UE behaviour - if the UE is not configured for PUSCH transmissions on active UL BWP of carrier of serving cell , or if srs-PowerControlAdjustmentStates indicates separate power control adjustment states between SRS transmissions and PUSCH transmissions, and tpc-Accumulation is provided, and the UE detects a DCI format 2_3 symbols before a first symbol of SRS transmission occasion , where absolute values of are provided in Table 7.1.1-1

Consequently, we have the following proposal and corresponding proposed text change for clarifying this issue in Rel-16. It is expected that UEs and networks are implemented in accordance to this TP and therefore no change is required on UE and network implementation.

Proposal: Absolute TPC is received from scheduling DCI for DG PUSCH, or latest common DCI KPUSCH, min symbols before transmission for CG PUSCH.

Proposed text change: {38.213: 7.1.1} in Rel-16

- 7.1.1 UE behaviour ... - is the PUSCH power control adjustment state for active UL BWP of carrier of serving cell and PUSCH transmission occasion if the UE is provided tpc-Accumulation, where - absolute values are given in Table 7.1.1-1 - If a PUSCH transmission is scheduled by a DCI format, is provided by the TPC command value in the DCI format - If a PUSCH transmission is configured by ConfiguredGrantConfig, is provided by the TPC command value in the latest DCI format 2_2 that the UE receives symbols before PUSCH transmission occasion . - If the UE receives a random access response message in response to a PRACH transmission on active UL BWP of carrier of serving cell as described in Clause 8 ...

- Conclusion

In this contribution, we discuss the issue on obtaining  for a given DG/CG PUSCH transmission with an absolute CL-PC. We have the following proposal and proposed change as shown above for Rel-16. The corresponding draft CR can be found in [2].

Proposal: Absolute TPC is received from scheduling DCI for DG PUSCH, or latest common DCI KPUSCH, min symbols before transmission for CG PUSCH.

- References

- 3GPP TS 38.213-g40 NR; Physical layer procedures for control

- R1-2100284 Draft CR on closed-loop power control with an absolute TPC command, ZTE

- Chairman's Notes RAN1#103-e final