3GPP TSG RAN WG1 #104-e                                       R1-2100470

e-Meeting, January 25th – February 5th, 2021

Source:	vivo

Title:	Discussion on mechanisms to improve reliability for RRC_CONNECTED UEs

Agenda Item:	8.12.2

Document for:  Discussion and Decision

- Introduction

In RAN#86 meeting, the work item of NR support of Multicast and Broadcast Services has been approved [1]. The followings have been identified as the objectives related with physical layer of the work item.

- Specify RAN basic functions for broadcast/multicast for UEs in RRC_CONNECTED state [RAN1, RAN2, RAN3]:

  - Specify a group scheduling mechanism to allow UEs to receive Broadcast/Multicast service [RAN1, RAN2]

    - This objective includes specifying necessary enhancements that are required to enable simultaneous operation with unicast reception.

  - Specify required changes to improve reliability of Broadcast/Multicast service, e.g. by UL feedback. The level of reliability should be based on the requirements of the application/service provided. [RAN1, RAN2]

- Specify RAN basic functions for broadcast/multicast for UEs in RRC_IDLE/ RRC_INACTIVE states [RAN2, RAN1]:

  - Specify required changes to enable the reception of Point to Multipoint transmissions by UEs in RRC_IDLE/ RRC_INACTIVE states, with the aim of keeping maximum commonality between RRC_CONNECTED state and RRC_IDLE/RRC_INACTIVE state for the configuration of PTM reception. [RAN2, RAN1].

Note: the possibility of receiving Point to Multipoint transmissions by UEs in RRC_IDLE/ RRC_INACTIVE states, without the need for those UEs to get the configuration of the PTM bearer carrying the Broadcast/Multicast service while in RRC CONNECTED state beforehand, is subject to verification of service subscription and authorization assumptions during the WI.

In RAN1 #102 e-meeting, the following agreements was made regarding the mechanisms to improve reliability for RRC_CONNECTED UEs [2][3].

Agreements:

- For RRC_CONNECTED UEs, HARQ-ACK feedback is supported for multicast and no additional evaluation is needed to justify this.

  - FFS: The detailed HARQ-ACK feedback solutions, e.g., ACK/NACK based, NACK-only based.

  - FFS: HARQ-ACK feedback can be optionally disabled and/or enabled.

Agreements:

- For RRC_CONNECTED UEs, at least support slot-level repetition for group-common PDSCH.

  - FFS: whether enhancement is needed

Agreements:

- For RRC_CONNECTED UEs, existing CSI feedback can be used for multicast transmission.

  - FFS: whether enhancement is needed

In RAN1 #103 e-meeting, the following agreements was made regarding the mechanisms to improve reliability for RRC_CONNECTED UEs [3].

Agreements:

For RRC_CONNECTED UEs receiving multicast, at least for PTM scheme 1, support at least one of the following:

- ACK/NACK based HARQ-ACK feedback for multicast,

  - From per UE perspective, UE feedback ACK or NACK.

  - From UEs within the group perspective,

    - FFS: PUCCH resource configuration for ACK/NACK feedback e.g., shared or separate PUCCH resources.

  - FFS details including conditions for it to be used

- NACK-only based HARQ-ACK feedback for multicast,

  - From per UE perspective, UE only feedback NACK.

  - From UEs within the group perspective, further down-select between:

    - FFS: PUCCH resource configuration for NACK only feedback.

  - FFS details including conditions for it to be used

- To decide in RAN1#104-e whether or not to support only one or both of the above schemes

  - If both are supported, FFS configuration/selection of ACK/NACK-based and NACK-only based HARQ-ACK feedback

Agreements:

For RRC_CONNECTED UEs receiving multicast, for ACK/NACK based HARQ-ACK feedback if supported for group-common PDCCH scheduling, PUCCH resource configuration for HARQ-ACK feedback from per UE perspective is, down-select one of the following options:

- Option 1: shared with PUCCH resource configuration for HARQ-ACK feedback for unicast

- Option 2: separate from PUCCH resource configuration for HARQ-ACK feedback for unicast

- Option 3: Option 1 or option 2 based on configuration

Agreements:

For RRC_CONNECTED UEs receiving multicast, for NACK-only based HARQ-ACK feedback if supported for group-common PDCCH scheduling, PUCCH resource configuration for HARQ-ACK feedback from per UE perspective is separate from PUCCH resource configuration for HARQ-ACK feedback for unicast.

- FFS PUCCH format

Agreements:

Enabling/disabling HARQ-ACK feedback for MBS is supported, further down-select between:

- Option 1: DCI

- Option 2: RRC configures enabling/disabling

- Option 3: RRC configures the enabling/ disabling function and DCI indicates enabling /disabling

- FFS: Option 4: MAC-CE indicates enabling/disabling

- FFS: Option 5: RRC configures the enabling/ disabling function and MAC-CE indicates enabling /disabling

Agreements:

For slot-level repetition for group-common PDSCH of RRC_CONNECTED UEs, for indicating the repetition number, further down-select among:

- Opt 1: by DCI

- Opt 2: by RRC

- Opt 3: by RRC+DCI

- FFS: Opt 4: by MAC-CE

- FFS: Opt 5: by RRC+MAC-CE

- FFS details for each option.

- FFS further enhancements for configuration of slot-level repetition

Agreements:

From the perspective of RRC_CONNECTED UEs receiving multicast, at least for PTM scheme 1 initial transmission, retransmission supports, for the purpose of down-selection, options are:

- Option 1: group-common PDCCH scheduled group-common PDSCH

- Option 2: UE-specific PDCCH scheduled PDSCH

  - Alt 1: PDSCH is UE-specific PDSCH

  - Alt 2: PDSCH is group-common PDSCH

- Option 3: both option 1 and option 2

- FFS other options

- FFS CBG based retransmission

Agreements:

FFS whether CSI feedback enhancement is needed for MBS, including but not limited:

- New CQI measurement

- New CSI report formats

- Targeted BLER

- CSI-RS configuration

- A-CSI-RS transmission triggering

- SRS configuration

Agreements:

For ACK/NACK based HARQ-ACK feedback if supported, both Type-1 and Type-2 HARQ-ACK codebook are supported for RRC_CONNECTED UEs receiving multicast,

        FFS details of HARQ-ACK codebook design.

         FFS whether enhanced Type-2 and/or Type-3 HARQ-ACK codebook is supported or not.

In this contribution, we will make discussions on the mechanisms to improve reliability for RRC_CONNECTED UEs.

- Discussion

PDSCH repetition

In NR, PDSCH transmission with repetition is supported to improve reliability, where the number of PDSCH repetitions can be semi-statically configured or dynamically indicated by corresponding DCI. For MBS, to improve reliability of Broadcast/Multicast service, it was agreed in the previous e-meeting that for RRC_CONNECTED UEs, at least support slot-level repetition for group-common PDSCH. For slot-level repetition for group-common PDSCH of RRC_CONNECTED UEs, for indicating the repetition number, same mechanisms as that in NR R15/16 can be used, i.e., it can be either configured by RRC or indicated by DCI. There is no motivation to introduce new option for indicating the repetition number for group-common PDSCHs.

Considering that differnet MBS services which may be assocated with different group common RNTIs can have different reliablity requirements, it is sugested to support seperate configurations for PDSCH repetition number for different MBS services.

Proposal 1: For PDSCH repetition of group-common PDSCH,

- NR Rel-15/16 existing options (i.e., RRC or DCI) for indicating the repetition number should be adopted.

- The PDSCH repetition numbers for different MBS services should be separately configured.

HARQ-ACK feedback

In the last e-meeting, it was agreed that for RRC_CONNECTED UEs, HARQ-ACK feedback is supported for multicast. Considering that the level of reliability should be based on the requirements of the application/service provided, whether to transmit HARQ-ACK feedback should be configurable so that a UE can transmit HARQ-ACK for a PDSCH flexibly based on the QoS requirement of multicast service.

Proposal 1: HARQ-ACK feedback for multicast should be RRC configurable.

Regarding HARQ-ACK for multicast, the followings have been identified as some issues to be discussed/decided.

- ACK/NACK feedback or NACK only feedback

Currently, there are two supported HARQ-ACK feedback modes which can be considered for MBS service, i.e., ACK/NACK feedback and NACK only feedback.

- Option 1: dedicated resource for ACK/NACK feedback

  - For the ACK/NACK feedback, a UE will generate ACK or NACK for a TB based on the TB decoding result, and the PUCCH resources for HARQ-ACK feedback needs to be orthogonal among different UEs. gNB can fully get the PDSCH transmission states for each UE. Therefore, it is natural that MBS PDSCH retransmission can be based on unicast PDSCH per UE basis. However, as discussed in our companion contribution [4], it will result in lager specification impact for MBS transmission with PTM scheme 1, while it is easy to support if PTM scheme 2 is supported for MBS transmission.

- Option 2: common resource for NACK only feedback

  - For the NACK only mode, only when a TB is decoding failed, a UE needs to transmit a NACK. For this feedback mode, the PUCCH resources for HARQ-ACK feedback among different UEs can be group common. Thus, it can be benefit from PUCCH resource consumption. However, gNB is unable to tell which UEs or how many UEs are transmitting NACK due to the group common resource. Thus, it is not possible to support MBS PDSCH retransmission through unicast PDSCH. gNB can’t tell it is ACK or DTX even when the feedback resource is UE-specific. In addition, it will bring some additional issues when considering multiplexing between NACK for multicast PDSCH and other UCI, such as HARQ-ACK for unicast PDSCH, CSI, SR or multiplexing with PUSCH.

Some comparisons of these two HARQ-ACK feedback modes are summarized in the following Table 1.

Table 1 Comparisons of two different HARQ-ACK feedback modes

| HARQ-ACK feedback mode | ACK/NACK feedback | NACK only feedback |
| --- | --- | --- |
| PUCCH resource(s) | Orthogonal among UEs in an MBS group | Same PUCCH resource can be shared among UEs in an MBS group |
| HARQ-ACK information reception for gNB | Full HARQ-ACK information, i.e., ACK, NACK or DTX can be got from each UE. | gNB can determine based on simple energy detection on PUCCH, but gNB can’t be told how many UEs report NACK if the PUCCH resource is group common; gNB can’t determine whether the HARQ of one UE is ACK or DTX if PUCCH resource is UE-specific. |
| MBS PDSCH retransmission | Can be retransmitted by multicast or unicast PDSCH | Can only be retransmitted by multicast PDSCH |
| Multiplexing/prioritization rule of HARQ-ACK for MBS and HARQ-ACK for unicast, CSI, SR or PUSCH | Can be multiplexed with HARQ-ACK for unicast PDSCH and/or other types of UCI, as well as PUSCH transmission. | It’s hard to support multiplexing of HARQ-ACK for MBS and HARQ-ACK for unicast PDSCH and/or other types of UCI, as well as PUSCH transmission. |

To make more comparisons between these two HARQ-ACK feedback modes, system-level simulation was conducted. The evaluation results in terms of spectral efficiency (SE) and resource utilization (RU) for the two HARQ-ACK feedback modes are shown in Table 2. The video traffic with 720p and 30fps is evaluated. UEs belong to same beam are within a one MBS group. The retransmission methods for ACK/NACK feedback mode and NACK only feedback modes are unicast and multicast respectively. Other detailed simulation assumptions are provided in Table 4 in Appendix.

Table 2 Evaluation results for different HARQ-ACK feedback modes

| Feedback mode | Cell average spectral efficiency (bps/Hz/TRP) | Resource utilization |
| --- | --- | --- |
| ACK/NACK feedback | 1.6372 | 0.5248(100%) |
| NACK only feedback | 1.6420 | 0.5535(+5.5%) |

From the simulation results in Table 2, it can be observed that:

- The ACK/NACK feedback mode outperforms with about 5.18% gains in term of RU than the NACK only feedback mode. Compared to the retransmission by group common PDSCH in NACK only feedback mode, the MCS levels of retransmission in ACK/NACK feedback mode are higher due to the UE-specific CSI, which saves more resources, resulting in the lower resource utilization.

- For the cell spectral efficiency, the performances of the two HARQ feedback mode are similar with slight gain of the NACK only feedback scheme mode. The reason of similar results is that both feedback modes can complete the packet transfer in packet interval of generation time, and the gain in SE largely results from the fact that the retransmission MCS of the NACK only feedback mode is lower than that of the ACK/NACK mode, resulting in a lower packet drop probability than that of the ACK/NACK feedback mode.

Observation 1: There is about 5.18% gain in term of RU of the ACK/NACK feedback mode than the NACK only feedback mode.

Observation 2: The NACK only feedback mode has slight gain in terms of SE compared with the ACK/NACK feedback mode.

From the above simulation, it can be observed that using PTP for retransmission can get some gains comparing with using PTM for retransmission. That is the motivation to support ACK/NACK for group-common PDSCH. It is noted that RAN2 has agreed to support dynamic switching between PTP transmission and PTM transmission. This can somehow get similar gain as using PTP transmission for retransmission. To compare different scheduling schemes, another system-level simulation was conducted. In the simulation, three scheduling schemes as followings are evaluated.

- Scheme 1: using PTM for initial transmission and the retransmission is based on unicast only

- Scheme 2: using PTM for initial transmission and the retransmission is based on dynamic adaption between unicast and multicast

- Scheme 3: for initial transmission, dynamic adaption between unicast and multicast; For retransmission, unicast is used if the initial transmission is unicast based, and multicast is used if the initial transmission is multicast based.

In scheme 3, the UE is selected to use unicast for initial transmission when the following conditions are met simultaneously:

- The UE’s transmission MCS is the minimum one in the group.

- The deviation to second lowest MCS in the group is larger than or equal to a threshold, e.g. three MCS grade.

- The UE’s transmission MCS is lower than a fix MCS grade, e.g., MCS 4.

The evaluation results in terms of SE and RU for the three scheduling schemes are shown in Table 3. The general simulation parameters are same shown in Table 3 except that the wide beam of [96,0] is used for BS configuration.

Table 3 Evaluations results for different scheduling schemes

| Scheduling scheme | Cell average spectral efficiency(bps/Hz/TRP) | Resource utilization |
| --- | --- | --- |
| Scheme 1 Multicast init + unicast ReTx | 1.6191 | 0.4046 |
| Scheme 2 Multicast init + dynamic unicast/multicast ReT | 1.6197 | 0.3989 |
| Scheme 3 Dynamic unicast/multicast for init and ReTx | 1.6177 | 0.3949 |

From the simulation result, in terms of cell average spectral efficiency, it is very close among the three scheduling schemes. However, slightly more gains can be observed from the resource utilization of scheme 3 compared to the scheme 1 and scheme 2, which implies that the ACK/NACK feedback for PTM transmission can’t bring more benefits when comparing with dynamic switching between PTP and PTM for initial transmission.

Observation 3: The cell average spectral efficiency between different scheduling schemes, e.g., dynamic unicast/multicast or multicast for initial transmission, dynamic unicast/multicast or unicast for retransmission, are very close to each other.

Observation 4: The resource utilization of the dynamic unicast/multicast for initial transmission + NACK only outperforms PTM transmission scheme 1 with ACK/NACK feedback.

Based on the analysis, we can find that ACK/NACK feedback for PTM transmission and get some gains in terms of RU comparing with NACK only feedback. However, its performance is less than that of dynamic switching between PTP and PTM for initial transmission which has been supported by RAN2. Considering the fact that ACK/NACK feedback for PTM transmission scheme 1 will result in lager specification impact, while it is easy to support for PTM transmission scheme 2, the following proposal is made.

Proposal 2: For RRC_CONNECTED UE,

- NACK only feedback is support for multicast.

- ACK/NACK feedback is not supported for multicast with PTM transmission scheme 1.

- ACK/NACK feedback can be supported for multicast with PTM transmission scheme 2 if PTM transmission scheme 2 is supported for MBS transmission.

For RRC_CONNECTED UEs receiving multicast, for ACK/NACK based HARQ-ACK feedback if supported for PTM scheme 2, PUCCH resource configuration for HARQ-ACK feedback from per UE perspective can be shared with PUCCH resource configuration for HARQ-ACK feedback for unicast if it is configured.

- HARQ-ACK feedback is per PDSCH or is in aggregated manner.

In NR, for unicast PDSCH transmission, 1-bit/2-bit HAR-ACK information for each unicast PDSCH is transmitted, which depends on the number of TBs of each PDSCH and whether the UE is configured with spatial bundling. Noting that, for MBS PDSCH transmission, the level of reliability is based on the requirements of the application/service provided. In other words, it may not be necessary to support PDSCH retransmission for a certain MBS service and HARQ-ACK feedback for link adaption is enough in that case. If multiple HARQ-ACK transmissions are in the same slot, the HARQ-ACK feedback payload may be very large. In order to save HARQ-ACK payload, aggregated HARQ-ACK feedback, i.e., 1-bit HARQ-ACK for multiple PDSCHs associated with the same PUCCH, can be considered for HARQ-ACK payload offload and power consumption reduction.

- HARQ-ACK codebook for multicast PDSCH

In the last meeting, it was agreed that for ACK/NACK based HARQ-ACK feedback if supported, both type 1 and type 2 HARQ-ACK codebook are supported for RRC_CONNECTED UEs receiving multicast.

For type 1 HARQ-ACK codebook, the existing codebook construction mechanism can be a starting point. More specifically, for the case that UE is configured with no unicast service and no more than one FDMed multicast PDSCH, the existing mechanism can be reused to construct a codebook. For the case UE is also configured with unicast service, the codebook construction may depend on the discussion of multiplexing between HARQ-ACK for multicast PDSCH and unicast PDSCH. If multiplexing is supported, it also has relation with the multiplexing scheme between unicast PDSCH and multicast PDSCH. For example, if a UE is configured to receive unicast PDSCH and multicast PDSCH in a slot in FDM manner, the UE may need to construct the HARQ-ACK codebook for unicast PDSCH and multicast PDSCH separately, and then concatenate the two HARQ-ACK codebooks together. Similarly, for the case UE is configured to receive more than one multicast PDSCH in a slot in FMD manner, UE may also need to construct the HARQ-ACK codebook for different multicast PDSCHs separately, and then concatenate these HARQ-ACK codebooks together.

For type 2 HARQ-ACK codebook, the codebook construction may depend on the scheduling scheme for group-common PDSCH. If multicast scheduling is based on PTM scheme 2, then for type 2 HARQ-ACK codebook, the current mechanism can be reused. If ACK/NACK for PTM scheme 1 for multicast is supported, then if UE is configured with multiple MBS services or simultaneous receptions of unicast and multicast, separate DAI counting for different g-RNTI or separate DAI counting for unicast and multicast is needed considering that different UEs may be configured with different g-RNTIs or have different unicast receptions.

In addition, enhanced type 2 and type 3 HARQ-ACK codebook should not be precluded. According to NRU discussion, enhanced type 2 and type 3 HARQ-ACK codebook are extended to licensed band. For MBS, these two types of codebook can also be used. For example, gNB can trigger UE to feedback type 3 HARQ-ACK codebook to get the information of PDSCH reception states in a period and use this information for link adaption. Another example for enhanced type 2 codebook, gNB can group multicast PDSCH as group 0 and unicast PDSCH as group 1, and then gNB can trigger UE to feedback HARQ-ACK for multicast PDSCH by UE-specific PDCCH, which can solve the issue that group common PDCCH can't indicate UE-specific PUCCH resource with one PRI.

Proposal 3: For ACK/NACK based HARQ-ACK feedback if supported, for type 1 HARQ-ACK codebook for RRC_CONNECTED UEs receiving multicast.

- Reuse the existing mechanism to construct a HARQ-ACK codebook for multicast PDSCHs.

- If UE configured to receive multiple FDM PDSCHs in a slot, UE constructs a HARQ-ACK codebook for these multiple PDSCHs separately and then concatenate the codebooks together.

Proposal 4: For ACK/NACK based HARQ-ACK feedback if supported, for type 2 HARQ-ACK codebook for RRC_CONNECTED UEs receiving multicast.

- Reuse the existing mechanism to construct a HARQ-ACK codebook for multicast PDSCHs.

- If UE is configured with multiple g-RNTIs or UE is configured with simultaneous unicast PDSCH reception,

  - If PTM transmission scheme 1 is used for group-common PDSCH, separate DAI counting for different g-RNTIs or g-RNTI and c-RNTI is used.

- If PTM transmission scheme 2 is used for group-common PDSCH, the existing mechanism can be reused to construct a HARQ-ACK codebook for different PDSCHs.

Proposal 5: For ACK/NACK based HARQ-ACK feedback if supported, support enhanced type 2 and type 3 HARQ-ACK codebook for RRC_CONNECTED UEs receiving multicast.

- Whether HARQ-ACK for multicast PDSCH and unicast PDSCH be multiplexed in one HARQ-ACK codebook or not

In NR, a UE will construct one HARQ-ACK codebook in one slot if multiple unicast PDSCHs are scheduled to be transmitted in the same slot, and the UE would transmit the HARQ-ACK codebook on one PUCCH determined based on the HARQ-ACK payload and PRI indicated by the last DCI. One issue needs to be discussed is whether HARQ-ACK for multicast PDSCH and unicast PDSCH can be multiplexed in one HARQ-ACK codebook or not. Some potential issues as follows are identified if multiplexing is supported.

- For type 1 HARQ-ACK codebook, a UE will construct the HARQ-ACK codebook based on the TDRA table of PDSCH transmission, if different TDRA tables are used for multicast PDSCH and unicast PDSCH, when determining candidate PDSCH reception occasions, it may need to take the union of TDRA tables for multicast and unicast PDSCH.

- In addition, type 1 codebook can’t support HARQ-ACK feedback for FDMed PDSCHs, then separate construction of HAREQ-ACK codebooks for multicast PDSCH and unicast PDSCH may be needed if they are transmitted in FDMed manner.

- For type 2 HARQ-ACK codebook, DAI is needed to guarantee same understanding on HARQ-ACK payload and order between gNB and UE. If multiplexing of HARQ-ACK for multicast PDSCH and unicast PDSCH in one HARQ-ACK codebook is supported, separate DAI counting may be needed since a UE can be interested in one or multiple MBS services.

- For the determination of PUCCH resource, it is possible that the DCI for multicast PDSCH is the last DCI, so the determination of PUCCH resource may need to be discussed.

Proposal 6: Regarding HARQ-ACK for multicast PDSCH, the followings need to be discussed/decided.

- Aggregated HARQ-ACK feedback, i.e., 1-bit HARQ-ACK for multiple PDSCHs can be considered

- In case of simultaneous multicast and unicast traffic for the same UE, whether HARQ-ACK for multicast PDSCH and unicast PDSCH can be multiplexed in one HARQ-ACK CB

- Conclusion

In this contribution, we make discussions on the mechanisms to improve reliability for RRC_CONNECTED UEs, and have the following observations and proposals.

Observation 1: There is about 5.18% gain in term of RU of the ACK/NACK feedback mode than the NACK only feedback mode.

Observation 2: The NACK only feedback mode has slight gain in terms of SE compared with the ACK/NACK feedback mode.

Observation 3: The cell average spectral efficiency between different scheduling schemes, e.g., dynamic unicast/multicast or multicast for initial transmission, dynamic unicast/multicast or unicast for retransmission, are very close to each other.

Observation 4: The resource utilization of the dynamic unicast/multicast for initial transmission + NACK only outperforms PTM transmission scheme 1 with ACK/NACK feedback.

Proposal 1: For PDSCH repetition of group-common PDSCH,

- NR Rel-15/16 existing options (i.e., RRC or DCI) for indicating the repetition number should be adopted.

- The PDSCH repetition numbers for different MBS services should be separately configured.

Proposal 2: For RRC_CONNECTED UE,

- NACK only feedback is support for multicast.

- ACK/NACK feedback is not supported for multicast with PTM transmission scheme 1.

- ACK/NACK feedback can be supported for multicast with PTM transmission scheme 2 if PTM transmission scheme 2 is supported for MBS transmission.

Proposal 3: For ACK/NACK based HARQ-ACK feedback if supported, for type 1 HARQ-ACK codebook for RRC_CONNECTED UEs receiving multicast.

Proposal 4: For ACK/NACK based HARQ-ACK feedback if supported, for type 2 HARQ-ACK codebook for RRC_CONNECTED UEs receiving multicast.

- Reuse the existing mechanism to construct a HARQ-ACK codebook for multicast PDSCHs.

- If UE is configured with multiple g-RNTIs or UE is configured with simultaneous unicast PDSCH reception,

  - If PTM transmission scheme 1 is used for group-common PDSCH, separate DAI counting for different g-RNTIs or g-RNTI and c-RNTI is used.

- If PTM transmission scheme 2 is used for group-common PDSCH, the existing mechanism can be reused to construct a HARQ-ACK codebook for different PDSCHs.

Proposal 5: For ACK/NACK based HARQ-ACK feedback if supported, support enhanced type 2 and type 3 HARQ-ACK codebook for RRC_CONNECTED UEs receiving multicast.

Proposal 6: Regarding HARQ-ACK for multicast PDSCH, the followings need to be discussed/decided.

- Aggregated HARQ-ACK feedback, i.e., 1-bit HARQ-ACK for multiple PDSCHs can be considered

- In case of simultaneous multicast and unicast traffic for the same UE, whether HARQ-ACK for multicast PDSCH and unicast PDSCH can be multiplexed in one HARQ-ACK CB

- References

- RP-193248, ‘New Work Item on NR support of Multicast and Broadcast Services’, Huawei

- RAN WG1 #102 e-meeting Chairman’s Notes

- RAN WG1 #103 e-meeting Chairman’s Notes

- R1-2100469, ‘Discussion on mechanisms to support group scheduling for RRC_CONNECTED UEs’, vivo

- Appendix

Table 3 Simulation assumptions for evaluation of multicast

| Parameters | Value |
| --- | --- |
| Layout | UMa in TR38.901 |
| Inter-BS distance | 500 m |
| Carrier frequency | 4GHz |
| Simulation bandwidth | 20 MHz, 8.2% Guard Band |
| Frame structure | DDDDDDDSUU, 10:2:2 for S |
| SCS / Cyclic Prefix | 30 KHz / NCP |
| Channel model | UMa in TR 38.901 |
| BS antenna configuration | (M, N, P, Mg, Ng; Mp, Np) = (8, 8, 2, 1, 1; 1, 8) (dH,dV) = (0.5, 0.8)λ +45°, -45° polarization |
| UE antenna configuration | (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 2) (dH,dV) = (0.5, N/A)λ 0°,90° polarization |
| BS beam set | For direction of BS analog beam steering (in LCS): Azimuth angle φi = [0] Zenith angle θj = [90 95 100 105] degree NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. |
| Traffic model | Traffic mode: video Traffic frequency: 30fps Traffic Image resolution: 720*1280p |
| Transmit power | 49 dBm at BS |
| Antenna height | asign with TR38.901 |
| Antenna gain | 8 dBi at BS and 0 dBi at UE |
| Noise figure | 9 dB at UE |
| UE distribution | 80% indoor, with 3km/h 20% outdoor, with 3 km/h |
| Number of UEs per cell | 10 |
| Schedule method | Beam round robin and TDM scheduled |
| Schedule granularity | Unicast: subband; Multicast: wideband |
| PMI CQI feedback granularity | Unicast: subband; Multicast: wideband |
| Max retransmission time | 4 |
| MCS table | EMBB_CP_OFDM_64QAM |
| Receiver | MMSE-IRC |