3GPP TSG RAN WG1 #104-e		                                    		R1-2100572

e-Meeting, January 25 – February 5, 2021

Agenda Item:	8.14.2

Source:	InterDigital Inc.

Title:	Discussion on Evaluation Methodologies for XR applications

Document for:	Discussion and Decision

# Introduction

In RAN#103-e meeting, several agreements were made on evaluation assumptions for XR [1], including.

Agreement 3: Adopt the following deployment for XR/CG evaluations - Indoor hotspot: FR1 and FR2 - Detailed definition of Indoor hotspot refers to TR 38.913. - Channel model: InH. Detailed definition of InH refers to TR 38.901. - Dense urban: FR1 and FR2 - Detailed deployment refers to TR 38.913, where single layer with Marco layer is assumed. - Channel model: UMi. Detailed definition of UMi refers to TR 38.901. FFS: Whether to prioritize FR1 for evaluation. Note 1: When selecting the deployment and evaluation assumptions for XR/CG evaluations, it is up to company to evaluate FR1 or FR2 or both for the frequency range. Note 2: It does not mean that all applications are evaluated for all the deployment scenarios. Agreement 4: Urban Macro can be reported for XR/CG evaluations only for FR1. - FFS: whether Uma is optional or not - Following parameters can be assumed. Parameter; Proposed value. Urban Macro (FR1). Layout; 21cells with wraparound ISD = 500 m. BS Tx power; FR1: 49 dBm/20 MHz. Agreement 6: System capacity is defined as the maximum number of users per cell with at least X % of UEs being satisfied. - X=90 (baseline) or 95 (optional) - Other values of X can also be evaluated optionally Note: The exact ‘satisfied’ requirements will be discussed separately FFS: how to calculate the percentage of satisfied users across multiple drops of simulations Agreement 8: Adopt the following UE distribution for XR/CG evaluation for outdoor scenario - For outdoor scenario: - FR1: 80% indoor, 20% outdoor - FR2: 100% outdoor Other UE distribution can be evaluated optionally. Agreement 12: Adopt the simulation assumptions in table 3 as below Table 3: Simulation assumptions for XR evaluation (Part 3) Power control parameter; Companies should report. Transmission scheme; Companies should report. Scheduler; SU/MU-MIMO PF scheduler (company to report SU or MU), other scheduler (e.g., delay aware scheduler) is up to companies report. CSI acquisition; Realistic Both CSI feedback and SRS are considered Companies should report • CSI feedback delay, CSI report periodicity, whether using CSI quantization, CSI error model or not, • Assumptions on SRS: periodicity, processing gain, processing delay, etc. PHY processing delay; Baseline: UE PDSCH processing Capability #1 Optional: UE PDSCH processing Capability #2 Companies should report gNB processing delay, e.g. DL NACK to retransmission delay, UL previous transmission to current transmission delay and etc.. PDCCH overhead; Companies should report. DMRS overhead; Companies should report. Target BLER; Companies should report. Max HARQ transmission; Companies should report. Agreement 13: The following aspects are to be discussed after traffic model is stable. - For the system capacity definition, how to determine whether a UE is satisfied or not is to be deferred until the exact traffic model along with how to measure E2E user experience is available. Additional metrics to be collected will be further discussed after traffic model is stable. - Various options for traffic arrival offset among UEs per cell were proposed by companies, e.g., even offset, random offset, no offset. It will be discussed after traffic model is determined. Agreement 20. - RAN1 continues to discuss evaluation methodologies for UE power consumption and system capacity. - RAN1 is to discuss whether/how to study/evaluate mobility and coverage at a later stage, e.g., starting from Q1 2021.

.

In additions to the agreements on traffic models and on AR1/AR2 applications in SA4 [2], a number of issues on evaluation assumptions have been left open for future discussion. In this contribution, we provide our view on some of the remaining issues on evaluation methodologies as well as simulation assumptions.

# KPIs and Assumptions for Evaluations

The traffic characteristics for majority of the XR use-cases described in SID [2] indicate medium to very high throughput requirement both in DL and UL. RAN evaluation for XR and Cloud Gaming traffic over NR should consider capacity, coverage, UE power consumption and mobility (e.g. low) as KPIs while taking into account of various traffic requirements including throughput, latency and reliability. The traffic characteristics for each use-case determines the evaluation assumptions and methodology.

5G-NR

Control Channel

gNB

Data Channel

Remote XR Server

Edge/Cloud Services

XR Devices

For network evaluation, the traffic models need to be simplified as a parametric input for system-level and link-level simulations. In the illustration in Fig.1, XR devices can include a wide class with various capabilities, all of which operate under a given traffic characteristics and network performance requirement (throughput, latency, reliability). However, it is also assumed that a single XR device may exchange multiple parallel traffic streams with different characteristics and requirements. It might, therefore, be necessary to put in place characteristic of a traffic stream, to/from a single user, which multiplexes eMBB as a special use case.

UE

Figure 1  Simplified Network deployment illustration for link-level evaluation

MPEG MIV

Content delivery/Rendering

Services,

Content

Traffic model generator

MPEG MIV

Matching RAN KPI  to

QoE

TX Scheduler

Assumptions on air interface

Transmitter

Channel

RX Scheduler

Reciever

Throughput, BLER

Link-level and system-level evaluations require simplified and parametric input characterizing the XR-traffic. Since a single traffic model cannot characterize all XR traffics, RAN1 should consider proposals to translate the requirements and traffic models for each XR use case detailed in our companion contribution [3] to a parametric traffic model that can be used to evaluate network deployment scenarios and E2E link performance. The traffic model generator provides the input to RAN link-level evaluation, which directly impacts the assumption on the scheduler, is illustrated in Figure 2 below. The scheduler setting during RAN evaluation is based on the output of the traffic model generator. A generalized/parametric XR traffic model generator with configurable parameters as described in [3], can be used for all XR applications to obtain parameters required in the scheduler.

Figure 2: Block diagram for link evaluation

Figure 2: Block diagram for link evaluation

# Remaining Issues on Evaluations Methodologies

## 3.1 System Capacity

System level simulation will be used to evaluate system capacity which, according to the agreements in RAN#103-e [1], is defined as the maximum number of users per cell with at least X % of UEs being satisfied. The agreement on system capacity furthermore indicates that X = 90 can be the baseline with X=95 being optional. The requirement for satisfied users can be specified by a measure of link quality (KPI) evaluated for each XR application based on their traffic parameters such as latency settings, frame size and rate, code rate, etc.

Proposal 1: 	Given the XR applications, define user experience satisfaction by the percentage (70%, 90%) of achieved maximum throughput, which is derived from the traffic parameters of the XR application.

With the definition of satisfied UEs in Proposal 1, evaluation of percentage of satisfied UEs can be done by analyzing CDF of UEs achieving an average throughput above a given value in b/s from which the percentage of satisfied UEs can be determined.

Proposal 2: 	Evaluate capacity for XR applications by analyzing CDF of UEs achieving an average throughput that is at least equal to the required throughput for the given XR application.

## 3.2 Coverage

In RAN #103-e, it was agreed that discussions on evaluation of coverage performance for XR is FFS starting form Q1. For most XR applications, the majority consensus is that the evaluation assumptions developed in Rel-17 coverage enhancement can be reused. Moreover, coverage evaluation should take into account the tradeoff to the % of satisfied UEs (capacity) as well as power consumption.

Proposal 3: 	For the XR applications, consider reusing evaluation methodologies discussed in Rel-17 coverage enhancement for coverage evaluation [8].

Evaluation of coverage shall consider various degree of mobility. Even though separate detailed discussion on evaluating mobility performance should not be prioritized at this time, coverage evaluation can consider prioritizing low mobility scenario, especially for supporting AR1/AR2 applications.

## 3.3 UE Power Consumption

Both capacity and coverage performance evaluations are impacted by the power saving technique assumed. For the evaluations of capacity, both random offset and uniform offset can be considered. The case for using different offsets for traffic arrival may be useful for determining the tradeoff between capacity and UE power savings. Link-Level evaluation of UE power consumption which also takes into account evaluation of throughput as well as latency is therefore necessary to evaluate baseline performance. The various aspects of UE settings which affect power consumption, such as the link adaptation technique, transmission scheme and scheduling techniques are considered. For analyzing performance of power saving techniques (e.g., DRX configuration, BWP switching, scheduling techniques, etc.) system level simulation can be used. As a start, power consumption evaluation methodology specified in TR38.840 [6] can be employed. However, discussion still need to be had on the tradeoff between power consumption vs. capacity, how much priority UE power consumption evaluation need to be given compared to other evaluations and on how to prioritize SL over LL evaluation or vice versa.

Proposal 4: 	Study aspects related to tradeoff between UE power savings and capacity in SL and LL evaluations

# Remaining Issues on Simulation Assumptions

In this section, we lay out some of the remaining assumption which have been left open for both link-level and system level evaluations. Most XR-use cases/applications are expected to have stringent requirements for reliability similar to Rel-16 URLLC [7]. However, unlike in URLLC, where evaluation of reliability is typically under low load assumption, evaluation of reliability for XR/CG use cases should also include high load assumption. Moreover, link-level evaluation should include multiplexing of different data streams in a single carrier with different requirements for reliability and latency requiring a new set of traffic models.

## 4.1 Link level evaluation

Link-level evaluation is used to evaluate performance of the air interface to fulfil a given user throughput requirement with associated requirement on reliability and latency for various assumptions. In Table 1 below, we give the major assumptions which have been agreed upon and include remaining assumptions which have been left for further discussion.

Table 1: Assumptions for Link-level evaluation of DL data channel

| Parameter | FR1 | FR2 |
| --- | --- | --- |
| Duplexing | TDD | TDD |
| TDD pattern | - DDDSU - DDDUU | - DDDSU |
| gNB Antenna configuration | 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1,8,2) 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) | 2 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (4,8,2,2,2;1,1) |
| UE antenna configuration | [M, N, P, Mg, Ng; Mp, Np]=[1, 2, 2, 1, 1; 1, 2] | [M, N, P, Mg, Ng; Mp, Np]=[1, 1, 2, 1, 1; 1, 1] |
| Carrier frequency | 4 GHz | 30 GHz |
| System BW | 100 MHz | 400 MHz |
| MCS | QPSK, 16-QAM, 64-QAM, 256-QAM |  |
| ScS | 30kHz | 120kHz |
| Propagation condition | CDL-A/C |  |
| Delay Spread | Up to 300ns | Up to 30ns |
| DMRS | 1 + 1 |  |
| Channel estimation | Realistic |  |
| Precoder | Companies should report |  |
| UE Velocity | 3km/h |  |
| HARQ config. (Max # of retransmission) | Up to 2 for start |  |
| Target BLER | 0.1% - 10% |  |
| Receiver Type | MMSE |  |

Proposal 5: 	Reuse evaluations assumption including Scheduling/HARQ/CSI processing used for Rel-16 URLLC, even in high load scenarios where requirement on reliability is stringent.

Proposal 6: 	Configurations for scheduling/HARQ/CSI processing should be based on the power control parameters under consideration.

## 4.2 System level evaluation

System level simulation will evaluate capacity and coverage performance for various deployment scenarios, UE distribution, traffic model and requirements. Depending on the specific XR use case considered, the evaluation assumptions should take into account of the expected high UL/DL throughput in most XR applications compared to eMBB, which can consequently impact the coverage and capacity performance. Several specific assumptions related to evaluation of capacity employing system level evaluation are deferred pending the completion of stable XR traffic model by SA4. Capacity be evaluated for both indoor hotspot and dense Urban channel models for both FR-1 and FR-2.

Table 2: Assumptions for System-level simulation for capacity

| Parameters | Indoor hotspot FR1/FR2 |  | Dense urban FR1/FR2 |  |
| --- | --- | --- | --- | --- |
|  | FR1 | FR2 | FR1 | FR2 |
| Layout | 120m x 50m ISD: 20m TRP numbers: 12 |  | 21cells with wraparound ISD: 200m |  |
| Carrier frequency | 4GHz | 30GHz | 4GHz | 30GHz |
| scs | 30kHz | 120kHz | 30kHz | 120kHz |
| BSTx power | 24 dBm | 23dBm | 44dBm | 40dBm |
| BW | 100 MHz | 80Mz |  |  |
| BS antenna configuration | [M, N, P, Mg, Ng; Mp, Np] = (4,4,2,1,1;4,4) | [M, N, P, Mg, Ng; Mp, Np] = (16, 8, 2,1,1;1,1) | (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1,8,2) | (M, N, P, Mg, Ng; Mp, Np) = (4,8,2,2,2;1,1) |
| BS antenna height | 3m |  | 25m |  |
| BS antenna tilt | 90 |  | 12 |  |
| BS antenna element gain | 5 dBi, |  | 8 dBi |  |
| BS receiver noise figure | 5 dB | 7 dB | 5 dB | 7 dB |
| UE antenna height | 1.5 m |  |  |  |
| UE antenna gain | 0 dBi | 5 dBi | 0 dBi | 5dBi |
| UE noise figure | 9 dB | 13 dB | 9 dB | 13 dB |
| Traffic model | Employ Traffic model specified in Table 1 of an accompanying contribution [3] for each XR application groups. |  |  |  |
| UE distribution | 100% Indoors |  | 20% outdoors 80% Indoors | 100% Outdoors |
| BS and UE receiver | MMSE-IRC |  |  |  |
| Channel estimation | Practical |  |  |  |

Additional simulation parameters for BS and UE antenna radiation pattern and taken from TR 38.901 [7] are given in Table 3.1 and Table 3.2. Table 3.2 applies for FR-2 only.

Table 3.1 Antenna radiation pattern for BS

| Vertical cut of the radiation power pattern (dB) |  |
| --- | --- |
| Horizontal cut of the radiation power pattern (dB) |  |
| 3D radiation power pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5 dBi for FR1 and 8 dBi for FR2 |

Table 3.2 Antenna radiation pattern for UE

| Parameter | Values |
| --- | --- |
| Antenna element radiation pattern in θ'' dim (dB) |  |
| Antenna element radiation pattern in φ'' dim (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |

System level simulation for coverage performance can use simulation assumptions specified for Rel-17 Coverage enhancement work [8].

Proposal 7: 	As a start, re-use evaluation assumptions for system level simulation applied in Rel-17 coverage enhancement for coverage evaluation [8].

# Conclusion.

In this contribution, the following proposals are put forward:

Proposal 1: 	Given the XR applications, define user experience satisfaction by the percentage (70%, 90%) of achieved maximum throughput, which is derived from the traffic parameters of the XR application.

Proposal 2: 	Evaluate capacity for XR applications by analyzing CDF of UEs achieving an average throughput that is at least equal to the required throughput for the given XR application.

Proposal 3: 	For the XR applications, consider reusing evaluation methodologies discussed in Rel-17 coverage enhancement for coverage evaluation [8].

Proposal 4: 	Study aspects related to tradeoff between UE power savings and capacity in SL and LL evaluations

Proposal 5: 	Reuse evaluations assumption including Scheduling/HARQ/CSI processing used for Rel-16 URLLC, even in high load scenarios where requirement on reliability is stringent.

Proposal 6: 	Configurations for scheduling/HARQ/CSI processing should be based on the power control parameters under consideration.

Proposal 7: 	As a start, re-use evaluation assumptions for system level simulation applied in Rel-17 coverage enhancement for coverage evaluation [8].

# References

RP-201145, Rel-17 SID on Study on XR Evaluations, Qualcomm, June 2020

RAN1 Chairman notes, RAN1#103-e, Dec 2020

R1-2100571, Discussion on XR applications and traffic models, InterDigital, Jan 2021

Tdoc S4aV200617 FS_XRTraffic: Parameters for XR conversational, Dec 2020

3GPP TR 26.928, Extended Reality (XR) in 5G, v.16.0.0, Mar. 2020

3GPP TR 38.830 Study on User Equipment (UE) power saving in NR

3GPP TR 38.824, Study on physical layer enhancements for NR ultra-reliable and low latency case (URLLC)

3GPP TR 38.830 Study on NR Coverage Enhancements

3GPP TR 38.901 Study on channel model for frequencies from 0.5 to 100 GHz