3GPP TSG RAN WG1 #104-e		R1-2100583

e-Meeting, January 25th – February 5th, 2021

Agenda Item: 8.1.4

Source: MediaTek Inc.

Title: CSI enhancement for NCJT and FR1 FDD reciprocity

Document for: Discussion & Decision

# Introduction

The objective for this agenda item, stated in [1], is given by

Enhancement on CSI measurement and reporting:

a.	Evaluate and, if needed, specify CSI reporting for DL multi-TRP and/or multi-panel transmission to enable more dynamic channel/interference hypotheses for NCJT, targeting both FR1 and FR2

b.	Evaluate and, if needed, specify Type II port selection codebook enhancement (based on Rel.15/16 Type II port selection) where information related to angle(s) and delay(s) are estimated at the gNB based on SRS by utilizing DL/UL reciprocity of angle and delay, and the remaining DL CSI is reported by the UE, mainly targeting FDD FR1 to achieve better trade-off among UE complexity, performance and reporting overhead

In this contribution, we continue the discussion of CSI enhancement based on the outcome of the RAN1#102e and #103e meetings [2][3].

# CSI Enhancement: NCJT

In the RAN1#103e meeting, we have the following agreement on CMR/IMR configuration/association [3]:

Agreement For NCJT CSI measurement configured with single reporting setting, study following measurement resource configuration/association mechanism - Whether/how to support interference measurement based on NZP CSI-RS given by nzp-CSI-RS-ResourcesForInterference or based on CSI-IM given by csi-IM-ResourcesForInterference - Whether/how to interpret measurement based on CMRs associated with different TRPs/TCI states respectively for a NCJT measurement hypothesis - CMR/IMR resource configuration restrictions/associations, e.g. for reference resource/time domain behavior/frequency domain behavior Note that RAN1 shall strive for commonality of CSI measurement/reporting mechanisms for NCJT CSI measurement configured by single or two reporting settings

We do not see a strong need of joint operation of multi-TRP with NCJT and MU-MIMO, so it suffices to support interference measurement based on CSI-IM given by csi-IM-ResourcesForInterference in order to tackle inter-cell interference.

Proposal 1: For NCJT CSI measurement configured with single reporting setting, only support interference measurement based on CSI-IM.

TS 38.214 says that “For CSI measurement(s) other than L1-SINR, a UE assumes each NZP CSI-RS port configured for interference measurement corresponds to an interference transmission layer.” That is, UEs assume the measured interference as is without any additional assumption, e.g., on precoders. Since for NCJT a UE can assist the gNB determining precoders for each TRP, technically all layers from an NCJT transmission are desired signals, rather than interference. With or without IMR, so long as UEs reports an RI larger than one, there is naturally inter-layer interference. How to interpret CMRs associated with different TRPs/TCI states should be up to UE implementation, similar to R15/R16 on inter-layer interference.

Proposal 2: How to interpret the two CMRs configured for an NCJT measurement hypothesis can be up to UE implementation.

In the RAN1#103e meeting, we have the following agreement on UE reporting mechanism [3]:

Agreement For a CSI reporting setting, support one or more of the following UE reporting mechanism: - Alt 1: the UE can be expected to report one CSI associated with the best single-TRP measurement hypothesis and one CSI associated with the best NCJT measurement hypothesis, if configured - FFS omission of CSI associated with NCJT measurement hypothesis - Alt 2: the UE can be expected to report one CSI associated with the best one among NCJT and/or single-TRP measurement hypotheses, if configured - FFS how to report recommended measurement hypothesis associated with that CSI report - Alt 3: the UE can be expected to report two CSIs associated with the two best single-TRP measurement hypotheses associated with CMRs from two TRPs and one CSI associated with the best NCJT measurement hypothesis, if configured - FFS omission of CSI associated with NCJT measurement hypothesis - Whether/How to report a subset of the CSI report quantities - FFS: CSI reporting configuration details Note supporting which one or more mechanisms is to be determined in RAN1#104-e

Alt. 1 is flexible for gNB’s scheduling among SU-MIMO with/without NCJT and MU-MIMO. Although Alt. 3 is even more flexible, the incurred overhead may be too large.  On the contrary, Alt. 2 is a good solution to limit the overhead. Support of Alt. 1 and Alt. 2 should be sufficient for most use cases of NCJT.

Proposal 3: For a CSI reporting setting, support Alt. 1 and Alt. 2 as UE reporting mechanism.

For omission of CSI associated with NCJT measurement hypothesis in Alt. 1, there are at least the following three options:

1) Only report the CSI associated with the best single-TRP measurement hypothesis

2) Only report the CSI associated with the best NCJT measurement hypothesis

3) Fall back to Alt. 2, i.e., report one CSI associated with the best one among NCJT and/or single-TRP measurement hypotheses

We note that the payload of Option 1 cannot be larger than Options 2 and 3, so there is no need to further consider the case where the CSI associated with the best NCJT measurement hypothesis cannot be accommodated but the CSI associated with the best single-TRP measurement hypothesis can. Thus, Option 1 is simpler and the gNB can at least get a CSI assuming no additional rules for Options 2 and 3.

Proposal 4: For Alt. 1, the CSI associated with the best NCJT measurement hypothesis has a lower reporting priority than the CSI associated with the best single-TRP measurement hypothesis.

To tackle the issue that the payload sizes for single-TRP and NCJT are different, the CSI can comprise of two parts, similar to the current NR design: Part 1 has a fixed payload size and is used to identify the number of information bits in Part 2. Then, if one of the RIs can be reported as zero, we know that the reported CSI is associated with the best single-TRP measurement hypothesis.

Proposal 5: For Alt. 2, the second RI can be reported as 0 to signal the best single-TRP measurement hypothesis.

In the RAN1#103e meeting, we have the following agreement on reporting quantity [3]:

Agreement For a CSI report associated with a Multi-TRP/panel NCJT measurement hypothesis configured by single CSI reporting setting, the UE is expected to report - two RIs, two PMIs, two LIs and one CQI per codeword, for single-DCI based NCJT when the maximal transmission layers is less than or equal to 4 - FFS: Maximal transmission layers larger than 4 - FFS: Whether/how a subset of above reporting quantities are allowed to be configured to the UE - FFS: whether/how to support two RIs, two PMIs, two LIs and two CQIs, for multi-DCI based NCJT - FFS: whether/how to support CRI(s) to be reported in a CSI - FFS: restrictions among reported CSI quantities, e.g. among reported RIs and PMIs - FFS: whether/how to support non-PMI based port-selection - FFS: whether/how to support single value of reported LI Note that other NCJT CSI measurement/reporting enhancement for other scenarios is not precluded, e.g. for HST-SFN

If multiple NCJT measurement hypotheses and/or multiple single-TRP hypotheses are configured in one CSI-ReportConfig, then it is natural to use CRI(s) to signal the selected measurement hypotheses. Since currently the mapping between CMR and CSI-IM based IMR is one-to-one, we need to introduce a mapping table to map multiple CMRs to a CRI, where the IMRs are mapped to CRIs by the natural index order. An example is given in Figure 1.

Proposal 6:  For an NCJT interference hypothesis, the corresponding CRI is associated with two CMRs, whereas the mapping from CRI to IMR remains one-to-one.

Figure 1: Example of mapping multiple CMRs to CRI

In the RAN1#98 meeting, we have the following agreements:

Agreement For single-DCI based NJCT transmission, at least for eMBB, with regarding to following design principles for DMRS entries: - Principle 1: No consensus to support 1+3 and/or 3+1 layer combinations from two TRPs indicated by antenna port field. - Principle 2: No consensus to have additional specification support for MU cases - Principle 3: No consensus to have additional specification support for two CWs

The reported RIs should be restricted such that there exist the corresponding layer combinations for the DMRS entries. For example, the reported RIs cannot be (1, 3) or (3, 1) since the current DMRS entries do not support these layer combinations. The allowed RI pairs for an NCJT measurement hypothesis assuming the maximal transmission layers less than or equal to 4 should be (1, 1), (1, 2), (2, 1), (2, 2). The same principle should be followed when considering maximal transmission layers larger than 4.

Proposal 7: The allowed RI pairs for an NCJT measurement hypothesis assuming the maximal transmission layers less than or equal to 4 should be (1, 1), (1, 2), (2, 1), (2, 2).

Non-PMI based port-selection should be supported for NCJT to benefit from channel reciprocity, where the precoding matrices are determined based on SRS. To do so, each rank can be associated with two CSI-RS resources with different TCI states. For simplicity, the two CSI-RS resource can have the same number of ports. Then, the port association can be determined by introducing a sub-rank indicator indicating number of layers for the first CSI-RS resource. For example, with the sub-rank indicator 2 and the port index sequence (0, 1, 4, 5), we know that ports 0, 1 of the first CSI-RS resource and ports 4, 5 of the second CSI-RS resource should be used. With the current restriction on RI pairs where the difference between two RIs is at most one, the association can be further simplified: If the rank is even, the first half of indices are of the first CSI-RS resource and the second half are of the second CSI-RS resource. If the rank is odd, then it suffices to have a 0-1 indicator telling which CSI-RS resource is associated with more layers.

Proposal 8: Non-PMI based port-selection is supported for a CSI report associated with an NCJT measurement hypothesis configured by single CSI reporting setting.

Following the above discussion, the higher layer parameter reportQuantity can be 'cri-RI-PMI-CQI ', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI', and 'cri-RI-LI-PMI-CQI' for a CSI report associated with an NCJT measurement hypothesis configured by single CSI reporting setting. In particular, for 'cri-RI-i1' and 'cri-RI-i1-CQI', two i1 values can be reported.

Proposal 9: The higher layer parameter reportQuantity can be 'cri-RI-PMI-CQI ', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI', and 'cri-RI-LI-PMI-CQI' for a CSI report associated with an NCJT measurement hypothesis configured by single CSI reporting setting.

# CSI Enhancement: FR1 FDD Reciprocity

In RAN1 #103-e, the following agreements were made for Rel-17 port selection codebook enhancement in FR1 FDD [3]:

- Port selection codebook enhancements utilizing DL/UL reciprocity of angle and/or delay is supported in Rel-17.

- Study following alternatives, and select one or a combination of multiple alternatives for Rel-17 in RAN1 104-e:

Channel reciprocity usually does not hold in FDD systems because of different operating frequencies of uplink (UL) and downlink (DL) transmission. However, both the UL and DL transmissions share the same propagation environment between the base station and UE. The UL and DL signals propagate along the same paths and are reflected by the same scatterers. Because of this, the spatial directions (angles) and propagation delays are expected to be almost the same in UL and DL directions. The Rel-15 and Rel-16 Type II port selection (PS) codebooks already allow angle domain reciprocity to be exploited for DL CSI estimation and reporting. Utilizing the angle domain information of the UL channel, it is possible for the gNB to beamform CSI-RS towards specific directions. Upon reception of beamformed CSI-RS, UE can select appropriate spatial directions via PS codebook. Further, using enhanced Type II PS codebook, UE can reduce feedback overhead by reporting only a few frequency selective precoder coefficients corresponding to dominant spatial directions and dominant delay taps. The intention of Rel-17 enhancement is to utilize delay reciprocity in addition to angle reciprocity to beamform CSI-RS towards dominant propagation paths, in order to trade-off between performance, UE complexity and reporting overhead [1].

Utilizing delay reciprocity, it is possible for the gNB to pre-compensate the delay taps in each spatial direction (beam) to the zero-lag position using CSI-RS precoding. When the gNB chooses to pre-compensate only a single dominant delay tap (M=1) in each beam, it is equivalent that the UE measures a frequency flat channel over the entire bandwidth. The UE then calculates and reports a single wideband precoder. If the gNB pre-compensates M>1 dominant delay taps in each beam, the channel measured by the UE in the mth precoded CSI-RS port corresponds to the DL channel in the mth delay tap, m=1,…M. The UE can further down-select a subset M1≤M delay taps by means of frequency domain (FD) port selection. In this case, the UE can still calculate a frequency selective precoder, where the frequency selectivity is reduced from N3 to M, N3 being the number of PMI subbands. This reduces the UE complexity and reporting overhead.

Proposal 10: Port selection codebook enhancements utilizing DL/UL reciprocity of angle and delay should be supported in Rel-17.

We will describe the following two cases for pre-compensation per beam: M=1 and M>1 dominant delay tap(s).

## Pre-compensation of M=1 dominant delay tap per beam

  - gNB procedure for precoding CSI-RS

Assume that the NR×P time domain DL channel estimated by the gNB based on the UL channel estimate is

HDLt=λ11δ(t-τ1)λ12δ(t-τ2)⋯λ1Pδ(t-τP)λ21δ(t-τ1)λ22δ(t-τ2)⋯λ2Pδ(t-τP)⋮⋮⋱⋮λNR1δ(t-τ1)λNR2δ(t-τ2)⋯λNRPδ(t-τP),

where NR is the number of receive antenna elements at the UE, P is the number of dominant beams, τp is the single dominant delay in beam p=1,…P, and λip is the complex channel gain between the ith receive antenna and pth beam. With this information, the gNB can shift the dominant delay tap τp in beam p to the zero lag position using the frequency domain dual operation of ej2πτpnc[n]⟷δ(t+τp)c(t), where c[n] is the CSI reference signal in frequency unit n=1,…N3. We can then describe the procedure for CSI-RS precoding as follows:

- gNB obtains the spatial domain (SD) and frequency domain (FD) bases from the UL channel estimate as

WSD=s0s1⋯sP-1∈CNT×P,

WFD=f0f1⋯fP-1∈CN3×P,

where sp∈CNT×1 is the SD basis vector and fp∈CN3×1 is the FD basis vector for the pth beam. NT is the number of transmit antennas at gNB. The nth element of fp is of the form discussed above, i.e., fnp=1N3ej2πτpn, by which the single dominant delay tap τp in beam p is shifted to the zero lag position when received by the UE.

- The NT×P SD-FD precoder for a P port CSI-RS in the nth subband is then obtained as

WSD-FDn=fn0s0fn1s1⋯fn(P-1)sP-1

Figure 2: Example scenario with P=4 dominant beams out of 6 beams

In Figure 2, we illustrate an example scenario with P=4 dominant beams out of 6 beams. The dominant beams have indices 0, 1, 3 and 5. In these dominant beams, the gNB pre-compensates a single delay tap with index 1, 5, 3 and 7 respectively. The 4 dominant SD-FD pairs used for CSI-RS precoding is therefore (0-1), (1-5), (3-3) and (5-7).

  - UE procedure for processing CSI-RS and reporting CSI

With the dominant delay tap in each beam pre-compensated to the zero-lag, the UE can obtain the effective DL channel by summing the channel estimated in all subbands as

Heff=n=1N3HnWSD-FD[n]∈CNR×P,

where Hn∈CNR×NT is the actual DL channel and HnWSD-FD[n] is the UE estimated channel in the nth subband. The UE can further select L≤P dominant beams by means of a free PS matrix W1∈NP×L in a polarization specific manner. Each column of W1 has a single non-zero entry of 1. After selection of L dominant beams, the UE can obtain the matrix W2∈CL×R coefficients linearly combining the L dominant beams with R transmission layers by the SVD of HeffW1 as

HeffW1=UΣVH

where W2 consists of the first R columns of V. The single wideband precoder computed and reported by the UE is W=W1W2. The corresponding subband CQIs are obtained and reported as post-processing functions of HnWSD-FDnW1W2, n=1, …N3.

Using the CSI report from the UE, the gNB can use the effective precoder for data transmission in subband n as WSD-FDnW1W2.

Proposal 11: For the case of a single dominant tap in each beam pre-compensated by the gNB by means of precoded CSI-RS, the codebook structure is W=W1W2, where W1∈NP×L is a free port selection matrix to choose L ports out of P CSI-RS ports in a polarization specific manner and each column of W1 has a single non-zero entry of 1.

  - Simulation results

The simulation assumption follows the agreed EVM assumption in 102e meeting [4]. The detailed assumptions are shown in Appendix. The baseline performance is R16 PS Type II with 8 CSI-RS ports. For Rel-17 design, the port selection ratio β={0.75, 0.5, 0.25, 0.125} is considered. The selected number of port for each layer is L = P × β. The traffic load with RU is 70% and SU/MU-MIMO adaptation with up to 2 rank is adopted in the simulation.

Figure 3 shows Rel-17 design with 32 CSI-RS ports yields about 10% UPT gain at lower feedback overhead regime. However, the gain curve shows a saturation trend at high feedback overhead regime and the performance is worse than Rel-16 PS Type II design. The reason is that only M=1 dominant delay tap is adopted per beam, i.e., only one channel tap delay (FD basis) is considered for each beam (SD basis). In this case, the SD-FD pair is one-to-one mapping where the main beams lose the channel information of second or third channel delay taps. Based on the results, we have the following observation and proposal:

Observation 1: Rel-17 design considering one dominant delay tap per beam can yield 10% UPT gain at the low feedback overhead regime. However in higher feedback overhead regime, the Rel-17 design shows a worse performance than R16 PS Type II.

Observation 2:  Pre-compensation of M=1 dominant delay tap per beam suffers the channel information loss of second or third strongest delay taps. UE feedback of more coefficients on the insignificant beams won’t yield more gain.

Proposal 12: In order to capture more channel information on the main beam, delay pre-compensation of M>1 dominat delay taps per beam should be further investigated in RAN1.

Figure 3: Comparison of R16/R17 PS codebooks

## Pre-compensation of M>1 dominant delay tap per beam

  - gNB procedure for precoding CSI-RS

With more than one dominant delay tap per beam, the NR×P time domain DL channel estimated by the gNB based on the UL channel estimate is

HDLt=m=1Mλ11(m)δ(t-τ1m)m=1Mλ12(m)δ(t-τ2m)⋯m=1Mλ1P(m)δ(t-τPm)m=1Mλ21(m)δ(t-τ1m)m=1Mλ22(m)δ(t-τ2m)⋯m=1Mλ2P(m)δ(t-τPm)⋮⋮⋱⋮m=1MλNR1(m)δ(t-τ1m)m=1MλNR2(m)δ(t-τ2m)⋯m=1MλNRP(m)δ(t-τPm)

where τpm is the mth dominant delay in beam p=1,…P, m=1, …M, and λip(m) is the complex channel gain between the ith receive antenna and pth beam in the mth delay. For pre-compensating a single dominant tap in each beam, we saw that the number of CSI-RS ports needed is P. With M dominant taps in each beam, the number of CSI-RS ports needed is PM, such that the CSI-RS ports m-1P+1, m-1P+2, …mP compensate the mth delay tap in each of the P beams. The corresponding SD-FD precoder for CSI-RS in subband n is

WSD-FD(m)n=fn0(m)s0fn1(m)s1⋯fn(P-1)(m)sP-1

where fnp(m)=1N3ej2πτpmn, n=1, …N3, p=1,…P, m=1, …M shifts the mth delay tap in the pth beam to the zero-lag position.

  - UE procedure for processing and reporting CSI-RS

As seen before, UE can estimate the channel in the M dominant delays in each of the P beams by summing the estimated channel in all subbands. The UE can select L≤P beams using free PS matrix W1∈NP×L in a polarization specific manner. UE can further freely select M1≤M dominant delays using an FD port selection matrix Wf∈NM×M1 specific to each transmission layer. However, the frequency selectivity of the channel measured by UE is reduced from N3 to M, as FD precoding is unknown to the UE. For this reduced frequency selectivity, RAN1 should further discuss the PMI component W2 and CQI computing and reporting mechanisms. It is to be noted that such a study was agreed in RAN1 #102-e [2]. However, companies could not comprehensively discuss this in RAN1 #103-e.

If the FD basis vectors are chosen as DFT vectors (which is assumed throughout the analysis for the case of M>1), indicating the indices of the vectors to the UE can recover the original frequency selectivity of N3, but would lead to DL signaling overhead. Further, UE complexity and reporting overhead are not reduced significantly by doing so, which is contrary to the main aim of Rel-17 enhancement.

Proposal 13: For the case of more than one dominant tap in each beam pre-compensated by the gNB by means of precoded CSI-RS, the codebook structure is W=W1W2WfH, where W1∈NP×L and Wf∈NM×M1 are free SD and FD port selection matrices to choose L out of P beams in a polarization specific manner, and M1 out of M delays, such that each column of W1 and Wf have a single non-zero entry of 1.

Proposal 14: RAN1 should further discuss the PMI component W2 and CQI reporting mechanism considering delay pre-compensation using FDD reciprocity.

For the special case when τpm≈τm for all p=1,…P, the SD-FD precoder in subband n for CSI-RS compensating the mth delay tap in the P beams is

WSD-FD(m)n=fnms0fnms1⋯fnmsP-1

where fnm=1N3ej2πτmn. This means that there is a dominant path on all the P beams at delay τm. The overall SD-FD precoder compensating all the M delays in the P beams then reduces to the form

WSD-FD=WFD⊗WSD∈CNTN3×PM

with WSD=s0s1⋯sP-1∈CNT×P and WFD=f0f1⋯fM-1∈CN3×M. The NT×PM precoder for a PM port CSI-RS in subband n will be then the n-1NT+1 to nNT rows of  WSD-FD. For this case, the SD and FD port selection operations to select L out of P beams and M1 out of M respectively can be represented by WSDW1 and WFDWf. The selected SD-FD bases can then be represented by WFDWf⊗WSDW1∈CNTN3×LM1. By the property of Kronecker product, this can be represented by

WFDWf⊗WSDW1=(WFD⊗WSD)(Wf⊗W1)

The gNB can use the precoder (WFD⊗WSD) for CSI-RS, and the precoder for data transmission has the same structure as Proposal 13.

In this case, the CSI-RS port count needed is high. For example, considering P=8 beams and M=6 delays would lead to a CSI-RS port of PM=48. We feel that increasing the CSI-RS port count to such high numbers is out of the scope of Rel-17 enhancement. To avoid this, and yet maintain the same numbers for P and M for optimal performance, we propose using DFT bases for FD and signaling to the UE the offset of the M-MP FD bases with respect to the MP FD bases used in CSI-RS precoding. With this, the CSI-RS port count would be PMP. To see this, consider a simple example with M=2 FD bases f1 and f2. With DFT bases, the individual elements of f1 and f2 can be related as fn2=ej2πknfn1. By dynamically indicating the value of k to the UE, the gNB can use only a single FD basis f1 for precoding a P port CSI-RS. The precoded CSI-RS for the second FD basis can be derived by the UE from the channel measured in the first P port CSI-RS and the value of k. In this example, MP=1=M/2.

Proposal 15: When M>1 delay taps are pre-compensated by the gNB using precoded CSI-RS in each of the P beams, gNB can use MP DFT FD bases for CSI-RS precoding and indicate the offset of the remaining M-MP FD bases via dynamic signaling to the UE.

# Conclusion

In summary, based on the above discussion we have the following observations and proposals:

Observation 1: Rel-17 design considering one dominant delay tap per beam can yield 10% UPT gain at the low feedback overhead regime. However in higher feedback overhead regime, the Rel-17 design shows a worse performance than R16 PS Type II.

Observation 2:  Pre-compensation of M=1 dominant delay tap per beam suffers the channel information loss of second or third strongest delay taps. UE feedback of more coefficients on the insignificant beams won’t yield more gain.

Proposal 1: For NCJT CSI measurement configured with single reporting setting, only support interference measurement based on CSI-IM.

Proposal 2: How to interpret the two CMRs configured for an NCJT measurement hypothesis can be up to UE implementation.

Proposal 3: For a CSI reporting setting, support Alt. 1 and Alt. 2 as UE reporting mechanism.

Proposal 4: For Alt. 1, the CSI associated with the best NCJT measurement hypothesis has a lower reporting priority than the CSI associated with the best single-TRP measurement hypothesis.

Proposal 5: For Alt. 2, the second RI can be reported as 0 to signal the best single-TRP measurement hypothesis.

Proposal 6:  For an NCJT interference hypothesis, the corresponding CRI is associated with two CMRs, whereas the mapping from CRI to IMR remains one-to-one.

Proposal 7: The allowed RI pairs for an NCJT measurement hypothesis assuming the maximal transmission layers less than or equal to 4 should be (1, 1), (1, 2), (2, 1), (2, 2).

Proposal 8: Non-PMI based port-selection is supported for a CSI report associated with an NCJT measurement hypothesis configured by single CSI reporting setting.

Proposal 9: The higher layer parameter reportQuantity can be 'cri-RI-PMI-CQI ', 'cri-RI-i1', 'cri-RI-i1-CQI', 'cri-RI-CQI', and 'cri-RI-LI-PMI-CQI' for a CSI report associated with an NCJT measurement hypothesis configured by single CSI reporting setting.

Proposal 10: Port selection codebook enhancements utilizing DL/UL reciprocity of angle and delay should be supported in Rel-17.

Proposal 11: For the case of a single dominant tap in each beam pre-compensated by the gNB by means of precoded CSI-RS, the codebook structure is W=W1W2, where W1∈NP×L is a free port selection matrix to choose L ports out of P CSI-RS ports in a polarization specific manner and each column of W1 has a single non-zero entry of 1.

Proposal 12: In order to capture more channel information on the main beam, delay pre-compensation of M>1 dominat delay taps per beam should be further investigated in RAN1.

Proposal 13: For the case of more than one dominant tap in each beam pre-compensated by the gNB by means of precoded CSI-RS, the codebook structure is W=W1W2WfH, where W1∈NP×L and Wf∈NM×M1 are free SD and FD port selection matrices to choose L out of P beams in a polarization specific manner, and M1 out of M delays, such that each column of W1 and Wf have a single non-zero entry of 1.

Proposal 14: RAN1 should further discuss the PMI component W2 and CQI reporting mechanism considering delay pre-compensation using FDD reciprocity.

Proposal 15: When M>1 delay taps are pre-compensated by the gNB using precoded CSI-RS in each of the P beams, gNB can use MP DFT FD bases for CSI-RS precoding and indicate the offset of the remaining M-MP FD bases via dynamic signaling to the UE.

# References

- RP-193133, “Further enhancements on MIMO for NR,” Samsung

- Chairman’s Notes, RAN1 #102e, e-Meeting, Aug. 17-28, 2020.

- Chairman’s Notes, RAN1 #103e, e-Meeting, Oct 26–Nov.13, 2020.

- R1-2006973, Discussion Summary for CSI enhancements MTRP and FR1 FDD reciprocity, RAN#102e, E-meeting, Aug. 17-28, 2020.

# Appendix

SLS assumptions for FR1 FDD reciprocity

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense urban macro |
| Frequency Range |  | 2 GHz with duplexing gap of 200 MHz between DL and UL |
| Inter-BS distance |  | 200 m |
| Channel model |  | Based on TR 38.901 with the reciprocity model of DL/UL channel in Section 5.3 of TR 36.897 |
| Antenna setup and port layouts at gNB |  | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ 100 deg subarray downtilt (zenith angle) |
| Antenna setup and port layouts at UE |  | 4RX: (2,1,2,1,1,1,1) |
| BS Tx power |  | 44 dBm |
| BS antenna height |  | 25 m |
| UE antenna height & gain |  | According to TR36.873 |
| UE receiver noise figure |  | 9 dB |
| Modulation |  | Up to 256QAM |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15kHz |
| Simulation bandwidth |  | 20 MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU/MU-MIMO adaptation with up to 2 rank |
| CSI feedback |  | CSI feedback periodicity: 5 ms Scheduling delay: 4 ms |
| Traffic model |  | FTP model 1 with packet size 0.5 Mbytes |
| Traffic load (Resource utilization) |  | 70% |
| UE distribution |  | 80% indoor (3km/h), 20% outdoor (30km/h) |
| UE receiver |  | MMSE-IRC |
| Baseline for performance evaluation |  | Rel-16 PS eTypeII |
| SRS modeling for UL channel estimation |  | SRS periodicity = 5 ms SRS error modelling according to Table A.1-2 in TR 36.897 with Δ = 9 dB |