3GPP TSG RAN WG1 Meeting #104e	R1-2100700

e-Meeting, January 25th – February 5th, 2021

Source: 	ZTE

Title: 	Draft TP for half-duplex operation

Agenda item:	7.2.12

Document for:	Discussion/Decision

# Introduction

During RAN1#104e, one LS was sent from RAN2 to RAN1 on half-duplex operation [1]. The following questions were asked by RAN2.

RAN2 noticed that while the capability half-DuplexTDD-CA-SameSCS-r16 (corresponding to RAN1 feature group 14-5) was introduced in RAN2#111-e, the corresponding configuration parameter (which was included in the RAN1 feature list in R1-2003190) has mistakenly not been introduced in the September 2020 version of the RRC specification (V16.2.0). RAN2 would like to inform RAN1 that the configuration parameter directionalCollisionHandling-r16 has now been introduced as part of ServingCellConfig, i.e. as a per-serving cell parameter. RAN2 also thought that the parameter should only apply within the same frequency range and cell group: That is, only the cells configured with directionalCollisionHandling-r16 within the same cell group and frequency range are considered for determining the corresponding R16 half-duplex behavior for each case. RAN2 would like to ask whether there should be additional network restrictions: currently the RAN2 field description assumes this would only apply to TDD CA for cells with the same SCS, but it was not clear if it would always apply to all TDD CA cells with the same SCS or whether it's up to network it is configured only for some cells?

In this contribution, we provide our analysis for the above questions raised by RAN2.

# Discussion

First of all, half-duplex operation is designed for CA. Thus, it is straightforward that the configuration is per cell group.

Typically, half-duplex operation is applied such that different carriers can share some of the components of the RF chain. FR1 and FR2 can’t share the same RF chain due to the big requirement difference. Thus, the half-duplex operation configuration is applied per frequency range.

Lastly, it seems doesn’t hurt to allow network to configure half-duplex operation to some of the cells, but not to all the TDD CA cells with the same SCS.

To reflect the information indicated in RAN2’s LS, the following TP is proposed.

Proposal 1: Adopt the following TP1 for half-duplex operation section 11.1 of TS38.213.

## TP1: ## 11.1 Slot configuration < ------------------------------ Other parts are omitted ------------------------------ > If a UE is scheduled by a DCI format to receive PDSCH over multiple slots, and if tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, indicate that, for a slot from the multiple slots, at least one symbol from a set of symbols where the UE is scheduled PDSCH reception in the slot is an uplink symbol, the UE does not receive the PDSCH in the slot. If a UE is scheduled by a DCI format to transmit PUSCH over multiple slots, and if tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, indicates that, for a slot from the multiple slots, at least one symbol from a set of symbols where the UE is scheduled PUSCH transmission in the slot is a downlink symbol, the UE does not transmit the PUSCH in the slot. If a UE - is configured with multiple serving cells with directionalCollisionHandling-r16 = 'enable' per frequency range, and - is not capable of simultaneous transmission and reception on any of the multiple serving cells, and - indicates support of capability for half-duplex operation in CA with unpaired spectrum, and - is not configured to monitor PDCCH for detection of DCI format 2-0 on any of the multiple serving cells, the UE determines a reference cell for a symbol as an active cell with the smallest cell index among serving cells where the symbol is configured as - downlink, or uplink, as indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated - uplink, if the symbol is flexible and the UE is configured to transmit SRS, PUCCH, PUSCH, or PRACH on the symbol - downlink, if the symbol is flexible and the UE is configured to receive PDCCH, PDSCH or CSI-RS on the symbol If a UE - is configured with multiple serving cells in a frequency band with directionalCollisionHandling-r16 = 'enable', and - is not capable of simultaneous transmission and reception on any of the multiple serving cells, and - indicates support of capability for half-duplex operation in CA with unpaired spectrum, and - is not configured to monitor PDCCH for detection of DCI format 2_0 on any of the multiple serving cells, the UE does not expect - a symbol to be indicated as downlink or uplink on the reference cell and as uplink or downlink on another cell, respectively, by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated, - tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigDedicated to indicate a symbol as downlink on the reference cell and to detect a DCI format scheduling a transmission on the symbol on another cell, and - to be configured by higher layers to receive PDCCH, PDSCH, or CSI-RS on a flexible symbol on the reference cell and to detect a DCI format scheduling a transmission on the symbol on another cell. If the reference cell and another cell for a UE operate in different frequency bands and if the UE - is configured with multiple serving cells with directionalCollisionHandling-r16 = 'enable' per frequency range, and - is not capable of simultaneous transmission and reception on any of the multiple serving cells, and - indicates support of capability for half-duplex operation in CA with unpaired spectrum, and - is not configured to monitor PDCCH for detection of DCI format 2-0 on any of the multiple serving cells, the UE - UE assumes symbol as flexible, is not required to receive higher layer configured PDCCH, PDSCH, or CSI-RS and not expected to transmit higher layers configured SRS, PUCCH, PUSCH, or PRACH, when tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated indicates symbol as downlink or uplink on the other cell and as uplink or downlink for the reference cell, respectively, - transmits a signal/channel scheduled by a DCI format on a symbol of the other cell when the symbol is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigDedicated for the reference cell, - is not required to receive a higher layer configured PDCCH, PDSCH, or CSI-RS on flexible symbols on the reference cell in a set of symbols, if the UE detects a DCI format scheduling a transmission on one or more symbols in the set of symbols on the other cell. If a UE - is configured with multiple serving cells with directionalCollisionHandling-r16 = 'enable' per frequency range, and - is not capable of simultaneous transmission and reception on any cell from the multiple serving cells, and - indicates support of capability for half-duplex operation in CA with unpaired spectrum, and - is not configured to monitor PDCCH for detection of DCI format 2-0 on any of the multiple serving cells, the UE - does not expect tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated for the reference cell to indicate a symbol as uplink and to detect a DCI format scheduling a reception on the symbol on another cell - does not expect to be configured by higher layers to transmit SRS, PUCCH, PUSCH, or PRACH on a flexible symbol on the reference cell and to detect a DCI format scheduling a reception on the symbol on another cell - does not transmit a PUCCH, PUSCH or PRACH that is configured by higher layers on a set of symbols on another cell if at least one symbol from the set of symbols is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or is a symbol corresponding to a PDCCH, PDSCH, or CSI-RS reception that is configured by higher layers on the reference cell - does not transmit a SRS that is configured by higher layers on a set of symbols on another cell if the set of symbols is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or corresponds to a PDCCH, PDSCH or CSI-RS reception that is configured by higher layers on the reference cell - does not receive a PDCCH, PDSCH or CSI-RS that is configured by higher layers on a set of symbols on another cell if at least one symbol from the set of symbols is indicated as uplink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or is a symbol corresponding to a SRS, PUCCH, PUSCH, or PRACH transmission that is configured by higher layers on the reference cell - assumes a symbol indicated as downlink or uplink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated on another cell to be flexible, if the UE is respectively configured by higher layers to transmit SRS, PUCCH, PUSCH, or PRACH or to receive PDCCH, PDSCH, or CSI-RS on the reference cell - does not expect to detect a first DCI format scheduling a transmission or reception on a symbol on a first cell and a second DCI format scheduling a reception or transmission on the symbol on a second cell, respectively

# Conclusion

In this contribution, the following TP is proposed to reflect the information indicated by RAN2’s LS.

Proposal 1: Adopt the following TP1 for half-duplex operation section 11.1 of TS38.213.

## TP1: ## 11.1 Slot configuration < ------------------------------ Other parts are omitted ------------------------------ > If a UE is scheduled by a DCI format to receive PDSCH over multiple slots, and if tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, indicate that, for a slot from the multiple slots, at least one symbol from a set of symbols where the UE is scheduled PDSCH reception in the slot is an uplink symbol, the UE does not receive the PDSCH in the slot. If a UE is scheduled by a DCI format to transmit PUSCH over multiple slots, and if tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, indicates that, for a slot from the multiple slots, at least one symbol from a set of symbols where the UE is scheduled PUSCH transmission in the slot is a downlink symbol, the UE does not transmit the PUSCH in the slot. If a UE - is configured with multiple serving cells with directionalCollisionHandling-r16 = 'enable' per frequency range, and - is not capable of simultaneous transmission and reception on any of the multiple serving cells, and - indicates support of capability for half-duplex operation in CA with unpaired spectrum, and - is not configured to monitor PDCCH for detection of DCI format 2-0 on any of the multiple serving cells, the UE determines a reference cell for a symbol as an active cell with the smallest cell index among serving cells where the symbol is configured as - downlink, or uplink, as indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated - uplink, if the symbol is flexible and the UE is configured to transmit SRS, PUCCH, PUSCH, or PRACH on the symbol - downlink, if the symbol is flexible and the UE is configured to receive PDCCH, PDSCH or CSI-RS on the symbol If a UE - is configured with multiple serving cells in a frequency band with directionalCollisionHandling-r16 = 'enable', and - is not capable of simultaneous transmission and reception on any of the multiple serving cells, and - indicates support of capability for half-duplex operation in CA with unpaired spectrum, and - is not configured to monitor PDCCH for detection of DCI format 2_0 on any of the multiple serving cells, the UE does not expect - a symbol to be indicated as downlink or uplink on the reference cell and as uplink or downlink on another cell, respectively, by tdd-UL-DL-ConfigurationCommon or by tdd-UL-DL-ConfigurationDedicated, - tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigDedicated to indicate a symbol as downlink on the reference cell and to detect a DCI format scheduling a transmission on the symbol on another cell, and - to be configured by higher layers to receive PDCCH, PDSCH, or CSI-RS on a flexible symbol on the reference cell and to detect a DCI format scheduling a transmission on the symbol on another cell. If the reference cell and another cell for a UE operate in different frequency bands and if the UE - is configured with multiple serving cells with directionalCollisionHandling-r16 = 'enable' per frequency range, and - is not capable of simultaneous transmission and reception on any of the multiple serving cells, and - indicates support of capability for half-duplex operation in CA with unpaired spectrum, and - is not configured to monitor PDCCH for detection of DCI format 2-0 on any of the multiple serving cells, the UE - UE assumes symbol as flexible, is not required to receive higher layer configured PDCCH, PDSCH, or CSI-RS and not expected to transmit higher layers configured SRS, PUCCH, PUSCH, or PRACH, when tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated indicates symbol as downlink or uplink on the other cell and as uplink or downlink for the reference cell, respectively, - transmits a signal/channel scheduled by a DCI format on a symbol of the other cell when the symbol is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigDedicated for the reference cell, - is not required to receive a higher layer configured PDCCH, PDSCH, or CSI-RS on flexible symbols on the reference cell in a set of symbols, if the UE detects a DCI format scheduling a transmission on one or more symbols in the set of symbols on the other cell. If a UE - is configured with multiple serving cells with directionalCollisionHandling-r16 = 'enable' per frequency range, and - is not capable of simultaneous transmission and reception on any cell from the multiple serving cells, and - indicates support of capability for half-duplex operation in CA with unpaired spectrum, and - is not configured to monitor PDCCH for detection of DCI format 2-0 on any of the multiple serving cells, the UE - does not expect tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated for the reference cell to indicate a symbol as uplink and to detect a DCI format scheduling a reception on the symbol on another cell - does not expect to be configured by higher layers to transmit SRS, PUCCH, PUSCH, or PRACH on a flexible symbol on the reference cell and to detect a DCI format scheduling a reception on the symbol on another cell - does not transmit a PUCCH, PUSCH or PRACH that is configured by higher layers on a set of symbols on another cell if at least one symbol from the set of symbols is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or is a symbol corresponding to a PDCCH, PDSCH, or CSI-RS reception that is configured by higher layers on the reference cell - does not transmit a SRS that is configured by higher layers on a set of symbols on another cell if the set of symbols is indicated as downlink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or corresponds to a PDCCH, PDSCH or CSI-RS reception that is configured by higher layers on the reference cell - does not receive a PDCCH, PDSCH or CSI-RS that is configured by higher layers on a set of symbols on another cell if at least one symbol from the set of symbols is indicated as uplink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or is a symbol corresponding to a SRS, PUCCH, PUSCH, or PRACH transmission that is configured by higher layers on the reference cell - assumes a symbol indicated as downlink or uplink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated on another cell to be flexible, if the UE is respectively configured by higher layers to transmit SRS, PUCCH, PUSCH, or PRACH or to receive PDCCH, PDSCH, or CSI-RS on the reference cell - does not expect to detect a first DCI format scheduling a transmission or reception on a symbol on a first cell and a second DCI format scheduling a reception or transmission on the symbol on a second cell, respectively

# Reference

- R1-2100023, LS on half-duplex operation, RAN1#104e.